3GPP TSG RAN WG1 #112bis-e			R1-2303710

e-Meeting, April 17th – 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on evaluation on NR duplex evolution

Agenda Item:	9.3.1

Document for: 	Discussion

- Introduction

At the RAN#94e meeting, a new SID [1] on “Study on evolution of NR duplex operation” was approved. The detailed objectives are as follows.

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss on the evaluation of NR duplex evolution.

- System level simulation

  - SLS calibration results

Based on simulation and calibration assumptions for the SLS that were agreed in the RAN1 meetings, the calibration has been performed for Urban Macro (FR1) and Dense Urban Macro (FR 2-1). Coupling loss with both large and small scale fading for gNB-UE, gNB-gNB, and UE-UE are shown in Fig.1, and detail assumption for the calibration is summarized in Appendix A.

- FR1 : Urban Macro

- FR2-1 : Dense Urban Macro

Figure 1. SLS calibration results

- Link Level Simulation for coverage metric

  - Evaluation assumptions for link level Simulation

At the RAN1#112 meeting, the assumptions for the evaluation using link level simulation (LLS) were discussed and following agreement was made [2].

Agreement Regarding the schemes for link level evaluation of PUSCH coverage performance, - For baseline legacy TDD, consider - Single slot PUSCH transmission - For SBFD, consider the following techniques of coverage enhancement: - Case 2: SBFD with PUSCH repetition type A - Case 3: SBFD with TBoMS PUSCH - Case 4: SBFD with PUSCH repetition type A and joint channel estimation - FFS: Joint channel estimation across SBFD and non-SBFD slots - Case 5: SBFD with TBoMS PUSCH and joint channel estimation - FFS: Joint channel estimation across SBFD and non-SBFD slots - UL coverage metrics are obtained using link budget template and TDD/SBFD required SINR for target data rate. Note: Evaluation accounts for different SINR level between SBFD and non-SBFD slots Agreement For link level evaluation of coverage performance for SBFD, the following interference components are added per each receive chain to the UL channel at SBFD symbols: - Self-interference, modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense similar to SLS. - Co-site inter-sector interference, modelled as additive white gaussian noise with fixed INR = - X dB based on assumption of co-site isolation αco-site - Inter-site gNB-gNB co-channel inter-subband CLI, - Alt-1: the value of interference power is selected according to the INR distribution drawn based on the statistics from SLS. - Alt-2: the value of interference power is determined based on the inter-site gNB-gNB co-channel inter-subband CLI model agreed for SLS taking into account the locations of victim gNB and several aggressor gNBs, and the gNB-gNB channel model FFS: Receiver blocking model

Regarding the schemes for link level evaluation of PUSCH coverage performance for SBFD, case 2 (SBFD with PUSCH repetition type A) has been selected for our initial LLS evaluation to see the baseline performance. For link level evaluation of coverage performance for SBFD, the presence of interference in SBFD and UL slots should be considered. The configuration of interference for SBFD and UL slots is as follows:

- SBFD: The slot is subject to interference from the DL subband

- UL: Since there is no DL subband allocation in the UL slot, the slot is not subject to interference from the DL subband

The respective interference modeling schemes based on above agreement in RAN1#112 are as follows:

- Self-interference is modelled as additive white gaussian noise with fixed INR = - 6 dB

- Co-site inter-sector interference is modelled as additive white gaussian noise with fixed INR = - 2 dB

- Inter-site gNB-gNB co-channel inter-subband CLI is modelled as additive white gaussian noise with fixed INR = 6dB for FR1, and INR = -15dB for FR2, and the value is calculated as follows:

  - Input signal power transmitted by aggressor gNB at victim gNB is calculated with using CDF of 95% for gNB-to-gNB coupling loss from SLS, and Tx power and antenna gain.

  - Interference level at UL subband is calculated using the formula defined for SLS.

Other evaluation assumptions are summarized in Appendix B.

  - Initial evaluation results

The evaluation results are shown in Figure 2.

(a)  FR1: Urban Macro

(b)  FR2-1: Dense Urban Macro Layer

Figure 2. Evaluation results of LLS

The evaluation results in FR1 did not show any gain by increased UL transmission opportunities. In this evaluation, the first PUSCH is transmitted in the non-SBFD slot (e.g., UL slot or non-SBFD flexible slot) and the succeeding PUSCH repetitions are transmitted in UL subband in the SBFD slots. Therefore, the results mean that the impact of performance degradation due to interference cancels the gain of PUSCH repetitions using SBFD slots, so that PUSCH repetitions using SBFD slots can’t contribute the improvement of coverage performance. On the other hand, in the case of FR2-1, the gNB transmission power is smaller and the gNB-to-gNB coupling loss is larger than that in FR1. Accordingly, the impact of inter-site gNB-gNB co-channel inter-subband CLI becomes smaller and the effect of interference itself is reduced. Thus, in the case of FR2-1, although the SBFD slot suffers from interference, the increased transmission opportunities resulted in a gain of about 4 dB.

Observation 1: In the case of FR1, SINR improvement is not found for SBFD operation with PUSCH repetition, since interference for SBFD is too strong.

Observation 2: In the case of FR2-1, SINR improvement of about 4 dB is expected for SBFD with PUSCH repetition assuming smaller inter-site interference in SBFD slot compared with that in FR1.

- Conclusion

In this contribution, we discussed on the evaluation of NR duplex operation. Based on the discussion we made the following proposals.

Observation 1: In the case of FR1, SINR improvement is not found for SBFD operation with PUSCH repetition, since interference for SBFD is too strong.

Observation 2: In the case of FR2-1, SINR improvement of about 4 dB is expected for SBFD with PUSCH repetition assuming smaller inter-site interference in SBFD slot compared with that in FR1.

References

- 3GPP, RP-213591, CMCC, “Study on evolution of NR duplex operation,” Dec. 2021.

- 3GPP, RAN1#112, RAN1 Chairman’s Notes, Mar. 2023.

Appendix A: Calibration parameters for SLS

Table A-1:  Calibration parameters for SLS.

|  | Urban Macro (FR1) | Dense Urban Macro (FR2-1) |
| --- | --- | --- |
| Carrier frequency | 4 GHz | 30 GHz |
| System bandwidth | 100 MHz | 100 MHz |
| Numerology | 14 OFDM symbol slot | 14 OFDM symbol slot |
|  | SCS = 30 kHz | SCS = 120 kHz |
| BS transmit power: Legacy TDD | 53 dBm for 100MHz | 40 dBm for 100 MHz |
| UE Tx power | 23 dBm | 23 dBm |
| Layout | Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |  |
| Inter-BS (2D) distance | 500 m | 200 m |
| Minimum BS-UE (2D) distance | 35 m | 35 m |
| Minimum UE-UE (2D) distance | 1 m | 1 m |
| UE distribution | UE Clustering | UE Clustering |
| BS antenna height | 25 m | 25 m |
| UE antenna height | 1.5 m | 1.5 m |
| gNB-UE Channel model (both large and small scale fading) | Macro-to-UE: UMa in TR 38.901; AAS and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 |  |
| gNB-gNB Channel model (both large and small scale fading) | Macro-to-Macro: UMa in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. |  |
| UE-UE Channel model (both large and small scale fading) | UE-to-UE: UMi-Street canyon in TR 38.901 |  |
| BS antenna array configuration for legacy TDD | (M,N,P,M_g,N_g;M_p,N_p ) = (8,8,2,1,1; 2,8) | (M,N,P,M_g,N_g;M_p,N_p ) = (4,16,2,2,2; 1,1) |
|  | (d_H,d_V )= (0.5, 0.8)λ, +45°/-45° polarization | (d_H,d_V )= (0.5, 0.5)λ, +45°/-45° polarization |
| BS antenna array configuration for SBFD | l Option 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD | l Option 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD |
| UE antenna configuration | 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1), (dH,dV) = (0.5, 0.5)λ, 0°,90° polarization |
|  | 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |  |

Appendix B: Evaluation assumptions for LLS

Table B-1:  Evaluation assumptions for LLS.

|  | FR1 |  | FR2 |  |
| --- | --- | --- | --- | --- |
| Categorize | Legacy TDD | SBFD | Legacy TDD | SBFD |
| Channel | Urban | Urban | Dense Urban | Dense Urban |
| Target data rate | 1 Mbps | 1 Mbps | 5 Mbps | 5 Mbps |
| Target metric | 10% iBLER | 10% iBLER | 10% iBLER | 10% iBLER |
| Center frequency | 4GHz |  | 28GHz |  |
| Frequency bandwidth | 100MHz |  | 200MHz |  |
| Maximum number of PRBs | 273 | 273 < D, U, G > = <104, 55, 5> | 132 | 132< D, U, G > = <52, 26, 1> |
| SCS | 30kHz |  | 120kHz | 120kHz |
| TDD pattern (DL : UL) | DDDSU | XXXXU | DDDSU | XXXXU |
| LoS/NLoS | NLoS | NLoS | - | - |
| Channel model | TDL-C | TDL-C | TDL-C | TDL-C |
| Delay spread | 300 | 300 | 100 | 100 |
| Mobile speed | 3 km/h | 3 km/h | 3 km/h | 3 km/h |
| PRB allocation | 55 | 55 | 26 | 26 |
| Symbol allocation (S, L) | S = 0, L = 14 | S = 0, L = 14 | S = 0, L = 14 | S = 0, L = 14 |
| Number of repetition | - | 4 | - | 4 |
| Number of TBoMS slot | - | - | - | - |
| Joint Channel Estimation | Disabled | Disabled | Disabled | Disabled |
| Frequency hopping (offset) | - | - | - | - |
| HARQ | Disabled | Disabled | Disabled | Disabled |
| Number of HARQ process | - | - | - | - |
| Number of layers | 1 | 1 | 1 | 1 |
| Number of Tx antennas | 2 | 2 | 2 | 2 |
| Number of Rx antennas | 4 | 4 | 2 | 2 |
| Number of codewords | 1 | 1 | 1 | 1 |
| Code rate per TB | 0.188476563 | 0.188476563 | 0.513671875 | 0.513671875 |
| Modulation | QPSK | QPSK | QPSK | QPSK |
| MCS (MCS table) | 8 | 8 | 13 | 13 |
| Channdel coding | LDPC | LDPC | LDPC | LDPC |
| Waveform | DFT-S-OFDM | DFT-S-OFDM | DFT-S-OFDM | DFT-S-OFDM |
| Estimated max data rate | 1.198 | 1.198 | 5.672 | 5.672 |
| DM-RS ports | 1000 | 1000 | 1000 | 1000 |
| PUSCH mapping type | Type A | Type A | Type A | Type A |
| DMRS Type A position | pos2 | pos2 | pos2 | pos2 |
| DMRS Length | 1 | 1 | 1 | 1 |
| Additional DM-RS symbol positions | pos1 | pos1 | pos2 | pos2 |
| DM-RS configuration type | Type1 | Type1 | Type1 | Type1 |
| Multiplexing with data | No | No | No | No |