3GPP TSG RAN WG1 #112bis-e			  R1-2303711

e-Meeting, April 17th – April 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for: 	Discussion and Decision

- Introduction

At RAN#95-e meeting, revised SID on NR duplex operation enhancements was approved with the objective as follows [1]:

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhance - ement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, possible solutions for subband non-overlapping full duplex and impact to legacy operation are discussed.

- Discussions

2.1 DL/UL subband time and frequency resource allocation

In Rel-15/16, gNB can configure/indicate a symbol as “DL” or “UL” or “flexible”, via time domain slot format indication. In Rel-18, with subband non-overlapping full duplex (SBFD) at gNB, both DL subband(s) and UL subband(s) can be configured/indicated on the same symbol.

2.1.1 SBFD DL/UL subband frequency location

During RAN1#110bis-e meeting, it was agreed that at least UL subband frequency domain location should be explicitly indicated. Besides UL subband, the frequency location of DL subband also needs to be known by UE. Explicit indication of DL subband frequency location is the most straightforward solution. Implicit indication is also possible. For example, if guard band between DL and UL subbands is not required, the DL subband frequency location can be implicitly derived based on the UL subband frequency location. If guard band is required, the DL subband frequency location can be determined based on UL subband frequency location and guard band location (or size). For this case, explicit indication of guard band size or location should be supported.

Proposal 1:

- If guard band between DL and UL subband in SBFD symbol is required, either DL subband  frequency location or guard band size should be explicitly indicated.

- If guard band between DL and UL subband in SBFD symbol is not required, DL subband  frequency location can be implicitly determined based on UL subband frequency location.

Dynamic indication of DL/UL subband frequency domain allocation will make it more difficult for higher-layer configured channels/signals to avoid overlapping with unavailable frequency resources, and also more difficult for UE to filter interference. Therefore, dynamic indication of DL/UL subband frequency domain allocation is not preferred.

Proposal 2: NOT support dynamic indication of DL/UL subband frequency domain location.

2.1.2 SBFD and non-SBFD symbols in one slot

At RAN1#112 meeting, it was agreed to study whether or not a slot can consist of SBFD and non-SBFD symbols.

The motivation to limit a slot not consisting SBFD and non-SBFD symbols comes from avoiding a channel/signal across SBFD and non-SBFD symbols in one slot. Some companies observed issues for this case, e.g. determination of transmission parameters for the channel/signal (e.g. MCS, FDRA, power, etc.) in the slot would be complicated.

In our view, it is too restricted if not allowing SBFD and non-SBFD symbols in one slot, especially considering the legacy TDD pattern configuration/indication is for per symbol granularity. For example, there may be slots consisting of DL and UL symbols. With this limitation, DL symbols in such slot is not expected to be indicated/configured for SBFD operation. Taking DDDSU (S with both DL and UL symbols) as an example, SBFD operation can only occur in DL slot. For this case, switching points in one periodicity include: D -> SBFD -> D -> U. However, if the SBFD symbol can be in the S slot, the switching point in one periodicity can be simpler: D -> SBFD -> U.

If it is necessary to avoid a channel/signal across SBFD and non-SBFD symbols in one slot, gNB can avoid scheduling/configuring such resource allocation, or a rule can be defined for such case, e.g. not receiving or transmitting channel/signal across SBFD and non-SBFD symbols in one slot.

Proposal 3: Support a slot consisting of both SBFD and non-SBFD symbols.

2.1.3 dynamic SBFD operation

At RAN1#112 meeting, whether/how to support dynamic SBFD operation in symbol configured as DL and flexible by TDD-UL-DL-ConfigCommon was discussed.

Agreement For dynamic SBFD, - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are not allowed - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are allowed Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency. For each option, additional conditions may apply to determine whether the option is applicable.

We think the potential benefit of dynamic SBFD is more flexibility. For example, gNB can adjust SBFD symbol location (e.g. dynamically enabling/disabling SBFD operation in certain symbol) according to timely DL/UL traffic characteristics. It may be useful for the case when multiple types of unpredictable services with quite different requirement/characteristics in the same system.

For dynamic SBFD operation in DL symbol, when a SBFD DL symbol configured with UL subband is dynamically indicated as non-SBFD symbol, it can be regarded as no DL or UL subband division on the symbol. Legacy DL symbol operation on the symbol can be reused. DL reception in any RB within DL BWP is allowed. Therefore, option 2 can be supported when gNB dynamically disable SBFD operation on a DL symbol.

For dynamic SBFD operation in flexible symbol, when a SBFD flexible symbol configured with UL subband is dynamically indicated as non-SBFD symbol, it can be regarded as no DL or UL subband division on the symbol. Legacy flexible symbol operation on the symbol can be reused. DL reception in any RB within DL BWP is allowed, and UL transmission in any RB within UL BWP is allowed. Therefore, option 3 is supported when gNB dynamically disable SBFD operation on a flexible symbol.

Proposal 4: Support dynamic SBFD for SBFD operation in DL symbol and flexible symbol.

- For SBFD DL symbol, option 2 is applied when the symbol is indicated as non-SBFD symbol by dynamic indication.

- For SBFD flexible symbol, option 3 is applied when the symbol is indicated as non-SBFD symbol by dynamic indication.

2.1.4 flexible symbol considering dedicated TDD pattern and SFI

In previous meetings, UE behavior in SBFD symbol configured as flexible by TDD-UL-DL-ConfigCommon was discussed. It was not touched whether SBFD operation can be supported on certain or all types of symbols configured as flexible by TDD-UL-DL-ConfigCommon. Companies think it can be separately discussed.

In our understanding, regarding whether/how to support SBFD operation in symbol configured as flexible by TDD-UL-DL-ConfigCommon, dedicated TDD configuration (i.e. TDD-UL-DL-Dedicated) and/or SFI indication need to be considered, since link direction of a symbol configured flexible in TDD-UL-DL-ConfigCommon may be changed by TDD-UL-DL-Dedicated or SFI.

- Case 1: a symbol configured as flexible in TDD-UL-DL-ConfigCommon and configured as DL in TDD-UL-DL-Dedicated

- Case 2: a symbol configured as flexible in TDD-UL-DL-ConfigCommon and configured as UL in TDD-UL-DL-Dedicated

- Case 3: a symbol configured as flexible in TDD-UL-DL-ConfigCommon and configured as flexible in TDD-UL-DL-Dedicated

For case 1, the symbol is a semi-static DL symbol from UE perspective. Same SBFD operation behavior as in SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon can be reused.

For case 2, the symbol is a semi-static UL symbol from UE perspective. SBFD operation in such symbol should be de-prioritized.

For case 3, the symbol is a semi-static flexible symbol from UE perspective, and UE may depend on SFI to determine the direction of the symbol. Three sub use cases can be considered.

- Case 3-1: UE is not configured to monitor SFI

- Case 3-2: UE is configured to monitor SBFD and the SFI indicates DL/UL/flexible for the symbol

- Case 3-3: UE is configured to monitor SBFD but UE doesn’t detect SFI for the symbol

For case 3-1, impact of SFI indication doesn’t need to be considered. SBFD operation can be supported in the symbol. For case 3-2, it is possible to support SBFD operation in SFI indicated DL/flexible symbol. Whether to support needs to consider aspects including flexibility, complexity, SFI missing issue, etc. For case 3-3, the symbol should not be used for DL or UL in legacy NR. Therefore, SBFD operation should not be supported in the symbol.

Proposal 5: Study whether/how to support SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon and configured as flexible in TDD-UL-DL-Dedicated.

- At least support SBFD operation in such symbol when SFI is not configured.

- Further study whether/how to support SBFD operation in symbol indicated as DL/flexible by SFI.

2.2 Potential enhancement on DL/UL reception/transmission

2.2.2 CSI-RS resource

At RAN1#112 meeting, it was agreed to study frequency resource allocation for CSI-RS across DL subbands.

Agreement: Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options: - Option 1: Two contiguous CSI-RS resources that are linked - Option 2: One CSI-RS resource - Option 2-1: Non-contiguous CSI-RS resource allocation - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

Both option 1 and option 2 can achieve CSI-RS frequency resource across non-contiguous DL subbands. For option 1, in order to better avoid overlapping with UL subband, non-contiguous RB allocation for CSI-RS or narrow band RB allocation for CSI-RS (e.g. smaller value than 24 for number of RBs) can be considered. Considering the limitation on number of configured CSI-RS resources, option 2 is slightly preferred.

The difference between option 2-1 and option 2-2 is whether to enhance the explicit CSI-RS frequency resource configuration, or to define UE rule to exclude unavailable resource. For option 2-1, it is more straightforward. And frequency resource for different CSI-RS occasions in SBFD or non-SBFD symbols are always same as the configured frequency resource. For option 2-2, it is possible that different CSI-RS occasions of the same CSI-RS resource index may have different frequency resources, e.g. CSI-RS occasions on SBFD symbols and CSI-RS occasions on non-SBFD symbols. In our view, different frequency resources for different CSI-RS occasions of the same CSI-RS resource index may bring some impact.  For example, if common CSI-RS resource configuration and/or common CSI reporting for non-SBFD and SBFD symbols, the case that different frequency resource for different CSI-RS occasions may be not good. For example, if frequency resource for CSI-RS used for channel measurement and CSI-RS used for interference measurement for the same CSI reporting are different, the CSI result may be not accurate. Therefore, we think option 2-1 is a simpler solution.

Proposal 6: Study one CSI-RS resource with non-contiguous frequency resource allocation in two DL subbands in SBFD symbols with higher priority.

2.2.1 Transmission/receptions across SBFD and non-SBFD symbols in different slots

At RAN1#112 meeting, it was agreed to study whether UL transmissions and DL receptions in different slots can be in SBFD symbols and non-SBFD symbols.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) - Study the following options for SBFD-aware UEs: - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following: - PDSCH/PUSCH/PUCCH repetitions - SPS PDSCH/CG PUSCH - TBoMS - Multi-PUSCH/PDSCH scheduled by a single DCI - Periodic/semi-persistent SRS/CSI-RS/PUCCH - PDCCH

For PDSCH/PUSCH/PUCCH repetitions or TBoMS PUSCH across SBFD and non-SBFD symbols in different slots, restricting transmissions/receptions to only SBFD symbols or only non-SBFD symbols may lead to latency issue or coverage/reliability issue. For example, the number of repetitions or number of TBoMS PUSCH slots would be limited if only considering consecutive non-SBFD/SBFD symbols. If non-consecutive non-SBFD/SBFD symbols can be considered, the latency would be increased. Therefore, Option 1 is not preferred.

For SPS PDSCH/CG PUSCH, PDCCH, and periodic/semi-persistent SRS/CSI-RS/PUCCH, option 1 can be realized by configuring separate SPS/CG/CSI-RS/SRS/PUCCH/PDCCH configurations for SBFD and non-SBFD symbols. Possible drawback of option 1 may be the limitation on number of SPS/CG/CSI-RS/SRS/PUCCH/PDCCH configurations.

Based on above analysis, option 2 is preferred for both cases.

If option 2 is applied, when the transmissions/receptions across SBFD and non-SBFD symbols, separate parameters (e.g. FDRA, FH, power control, etc.) for transmissions/receptions on SBFD symbols and on non-SBFD symbols may be beneficial.

Proposal 7: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, study potential enhancement to support transmissions/receptions can be in SBFD symbols and non-SBFD symbols.

2.2.3 CSI reporting

At RAN1#112 meeting, it was agreed to study whether same or separate CSI reporting for CSI report associated with periodic/semi-persistent CSI-RS across SBFD symbols and non-SBFD symbols in different slots.

Agreement: For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols): - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

Considering the interference level at SBFD symbol/slot and at non-SBFD symbol/slot may be different, separate CSI reporting for SBFD symbols and non-SBFD symbols would be beneficial.

To achieve separate CSI reporting, two possible methods can be considered:

- Alt 1: One CSI report includes only one type of measurement, i.e. one CSI report includes either CSI measurement result at non-SBFD symbol/slot or at SBFD symbol/slot. For example, the measurement result type is configured in CSI reporting configuration.

- Alt 2: One CSI report can include two types of measurement, i.e. one CSI report can include both CSI measurement results at non-SBFD symbol/slot and at SBFD symbol/slot.

Fig 1: Example of CSI reporting

As in the Fig 1 example, for CMR configuration with CRI#1, if using Alt 1, UE will report measurement result at either CSI-RS occasion #1 or CSI-RS occasion #2. If using Alt 2, for CMR configuration with CRI#1, UE will report measurement results at CSI-RS occasion#1 and at CSI-RS occasion #2 in one CSI reporting instance. The benefit of Alt 2 is CSI measurement for SBFD and for non-SBFD can be included in one reporting instance, thus leading to smaller number of CSI reporting instances than Alt 1. From simplicity perspective, Alt 1 is much simpler. From our perspective, Alt 1 is slightly preferred.

Proposal 8: For CSI report associated with periodic/semi-persistent CSI-RS across SBFD symbols and non-SBFD symbols in different slots, study separate CSI reporting for SBFD and non-SBFD symbols.

- Study whether one CSI reporting instance includes measurement results only for SBFD or non-SBFD symbol, or can include measurement results for both SBFD and non-SBFD symbols.

2.2.4 PUSCH/PUCCH

In previous meetings, potential issues/solutions for UL/DL transmissions/receptions collision with DL/UL subband(s) were discussed and identified, e.g. whether separate parameter configuration, partial PRG overlapping, available slot counting, PUSCH/PUCCH frequency hopping, etc. In this section, some issues still not addressed for PUCCH/PUSCH are proposed.

Segmentation for PUSCH repetition type B

For PUSCH repetition type B, nominal repetition may be segmented into actual repetitions around semi-static DL symbol, SSB symbol or RRC configured invalid symbol, as shown in Fig 2. Considering SBFD operation with UL subabnd in semi-static DL symbol, impact on PUSCH repetition type B segmentation should be studied.

Fig 2: Example of PUSCH repetition type B

Proposal 9: Study SBFD impact on PUSCH repetition type B segmentation.

SPS HARQ-ACK deferrring

For PUCCH, SPS HARQ-ACK deferring is introduced in Rel-17. When a PUCCH for SPS HARQ-ACK overlaps with semi-static DL symbol or SSB symbol, UE will defer the PUCCH until no overlapping with semi-static DL symbol or SSB symbol, as shown in Fig 3. Considering SBFD operation with UL subabnd in semi-static DL symbol, impact on SPS HARQ-ACK deferring should be studied.

Figure 3: Example of SPS HARQ-ACK deferring

Proposal 10: Study SBFD impact on SPS HARQ-ACK deferring.

2.2.5 collision of UL transmission in UL subband with DL reception in DL subband(s)

For such collision cases, the key issue is how to determine whether UE should perform DL reception or UL transmission in the same SBFD symbol.

Fig 4: Example of DL channel/signal in DL subband and UL channel/signal in UL subband in the same SBFD symbol

The simplest solution would be to avoid such case. However, it is too restricted, especially for higher layer configured channels/signals.

Another solution is that UE determines between DL reception or UL transmission in the symbol based on explicit indication of SBFD symbol direction. For example, gNB signals an explicit indication that the direction of the SBFD symbol is DL. In this case, UE will determine to cancel the UL transmission in the UL subband, and perform DL reception in DL subband of the SBFD symbol. This solution requires additional signal overhead for SBFD symbol direction.

Another solution is that UE determines between DL reception or UL transmission in the symbol based on predefined rules. This solution doesn’t require explicit indication for direction of SBFD symbol. And it is flexible to separately consider different rules for different cases. When defining rules, multiple types of collision cases can be further divided, based on whether scheduled or configured DL/UL channel/signal and with or without repetitions. For example, the DL channel/signal may be PDCCH, or scheduled DL channel/signal without repetition, or scheduled DL channel/signal with repetitions, or configured DL channel/signal with or without repetitions, and the UL channel/signal may be scheduled UL channel/signal without repetition, or scheduled UL channel/signal with repetitions, or configured UL channel/signal with or without repetitions. Any combination of DL channel/signal type and UL channel/signal type may need separate consideration.

Type 1 collision case: The DL and UL channels/signals are scheduled by DCI and no repetitions.

This case can be avoided by gNB scheduling.

Fig 5: DG DL vs. DG UL

Type 2 collision case: Only DL (or UL) channel/signal is scheduled by DCI and no repetitions, while UL (or DL) channel/signal is configured by higher layer (with or without DCI/MAC CE activation).

For this case, the DL (or UL) channel scheduled by DCI without repetitions can be prioritized. For example, if the DL channel/signal in DL subband in the symbol is higher layer configured, while a PUSCH in UL subband in the symbol is scheduled by DCI, UE can determine to transmit the PUSCH in UL subband in the symbol, and cancel DL reception in DL subband in the symbol.

Fig 6: DG DL (or UL) vs. CG UL (or DL)

Type 3 collision case: The DL and UL channels/signals are configured by higher layer.

For this case, certain rule can be defined based on DL/UL channel type and/or with or without DCI/MAC CE activation. For example, DL/UL channel/signal is prioritized over UL/channel/signal (e.g. SPS PDSCH prioritized over CG PUSCH), or certain channel/signal type is prioritized over other channel/signal types (e.g. control channel prioritized over data channel), or channel with DCI/MAC CE activation is prioritized over channel/signal with DCI/MAC CE activation (e.g. semi-persistent CSI-RS prioritized over periodic SRS).

Fig 7: CG DL vs. CG UL

Type 4 collision case: The DL and/or UL channels/signals are with multiple repetitions.

For this case, the latter repetitions other than the first repetition can be regarded as channel/signal without scheduling DCI. Then rules defined for other collision types can be applied.

For example, for multiple PDSCH repetitions scheduled by DCI in DL subband and PUSCH without repetitions scheduled by DCI in UL subband in the same SBFD symbol, the DG PUSCH without repetitions can be prioritized.

Fig 8: DL vs. UL with repetitions

Proposal 11: For DL channel/signal in DL subband and UL channel/signal in UL subband in the same SBFD symbol, study predefined rules to determine whether UE will receive in DL subband or transmit in UL subband in the same SBFD symbol.

- Type 1 collision case: The DL and UL channels/signals are scheduled by DCI without repetitions.

- Type 2 collision case: Only DL (or UL) channel/signal is scheduled by DCI without repetitions, while UL (or DL) channel/signal is configured by higher layer (with or without DCI/MAC CE activation).

- Type 3 collision case: The DL and UL channels/signals are configured by higher layer.

- Type 4 collision case: The DL and/or UL channels/signals are with multiple repetitions.

2.2.6 UL cancellation DCI

For UL cancellation DCI format 2_4, the time domain position for the indicated resources is obtained by excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon.

## 11.2A Cancellation indication If a UE is provided UplinkCancellation, the UE is provided, in one or more serving cells, search space sets for monitoring the first PDCCH candidate with a CCE aggregation level of LCI CCEs of each search space set for detection of a DCI format 2_4 [5, TS 38.212] with a CI-RNTI provided by ci-RNTI as described in clause 10.1. UplinkCancellation additionally provides to the UE - a set of serving cells, by ci-ConfigurationPerServingCell, that includes a set of serving cell indexes and a corresponding set of locations for fields in DCI format 2_4 by positionInDCI - a number of fields in DCI format 2_4, by positionInDCI-forSUL, for each serving cell for a SUL carrier, if the serving cell is configured with a SUL carrier - an information payload size for DCI format 2_4 by dci-PayloadSize-ForCI - an indication for time-frequency resources by timeFrequencyRegion For a serving cell having an associated field in a DCI format 2_4, for the field denote by - NCI a number of bits provided by ci-PayloadSize - BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, from a number of symbols that - is provided by timeDurationforCI in timeFrequencyRegion, if the PDCCH monitoring periodicity for the search space set with the DCI format 2_4 is one slot and there are more than one PDCCH monitoring occasions in a slot, or - is equal to the PDCCH monitoring periodicity, otherwise. - GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion

Considering SBFD operation with UL subband in DL symbol, there may be UL transmission in DL symbol. With the legacy rule, UL transmission in UL subband in SBFD symbol configured as DL can’t be cancelled by DCI 2_4. Therefore, impact on UL cancellation DCI format 2_4 should be studied.

Proposal 12: Study SBFD impact on UL cancellation DCI format 2_4.

2.3 SBFD operation in RRC idle/in-active mode

SSB and type-0 PDCCH are cell specific channel/signal for both legacy UEs without SBFD capability and SBFD capable UEs. Assuming DL and UL subbands on SSB or type-0 PDCCH symbol, it may cause issue for legacy UE without SBFD capability. Therefore, SBFD operation on SSB or CORESET#0 symbol should be avoided.

Proposal 13: Not support SBFD operation with UL subband on SSB symbol or CORESET #0 symbol.

In legacy RACH procedure, valid RO is determined based on semi-static TDD pattern configuration. RO overlapping with semi-static DL symbol would be regarded as invalid RO. For SBFD, when UL subband  is configured in a DL symbol, PRACH and Msg 3 PUSCH transmission in UL subband of the SBFD symbol would be beneficial for RACH procedure latency reduction. In legacy RACH procedure, UE doesn’t monitor RAR in symbol(s) with valid RO. If RO in UL subband in SBFD symbol is regarded as valid, it should not be precluded that UE can monitor RAR in DL subband of the symbol, when UE doesn’t transmit PRACH on any valid RO in the symbol.

Fig 9: ROs in SBFD symbol

Proposal 14:

- Support PRACH and Msg 3 PUSCH transmission in UL subband in SBFD symbol.

- Support UE monitoring RAR in DL subband in SBFD symbol, if UE doesn’t transmit PRACH in any valid RO on the symbol.

2.4 UE-to-UE CLI measurement and reporting

As guided by RAN1#109-e agreements, interference management enhancements common for dynamic TDD and SBFD will be discussed in AI 9.3.3, and interference management enhancements specific for SBFD will be discussed in AI 9.3.2, including gNB self-interference, UE-to-UE inter-subband CLI, and gNB-to-gNB inter subbband CLI.

At RAN1#112 meeting, it was agreed to study three methods for inter-subband CLI measurement.

Agreement For inter-UE inter-subband CLI measurement, study at least the following methods: - Method#1: victim UE measures RSSI within DL subband - FFS: Whether SINR can be measured - Method#2: victim UE measures RSRP of aggressor UE within UL subband - Method#3: victim UE measures RSSI within UL subband - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP.

For method #1, RSSI measurement within DL subband is already supported in Rel-16. Some SBFD specific aspects can also be considered. For example, for the legacy CLI-RSSI measurement, one contiguous subband can be configured for CLI-RSSI measurement resource. With SBFD operation, there may be non-contiguous DL subbands in SBFD symbol.

For method #2 and method #3, UE measurement in UL BWP is not supported in Rel-16. As UL subband is configured for UL transmission, when and how to perform CLI measurement in UL subband needs to be studied. For example, whether UE always performs CLI measurement within UL subband as long as such CLI measurement is configured, or additional conditions to enable UE measuring CLI within UL subband (e.g. triggered by DCI, or dynamic SBFD indication disables SBFD operation on the symbol, etc.)? And whether UE only measures SRS-RSRP/CLI-RSSI within the UL subband in SBFD symbol, or it’s possible to measure on resources within and outside UL subband in SBFD symbol? Regarding measurement RS and reporting framework for SRS-RSRP/CLI-RSSI measurement within ULsubband, Rel-16 SRS-RSRP and CLI-RSSI measurement RS and reporting framework, and/or Rel-18 enhanced SRS-RSRP/CLI-RSSI measurement RS and reporting framework can be the baseline.

Moreover, with SBFD operation, CLI interference level at different RB locations may vary. For method #1, #2, and #3, CLI reporting with finer granularity is beneficial to identify different interference levels at different RB locations. Therefore, multiple subbands for CLI measurement resource would be beneficial.

Proposal 15: For UE-to-UE CLI measurement,

- Support non-contiguous frequency resource configuration for CLI-RSSI measurement across DL subbands.

- Study when and how to measure RSRP/RSSI within UL subband, e.g. additional condition for UE to measure CLI within UL subband.

- Study measurement and reporting for CLI-RSSI within DL subband and CLI-RSSI/RSRP within UL subband, with multiple subbands as CLI-RSSI/RSRP measurement resource.

- Conclusion

In this contribution, we discussed subband non-overlapping full duplex. We have the following observations and proposals:

Proposal 1:

- If guard band between DL and UL subband in SBFD symbol is required, either DL subband  frequency location or guard band size should be explicitly indicated.

- If guard band between DL and UL subband in SBFD symbol is not required, DL subband  frequency location can be implicitly determined based on UL subband frequency location.

Proposal 2: NOT support dynamic indication of DL/UL subband frequency domain location.

Proposal 3: Support a slot consisting of both SBFD and non-SBFD symbols.

Proposal 4: Support dynamic SBFD for SBFD operation in DL symbol and flexible symbol.

- For SBFD DL symbol, option 2 is applied when the symbol is indicated as non-SBFD symbol by dynamic indication.

- For SBFD flexible symbol, option 3 is applied when the symbol is indicated as non-SBFD symbol by dynamic indication.

Proposal 5: Study whether/how to support SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon and configured as flexible in TDD-UL-DL-Dedicated.

- At least support SBFD operation in such symbol when SFI is not configured.

- Further study whether/how to support SBFD operation in symbol indicated as DL/flexible by SFI.

Proposal 6: Study one CSI-RS resource with non-contiguous frequency resource allocation in two DL subbands in SBFD symbols with higher priority.

Proposal 7: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, study potential enhancement to support transmissions/receptions can be in SBFD symbols and non-SBFD symbols.

Proposal 8: For CSI report associated with periodic/semi-persistent CSI-RS across SBFD symbols and non-SBFD symbols in different slots, study separate CSI reporting for SBFD and non-SBFD symbols.

- Study whether one CSI reporting instance includes measurement results only for SBFD or non-SBFD symbol, or can include measurement results for both SBFD and non-SBFD symbols.

Proposal 9: Study SBFD impact on PUSCH repetition type B segmentation.

Proposal 10: Study SBFD impact on SPS HARQ-ACK deferring.

Proposal 11: For DL channel/signal in DL subband and UL channel/signal in UL subband in the same SBFD symbol, study predefined rules to determine whether UE will receive in DL subband or transmit in UL subband in the same SBFD symbol.

- Type 1 collision case: The DL and UL channels/signals are scheduled by DCI without repetitions.

- Type 2 collision case: Only DL (or UL) channel/signal is scheduled by DCI without repetitions, while UL (or DL) channel/signal is configured by higher layer (with or without DCI/MAC CE activation).

- Type 3 collision case: The DL and UL channels/signals are configured by higher layer.

- Type 4 collision case: The DL and/or UL channels/signals are with multiple repetitions.

Proposal 12: Study SBFD impact on UL cancellation DCI format 2_4.

Proposal 13: Not support SBFD operation with UL subband on SSB symbol or CORESET #0 symbol.

Proposal 14:

- Support PRACH and Msg 3 PUSCH transmission in UL subband in SBFD symbol.

- Support UE monitoring RAR in DL subband in SBFD symbol, if UE doesn’t transmit PRACH in any valid RO on the symbol.

Proposal 15: For UE-to-UE CLI measurement,

- Support non-contiguous frequency resource configuration for CLI-RSSI measurement across DL subbands.

- Study when and how to measure RSRP/RSSI within UL subband, e.g. additional condition for UE to measure CLI within UL subband.

- Study measurement and reporting for CLI-RSSI within DL subband and CLI-RSSI/RSRP within UL subband, with multiple subbands as CLI-RSSI/RSRP measurement resource.

References

- RP-220633, Revised SID: Study on evolution of NR duplex operation, March 2022, CMCC

- 3GPP, RAN1#109-e meeting, Chairman’s notes

- 3GPP, RAN1#110 meeting, Chairman’s notes

- 3GPP, RAN1#110bis-e meeting, Chairman’s notes

- 3GPP, RAN1#111 meeting, Chairman’s notes

- 3GPP, RAN1#112 meeting, Chairman’s notes