3GPP TSG RAN WG1 #112bis-e	R1-2303714

e-Meeting, April 17th – 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on channel design framework in SL-U

Agenda Item:	9.4.1.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#112 meeting [1], there was discussion on channel design framework in SL-U. In this contribution, we share our further views on channel design framework in SL-U.

- Discussions

  - PSCCH/PSSCH structure

    - Sub-channel definition / indication

Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - 1 sub-channel equals K interlace - FFS: whether K is fixed as 1 or (pre-)configured - Discuss whether one or both of the following alternatives are supported - Alt 1: 1 sub-channel is confined within 1 RB set - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding 1 sub-channel equals K interlace(s) - At least K=1 and K=2 is supported for 15 kHz SCS - At least K=1 is supported for 30 kHz SCS - FFS: details related to multiple RB sets Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - When more than one RB set is used for transmissions, down-select one of the followings - Option A: Support that the used interlace index(s) in different RB sets are always the same - Option B: Support that the used interlace index(s) in different RB sets can be different - FFS details Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - Down-select one of the followings - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s) - Option 2: Support explicitly indicating at least the used sub-channel index(s) - At least RB set index(s) is not explicitly indicated - FFS details Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

As agreed, each sub-channel is defined and indexed per RB set. Under this definition, one important aspect to be solved is sub-channel – interlace association. In our view, the same interlace index should be associated with the same sub-channel index across different RB sets for PAPR reduction. For example, when sub-channel 0 is allocated for both RB-set 0 and RB-set 1, sub-channel 0 at RB-set 0 and sub-channel 0 at RB-set 1 should be associated with the same interlace. This can achieve lower PAPR. There is no motivation to allow the other definitions.

Fig.1: Sub-channel – interlace association

Proposal 1:

- For each RB set, sub-channel 0 is defined with interlace 0, sub-channel 1 is defined with interlace 1, and so on.

Besides, sub-channel indication mechanism is another important aspect. According to the agreed sub-channel definition, at least indication of sub-channel index(s) and RB-set index(s) is necessary. For how to indicate sub-channel(s) and RB set(s), possible directions would be the following.

- 1) A single set of sub-channels and one or more RB sets are indicated, i.e., selected sub-channel indexes for transmission across multiple RB sets are common among the RB sets

- 2) A set of sub-channels per RB set and one or more RB sets are indicated, i.e., selected sub-channel indexes for transmission across multiple RB sets can be different among the RB sets

Key point would be overhead, flexibility, and PAPR. We prefer good balance among these points. 1) is too restrictive from flexibility perspective, while 2) can achieve better flexibility; we believe that this flexibility performance is important since SL resource allocation flexibility is not so high compared to NR-U scheduling and UE autonomous resource selection in low resource granularity leads to quite poor resource efficiency. However, 2) requires sub-channel indication field per RB-set and as a result 2) is worse than 1) from overhead perspective. Regarding PAPR performance, 1) achieves good PAPR always while 2) can achieve the same PAPR if the same interlace is selected across multiple RB-sets and leads to higher PAPR if different interlace is used across multiple RB-sets.

Considering the above performance analysis, we believe that a middle-ground way between 1) and 2) is the best way. That is, a set of sub-channels per M RB sets and which RB sets are used are indicated. In this way, overhead is lower than 2) and at the same time flexibility can be better than 1). This is illustrated below with M = 2 in case of an RP is defined with 4 RB-sets. Here, ‘sub-channel 0’ for RB-set 0 and RB-set 1 together is indicated, and ‘sub-channel 4’ for RB-set 2 and RB-set 3 together is indicated, in a SCI.

Fig.2: Sub-channel indication per M RB sets

Proposal 2:

- For PSCCH/PSSCH resource indication, the number N of RB sets, starting RB set, and a set of sub-channels per M RB sets are indicated in a SCI.

  - M is (pre-)configured.

    - Intra-cell guard band (ICGB) usage

Usage of ICGB is a remaining issue to be solved. In our understanding, when the same interlace is used for transmissions across multiple adjacent RB sets, it would be straightforward to use the corresponding PRBs within the ICGB. When different interlace is used for transmissions across multiple adjacent RB sets, PRBs corresponding to either RB-set can be used from the ICGB.

Meanwhile, the key point in the operation is that TBS determination should not consider these PRBs. For instance, the initial TX of a TB is performed over multiple RB-sets. Then, retransmission of the same TB can be performed within only a single RB-set. If the number of PRBs within the ICGB is used for TBS determination, the total number of PRBs becomes different between the initial TX and the retransmission. In SL, reserved MCS indices are not available. Easiness to derive the same TBS value for transmissions is quite important.

Proposal 3:

- When a PSCCH/PSSCH is mapped across multiple adjacent RB sets and

  - If the same interlace index is used among the RB sets, PRBs in the same interlace within the intra-cell guard band are used.

  - If different interlace index is used among the RB sets, PRBs corresponding to either interlace (e.g., interlace at RB set with smaller index) within the intra-cell guard band are used.

- In TBS determination, the number of PRBs for a PSCCH/PSSCH TX does not include the number of PRBs allocated from intra-cell guard band.

    - Channel/signal mapping

Agreement For PSCCH and PSSCH in SL-U: - PSCCH is transmitted within 1 sub-channel - At least support Option 1 below - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH - Note: the lowest sub-channel may not be entirely contained in the lowest RB set - FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below - Option 2: PSCCH locates in every RB set of corresponding PSSCH - Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs. - FFS other details

For interlaced structure, how to map PSCCH/PSSCH including RS needs to be clarified.

      - Control/Data/RS mapping

In interlaced RB-based structure, how to map control/data/RS is an issue to be solved. In short, our view is that the conventional mapping is applied to IRBs and then IRB-to-PRB mapping is performed. We do not see any issue on this simplest way. Other procedure would be large spec impact; thus the above procedure should be supported unless critical issue is found.

Fig.3: Control/Data/RS mapping for interlaced RB-based structure

Proposal 4:

- For interlaced RB-based structure, PSCCH/PSSCH is mapped on IRBs as on PRBs in Rel-16/17, and then IRB-to-PRB mapping is performed.

    - Maximum 2 candidate starting symbols

Maximum 2 candidate starting symbols were agreed as a working assumption in order to solve resource efficiency issue due to transmission cancellation due to LBT failure. For details, the several agreements were reached additionally. Furthermore, we think there are at least five issues to be discussed.

      - Availability

Agreement Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH.

Although a mechanism of max 2 candidate starting symbols was agreed, our understanding is that allowing TX from the 2nd starting position without any restriction was not intended. For example, allowing TX from the 2nd starting position without TX attempt from the 1st starting position is not reasonable way in consideration of the abovementioned motivation of this feature. Based on this, the following restriction in addition to the agreed restriction should be clearly described in specifications.

Proposal 5:

- 2nd starting symbol is available only when the following condition is met.

  - The UE attempted the transmission from the 1st starting symbol, but the corresponding LBT was failed.

      - Wideband operation

For a transmission across multiple RB sets, e.g., partial TX from the 1st starting position and the remaining TX from the 2nd TX starting position would not be intended for this feature. Although it may be understood as this is clear according to the previous agreement for wideband operation not considering two candidate starting symbols, we believe that it would be better to agree the following proposal explicitly to avoid misalignment among companies.

Fig.4: Usage of two candidate starting symbols in case of multiple RB-sets

Proposal 6:

- When a transmission is performed across multiple RB sets,

  - If the 1st starting position is available on all the RB sets, the TX is performed from the 1st starting symbol.

  - If the 1st starting position is not available on at least one of the RB sets, the 2nd starting symbol is attempted for the TX.

    - If the 2nd starting position is available on all the RB sets, the TX is performed from the 2nd starting symbol; otherwise, the TX is dropped.

      - PSCCH/PSSCH structure for TX from the 2nd starting symbol

Although several agreements were reached for maximum 2 candidate starting symbols, exact PSCCH/PSSCH structure has not identified yet. In our understanding, the following two structures are considerable:

- 1) PSCCH/PSSCH is generated for the number of PSCCH/PSSCH symbols according to the 2nd starting symbol.

- 2) PSCCH/PSSCH is generated for the number of PSCCH/PSSCH symbols according to the 1st starting symbol and the last several symbols over the slot are cut-off.

These two structures can be illustrated as below. The latter would be common understanding, thus we suggest the following proposal for clarification.

1)	 						2)

Fig.5: Two possible PSCCH/PSSCH generation for TX from the 2nd starting symbol

Proposal 7:

- For PSCCH/PSSCH transmissions starting from the 2nd starting symbol (Xth symbol),

  - PSCCH/PSSCH is generated according to the number of the actual PSCCH/PSSCH transmission, i.e., duration between Xth symbol and 13th symbol.

      - (Pre-)configuration parameters

In R16/17 SL, each parameter is (pre-)configured according to the starting symbol and the number of SL symbols. When two starting symbols are available, it would be difficult to optimize the parameters. For example, transmission starting from the 2nd starting symbol does not have sufficient amount of resources compared to transmission starting from the 1st starting symbol. Then, a possible solution is to allow different number of PSCCH symbols. Three symbols PSCCH is used for transmission from the 1st starting symbol, and two symbols PSCCH is applied for transmission from the 2nd starting symbol. Similar discussion can be considered for other parameters.

Fig.6: Example of separate parameters for two candidate starting symbols

Proposal 8:

- At least the following parameters are (pre-)configured for each of the 1st starting symbol and the 2nd starting symbol.

  - SL-PSCCH-Config (e.g., sl-TimeResourcePSCCH, sl-FreqResourcePSCCH)

  - SL-PSSCH-Config (e.g., sl-Scaling)

  - SL-MinMaxMCS-List

  - sl-PowerControl (e.g., sl-P0-PSSCH-PSCCH)

  - SL-CSI-RS-Config (e.g., sl-CSI-RS-FirstSymbol)

      - AGC symbols

When two starting symbols are available, one important issue is 2nd AGC symbol. If all SL UEs are aligned for starting symbol at the same slot, there is no need to define 2nd AGC symbol. However, we believe that the situation is not guaranteed. In SL, due to hidden-node issue, it could occur that some UEs start from 1st starting symbol and other UEs start from 2nd starting symbol within the same slot. From this point, it would be true that RX UE always needs to perform AGC twice within a slot.

Observation 1:

- Due to hidden-node issue, it could occur that some UEs start from 1st starting symbol and other UEs start from 2nd starting symbol within the same slot.

Meanwhile, this 2nd AGC operation within a slot at RX UE side does not mean that new AGC symbol shall be defined. Rather, from TX UE perspective, the issue is that the symbol for the 2nd AGC may not be used for data-decoding at RX UE side. What TX UE should do is to consider the 2nd AGC behavior in TBS/MCS/coding-rate determination, and thus there is no necessity to define explicit 2nd AGC symbol (i.e., symbol copy is unnecessary). For resource mapping, TX UE just follows the R16/17 mechanism, and RX UE may use the mapped data at the 2nd AGC timing, may not. It is up to RX UE implementation.

Fig.7: 2nd AGC issue – no spec change of TX UE is necessary

Proposal9:

- When two starting symbols are available,

  - It is assumed from TX UE that RX UE always performs 2nd AGC at the 2nd starting symbol

    - Note: RX UE can use the 2nd starting symbol for decoding by up to UE implementation

  - If a TX UE will transmit a PSCCH/PSSCH from the 1st starting symbol, no dedicated 2nd AGC symbol is defined (i.e., no symbol copy around the 2nd starting symbol)

    - Note: TX UE can consider the 2nd AGC in TBS/MCS/coding-rate determination

  - Note: no spec impact is assumed

      - Processing time constraints

When TX is started from the 2nd starting symbol, timing of completing decoding of each channel would change. For example, PSSCH decoding timing may be later compared to the case where TX is started from the 1st starting symbol since decoding completion of 1st SCI and 2nd SCI will be later. Correspondingly, PSCCH/PSSCH to PSFCH slot offset should be larger than the existing sl-MinTimeGapPSFCH values. Besides, PSCCH decoding timing will be later since PSCCH location becomes later. Timing of obtaining reservation information becomes later correspondingly; hence ending timing of sensing window should be modified so that excessively small processing time is not required.

Fig.8: Issue - existing processing time constraints with 2nd starting symbol
(upper: PSSCH to PSFCH, lower: sensing to exclusion)

Proposal 10:

- Update the following processing time constraints.

  - PSCCH/PSSCH to PSFCH slot offset is sl-MinTimeGapPSFCH + 1.

  - The last monitored PSCCH occasion for a resource selection is the first PSCCH occasion at the last slot within the sensing window.

    - TBS determination

Working assumption If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - At least for COT initiation, TBS is determined based on a reference number of symbols as follows: - Option 4: The reference number of symbols is determined by (pre-)configuration - FFS: value range - FFS: whether a different reference number of symbols is needed for transmission in a shared COT

The above working assumption was reached for TBS determination in case of max 2 candidate starting symbols. In our view, correspondingly, how to use ‘PSFCH overhead indication’ field in SCI-1 is now an issue to be discussed.

In R16/17 SL, which length is used for TBS determination is dynamically indicated. A single TB can be transmitted at both non-PSFCH slot and PSFCH slot, and the dynamic indication is used for TBS alignment for such a case. A: PSCCH/PSSCH duration at a non-PSFCH slot; B: PSCCH/PSSCH duration at a PSFCH slot. When a (pre-)configured reference number of symbols is used for TBS determination, usage of this indication field becomes unclear.

Our understanding of why the reference number is introduced is that TBS alignment is achieved in any case; that is, further 3 symbols subtraction seems to be unnecessary. Such a unrequired field can be removed to improve PSCCH decoding performance while the field size is 1 bit as maximum.

Proposal 11:

- When 2 candidate starting symbols are available, ‘PSFCH overhead indication’ field in SCI-1 is always 0 bit.

Besides, one FFS is that whether a different reference number of symbols is needed for transmission in a shared COT. An important aspect for TBS determination is to maintain TBS among (re)transmissions for a TB. A TB can be transmitted in both a COT initiating PSCCH/PSSCH and PSCCH/PSSCH within a shared COT. Different reference number for a shared COT leads to TBS different from that for a COT initiating PSCCH/PSSCH

Proposal 12:

- Not support a different reference number for a shared COT.

    - Contiguous RB-based

Another remaining issue is contiguous RB-based structure. Although contiguous RB-based structure would be available in some scenarios, we believe that contiguous RB-based structure should be deprioritized in R18 SL-U in consideration of the remaining available time for R18 discussion. Interlace RB-based structure is applicable for any situation (while it may not be optimal for some scenarios), and thus to support contiguous RB-based structure is not essential.

Proposal 13:

- Deprioritize contiguous RB-based structure in R18 SL-U discussion.

  - PSFCH structure

    - Too small PSFCH capacity

Agreement To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details. Agreement Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace - Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U - Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - FFS: value of K2, K4 - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case - FFS: whether/how to reduce PAPR of PSFCH transmission

One issue on PSFCH format with interlaced structure as PUCCH format 0 in NR-U is small PSFCH capacity. In Rel-16/17, when X PRBs and Y CSs are available for PSFCH, there are XY PSFCH resources in a PSFCH occasion. However, if the interlaced structure of PUCCH format 0 is reused for PSFCH format 0 without any enhancement (Alt 2-3a), each PSFCH resource occupies multiple PRBs and hence available PSFCH resources become much less than XY. In NR-U, the capacity issue is not critical since any time-frequency resources are available for PUCCH; but this is not the case in SL-U, where PSFCH can be transmitted at only (pre-)configured and associated PSFCH occasion.

Firstly, we share our view on Alt 2-3a/2-4a/3-2a below:

- Alt 2-3a: This alternative seems to lead to two issues.

  - Capacity: Abovementioned capacity issue is critical and thus enhancement should be introduced rather than just using one interlace.

    - For example, when PSFCH periodicity = 4 slots, SCS = 30 kHz, and 5 sub-channels are available for PSSCH, each PSFCH occasion has 30 = 5 interlaces x 6 cyclic shifts as maximum. Each PSFCH occasion requires to accommodate all PSFCH transmissions corresponding to 20 PSSCH resources = 4 slots x 5 sub-channels. The following issues are non-negligible:
A) Support of GC option 2. Only 30 resources are available in each PSFCH occasion for 20 PSSCH resources. There is no sufficient resource for GC option 2 even for only 2 UEs per group. 
B) Interference management. In R16/17, the number of CSs in each PRB can be (pre-)configured from 1/2/3/6. If lower interference is essential in a scenario and 1/2/3 is configured for the number of CSs, less than 20 resources are available in each PSFCH occasion for 20 PSSCH resources. ‘6’ must be selected always.

  - Spec impact: Although this alternative is proposed from perspective of smaller spec impact, we do not think the spec impact is not so small.

    - In Rel-16/17, a PSSCH resource is associated with one or more dedicated PSFCH PRBs. In other words, CDM is not applied between PSFCH resources for PSSCH-resource A and PSFCH resources for PSSCH-resource B. However, as analyzed above, it seems that CDM is essential between PSFCH resources for PSSCH-interlace A and PSFCH resources for PSSCH-interlace B. Legacy mapping of PSSCH-PSFCH is not applicable and hence anyhow large spec impact is assumed.

- Alt 2-4a: It seems that this mechanism does not solve the capacity issue but is a solution to obtain lower PAPR. We are fine to discuss Alt 2-4a for better PAPR performance, but the discussion should be done separately.

- Alt 3-2a: this alternative leads to high PSD which would violate PSD limit. Although multiple common PRBs at both edges can be used for lower PSD, the solution degrades capacity performance.

Observation 2:

- For Alt 2-3a of PSFCH structure, it seems that there are at least the following two issues:

  - Capacity issue: quite difficult to support GC option2, impossible to configure smaller number of CSs for a scenario of high frequency selectivity

  - Spec impact: impossible to apply legacy mapping of PSSCH-PSFCH (i.e., separate PSFCH PRB(s) per PSSCH resource)

Proposal 14:

- For PSFCH structure,

  - Drop Alt 2-3a/3-2a.

  - Separately discuss Alt 2-4a with respect to PAPR reduction.

Then, our view on Alt 1-1a/2-2a is the following:

- Alt 1-1a: In our understanding, all PRBs from the common interlace shall be used in this alternative. This implies that power allocated for dedicated PRBs cannot be high. Detection performance is limited. This is true even when K3 becomes larger.

- Alt 2-2a: It was pointed out that large interface from common CS to dedicated CS is observed. This issue seems true; thus, the same PRB as that for common PRB should not be used for a dedicated transmission.

To solve this situation, we believe that Alt 1-1a and Alt 2-2a should be combined into a single solution. A key concept of Alt 2-2a is to achieve capacity enhancement without increasing of the number of PRBs for a PSFCH TX to avoid smaller power allocation for information TX part. This mechanism can solve the issue on Alt 1-1a.

A combination of Alt 1-1a and Alt 2-2a is illustrated as below. A common interlace with N PRBs is defined for any UE’s PSFCH transmission. Then K3 dedicated PRBs with a dedicated cyclic shift are allocated for each UE’s TX, and at the same time K3 PRBs from the N PRBs are not used for each UE’s TX. That is, a UE transmits total N PRBs for a PSFCH, where K3 PRBs are dedicated for the PSFCH and (N–K3) PRBs are common among UEs. How (N–K3) PRBs are used from the common interlace should be determined such that PSD is lower. A PRB that belongs to the common interlace and is the closest to one of K3 dedicated PRBs should be one of (N–K3) PRBs.

- K3 = 1

- K3 = 2

Fig.9: A combination of Alt 1-1a and Alt 2-2a

Proposal 15:

- For interlaced PSFCH format 0, support a combination of Alt 1-1a and Alt 2-2a.

  - Each PSFCH transmission occupies the following

    - K3 dedicated PRB(s) with a dedicated cyclic shift among PRBs not used for common interlace.

    - PRBs within 1 common interlace, except for PRBs within the common interlace that are the closest to each of K3 dedicated PRB(s); i.e., PRBs within the common interlace that are the closest to each of K3 dedicated PRB(s) are dropped/not used.

    - Location of PSFCH resources

One key point in SL-U would be whether different RB-sets (LBT channels) between PSSCH and PSFCH is allowed or not. In Rel-16/17, naturally there is no restriction on the association from perspective of frequency-domain resource. However, in SL-U, it would be valid that the same RB-set is better so that the two TXs can be performed within the same COT. If they are at different RB-sets, basically COT sharing cannot be applied for the two TXs and as the result, PSFCH transmission skipping due to LBT failure occurs more frequently.

Proposal 16:

- The same RB-set is assigned for PSCCH/PSSCH resource and the corresponding PSFCH resource.

    - PSFCH drop due to LBT failure

Agreement For PSFCH and SL-HARQ in SL-U: - At least R16 NR SL PSFCH format 0 is supported - FFS whether to introduce new PSFCH format - FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc. - FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc. - FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc. - FFS: whether/how to address PSFCH and related PSSCH in different COTs Agreement To address PSFCH transmission dropping due to LBT failure: - Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Down-select one or support both of the followings - Option 1: Such PSFCH occasion(s) are (pre-)configured - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc. - FFS other details

In SL-U, any transmission is failed more frequently compared to in licensed spectrum or ITS-band due to LBT failure. Frequent PSFCH dropping would be a big issue since corresponding PSSCH retransmissions will occur and thus, performance of resource efficiency and reliability/latency becomes worse. Some enhancement to solve this issue would be necessary. Based on this, now we have two options as solution of the issue.

From these options, our preference is Option 1. Option 2 causes too complicated PSFCH conflict avoidance. R16/17 PSFCH mechanism, i.e., fixed association between PSCCH/PSSCH and PSFCH, was made such that PSFCH resource conflict is avoided without reservation/sensing-like feature. It was concluded that dynamic mechanism is difficult/quite complicated for SL where there is no center scheduler unlike DL/UL. We do not think this situation is changed.

For details of Alt 1, the association concept should be maintained. That is, PSFCH resource overlap among UEs for different PSSCH transmissions should be avoided by spec definition and (pre-)configuration.

Proposal 17:

- For determination of PSFCH occasion, support Option 1 with the following note.

  - Note: PSFCH resources associated with a PSSCH resource are not overlapped with PSFCH resources associated with other PSSCH resources.

  - S-SSB structure

Agreement For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc. - FFS gap of 0 - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS. Agreement Regarding S-SSB, RAN1 further study the following: - How to transmit S-SSB when a SL BWP contains multiple RB sets

    - Structure

For S-SSB structure, we believe that interlaced structure is better since higher energy per RE can bring better S-SSB detection performance. We do not see any motivation to apply interlaced structure only for PSBCH; thus Option 1-1 should be supported.

On interlaced S-SSB structure, each symbol of the conventional S-SSB is composed of 11 PRBs (PSBCH) or 127 Res (S-PSS/S-SSS). There are cases where interlaced S-SSB structure can be mapped within the available PRBs of a single 20 MHz; for example, when a single 20 MHz is available with 30 kHz SCS, 51 PRBs are available at the channel as defined in 38.101-1, and 11 PRBs/127 REs can be mapped within the 51 PRBs with intervals of 5 PRBs. This is illustrated below. Note that 50 PRBs should be (pre-)configured for a resource pool so that unequal sub-channel size is avoided.

Fig.10: Option 1-1 when only a single RB-set is defined.

On the other hand, 38.101-1 defines for NR-U that 49/50/51 PRBs are configurable for each RB-set except for intra-cell guard when wideband operation can be performed. For transmission of the interlaced S-SSB structure, it seems that 51 PRBs should be available at the RB-set. However, if 51 PRBs are (pre-)configured for each RB-set, this leads to unequal sub-channel size, which should not be allowed as in R16/17. It would be impossible to avoid this situation by RP (pre-)configuration. Therefore, we believe that a possible way is that 50 PRBs are defined/(pre-)configured for each RB-set, and S-SSB transmission can use one adjacent PRB outside of an RB-set.

Fig.11: Option 1-1 when two RB-sets are defined.

Proposal 18:

- For S-SSB structure, support Option 1-1.

  - When 30 kHz SCS and multiple RB sets are configured, interlaced S-SSB is mapped within an RB-set and a PRB within ICGB adjacent to the RB-set.

On exemption of OCB requirement, we believe that the exemption should be deprioritized to avoid complex operation/specification.

Proposal 19:

- Exemption of OCB requirement is not applied to S-SSB TX.

    - Additional candidate S-SSB occasions

Agreement Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings: - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured - FFS details, e.g., value of K, details on gap length, etc. - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number Agreement Regarding additional candidate S-SSB occasions: - In the same S-SSB period, RAN1 further study the followings: - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions - FFS details Agreement Down-select one or support both of the followings: - Option 1: Additional candidate S-SSB occasions are excluded from resource pool - Option 2: Additional candidate S-SSB occasions belong to resource pool - Note: Companies are encouraged to consider aspects including: S-SSB resource overhead, Tx/Rx UE behavior (e.g., whether any blind detection in Option 2), applicable scenarios, etc.

      - Number/Location

For the number/location of additional S-SSB occasions, our view is that simple solution is enough.

Proposal 20:

- For the number and location of additional S-SSB occasion, support Option 1.

  - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

      - vs RP

It was agreed that conventional S-SSB slots are excluded from SL RP as in R16/17, but additional candidate S-SSB occasions have not been concluded yet. In our view, the additional S-SSB slots should also be excluded from SL RP as the conventional S-SSB slots. If additional S-SSB slots are included in RP and thereby intra/inter-UE multiplexing/prioritization with PSSCH is performed, resource allocation mechanism becomes quite complicated. Besides, a lot of UEs may perform S-SSB transmission at the additional S-SSB slots; that is, they cannot communicate each other. Benefit to use the slots for e.g. data transmission is highly questionable.

Proposal 21:

- For RP vs additional S-SSB slots, support Option 1.

  - Option 1: Additional candidate S-SSB occasions are excluded from resource pool

      - How to use

Regarding how to use additional S-SSB occasions, RAN1 agreed four alternatives as captured above. For down-selection, it seems that there are two important aspects – 1) if each UE uses all occasions, the TXs would bring trouble to other system; 2) no TX would cause COT loss due to other systems. These are in the relationship of trade-off. Thus, changing behavior based on situations is better.

Proposal 22:

- For usage of additional S-SSB occasions, support Alt 3.

  - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

- Conclusion

In this contribution, we discussed channel design framework in SL-U. Observations/Proposals are summarized as following:

Proposal 1:

- For each RB set, sub-channel 0 is defined with interlace 0, sub-channel 1 is defined with interlace 1, and so on.

Proposal 2:

- For PSCCH/PSSCH resource indication, the number N of RB sets, starting RB set, and a set of sub-channels per M RB sets are indicated in a SCI.

  - M is (pre-)configured.

Proposal 3:

- When a PSCCH/PSSCH is mapped across multiple adjacent RB sets and

  - If the same interlace index is used among the RB sets, PRBs in the same interlace within the intra-cell guard band are used.

  - If different interlace index is used among the RB sets, PRBs corresponding to either interlace (e.g., interlace at RB set with smaller index) within the intra-cell guard band are used.

- In TBS determination, the number of PRBs for a PSCCH/PSSCH TX does not include the number of PRBs allocated from intra-cell guard band.

Proposal 4:

- For interlaced RB-based structure, PSCCH/PSSCH is mapped on IRBs as on PRBs in Rel-16/17, and then IRB-to-PRB mapping is performed.

Proposal 5:

- 2nd starting symbol is available only when the following condition is met.

  - The UE attempted the transmission from the 1st starting symbol, but the corresponding LBT was failed.

Proposal 6:

- When a transmission is performed across multiple RB sets,

  - If the 1st starting position is available on all the RB sets, the TX is performed from the 1st starting symbol.

  - If the 1st starting position is not available on at least one of the RB sets, the 2nd starting symbol is attempted for the TX.

    - If the 2nd starting position is available on all the RB sets, the TX is performed from the 2nd starting symbol; otherwise, the TX is dropped.

Proposal 7:

- For PSCCH/PSSCH transmissions starting from the 2nd starting symbol (Xth symbol),

  - PSCCH/PSSCH is generated according to the number of the actual PSCCH/PSSCH transmission, i.e., duration between Xth symbol and 13th symbol.

Proposal 8:

- At least the following parameters are (pre-)configured for each of the 1st starting symbol and the 2nd starting symbol.

  - SL-PSCCH-Config (e.g., sl-TimeResourcePSCCH, sl-FreqResourcePSCCH)

  - SL-PSSCH-Config (e.g., sl-Scaling)

  - SL-MinMaxMCS-List

  - sl-PowerControl (e.g., sl-P0-PSSCH-PSCCH)

  - SL-CSI-RS-Config (e.g., sl-CSI-RS-FirstSymbol)

Observation 1:

- Due to hidden-node issue, it could occur that some UEs start from 1st starting symbol and other UEs start from 2nd starting symbol within the same slot.

Proposal9:

- When two starting symbols are available,

  - It is assumed from TX UE that RX UE always performs 2nd AGC at the 2nd starting symbol

    - Note: RX UE can use the 2nd starting symbol for decoding by up to UE implementation

  - If a TX UE will transmit a PSCCH/PSSCH from the 1st starting symbol, no dedicated 2nd AGC symbol is defined (i.e., no symbol copy around the 2nd starting symbol)

    - Note: TX UE can consider the 2nd AGC in TBS/MCS/coding-rate determination

  - Note: no spec impact is assumed

Proposal 10:

- Update the following processing time constraints.

  - PSCCH/PSSCH to PSFCH slot offset is sl-MinTimeGapPSFCH + 1.

  - The last monitored PSCCH occasion for a resource selection is the first PSCCH occasion at the last slot within the sensing window.

Proposal 11:

- When 2 candidate starting symbols are available, ‘PSFCH overhead indication’ field in SCI-1 is always 0 bit.

Proposal 12:

- Not support a different reference number for a shared COT.

Proposal 13:

- Deprioritize contiguous RB-based structure in R18 SL-U discussion.

Observation 2:

- For Alt 2-3a of PSFCH structure, it seems that there are at least the following two issues:

  - Capacity issue: quite difficult to support GC option2, impossible to configure smaller number of CSs for a scenario of high frequency selectivity

  - Spec impact: impossible to apply legacy mapping of PSSCH-PSFCH (i.e., separate PSFCH PRB(s) per PSSCH resource)

Proposal 14:

- For PSFCH structure,

  - Drop Alt 2-3a/3-2a.

  - Separately discuss Alt 2-4a with respect to PAPR reduction.

Proposal 15:

- For interlaced PSFCH format 0, support a combination of Alt 1-1a and Alt 2-2a.

  - Each PSFCH transmission occupies the following

    - K3 dedicated PRB(s) with a dedicated cyclic shift among PRBs not used for common interlace.

    - PRBs within 1 common interlace, except for PRBs within the common interlace that are the closest to each of K3 dedicated PRB(s); i.e., PRBs within the common interlace that are the closest to each of K3 dedicated PRB(s) are dropped/not used.

Proposal 16:

- The same RB-set is assigned for PSCCH/PSSCH resource and the corresponding PSFCH resource.

Proposal 17:

- For determination of PSFCH occasion, support Option 1 with the following note.

  - Note: PSFCH resources associated with a PSSCH resource are not overlapped with PSFCH resources associated with other PSSCH resources.

Proposal 18:

- For S-SSB structure, support Option 1-1.

  - When 30 kHz SCS and multiple RB sets are configured, interlaced S-SSB is mapped within an RB-set and a PRB within ICGB adjacent to the RB-set.

Proposal 19:

- Exemption of OCB requirement is not applied to S-SSB TX.

Proposal 20:

- For the number and location of additional S-SSB occasion, support Option 1.

  - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

Proposal 21:

- For RP vs additional S-SSB slots, support Option 1.

  - Option 1: Additional candidate S-SSB occasions are excluded from resource pool

Proposal 22:

- For usage of additional S-SSB occasions, support Alt 3.

  - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

References

- 3GPP RAN1#112 meeting,	chair’s notes