3GPP TSG RAN WG1 #112bis-e			     R1-2303716

e-meeting, April 17th – 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on enhanced sidelink operation on FR2 licensed spectrum

Agenda Item:	9.4.3

Document for: 	Discussion and Decision

- Introduction

At the RAN1#112 meeting [1], there was discussion on enhanced sidelink operation on FR2 licensed spectrum. In this contribution, we share our views on enhanced sidelink operation on FR2 licensed spectrum.

Agreement For sidelink beam management, RAN1 is to study - how transmit beam(s) training and/or receive beam(s) training is performed - whether and how spatial related information (e.g., TCI, QCL, beam ID, etc) information could be identified - the relationship between PC5 unicast link establishment and sidelink initial beam pairing (e.g., whether initial beam pairing procedure starts before, during or after sidelink unicast link establishment procedure.) Agreement RAN1 is to study sidelink beam measurement and reporting schemes (e.g., periodicity, contents, container, timing, procedure, etc.) for sidelink beam maintenance. Agreement RAN1 is to study sidelink beam indication and switching schemes (e.g., framework, general procedure, contents, signaling, timing, etc.) for sidelink beam maintenance. Agreement RAN1 is to study the information related to a sidelink beam failure instance that the PHY layer provides to the MAC layer.

- Discussion

  - Initial beam pairing

At the previous meeting, it was discussed which reference signal or channel should be studied for initial beam pairing. The candidates are S-SSB, SL CSI-RS, DMRS of PSSCH/PSCCH or PSFCH. In our views, which signal/channel is appropriate for initial beam pairing depends on when it is performed.

If it is performed after the unicast link establishment procedure, PC5-RRC configurations can be used to configure the reference signal and/or measurements for initial beam pairing. In this sense, the use of SL CSI-RS is a natural choice.

If it is performed during unicast link establishment, we assume that the UE initiating initial beam pairing (TX UE) would transmit beamformed PSSCHs conveying PC5-S and/or PC5-RRC signalling repeatedly with beam sweeping and the UE responding the initiation of initial beam pairing (RX UE) would measure the strength of reference signal (beam) and try to decode these PSSCHs, and then RX UE would transmit information that include which TX beam is best. That is, in that case, DMRS of PSSCH/PSCCH is used for the initial beam pairing.

If it is performed before unicast link establishment, reference signal that can be transmitted regardless of PC5-RRC connection, should be considered as a candidate, such as S-SSB, SL CSI-RS enhanced to be transmitted before PC5-RRC connection or PSFCH as responding channel.

- Which reference signal/channel is appropriate for initial beam pairing depends on when initial beam pairing is performed.

In our opinion, initial beam pairing should be performed during or before unicast link establishment. For the case where it is performed after unicast link, we understand that it may only require a simple mechanism and minimal specification impact. However, the benefit of such a beam pairing mechanism seems to be small because, in this case, the SL UEs were already able to exchange information and establish a unicast link without the specification related to SL beam management. To ensure the coverage of SL FR2 unicast link, beam-related beahvior for PC5-RRC connection establishment should not be left to UE implementation but explicitly specified in RAN1.

- Initial beam pairing should be performed either during or before unicast link establishment procedure

Some companies mentioned that the option of using DMRS for initial beam pairing should be precluded. We cannot support that at this stage. In Uu, the initial access procedure is initiated by SSB/PRACH, so the initial beam pairing also uses the relevant RS/channel. In sidelink unicast, the link establishment procedure is initiated by PSCCH/PSSCH, so it is natural to consider this option.

In addition, it is our understanding that PSFCH is assumed to be used only to respond the signal/channel initiating initial beam pairing, not to initiate the initial beam pairing. This should be clarified.

- Study whether and how DMRS of PSSCH/PSCCH and PSFCH could be enhanced for SL initial beam pairing performed during unicast link establishment procedure

  - PSFCH is assumed to be used only for responding the initiation of initial beam pairing

- Study whether and how S-SSB, CSI-RS and PSFCH could be enhanced for SL initial beam pairing performed before unicast link establishment procedure

  - PSFCH is assumed to be used only for responding the initiation of initial beam pairing

  - Beam maintenance

To clarify the scope of studying beam maintenance procedure, it should be confirmed that it is performed only after PC5-RRC connection is established. Beam maintenance is assumed to use PC5-RRC parameters related to the spatial related information, the resource configuration of reference signal or measurement/report configuration.

- SL beam maintenance procedure should be performed after PC5-RRC connection establishment procedure (and initial beam pairing)

For which reference signal is used, following the description of WID’s objectives, CSI-RS can be considered as the first target of this study. The existing CSI framework and signaling are reused to minimize the specification impact.

- Prioritize studying SL CSI-RS for SL beam maintenance

For the details of SL CSI-RS for beam maintenance, the discussion points are whether to support CSI-RS only TX, multiple beam CSI-RS TX within one slot (intra-slot beam sweeping), and periodic CSI-RS TX or repeating CSI-RS TX with the same beam.

In our view, whether to support intra-slot beam sweeping and/or CSI-RS only transmission is more important topic, which may change the SL slot structure and cause a lot of PHY layer spec. impact. Whether to introduce the periodic or repetitive CSI-RS transmissions would be the issue mainly related to the higher layer signaling structure.

Note that performing TX beam sweeping would change the receive power from the perspective of surrounding RX UEs (not a beam measuring UE) and if without AGC symbol, which leads to AGC issue in surrounding SL UEs trying to perform sensing or data reception.

For CSI-RS only transmission, there are several possibilities, such as CSI-RS without PSCCH and PSSCH, CSI-RS only without PSSCH, CSI-RS without data but with SCI. RAN1 should discuss this point.

- Study firstly how/whether to support SL CSI-RS only transmission and/or intra-slot beam sweeping, which would change SL slot structure

  - For SL CSI-RS only transmission, study which structure to be used such as SL CSI-RS without PSCCH/PSSCH, only without PSSCH or without data but with SCI.

  - For intra-slot beam sweeping, study how to resolve the AGC issue.

Regarding the above details of CSI-RS, we support CSI-RS for beam maintenance if CSI-RS transmission without data and intra-slot beam sweeping is feasible. This is because the advantages of CSI-RS over DMRS or S-SSB appear to be the flexibility of transmission timing and possible resource efficiency for transmitting multiple beams. If CSI-RS TX for beam maintenance is only allowed to be multiplexed with data (MAC PDU) and applied to the same beam within a slot, it is not much different from the situation where each data TX is repeatedly transmitted with beam sweeping and RX UE performs the feedback to TX UE by measuring the strength DMRS of each beam i.e., DMRS for beam maintenance.

- The advantages of SL CSI-RS for SL beam maintenance are considered to be minimum resources and flexible resources to maintain the paired beam.

- Support SL CSI-RS for SL beam maintenance if SL CSI-RS transmission without data and intra-slot beam sweeping are feasible

  - Otherwise, support DMRS or S-SSB for SL beam maintenance

  - Beam failure recovery

We support studying beam failure recovery (BFR) mechanism. It is useful when CSI report is not triggered or by using beam maintenance cannot follow the rapid change of the relative location of SL UEs. It would be typical environment for SL FR2 use cases. Here we note that the motivation of BFR is that a beam pairing should be recovered faster than the SL RLF mechanism.

- Study the physical layer design for SL beam failure recovery (BFR), including SL beam failure recovery request (BFRQ), SL beam failure recovery response (BFRR) and the general procedure.

  - BFR should be designed to ensure faster recovery than SL RLF

Regarding the criteria of beam failure instance, HARQ DTX and L1-RSRP were discussed at the previous meeting. We believe that the appropriate criteria depend on whether periodic/frequent reference signals (beams) are transmitted. If periodic/frequent reference signals are present, RX SL UE can monitor the strength of each beam (e.g., L1-RSRP) intermittently and, if BFR is triggered, search for a new candidate beam based on periodic/frequent reference signals.

If periodic/frequent RSs are not supported or configured, the BFI criteria can only be transmission failure (e.g., HARQ DTX). In this case, RX SL UE may not be able to search for a new candidate beam by trying to receive RSs pointing the direction of RX SL UE when BFR occurs. Then, RX SL UE would have to start from TX beam sweeping, which is expected to take longer to recover beam pairing; That point should be studied to achieve fast recovery for HARQ DTX as the BFI criteria.

- In the case where periodic RSs are used to maintain the beam pairing, RX SL UE can detect the degradation of main beam based on L1-RSRP and search for a new candidate beam based on periodic RS.

- In other cases, transmission failure (e.g., HARQ DTX) can be considered as BFI criteria.

- Support L1-RSRP as the BFI criteria if periodic reference signal is supported and configured for beam maintenance.

- For HARQ DTX as the BFI criteria, study how to determine a new candidate beam to quickly recover the beam pairing.

  - Directional TX/RX/sensing issues in RA mode 2

At the previous meeting, it was discussed whether to include studying the case where multiple unicast links are established in one SL UE. We get to understand this study is a lower priority than the studies on basic features such as initial beam pairing, beam maintenance, beam failure recovery for single unicast link. But still, we believe that it is an essential and important topic to achieve useful SL FR2 system. As agreed in the evaluation methodology for SL FR2, cluster-based deployment of SL FR2 UEs is a typical case.

- Study the case where one SL UE has multiple unicast links on FR2.

  - It is a lower priority than the studies on basic features such as initial beam pairing, beam maintenance, beam failure recovery for single unicast link.

The typical beam management operation on FR2 is specified as the spatial domain filters, which is assumed to realize analog beamforming. This implies only one specific spatial domain filter that has a specific direction can be applied at each moment. On the other hand, the current SL RA mode 2 mechanism relies on reservation signaling for the future resources and sensing it to avoid the collision on the same resource. Each SL UE performs reservation and sensing for each SL UE’s surroundings to ensure that its future transmission would not interfere with other SL UEs. In our view, it’s an important discussion point whether, transmission and reception with only a specific direction are adequate for the essential concept of reservation and sensing.

Assuming the existing concept of reservation and sensing, the current specification of resource allocation and sensing should be enhanced.

- For sensing, without any specification on RX spatial domain filter for sensing, SL UEs probably miss a lot of reservation signals and do not perform sensing properly.

- For resource allocation, without any specification on resource selection considering TX/RX spatial domain filter,

  - From TX UE side, when there are multiple TXs at the same time and if they are associated with different TX spatial domain filter, only either TX can be performed with the optimal TX spatial domain filter. For example, the remaining TX(s) is dropped.

  - From RX UE side, when there are multiple RXs at the same time and if they are associated with different RX spatial domain filter, only either RX can be performed with the optimal RX spatial domain filter. For example, the remaining RX(s) is attempted with non-optimal RX spatial domain filter.

- Study how to determine RX beam filter to perform sensing.

Figure 2: The image of sensing failure without RX beam spec.

- In TX UE side, study how to avoid the case to perform multiple TXs to use different TX spatial filters at the same time (one slot)

- In RX UE side, study how to avoid the case to perform multiple RXs to use different RX spatial filters at the same time (one slot)

- Study how to determine the PSFCH TX/RX beam from multiple beam candidates and/or how to avoid/handle the case where multiple PSFCH TX/RXs are performed in a slot, when multiple unicast links are established.

Figure 3: The image of being impossible for simultaneous TXs in one slot with different TX beams from TX UE side

And even after TX/RX beams for the specific unicast link are determined, from the RX SL UE side, it is unclear which timing the RX UE should use the determined RX beam at. It is not rare case that a UE has connections with multiple UEs and accordingly the UE needs to switch RX beam often. In this case, even when the TX UE attempts a TX on a non-reserved resource, e.g., at slot n, the RX UE requires to know that the corresponding RX beam should be used at the slot n so that the TX is received successfully. Some enhancement mechanism for this purpose should be introduced.

- RAN1 is to study how to determine one RX beam from multiple RX beam candidates for multiple unicast links in slot n, for a data reception from a specific TX UE at slot n, at least if the TX is performed on a non-reserved resource.

Figure 4: The image of how RX UE determines to use the paired beam at slot n for a data reception

- Conclusion

In this contribution, we discussed enhanced sidelink operation on FR2 licensed spectrum. Observations/Proposals are summarized as following:

- Which reference signal/channel is appropriate for initial beam pairing depends on when initial beam pairing is performed.

- Initial beam pairing should be performed either during or before unicast link establishment procedure

- Study whether and how DMRS of PSSCH/PSCCH and PSFCH could be enhanced for SL initial beam pairing performed during unicast link establishment procedure

  - PSFCH is assumed to be used only for responding the initiation of initial beam pairing

- Study whether and how S-SSB, CSI-RS and PSFCH could be enhanced for SL initial beam pairing performed before unicast link establishment procedure

  - PSFCH is assumed to be used only for responding the initiation of initial beam pairing

- SL beam maintenance procedure should be performed after PC5-RRC connection establishment procedure (and initial beam pairing)

- Prioritize studying SL CSI-RS for SL beam maintenance

- Study firstly how/whether to support SL CSI-RS only transmission and/or intra-slot beam sweeping, which would change SL slot structure

  - For SL CSI-RS only transmission, study which structure to be used such as SL CSI-RS without PSCCH/PSSCH, only without PSSCH or without data but with SCI.

  - For intra-slot beam sweeping, study how to resolve the AGC issue.

- The advantages of SL CSI-RS for SL beam maintenance are considered to be minimum resources and flexible resources to maintain the paired beam.

- Support SL CSI-RS for SL beam maintenance if SL CSI-RS transmission without data and intra-slot beam sweeping are feasible

  - Otherwise, support DMRS or S-SSB for SL beam maintenance

- Study the physical layer design for SL beam failure recovery (BFR), including SL beam failure recovery request (BFRQ), SL beam failure recovery response (BFRR) and the general procedure.

  - BFR should be designed to ensure faster recovery than SL RLF

- In the case where periodic RSs are used to maintain the beam pairing, RX SL UE can detect the degradation of main beam based on L1-RSRP and search for a new candidate beam based on periodic RS.

- In other cases, transmission failure (i.e., HARQ DTX) can be considered as BFI criteria.

- Support L1-RSRP as the BFI criteria if periodic reference signal is supported and configured for beam maintenance.

- For HARQ DTX as the BFI criteria, study how to determine a new candidate beam to quickly recover the beam pairing.

- Study the case where one SL UE has multiple unicast links on FR2.

  - It is a lower priority than the studies on basic features such as initial beam pairing, beam maintenance, beam failure recovery for single unicast link.

- Study how to determine RX beam filter to perform sensing.

- In TX UE side, study how to avoid the case to perform multiple TXs to use different TX spatial filters at the same time (one slot)

- In RX UE side, study how to avoid the case to perform multiple RXs to use different RX spatial filters at the same time (one slot)

- Study how to determine the PSFCH TX/RX beam from multiple beam candidates and/or how to avoid/handle the case where multiple PSFCH TX/RXs are performed in a slot, when multiple unicast links are established.

- RAN1 is to study how to determine one RX beam from multiple RX beam candidates for multiple unicast links in slot n, for a data reception from a specific TX UE at slot n, at least if the TX is performed on a non-reserved resource.

References

- 3GPP RAN1#112, chair’s notes