3GPP TSG RAN WG1#112bis-e	 	 	R1-2303719

e-Meeting, April 17th – April 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on bandwidth aggregation for positioning measurements

Agenda Item:	9.5.4

Document for: 	Discussion and Decision

- Introduction

At the RAN#98-e meeting, new WID on Rel-18 NR positioning was agreed [1]. The work item includes objectives related to improved positioning accuracy as follows:

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4].

  - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2].

    - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference).

  - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4].

  - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements.

  - Specify measurements that are limited to a single carrier/PFL.

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

In this contribution, we present our views on bandwidth aggregation for positioning measurements for Rel-18 NR positioning.

- Timing error between carriers

In the current NR positioning, DL-PRS/SRS resources for positioning can be configured up to 272 PRBs. 272 PRBs corresponds to about 50 MHz bandwidth at 15 kHz subcarrier spacing. In Rel-17 SI phase, RAN1 studied the positioning accuracy performance gain of bandwidth aggregation of DL-PRS/SRS for positioning and concluded that bandwidth aggregation under certain scenarios, configurations, and assumptions on modelled impairments can improve positioning accuracy for timing-based methods (e.g., DL-TDOA, UL-TDOA and Multi-RTT) [2]. In Rel-18 SI phase, RAN4 studied the feasibility of bandwidth aggregation for positioning and concluded that bandwidth aggregation for intra-band contiguous carriers using single antenna is feasible from RRM perspective [3].

In NR specifications, Time Alignment Error (TAE) requirement which applies to Tx frame timing in carrier aggregation is specified. TAE requirement is specified as ns order timing error between carriers transmitted from different antenna connectors for MIMO transmission. For example, TAE is allowed up to 260 ns in intra-band contiguous carriers.

As we mentioned above, in case of Rel-18 bandwidth aggregation for positioning, PRS/SRS for positioning are transmitted at single antenna only. Therefore, large timing error between carriers such as TAE (e.g., 260 ns) may not occur. However, given that even 1 ns timing error corresponds to 30 cm positioning error for timing-based methods, whether Tx timing error between carriers occurs or not even for single antenna transmission of DL-PRS/SRS for positioning should be discussed at first. In addition, if Tx timing error between carriers occurs and impacts on DL-PRS/SRS for positioning measurement results based on bandwidth aggregation, some compensation scheme for the Tx timing error between carriers may be needed. For example, some features regarding Timing Error Group (TEG) specified in Rel-17 may be reused to compensate Tx timing error between carriers.

Proposal 1:

- For bandwidth aggregation for positioning, whether Tx timing error between carriers occur or not even for single antenna transmission of DL-PRS/SRS for positioning should be discussed.

Observation 1:

- For bandwidth aggregation for positioning, some compensation scheme for Tx timing error between carriers may be needed.

- Relationship between UL CA and SRS bandwidth aggregation

Regarding relationship between UL CA and SRS bandwidth aggregation, the following agreement was made at the last meeting [4].

Agreement

Study the relationship between UL communication CA and SRS bandwidth aggregation, including

- Whether to support the decoupling of the SRS bandwidth aggregation and the communication carrier aggregation for UE capabilities.

- Whether to support the configuration of SRS BW aggregation not limited by the allowed configuration of communication CA, i.e. SRS outside BWP and across carriers.

In our understanding, even if a Rel-18 UE has legacy UL communication CA capabilities, the UE does not always support SRS bandwidth aggregation features. To support SRS bandwidth aggregation features, legacy UL communication CA capabilities are not mandatory since the functions required for carrier aggregation for measurement purpose is different from that for carrier aggregation for communication purpose. Thus, from perspective of the backward compatibility, legacy UL communication CA capabilities and SRS band width aggregation capabilities should be decoupled.

Proposal 2:

- Legacy UL communication CA capabilities and SRS band width aggregation capabilities should be decoupled.

- Carrier indication for bandwidth aggregation

Regarding carrier indication for bandwidth aggregation, the following agreements were made at the last meeting [4].

Agreement

Support joint measurement and report for the PRS resources aggregated across the PFLs for DL-TDOA and multi-RTT positioning methods

- In a measurement report element, single RSTD or single UE Rx-Tx time difference is reported for the PRS resources across aggregated PFLs

  - FFS: RSRP, RSRPP

- FFS: In a measurement report, PFL aggregation indication is supported to indicate whether/which PFLs are aggregated for the PRS measurement

- FFS whether to use PRS assistance data or use location information request message to indicate UE to perform joint measurement across aggregated PFLs

- FFS RSTD reference configuration or report should be enhanced

Agreement

Support joint measurement and report for the SRS resources across the aggregated carriers for UL-TDOA and Multi-RTT positioning methods

- Single UL RTOA or gNB Rx-Tx time difference is reported for the SRS resources across aggregated carriers

  - FFS: RSRP or RSRPP

- FFS: SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which carriers are aggregated for the joint SRS measurement

- Support LMF to request gNB for the UL positioning measurement from aggregated SRS resources across multiple CCs

In Rel-17 NR, measurement of DL-PRS/SRS resource and the measurement reporting are operated on each PFL. To support DL-PRS/SRS bandwidth aggregation, it necessary to introduce mechanism to report and/or indicate which carriers are to be aggregated. One straightforward solution is reporting aggregated DL-PRS/SRS resource (set) IDs on each occasion of measurement result reporting. However, it may have concern that signaling overhead is too large. From perspective of signaling overhead, one possible solution is to introduce group ID providing aggregated DL-PRS/SRS resource (set) IDs.

Proposal 3:

- Consider introducing group ID providing aggregated DL-PRS/SRS resource (set) IDs.

- Conclusion

In this contribution, we discussed on bandwidth aggregation for positioning measurements for Rel-18 NR positioning. Based on the discussion, we made following observation and proposals.

Observation 1:

- For bandwidth aggregation for positioning, some compensation scheme for Tx timing error between carriers may be needed.

Proposal 1:

- For bandwidth aggregation for positioning, whether Tx timing error between carriers occur or not even for single antenna transmission of DL-PRS/SRS for positioning should be discussed.

Proposal 2:

- Legacy UL communication CA capabilities and SRS band width aggregation capabilities should be decoupled.

Proposal 3:

- Consider introducing group ID providing aggregated DL-PRS/SRS resource (set) IDs.

References

- RP-223549, 3GPP RAN#98-e, “New WID on Expanded and Improved NR Positioning”, Electronic Meeting, Dec. 2022.

- 3GPP TR 38.857 V17.0.0 (2021-03), Study on NR Positioning Enhancements

- 3GPP TR 38.859 V18.0.0 (2022-12), Study on expanded and improved NR positioning

- 3GPP RAN1#112, Chairman’s Notes, Feb. 2023.