3GPP TSG RAN WG1 #112bis-e			R1-2303721

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.6.1

Source: 	NTT DOCOMO, INC.

Title: 	Discussion on further UE complexity reduction for eRedCap

Document for:	Discussion and Decision

- Introduction

A new WID on further NR RedCap UE complexity reduction (FS_NR_redcap_enh) was approved at the RAN#97-e meeting and revised at the RAN#98-e meeting [1]. The objectives of the WI are shown below.

The objective is to specify support for the following enhancements: Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2 and CT1 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99-e regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

In this contribution, we discuss on further UE complexity reduction for Rel-18 eRedCap.

- Discussion

In the following subsections, we provide the discussion for UE BB bandwidth reduction and UE peak rate reduction for further UE complexity reduction.

  - UE BB bandwidth reduction

    - Separate initial DL/UL BWP

At the RAN1#110bis-e meeting, the following agreement was made for separate initial DL/UL BWP;

Agreement: For a cell supporting both Rel-17 and Rel-18 RedCap UEs, - The Rel-18 RedCap UEs can share the same separate initial DL/UL BWP as the Rel-17 RedCap UEs. - FFS: whether to support an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs

Based on the agreement, the following two cases are supported so far;

- Case 1a: For a cell supporting both Rel-17 and Rel-18 RedCap UEs, non-RedCap, Rel-17 and Rel-18 RedCap UEs share the initial BWP.

- Case 1b: For a cell supporting both Rel-17 and Rel-18 RedCap UEs, Rel-17 and Rel-18 eRedCap UEs share the separate initial BWP.

Fig.1a  Initial BWP configuration for Case 1a          Fig.1b  Initial BWP configuration for Case 1b

In addition, the following conclusion was made at RAN1#112 meeting, and hence the case where a separate initial BWP is configured for Rel-17 and another separate initial BWP is configured for Rel-18 eRedCap UE would not be discussed further in RAN1.

Conclusion: There is no consensus to continue discussion on “whether additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is allowed to be configured by the SIB in the cell”.

Given the situation, it is unclear so far whether to support the below cases 1c and 1d from RAN1 perspective.

- Case 1c: For a cell supporting only Rel-18 eRedCap UE and not supporting Rel-17 RedCap UE, separate initial BWP specific to Rel-18 eRedCap UE is configured.

- Case 1d: For a cell supporting both Rel-17 and Rel-18 RedCap UEs, non-RedCap and Rel-17 RedCap UEs share the initial BWP and separate initial BWP specific to Rel-18 eRedCap UE is configured.

Fig.1c  Initial BWP configuration for Case 1c              Fig.1d  Initial BWP configuration for Case 1d

Regarding the Case 1c, this is exactly the same operation as Rel-17 RedCap UEs, i.e., initial BWP which exceeds 20 MHz is configured for non-RedCap UEs and a separate initial BWP which does not exceed 20MHz is configured for Rel-18 eRedCap UEs. Therefore, for the same reason as Rel-17 RedCap UE that the initial BWP should be configured within 20MHz, this case should be supported. For this case, Rel-17 framework can be simply reused, but the signaling details, i.e., whether to reuse/reinterpret the signaling for Rel-17 RedCap etc., would be discussed in RAN2.

Regarding the Case 1d, we see the clear benefits to support this case considering the target use cases for Rel-18 eRedCap. In Rel-18, this WI is targeting low-end devices and cost/complexity need to be further reduced compared to Rel-17 RedCap. Accordingly, it is expected that the number of Rel-18 eRedCap devices in the NW would be largely increased compared to non-RedCap/Rel-17 RedCap UEs. As captured in WID objective, the NW would accommodate non-RedCap UE, Rel-17 eRedCap UE and Rel-18 eRedCap UE at most, and hence such case should be taken into account. In our view, for such case, the congestion on the resources for random access can be the bottleneck for the NW and is highly concerned. To address this concern, enhancements from Rel-17 RedCap in terms of capacity for random access should be considered. For the Case 1d, random access resources can be offloaded to Rel-18 eRedCap specific initial BWP and the congestion on random access can be resolved.

Furthermore, if Rel-18 eRedCap specific initial BWP can be configured, NW can configure much smaller BWP size for the Rel-18 eRedCap UE than that for legacy UEs. For example, in FDD band which has 20 MHz CBW, initial BWP for Rel-17 RedCap and non-RedCap UE is configured as 20 MHz and one separate initial BWP for Rel-18 eRedCap UE is configured as 5 MHz. As for this operation, during random access and after random access, the Rel-18 eRedCap UE can operate within 5MHz BWP while the wider BWP operation is still allowed for legacy UEs even for random access. BWP size is important factor for UE power saving, and hence it is meaningful to allow such BWP limiting operation for Rel-18 eRedCap targeting power consumption sensitive devises compared to legacy UEs.

It should be noted that it is up to NW whether to use such initial BWP configuration specific to Rel-18 eRedCap, i.e., NW can configure a separate initial BWP specific to Rel-18 eRedCap if the random access capacity/power consumption is concerned, otherwise, initial BWP can be shared with legacy UEs.

In addition, it should be also noted that this operation does not intend to change the separate initial DL/UL BWP framework in Rel-17 and there seems no/little RAN1 impact, i.e., the possible specification impact is only introducing new RRC parameter for the configuration related to the separate initial DL/UL BWP for Rel-18 eRedCap. From both UE perspective, only one SIB-configured separate initial BWP is configured and this is exactly the same operation as Rel-17 RedCap UEs. More specifically, while the details of signalling should be discussed in RAN2, for example (for DL BWP), if initialDownlinkBWP-RedCap-r18 is configured for Rel-18 eRedCap UE in SIB1, the UE uses the initial BWP associated with the configurations of initialDownlinkBWP-RedCap-r18.

To summarize, we believe Case 1d BWP operations is beneficial for offloading of random access and also potentially beneficial from power saving perspective. Therefore, we propose to support these operations in Rel-18.

Proposal 1:

- For a cell supporting Rel-18 RedCap UE but not supporting Rel-17 RedCap UE, a separate initial DL/UL BWP specific to Rel-18 eRedCap should be supported.

- For a cell supporting both Rel-17 RedCap and Rel-18 RedCap UEs,

  - a separate initial DL/UL BWP specific to Rel-18 eRedCap should be supported when non-RedCap and Rel-17 RedCap UEs share initial DL/UL BWP.

- The details for the signalling of initial BWP configuration should be up to RAN2.

    - Timeline between Msg2 and Msg3

At the RAN1#112, the following agreement was made;

Agreement: For the earlier RAN1 agreement achieved in RAN1#111 as following, For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH. For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS - Note: Single Value pair for X is to selected for SCSs

In the current specification, the minimum time separation between the last symbol of RAR PDSCH and the first symbol of Msg3 PUSCH is specified as NT,1 + NT,2 + 0.5 ms. This timeline is specified regardless of the RAR PDSCH BW, however, the capable BB BW per slot is restricted for Rel-18 eRedCap and the timeline may need to be extended depending on the PDSCH BB BW.

According to the above agreement, when the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, the minimum time separation between RAR and Msg3 is extended as NT,1 + NT,2 + 0.5 + X ms for Rel-18 eRedCap UE and the exact value of X would be down-selected from [0.5/0.25 or 1/0.5 or 2/1] for 15/30 kHz SCS.

According to the current specification, NT,1 + NT,2 + 0.5 equals to 31/39 symbols for 15/30 kHz SCS respectively when dmrs-AdditionalPosition ≠ 'pos0'. As for Msg3 PUSCH scheduling by UL grant in RAR, the offset from RAR PDSCH reception slot n is n + K2 + ∆, where K2 is indicated based on the default TDRA table for PUSCH (table6.1.2.1.1-2/6.1.2.1.1-4 in TS 38.214) and ∆ is specified for each SCS (table 6.1.2.1.1-5 in TS 38.214).

Table 6.1.2.1.1-2: Default PUSCH time domain resource allocation A for normal CP [3]

Table 6.1.2.1.1-4: Definition of value j [3]

Table 6.1.2.1.1-5: Definition of value Δ [3]

When K2 is indicated as j, the offset form RAR PDSCH slot to Msg3 PUSCH slot corresponds to 3/4 slots (= 42/56 symbols) for 15/30 kHz SCS respectively. The time separation between RAR PDSCH and Msg3 PUSCH can be shortest when the PDSCH ends in the last symbol in the slot n and the PUSCH starts from very first symbol in the slot n + K2 + ∆. For this case, the difference between the processing time for RAR PDSCH reception and Msg3 PUSCH transmission (i.e., NT,1 + NT,2 + 0.5 ms) and minimum possible scheduling offset between RAR PDSCH reception and Msg3 PUSCH transmission (i.e., K2 + ∆) is 11/17 symbols (= 42/56 symbols - 31/39 symbols) = 0.79/0.61 ms for 15/30 kHz SCS respectively.

If the processing time for RAR PDSCH reception and Msg3 PUSCH transmission is expanded as NT,1 + NT,2 + 0.5 + X ms and X is larger than 0.79/0.61 ms for 15/30 kHz SCS, the valid K0/K2 and SLIV configuration for RAR PDSCH/Msg3 PUSCH would be limited. To avoid such restriction on RAR and Msg3 TDRA, smaller value for X is preferable, e.g., 0.5/0.25 ms for 15/30 kHz SCS, otherwise, separate early indication is essential for NW to apply different handling between Rel-18 eRedCap and legacy UEs.

It should be noted that it was agreed at RAN#99 meeting that peak rate reduction to 10 Mbps can be supported as standalone feature for Rel-18 eRedCap, and of course this standalone UE does not support the features as for UE BB bandwidth reduction including extension of this RAR-Msg3 timeline. However, it was also agreed that Rel-18 eRedCap UEs support peak rate reduction as add-on feature and standalone feature use the same early indication. In that sense, if default PUSCH TDRA table is extended, e.g., new default PUSCH TDRA table is specified, for Rel-18 eRedCap UE which supports peak rate reduction as add-on feature to improve the configuration flexibility on TDRA, the standalone UE cannot operate with the extended default PUSCH TDRA table, then the PUSCH TDRA misalignment may happen. Therefore, the default PUSCH TDRA table cannot be extended even for Rel-18 eRedCap UEs. This also implies that smaller value of X is preferable to maximize the valid configuration of TDRA for RAR PDSCH and Msg3 PUSCH.

Proposal 2:

When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, support X for the minimum time separation between RAR and Msg3 NT,1 + NT,2 + 0.5 + X ms for Rel-18 eRedCap UE as 0.5/0.25 ms for 15/30 kHz SCS.

    - Separate early indication

At the RAN#98-e meeting, it was agreed to include the separate early indication of Rel-18 eRedCap as the scope of this WI. Therefore, at least either Msg1 or Msg3 based separate early indication would be supported.

At RAN2#121 meeting, the following agreement was made;

Introduce Msg3/MsgA PUSCH based early indication for Rel-18 eRedCap. FFS how to implement this in the spec (e.g., new LCIDs or not). We will wait for RAN1 progress to see if there is a need for a Msg1 early indication for eRedCap.

Based on the agreement, RAN1 should clarify whether Msg1-based separate early indication is necessary for Rel-18 eRedCap.

At the RAN1#111 meeting, the following agreements were made;

Agreement: For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

Based on the agreement, a cell supporting Rel-18 eRedCap UE has to ensure that the BW for Msg3 is not to exceed 5 MHz for Rel-18 eRedCap UEs. And as for RAR, the BW needs to be within 5 MHz, otherwise, the TDRA of RAR and Msg3 would be restricted depending on the value of X for the RAR and Msg3 processing timeline discussed in section 2.1.2. For example, if separate early indication via Msg1 is not supported, RAR/Msg3 need to be configured within 5MHz or TDRA of RAR/Msg3 is restricted even for non-RedCap/Rel-17 RedCap UEs. It should be allowed for NW to avoid such restrictions on legacy UEs and potential impacts on the current deployment, and hence a separate early indication via Msg1 is necessary.

Several companies insisted that the payload size for RAR is relatively small and 5MHz BW is sufficient even for legacy UEs, however, there are some cases that the payload size can be large so that BW larger than 5MHz is required. For example, multiple RAR massages for different UEs can be multiplexed in a single MAC PDU which is transmitted in a RAR PDSCH, and then the payload size can be large so that 5MHz BW is not sufficient. Therefore, the payload size for RAR is not always small and 5MHz BW is not sufficient for some cases.

In addition, if initial BWP operation described in Fig.1d in section 2.1.1 is supported, NW has to know in which initial BWP of shared one for non-RedCap/Rel-17 RedCap UE or specific one for Rel-18 eRedCap to transmit RAR, then Msg1-based separate early indication is required.

Whether to support the Msg1-based separate early indication was extensively discussed at the previous meetings and there seems no drawback to support from RAN1 perspective while majority of companies see the necessity on it. Accordingly, RAN1 should conclude that Msg1-based separate early indication is necessary.

Regarding specification impacts to support the separate early indication via Msg1, the only expected impact is for RAN2 spec. While the details should be discussed in RAN2, the Rel-17 framework can be easily reused, i.e., RACH partitioning is configured by gNB and a UE transmits PRACH based on its capability, thus we don’t see any concern on the workload. In addition, though the complexity on PRACH partitioning is concerned, whether to use Msg1-based separate early indication is up to NW, and hence it would not be the reason not to support the separate early indication as specification.

Given that the following operation as for 2-step RACH for Rel-18 eRedCap was agreed at RAN1#112 meeting, 2-step RACH should also be taken into account.

Agreement: For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

Similar to Rel-17 RedCap, separate early indication based on MsgA PRACH should be supported if Msg1-based separate early indication for REl-18 eRedCap is supported.

Proposal 3:

A separate early indication for Rel-18 eRedCap via Msg1 and MsgA PRACH should be supported.

- Rel-17 early indication framework is reused for the separate early indication.

- Details for PRACH partitioning is up to RAN2.

    - PUCCH capacity for Msg4 HARQ-ACK

In the current specification, if intra-slot frequency hopping is disabled, only one base sequence is generated for the PUCCH transmission. On the other hand, if intra-slot frequency hopping is enabled, one base sequence is generated per hop for the PUCCH transmission as shown in Fig. 3. Therefore, if a PUCCH without intra-slot frequency hopping transmitted from RedCap/eRedCap UE and a PUCCH with intra-slot frequency hopping transmitted from non-RedCap UE are overlapped, they would interfere with each other irrespective of the applied CS since they have high cross-correlation.

Fig.3  Base sequence(s) for PUCCH with/without intra-slot frequency hopping (current spec)

As discussed in the section 2.1.1, especially for a cell supporting Rel-18 eRedCap, we believe it is essential to ensure the user multiplexing capacity for random access. However, as explained above, PUCCH resources cannot be multiplexed with CS when the PUCCH intra-slot frequency hopping is disabled. As a result, the capacity for common PUCCH cannot be ensured with CS. The user multiplexing capacity can be provided with FDM, i.e., additional PRB offset which was supported for Rel-17 RedCap. However, especially for the case when SCS is 30 kHz SCS and the separate initial BWP is configured as 5MHz for Rel-18 eRedCap, the max number of PRB is [11] and the additional PRB offset can be affect to the PUSCH fragmentation even if the PRB offset is 2 RBs for common PUCCH. Therefore, considering such case, it is preferable to support the multiplexing with CS.

Even if intra-slot frequency hopping is disabled, two base sequences can be generated as if intra-slot frequency hopping is enabled for the PUCCH transmission as shown in Fig. 4. In this case, even if a PUCCH without intra-slot frequency hopping transmitted from Rel-18 eRedCap UE and a PUCCH with intra-slot frequency hopping transmitted from legacy UE are overlapped, their interference can be sufficiently suppressed if different CS is applied to each UE. Therefore, we propose following:

Proposal 4: When intra-slot PUCCH frequency hopping within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Msg4/MsgB for RedCap UEs is disabled, the UE generates two base sequences for the PUCCH as if intra-slot frequency hopping is enabled for the PUCCH transmission.

Fig.4  Base sequences for PUCCH with/without intra-slot frequency hopping (proposal)

    - Scheduling restrictions on PDSCH

FDM on unicast PDSCH and broadcast PDSCH

At the RAN1#111/112 meeting, the following agreements were made;

Agreement: (RAN1#112) For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS Note: No intention to change the RAN4 RF specifications about maximum transmission PRB number Agreement: (RAN1#111) - For UE BB complexity reduction, a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The number of PRB scheduled in DCI is not larger than the maximum number of PRB agreed in previous agreement from 110b-e

According to the agreements, the number of PRBs that a UE can process is restricted as 5MHz per slot and the number of PRBs for unicast PDSCH cannot be larger than 5MHz for Rel-18 eRedCap UE not to extend the PDSCH processing time for HARQ-ACK preparation.

According to the following excerption from TS38.214 [3], one PDSCH scheduled with SI-RNTI and one unicast PDSCH can be FDMed in the overlapped time domain resource or SI acquisition time can overlap with the processing of unicast PDSCH decoding.

The UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if another PDSCH in the same cell scheduled with RA-RNTI or MSGB-RNTI partially or fully overlap in time. The UE in RRC_IDLE and RRC_INACTIVE modes shall be able to decode two PDSCHs each scheduled with SIRNTI, P-RNTI, RA-RNTI or TC-RNTI, with the two PDSCHs partially or fully overlapping in time in non-overlapping PRBs. … On a frequency range 1 cell, the UE shall be able to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CSRNTI and, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs, unless the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CSRNTI requires Capability 2 processing time according to clause 5.3 in which case the UE may skip decoding of the scheduled PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI.

Based on the specification, accordingly, even if the number of PRBs for unicast PDSCH is smaller than or equal to 5MHz, broadcast PDSCH can be FDMed and the total number of PRBs for the unicast PDSCH and the broadcast PDSCH in a slot can be larger than 5MHz. However, in our understanding, even if the total number of FDMed unicast PDSCH and the broadcast PDSCH in a slot is larger number of PRBs than 5MHz, Rel-18 eRedCap UE can process without any additional specification impact since there is no timeline requirement on reception of PDSCH scheduled with SI-RNTI and SI acquisition and it can be processed across multiple slots.

Based on the above, we propose to support the case where one PDSCH scheduled with SI-RNTI and one unicast PDSCH can be FDMed in the overlapped time domain resource or SI acquisition time can overlap with the processing of unicast PDSCH decoding without any specification change.

Proposal 5:

Support the case where one unicast PDSCH scheduled with SI-RNTI or SI acquisition triggered by P-RNTI and another PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI are overlapped in time domain and the total number of PRBs for these PDSCHs is larger than 5MHz while the number of PRB for the unicast PDSCH does not exceed 25/12 RBs for 15/30 kHz SCS.

Msg4 resource allocation

While the similar operation has been already agreed for unicast PDSCH, no consensus was achieved for Msg4 whether the larger number of RBs than 25/12 can be allocated, then the following agreement was made at the last RAN1 meeting;

Working assumption: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

In our understanding, the allocated number of RBs can be larger than 25/12 RBs for 15/30 kHz SCS for Rel-18 eRedCap before the contention resolution. For example, when collision happened between non-RedCap/Rel-17 RedCap and Rel-18 eRedCap UEs during random access, the Rel-18 eRedCap UEs tries to receive the Msg4 PDSCH while NW intends to schedule Msg4 for non-RedCap/Rel-17 RedCap UE. For this case, when the Rel-18 eRedCap UE decodes the PDCCH which schedules Msg4 for legacy UE, the UE can find that collision happened and the Msg4 is not transmitted for the Rel-18 eRedCap UE since FDRA in the DCI is larger than 25/12 RBs for 15/30 kHz SCS, then re-try the RACH procedure. In fact, this may happen even for legacy UEs, i.e., contention happened for different UEs and more than one UEs use the same TC-RNTI to receive Msg4 which is not scheduled for the UE.

It can be clarified if necessary that Rel-18 eRedCap UE initiates the RACH procedure again when the larger number of RBs than 25/12 for 15/30 kHz SCS are allocated for Msg4.

However, still Rel-18 eRedCap UE is not required to process larger number of RBs 25/12 for 15/30 kHz SCS, and hence this working assumption itself should be agreed.

Proposal 6:

Confirm the following working assumption;

Working assumption:

- For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

  - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

  - UE peak rate reduction

    - Constraint relaxation on peak rate calculation

In addition to UE BB bandwidth reduction to 5MHz in FR1, another complexity reduction feature, further UE peak data rate reduction, is captured in WID objective. For Rel-17 RedCap, the target peak data rate was set as 150 Mbps. In Rel-18, the target peak data rate is reduced to 10 Mbps for further UE complexity reduction.

During the SI phase for Rel-18 eRedCap, three options of peak rate reduction was studied and it was concluded at the RAN1#110 and RAN#97-e meetings that peak rate reduction option PR1 in SI, i.e., relaxation of the constraint (vLayers·Qm·f ≥ 4), is captured as the discussion scope for peak rate reduction in the WID.

As captured in TR38.865 section 7.3, significant impacts for PR1 are not expected, however, at least the exact value of relaxed constraint for peak rate reduction should be decided and the following agreements were made at the previous RAN1 meetings;

Agreement: - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X - If UE peak data rate reduction is supported as a standalone feature, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ Y. - FFS: the value of Y - Note: Whether this option is supported will be decided in RAN plenary. Agreement: Revise the earlier agreement by removing the square brackets like this: - The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306. - The same value for X is used for DL and UL Agreement: For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2. - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X.

In addition, at the RAN#99 meeting, the following proposal was endorsed, and hence the peak rate reduction can be supported as standalone feature as Rel-18 eRedCap targeting 10 Mbps;

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

Regarding the data rate for NR, it is calculated by the following equation according to TS38.306 [3];

wherein

J is the number of aggregated component carriers in a band or band combination

Rmax = 948/1024

For the j-th CC,

is the maximum number of layers

is the maximum modulation order

is the scaling factor

is the numerology

is the average OFDM symbol duration in a subframe for numerology

is the maximum RB allocation in bandwidth  with numerology

is the overhead and takes the following values. [0.14], for frequency range FR1 for DL

In the current specification, the constraint on vLayers·Qm·f is specified as vLayers·Qm·f ≥ 4. As explained above, this constraint would be relaxed for further peak data reduction, i.e., vLayers·Qm·f can be smaller than 4.

As concluded at the previous RAN plenary meetings, the minimum peak rate should be 10 Mbps for both peak rate reduction as add-on feature for UE BB bandwidth reduction and standalone feature.

For the case where peak rate reduction is the add-on feature, based on the agreement, the value X would be down-selected from 3 or 3.2. In our view, it can be allowed that the peak rate is bit smaller than 10 Mbps as 9.6 Mbps for 30 kHz SCS. Accordingly, we are open to discuss the constraint between 3 or 3.2. Furthermore, if the constraint is relaxed to 3 or 3.2, new scaling factor value can be introduced to meet the constraint. More specifically, when the scaling factor is 0.5 or 0.54, the relaxed constraint 3 or 3.2 can be achieved with 1 MIMO layer and 64 QAM.

Proposal 7: When the peak rate reduction is supported as add-on feature for UE BB bandwidth reduction,

- The constraint on the vLayers·Qm·f , i.e., X in the agreement, is relaxed to 3 or 3.2.

- New value for scaling factor 0.5 or 0.54 is introduced.

For the case where peak rate reduction is the standalone feature, similar to the add-on feature, it can be allowed that the peak rate is bit smaller than 10 Mbps as 9.9 Mbps for 15 kHz SCS and 9.6 Mbps for 30 kHz SCS. Accordingly, we are open to discuss the constraint between 0.7 or 0.75. Furthermore, if the constraint is relaxed to 0.7 or 0.75, new scaling factor value can be introduced to meet the constraint. More specifically, when the scaling factor is 0.12 or 0.125, the relaxed constraint 0.7 or 0.75 can be achieved with 1 MIMO layer and 64 QAM.

Proposal 8: When the peak rate reduction is supported as standalone feature,

- The constraint on the vLayers·Qm·f is relaxed to 0.7 or 0.75.

- New value for scaling factor 0.12 or 0.125 is introduced.

Table 1: Peak data rate for 15 kHz SCS [Mbps]

The red value represents the closest value larger than the target peak rate of 10Mbps.

| Number of PRB | v∙Q∙f | Peak rate |
| --- | --- | --- |
| 25 | 1.4 | 5.4 |
|  | 3 | 10.0 |
|  | 3.2 | 10.7 |
| 106 | 0.7 | 9.9 |
|  | 0.75 | 10.6 |
|  | 1 | 14.2 |

Table 2: Peak data rate for 30 kHz SCS [Mbps]

The red value represents the closest value larger than the target peak rate of 10Mbps.

| Number of PRB | v∙Q∙f | Peak rate |
| --- | --- | --- |
| 12 | 3 | 9.6 |
|  | 3.2 | 10.3 |
|  | 4 | 12.8 |
| 51 | 0.7 | 9.6 |
|  | 0.75 | 10.3 |
|  | 1 | 13.6 |

- Conclusion

In this contribution, we discussed UE BB bandwidth reduction and UE peak data rate reduction for further UE complexity reduction. Based on the discussion, we made following proposals.

Proposal 1:

- For a cell supporting Rel-18 RedCap UE but not supporting Rel-17 RedCap UE, a separate initial DL/UL BWP specific to Rel-18 eRedCap should be supported.

- For a cell supporting both Rel-17 RedCap and Rel-18 RedCap UEs,

  - a separate initial DL/UL BWP specific to Rel-18 eRedCap should be supported when non-RedCap and Rel-17 RedCap UEs share initial DL/UL BWP.

- The details for the signalling of initial BWP configuration should be up to RAN2.

Proposal 2:

When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, support X for the minimum time separation between RAR and Msg3 NT,1 + NT,2 + 0.5 + X ms for Rel-18 eRedCap UE as 0.5/0.25 ms for 15/30 kHz SCS.

Proposal 3:

A separate early indication for Rel-18 eRedCap via Msg1 and MsgA PRACH should be supported.

- Rel-17 early indication framework is reused for the separate early indication.

- Details for PRACH partitioning is up to RAN2.

Proposal 4: When intra-slot PUCCH frequency hopping within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Msg4/MsgB for RedCap UEs is disabled, the UE generates two base sequences for the PUCCH as if intra-slot frequency hopping is enabled for the PUCCH transmission.

Proposal 5:

Support the case where one unicast PDSCH scheduled with SI-RNTI or SI acquisition triggered by P-RNTI and another PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI are overlapped in time domain and the total number of PRBs for these PDSCHs is larger than 5MHz while the number of PRB for the unicast PDSCH does not exceed 25/12 RBs for 15/30 kHz SCS.

Proposal 6:

Confirm the following working assumption;

Working assumption:

- For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

  - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Proposal 7: When the peak rate reduction is supported as add-on feature for UE BB bandwidth reduction,

- The constraint on the vLayers·Qm·f , i.e., X in the agreement, is relaxed to 3 or 3.2.

- New value for scaling factor 0.5 or 0.54 is introduced.

Proposal 8: When the peak rate reduction is supported as standalone feature,

- The constraint on the vLayers·Qm·f is relaxed to 0.7 or 0.75.

- New value for scaling factor 0.12 or 0.125 is introduced.

References

[1] RP-223544, 		Enhanced support of reduced capability NR devices,	Ericsson.

[2] R1-2212982, 	RAN1 agreements for Rel-18 NR RedCap,		Ericsson.

[3] TS38.214 V17.5.0, 	NR; Physical layer procedures for data.