3GPP TSG RAN WG1 #112bis-e			     R1-2303723

e-Meeting, April 17th – 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on enhancements on Cell DTX/DRX mechanism

Agenda Item:	9.7.2

Document for: 	Discussion and Decision

- Introduction

At RAN#98-e meeting, WID on network energy savings was approved with the objectives as follows [1]:

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2] - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]. - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]

In this contribution, we share our views on enhancements on Cell DTX/DRX mechanism.

- Discussion

Cell DTX/DRX mechanism has been studied during the SI and captured in the TR of network energy savings [2]. The motivation of Cell DTX/DRX is to reduce gNB power consumption by informing UEs of Cell inactivity periods for transmission or reception, which potentially provides longer opportunities for cell inactivity. At least the following aspects should be discussed for Cell DTX/DRX mechanism; 1) UE behavior for Cell DTX/DRX, 2) configuration and activation/deactivation mechanism, and 3) alignment between Cell DTX and UE DRX.

Firstly, the UE behavior for Cell DTX/DRX needs to be clarified, especially during Cell DTX/DRX inactive periods. At the last meeting, the following agreement was achieved for further discussion on channels/signals, which the UE may be expected to not transmit or receive during Cell inactivity periods of Cell DTX/DRX [3]:

Agreement At least the following candidate signals/channels for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, are considered from RAN1 perspective for further discussion. The exact set of signals/channels that the UE may be expected to not transmit or receive is FFS. - DL - Periodic/Semi-persistent CSI-RS (including TRS) - PRS - PDCCH scrambled with UE specific RNTI - PDCCH in Type-3 CSS - SPS-PDSCH - UL - SR - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - CG-PUSCH Other signals/channels are not precluded

As described in the WID [1], impact on channels/signals for legacy UEs and IDLE/INACTIVE UEs should be avoided. Besides, dynamic channels/signals for both DL and UL can be allowed to transmit because the transmissions can be avoided by gNB implementation if NW energy saving is demanded. Therefore, UE should be expected to receive SSB, SIB, Paging PDSCH, PDCCH in Type0-PDCCH CSS, Type0A-PDCCH CSS, Type1-PDCCH CSS, and Type2-PDCCH CSS, and dynamic PDSCH for DL, and to transmit PRACH, aperiodic CSI report on PUSCH, aperiodic SRS, HARQ-ACK for dynamic PDSCH reception, HARQ-ACK for SPS PDSCH activation/deactivation, and dynamic grant PUSCH for UL, during the Cell inactivity periods. Other channels/signals can be considered for the candidate channels/signals, which the UE may be expected to not transmit or receive during Cell inactivity periods of Cell DTX/DRX. In summary, Table.1 can be starting point for further discussion.

Table. 1 UE transmission/reception restriction during Cell inactivity periods of Cell DTX/DRX

| DL/UL | Channel/signal | View |
| --- | --- | --- |
| DL | Periodic/Semi-persistent CSI-RS (FFS: for tracking) | Can be dropped. Regarding CSI-RS for tracking, it needs to review impact on time/freq. synchronization. |
|  | PRS | Can be dropped |
|  | PDCCH in Type 3-PDCCH CSS and USS | Can be dropped |
|  | SPS PDSCH | Can be dropped but no scheduling restriction should be imposed (i.e., gNB does not need to ensure that configuration of SPS PDSCH conflicts Cell inactivity periods) |
|  | SSB | No impact as noted in WID |
|  | SIB | No impact as noted in WID |
|  | Paging PDSCH | No impact as noted in WID |
|  | PDCCH in Type0-PDCCH CSS, Type0A-PDCCH CSS, Type1-PDCCH CSS, and Type2-PDCCH CSS | No impact as noted in WID |
|  | Dynamic PDSCH | Can be avoided by gNB implementation |
| UL | SR | Can be dropped |
|  | Periodic/Semi-persistent CSI report | Can be dropped |
|  | Periodic/Semi-persistent SRS | Can be dropped |
|  | HARQ-ACK for SPS PDSCH reception | Can be dropped |
|  | CG PUSCH | Can be dropped |
|  | PRACH | No impact as noted in WID |
|  | Dynamic grant PUSCH | Can be avoided by gNB implementation |
|  | Aperiodic CSI report on PUSCH | Can be avoided by gNB implementation |
|  | Aperiodic SRS | Can be avoided by gNB implementation |
|  | HARQ-ACK for dynamic PDSCH reception | Can be avoided by gNB implementation |
|  | HARQ-ACK for SPS PDSCH activation/deactivation | Can be avoided by gNB implementation |

Proposal 1:

UE behavior during Cell DTX/DRX inactivity periods should be further discussed. Table.1 can be starting point.

Secondly, configuration and activation/deactivation mechanisms need to be clarified. For configuration of Cell DTX/DRX, cell-specific or UE-specific manner can be considered. It would be straightforward to apply UE-specific manner to avoid legacy UE impact. Regarding activation and deactivation mechanisms of Cell DTX/DRX, semi-static or dynamic indication can be considered. To achieve flexible and fast activation and deactivation, dynamic indication would be more appropriate. If dynamic indication is supported, unit of activation/deactivation needs to be clarified, for which Cell-specific, UE-specific, or group-common based indication can be considered. To avoid legacy UE impact, UE-specific or group-common based indication should be preferable.

Proposal 2:

Configuration and activation/deactivation mechanisms of Cell DTX/DRX should be discussed.

Lastly, alignment between Cell DTX and UE DRX should also be clarified. It would be beneficial to align Cell DTX and UE DRX to efficiently reduce UE energy consumption in addition to gNB energy consumption. However, its necessity may depend on how the UE behavior during Cell DTX inactive periods is defined. For example, dynamic channels/signals are allowed to be transmitted during Cell DTX inactivity periods, the UE should wake-up for receiving them. Thus, there is no need to do such an alignment in this case. Otherwise, alignment between Cell DTX and UE DRX would bring better UE energy savings since the UE will wake-up less frequently compared to no alignment case.

Proposal 3:

Alignment between Cell DTX and UE DRX should be discussed in accordance with the UE behavior during Cell DTX inactivity periods.

- Conclusion

In this contribution, we present the following proposals for enhancements on Cell DTX/DRX mechanism.

Proposal 1:

UE behavior during Cell DTX/DRX inactivity periods should be further discussed. Following table can be starting point.

| DL/UL | Channel/signal | View |
| --- | --- | --- |
| DL | Periodic/Semi-persistent CSI-RS (FFS: for tracking) | Can be dropped. Regarding CSI-RS for tracking, it needs to review impact on time/freq. synchronization. |
|  | PRS | Can be dropped |
|  | PDCCH in Type 3-PDCCH CSS and USS | Can be dropped |
|  | SPS PDSCH | Can be dropped but no scheduling restriction should be imposed (i.e., gNB does not need to ensure that configuration of SPS PDSCH conflicts Cell inactivity periods) |
|  | SSB | No impact as noted in WID |
|  | SIB | No impact as noted in WID |
|  | Paging PDSCH | No impact as noted in WID |
|  | PDCCH in Type0-PDCCH CSS, Type0A-PDCCH CSS, Type1-PDCCH CSS, and Type2-PDCCH CSS | No impact as noted in WID |
|  | Dynamic PDSCH | Can be avoided by gNB implementation |
| UL | SR | Can be dropped |
|  | Periodic/Semi-persistent CSI report | Can be dropped |
|  | Periodic/Semi-persistent SRS | Can be dropped |
|  | HARQ-ACK for SPS PDSCH reception | Can be dropped |
|  | CG PUSCH | Can be dropped |
|  | PRACH | No impact as noted in WID |
|  | Dynamic grant PUSCH | Can be avoided by gNB implementation |
|  | Aperiodic CSI report on PUSCH | Can be avoided by gNB implementation |
|  | Aperiodic SRS | Can be avoided by gNB implementation |
|  | HARQ-ACK for dynamic PDSCH reception | Can be avoided by gNB implementation |
|  | HARQ-ACK for SPS PDSCH activation/deactivation | Can be avoided by gNB implementation |

Proposal 2:

- Configuration and activation/deactivation mechanisms of Cell DTX/DRX should be discussed.

Proposal 3:

- Alignment between Cell DTX and UE DRX should be discussed in accordance with the UE behavior during Cell DTX inactivity periods.

References

- RP-223540, New WID: Network energy savings for NR, Dec. 2022, Huawei.

- TR 38.864, v1.0.0

- 3GPP, RAN1#112 meeting, Chairman’s notes