3GPP TSG RAN WG1 #112bis-e			  R1-2303724

e-Meeting, April 17th – 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on XR specific capacity improvement enhancements

Agenda Item:	9.8.1

Document for: 	Discussion and Decision

- Introduction

At RAN#98-e meeting, WID on XR enhancements for NR was approved with the objective as follows [1]:

Specify the enhancements related to power saving: - DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2). Specify the enhancements related to capacity: - Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1); - BSR enhancements including at least new BS Table(s); (RAN2); - Delay reporting of buffered data in uplink; (RAN2); - Provision of XR traffic assistance information for DL and UL (e.g. periodicity); (RAN2); - Discard operation of PDU Sets (RAN2); Specify the enhancements for XR Awareness (RAN2, RAN3): TBD (detailed objectives will be further clarified at RAN#99 based on the conclusions of TR38.835, and work to be started only after RAN#99)

In this contribution, solutions for XR capacity enhancement are discussed.

- Discussions

2.1 Multiple CG PUSCH occasions in a CG period

Time domain resource allocation

At RAN1#112 meeting, possible alternatives for TDRA of multiple CG PUSCH occasions were agreed for further study.

Agreement For determination of the time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG, the following alternatives for further study: - Alt-A: TDRA determination based on repetition framework. - Alt-A1: Follow the time domain resource mapping of Type A repetition - N configured by higher layers or indicated by activation DCI - Single SLIV is determined from TDRA - The same SLIV in N PUSCH in consecutive slots per CG period - FFS for non-consecutive slots - FFS details, including related RRC parameters - Alt-A2: Follow the time domain resource mapping of Type B repetition - N configured by higher layers or indicated by activation DCI - Single SLIV is determined from TDRA - The SLIV used for 1st PUSCH per CG period. - N consecutive nominal PUSCHs with same duration per CG period - Note: N is not necessarily the repetition factor. FFS details, including related RRC parameters - Alt-B: TDRA determination based on NR-U framework - N and M configured by higher layers - Single SLIV is determined from TDRA. - The SLIV used for 1st PUSCH per CG period. - M consecutive PUSCH TOs with same duration in slot. The M PUSCH TOs are used in N consecutive slots per CG period - Note: N and M are configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. M and N configuration is independent from cgRetransmissionTimer configuration. - FFS details, including related RRC parameters - Alt-C: TDRA determination based on single DCI scheduling multiple PUSCHs - Alt-C1: Follow Rel-16 single DCI scheduling multiple PUSCHs - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with k2-r16 - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period - Note: N PUSCH TOs should be consecutive PUSCH TOs in consecutive slots. - FFS details, including related RRC parameters - Alt-C2: Follow Rel-17 single DCI scheduling multiple PUSCHs - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with extendedK2-r17 - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period - Note: N PUSCH TOs can be non-consecutive PUSCHs and/or in non-consecutive slots. - FFS details, including related RRC parameters

From specification impact perspective, new parameter for number of PUSCH occasions in one CG period need to be configured/indicated for Alt-A1 and Alt-A2. Moreover, for Alt-A2, UE behavior needs to be defined for the case when a nominal PUSCH occasion across slot boundary or DL/UL boundary. For example, a nominal PUSCH occasion is split into multiple actual PUSCH occasions when across slot boundary or DL/UL boundary, which is too complicated. For Alt-B, Rel-16 NR-U framework can work without any enhancement. For Alt-C1, Rel-16 single DCI scheduling multiple PUSCHs scheme can work without any enhancement. For Alt-C2, Rel-17 single DCI scheduling multiple PUSCHs scheme can work by simply extending the applicability to FR1/FR2-1, without any configuration or DCI design impact. To summary, Alt B/Alt-C1/Alt-C2 can work for multi-PUSCHs CG, with no/little additional specification impact.

From flexibility perspective, Alt-C2 is the most flexible method by separately indicating TDRA for each CG PUSCH occasion. However, multiple CG PUSCH occasions determined by Alt-A1/Alt-A2/Alt-B/Alt-C1 should be consecutive or in consecutive slots.

Considering specification impact and flexibility, Alt-C2 is preferred.

Proposal 1: Support Alt C-2 for TDRA of multiple CG PUSCHs in one CG period, i.e. following Rel-17 single DCI scheduling multiple PUSCHs.

In current spec, UE can’t validate a DCI for CG PUSCH activation if the DCI indicates more than one SLIV by the TDRA field according to the highlighted description below. If separate TDRA indication for each CG PUSCH occasion is achieved by TDRA field in the activation DCI indicating multiple SLIVs, the limitation should be relaxed.

TS38.213 10.2 PDCCH validation for DL SPS and UL grant Type 2 A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if - the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI or a G-CS-RNTI provided by g-cs-RNTI, and - the new data indicator field in the DCI format for the enabled transport block is set to '0', and - the DFI flag field, if present, in the DCI format is set to '0', and - the time domain resource assignment field in the DCI format indicates a row with single SLIV, and - if validation is for scheduling activation and if the PDSCH-to-HARQ_feedback timing indicator field in the DCI format is present, the PDSCH-to-HARQ_feedback timing indicator field does not provide an inapplicable value from dl-DataToUL-ACK-r16.

Proposal 2: Relax the limitation for validation of CG PUSCH activation DCI, when the TDRA field in the activation DCI indicates multiple SLIVs.

For Rel-17 multi-PUSCH scheduling in FR2-2, joint operation of multiple PUSCH repetitions and single DCI scheduling multiple PUSCHs is not supported due to UE and operation complexity. Similarly, joint operation of multiple CG PUSCH occasions in a CG period and CG PUSCH repetition is not preferred from the similar reasons.

For example, if rep-K is configured with value K in ConfiguredGrantConfig, and the TDRA field in the activation DCI indicates multiple SLIVs, UE may transmit on the multiple CG PUSCH occasions in one CG period, with each CG PUSCH occasion with only one repetition.

Proposal 3: Not support joint operation of multiple CG PUSCH occasions in a CG period and CG PUSCH repetitions.

- For example, if rep-K is configured with value K in ConfiguredGrantConfig, and the TDRA field in the activation DCI indicates multiple SLIVs, UE may transmit on the multiple CG PUSCH occasions in one CG period, with each CG PUSCH occasion with single repetition.

HARQ process ID

At RAN1#112 meeting, HARQ process ID determination issue was discussed, and possible alternatives were identified.

Agreement For determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH, consider the following alternatives: - Alt. 1: The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration. - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period. - Alt 1-1; X = 1 - Alt 1-2: X is the number of configured PUSCHs in a period - Alt 1-3: X is provided by RRC configuration. - FFS details - Alt. 2: Support that UE can decide, as in NR-U, the HARQ IDs for the multiple CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration - FFS details - Alt. 3: The HARQ process ID for the configured PUSCHs in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured. - FFS on potential enhancements different from previous alternatives - Alt 3-1: Note: Same HP ID would be used for all PUSCHs within a period. - FFS details - Alt 3-2: Note: Different HP ID could be used for all PUSCHs within a period. - FFS details - Alt. 4: The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured. - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period - FFS on potential enhancements different from previous alternatives - Alt 5: Support that UE can decide, as in NR-U, the HARQ IDs for the first CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period - FFS details - Alt 6: FFS other solutions Note: The case of one TB map to multiple PUSCHs is not considered here.

Principle of Alt 3 is to apply legacy CG HP ID calculation procedure separately for each configured PUSCH. If without any enhancement, only one HP ID can be obtained for per CG period. If same HP ID is applied, only single TB can be transmitted on the multiple CG occasions, which is not the motivation to study multi-PUSCHs CG in Rel-18. If different HP IDs are applied, how to obtain different HP IDs by legacy CG procedure needs to be studied.

Principle of Alt 1 and Alt 4 is to reuse HP ID increasing rule in single DCI scheduling multiple PUSCHs scheme, i.e. the HP ID for the first PUSCH is indicated and remaining PUSCHs are determined by incrementing the HP ID of the previous PUSCH. Difference between Alt 1 and Alt 4 lies in whether to enhance the determination for the first configured/valid PUSCH.

- For Alt 4, if legacy CG HP ID calculation procedure without any enhancement is applied to determine the HP ID for the first configured/valid PUSCH, HP ID of the second configured/valid CG PUSCH in a previous CG period may be equal to the HP ID of the first configured/valid CG PUSCH in the subsequent CG period. It may cause HP ID collision issue, e.g. when the first configured/valid PUSCH in a later CG period is transmitted before HARQ procedure of the second configured/valid PUSCH in a previous CG period is not completed. For example, assuming the HP ID for the first configured/valid CG PUSCH in a previous CG period is N, then the HP ID for the second configured/valid CG PUSCH in a previous CG period is N+1. However, the HP ID for the first configured/valid CG PUSCH in the next CG period is also equal to N+1.

- For Alt 1, the gap between CG PUSCH occasions with same HP ID would be larger by Alt 1-2 and Alt 1-3.

Regarding “configured/valid” in the Alt 1, using “valid” can save HP ID resource, i.e. the enhanced HP ID calculation procedure is applied to determine HP ID for the 1st valid PUSCH, and HP ID incrementation for remaining CG PUSCHs only consider valid PUSCHs. Moreover, in Rel-17 single DCI scheduling multi-PUSCHs, the HP ID is indicated for the first valid PUSCH, and incrementing of HP ID only counts valid PUSCHs.

Principle of Alt 2 is to reuse HP ID determination rule in licensed spectrum for each CG PUSCH, i.e. UE selects HP ID and reports the HP ID in CG-UCI. Alt 2 may cause UL signaling overhead issue.

Principle of Alt 5 is to determine HP ID for remaining CG PUSCHs based on HP ID of the first CG PUSCH, where HP ID determination rule in licensed spectrum is applied for the first CG PUSCH. In our understanding, Alt 5 can’t work since gNB may not know which configured CG PUSCH is the first transmitted PUSCH. Therefore, misalignment understanding for HP IDs of PUSCHs in one CG period is possible.

Based on above analysis, Alt 1 is preferred. The enhanced HP ID calculation procedure is applied to determine HP ID for the 1st valid PUSCH, and HP ID incrementation for remaining CG PUSCHs only consider valid CG PUSCHs.

Proposal 4: Support Alt 1 for HP ID determination of multiple CG PUSCHs in one CG period, i.e.

- The HARQ process ID for the first valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration.

  - X is the number of configured PUSCHs in a period, or X is provided by RRC configuration.

- The HARQ process ID of the remaining valid PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding valid PUSCH in the period.

MCS/FDRA

At RAN1#112 meeting, it was agreed that the configuration/indication parameters except MCS and FDRA of CG PUSCHs in a multi-PUSCHs CG configuration are the same, while the MCS and FDRA parameters are still FFS. In legacy CG, same MCS and FDRA parameters are applied to CG PUSCHs in different CG periods of the same CG configuration. The motivation to apply different MCS and FDRA parameters to multiple CG PUSCHs in one CG period is not justified so far. Therefore, same MCS/FDRA parameter indication/configuration for multiple CG PUSCHs in one CG period should be applied, unless the benefit of applying different indications/configurations is justified.

Proposal 5: Support same MCS/FDRA parameter indication/configuration for multiple CG PUSCHs in one CG period.

2.2 UE reporting indication of unused CG occasions by UCI

At RAN1#112 meeting, it was agreed that the physical channel to report UCI for indication of unused TOs is CG PUSCH. Remaining issues include UCI format, determination of CG PUSCHs carrying the UCI, detailed design for UCI indicating unused TOs.

UCI format

Three alternatives were agreed for down-selection for the UCI format.

Agreement Consider the following alternatives for “the UCI that provides information about unused CG PUSCH transmission occasions” for down-selection or revision - Alt. 1: “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI. - FFS on details - Alt. 2: “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI. - FFS on details - Alt. 3: “The UCI that provides information about unused CG PUSCH transmission occasions” replaces/re-purposes some field(s) of the CG-UCI. - FFS on details

The legacy CG-UCI only exists in unlicensed spectrum, to indicate information like HPN, NDI, COT sharing information, etc.

For licensed spectrum, the legacy CG-UCI indication bits are not needed. Alt-2 should not be a good solution for licensed spectrum, which causes signaling overhead with useless fields. Selection between Alt-1 and Alt-3 may depend on the size of UCI to indicate unused CG occasions. For example, if the size of UCI to indicate unused CG occasions is much smaller than the legacy CG-UCI size, Alt-3 also wastes UL resource. Otherwise, if the size of UCI to indicate unused CG occasions is larger than the legacy CG-UCI size, repurposing the legacy CG UCI fields is not enough. Therefore, Alt-1 is preferred for licensed spectrum. Multiplexing/encoding of the new UCI on CG PUSCH can reuse multiplexing/encoding procedures for legacy CG-UCI on CG-PUSCH in unlicensed spectrum.

For unlicensed spectrum, the legacy CG-UCI indication bits are useful. Alt-3 should not be supported for unlicensed spectrum. From UL signaling overhead perspective, Alt-1 and Alt-2 are similar. From UCI encoding/multiplexing perspective, Alt-1 will require more specification effort for encoding/multiplexing a new UCI and legacy CG-UCI on one CG PUSCH. Therefore, Alt-2 to add new bits in legacy CG-UCI to indicate unused CG occasions is preferred for unlicensed spectrum.

Proposal 6:

- In licensed spectrum, support Alt 1, i.e. “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI.

  - Multiplexing/encoding of the new UCI on CG PUSCH can reuse multiplexing/encoding procedures for legacy CG-UCI on CG-PUSCH in unlicensed spectrum.

- In unlicensed spectrum, support Alt 2, i.e. “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI.

CG PUSCH carrying the UCI

At RAN1#112 meeting, four options were identified for further down-selection.

Agreement For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, the following options for further down-scoping with possible revision, are considered for the transmission occasion of the UCI: - Option 1: A transmitted CG PUSCH, includes the UCI. - FFS details - Option 2: A transmitted CG PUSCH includes the UCI, if it is transmitted in an occasion determined by RRC. - FFS details - Option 3: A transmitted CG PUSCH includes the UCI, if it is transmitted in a pre-defined transmission occasion. - FFS details - Example of a pre-determined occasion: 1st configured PUSCH TO in a CG period or 1st configured PUSCH TO in a multiple CG periods - Option 4: A transmitted CG PUSCH includes the UCI, if it is transmitted in a transmission occasion determined satisfying given condition(s). - FFS details - Examples of a condition: A first transmitted PUSCH in a CG period, or a first PUSCH transmission within a multiple of CG periods. Other options are not precluded. Proponent companies to provide details.

From signaling overhead perspective, Option 1 may cause the most UL signaling overhead. From blind decoding perspective, Option 4 may cause gNB and UE understanding misalignment issue since gNB is not sure about which CG PUSCH is the first actually transmitted PUSCH. Therefore, Option1 and Option 4 are not preferred. Option 2 and Option 3 both can work and UL signaling overhead can be similar.

Proposal 7: For determining which CG PUSCH includes the UCI, option 1 and option 4 are not supported. Support option 2 or option 3.

Detailed design for UCI indicating unused TOs

Agreement For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the following options for further down-scoping, are considered for the information provided by the UCI: - Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused” - Option 1-1: The UCI provides the number of consecutive TO(s) in time domain. - Applicable numbers can be determined from information obtained from configuration. - FFS details - Option 1-2: The UCI provides a time duration/range that includes the consecutive TO(s) in time domain. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” (consecutive/non-consecutive TO(s) in time domain) - Option 2-1: The UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2-2: The UCI provides a bitmap where a bit corresponds to TOs within a time duration/range. The bit indicates whether all TOs within the time duration/range are “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - FFS whether/how the unused TO(s) can be associated to multiple CG configuration. - Other options are not precluded. Proponent companies to provide details.

The first question to be discussed is whether the UCI can indicate unused TOs of same or multiple CG configurations. From complexity perspective, association of unused TO indication with different CG configurations would be more complicated than unused TO indication only applying to TOs with the same CG configuration. Considering the limited TUs in Rel-18, unused TO indication only applying to TOs with the same CG configuration is preferred. Moreover, considering the main motivation to support UE reporting unused CG occasion information is the case of multiple CG occasions in one CG period, it is preferred to only focus on UCI indication design for the case of multi-PUSCHs CG.

Proposal 8: UCI indicates unused TOs of the same CG configuration as CG PUSCH carrying the UCI.

Regarding whether the UCI indicates consecutive or non-consecutive unused TOs, Option 2 can indicate consecutive or non-consecutive unused CG occasions, while option 1 can only indicate consecutive unused CG occasion. Option 2 is more flexible, at the cost of UCI size (if assuming same number of total indicated TOs). Indication of unused TOs is mainly for the case when there is an XR packet to be transmitted and UE only needs part of configured CG resources. From latency perspective, the used TOs will be consecutive TOs. Therefore, the unused TOs are very likely to be consecutive. Option 1 to indicate consecutive unused TOs is enough. Down-selection between option 1-1 and option 1-2 can rely on trade-off between UCI payload and indication granularity. Option 1-1 to directly indicate number of unused TOs is slightly preferred.

Proposal 9: For UCI indicating unused TOs, support option 1, i.e. the UCI indicates consecutive unused TOs.

For each option (i.e. either indicating consecutive or non-consecutive unused TOs), range of the indicated TOs by the UCI needs to be discussed. The key issue is the first TO for the UCI indication. Two alternatives can be considered:

- Alt 1: to define the first TO by specification, e.g. the first TO with X slots/symbols/TOs/CG periods after the ending/starting symbol of CG PUSCH carrying the UCI. The definition of X may need to consider required time for gNB to utilize the unused CG occasion information for other UEs.

- Alt 2: the UCI directly indicates the offset between the UCI and the first indicated TO, e.g. Y slots/symbols/TOs/CG periods after the ending/starting symbol of CG PUSCH carrying the UCI. Limitation on candidate values of the offset may be necessary, e.g. Y is no smaller than a certain value, in order to leave time for gNB to utilize the unused CG occasion information for other UEs.

Alt 2 is more flexible than Alt 1, at the cost of UCI payload. Selection between Alt 1 and Alt 2 may depend on which alternative is selected in previous issues. For example, if each CG PUSCH includes the UCI, Alt 1 is enough. If the UCI can only indicate consecutive unused TOs, and the UCI can occur only in certain CG PUSCH, Alt 2 is more suitable.

Proposal 10: The first TO for the UCI indication range can be determined considering following two alternatives:

- Alt 1: the first TO with X slots/symbols/TOs/CG periods after the ending/starting symbol of CG PUSCH carrying the UCI.

  - The definition of X may need to consider required time for gNB to utilize the unused CG occasion information for other UEs.

- Alt 2: the first indicated TO is indicated by the UCI, e.g. Y slots/symbols/TOs/CG periods after the ending/starting symbol of CG PUSCH carrying the UCI.

  - Limitation on candidate values of the offset may be necessary, e.g. Y is no smaller than a certain value, in order to leave time for gNB to utilize the unused CG occasion information for other UEs.

In Rel-17, single TB processing over multiple slots is supported for dynamically scheduled PUSCH and also CG PUSCH. It needs to be clarified whether such CG-UCI indication can be reported on a CG PUSCH configured with single TB processing over multiple slots. If yes, how to multiplex the CG-UCI with the CG PUSCH in multiple slots need to be studied, e.g. multiplexed on the first slot or multiplexed on each slot.

Proposal 11: Study whether/how to support such CG-UCI indication along with CG PUSCH configured with single TB processing over multiple slots.

- Conclusion

In this contribution, we discussed possible solutions for XR capacity improvement. We have following

proposals:

Proposal 1: Support Alt C-2 for TDRA of multiple CG PUSCHs in one CG period, i.e. following Rel-17 single DCI scheduling multiple PUSCHs.

Proposal 2: Relax the limitation for validation of CG PUSCH activation DCI, when the TDRA field in the activation DCI indicates multiple SLIVs.

Proposal 3: Not support joint operation of multiple CG PUSCH occasions in a CG period and CG PUSCH repetitions.

- For example, if rep-K is configured with value K in ConfiguredGrantConfig, and the TDRA field in the activation DCI indicates multiple SLIVs, UE may transmit on the multiple CG PUSCH occasions in one CG period, with each CG PUSCH occasion with single repetition.

Proposal 4: Support Alt 1 for HP ID determination of multiple CG PUSCHs in one CG period, i.e.

- The HARQ process ID for the first valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration.

  - X is the number of configured PUSCHs in a period, or X is provided by RRC configuration.

- The HARQ process ID of the remaining valid PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding valid PUSCH in the period.

Proposal 5: Support same MCS/FDRA parameter indication/configuration for multiple CG PUSCHs in one CG period.

Proposal 6:

- In licensed spectrum, support Alt 1, i.e. “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI.

  - Multiplexing/encoding of the new UCI on CG PUSCH can reuse multiplexing/encoding procedures for legacy CG-UCI on CG-PUSCH in unlicensed spectrum.

- In unlicensed spectrum, support Alt 2, i.e. “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI.

Proposal 7: For determining which CG PUSCH includes the UCI, option 1 and option 4 are not supported. Support option 2 or option 3.

Proposal 8: UCI indicates unused TOs of the same CG configuration as CG PUSCH carrying the UCI.

Proposal 9: For UCI indicating unused TOs, support option 1, i.e. the UCI indicates consecutive unused TOs.

Proposal 10: The first TO for the UCI indication range can be determined considering following two alternatives:

- Alt 1: the first TO with X slots/symbols/TOs/CG periods after the ending/starting symbol of CG PUSCH carrying the UCI.

  - The definition of X may need to consider required time for gNB to utilize the unused CG occasion information for other UEs.

- Alt 2: the first indicated TO is indicated by the UCI, e.g. Y slots/symbols/TOs/CG periods after the ending/starting symbol of CG PUSCH carrying the UCI.

  - Limitation on candidate values of the offset may be necessary, e.g. Y is no smaller than a certain value, in order to leave time for gNB to utilize the unused CG occasion information for other UEs.

Proposal 11: Study whether/how to support such CG-UCI indication along with CG PUSCH configured with single TB processing over multiple slots.

References

- RP-223502, New WID on XR Enhancements for NR

- 3GPP, RAN1#112 meeting, Chairman’s notes