3GPP TSG RAN WG1 #112bis-e			R1-2303727

e-Meeting, April 17th – April 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on L1 enhancements for inter-cell mobility

Agenda Item:	9.10.1

Document for: 	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID of ‘Further NR mobility enhancements’ was approved and the WID was further updated in RAN#97-e meeting [1].

In this contribution, we mainly discuss the enhancements for L1/L2 triggered inter-cell mobility (LTM), including L1 beam measurement and reporting, L1 beam indication, and L1/L2 cell switch indication.

- Scenario and procedure

In previous RAN1 meetings, the scenario and procedure of Rel-18 LTM were discussed. And RAN1 has made following agreements.

Agreement

-      RAN1 to further study the potential RAN1 enhancements and spec impact to perform at least the following procedures prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay / interruption

-        DL synchronization for candidate cell(s)

-       TRS tracking for candidate cell(s)

-       CSI acquisition for candidate cell(s)

-       Activation/Selection of TCI states for candidate cell(s), if feasible

-       Note: Uplink synchronization aspect will not be discussed under this A.I.

-       FFS: Whether the above procedures prior to the reception of L1/L2 cell switch command can be performed on candidate cell when it is deactivated SCell (if defined in RAN2)

-      Detailed discussion will be commenced after receiving RAN2 LS.

Agreement

- Regarding the potential RAN1 enhancements to reduce the handover delay / interruption for Rel-18 LTM

  - Support at least DL synchronization for candidate cell(s) based on at least SSB before cell switch command

    - Further study the necessary mechanism, e.g. signaling and UE capability

To reduce the latency between reception of cell switch command and data transmission on target cell, it is beneficial to perform and finish following procedures for candidate cells before cell switch happens, including DL/UL synchronization, TRS tracking, CSI acquisition, and activation of TCI states for candidate cells. Based on UE reported L1 beam measurement results, NW can select one or multiple target candidate cells to activate TCI states and to perform CSI acquisition. In last RAN1 meeting, DL synchronization for candidate cells before cell switch command has been supported, but other procedures are still for further study. We think it is better to discuss and decide those procedures early so we can have an overall understanding of the whole Rel-18 LTM procedure. On the other hand, RAN2 has agreed that SCell can be also configured as candidate cell. Note that above procedures has been supported for those candidate cells which are current activated SCells. Thus, above procedures prior to the reception of L1/L2 cell switch command can be applied to deactivated SCells as well. In this case, the UE behavior on deactivated SCells can be enhanced.

Proposal 1

- Support to perform following procedures prior to the reception of cell switch command,

  - TRS tracking for candidate cell(s)

  - CSI acquisition for candidate cell(s)

  - Activation/Selection of TCI states for candidate cell(s)

- Note above procedures prior to the reception of cell switch command have been supported for the candidate cells which are current activated SCell

- Above procedures prior to the reception of cell switch command can be supported for a candidate cell if it is deactivated SCell

- L1 beam measurement and reporting

L1 measurement RS

On the L1 measurement RS, following agreement was made in last RAN1 meeting. SSB based L1-RSRP has been supported. To facilitate following CSI measurement on candidate cells, we think CSI-RS based L1 measurement should be also supported. The configured CSI-RS can be associated with serving cells or candidate cells, and both L1 intra-frequency and inter-frequency measurements can be supported. For the CSI-RS type, we think at least CSI-RS for BM can be configured as CMR, while other CSI-RS types can be further studied, e.g., CSI-RS for tracking and CSI-RS for CSI configured as CMR, or CSI-IM (ZP-CSI-RS) configured as IMR.

Agreement

- For candidate cell measurement for Rel-18 LTM,

  - SSB based L1-RSRP is supported for intra-frequency measurement

  - SSB based L1-RSRP is supported for inter-frequency measurement from RAN1 point of view

  - FFS: L1-SINR, CSI-RS based L1-RSRP

Proposal 2

- Support to use CSI-RS associated with serving cells and candidate cells for both L1 intra-frequency and inter-frequency measurements.

  - At least support CSI-RS for BM to be configured as CMR.

  - Open to support CSI-RS for tracking and CSI-RS for CSI to be configured as CMR, and CSI-IM (ZP-CSI-RS) to be configured as IMR.

L1 measurement quantity

On the L1 measurement quantity, following agreement has been made in previous RAN1 meeting.

Agreement

- For candidate cell measurement for Rel-18 L1/L2 mobility,

  - L1-RSRP is supported for intra-frequency candidate cell measurement.

  - Further study the following measurement quantities for candidate cell measurement

    - L1-RSRP for inter-frequency (if supported)

    - L1-SINR for intra-frequency and inter-frequency (if supported)

- FFS: to assess the use case and the benefit of UL measurement instead of/in addition to DL L1 measurement, which includes:

  - How the UL measurement result is used, e.g. handover decision

  - Signals/channels used for UL measurement, e.g. SRS

  - Spec impact including other WGs, e.g. definition of gNB measurement, interface to transfer RS configuration or measurement results

  - Note: The next discussion will take place based on companies’ contribution in future meeting.

On L1 measurement quantity, since L1-RSRP and L1-SINR have been supported in Rel-17 for intra-frequency measurement, they should be supported in Rel-18 LTM for intra-frequency measurement as well. For inter-frequency measurement, L1-RSRP is less useful while L1-SINR is more important as it can reflect different interference levels on different frequencies. For example, in intra-band co-located CA scenario, L1-RSRP value would be almost the same on different CCs, and there is no point to use inter-frequency L1-RSRP measurement in the scenario. However, interference level would be different on different CCs, and L1-SINR reporting is beneficial for gNB to know interference level on each CC.

Some companies said that each L1-SINR measurement result is an instant value, which is not proper for L1 mobility decision. For this issue, following two methods can be studied to mitigate the impact brought by varying interference.

- For inter-frequency measurement, L1-RSRP can be used together with L1-SINR to select a beam with both good signal quality and low interference. Otherwise, a selected beam based on L1-SINR only may have low instant interference and low signal quality, which is not good for coverage.

- UE/event triggered report can be introduced, with the new event(s) considering time domain and/or spatial domain variations, e.g., a report is triggered when certain measurement threshold condition is met for a time duration or multiple beams. In this case, the impact of varying interference can be also mitigated. More details on UE/event triggered report are discussed in following sessions.

Proposal 3

- Support L1-SINR for intra-frequency and inter-frequency measurement. Further study following two methods to mitigate the impact of varying interference.

  - Use L1-RSRP and L1-SINR together to select the beams to report.

  - Introduce UE/event triggered reporting, with the new event(s) considering time domain and/or spatial domain variations, e.g., a report is triggered when certain measurement threshold condition is met for a time duration or multiple beams.

L1 measurement configuration/reporting

In last RAN1 meeting, following was agreed on configuration parameters.

Agreement

- For L1-RSRP measurement RS configuration

  - For SSB based L1-RSRP measurement:

    - As a starting point, at least the following information needs to be provided to a UE, e.g.

      - For intra- and inter- frequency: PCI or logical ID (e.g., as being defined in R17 ICBM), time domain (e.g. SMTC or periodicity and SSB position in burst)

      - For inter-frequency: frequency domain location (e.g. center frequency), SCS

      - FFS: transmission power (for pathloss calculation)

    - Note: other parameters included in the configuration can be further discussed

    - Including above agreement into the LS

In Rel-17 ICBM, the configuration of SSB transmission power is supported. In Rel-18 LTM, for measurement configuration, the SSB transmission power is also needed for pathloss calculation for UE power control procedure. In addition, for candidate cells on the same frequency as serving cell, considering unsynchronized NW, the SSB SCS, halfFrameIndex, and SFN offset could be different, which are also needed for intra-frequency and inter-frequency measurement configuration.

Proposal 4

- For SSB based L1-RSRP measurement for candidate cells, following parameters can be configured,

  - For intra-frequency: SSB transmission power, SSB SCS, halfFrameIndex, SFN offset

  - For inter-frequency: SSB transmission power, halfFrameIndex, SFN offset

In last RAN1 meeting, following was proposed by FL for L1 beam measurement/reporting.

- Important discussion in RAN1#112bis

  - FFS whether the configured candidate cell(s) can be activated

  - FFS how to choose the beams to be reported from multiple candidate cells, e.g. from all configured/activated candidate cells, from each candidate cell, from each group of candidate cells, from selected candidate cells

For the maximum number of configured candidate cells for L1 measurement/reporting, it is sufficient to support 7 for intra-frequency measurement, which is the same as Rel-17 ICBM. For inter-frequency measurement, considering the potential large number of CCs in CA scenario, a larger value may be needed. Note that at least the maximum number of configured candidate cells by RRC signaling can be larger than 7. If a large number of candidate cells are configured by RRC and up to 7 candidate cells are configured for L1 measurement/reporting, with UE movement, RRC-reconfiguration of candidate cells for L1 measurement/reporting may be needed and may happen frequently. To avoid frequent RRC-reconfiguration, MAC CE can be used to activate/deactivate beams/cells for L1 measurement. For CMR configuration, the cell index should be explicitly configured for SSB/CSI-RS in configuration signaling.

Proposal 5

- On L1 measurement configuration,

  - For the maximum number of candidate cells for L1 measurement/reporting,

    - Support 7 (same as Rel-17 ICBM) for intra-frequency measurement

    - Support to study larger value than 7 for inter-frequency measurement

  - Support to study MAC CE based activation/deactivation of candidate beams/cells for L1 measurement.

  - Support to explicitly configure cell index for SSB/CSI-RS in CSI configuration signaling.

In a single reporting instance, for the maximum number of reported beams across all measurement cells/RSs and frequencies, at least 4 can be considered for intra-frequency measurement report and new value larger than 4 can be considered for inter-frequency measurement report.

For beam selection for intra-frequency measurement, either L1-RSRP or L1-SINR can be used, same as legacy. For beam selection for inter-frequency measurement, as discussed above, L1-RSRP and L1-SINR can be used together to select the beams with both good signal strength and low interference. If both L1-RSRP and L1-SINR are used for beam selection, the beams can be selected from all the candidate cells. But if only L1-RSRP is used for beam selection, to consider the difference across frequencies, it is better to select beams per frequency. Another issue is whether to always include serving cell’s beam into the report. Since NW also configures separate L1 beam measurement/reporting for each serving cell for dynamic data scheduling, NW could obtain the L1 measurement results of serving cell from separate CSI report. Thus, there is no problem if the L1 beam report for candidate cells does not include serving cell’s beam.

Regarding the reporting format, we think the legacy reporting format in Rel-17 ICBM can be reused for Rel-18 LTM for both intra-frequency measurement and inter-frequency measurement. Since the reported re-indexing index of configured RS could imply the actual RS and its PCI, no new reporting content is needed to indicate cell index of each reported RS.

Proposal 6

- In a single reporting instance, for the maximum number of reported beams across all measurement cells/RSs and frequencies for Rel-18 L1/L2 LTM,

  - 4 or larger value can be supported for intra-frequency measurement report,

  - Larger value than 4 can be supported for inter-frequency measurement report.

- For beam selection for inter-frequency measurement,

  - L1-RSRP and L1-SINR can be used together to select the beams from all configured/activated cells.

  - If only L1-RSRP is used for beam selection, beams can be selected from configured/activated cells per frequency. In this case, the number of selected frequencies can be also configured.

- For reporting format, the legacy reporting format in Rel-17 ICBM can be reused for Rel-18 LTM for both intra-frequency and inter-frequency measurements.

For gNB scheduled L1 measurement report for Rel 18 LTM, semi-persistent report on PUSCH and aperiodic report on PUSCH have been supported. We think periodic report on PUCCH and semi-persistent report on PUCCH should be also supported as legacy. Some companies said that the UCI reporting payload will increase and thus PUCCH is not proper. However, the number of reported beams can be configured and controlled by the NW, which means that the UCI reporting payload may not increase based on configuration. To ensure NW scheduling and configuration flexibility, we think beam reporting on PUCCH can be supported.

Proposal 7

- For gNB scheduled L1 measurement report for Rel 18 LTM, support periodic report on PUCCH and semi-persistent report on PUCCH.

UE/event triggered L1 measurement/reporting

Since the triggering of cell switch is based on L1 measurement results, some enhancements are needed for current L1 beam measurement/reporting. For example, L1 measurement results may vary fast, thus it is difficult for NW to make proper mobility decision based on varied measurement results. If NW makes improper decision on L1/L2 cell switch, frequent cell switch or frequent ping-pong may happen. In addition, cell switch decision is made by NW based on L1 measurement reporting from UE, since NW does not know the variation of DL channel condition timely, the cell switch decision may be late in some cases. In addition, in order to timely know the DL channel condition, frequent beam reporting can be configured, but it will increase CSI reporting overhead. Hence, we could observe three main motivations for L1 beam measurement/reporting enhancement.

Observation 1

- L1 beam measurement/reporting enhancement is needed for following motivations.

  - robustness (e.g., to avoid frequent cell switch or ping-pong)

  - timely reporting (e.g., to avoid late cell switch)

  - overhead reduction (e.g., to avoid frequent UE reporting to reduce reporting overhead)

Agreement

- For L1 measurement report for Rel-18 L1/L2 mobility, if UE event triggered report for L1 measurement is supported based on further study

    - At least the following aspects may be considered

      - How to define UE event and exact definition of events,

      - Report container

      - Resource allocation/assignment for UE event triggered report

      - Necessity of indication to gNB when the condition UE event is met, and how

      - Necessity to define the condition to start/stop the reporting,

      - Contents of the report/reporting format, PCI, RS ID, measurement result etc.

      - The interaction with filtered L1 measurement results (if supported)

      - Support of simultaneous configuration of both UE event triggered and any of periodic/semi-persistence/aperiodic reporting, and solutions when both of them are configured.

      - Report destination, whether the report is sent to serving cell only or can be sent to one or more candidate cell(s).

      - Benefit when L3 measurement is involved

For above motivations, UE/event triggered report could address them. UE/event triggered report was discussed in last meeting and following agreement has been made.

For the event, we think legacy L3 events can be the starting point, e.g., A3 (Neighbour becomes offset better than SpCell), and A5 (SpCell becomes worse than threshold1 and neighbour becomes better than threshold2). To ensure robustness of the report, legacy L3 events can be enhanced considering time domain and/or spatial domain variations, e.g., a report is triggered when certain measurement threshold condition is met for a time duration or multiple beams. Similar as L3 events, the new event is configured by gNB and UE operates following the configured event. Thus, when the event is met and reporting is triggered, UE just needs to report the selected RS and measurement results, like L1 beam reporting. There is no need to report the condition to trigger the report.

Regarding the report container, it is straightforward to use MAC CE, which does not require preconfigured resources. Since the MAC CE is sent on PUSCH, it is also straightforward to be sent to serving cell only. For the reporting contents, it could reuse the design for L1 beam measurement/reporting. The format of the MAC CE can be left to RAN2.

When UE/event triggered report is configured, it is up to NW whether to configure gNB scheduled L1 measurement report at the same time, as they are separate configurations.

Proposal 8

- Support UE/event triggered L1 beam measurement/reporting with following design details.

  - Legacy L3 events can be starting point, e.g., A3 (Neighbour becomes offset better than SpCell), or A5 (SpCell becomes worse than threshold1 and neighbour becomes better than threshold2).

    - Legacy L3 events can be enhanced considering time domain and/or spatial domain variations, e.g., a report is triggered when certain measurement threshold condition is met for a time duration or multiple beams.

  - UE/event triggered report is reported by MAC CE. The report contents can reuse the design for L1 beam reporting.

  - UE/event triggered report is sent to serving cell only.

  - Support separate and simultaneous configuration of both UE/event triggered report and gNB scheduled L1 measurement report.

- L1 beam indication and L1/L2 cell switch indication

TCI framework

On TCI framework used for Rel-18 LTM, following agreement was made in previous RAN1 meeting.

Agreement

- The beam indication of candidate cell(s) for Rel-18 LTM should be designed based on the following:

  - Beam indication for Rel-18 LTM is designed based on Rel-17 unified TCI framework, if both serving cell and candidate cell support Rel-17 unified TCI framework

  - FFS: whether/how to design mechanism for Beam indication for Rel-18 LTM when at least one from serving cell and candidate cell supports only Rel-15 TCI framework.

  - Note: How and whether to indicate the new serving cell(s) and timing for beam indication are separately discussed

There is an FFS to discuss the scenario when at least one from serving cell and candidate cell supports only Rel-15 TCI framework. We think it is a possible scenario in NW that different cells in inter-band scenario are configured with different TCI frameworks. Thus, it is beneficial to consider this scenario. On the other hand, considering current workload, this scenario has lower priority than Rel-17 unified TCI framework and can be discussed later if time allows.

Proposal 9

- Open to discuss the FFS scenario as a low priority scenario if time allows.

  - FFS: whether/how to design mechanism for Beam indication for Rel-18 LTM when at least one from serving cell and candidate cell supports only Rel-15 TCI framework.

In last RAN1, following agreement was made on beam indication.

Agreement

- At least for Rel-17 unified TCI framework based beam indication included in cell switch command (i.e. scenario 2), beam indication applies to signals/channels that follow or are configured to follow Rel-17 unified TCI at the target cell(s)

- FFS: beam indication for mTRP case

If a candidate cell can be preconfigured as mTRP by RRC signaling, group-based beam reporting may be needed for the candidate cell to obtain two beams for mTRP operation before cell switch command. In addition, TCI state activation for a candidate cell should also consider the activation of two beams for a DCI codepoint. Such design will make the Rel-18 LTM configuration and procedure complicated. Thus, it is not preferred. The simple method is to not consider mTRP case in Rel-18 LTM mobility procedure. After cell switch command, if a candidate cell becomes serving cell, NW can configure group-based beam reporting and mTRP operation if needed by RRC re-configuration signaling for the serving cell.

Proposal 10

- Do not consider mTRP case in Rel-18 LTM mobility procedure.

Beam indication timing

On beam indication timing for candidate cells, following agreement was made in last meeting.

Agreement

- For beam indication timing for Rel-18 LTM,

    - Support Scenario 2: Beam indication together with cell switch command,

      - For Rel-17 unified TCI framework,

        - Beam indication indicates TCI state for each target serving cell

  - FFS: Scenario 1: Beam indication before cell switch command

  - FFS: Scenario 3: Beam indication after cell switch command

- FFS: Activation of TCI state(s) of target serving and/or candidate cell(s).

First, we do not support scenario 3. If the first beam indication for target candidate cell is after cell switch command, it causes additional delay before data transmission on target cell. Second, in our understanding, scenario 1 has been supported for following two cases, which means that beam indication before cell switch command already happens for the two cases,

- if the target cell has been configured as Rel-17 ICBM;

- if the target cell is one of current serving cells.

On the other hand, scenario 2 has been agreed, and the beam indication is carried by MAC CE. It is simpler to assume the beam indication field always exists in cell switch command. Therefore, if companies could have common understanding that Rel-17 ICBM and Rel-18 LTM can be configured and operated simultaneously for a UE, we think there is no need to further discuss scenario 1.

Rel-17 ICBM and Rel-18 LTM have separate RRC signaling to configure candidate cells. If a candidate cell with the same PCI is configured both in Rel-17 ICBM configuration signaling and Rel-18 LTM configuration signaling, both Rel-17 ICBM and Rel-18 LTM can be performed for this cell. There is no specific enhancement needed for such operation. NW will ensure the configuration for this cell configured in Rel-17 ICBM signaling and in Rel-18 LTM signaling to be the same.

Proposal 11

- Simultaneous operation of R17 ICBM and R18 LTM is supported.

  - No specific enhancement is needed. For a candidate cell with the same PCI configured in both Rel-17 ICBM configuration signaling and Rel-18 LTM configuration signaling, both Rel-17 ICBM and Rel-18 LTM can be performed for this cell.

To enable beam indication together with cell switch command, TCI states activation of candidate cells should be supported before cell switch command. Thus, MAC CE can be used to activate TCI states for candidate cells before cell switch command, and the beam indication in cell switch command is selected from the activated TCI states. To make the procedure simpler, one MAC CE can be used to activate TCI states for multiple candidate cells.

Proposal 12

- Support to use one MAC CE to activate TCI states for multiple candidate cells.

- The beam indication in cell switch command is selected from the activated TCI states.

Cell switch command

[FL proposal 5-4-v1] Wed

- From RAN1 point of view, at least the following information needs to be included in the cell switch command, which is conveyed by MAC CE

  - Information to identify the target cell(s)

    - The details including bit number are designed by RAN2

  - FFS: TA related information (up to the discussion in A.I. 9.12.2)

  - Beam Indication for the target SpCell

  - ID of the active DL and UL BWPs for the target SpCell

- [Study further the necessity/effectuality and benefit of the following field and corresponding UE procedure]

  - Triggering of aperiodic TRS transmitted from the target cell

  - Triggering the CSI acquisition of the target cell and reporting to the target cell

  - Triggering of aperiodic SRS transmission to the target cell

- FFS: the presence of each field (i.e. always present or configurable)

- FFS: the bit size of each field, or can be felt to RAN2

RAN2 has agreed to carry cell switch indication by MAC CE. For the contents in cell switch command MAC CE, we had some discussions in last meeting and FL had a following proposal.

First, we think some fields should always exist in the MAC CE, e.g., cell identify/cell group identify, TCI state ID/beam indication, and DL/UL BWP indication. Second, for some other fields, e.g., TA related information, whether such fields exist or not can be indicated by additional field in MAC CE. This is because, for some target cells, the TA has been maintained by UE and there is no need to indicate the TA value again. Third, for the other fields with additional functions, e.g., triggered aperiodic TRS transmission, triggered CSI acquisition and reporting, triggered aperiodic SRS transmission, we think they may be beneficial to enable fast data transmission on target cell, if TRS tracking and CSI acquisition procedures for candidate cells prior to the reception of cell switch command are not supported. Thus, they can be supported as well. But similar as TA, whether such fields exist or not can be indicated by additional field in MAC CE, if supported.

Proposal 13

- For the contents in cell switch indication MAC CE,

  - following fields always exist,

    - Cell identity / Cell group identity

    - TCI state ID/Beam indication

    - DL/UL BWP indication

  - Whether other fields exist or not can be indicated by additional fields in MAC CE, e.g.,

    - TA related information

    - Triggered aperiodic TRS transmission on target cell

    - Triggered CSI acquisition and reporting on target cell

    - Triggered aperiodic SRS transmission on target cell

- Conclusion

In this contribution, we discussed the L1 enhancements for inter-cell mobility. We have following observations and proposals.

Observation 1

- L1 beam measurement/reporting enhancement is needed for following motivations.

  - robustness (e.g., to avoid frequent cell switch or ping-pong)

  - timely reporting (e.g., to avoid late cell switch)

  - overhead reduction (e.g., to avoid frequent UE reporting to reduce reporting overhead)

Proposal 1

- Support to perform following procedures prior to the reception of cell switch command,

  - TRS tracking for candidate cell(s)

  - CSI acquisition for candidate cell(s)

  - Activation/Selection of TCI states for candidate cell(s)

- Note above procedures prior to the reception of cell switch command have been supported for the candidate cells which are current activated SCell

- Above procedures prior to the reception of cell switch command can be supported for a candidate cell if it is deactivated SCell

Proposal 2

- Support to use CSI-RS associated with serving cells and candidate cells for both L1 intra-frequency and inter-frequency measurements.

  - At least support CSI-RS for BM to be configured as CMR.

  - Open to support CSI-RS for tracking and CSI-RS for CSI to be configured as CMR, and CSI-IM (ZP-CSI-RS) to be configured as IMR.

Proposal 3

- Support L1-SINR for intra-frequency and inter-frequency measurement. Further study following two methods to mitigate the impact of varying interference.

  - Use L1-RSRP and L1-SINR together to select the beams to report.

  - Introduce UE/event triggered reporting, with the new event(s) considering time domain and/or spatial domain variations, e.g., a report is triggered when certain measurement threshold condition is met for a time duration or multiple beams.

Proposal 4

- For SSB based L1-RSRP measurement for candidate cells, following parameters can be configured,

  - For intra-frequency: SSB transmission power, SSB SCS, halfFrameIndex, SFN offset

  - For inter-frequency: SSB transmission power, halfFrameIndex, SFN offset

Proposal 5

- On L1 measurement configuration,

  - For the maximum number of candidate cells for L1 measurement/reporting,

    - Support 7 (same as Rel-17 ICBM) for intra-frequency measurement

    - Support to study larger value than 7 for inter-frequency measurement

  - Support to study MAC CE based activation/deactivation of candidate beams/cells for L1 measurement.

  - Support to explicitly configure cell index for SSB/CSI-RS in CSI configuration signaling.

Proposal 6

- In a single reporting instance, for the maximum number of reported beams across all measurement cells/RSs and frequencies for Rel-18 L1/L2 LTM,

  - 4 or larger value can be supported for intra-frequency measurement report,

  - Larger value than 4 can be supported for inter-frequency measurement report.

- For beam selection for inter-frequency measurement,

  - L1-RSRP and L1-SINR can be used together to select the beams from all configured/activated cells.

  - If only L1-RSRP is used for beam selection, beams can be selected from configured/activated cells per frequency. In this case, the number of selected frequencies can be also configured.

Proposal 7

- For gNB scheduled L1 measurement report for Rel 18 LTM, support periodic report on PUCCH and semi-persistent report on PUCCH.

Proposal 8

- Support UE/event triggered L1 beam measurement/reporting with following design details.

  - Legacy L3 events can be starting point, e.g., A3 (Neighbour becomes offset better than SpCell), or A5 (SpCell becomes worse than threshold1 and neighbour becomes better than threshold2).

    - Legacy L3 events can be enhanced considering time domain and/or spatial domain variations, e.g., a report is triggered when certain measurement threshold condition is met for a time duration or multiple beams.

  - UE/event triggered report is reported by MAC CE. The report contents can reuse the design for L1 beam reporting.

  - UE/event triggered report is sent to serving cell only.

  - Support separate and simultaneous configuration of both UE/event triggered report and gNB scheduled L1 measurement report.

Proposal 9

- Open to discuss the FFS scenario as a low priority scenario if time allows.

  - FFS: whether/how to design mechanism for Beam indication for Rel-18 LTM when at least one from serving cell and candidate cell supports only Rel-15 TCI framework.

Proposal 10

- Do not consider mTRP case in Rel-18 LTM mobility procedure.

Proposal 11

- Simultaneous operation of R17 ICBM and R18 LTM is supported.

  - No specific enhancement is needed. For a candidate cell with the same PCI configured in both Rel-17 ICBM configuration signaling and Rel-18 LTM configuration signaling, both Rel-17 ICBM and Rel-18 LTM can be performed for this cell.

Proposal 12

- Support to use one MAC CE to activate TCI states for multiple candidate cells.

- The beam indication in cell switch command is selected from the activated TCI states.

Proposal 13

- For the contents in cell switch indication MAC CE,

  - following fields always exist,

    - Cell identity / Cell group identity

    - TCI state ID/Beam indication

    - DL/UL BWP indication

  - Whether other fields exist or not can be indicated by additional fields in MAC CE, e.g.,

    - TA related information

    - Triggered aperiodic TRS transmission on target cell

    - Triggered CSI acquisition and reporting on target cell

    - Triggered aperiodic SRS transmission on target cell

References

- 3GPP RP-222332, “Revised WID on Further NR mobility enhancements”, RAN#97e, Sep. 2022.