3GPP TSG RAN WG1 #112bis-e			R1-2303728

e-Meeting, April 17th – April 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Timing advance enhancement for inter-cell mobility

Agenda Item:	9.10.2

Document for: 	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID of ‘Further NR mobility enhancements’ was approved and the WID was further updated in RAN#97-e meeting [1].

In this contribution, we mainly discuss the enhancements for timing advance for L1/L2 triggered inter-cell mobility (LTM).

- Initial TA acquisition of candidate cells

2.1 Enhancements of PDCCH ordered RACH

In previous RAN1 meeting, enhancement for PDCCH ordered RACH of candidate cells in Rel-18 LTM was discussed. And RAN1 has made following agreements in common for PDCCH ordered RACH with/without RAR.

Agreement For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated - If reception of RAR is not configured/indicated (without RAR) - TA value of candidate cell is indicated in cell switch command - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order - If reception of RAR is configured/indicated (with RAR), FFS - whether RAR is received from serving cell or candidate cell - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE - content of RAR - FFS: signaling for configuration/indication of whether RAR needs to be received - UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM - Send LS to RAN2 and RAN3 to check the feasibility about this agreement Note: Definition of candidate cells is up to RAN2 Agreement The PDCCH order from the source cell contains the indication of candidate cell. - The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity Agreement For PDCCH-order based RACH for TA measurement for candidate cells, legacy CBRA is not supported Agreement Whether RAR needs to be received is configured by RRC. Agreement study at least the following issues on PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured - Whether gap between the DCI and PRACH longer than timeline defined in spec is needed - Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx

We should discuss details of PDCCH ordered RACH to complete it. Thus, we suggest four proposals below. Firstly, we should discuss what unit configuration for RAR reception is set in. There can be three alternatives: Alt 1: per

Figure 1: Alternatives on configuration unit of RAR reception

candidate cell, Alt 2: for all/per non-serving cells, Alt 3: for all candidate cells. They are shown in Figure 1. For Alt 1, NW has full flexibility to decide whether RAR reception is needed per candidate cell. For Alt 2, NW can configure it for all non-serving cells or per non-serving cell. For current serving cells, RAR can be always received as like legacy. For Alt 3, NW can configure it for all candidate cells. It is simple, but NW doesn’t have flexibility. We analyze this from NW architecture perspective. If all the candidate cells are in intra-DU scenario, there is no problem for all alternatives. However, if a candidate cell in inter-DU is included in candidate cell configuration, different configuration for RAR reception should be considered for different candidate cells, (i.e., Alt 1/2 is good.). This is because that configuration of RAR reception can be different between a candidate cell in intra-DU and in inter-DU. At least for a candidate cell in inter-DU, it is better to not configure RAR reception because if RAR is received on serving cell, it may lead to latency on inter-DU information exchange, and if RAR is received on the candidate cell on which PRACH is sent, it requires additional configuration of Type-1 PDCCH CSS of the candidate cell.

Observation 2.1

PDCCH ordered RACH with RAR for a candidate cell in inter-DU is not beneficial from latency or additional configuration perspective.

Thus, we prefer Alt 1 considering NW architecture and configuration flexibility.

Proposal 2.1

Whether RAR needs to be received is configured by RRC per candidate cell.

Secondly, we should discuss baseline UE approach for PDCCH ordered RACH. In previous RAN1 meeting, the combination with RAR only and without RAR only was supported. We prefer ‘with RAR only’. If ‘without RAR only’ is baseline, on current serving cell, legacy procedure cannot be performed.

Proposal 2.2

Support ‘with RAR only’ as baseline at least for current serving cell which is also candidate cell.

Thirdly, we should discuss the type of PRACH resource. In previous RAN1 meeting, some companies have concern about that only CFRA is supported. However, we are not sure how CBRA without Msg.2/3/4 works. If CBRA without Msg.2/3/4 is used, we are not sure how NW knows which UE is for Rel-18 LTM and how UE recognizes its RNTI without Msg.2/3/4. Thus, we think CFRA should be baseline for PDCCH ordered RACH for Rel-18 LTM.

Proposal 2.3

For PDCCH ordered RACH for TA acquisition, CFRA should be baseline.

Fourthly, we should discuss indication of cell identity in DCI format 1_0 for PDCCH order. For PDCCH ordered RACH for Rel-18 LTM, one candidate cell can be indicated by PDCCH order. If we consider the number in Rel-17 ICBM as baseline, there can be at least 8 candidate cells (i.e., SpCell is included.). Thus, at least 3 reserved bits in DCI format 1_0 for PDCCH order can be used for indication of cell identity. The correspondence between DCI bits and candidate cells is shown in Figure 2.

Proposal 2.4

At least 3 reserved bits in DCI format 1_0 for PDCCH order can be used for indication of cell identity.

Figure 2: The correspondence between DCI bits and candidate cells

With RAR

On ‘with RAR’, we should discuss two FFSs. Firstly, we should discuss whether RAR is received from serving cell or candidate cell. As we analyze above, RAR reception for a candidate cell in inter-DU is not beneficial from latency or additional configuration perspective. Thus, we discuss for a candidate cell in intra-DU. For candidate cell in intra-DU, we can assume that there is no concern like above. Thus, it is better to receive from SpCell like legacy CA.

Proposal 2.5

For a candidate cell which is intra-DU candidate cell, RAR should be received on SpCell.

Secondary, we should discuss RAR contents. In previous RAN1 meeting, RAN1 has made following agreement for RAR contents.

Agreement If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell. - The maximum number of TA values memorized by UE is a UE capability - FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR

We assume that one candidate cell configured with RAR reception can be triggered by PDCCH order, like legacy CA because it can be complex to trigger PDCCH ordered RACH for multiple candidate cells configured with RAR reception. Thus, UE knows for which cell the RAR is intended, so candidate cell ID may not be needed. However, UE ID should be contained in RAR like legacy for Rel-18 LTM.

Proposal 2.6

If reception of RAR is configured, RAR should contains UE ID.

Without RAR

On ‘without RAR’, we should discuss three FFSs. Firstly, we should discuss re-transmission of PRACH when RAR reception is not configured. In previous RAN1 meeting, RAN1 has made following agreement.

Agreement on whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives. - Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1) - Alt 2: UE autonomous Re-transmission of PRACH is allowed, The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax

In legacy PDCCH ordered RACH, UE autonomously retransmissions PRACH with power ramping when RAR is not received during RAR window. However, when candidate cell is configured without RAR reception, if UE always retransmits PRACH the maximum number of times, it wastes of resources as NW may have successfully decoded the PRACH preamble. Thus, UE autonomous re-transmission of PRACH should not be allowed. If NW cannot receive PRACH, NW can initiate new PDCCH ordered RACH. Also, the parameter of PreambleTransMax = 1 should not be used to achieve this because if this candidate cell becomes new serving cell, even for legacy PRACH procedure, PreambleTransMax = 1 is used in this case. It is unreasonable. Thus, we support Alt1 without the example method.

Proposal 2.7

Support Alt 1: UE autonomous re-transmission of PRACH is not allowed.

- Define above UE behavior in spec.. Not support to set PreambleTransMax = 1 to achieve above. The parameter of PreambleTransMax follows legacy configuration, which can be used for legacy RACH procedure.

Secondary, we should discuss how UE determines the transmit power of subsequent PRACH by PDCCH order. We think that reusing the parameter of legacy power ramping (e.g., PREAMBLE_RECEIVED_TARGET_POWER and PREAMBLE_POWER_RAMPING_COUNTER) is reasonable because it is designed for PRACH retransmission. However, UE cannot know when to increment counter for power, so a certain condition can be needed. We can consider two conditions: Alt 1: Reception of PDCCH order for same candidate cell on the same beam, (e.g., same candidate cell ID with same SS/PBCH index), Alt 2: Explicit indication in PDCCH order, (e.g., the 1 reserved bit can be used for indication whether to increase power or not.).

Proposal 2.8

UE increased PRACH power and increments counter for power by one of the following alternatives:

- Alt 1: Reception of PDCCH order for same candidate cell on the same beam, (e.g., same candidate cell ID with same SS/PBCH index)

- Alt 2: Explicit indication in PDCCH order, (e.g., the 1 reserved bit can be used to indicate whether to increase power or not.)

Proposal 2.9

For PDCCH ordered RACH for candidate cell, when reception of RAR is not configured,

- the parameter for power ramping, (e.g., PREAMBLE_RECEIVED_TARGET_POWER and PREAMBLE_POWER_RAMPING_COUNTER) can be reused for the transmit power of subsequent PRACH triggered by PDCCH order, if power ramping occurs.

2.2 UE-based TA measurement

In previous RAN1 meeting, we discussed UE-based TA measurement. However, there is no agreement on it because time is not enough for discussion and RAN1 spec. impact is not clear. So, we should further discuss it. Procedure of UE-based TA measurement in our assumption is shown in Figure 3. To support UE based TA measurement, some enhancements are needed, so, we analyse it below. Firstly, UE needs to be notified by NW whether to perform UE based TA measurement or not. For UE-based TA measurement, a reference cell needs to be set up for the UE to measure the DL difference timing. The reference cell to be set should be configured as part of the candidate cell configuration, as it is simple for the UE to refer to the same cell after cell switch. Secondly, for UE based TA measurement, an offset value from the reference cell needs to be configured for the UE to calculate the DL difference timing when DL is unsynchronized between reference cell and a candidate cell. Since this offset value is a fixed value assuming that the reference cell is fixed, it is considered better to be configured as part of candidate cell configuration.

Proposal 2.10

For UE based TA measurement, the reference cell and the offset values should be configured as part of candidate configuration.

Figure 3: Assumed procedures of UE-based TA measurement.

Thirdly, for UE-based TA measurement, it needs to be considered when to trigger it. NW needs to know whether UE-based TA measurement is performed or not for candidate cell (i.e., NW needs to know whether UE has available TA for candidate cell.). Thus, UE-based TA measurement can be triggered by NW. Considering PDCCH ordered RACH has been supported for Rel-18 LTM, it can be also used to trigger UE-based TA measurement.

Proposal 2.11

PDCCH order can be reused to trigger UE based TA measurement for candidate cell(s).

Fourthly, for UE-based TA measurement, it needs to be considered when to apply the TA of candidate cell. As for the TA derived by UE-based TA measurement, it is considered appropriate to be applied after cell switch command, since it is for a candidate cell. Also, after the cell switch, the candidate cell to which the TA is applied becomes a serving cell, and the subsequent TA derivation is considered to follow the legacy procedure, so it is better to apply it to the first UL transmission.

Proposal 2.12

For UE based TA measurement, TA of target cell is applied after cell switch command for first UL transmission.

Fifthly, we think UE-based TA measurement can UE optional. And new UE capability can be introduced for it. For TA acquisition of candidate cells, PDCCH ordered RACH should be supported by default.

Proposal 2.13

For TA acquisition of candidate cells for Rel-18 LTM, PDCCH ordered RACH is supported by default.

- UE-based TA measurement is UE optional.

- Configuration for TA acquisition

In 110bis-e meeting, TA acquisition of the candidate cell(s) before cell switch command is received in L1/L2 based mobility was agreed, but whether this can be applied to the candidate cell when it is deactivated SCell was FFS. This is because some companies said that definition of deactivated SCell as candidate cells is ambiguous. On the other hand, in the RAN2#119bis meeting, the following agreements were made.

・A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered. ・For L1L2 mobility, Target Pcell/SCell can be current SCell/PCell, i.e., current SCell/PCell can be configured as candidates.

From the agreements above, we can assume that current PCell/SCell can be configured as candidate cells by RRC signaling. In other words, these agreements clarify that deactivated SCell is supported as the candidate cell. So, we can support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility when it is deactivated SCell.

Proposal 3.1

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility when it is deactivated SCell.

For TA acquisition of a candidate cell before cell switch command is received, the association between TA/TAG and candidate cell was discussed in the last meeting, where explicit and implicit association were raised. On the other hand, in the RAN2#119bis meeting, the following agreement was made.

RAN2 assumes that candidate cell configuration can only be modified / released by Network.

From the agreements above, it is simple and straightforward to consider explicit association, e.g., the association is provided as a part of candidate cell(s) configuration. If a candidate cell is a deactivated SCell, since it is not decided yet whether any signaling/information can be transmitted on deactivated SCell, implicit association is not desirable. In addition, even if some signaling transmission may be allowed on deactivated SCell, it may not apply to any deactivated SCell. Thus, explicit association should be supported.

Proposal 3.2

For TA acquisition of a candidate cell before cell switch command is received, support Alt2: Associate TA/TAG and candidate cell explicitly.

- The association is provided as a part of candidate cell(s) configuration which is CellGroupConfig of candidate cell.

- This also applied to candidate cell when it is deactivated SCell.

- Conclusion

In this contribution, we discussed enhancements on timing advance management. Based on the discussion, we made following observations and proposals.

Observation 2.1

PDCCH ordered RACH with RAR for a candidate cell in inter-DU is not beneficial from latency or additional configuration perspective.

Proposal 2.1

Whether RAR needs to be received is configured by RRC per candidate cell.

Proposal 2.2

Support ‘with RAR only’ as baseline at least for current serving cell which is also candidate cell.

Proposal 2.3

For PDCCH ordered RACH for TA acquisition, CFRA should be baseline.

Proposal 2.4

At least 3 reserved bits in DCI format 1_0 for PDCCH order can be used for indication of cell identity.

Proposal 2.5

For a candidate cell which is intra-DU candidate cell, RAR should be received on SpCell.

Proposal 2.6

If reception of RAR is configured, RAR should contains UE ID.

Proposal 2.7

Support Alt 1: UE autonomous re-transmission of PRACH is not allowed.

- Define above UE behavior in spec.. Not support to set PreambleTransMax = 1 to achieve above. The parameter of PreambleTransMax follows legacy configuration, which can be used for legacy RACH procedure.

Proposal 2.8

UE increments counter for power by one of the following alternatives:

- Alt 1: Reception of PDCCH order for same candidate cell on the same beam, (e.g., same candidate cell ID with same SS/PBCH index)

- Alt 2: Explicit indication in PDCCH order, (e.g., the 1 reserved bit can be used to indicate whether to increase power or not.)

Proposal 2.9

For PDCCH ordered RACH for candidate cell, when reception of RAR is not configured,

- the parameter for power ramping, (e.g., PREAMBLE_RECEIVED_TARGET_POWER and PREAMBLE_POWER_RAMPING_COUNTER) can be reused for the transmit power of subsequent PRACH triggered by PDCCH order, if power ramping occurs.

Proposal 2.10

For UE based TA measurement, the reference cell and the offset values should be configured as part of candidate configuration.

Proposal 2.11

PDCCH order can be reused to trigger UE based TA measurement for candidate cell(s).

Proposal 2.12

For UE based TA measurement, TA of target cell is applied after cell switch command for first UL transmission.

Proposal 2.13

For TA acquisition of candidate cells for Rel-18 LTM, PDCCH ordered RACH is supported by default.

- UE-based TA measurement can UE optional.

Proposal 3.1

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility when it is deactivated SCell.

Proposal 3.2

For TA acquisition of a candidate cell before cell switch command is received, support Alt2: Associate TA/TAG and candidate cell explicitly.

- The association is provided as a part of candidate cell(s) configuration which is CellGroupConfig of candidate cell.

- This also applied to candidate cell when it is deactivated SCell.

Reference

[1] 3GPP RP-222332, “Revised WID on Further NR mobility enhancements”, RAN#97e, Sep. 2022.