3GPP TSG RAN WG1 #112bis-e			  R1-2303731

e-Meeting, April 17th – April 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on PRACH coverage enhancements

Agenda Item:	9.12.1

Document for: 	Discussion and Decision

- Introduction

At RAN#96-e meeting, revised WID on further NR coverage enhancements for NR was approved with the objective as follows [1]:

- Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beams for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable. - Study and if necessary specify following power domain enhancements - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1) - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1) - Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

In this contribution, remaining issues for PRACH coverage enhancement are discussed.

- Discussions

2.1 Multiple PRACH transmissions with the same Tx beam

Issue 1: Application use case

According to the WID objectives, multiple PRACH transmissions with the same Tx beam will be specified for 4-step RACH procedure in Rel-18. Considering there are multiple RACH types, e.g., contention free RACH (CFRA) and contention based RACH (CBRA), application use cases for multiple PRACH transmissions needs to be clarified first.

From specification perspective, multiple PRACH transmissions for CFRA may have smaller impact than multiple PRACH transmissions for CBRA, and also smaller impact on legacy UEs.

From coverage extension perspective, the realistic coverage would generally be determined based on initial access signal/channels. Thus, PRACH coverage extension during initial access (e.g., CBRA) would be very important. For RACH during CONNECTED MODE (e.g., CFRA and CBRA), PRACH coverage extension may also be needed. For example, multiple PRACH transmissions for CFRA can improve PRACH detection rate in SNR limited scenarios, e.g. for handover and beam failure recovery cases.

Therefore, multiple PRACH transmissions should be supported for both CBRA and CFRA, considering both RACH during initial access and RACH in CONNECTED mode in Rel-18.

Proposal 1: Support multiple PRACH transmissions for RA during initial access (e.g. CBRA) and RA during CONNECTED MODE (e.g. CFRA and CBRA).

Issue 2: Number of multiple PRACH transmissions

At RAN1#112 meeting, it was agreed gNB can configure one or multiple values from {2, 4, 8} for the number of PRACH transmissions.

Agreement For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions. - If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported. - FFS: details Agreement Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx beams.

When multiple candidate values for number of multiple PRACH transmissions are configured, how to determine the value needs to be discussed.

When UE-specific signaling is not available for RA, e.g., RA initiated for initial access, it should be allowed that UE determines the number of PRACH transmissions (including whether single or multiple PRACH transmissions) based on certain condition(s) by itself. It was agreed in RAN1#111 meeting that at least RSRP of the selected SSB/CSI-RS is used to determine the number of PRACH transmissions for the first RACH attempt. How to determine the number of PRACH transmission for other RACH attempts than the first RACH attempt is still not clear. In our understanding, RSRP of the selected SSB/CSI-RS is helpful to reflect the current channel condition and required SNR gap for any RACH attempt. Therefore, RSRP threshold(s) should be considered to determine the number of repetitions for any RACH attempt. The number of SS-RSRP thresholds is dependent on the configured number of candidate values for the number of multiple PRACH transmissions. For example, if candidate values for multiple PRACH transmissions configured by gNB are {2, 4, 8}, UE can determine the number of PRACH transmissions (from 1, 2, 4, 8) by comparing the configured SS-RSRP thresholds and RSRP of the selected CSI-RS/SSB.

For PDCCH ordered RA, it is also possible to indicate the number of transmissions in PDCCH order. Following PDCCH order indication is simpler and more efficient than UE autonomous determination based on certain condition. A possible method is to indicate the value via ‘reserved bits’ in the DCI directly. Another method is that the PDCCH order indicates multiple mask indexes when triggering RA with multiple PRACH transmissions, with each mask index for corresponding PRACH transmission. For this case, the number of PRACH transmissions is implicitly determined by the number of mask indexes.

Proposal 2: When multiple values are configured for the number of multiple PRACH transmissions,

- For RA without PDCCH order, UE determines the number of PRACH transmissions based on RSRP of the selected SSB/CSI-RS and configured SSB/CSI-RS RSRP thresholds.

- For PDCCH ordered RA, the PDCCH order DCI indicates the number of PRACH transmissions, which can be explicitly indicated by ‘reserved bits’, or implicitly indicated by the number of indicated mask indexes.

Issue 3: PRACH resource for multiple PRACH transmissions

PRACH resource for single PRACH transmission and multiple PRACH transmissions

At RAN1#111 meeting, it was agreed to differentiate between multiple PRACH transmissions and single PRACH transmissions. For this purpose, two WAs were made during RAN1#112 meeting.

Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs. - Note: Separate RO means that the RO is separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

If RO resource(s) are shared by UEs configured with multiple PRACH transmissions and single PRACH transmission are configured, the latency of multi-PRACH operation is largely dependent on configured number of ROs per SSB. For example, if only one RO per SSB in one association period is configured, up to 8 PRACH transmissions will span over 8 association periods. On the other hand, if latency issue is addressed when gNB determining PRACH resource configuration parameter, it implies the multi-PRACH feature has impacted legacy UEs. Separate RO resource for multi-PRACH can minimize the impact on Rel-15 UE PRACH resource configuration. Therefore, we prefer to confirm the WA to use separate RO resource for single PRACH transmission and multiple PRACH transmissions. Regarding whether to also confirm both WAs, we think it may lead to more specification effort. Considering the remaining meetings, we prefer to only focus on separate RO resource for single PRACH transmission and multiple PRACH transmissions.

Proposal 3: Confirm the WA to use separate RO resource for single PRACH transmission and multiple PRACH transmissions.

RO group for multiple PRACH transmissions

At RAN1#112 meeting, it was agreed to support configuration of multiple values for number of PRACH transmissions, and also support to differentiate between different number of multiple PRACH transmissions. Meanwhile, “RO group” consisting valid ROs for a specific number of multi-PRACH was agreed.

Agreement For multiple PRACH transmissions with same Tx beam, "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions. Note 1: All ROs in one RO group is associated with the same SSB(s). Note 2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. Note 3: whether/how to define “RO group” in specification will be discussed separately Note 4: Valid RO(s) refers to what is defined in existing specification FFS: whether and how to address collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features, e.g., 2-step RACH. FFS: the time span of RO group. FFS: whether and how ROs can be shared between different RO groups for different number of multiple PRACH transmissions. FFS: other details

Firstly, we want to clarify the understanding of “RO group”, since we think the current definition of “RO group” is not clear. There may be two understandings:

- Opt-A: RO group only defines time span for valid ROs associated with the same SSB/CSI-RS for multi-PRACH, i.e. there may be one or multiple FDMed valid ROs associated with the same SSB/CSI-RS in the same time occasion.

- Opt-B: RO group defines time and frequency locations for valid ROs of multi-PRACH, i.e. only one valid RO in frequency domain for per time occasion.

Fig 1: two possible definitions for “RO group”

For opt-A definition of “RO group”, UE selects one RO group associated with the selected SSB/CSI-RS for a specific number of PRACH transmissions, then UE selects TDMed ROs in the selected RO group. Then UE transmits multiple PRACH transmissions on the selected ROs. For opt-B definition of “RO group”, UE selects one RO group associated with the selected SSB/CSI-RS for a specific number of PRACH transmissions. Then UE transmits multiple PRACH transmissions on the selected ROs. Opt-B definition is more straightforward.

Proposal 4: Clarify one RO group consists of N valid TDMed ROs associated with the same SSB/CSI-RS for a specific multiple number of PRACH transmissions (e.g., N as the number of PRACH transmissions), with single valid RO associated with the same SSB/CSI-RS in per time occasion.

- RO group should define an exact RO in frequency domain

Possible limitations on RO groups for same or different number of PRACH transmissions may need to be considered.

For RO groups for the same number of PRACH transmissions, collision among different RO groups may cause some issue. For example, if different UEs transmits the same number of multi-PRACH on collided RO groups as in Fig 2, when gNB detects 4 PRACH transmissions with preamble index for 2 PRACH transmissions in the 4 ROs, gNB is not able to differentiate the 4 PRACH transmissions, and gNB doesn’t know when to send RAR. Therefore, RO overlapping between multiple RO groups for same number of PRACH transmissions should be avoided.

Fig 2: collision for RO groups for the same number of PRACH transmissions

Proposal 5: One RO should not be included in multiple RO groups associated with the same SSB/CSI-RS for the same number of PRACH transmissions.

In order to support differentiation between different number of multiple PRACH transmissions, separate PRACH resources for different number of PRACH transmissions are needed. Following two alternatives can be considered:

- Alt 1: Separate preamble resources on shared RO resources for different number of multiple PRACH transmissions.

- Alt 2: Separate RO resources for different number of multiple PRACH transmissions.

For Alt 1, RO groups for different number of PRACH transmissions can be overlapped. For Alt 2, ROs in RO groups for different number of PRACH transmissions should be not overlapped.

Alt 2 needs more RO resource than Alt 1, while Alt 2 may be easier to implement joint detection for each number of repetitions. For Alt 1, the number of PRACH transmissions is obtained based on decoded preamble, gNB is not certain about the exact RO group location before decoding the preamble on a RO included in multiple RO groups. If gNB can only perform joint detection for multi-PRACH (i.e. decoding one of multiple PRACH transmissions is not feasible), in a RO included in RO groups for different number of PRACH transmissions, gNB needs to try to detect multi-PRACH in multiple RO groups with different preamble resource. For Alt 2, for each RO, the RO group including the RO is certain. gNB can perform joint detection on ROs within one RO group.

Proposal 6: To differentiate different number of PRACH transmissions, support separate RO resource for different number of PRACH transmissions.

Frequency locations for ROs within a RO group

If there is only one valid RO in frequency domain on one time occasion for one SSB/CSI-RS, frequency location for each RO within one RO group is expected to be the same.

If there are multiple valid FDMed ROs in one time occasion for one SSB/CSI-RS, same or different frequency domain locations for different ROs within one RO group are possible. For this case, PRACH hopping like scheme can be achieved by defining/indicating a frequency index offset among different ROs within one RO group. Three examples are provided.

Example 1: If frequency index #m (frequency index among FDMed ROs associated with the same SSB/CSI-RS in the same time occasion) for the kth RO in RO group, the frequency index for the (k+1)th RO in the RO group would be #(m+offset_index) mod N, where N is the number of FDMed ROs for the SSB/CSI-RS in the one time occasion, and offset_index is the frequency index offset.

Example 2: For odd-indexed RO (e.g. 1st, 3rd, 5th, 7th RO) in one RO group, frequency index is the same as the first RO (e.g. frequency index #m). For even-indexed RO (e.g. 2nd, 4th, 6th, 8th RO) in one RO group, frequency index is (m+ offset_index) mod N, where N is the number of FDMed ROs for the SSB in the one time occasion, and offset_index is the frequency offset.

Example 3: For the first K/2 TDMed ROs in one RO group,  frequency index is the same as the first RO (e.g. frequency index #m). For the last K/2 ROs in RO group,  frequency index is (m+ offset_index) mod N, where N is the number of FDMed ROs for the SSB in the one time occasion, and offset_index is the frequency offset.

Fig 3: different frequency locations for ROs within one RO group

In example 1, K PRACHs are transmitted on min(K, N) frequency hops with each PRACH transmission shifted in frequency domain relative to the previous PRACH transmission. In example 2 and 3, K PRACHs are transmitted on 2 hops. The example 2 is similar to PUSCH/PUCCH frequency hopping scheme without DMRS bundling, while example 3 is similar to PUSCH/PUCCH frequency hopping case with two DMRS bundle groups.

For different RO frequency location case, frequency domain diversity gain may be obtained, which is beneficial for PRACH coverage.

Proposal 7: Support same or different frequency locations for TDMed ROs within one RO group.

- when there are multiple valid FDMed ROs in one time occasion for one SSB/CSI-RS, different frequency domain locations for different ROs within one RO group can be achieved by indicating/defining a frequency domain index offset among different ROs within one group.

Issue 4: PRACH transmission power

At RAN1#112 meeting, it was agreed that power ramping is not applied for PRACH transmissions within one RACH attempt.

Agreement For multiple PRACH transmissions with same Tx beam in one RACH attempt, transmission power ramping is not applied within one RACH attempt.

There may be some cases when updated measurement results for the selected SSB/CSI-RS during the multiple PRACH transmissions in one RACH attempt. For this case, it’s better to still use same measurement results to multiple PRACH transmissions within one RACH attempt, since the multiple transmissions are determined based on one measurement result of the same CSI-RS/SSB. Moreover, the updated measurement result is considered, the number of PRACH transmissions corresponding to the latest measurement results may also be changed. For this case, the situation would be more complicated if number of PRACH transmissions is updated. If same measurement result is applied for the multiple PRACH transmissions, the transmission power of each PRACH transmission is the same.

Proposal 8:  For multiple PRACH transmissions with same Tx beam in one RACH attempt, transmission power is the same during the multiple PRACH transmissions.

Issue 5: RAR reception

In RAN1#112 meeting, it was agreed that single RAR window is supported for one RACH attempt, and the starting position and RA-RNTI still need to be further discussed.

Agreement For multiple PRACH transmissions with same Tx beam, only one RAR window is supported for RAR monitoring for one RACH attempt. - FFS: the start position of the RAR window. - FFS: RA-RNTI.

For RAR window, two alternatives were discussed:

- Alt 1: RAR window starts after the first PRACH transmission.

- Alt 2: RAR window starts after the last PRACH transmission.

For alt 1, longer RAR window may be needed, while legacy RAR window length can be reused for Alt 2. Possible benefit of Alt 1 may be early termination. However, early termination is based on the assumption that gNB tries to decode each PRACH transmission separately. It leads more gNB complexity than gNB only decoding after the last PRACH transmissions. Therefore, the benefit of early termination may be not important. Therefore, alt 2 is preferred.

For RA-RNTI, it is feasible to calculate based on the first or last PRACH transmission.

Proposal 9: For monitoring RAR for multiple PRACH transmissions within one RACH attempt,

- the RAR window starts after the last PRACH transmission.

- RA-RNTI is calculated based on the first or last PRACH transmission.

2.2 Multiple PRACH transmissions with different beams

At RAN1#112 meeting, a conclusion was made that multiple PRACH transmissions are only transmitted on ROs associated with the same SSB/CSI-RS. In our understanding, there are two possibilities for this case:

- Alt 1: UE uses different narrow Tx beams corresponding to the same SSB/CSI-RS.

- Alt 2: UE uses different Tx beams corresponding to different SSBs/CSI-RSs.

Alt 2 may lead to different Rx beams at gNB side in the same RO. Therefore, Alt 2 is not feasible from gNB blind decoding perspective. In our understanding, the Alt 1 case will cause little additional specification effort based on the specified multi-PRACH with same Tx beam. In other words, it can easily be supported in Rel-18.

Proposal 10: Support multiple PRACH transmissions over ROs associated with the same SSB/CSI-RS, where the different Tx beams corresponds to the same SSB/CSI-RS in Rel-18.

- Conclusion

In this contribution, we discussed possible solutions for PRACH coverage enhancements. We have following observation and proposals:

Proposal 1: Support multiple PRACH transmissions for RA during initial access (e.g. CBRA) and RA during CONNECTED MODE (e.g. CFRA and CBRA).

Proposal 2: When multiple values are configured for the number of multiple PRACH transmissions,

- For RA without PDCCH order, UE determines the number of PRACH transmissions based on RSRP of the selected SSB/CSI-RS and configured SSB/CSI-RS RSRP thresholds.

- For PDCCH ordered RA, the PDCCH order DCI indicates the number of PRACH transmissions, which can be explicitly indicated by ‘reserved bits’, or implicitly indicated by the number of indicated mask indexes.

Proposal 3: Confirm the WA to use separate RO resource for single PRACH transmission and multiple PRACH transmissions.

Proposal 4: Clarify one RO group consists of N valid TDMed ROs associated with the same SSB/CSI-RS for a specific multiple number of PRACH transmissions (e.g., N as the number of PRACH transmissions), with single valid RO associated with the same SSB/CSI-RS in per time occasion.

- RO group should define an exact RO in frequency domain

Proposal 5: One RO should not be included in multiple RO groups associated with the same SSB/CSI-RS for the same number of PRACH transmissions.

Proposal 6: To differentiate different number of PRACH transmissions, support separate RO resource for different number of PRACH transmissions.

Proposal 7: Support same or different frequency locations for TDMed ROs within one RO group.

- when there are multiple valid FDMed ROs in one time occasion for one SSB/CSI-RS, different frequency domain locations for different ROs within one RO group can be achieved by indicating/defining a frequency domain index offset among different ROs within one group.

Proposal 8:  For multiple PRACH transmissions with same Tx beam in one RACH attempt, transmission power is the same during the multiple PRACH transmissions.

Proposal 9: For monitoring RAR for multiple PRACH transmissions within one RACH attempt,

- the RAR window starts after the last PRACH transmission.

- RA-RNTI is calculated based on the first or last PRACH transmission.

Proposal 10: Support multiple PRACH transmissions over ROs associated with the same SSB/CSI-RS, where the different Tx beams corresponds to the same SSB/CSI-RS in Rel-18.

References

- RP-221858, Revised WID on Further NR coverage enhancements, June 2022, China Telecom

- 3GPP, RAN1#110bis-e meeting, Chairman’s notes

- 3GPP, RAN1#111 meeting, Chairman’s notes

- 3GPP, RAN1#112 meeting, Chairman’s notes