3GPP TSG RAN WG1 Meeting #112bis-e															R1-2303733

e-Meeting, April 17 – April 26, 2023

Source: 	NTT DOCOMO, INC.

Title:	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM

Agenda item:	9.14.3

Document for:	Discussion and Decision

- Introduction

In RAN#96e, a revised WID [1] for Rel-18 WI “Further NR Coverage Enhancements” was approved with the following objectives related to RAN1 WG:

- Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beams for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable. - Study and if necessary specify following power domain enhancements - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1) - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1) - Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

In this contribution, we discuss enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM.

- Discussion

Regarding the exact method for dynamic waveform switching, RAN1#111 and RAN1#112 made the following agreements and working assumption:

Agreement For DCI based solution, - For supported dynamically scheduled PUSCH, support dynamic waveform switching indication from UL scheduling DCI Note: “Supported dynamically scheduled PUSCH” is to be confirmed in further discussion Note: It does not imply that the waveform switching indication applies to other transmission or not - Indication from non-UL scheduling DCI is not supported. Note: the working assumption made in RAN1#110b-e for “Support at least one of the following options for the dynamic waveform indication in R18” does not need to be confirmed Working Assumption Support new 1-bit field for dynamic waveform indication from UL scheduling DCI - Note: no change of the current size alignment procedure between UL DCI and DL DCI Agreement For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI. Note: no change of the current size alignment procedure between UL DCI and DL DCI.

A key point of the last agreement, on top of the WA above, is that it only touches upon the case of single TB scheduled by single DCI. We believe new 1-bit field for DWS for that case is indeed straightforward, given we now consider non-fallback DCI. Note that we believe the agreement covers the case of PUSCH repetitions, irrespective of repetition types. We also believe it covers cross-carrier scheduling case as well.

Besides, the last agreement does not cover some other cases, e.g., more than 1 TB scheduled by single DCI supported in Rel-16 NR-U and Rel-17 beyond 52.6 GHz. We believe, even for this case, 1-bit field is enough. The reason is that for multi-PUSCH scheduling in Rel-16 NR-U and Rel-17 beyond 52.6 GHz, all the TBs (i.e., PUSCHs) scheduled by a single DCI is allocated in a single CC. Within a single CC, we do not assume that large scale channel condition (e.g., pathloss) changed significantly within a certain time span. Also, it is difficult for scheduler to expect the channel condition change (if any) within the time span of multiple PUSCHs, therefore, multiple bits in a DCI for DWS do not bring large benefit in this case.

Proposal 1: For more than 1 TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI, which is commonly applied to all the TBs.

For the applicable case of additional 1-bit solution in DCI for DWS, RAN1#112 agreed the following:

Conclusion There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18. Agreement Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant.

In Rel-18, DWS is not applicable to Type 1/2 CG PUSCH transmission.

Based on above, the following cases still remain on the table as the potential use cases of DWS with additional 1-bit in DCI:

- Case 1: PUSCH scheduled by RAR UL grant (i.e., msg3 PUSCH)

- Case 2: PUSCH scheduled by DCI format 0_0

- Case 3: PUSCH scheduled by DCI format 0_1/0_2

In our view, at least Case 3 (i.e., PUSCH scheduled by DCI format 0_1/0_2) should be considered for dynamic waveform switching as it is frequently used in the field. Moreover, we think the benefit of dynamic waveform switching is commonly valid irrespective of the PUSCH scheduling method. Therefore, we believe it would be good to maximize the use case in general.

For the other cases, we are open to consider at this stage. Meanwhile, we may need to consider another solution for the cases, since additional 1 bit in DCI related to the cases might be difficult.

.

Proposal 2: Support dynamic waveform switching at least for PUSCH scheduled by DCI format 0_1/0_2

- Open to consider any other type(s) of PUSCH transmission

There are some other issues that need to be resolved even when we focus on Case 3 above. One of the most significant issues would be DCI size determination. In Rel-17, the size of DCI format 0_1/0_2 is dependent on waveform configuration (i.e., transformPrecoder in PUSCH-Config IE). When the applied waveform is dynamically indicated, then it is not clear which waveform (i.e., CP-OFDM or DFT-S-OFDM) is considered for DCI size determination.

Here we have tried to identify the exact DCI field(s) in DCI format 0_1/0_2 whose size depends on waveform. We observe the same fields for both DCI formats, as listed below:

- TPMI

- Antenna ports

- PTRS – DMRS association

- DMRS sequence initialization

- FDRA

Among above, bitwidth of the fields other than FDRA is dependent of the configuration of transform precoder. For FDRA, there is no direct relationship between its bitwidth and transform precoder configuration.

For TPMI field, there are a number of variations for the bitwidth depending on the configuration (including transform precoder). To compare the case with DFT-S-OFDM and with CP-OFDM, the following combinations of DCI field entry tables specified in 38.212 should be considered:

- Comparison#1-1: Table 7.3.1.1.2-2 (CP-OFDM) vs. Table 7.3.1.1.2-3 (DFT-S-OFDM)

  - Conditions: 4 antenna ports, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower,

- Comparison#1-2: Table 7.3.1.1.2-2A/B (CP-OFDM) vs. Table 7.3.1.1.2-3A (DFT-S-OFDM)

  - Conditions: 4 antenna ports, txConfig = codebook, ul-FullPowerTransmission is configured to fullpowerMode1, maxRank=2, 3 or 4,

- Comparison#1-3: Table 7.3.1.1.2-4 (CP-OFDM) vs. Table 7.3.1.1.2-5 (DFT-S-OFDM)

  - Conditions: 2 antenna ports, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower,

- Comparison#1-4: Table 7.3.1.1.2-4A (CP-OFDM) vs. Table 7.3.1.1.2-5A (DFT-S-OFDM)

  - Conditions: 2 antenna ports, txConfig = codebook, ul-FullPowerTransmission is configured to fullpowerMode1

The categorization above is basically separated by Rel-16 capabilities of full-power UL transmission and antenna port implementation. Note that the table for CP-OFDM actually implies CP-OFDM (i.e., transform precoder is disabled) AND maxRank is larger than 1, as the TPMI table and the required bitwidth is equivalent when maxRank=1, irrespective of transform precoder enabled/disabled.

The results of the comparison for TPMI field above are as follows:

Table 2-1: Comparison of TPMI bitwidth

|  |  | Transform precoder |  |
| --- | --- | --- | --- |
|  |  | Disabled | Enabled |
| Comparison#1-1 | Non-coherent | 4 | 2 |
|  | Partial-coherent | 5 | 4 |
|  | Full-coherent | 6 | 5 |
| Comparison#1-2 | Non-coherent | 4 | 3 |
|  | Partial-coherent | 5 (maxRank=2) or 6 (maxRank=3/4) | 4 |
| Comparison#1-3 | Non-coherent | 2 | 1 |
|  | Full-coherent | 4 | 3 |
| Comparison#1-4 | Non-coherent | 2 | 2 |

According to above, bitwidth of TPMI in case of CP-OFDM with rank larger than 1 is always larger than  (or equal to) that of DFT-S-OFDM.

Observation 1: TPMI field bitwidth for CP-OFDM is always larger than (or equal to) that for DFT-S-OFDM

For antenna port field, similarly, the following comparisons should be considered:

- Comparison#2-1: Table 7.3.1.1.2-6/6A (DFT-S-OFDM) vs. Tables 7.3.1.1.2-8/9/10/11 (CP-OFDM)

  - Conditions: dmrs-Type=1, maxLength=1

- Comparison#2-2: Table 7.3.1.1.2-7/7A (DFT-S-OFDM) vs. Tables 7.3.1.1.2-12/13/14/15 (CP-OFDM)

  - Conditions: dmrs-Type=1, maxLength=2

The results of the comparison for Antenna port field above are as follows:

Table 2-2: Comparison of Antenna port field

|  | Transform precoder |  |
| --- | --- | --- |
|  | Disabled | Enabled |
| Comparison#2-1 | 3 | 2 |
| Comparison#2-2: | 4 | 4 |

According to above, bitwidth of Antenna port field in case of CP-OFDM with rank larger than 1 is always larger than (or equal to) that of DFT-S-OFDM, which is the same trend as for TPMI.

Observation 2: Antenna port field bitwidth for CP-OFDM is always larger than (or equal to) that for DFT-S-OFDM

The series of Comparison#2-x above bring us another point than DCI field size: as described above, for Antenna port field size determination, dmrs-Type=2 is actually not considered for DFT-S-OFDM. This is because the use of DMRS Type 2 is not considered much in the real operation due to its higher PAPR. From our perspective, this fact doesn’t need to be changed even in Rel-18, that is, adding the support of DMRS Type 2 for DFT-S-OFDM is not necessary.

Proposal 3: Discuss UE behaviour when DMRS Type 2 is configured for PUSCH, and DFT-S-OFDM is indicated by DCI

- Additional support of DMRS Type 2 for DFT-S-OFDM is not necessary

For PTRS – DMRS association field and DMRS sequence initialization field, they actually exist only when transform precoder is disabled.

Observation 3: For PTRS – DMRS association field and DMRS sequence initialization field, similar to TPMI/Antenna port fields, bitwidth in case of CP-OFDM is always larger than (or equal to) that for DFT-S-OFDM.

Looking at the observations above, for each of the fields whose bitwidth is impacted by the configuration of transform precoder, the bitwidth for CP-OFDM is always larger than or equal to that for DFT-S-OFDM. Due to this fact, when DWS is configured, by assuming CP-OFDM, a UE can always determine the same/larger number of bits for a DCI 0_1/0_2. And when DFT-S-OFDM is indicated by DWS field, the UE can interpret each field properly, by ignoring some bits (or even the whole field). Therefore, we believe, in case that DWS is configured, CP-OFDM should always be assumed for determining the total number of DCI bits (and the number of bits for each field as well).

Proposal 4: When dynamic waveform switching is configured, the size of DCI format 0_1/0_2  should be determined assuming CP-OFDM

There was an extensive discussion regarding the DCI size alignment is per-field or per-format. Moderator’s summary captures four possibilities, which we think covers all the potential directions:

FL proposal 2-4r2: RAN1 to select one of the following Options for DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2: Option 1: Per-format alignment - Step 1: Determine field sizes of DCI addressed to C-RNTI based on the waveform indicated by DWS field. - Step 2: If DCI is addressed to CS-RNTI, align field sizes to that of DCI addressed to C-RNTI based on the waveform indicated by DWS field using the existing C-RNTI/CS-RNTI alignment clause. - Select between: - Alt. A: - By specification, at least one field preceding NDI has size that depends on waveform (e.g. FDRA). - Note: This implies DWS field must be decoded before finding position of NDI field. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Impact on procedure for PDCCH validation of CG type 2: if DWS field value does not match waveform RRC-configured in ConfiguredGrantConfig for the CG, and NDI=0, it is an error case. - Alt. B: - By specification, no field preceding NDI has size that depends on waveform. - Note: It seems that it is currently the case for DCI format 0_1/0_2 in R17 (may need further checking) - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Note: This means DWS field can be positioned after NDI, but before the first field that depends on waveform. - No impact on procedure for PDCCH validation of CG type 2. UE can ignore value of DWS field in the validation. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). Option 2: Per-field alignment - Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Step 2: If DCI is addressed to CS-RNTI, align field sizes to those of C-RNTI according to existing alignment clause between CS-RNTI and C-RNTI. - Note: DWS field does not need to be decoded before finding position of NDI field or any other field. - Note: No constraint on position of DWS field in DCI (but positioning before any field that has waveform-dependent interpretation may help implementation). Option 3: Per-field alignment only for fields preceding NDI field - Step 1: For fields preceding NDI, apply per-field alignment as per Steps 1 and 2 of Option 2. - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Step 2: For fields following NDI, determine field sizes based on DWS as per Steps 1 and 2 of Option 1. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Note: Option 3 has same features as Option1/Alt-B with respect to impact on PDCCH validation and positioning of DWS field. In case all fields preceding NDI are guaranteed to have size independent of waveform by specification, Option 3 may not be relevant. Option 4: Per-field alignment only for DCI addressed to CS-RNTI - For DCI addressed to C-RNTI: - Step 1: Determine size of fields based on DWS indication. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Step 2: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - For DCI addressed to CS-RNTI: - Apply padding to each field, if necessary, to match the size of the corresponding field for the DCI addressed to C-RNTI and (DWS=CP-OFDM). - Note: This means DWS field does not need to be decoded before finding position of NDI field in the case of CS-RNTI. - Note: Field introduced in future specs needs to have size for CP-OFDM at least equal to size for DFT-S-OFDM, or otherwise cannot be used for CS-RNTI and NDI=1.

One important issue is the relationship between DCI for dynamic uplink grant and the one for Type 2 configured grant activation/deactivation. Those two types of DCI shares the same DCI format but its CRC is scrambled by different RNTI. So if those two DCI have different size, UE needs to have more hypothesis on DCI size for a DCI format per the (scheduled) cell, which requires more UE blind decoding effort. Given we do not support DWS for Type 2 CG PUSCH itself, we believe the impact of DWS on Type 2 CG PUSCH and its activation/deactivation should be minimized as much as possible. To achieve this direction, we believe RAN1 should go with per-field alignment, rather than per-format alignment. We would also like to note that per-field alignment is considered in case of cross-carrier scheduling only. In case of single cell scheduling, UE can always identify a single DCI size for a DCI format. If we go with per-format alignment, RAN1 needs to change this principle if DWS is configured. We do not think DWS deserves such a large impact on NR basis function.

Proposal 5: Support per-field DCI size alignment when DWS is configured

Another issue related to DCI field would be the interpretation of FDRA. In Rel-17, there are three resource allocation types in frequency domain: type 0, type 1 and type 2. Type 0 is actually applicable to PUSCH with CP-OFDM, while type 1 is applicable regardless of the applied waveform. Type 2 is intended for unlicensed band operation (i.e., interlaced mapping), which is also applicable regardless of the applied waveform.

There can be some error cases when considering potential combinations of waveform and FDRA type. For example, FDRA type 0 is not appliable to PUSCH with DFT-S-OFDM so far. But now, as we will add an explicit 1-bit DCI field to indicate DWS, a UE may be indicated such a combination.

In our view, this issue can easily be avoided by introducing some restrictions. For example, in case DWS is configured, and at least when DFT-S-OFDM is indicated, FDRA type (configured by resourceAllocation) can be restricted to be resourceAllocationType1 or dynamicSwitch. If the latter one (dynamicSwitch) is supported, then additional restriction to FDRA bit, e.g., MSB to be restricted to always be ‘1’ to indicate FDRA type 1, can be added. Based on these restrictions, what UE assumes for FDRA is basically aligned with the legacy specification, thus no implementation effort.

Proposal 6: When dynamic waveform switching is configured, and at least when DFT-S-OFDM is indicated, for FDRA,

- Specify the restriction on resourceAllocation to be resourceAllocationType1, or dynamicSwitch

- If dynamicSwitch is supported, additional restriction on MSB of FDRA to be ‘1’ should also be considered

In addition, the need of UE reporting was also discussed in RAN1#111, which resulted in the following agreement:

Agreement Study the necessity of the following potential enhancements to assist the scheduler in determining waveform switching: - Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform - Target waveform can be same or different from waveform of an actual PUSCH transmission - FFS target RB allocation and/or target modulation order can be same or different from respective properties of an actual PUSCH transmission - FFS determination of target waveform, target RB allocation, target modulation order - FFS details, e.g. report PCMAX,f,c or Type 1 power headroom for a waveform, or difference thereof between waveforms - PHR triggering enhancements, e.g. - Network-triggered PHR - PH becomes lower (higher) than a threshold - PHR triggered by waveform switching - Reporting of recommended waveform or request to switch waveform - Other solutions not precluded

In our understanding, the motivation of the UE reporting enhancement comes from the fact that the actual benefit of waveform switching may largely be varied case-by-case. For example, the advantage of CP-OFDM, such as flexibility on frequency domain resource allocation, may outweigh the disadvantage on its PAPR performance compared with DFT-S-OFDM. To identify whether dynamic waveform switching is really needed, gNB/NW has to understand the UE’s condition better in terms of e.g., PCMAX difference depending on waveform, remaining power budget, etc. Given that, we support to specify enhancements to assist the scheduler in determining waveform switching, such as PHR reporting.

One discussion point is the definition of reported information. As described above, the target of the enhancement should be to improve the knowledge at gNB/NW side regarding whether DWS is beneficial or not. For example, if PCMAX for CP-OFDM and DFT-OFDM are reported, gNB can easily understand the exact benefit of DWS.

Another discussion point is whether to enhance PHR triggering method. We are open to consider some enhancements, e.g., network-triggered PHR, to obtain PHR in a timely manner based on the actual necessity in the operation. Meanwhile, we would like to point out that even if the triggering method is enhanced, still PHR reporting information should also be enhanced because the metric reported in the current PHR is not clear enough from NW perspective to properly understand the condition of UE. To help understand it better, the difference between CP-OFDM and DFT-S-OFDM should be visible.

Proposal 6: Support UE reporting enhancement to assist the scheduler in determining dynamic waveform switching, such as PHR reporting

- E.g., reporting two PHR values, one for CP-OFDM and the other for DFT-S-OFDM

- Conclusion

Proposal 1: Confirm working assumption on supporting new 1-bit field for dynamic waveform switching from UL scheduling DCI, at least for DCI format 0_1

- Open to consider another solution for PUSCH scheduled by DCI format 0_2

Proposal 2: Support dynamic waveform switching at least for PUSCH scheduled by DCI format 0_1/0_2

- Open to consider any other type(s) of PUSCH transmission

Observation 1: TPMI field bitwidth for CP-OFDM is always larger than (or equal to) that for DFT-S-OFDM

Observation 2: Antenna port field bitwidth for CP-OFDM is always larger than (or equal to) that for DFT-S-OFDM

Proposal 3: Discuss UE behaviour when DMRS Type 2 is configured for PUSCH, and DFT-S-OFDM is indicated by DCI

- Additional support of DMRS Type 2 for DFT-S-OFDM is not necessary

Observation 3: For PTRS – DMRS association field and DMRS sequence initialization field, similar to TPMI/Antenna port fields, bitwidth in case of CP-OFDM is always larger than (or equal to) that for DFT-S-OFDM.

Proposal 4: When dynamic waveform switching is configured, the total size of DCI format 0_1/0_2 (and the bitwidth of each field) should be determined assuming CP-OFDM

Proposal 5: When dynamic waveform switching is configured, and at least when DFT-S-OFDM is indicated, for FDRA,

- Specify the restriction on resourceAllocation to be resourceAllocationType1, or dynamicSwitch

- If dynamicSwitch is supported, additional restriction on MSB of FDRA to be ‘1’ should also be considered

Proposal 6: Support UE reporting enhancement to assist the scheduler in determining dynamic waveform switching, such as PHR reporting

- E.g., reporting two PHR values, one for CP-OFDM and the other for DFT-S-OFDM

Reference

- RP-221858, Revised WID on Further NR coverage enhancements, June 2022, China Telecom

- R1-2106635, Rel-17 TEI proposals, August 2021