3GPP TSG RAN WG1 #112bis-e		                                                       R1-2303735

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.17.10

Source: 	NTT DOCOMO, INC.

Title: 	Draft RAN1 UE features list for Rel-18 Multi-carrier enhancements for NR

Document for:	Discussion and Decision

- Introduction

This contribution includes rapporteur’s initial draft RAN1 UE features list for Rel-18 Multi-carrier enhancements for NR based on agreements made for the WI so far.

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_MC_enh | 49-1 | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell included in a set of cells | - UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is included in a set of cells in same PUCCH group. - Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is one of SCells if set of cells includes only SCells. - Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). - Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} - UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 - HARQ feedback based on Type 1 HARQ codebook - FDRA field based co-scheduled cell indication |  | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell included in a set of cells | [Per BC] | N/A | N/A | N/A | [Agreement At least cases 1-1 and 1-2 on SCS are supported: - Case 1-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and same SCS is used among all the co-scheduled cells including the scheduling cell. - Case 1-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and same SCS is used among all the co-scheduled cells which may be same or different to the SCS of the scheduling cell.] [Agreement Followings are excluded from multi-cell PDSCH/PUSCH scheduling in Rel-18. - SCell schedules multiple cells including P(S)Cell - Different SCS among co-scheduled cells - Different carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2) among co-scheduled cells] [Agreement The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4.] | Optional with capability signaling |
| 49. NR_MC_enh | 49-1a | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | - UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. - Scheduling cell is PCell or SCell, and a set of cells includes only SCells. - Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). - Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} - UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 - HARQ feedback based on Type 1 HARQ codebook - FDRA field based co-scheduled cell indication | 6-10 (CCS with same SCS) | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell which is not included in a set of cells with same SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A | [Agreement At least cases 1-1 and 1-2 on SCS are supported: - Case 1-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and same SCS is used among all the co-scheduled cells including the scheduling cell. - Case 1-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and same SCS is used among all the co-scheduled cells which may be same or different to the SCS of the scheduling cell.] [Agreement Followings are excluded from multi-cell PDSCH/PUSCH scheduling in Rel-18. - SCell schedules multiple cells including P(S)Cell - Different SCS among co-scheduled cells - Different carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2) among co-scheduled cells] [Agreement The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4.] | Optional with capability signaling |
| 49. NR_MC_enh | 49-1b | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | - UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. - Scheduling cell is PCell or SCell, and a set of cells includes only SCells. - Scheduling cell and co-scheduled cells have different SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). - Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} - UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 - HARQ feedback based on Type 1 HARQ codebook - FDRA field based co-scheduled cell indication | 18-5 (DL CCS with different SCS) | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A | [Agreement At least cases 1-1 and 1-2 on SCS are supported: - Case 1-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and same SCS is used among all the co-scheduled cells including the scheduling cell. - Case 1-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and same SCS is used among all the co-scheduled cells which may be same or different to the SCS of the scheduling cell.] [Agreement Followings are excluded from multi-cell PDSCH/PUSCH scheduling in Rel-18. - SCell schedules multiple cells including P(S)Cell - Different SCS among co-scheduled cells - Different carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2) among co-scheduled cells] [Agreement The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4.] | Optional with capability signaling |
| 49. NR_MC_enh | 49-2 | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell included in a set of cells | - UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is included in a set of cells in same PUCCH group. - Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is one of SCells if set of cells includes only SCells. - Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). - Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} - UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 - FDRA field based co-scheduled cell indication |  | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell included in a set of cells | [Per BC] | N/A | N/A | N/A | [Agreement At least cases 1-1 and 1-2 on SCS are supported: - Case 1-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and same SCS is used among all the co-scheduled cells including the scheduling cell. - Case 1-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and same SCS is used among all the co-scheduled cells which may be same or different to the SCS of the scheduling cell.] [Agreement Followings are excluded from multi-cell PDSCH/PUSCH scheduling in Rel-18. - SCell schedules multiple cells including P(S)Cell - Different SCS among co-scheduled cells - Different carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2) among co-scheduled cells] [Agreement The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.] | Optional with capability signaling |
| 49. NR_MC_enh | 49-2a | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | - UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. - Scheduling cell is PCell or SCell, and a set of cells includes only SCells. - Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). - Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} - UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 - FDRA field based co-scheduled cell indication | 6-10 (CCS with same SCS) | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell which is not included in a set of cells with same SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A | [Agreement At least cases 1-1 and 1-2 on SCS are supported: - Case 1-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and same SCS is used among all the co-scheduled cells including the scheduling cell. - Case 1-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and same SCS is used among all the co-scheduled cells which may be same or different to the SCS of the scheduling cell.] [Agreement Followings are excluded from multi-cell PDSCH/PUSCH scheduling in Rel-18. - SCell schedules multiple cells including P(S)Cell - Different SCS among co-scheduled cells - Different carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2) among co-scheduled cells] [Agreement The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.] | Optional with capability signaling |
| 49. NR_MC_enh | 49-2b | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | - UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. - Scheduling cell is PCell or SCell, and a set of cells includes only SCells. - Scheduling cell and co-scheduled cells have different SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). - UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 - FDRA field based co-scheduled cell indication | 18-5b (CCS with different SCS) | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A | [Agreement At least cases 1-1 and 1-2 on SCS are supported: - Case 1-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and same SCS is used among all the co-scheduled cells including the scheduling cell. - Case 1-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and same SCS is used among all the co-scheduled cells which may be same or different to the SCS of the scheduling cell.] [Agreement Followings are excluded from multi-cell PDSCH/PUSCH scheduling in Rel-18. - SCell schedules multiple cells including P(S)Cell - Different SCS among co-scheduled cells - Different carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2) among co-scheduled cells] [Agreement The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.] | Optional with capability signaling |
| 49. NR_MC_enh | 49-3 | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | At least one of {49-1, 49-1a, 49-1b, 49-2, 49-2a, 49-2b} | Yes |  | UE does not support monitoring both legacy DCI format(s) (0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | [Per UE] | No | No | No | [Agreement For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, RAN1 specification supports monitoring the DCI format 0_X/1_X and DCI format 0_0/1_0, 0_1/1_1, and/or 0_2/1_2 (if supported by the UE), if configured from a same scheduling cell. - The DCI format 0_X/1_X and the DCI format 0_0/1_0/0_1/1_1/0_2/1_2 can be monitored simultaneously. ] [Note: this FG is referring to FG11-1a] | Optional with capability signaling |
| 49. NR_MC_enh | 49-4 | Multiple sets of cells | - Max number of sets of cells supported by UE in total is reported with candidate value set of {[2, 3, 4]} - Max number of sets of cells supported by UE for a same scheduling cell is reported with candidate value set of {[1, 2, 3, 4]} | At least one of {49-1, 49-1a, 49-1b, 49-2, 49-2a, 49-2b} | Yes |  |  |  |  |  |  | [Agreement Following is supported in Rel-18 multi-cell scheduling - A UE can be configured one or multiple sets of cells with each set configured for multi-cell scheduling using DCI format 0_X/1_X. - Up to 4 sets of cells can be configured per PUCCH group. - Up to N sets of cells can be configured and respectively scheduled by DCI format 0_X/1_X from a same scheduling cell. - The value of N is reported as UE capability.] |  |
| 49. NR_MC_enh | 49-5 | Type 2 HARQ CB support for DCI format 1_3 | HARQ feedback based on Type 2 HARQ codebook for PDSCHs scheduled by DCI format 1_3 | At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on Type 2 HARQ codebook for PDSCHs scheduled by DCI format 1_3 | [Per UE] | No | No | No | [Agreement HARQ-ACK codebook types (Type-1, Rel-15 Type-2, Rel-16 Type-3, Rel-17 Type-3) are applicable when multi-cell PDSCH scheduling is configured.] [Agreement For Type-2 HARQ-ACK codebook, two sub-codebooks are generated with a first sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single cell and a second sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling more than one cell. - Separate DAI counting for DCI(s) with each scheduling a single cell and DCI(s) with each scheduling more than one cell. - Type-2 HARQ-ACK codebook is generated by concatenating the first sub-codebook and the second sub-codebook.] [Note: this FG is referring to FG5-24] | Optional with capability signaling |
| 49. NR_MC_enh | 49-5a | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | 10-16 (Type 3 HARQ CB), At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on Type 3 HARQ codebook triggered by DCI format 1_3 | [Per band] | N/A | N/A | N/A | [Agreement HARQ-ACK codebook types (Type-1, Rel-15 Type-2, Rel-16 Type-3, Rel-17 Type-3) are applicable when multi-cell PDSCH scheduling is configured.] [Note: this FG is referring to FG10-16] | Optional with capability signaling |
| 49. NR_MC_enh | 49-5b | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | 25-6 (Enhanced Type 3 HARQ CB), At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on enhanced Type 3 HARQ codebook triggered by DCI format 1_3 | [Per band] | N/A | N/A | N/A | [Agreement HARQ-ACK codebook types (Type-1, Rel-15 Type-2, Rel-16 Type-3, Rel-17 Type-3) are applicable when multi-cell PDSCH scheduling is configured.] [Note: this FG is referring to FG25-6] | Optional with capability signaling |
| 49. NR_MC_enh | 49-6 | Co-scheduled cell indication based on co-scheduled cell indicator field in DCI format 1_3/0_3 | Co-scheduled cell indication based on co-scheduled cell indicator field in DCI format 1_3/0_3 - Combinations of co-scheduled cells are configured via RRC | At least one of {49-1, 49-1a, 49-1b, 49-2, 49-2a, 49-2b} | Yes |  | UE does not support co-scheduled cell indication based on co-scheduled cell indicator field in DCI format 1_3/0_3 | [Per UE] | No | No | No | [Agreement For a set of cells which is configured for multi-cell scheduling using DCI format 0_X and DCI format 1_X, support the following: •If table defining combinations of co-scheduled cells for the set of cells is configured, O an indicator in the DCI is included and points to one row of the table. O The table is configured by RRC signaling for the set of cells.] | Optional with capability signaling |
| 49. NR_MC_enh | 49-X | Supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands | Indicate supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands - Candidate value set is {switchedUL, dualUL, both} |  | Yes |  | [UL Tx switching across more than 2 bands cannot be supported for the band pair in the band comibination] | [Per band pair per band combination, details up to RAN2] | [N/A] | [N/A] | [N/A] | [Agreement Ask RAN2 to consider following alternatives for UE capability reporting about the supported UL Tx switching options - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination] [Agreement in RAN2#121 For UE capability of switching options, introduce a per-band-pair UE capability to report supported switching options for Rel-18 UL Tx switching.] | Optional with capability signaling |
| 49. NR_MC_enh | 49-Y | Minimum separation time for two uplink switching on more than 2 bands within any two consecutive reference slots | If two uplink switchings are triggered and UL transmissions involved in the two uplink switchings are on more than 2 bands within any two consecutive reference slots, then the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a maximum of X us and the switching gap required for the second uplink switching, and X us is reported with a candidate value set of {0us, 500us} | 49-X | Yes |  | [two uplink switching cannot be triggered in two consecutive reference slots for UL transmissions on more than 2 bands] | [Per BC] | [N/A] | [N/A] | [N/A] | [Agreement Confirm the working assumption with following updates - (working assumption) If two uplink switching are triggered and UL transmissions involved in the two uplink switching are on more than 2 bands within any two consecutive reference slots, then the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a maximum of X us and the switching gap required for the second uplink switching. - X us is subject to UE capability with a value set of {0us, 500us}] | Optional with capability signaling |

# Appendix A: Agreements made for multi-cell PUSCH/PDSCH scheduling with a single DCI

RAN1#109-e

Agreement

Agree the following terminologies ONLY for convenience of discussion:

- DCI format 0_X is used for scheduling multiple PUSCHs on multiple cells with one PUSCH per cell

- DCI format 1_X is used for scheduling multiple PDSCHs on multiple cells with one PDSCH per cell.

The above does not imply introducing new DCI format(s) at this point.

Agreement

- Different TBs are scheduled on different cells by DCI format 0_X.

- Different TBs are scheduled on different cells by DCI format 1_X.

Agreement

Fallback DCI (i.e., DCI formats 0_0 and 1_0) does not support multi-cell scheduling.

Agreement

The DCI for multi-cell scheduling is monitored only in USS set.

Agreement

- PDSCH cannot be scheduled by DCI format 0_X.

- PUSCH cannot be scheduled by DCI format 1_X.

Agreement

- All the co-scheduled cells by a DCI format 1_X and the scheduling cell are included in the same PUCCH group.

- FFS: All the co-scheduled cells by a DCI format 0_X and the scheduling cell are included in the same [cell or PUCCH group].

Agreement

- DCI format 0-X/1-X on a scheduling cell can be used to schedule PUSCHs/PDSCHs on multiple cells including the scheduling cell.

- DCI format 0-X/1-X on a scheduling cell can be used to schedule PUSCHs/PDSCHs on multiple cells not including the scheduling cell.

Agreement

- For a UE, the maximum number of cells scheduled by a DCI format 0_X can be same or different to the maximum number of cells scheduled by a DCI format 1_X.

Working Assumption

- All HARQ-ACK codebook types (Type-1/2/3) are applicable when multi-carrier PDSCH scheduling is configured.

Agreement

- One value for the maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is selected from {3, 4, 8}.

- For a UE, the maximum number of co-scheduled cells by a DCI format 0_X can be smaller than or equal to the maximum number supported in Rel-18.

Agreement

- One value for the maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is selected from {3, 4, 8}.

- For a UE, the maximum number of co-scheduled cells by a DCI format 1_X can be smaller than or equal to the maximum number supported in Rel-18.

Agreement

- (Working assumption) DCI format 0_X/1_X is a new DCI format for multi-cell scheduling

- DCI format 0_X can be used for single cell PUSCH scheduling.

- DCI format 1_X can be used for single cell PDSCH scheduling.

- FFS: UE monitors one of or both multi-cell scheduling DCI and legacy single cell scheduling DCI for a scheduled cell.

Agreement

- DCI format 0-X/1-X can be transmitted on PCell.

- DCI format 0-X/1-X can be transmitted on a SCell at least when the DCI format 0-X/1-X does not schedule PUSCH/PDSCH on PCell.

- FFS whether a DCI format 0-X/1-X can be transmitted on an SCell if the DCI format 0-X/1-X schedules PUSCH/PDSCH on PCell.

Agreement

Further study DCI size budget including below options for multi-cell scheduling DCI:

- Option 1: Existing DCI size budget is maintained per scheduled cell.

  - Alt 1-1: DCI size budget is maintained via DCI size alignment and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

  - Alt 1-2: DCI size budget is maintained via configured size for multi-cell scheduling DCI and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

  - Alt 1-3: DCI size budget is maintained via DCI size alignment and DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell.

- Option 2: Existing DCI size budget is not necessarily maintained per scheduled cell.

  - Alt 2-1: DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell.

  - Alt 2-2: DCI size budget of multi-cell scheduling DCI is not counted per serving cell and not considered in the related serving cell specific DCI size alignment procedure, e.g., for K co-scheduled cells, gNB guarantee the total budget of 3*K DCI sizes is not exceeded.

  - Alt 2-3: voiding the “3+1” limit for multi-cell scheduling

  - Alt 2-4: the DCI size budget for DCI size alignment can be separately configured for each cell

  - Alt 2-5: DCI size budget of the scheduling cell can be increased to account for the DCI format for multi-cell scheduling. Accordingly, the DCI size budget of a scheduled cell can be reduced.

- Other options/alternatives could be considered.

Agreement

Further study BD/CCE counting for multi-cell scheduling DCI based on below options:

- Alt 1: counted on each co-scheduled cell

- Alt 2: counted only in one scheduled cell

- Alt 3: scaled down to each of co-scheduled cell according to the number of co-scheduled cells

- Alt 4: counted as part of the scheduling cell instead of each scheduled cell

- Alt 5: scaled down to each of scheduled cells excluding scheduling cell

- Alt 6: counted on each co-scheduled cell excluding scheduling cell

- Other alternatives could be considered.

Agreement

For multi-cell scheduling, the co-scheduled cells are indicated by DCI format 0_X/1_X. At least the following options are considered:

- Option 1: An indicator in the DCI points to one row of a table defining combinations of scheduled cells.

  - The table is configured by RRC signaling.

  - FFS: Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 2: An indicator in the DCI is a bitmap corresponding to a set of configured cells that can be scheduled by the DCI 0_X/1_X

  - FFS: Separate sets of configured cells for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 3: using existing field (e.g., CIF, FDRA) to indicate whether one or more cells are scheduled or not

- Other options are not precluded.

- Note: It does not preclude other DCI information fields (e.g., BWP) to be jointly indicated by the indicator of the co-scheduled cells.

Agreement

For design of multi-cell scheduling DCI, companies are encouraged to consider following types of DCI fields:

- Type-1 field: A single field indicating common information to all the co-scheduled cells or separate information to each of co-scheduled cells via joint indication or an information to only one of co-scheduled cells

- Type-2 field: Separate field for each of the co-scheduled cells, or each sub-group comprising one or more co-scheduled cells where a single field is commonly applied to the co-scheduled cells belonging to a same sub-group

- Type-3 field: Common or separate to each of the co-scheduled cells or to each sub-group.

  - FFS: whether it is dependent on explicit configuration or implicit condition (e.g., intra or inter band CA, FR1 or FR2).

- Other types are not precluded.

RAN1#110

Agreement

All the co-scheduled cells by a DCI format 0_X and the scheduling cell are included in the same PUCCH group.

Agreement

Confirm below working assumption reached in RAN1#109e meeting.

- (Working assumption) DCI format 0_X/1_X is a new DCI format for multi-cell scheduling

Working Assumption

For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cell.

- The DCI format 0_X/1_X and the legacy DCI format(s) can be monitored simultaneously.

  - FFS: whether monitoring of the DCI format 0_X/1_X and the legacy DCI format(s) is supported for one, a subset, or all cells within the set of cells.

- FFS: number of different DCI sizes for 0_X/1_X and for legacy DCI formats

- FFS: whether to support a subset or all legacy DCI format(s) to be monitored with DCI 0_X/1_X

Working Assumption

- The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4.

- The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.

FFS: The maximum number of configurable cells for co-scheduling

Agreement

For discussing field design of DCI format 0_X/1_X which schedules more than one cell, reformulate the types of DCI fields as below:

- Type-1 field:

  - Type-1A field: A single field indicating common information to all the co-scheduled cells

  - Type-1B field: A single field indicating separate information to each of co-scheduled cells via joint indication

  - Type-1C field: A single field indicating an information to only one of co-scheduled cells

- Type-2 field: Separate field for each of the co-scheduled cells

- Type-3 field: Common or separate to each of the co-scheduled cells, or separate to each sub-group, dependent on explicit configuration.

  - Note: One sub-group comprises a subset of co-scheduled cells where a single field is commonly applied to the co-scheduled cell(s) belonging to a same sub-group.

- Note: Handling of any parameters applicable to multi-cell scheduling where corresponding fields are not included in DCI format 0_X/1_X (if any) will be separately discussed.

Agreement

- For DCI format 1_X/0_X which can schedule more than one cell,

- Type-1 fields at least include below:

  - Type-1A:

    - Identifier for DCI formats

    - Downlink assignment index

    - TPC for scheduled PUCCH

    - PUCCH resource indicator

    - PDSCH-to-HARQ timing indicator

    - One-shot HARQ-ACK request

- Type-2 fields at least include below:

  - New data indicator per TB

  - Redundancy version per TB

- FFS: Other fields to be included in DCI format 1_X/0_X and which type of the fields belongs to.

- FFS: size for each field

Agreement

- When UE detects a DCI format 1_X scheduling a set of PDSCHs, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot , where  is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format and  is the last UL slot overlapping with the DL slot  for the reference PDSCH reception for slot-based PUCCH or an UL slot overlapping with the end of the reference PDSCH reception in DL slot  for sub-slot based PUCCH.

- FFS details of reference PDSCH

Agreement

- For Type-2 HARQ-ACK codebook, two sub-codebooks are generated with a first sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single cell and a second sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling more than one cell.

- Separate DAI counting for DCI(s) with each scheduling a single cell and DCI(s) with each scheduling more than one cell.

- FFS whether a DCI scheduling more than one cell is associated with the first sub-codebook or the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one.

- Type-2 HARQ-ACK codebook is generated by concatenating the first sub-codebook and the second sub-codebook.

- If at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling,

  - FFS: the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell;

- Otherwise, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to N, where N is the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE.

- HARQ-ACK information bits for co-scheduled PDSCHs by a DCI format 1_X is ordered based on serving cell indices associated with co-scheduled PDSCHs.

- HARQ-ACK bundling across co-scheduled cells is not supported for multi-cell scheduling.

Agreement

UE does not expect to be configured both CBG-based PDSCH/PUSCH transmission and the multi-cell PDSCH/PUSCH scheduling on the same or different cells within a same PUCCH group.

Agreement

- At least cases 1-1 and 1-2 on SCS are supported:

- Case 1-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and same SCS is used among all the co-scheduled cells including the scheduling cell.

- Case 1-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and same SCS is used among all the co-scheduled cells which may be same or different to the SCS of the scheduling cell.

- Case 1-3: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and different SCS is used among the co-scheduled cells including the scheduling cell.

- Case 1-4: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and different SCS is used among the co-scheduled cells.

- FFS: Whether Case 1-3 or 1-4 is additionally supported.

RAN#97-e

Updated proposal 4.1:

- Deprioritize any optimization for unlicensed spectrum operation for designing the multi-cell PUSCH/PDSCH scheduling in Rel-18.

Updated proposal 4.2:

- Enhanced Type-2 HARQ-ACK codebook is not supported for the multi-cell PUSCH/PDSCH scheduling in Rel-18.

- Type-1 HARQ-ACK codebook is supported only for the case where co-scheduled cells by a DCI format 1_X have same SCS/carrier type/duplex mode in Rel-18.

  - Additional restriction(s) can be discussed in RAN1

Updated proposal 4.3:

- Configuring more than one scheduling cell for DCI format 0_X/1_X for each scheduled cell is not supported for the multi-cell PUSCH/PDSCH scheduling in Rel-18.

Updated proposal 4.5:

- Followings are excluded from multi-cell PDSCH/PUSCH scheduling in Rel-18.

  - SCell schedules multiple cells including P(S)Cell

  - Different SCS among co-scheduled cells

  - Different carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2) among co-scheduled cells

  - Configuration of both multi-cell PDSCH/PUSCH scheduling and multi-TRP for a scheduled cell

  - Support for any sidelink scheduling

Conclusion:

- Following is excluded from multi-cell PDSCH/PUSCH scheduling in Rel-18.

  - PCell schedules multiple cells by DCI format 0_X/1_X when a sSCell is configured to schedule PCell

RAN1#110bis-e

Agreement

Confirm the following working assumption reached in RAN1#110 meeting.

Working Assumption

- The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4.

- The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.

- FFS: The maximum number of configurable cells for co-scheduling

Agreement

At least the following fields are excluded from DCI format 1_X/0_X:

- CBGTI

- CBGFI

- PDSCH group index

- New feedback indicator

- Number of requested PDSCH group(s)

- Sidelink assignment index

- Second TPC command for scheduled PUSCH

- Second SRS resource indicator

- Second Precoding information

- Second PTRS-DMRS association

- Second TPC command for scheduled PUCCH

Agreement

For DCI format 1_X/0_X, Type-1 fields at least include the following:

- Priority indicator

- Indicator of co-scheduled cells

- beta offset indicator

- CSI request

- UL-SCH indicator

- FFS: ChannelAccess-CPext

Agreement

Confirm below working assumption reached in RAN1#110 meeting with revision.

Working Assumption

- For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, RAN1 specification supports monitoring the DCI format 0_X/1_X and DCI format 0_0/1_0, 0_1/1_1, and/or 0_2/1_2 (if supported by the UE), if configured from a same scheduling cell.

- The DCI format 0_X/1_X and the DCI format 0_0/1_0/0_1/1_1/0_2/1_2 can be monitored simultaneously.

- Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell.

Agreement

For a set of cells co-scheduled by a DCI format 0_X/1_X, time domain resource allocations for the set of cells are jointly indicated by a single TDRA field in the DCI format 0_X/1_X.

- Separate {SLIV, mapping type, scheduling offset K0 (or K2)} is indicated for each of co-scheduled PDSCHs/PUSCHs.

- FFS details of the TDRA table design

Agreement

Confirm below working assumption:

Working Assumption

HARQ-ACK codebook types (Type-1, Rel-15 Type-2, Rel-16 Type-3, Rel-17 Type-3) are applicable when multi-cell PDSCH scheduling is configured.

Working Assumption

For a set of cells which is configured for multi-cell scheduling,

- Existing DCI size budget is maintained on each cell of the set of cells.

- DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells.

  - FFS which cell DCI size of the DCI format 0_X/1_X is counted on.

- BD/CCE of DCI format 0_X/1_X is counted on one cell among the set of cells.

  - FFS which cell BD/CCE of the DCI format 0_X/1_X is counted on.

- Search space of DCI format 0_X/1_X is configured on one cell of the set of cells and associated with the search space of the scheduling cell with the same search space ID.

  - FFS which cell the SS of the DCI format 0_X/1_X is configured on.

- FFS: How to address Rel-17 BD/CCE limit for any given cell (operating the feature under Rel-17 BD/CCE limit)

- Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell.

Agreement

- UE does not expect to be configured both multi-PDSCH scheduling and multi-cell PDSCH scheduling on the same or different cells within a same PUCCH group.

Agreement

- For Type-2 HARQ-ACK codebook, if at least one cell of a set of cells which can be co-scheduled by DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell of the set of cells is equal to M, where M is the maximum number of TBs which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE.

Agreement

- For Type-2 HARQ-ACK codebook, a DCI format 1_X scheduling more than one cell is associated with the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one.

- If a UE is scheduled by a DCI format 1_X to receive PDSCH over multiple cells, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a cell from the multiple cells, at least one symbol from a set of symbols where the UE is scheduled PDSCH reception in the cell is an uplink symbol, the UE does not receive the PDSCH in the cell.

- If a UE is scheduled by a DCI format 0_X to transmit PUSCH over multiple cells, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a cell from the multiple cells, at least one symbol from a set of symbols where the UE is scheduled PUSCH transmission in the cell is a downlink symbol, the UE does not transmit the PUSCH in the cell.

RAN1#111

Agreement

Confirm the RAN1#110bis-e working assumption with the following changes:

Working Assumption

For a set of cells which is configured for multi-cell scheduling,

- Existing DCI size budget is maintained on each cell of the set of cells.

- DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells.

  - DCI size of the DCI format 0_X/1_X is counted on the reference cell.

- BD/CCE of DCI format 0_X/1_X is counted on one cell among the set of cells.

  - BD/CCE of the DCI format 0_X/1_X is counted on the reference cell.

- Same reference cell is used for both DCI format 0_X and DCI format 1_X.

- The reference cell is

  - the scheduling cell if the scheduling cell is included in the set of cells and search space of the DCI format 0_X/1_X is configured only on the scheduling cell;

  - one cell of the set of cells which search space of DCI format 0_X/1_X is configured on and associated with the search space of the scheduling cell with the same search space ID if search space of the DCI format 0_X/1_X is configured on the cell in addition to the scheduling cell.

    - It is up to gNB on which cell the SS of the DCI format 0_X/1_X is configured on.

- To address Rel-17 BD/CCE limit for any given cell (operating the feature under Rel-17 BD/CCE limit)

  - For the reference cell, a total number of configured BD/CCEs for both DCI formats 0_X/1_X and legacy DCI formats (if configured) does not exceed the Rel-17 limits.

  - For other cells in the sets of cells, Rel-17 limits for PDCCH/DCI monitoring and BD/CCE counting rules for legacy DCI formats (not including DCI formats 0_X/1_X) apply

- Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e.,  and ) for PDCCH candidates for each scheduled cell.

Agreement

For a set of cells which is configured for multi-cell scheduling, up to 4 cells within the set of cells are supported.

- A DCI format 0_X/1_X can schedule PUSCH(s)/PDSCH(s) on a combination of co-scheduled cells among the same set of cells.

Agreement

For DCI format 1_X/0_X,

- Type-1 fields at least include below:

  - ChannelAccess-Cpext

  - TDRA

- Below fields are agreed to be supported for DCI format 0_X/1_X. FFS: Whether the fields are type1, type2, type configurable, or omitted. FFS: details on the fields (e.g. length, which legacy configurations are applicable), other fields.

  - HARQ process number

  - MCS (FFS: potential compression scheme)

  - Bandwidth part indicator

  - Frequency domain resource assignment (FFS: potential compression scheme)

  - VRB-to-PRB mapping

  - PRB bundling size indicator

  - Rate matching indicator

  - ZP CSI-RS trigger

  - Antenna port(s)

  - Transmission configuration indication

  - DMRS sequence initialization

  - Frequency hopping flag

  - TPC command for scheduled PUSCH

  - Precoding information and number of layers

  - PTRS-DMRS association

  - SRS request

  - SRS resource indicator

  - SRS offset indicator

  - PTRS-DMRS association

  - Open-loop power control parameter set indication

  - UL/SUL indicator

Note: RAN1 strives to minimize the number of fields which are type configurable.

Agreement

For monitoring PDCCH candidates for a set of cells which is configured for multi-cell scheduling, the n_CI in the search space equation is determined by a value configured for the set of cells by RRC signaling.

Agreement

The types for below fields in DCI format 1_X are listed (R1-2212924):

| Field | Type | Details (for information only) |
| --- | --- | --- |
| HARQ process number | Type 2 | Details in Section 7.1.1 |
| MCS | Alt 1: Type 2 (without compression) | Details in Section 7.1.2 |
| BWP indicator | Type 1A | Details in Section 7.1.3 |
| FDRA | Type 2 - Further consider larger RBG granularity than existing maximum specified or configured value for RA type 0 - Use large RBG-based RIV for RA type 1 based on R16 configurable granularities for DCI format 1_2 | Details in Section 7.1.4 |
| VRB-to-PRB mapping | Type 1A | Details in Section 7.1.5 |
| PRB bundling size indicator | Type 1A | Details in Section 7.1.6 |
| Rate matching indicator | Type 1B (up to 4 bits) | Details in Section 7.1.7 |
| ZP CSI-RS trigger | Type 1B (up to 3 bits) | Details in Section 7.1.8 |
| Antenna port(s) | Configurable between Type 1A and Type 2 | Details in Section 7.1.9 |
| TCI | Type 1B (up to 4 bits) | Details in Section 7.1.10 |
| DMRS sequence initialization | Type 1A | Details in Section 7.1.11 |
| SRS request | Type 1B (up to 4 bits) | Details in Section 7.1.12 |
| SRS offset indicator | Type 1B (up to 3 bits) | Details in Section 7.1.13 |

This does not imply that payload of DCI can be larger than what is supported for polar code in Rel-17.

FFS: Details

Agreement

- The types for below fields in DCI format 0_X are listed:

| Field | Type | Details (for information only) |
| --- | --- | --- |
| HARQ process number | Type 2 | Details in Section 7.2.1 |
| MCS | Alt 1: Type 2 (without compression) | Details in Section 7.2.2 |
| BWP indicator | Type 1A | Details in Section 7.2.3 |
| FDRA | Type 2 - Further consider larger RBG granularity than existing maximum specified or configured value for RA type 0 - Use large RBG-based RIV for RA type 1 based on R16 configurable granularities for DCI format 1_2 | Details in Section 7.2.4 |
| Frequency hopping flag | Type 1A | Details in Section 7.2.5 |
| TPC command for scheduled PUSCH | Type 2 | Details in Section 7.2.6 |
| Open-loop power control parameter set indication | Type 1A | Details in Section 7.2.7 |
| Antenna port(s) | Configurable between Type 1A and Type-2 | Details in Section 7.2.8 |
| Precoding information and number of layers | Configurable between Type 1A and Type-2 | Details in Section 7.2.9 |
| PTRS-DMRS association | Type 2 | Details in Section 7.2.10 |
| DMRS sequence initialization | Type 1A | Details in Section 7.2.11 |
| SRS request | Type 1B (up to 4 bits) | Details in Section 7.2.12 |
| SRS resource indicator | Configurable between Type 1A and Type-2 | Details in Section 7.2.13 |
| SRS offset indicator | Type 1B (up to 3 bits) | Details in Section 7.2.14 |
| UL/SUL indicator | FFS | Details in Section 7.2.15 |

This does not imply that payload of DCI can be larger than what is supported for polar code in Rel-17.

FFS: Details

RAN1#112

Agreement

For Type-2 HARQ-ACK codebook, for a set of cells which is co-scheduled by a DCI format 1_X, the reference PDSCH to determine DAI counting is the PDSCH with smallest serving cell index among the set of co-scheduled cells.

Agreement

- For a set of cells which is co-scheduled by a DCI format 1_X, the PDSCH with the smallest serving cell index among the set of co-scheduled cells is used to determine last DCI format for PUCCH determination among DCI formats within a same PDCCH MO.

- It is up to gNB implementation to resolve the last DCI format issue when both DCI format 1_X and other DCI format 1_0/1_1/1_2/1_X are received in a same PDCCH monitoring occasion on a same scheduling cell for scheduling PDSCHs on same scheduled cell.

Agreement

For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_X, the reference PDSCH is the PDSCH ending last as indicated in the DCI format 1_X among the set of co-scheduled PDSCHs.

Conclusion

Type-1 HARQ-ACK codebook is supported for multi-cell scheduling without K1 extension.

- UE expects HARQ-ACK information for all co-scheduled PDSCHs by DCI format 1_X can be mapped in the Type-1 HARQ-ACK codebook.

- Type-1 HARQ-ACK codebook is not enhanced for Rel-18 multi-cell scheduling.

Agreement

For a set of cells which is configured for multi-cell scheduling using DCI format 0_X/1_X, a joint TDRA table is configured by RRC signaling for the set of cells with each row in the table containing TDRA indexes for all cells within the set of cells.

- TDRA field in the DCI format 0_X/1_X belongs to Type-1B field.

- TDRA field in the DCI format 0_X/1_X indicates a row from the joint TDRA table.

- TDRA index for a cell points to a corresponding TDRA in the TDRA table applicable for DCI format 0-1/1-1.

Agreement

CSI request in DCI format 0_X belongs to Type-1C field.

- This field is applied to the cell with smallest serving cell index among the co-scheduled cells.

Agreement

UL-SCH indicator in DCI format 0_X belongs to Type-1C field.

- This field is applied to the cell with smallest serving cell index among the co-scheduled cells.

Agreement

Enhanced Type-3 codebook indicator in DCI format 1_X belongs to Type-1A field.

Agreement

HARQ-ACK retransmission indicator in DCI format 1_X belongs to Type-1A field.

Agreement

PUCCH Cell indicator in DCI format 1_X belongs to Type-1A field.

Agreement

For a set of cells configured for multi-cell scheduling using DCI format 0_X/1_X,

- the size of a Type-1A field in the DCI format 0_X/1_X is determined as maximum field size of active BWP among all cells within the set of cells.

- the size of a Type-1B field in the DCI format 0_X/1_X is equal to ceiling(log2(N)), where N is the number of rows in RRC-configured table with each row containing multiple indexes for all cells within the set of cells.

  - The Type-1B field indicates one row of the configured table

  - The Type-1B index for a cell points to a corresponding index in a RRC configured table applicable for DCI format 0_1/1_1 or MAC CE activated values.

- the size of a per cell Type-2 field in the DCI format 0_X/1_X is determined based on active BWP for each cell.

Agreement

For a set of cells which is configured for multi-cell scheduling using DCI format 0_X and DCI format 1_X, support the following:

- If table defining combinations of co-scheduled cells for the set of cells is configured,

  - an indicator in the DCI is included and points to one row of the table.

  - The table is configured by RRC signaling for the set of cells.

    - Separate tables are configured for downlink scheduling and uplink scheduling

  - The size of the indicator is equal to ceil(log2(N)), where N is the number of rows in the table.

  - The max number of rows in the table is 16

  - The size of the per-cell Type 2 fields for each co-scheduled cell does not change according to the indicated co-scheduled cell combination

  - The payload size of DCI format 1_X is derived by UE based on RRC configuration of the active BWP(s) of co-scheduled cell combinations within the set of cells.

    - The payload size of DCI format 1_X is the same for the active BWP(s) of all the co-scheduled cell combinations and equal to the largest payload size among the active BWP(s) of all the co-scheduled cell combinations determined by the co-scheduled cell combination table.

  - The payload size of DCI format 0_X is derived by UE based on RRC configuration of the active BWP(s) of co-scheduled cell combinations within the set of cells.

    - The payload size of DCI format 0_X is the same for the active BWP(s) of all the co-scheduled cell combinations and equal to the largest payload size among the active BWP(s) of all the co-scheduled cell combinations determined by the co-scheduled cell combination table.

- Otherwise,

  - The UE determines the actually scheduled cell(s) based on the FDRA field of each cell of the set of cells.

    - For Type 0 FDRA, all 0s indicates the cell is not scheduled.

    - For Type 1 FDRA, all 1s indicates the cell is not scheduled.

  - The size of the Type 2 fields for each cell does not change according to actually co-scheduled cells.

  - The payload size of DCI format 0_X is derived by UE based on RRC configuration of the active BWP(s) of all cells within the set of cells.

  - The payload size of DCI format 1_X is derived by UE based on RRC configuration of the active BWP(s) of all cells within the set of cells.

Agreement

Following is supported in Rel-18 multi-cell scheduling

- A UE can be configured one or multiple sets of cells with each set configured for multi-cell scheduling using DCI format 0_X/1_X.

- Up to 4 sets of cells can be configured per PUCCH group.

- When multiple sets of cells are configured,

  - a cell in one set of cells can’t be included in another set of cells.

  - n_CI value is independently configured for each set of cells.

  - reference cell for counting DCI size and BD/CCE of DCI format 0_X/1_X is independently determined for each set of cells.

  - search space configuration of DCI format 0_X/1_X is independently configured for each set of cells.

  - DCI size of DCI format 0_X is independently determined for each set of cells.

  - DCI size of DCI format 1_X is independently determined for each set of cells.

- The multiple sets of cells can be scheduled by DCI format 0_X/1_X from different scheduling cells.

- Up to N sets of cells can be configured and respectively scheduled by DCI format 0_X/1_X from a same scheduling cell.

  - The value of N is reported as UE capability.

  - An indicator is included in the DCI to indicate the scheduled set of cells,

    - The size of the indicator is equal to ceil(log2(N)), where N is the number of sets of cells.

  - Unique n_CI value is configured for each set of cells.

Agreement

- A new RBG size configuration “Configuration 3” is added with the following values and only used for DCI format 0_X/1_X for RA type 0.

- RBG size is configured per BWP per cell.

- Independent RA type configuration is applied per BWP per cell for multi-cell scheduling DCI.

Table 5.1.2.2.1-1 / Table 6.1.2.2.1-1: Nominal RBG size P

| Bandwidth Part Size | Configuration 1 | Configuration 2 | Configuration 3 |
| --- | --- | --- | --- |
| 1 – 36 | 2 | 4 | 8 |
| 37 – 72 | 4 | 8 | 16 |
| 73 – 144 | 8 | 16 | 32 |
| 145 – 275 | 16 | 16 | 32 |

Agreement

DCI format 0_X / 1_X with CRC scrambled by C-RNTI and MCS-C-RNTI is supported.

Agreement

For a set of cells which is configured for multi-cell scheduling using DCI format 0_X/1_X, if DCI size budget on the reference cell can’t be maintained after performing Rel-17 DCI size alignment procedures for legacy DCI formats (after step 4C), UE applies zero padding to whichever of DCI formats 0_X or 1_X that has a smaller size to have equal size.

Agreement

- Separate search space sets for DCI format 0_X/1_X and legacy DCI formats are independently configured

- Separate search space sets for DCI format 0_X and 1_X can be independently configured

Agreement

If the UE is configured with two SRS resource sets with ‘codebook’ or ‘non-codebook’, a PUSCH scheduled by DCI format 0_X is always associated with the first SRS resource set with ‘codebook’ or ‘non-codebook’.

Conclusion

PUSCH repetition Type B operation is not supported with DCI format 0_X (i.e. UE cannot be configured with PUSCH repetition Type B applicable for DCI format 0_1)

Agreement

New RRC parameter of RBG granularity for RA type 1 can be configured per BWP per cell for DCI format 0_X/1_X with same value range applicable for DCI 0_2/1_2.

Agreement

Size of RV field can be configured per BWP per cell for DCI format 0_X/1_X.

Agreement

Size of HPN field can be configured per BWP per cell for DCI format 0_X/1_X.

Agreement

- Priority indicator in DCI format 0_X belongs to Type-1A field.

  - The indicated priority is applied to all the co-scheduled PUSCH(s)

- Priority indicator in DCI format 1_X belongs to Type-1A field.

  - The indicated priority indicator is applied to the PUCCH.

- RRC parameters is introduced to configure the presence of priority indicator in DCI format 0_X/1_X

  - This parameter is per set of cells

Agreement

- ChannelAccess-Cpext in DCI format 1_X belongs to Type-1A field.

  - The indicated channel access information is applied to the PUCCH and/or SRS (whichever is first).

- ChannelAccess-Cpext-CAPC in DCI format 0_X belongs to Type-1A field.

  - The indicated code point is applied to all the co-scheduled PUSCHs and/or SRS (whichever is first) by DCI format 0_X.

Agreement

Beta_offset indicator in DCI format 0_X belongs to Type-1A field.

- This field is applied to the scheduled PUSCH(s) where the UCI is multiplexed.

Agreement

Inclusion of SCell dormancy indication in DCI format 0_X/1_X is configurable

Agreement

Inclusion of PDCCH monitoring adaptation indication in DCI format 0_X/1_X is configurable

Agreement

Inclusion of minimum applicable scheduling offset indicator in DCI format 0_X/1_X is configurable

# Appendix B: Agreements made for multi-carrier UL Tx switching scheme

RAN1#109-e

Conclusion

EN-DC cases are out of scope for Rel-18 UL Tx switching

Conclusion

UL only cell cases are out of scope for Rel-18 UL Tx switching

RAN1 Observation

Four contributions (R1-2203136, R1-2204724, R1-2204909, R1-2205131) from three companies show their evaluation results on UL Tx switching across 3 or 4 bands at RAN1#109-e meeting.

- All evaluation results show the performance gain of UL Tx switching across 4 bands compared with UL Tx switching across 2 bands, assuming TDD bands with different TDD UL/DL configurations are included in 4 bands.

  - Evaluation results in R1-2203136 show the performance gain of UL Tx switching across 4 bands compared with UL Tx switching across 3 bands.

  - Evaluation results in R1-2204724 show that the performance gain of UL Tx switching across 4 bands compared with UL Tx switching across 2 bands depends on achievable switching period, and the longer switching period for UL Tx switching across 4 bands compared with UL Tx switching across 2 bands leads to reduction of the performance gain. Other evaluation results did not consider the impact of longer switching period for UL Tx switching across 4 bands compared with UL Tx switching across 2 bands.

  - Evaluation results in 5131 observe that the gain highly depends on the scheduling mechanism.

  - The range of performance gains shown in four contributions varies depending on the simulation assumptions.

Agreement

Companies are encouraged to investigate pros and cons of following possible mechanisms for dynamic Tx carrier switching across the configured bands, and RAN1 strives for the down-selection at RAN1#110

- Alt.1: Dynamic Tx carrier switching can be across all the supported switching cases by the UE and based on the UL scheduling, i.e., via UL grant and/or RRC configuration for UL transmission

- Alt.2: NW indicates 2 bands out of the configured bands (3 or 4 bands) via DCI or MAC-CE, and dynamic Tx carrier switching between indicated bands is same as Rel-17

- Alt.3: One anchor band is selected among configured bands (3 or 4 bands), and dynamic Tx carrier switching can be performed only from the anchor band to a non-anchor band and from a non-anchor band to the anchor band

- Note: Other mechanisms are not precluded

Agreement

Send LS to RAN4 to ask their feedback on the potential increase of switching period and complexity in the case of UL Tx switching across 3 or 4 bands

- In the LS, observations based on the evaluation results and alternative switching mechanisms discussed in RAN1 are captured for the information to RAN4

- In the LS, RAN1 also asks RAN4 feedback on whether following assumption can be considered as baseline UE assumption/behavior even in case of the UL Tx switching across 3 or 4 bands

  - When one of the two Tx chains is triggered to switch from one band to another band, another Tx chain which is in any of bands is also not expected to be used for transmission during the switching period

LS is endorsed in R1-2205502.

Conclusion

If Rel-18 UL Tx switching is supported, following assumption is applied for Rel-18 UL Tx switching across up to 3 or 4 bands

- Only when the two Tx chains are linked to one NR band, the 2-ports UL transmission on the NR band is possible

RAN1 Observation

Following proposals to address the concern on UE/gNB complexity increase or scheduling restriction due to UL Tx switching across larger number of bands compared with Rel-16/17 are identified in contributions submitted at RAN1#109-e, and companies are encouraged to investigate pros and cons of the proposals so that one or some of them may be down-selected after the down-selection of the mechanism for dynamic Tx carrier switching across the configured bands

- UE can report the supports of only some of concurrent UL cases (combinations of 2 bands for concurrent UL transmissions)

- Switching across 0/1/2 ports is supported only for 2 configured bands out of 3 or 4 configured bands and other bands support switching across 0/1 port only

- Only switching across 0/1 port is supported across all configured bands when 3 or 4 bands are configured

- Prioritization rules between uplink carriers are specified

- No restriction on the UEs choice of MIMO capability on any of the bands/CCs involved in the UL Tx switching band combination is introduced

- After one RF state switch, the next RF state switch must occur after 14 symbols or later (FFS: which SCS is assumed for the symbol duration)

- Note: Other solutions are not precluded

- Note: each proposal assumes certain mechanism for dynamic Tx carrier switching across the configured bands, and hence some or all of the proposals may not be necessary depending on the down selection of the mechanism for dynamic Tx carrier switching across the configured bands

Conclusion

It is RAN1’s understanding that RAN4 should lead the discussion on UL Tx switching with multiple TAGs for both 2 bands case and more than 2 bands case

- For further discussion in RAN1 with regards to UL Tx switching with multiple TAGs, it will be discussed only if triggered by RAN4

- If it is decided to support UL Tx switching with multiple TAGs, it is RAN1's working assumption that the number of TAGs should be limited to up to 2

RAN1 Observation

Following possible switching configurations can be considered, and RAN1 may discuss if any of the following switching configurations need to be supported after making some progress on the discussion on the switching mechanism

- For 3 bands case

  - Switching configuration.3-1: all the 3 bands support up to 2Tx

  - Switching configuration.3-2: only 1 band out of 3 bands support up to 2Tx

  - Switching configuration.3-3: only 2 bands out of 3 bands support up to 2Tx

- For 4 bands case

  - Switching configuration.4-1: all the 4 bands support up to 2Tx

  - Switching configuration.4-2: only 1 band out of 4 bands support up to 2Tx

  - Switching configuration.4-3: only 2 bands out of 4 bands support up to 2Tx

  - Switching configuration.4-4: only 3 bands out of 4 bands support up to 2Tx

- Note: The Spec should not restrict which Tx chain is fixed or switched across certain bands.

RAN#96

RAN provides following guidance to RAN1/2/4.

- If Rel-18 UL Tx switching is supported,

  - RAN1/2/4 shall focus on defining necessary mechanisms and requirements for UL Tx switching across 3 or 4 different bands in Q3 2022

    - Inter-band UL-CA Option 1 (i.e., switched UL) and Option 2 (i.e., dual UL) without SUL band

    - Inter-band UL CA Option 1 (i.e., switched UL) for {SUL band + corresponding NUL band} + 1 or 2 other NUL band(s)

      - UL CA framework where UL CA is performed between NULs according to current RAN4 specifications should not be changed

      - Note: switching across any band in this scenario is not precluded

    - Intra-band two contiguous aggregated carriers within one non-SUL band out of 3 or 4 bands

  - Further check additional scenarios in RAN#97e, e.g.,

    - {SUL band + corresponding NUL band} + {SUL band + corresponding NUL band}

    - Simultaneous transmission across 2 bands in {SUL band + corresponding NUL band} + 1 or 2 other NUL band(s) (excluding simultaneous transmission between SUL and corresponding NUL)

  - Mechanisms/requirements should not introduce restrictions on what were already supported in current specifications for UL Tx switching

RAN1#110

Working Assumption

- If Rel-18 UL Tx switching is supported, following switching mechanism is considered as baseline for the Rel-18 UL Tx switching across 3 or 4 bands

  - Alt.1: Dynamic Tx carrier switching can be across all the supported switching cases by the UE and based on the UL scheduling, i.e., via dynamic grant and/or RRC configuration for UL transmission

- RAN1 will support one or more of following complexity reduction options, considering at least the potential additional preparation time, additional interruption time, and RF complexity for certain switching cases/patterns, if Rel-18 UL Tx switching is supported based on Alt.1, and companies are encouraged to investigate options with striving for down-selection at RAN1#110bis-e.

  - Option 1: UE is allowed to support only some of concurrent UL cases (band pairs)

    - FFS: at least one band pair should be supported as in Rel-17

    - FFS: for both 3 and 4 bands cases or only for 4 bands case

    - FFS: potential capability/RRC signaling

  - Option 2: UE is allowed to support 2 ports transmission only on some of bands out of configured bands for UL Tx switching

    - FFS: at least two bands should support up to 2 Tx as in Rel-17

    - FFS: for both 3 and 4 bands cases or only for 4 bands case

    - FFS: for both switched UL and dual UL cases or only for dual UL case

    - FFS: whether/how to reuse or extend existing capability/RRC signaling

  - Option 3: UE is allowed with more preparation procedure time (or interruption time) only for some specific switching cases/patterns

    - FFS: specific switching cases/patterns where more preparation procedure time (or interruption time) is necessary, e.g., switching patterns not existed in Rel-17

    - FFS: how long preparation procedure time and/or interruption time is necessary, and whether RAN4 involvement is necessary

    - FFS: whether/how to report/indicate the specific switching cases/patterns and/or value(s) of preparation procedure time (or interruption time)

    - FFS: what is the definition of preparation procedure time or interruption time, including whether interruption happens during the preparation procedure time and whether it includes switching period

    - FFS: whether/how long minimum interval between two succeeding UL Tx switching is necessary

  - Option 4: UE is allowed to support only some of band pairs for tx switching

    - FFS: at least one band pair should be supported as in Rel-17

    - FFS: for both 3 and 4 bands cases or only for 4 bands case

    - FFS: for switched UL and/or dual UL

    - FFS: potential capability/RRC signaling

  - Other options are not precluded

RAN#97-e

Updated proposal 3.1:

- Clarify that the number of TAGs is limited to up to 2 for both 2 bands switching and more than 2 bands switching cases

- Apply the proposed WID update in [5]

- Capture following conclusion in the meeting report of RAN#97-e.

Conclusion: for the work on UL Tx switching with 2 TAGs, RAN1/2 discussion can be triggered by RAN4 LS. No RAN1 spec impact is expected.

RAN1#110bis-e

Agreement

If Rel-18 UL Tx switching for 3 or 4 bands with dual UL is supported, UE is allowed to support only some of band pairs for concurrent UL transmission based on UE capability

- The supported band pair for concurrent transmission requires the support of UL CA on the corresponding band pair(s) by the UE

- Details on the UE capability such as how to report the support of dual UL and the supported band pair(s) for concurrent UL transmission are further discussed

- Details on the gNB configuration/indication such as how to indicate the band pair(s) UE should expect for concurrent UL transmission are further discussed

- Note: UE is also allowed to support all band pairs for concurrent transmission, and the design of Rel-18 UL Tx switching for 3 or 4 bands with dual UL does not impose any restriction

Agreement

If Rel-18 UL Tx switching for 3 or 4 bands is supported, UE is allowed to support only some of band(s) for up to 2 ports UL transmission based on UE capability

- Further down-select from the following alternatives

  - Alt.1: no restriction for both switched UL and dual UL and for both 3 bands and 4 bands

  - Alt.2: at least one band should support up to 2 ports UL transmission for both switched UL and dual UL and for both 3 bands and 4 bands

  - Alt.3: at least two bands should support up to 2 ports UL transmission for both switched UL and dual UL and for both 3 bands and 4 bands

- Details on the UE capability such as whether existing per-FS UL-MIMO capability can be reused or not are further discussed

- Details on the gNB configuration/indication such as whether/how to additionally indicate 2 ports UL transmission mode for a band/cell are further discussed

- Existing MIMO mechanism for MIMO mode indication should be reused

- Note: UE is also allowed to support all bands for up to 2 ports UL transmission, and the design of Rel-18 UL Tx switching for 3 or 4 bands does not impose any restriction

Agreement

If Rel-18 UL Tx switching for 3 or 4 bands is supported, following is considered as baseline.

- Existing conditions where the switching period is required can be reused for Rel-18 UL Tx switching with 3 or 4 bands when only two bands are involved in a switching

- New conditions where the switching period is required should be introduced for Rel-18 UL Tx switching with 3 or 4 bands when more than two bands are involved in a switching

  - For dual UL, following new conditions are considered

    - When the UE is to transmit a 1-port or 2-port transmission on one uplink carrier on one band (1st band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (2nd and 3rd band)

    - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 2T on a carrier on another band (3rd band)

    - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on one of the bands and another different band (1st or 2nd band, and 3rd band)

    - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (3rd and 4th band)

  - FFS for switched UL and/or for the case with complexity reduction option 1 or 2

  - FFS the same or different switch period for existing conditions and new conditions

Conclusion

No consensus in RAN1 on complexity reduction option 3

Agreement

- Consider following alternatives for UE capability reporting about the supported UL Tx switching options

  - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination

  - Alt.2: report {switchedUL, dualUL, both} for the band combination and report supported band pair for concurrent transmission for the band combination

- Consider following alternatives for gNB configuration regarding dual UL

  - Alt.1: configure {switchedUL, dualUL} in CellGroupConfig

  - Alt.2: configure {switchedUL, dualUL} for each band pair (combination of serving cells?)

  - Alt.3: at least configuration of supported band pair (combination of serving cells) for concurrent transmission

  - Alt.4: No configuration of supported band pair (combination of serving cells) for concurrent transmission, i.e., UE just assumes as it reports

Working Assumption

Specify UL Tx switching schemes across up to 4 bands in Rel-18

Working Assumption

If Rel-18 UL Tx switching for 3 or 4 bands is supported, both Switched UL and Dual UL are supported

Agreement

Confirm the following working assumption made at the RAN1#110 meeting.

Working Assumption

If Rel-18 UL Tx switching is supported, following switching mechanism is considered as baseline for the Rel-18 UL Tx switching across 3 or 4 bands

- Alt.1: Dynamic Tx carrier switching can be across all the supported switching cases by the UE and based on the UL scheduling, i.e., via dynamic grant and/or RRC configuration for UL transmission

Working Assumption

At least for dual UL, reuse existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState to solve the issue on ambiguous switching state at least for following cases

- Case#1 of the issue: two Tx chains are currently associated with band A, and next transmission is 1 port transmission on band B, but there are multiple possible switching cases where 1P on band B is supported

  - if twoT is indicated, both of two Tx chains are switched to band B

  - if oneT is indicated, one Tx chain is switched to band B while another Tx chain remains on band A

- Case#2 of the issue: two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, but there are multiple possible switching cases where 1P on band C is supported

  - if twoT is indicated, both of two Tx chains are switched to band C

  - if oneT is indicated, one Tx chain is switched to band C while how to determine the associated band for another Tx chain is FFS

    - Alt.1: based on gNB’s configuration/indication e.g., new RRC parameter

    - Alt.2: based on predefined rule

    - Other alternative is not precluded

- FFS for other potential cases

Agreement

Ask RAN2 to consider following alternatives for UE capability reporting about the supported UL Tx switching options

  - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination

  - Alt.2: report {switchedUL, dualUL, both} for the band combination and report supported band pair for concurrent transmission for the band combination

    - Note：If there is no report on the supported band pair(s) for concurrent transmission while the UE reports “dualUL” or “both” for the band combination, gNB may assume that the UE supports concurrent transmission on all the band pairs within the band combination

  - Alt.3: report {dualUL} for each band pair in the band combination

    - Note: Within the band combination, the UE shall be capable of being operated in switched UL mode for all band pairs

Agreement

Ask RAN2 to consider following alternatives and specify gNB configuration

  - Alt.1: configure {switchedUL, dualUL} for all serving cells (i.e., for the band combination)

  - Alt.2: configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination)

  - Alt.3: configure {switchedUL, dualUL} for all serving cells (i.e., for the band combination), and configure combination(s) of serving cells (i.e., as supported serving cell pair(s) for each band pair in the band combination) for concurrent transmission

Working assumption

Study the following alternatives for the minimum separation time between two UL Tx switchings for Rel-18 UL Tx switching schemes across up to 3 or 4 bands, and decide in RAN1#111 whether/which of the following alternatives is needed

- Alt.1: define 14 symbols based on a SCS (FFS on SCS) as minimum separation time between two UL Tx switchings

- Alt.2: define that no more than one uplink Tx switching within a reference slot based on a SCS (FFS on SCS)

- Alt.3: define X slots as minimum separation time between two UL Tx switchings where 3 bands are involved in total, and define Y slots as minimum separation time between two UL Tx switchings where 4 bands are involved in total, where X and/or Y is no less than 1 (FFS on X,Y, FFS reference SCS for the slots in case of multiple SCSs across carriers or expressed in unit of micro second)

- Alt.4: report the minimum separation time for different switching cases

- Other alternative is not precluded

- FFS: Applicable cases for the restriction

- Note: Companies are encouraged to provide detailed numbers of minimum separation time

Agreement

Consider following alternatives on the supported switching cases (Tx chain states) for each scenario

- Scenario#1: For switched UL, if UE supports up to 2 ports UL transmission on all the bands in the band combination,

  - Alt.1-1: only switching cases (Tx chain states) with 2T are assumed

    - In case of 3 bands, 3 switching cases ({2T,0T,0T}, {0T,2T,0T}, {0T,0T,2T}) are assumed

    - In case of 4 bands, 4 switching cases ({2T,0T,0T,0T}, {0T,2T,0T,0T}, {0T,0T,2T,0T}, {0T,0T,0T,2T}) are assumed

  - Alt.1-2: switching cases (Tx chain states) with 1T-1T can also be assumed

    - FFS: detailed switching cases to be assumed

- Scenario#2: For switched UL, if UE supports up to 2 ports UL transmission only on some of the bands,

  - Alt.2-1: for the band where 2 ports UL transmission is not supported, switching cases (Tx chain states) with 1T-1T can be assumed

    - FFS: detailed switching cases to be assumed with different number of bands supporting up to 2 ports UL transmission

  - Alt.2-2: only switching cases (Tx chain states) with 2T are assumed

    - Assumed switching cases are same as Scenario#1

  - Alt.2-3: switching cases (Tx chain states) with 1T-1T can also be assumed

    - FFS: detailed switching cases to be assumed

- FFS: Scenario#3: For dual UL, if UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission on all the bands in the band combination,

  - Alt.3-1: corresponding switching case(s) with 1T-1T for the band pair(s) are not assumed

    - FFS: if UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission only on some of the bands

  - Alt.3-2: corresponding switching case(s) with 1T-1T for the band pair(s) are assumed

    - Assumed switching cases are same as the case where UE supports dual UL for all band pairs in the band combination

Agreement

LS on UE capability and gNB configuration for UL Tx switching across 3 or 4 bands in Rel-18 is endorsed. Final LS in R1-2210724.

RAN1#111

Agreement

For switched UL, if UE supports up to 2 ports UL transmission on all the bands in the band combination, only switching cases (Tx chain states) with 2T are assumed

- Conclusion: In case of 3 bands, 3 switching cases ({2T,0T,0T}, {0T,2T,0T}, {0T,0T,2T}) are assumed

- Conclusion: In case of 4 bands, 4 switching cases ({2T,0T,0T,0T}, {0T,2T,0T,0T}, {0T,0T,2T,0T}, {0T,0T,0T,2T}) are assumed

- Based on the assumption, the switching gap is required for every UL transmission with changing transmitting band from preceding transmission in this scenario

Agreement

For switched UL, if UE supports up to 2 ports UL transmission only on some of the bands in the band combination, only switching cases (Tx chain states) with 2T are assumed

- Based on the assumption, the switching gap is required for every UL transmission with changing transmitting band from preceding transmission in this scenario

Agreement

For dual UL, if a UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission on all the bands in the band combination, corresponding switching case(s) with 1T-1T for the band pair(s) where concurrent transmission is not supported are not assumed

Agreement

For dual UL, if UE supports concurrent transmission on all band pairs and supports up to 2 ports UL transmission on all the bands in the band combination, all possible switching cases with 1T-1T and 2T are assumed

- In case of 3 bands, 6 switching cases ({2T,0T,0T}, {0T,2T,0T}, {0T,0T,2T}, {1T, 1T, 0T}, {1T, 0T, 1T}, {0T, 1T, 1T}) are assumed

- In case of 4 bands, 10 switching cases ({2T,0T,0T,0T}, {0T,2T,0T,0T}, {0T,0T,2T,0T}, {0T,0T,0T,2T}, {1T,1T,0T,0T}, {1T,0T,1T,0T}, {1T,0T,0T,1T}, {0T,1T,1T,0T}, {0T,1T,0T,1T}, {0T,0T,1T,1T}) are assumed

Agreement

For dual UL, if UE supports up to 2 ports UL transmission only on some of the bands in the band combination, corresponding switching case(s) with 2T for the band where up to 2 ports transmission is not supported are assumed

- If the UE does not support concurrent transmission on specific band pair(s) in the band combination, corresponding switching case(s) with 1T-1T for the band pair(s) where concurrent transmission is not supported are not assumed

Agreement

Following new conditions are applicable to dual UL only (i.e., not applicable to switched UL)

- When the UE is to transmit a 1-port or 2-port transmission on one uplink carrier on one band (1st band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (2nd and 3rd band)

- When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 2T on a carrier on another band (3rd band)

- When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on one of the bands and another different band (1st or 2nd band, and 3rd band)

- When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (3rd and 4th band)

Agreement

Following working assumption is confirmed with updates.

Working Assumption

At least for dual UL, reuse existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState to solve the issue on ambiguous switching state at least for following cases

- Case#1 of the issue: two Tx chains are currently associated with band A, and next transmission is 1 port transmission on band B, but there are multiple possible switching cases where 1P on band B is supported

  - if twoT is indicated, both of two Tx chains are switched to band B

  - if oneT is indicated, one Tx chain is switched to band B while another Tx chain remains on band A

- Case#2 of the issue: two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, but there are multiple possible switching cases where 1P on band C is supported

  - if twoT is indicated, both of two Tx chains are switched to band C

  - if oneT is indicated, one Tx chain is switched to band C while how to determine the associated band for another Tx chain is FFS

    - Alt.1: based on gNB’s configuration/indication e.g., new RRC parameter

    - Alt.2: based on predefined rule

    - Other alternative is not precluded

- FFS for other potential cases

Agreement

In Case#2 where two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, if oneT is indicated via uplinkTxSwitching-DualUL-TxState, one Tx chain is switched to band C and associated band for another Tx chain is determined by new RRC parameter which is down-selected from following alternatives.

- An associated band is configured for each band so that another Tx chain is associated with the configured band (as associated band for the transmitting band)

  - E.g., associated band for each transmitting band is configured as {B for A}, {A for B}, {A for C} and {C for D}.

    - When 1 port transmission on band C is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band B is switched to band C while another Tx chain associated with band A remains unchanged (because band A is associated band for band C)

    - When 1 port transmission on band D is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band A (or B) is switched to band D while another Tx chain associated with band B (or A) is switched to band C (because band C is associated band for band D)

If there is one band where concurrent transmission with any other band is not supported, NW does not configure an associated band for the band. In such case, even if oneT is configured, UE performs switching as twoT is configured when 1 port transmission on the band is scheduled

Agreement

There is no restriction on number of bands supporting up to 2 ports UL transmission for both switched UL and dual UL and for both 3 bands and 4 bands.

- It is up to UE capability to support 2 ports UL transmission on none/some/all of the 3 or 4 bands

- Note: UE with only 1 Tx chain is not expected to perform UL Tx switching (no spec impact)

Agreement

Confirm the following working assumption.

Working Assumption

Specify UL Tx switching schemes across up to 4 bands in Rel-18

Agreement

Following restrictions are applied for Rel-18 UL Tx switching across 3 or 4 bands.

- The UE does not expect to perform more than one uplink switching within a reference slot based on µUL = max(µUL, 1, µUL, 2, µUL, 3) in case of 3 bands, µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4) in case of 4 bands, where µUL, 1, µUL, 2, µUL, 3, µUL, 4 are SCSs of active UL bandwidth parts of the bands in the band combination

  - If there are two consecutive intra-band carriers in one band, µUL, 1 = max(µUL, 1-1, µUL, 1-2), where µUL, 1-1 and µUL, 1-2 are SCSs of active UL bandwidth parts of the carriers in the band

- (working assumption) If two uplink switching are triggered and result in UL transmissions on more than 2 bands within any two consecutive reference slots, then the time duration between the end of all transmission(s) prior to the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time

  - The minimum separation time is a sum of X us and the switching gap required for the second uplink switching.

  - X us is subject to UE capability with a value set of {0us, 500us}

RAN1#112

Agreement

Confirm the working assumption with following updates

(working assumption) If two uplink switching are triggered and UL transmissions involved in the two uplink switching are on more than 2 bands within any two consecutive reference slots, then the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time

- The minimum separation time is a maximum of X us and the switching gap required for the second uplink switching.

- X us is subject to UE capability with a value set of {0us, 500us}

Agreement

Alt.5: gNB configures priorities to each carrier/band.

- The gNB configures priority for each band. The UE determines the switching period location on either switching-from band(s) or switching-to band(s) that is involved in the UL Tx switching and is not with the highest priority band.

RAN2#119-e

- As a baseline, RAN2 reuse Rel-16/17 UL Tx switching band combination list (i.e. BandCombinationList-UplinkTxSwitch-r16) for Rel-18 UL Tx switching capability reporting.

- As a baseline, uplink bands for Rel-18 UL Tx switching are configured as in legacy way, i.e. by UplinkConfig.

- RAN2 waits for RAN1/4 input and then addresses the potential issues according to RAN1/4 indication, e.g.:

– whether the switching period is configured per band pair or per band combination on UE capability reporting.

– whether the switching option (i.e. switchedUL or dualUL) is configured per band pair or per band combination on UE capability reporting.

– how RRC configures a period location for each band pair within three or four bands on RRC configuration.

– how to configure a state of Tx chains after the UL Tx switching is not unique in Rel-18 framework on RRC configuration.

RAN2#119bis-e

Wait for RAN1/4 (No agreement)

RAN2#120

Multi-carrier UL Tx switching scheme

- R2 assumes For UE capability to report applicability of DL interruption for Rel-18 UL Tx switching, RAN2 reuses uplinkTxSwitching-DL-Interruption-r16 (no spec impact).

- R2 assumes to reuse the per band per BC capability, uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17, on UL-MIMO coherence for the 2Tx-capable UL band(s) for Rel-18 UL Tx switching (fallback description FFS).

RAN2#121

Multi-carrier UL Tx switching scheme

- For UE capability of switching options, introduce a per-band-pair UE capability to report supported switching options for Rel-18 UL Tx switching.

- configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination)

- For RRC configuration to clarify ambiguous Tx state, RAN2 should introduce an RRC configuration that associates a band to another band which the unused Tx chain is switched to when the switch is from concurrent transmission on two bands to 1 Tx transmission on another band.

- For UE capability of 2-port UL transmission, RAN2 reuse the per-FS UL-MIMO UE capability (no spec change).

- No technical agreements. Status as follows:

-	For the RAN4 UE capability of unaffected/maintained UL transmission on the unchanged Tx chain when the other Tx chain is switching, and RAN1 UE capability of minimum separation time, RAN2 wait for further RAN4/RAN1 agreements on the attributions, e.g. granularity.

-	For switching period, RAN2 wait for the RAN4 decision on whether different values can be reported in Rel-18 and whether/how to handle the 1Tx-1Tx switching.

-	Feature Set discussion (P5/P6 etc) is postponed, allow companies to digest.

-	The following is postponed: For Rel-18 UL Tx switching among 3/4 bands, existing signalling uplinkTxSwitching-DualUL-TxState-r17 is reused to indicate the state of Tx chains for dualUL mode.

-	For Rel-18 UL Tx switching among 3/4 bands, whether new signalling indicating the mode of 1Tx-2Tx switching, 2Tx-2Tx switching[, and 1Tx-1Tx switching] for each band pair is pending to RAN4/RAN1’s further discussion.

RAN4#104-e

Agreement from R4-2214463

Issue 2-1-1: Set of values for Tx switching period

RAN4 agreements have been captured in the reply LS to RAN1.

Issue 2-1-2: Granularity of Tx switching period

RAN4 agreements have been captured in the reply LS to RAN1.

Issue 2-1-3: Exact value of Tx switching period

Further discuss the two options in the next meeting:

Option 1: Reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18 Tx switching.

Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 Tx switching with 3/4 bands configured.

Sub-topic 2-2: Impact from switching of one Tx chain on the other Tx chain

Issue 2-2-1: Impact on the band with the number of Tx chain changed due to switching

RAN4 agreements have been captured in the reply LS to RAN1.

Issue 2-2-2: Impact on the band with the number of Tx chain unchanged due to switching

GTW Agreement:

As baseline UE assumption, neither of Tx chains is expected to be used for transmission on band C during the switching period.

RAN4 will further discuss optional advanced features to allow the other Tx chain can be expected to be used for transmission on band C during the switching period as advanced/optional UE assumption.

Sub-topic 2-3: Possible mechanisms for dynamic Tx carrier switching across the configured bands

Issue 2-3-2: Switching cases for CA option 2

This issue is covered in the switching mechanism discussion in RAN1.

Sub-topic 2-4: PUSCH preparation time

Issue 2-4-1: PUSCH preparation procedure time / scheduling delay

To be discussed in RAN1.

Sub-topic 2-5: Other issues

Issue 2-5-1: Concurrent UL transmission on 2 bands

RAN4 agreements have been captured in the reply LS to RAN1.

Issue 2-5-2: Number of bands supporting 2Tx / UL-MIMO

Issue 2-5-2A: On 2Tx

Covered in RAN1 discussion

Issue 2-5-2B: On UL-MIMO

Covered in RAN1 discussion

Issue 2-5-3: Support of intra-band UL CA

The following observation is aligned with the current WI scope approved in RAN plenary. Further update on the WI scope is not precluded and is up to RAN plenary decision.

Observation: Scope of the WI is limited such way that only one band among 3 or 4 bands that are part of the configured TX switching scheme can have intra-band UL CA configured.

Issue 2-5-4: RF requirements

No need to define RF requirements for UL CA with UL simultaneous transmission on 3 and 4 bands in the WI.

For the next meeting, encourage analysis/identification of the RAN4 RF requirements needed for this WI.

Issue 2-5-5: UE complexity aspect

GTW Agreement:

The complexity related aspects would be discussed in RAN4 in future meetings.

Agreement from R4-2214464

On the length of switching period:

- For UL switching period with Tx switching across 3 or 4 bands, RAN4 agreed to reuse the same set of values as in Rel-16/17, i.e., {35 us, 140 us, 210 us} for UL CA and SUL.

- The length of switching period is applied per band pair for each band combination.

- For each band pair, the switching period can be the same or different for 1Tx-2Tx switching and 2Tx-2Tx switching based on UE reporting, which is similar as in Rel-17.

  - Note: For UE reporting different periods for 1Tx-2Tx switching and 2Tx-2Tx switching for a band pair, similar to Rel-17, it is RAN4 understanding that the 2Tx-2Tx switching period is applied when 2Tx-2Tx switching mode is configured.

- For the same band pair, RAN4 has not concluded on whether the same or a different value can be reported for the specific band pair supporting Tx switching across 3 or 4 bands in Rel-18 compared to Tx switching across 2 bands specified in Rel-16/17.

On the UE complexity:

RAN4 has not identified any technical difficulty for UE to prevent realizing Tx switching across 3 or 4 bands.

RAN4 would like to recommend the UE memory sharing issue to be further discussed in RAN1 if necessary.

RAN4 has discussed the UE assumption/behavior considering two cases:

- Case 1: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on either band A or band B.

For Case 1, RAN4 agreed that neither of Tx chains is expected to be used for transmission during the switching period.

- Case 2: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on a different band (named “band C”).

For Case 2, RAN4 agreed that, as baseline UE assumption, neither of Tx chains is expected to be used for transmission on band C during the switching period.

RAN4 also discussed other related issues for UL Tx switching across 3 or 4 bands in single TAG scenario, and reached the following agreements:

For concurrent UL transmission on 2 bands:

For UL Tx switching across 3 and 4 bands, the support of concurrent UL transmission on 2 (out of 3 or 4) bands at least requires UL CA support on the corresponding band pair(s) by the UE.

Agreement from R4-2214465

Sub-topic 3.1 Tx switching with multiple TAGs

Issue 3-1-1 UL switching time

WF (as per agreement at the Aug 22 GTW)

Agreement:

RAN4 further discuss if the UL switching time is the same for single TAG and 2 TAGs

UL switching time should not include timing difference up to MTTD between two TAGs.

FFS:

UE may omit the uplink transmissions corresponding to any TAG during the UE switching time.

Issue 3-1-2: UL outage time

WF

Factors for UL outage time discussed in RF session, to be further checked in RRM session

UL switching time (UE capability)

The difference between the TA on the two TAGs, up to MTTD

Timing and measurement error

Issue 3-1-3: Sub-topic PUSCH preparation time

WF

The PUSCH preparation time includes the effect of TA and difference between multiple active component carriers in addition to the switching period (Rel-16)

RAN1 to confirm in case further clarification is needed

Issue 3-1-4: DL interruption time for Tx switching with multiple TAGs

WF

to be discussed in the RRM session

Issue 3-1-5 UE capability and release independence

WF

To be discussed after we know requirements and spec impact

Sub-topic 3-1-6  RAN4 CR text

WF

Further discuss at the next meeting

Issue 3-1-7 Need of RAN1 spec impact

WF

Inform RAN1 that the target scenario considered by RAN4 for the Rel-18 specification is multiple-TAG (dual-TAG) for switching between 2 bands..

Sub-topic 3-2 Additional issue for Tx switching accorss 3/4 bands with multiple TAGs

Issue 3-2-1 Target scenarios

Option 1: Limit number of TAGs to up to 2 for all the cases in the Rel-18  WI (QC, DCM, CTC, Samsung, OPPO, Xiaomi, Sony, OPPO, E///, Nokia)

Option 2: Limit number of TAGs for 3 and 4 band cases to 1 (QC, Nokia)

Option 3: In the scenario of multi-TAG, the switching time masks do not include timing advance difference but the timing advance difference should be considered with the switching time. (HW)

Option 4: focus on 2-TAG case at first, and then extend to the same number of bands if time allowed. (ZTE)

WF (as per agreement at the Aug 22 GTW)

Agreement:

Agree option 1.

RAN4#104bis-e

<R4-2217740>

Sub-topic 1-1: Switching period for Tx switching across 3/4 bands

Issue 1-1-1: Exact value of Tx switching period

Way forward:

For the exact value of Tx switching period for each band pair, further discuss the following options:

- Option 1: Reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18.

  - Note: with the understanding that the switching period in Rel-18 could be different for different band pairs, according to the granularity of per band pair per BC agreed in the last meeting.

- Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 band combination with 3/4 bands.

- Option 3: Option 1 for switchedUL, and option 2 for dualUL

Sub-topic 1-2: Impact from switching of one Tx chain on the other Tx chain

Issue 1-2: Impact on the band with the number of Tx chain unchanged due to switching (Case 2)

Way forward:

- For the impact on the band with the number of Tx chain unchanged due to switching, in addition to the baseline UE assumption agreed in RAN4 #104e, further discuss the two options in the next meeting:

  - Option 1: Introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged (i.e., one Tx chain is maintained on the band) during UL switching.

    - Potential proposals on the granularity of the optional UE capability can also be discussed.

  - Option 2: Do not define other optional features to allow the other Tx chain to be used for transmission on the band with the number of Tx chain unchanged during the switching period.

Issue 1-2A: Issue of two Tx chains switched between two different band pairs

Way forward:

- When two Tx chains are switched between two different band pairs with different lengths of switching periods, as baseline UE assumption, neither of the two Tx chains is expected to be used for transmission during the larger one of the two switching periods.

- Whether advanced optional UE ability will be considered is related to the discussion in Issue 1-2.

Sub-topic 1-3: RAN4 RF requirements

GTW Agreement

  - Following Rel-16/17 approach, common switching time mask requirements for “switched UL” case and “dual UL” case are to be defined.

  - Further discuss whether to define additional requirements for CA option 2, by considering the switching cases to be introduced in RAN1.

Sub-topic 1-4: Location of switching period

GTW Agreement

  - For single-TAG case, RAN4 agrees to reuse the Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) and discuss further details for Rel-18 Tx switching scenario in RAN1.

Sub-topic 1-5: Applicability of DL interruption

Way forward:

- For combinations of SUL+TDD and TDD+TDD CA with the same UL-DL pattern, DL interruption is not required.

- For the other duplex mode combinations, define different capabilities for UEs with and without DL interruption.

  - UE capability is defined as per band per band combination for each band pair supporting UL Tx switching.

Further discuss in the next meeting:

- For SUL+TDD+FDD combinations, for Tx switching between SUL and TDD bands, whether or not DL interruption on the SUL and TDD bands is required.

<R4-2217741>

LS from RAN4 to RAN1/2 (R4-2218841) was approved

<R4-2217742>

Sub-topic 2-1: UL switching time and outage time

Issue 2-1-1: UL switching time

WF UL switching time (Oct 12 GTW agreement):

the UL switching time is the same for single TAG and dual-TAG cases

Issue 2-1-1A: on the wording in the FFS bullet

WF on the wording in the FFS bullet:

continued discussion at the next meeting.

Issue 2-1-2: UL outage time and RAN4 CR text

Issue 2-1-2A: RAN4 CR text

WF RAN4 CR text:

continued discussion at the next meeting. Two options:

Option 1: modify the time mask for TX switching to include the case of dual TAG with different timing advance on the two TAGs.

Option 2: apply same approach as in R17 V2X that the time mask only contains the UE hardware requirement (switching period), and no TA difference included. The impact of Tx switching with multiple TAGs can be considered as scheduling restriction.

Issue 2-1-2B: UL outage time

WF UL outage time:

continued discussion at the next meeting.

Sub-topic 2-2: PUSCH preparation time

WF PUSCH preparation time (Oct 12 GTW agreement):

the timing difference effect and switching period have already been considered in the PUSCH preparation time in the current specification, and further specification update is not needed.

Sub-topic 2-3: Sceanrio

WF Scenario (Oct 12 GTW agreement):

The band combinations containing intra-band non-contiguous CA is not considered in the WI.

NR-DC with multi-TAG operation is not considered in the WI.

Sub-topic 2-4: Location of switching period

WF UL location of the switching period:

continued discussion at the next meeting. The location of the switching period must be specified.

<R4-2217258>

Topic #1:	DL interruption for Tx switching across 3/4 bands

<Agreement>: Issue 1-1: RRM impact due to Rel-18 Tx switching across 3 or 4 bands

- DL interruption requirements are to be specified

- No impact on the existing MTTD requirement due to the extension of UL TX switching for 2 bands from 1 TAG to 2 TAGs.

Issue 1-2: DL interruption length

<Agreement>:

For Rel-18 Tx switching across 3/4 bands with single TAG, reuse Rel-16/Rel-17 values for length of DL interruption.

< Way forward >:

For Rel-18 Tx switching across 3/4 bands with 2 TAGs,

- Option 1: reuse Rel-16/Rel-17 values for length of DL interruption.

- Option 2: interruption on other serving cells when UL Tx switching occurs across multiple bands shall be defined as:

Tinterrupt = ceil((switching period+2*TA adjustment uncertainty+2*MRTD-CP length)/symbol duration)+1

Where

- Switching period is being discussed in RF session and the outcome can be reused in interruption design.

- TA adjustment uncertainty remains same as legacy.

- MRTD=3us which is used to derive R16/R17 DL interruption length is not valid for non-collocated case, further discuss the concrete value of MTRD

<Agreement>: Issue 1-3: Starting point of the DL interruption

For R18 Tx switching across 3/4 bands with both single TAG and 2 TAGs, reuse the Rel-16/17 agreement on the starting symbol of DL interruption

<Agreement>: Issue 1-4: Interrupted cells (i.e., victim cells) where the DL interruption is allowed at UL Tx switching

- Option 1(Nokia, Apple, vivo): Wait for RAN1 /RF conclusion on the interrupted carriers at UL Tx switching across 3 or 4 bands.

- Option 2 (China Telecom, Huawei, Ericsson, Samsung): The capability uplinkTxSwitching-DL-Interruption-r16 is unaffected in the multiple TAG case.

- Option 2a (Samsung, Huawei): For Rel-18 TX switching across 3/4 bands, no reason to change the existing requirement structure and the signaling design for uplinkTxSwitching-DL-Interruption.

RRM can directly use the conclusion of RF session or other RAN group, when decisions are made. So no discussion in 2nd round discussion.

<Agreement>: Issue 1-5: DL interruption is applicable for ENDC

- Option 1: Same principle applies for ENDC, i.e., DL interruptions in TS 38.133 are not affected, even if the time masks in UE RF requirements might have to be different.

Conclusion: It is confirmed that RAN1 has the below agreement [RP-221436]. Following RAN1 agreement, the issue is closed.

RAN1#109-e Conclusion EN-DC cases are out of scope for Rel-18 UL Tx switching

< Way forward > Issue 1-6: Applicable scenario of DL interruption requirements

Candidate options:

- Option 1: reuse Rel-16/Rel-17 values for length of DL interruption. Define DL interruption requirements for Rel-18 Tx switching across 3 or 4 bands applicable to all scenarios including:

  - Inter-band UL-CA without SUL band

  - Inter-band UL CA for {SUL band + corresponding NUL band} + 1 or 2 other NUL band(s)

  - Intra-band two contiguous aggregated carriers within one non-SUL band out of 3 or 4 bands

  - {SUL band + corresponding NUL band} + {SUL band + corresponding NUL band}:

Topic #2: UL outage time for TX switching with 2 TAGs

Issue 2-1: Concept of UL outage time

Candidate options:

- Revised Option 1: The UL outage refers to the actual impacted UL OFDM symbols on serving carriers involved in the switching.

- New Option 2: The UL outage refers to the actual impacted UL OFDM symbols on other victim serving carriers NOT involved in the switching.

- Option 3: The UL outage refers to the possible range of interrupted UL OSs (like a potential interruption window)

Issue 2-2: Factors for UL outage time

To clarify, this issue is trigged by RF WF R4-2215163 (see below). RRM is just responsible to provide possible response to RF. Whether the UL outage time is captured in spec is still under discussion in RF session

## In RF WF [R4-2215163] ## Issue 3-1-2: UL outage time Factors for UL outage time discussed in RF session, to be further checked in RRM session - UL switching time (UE capability) - The difference between the TA on the two TAGs, up to MTTD - Timing and measurement error

Candidate options:

  - Option 1(China Telecom): UL outage time for Tx switching with 2 TAGs includes:

    - UL switching time (UE capability)

    - Half of the difference between the actual TAs on the two TAGs (i.e., not directly use the MTTD defined in TS 38.133)

    - Timing and measurement error, including:

a) BS synchronization accuracy: 3us defined in clause 7.4.2 of TS 38.133 can be used;

b) UE transmit timing error: the requirement defined in clause 7.1 of TS 38.133 can be seen an upper bound, and the sum of maximum UE transmit timing error is 1.56 us for the carriers with 2 TAGs

c) TA quantization error: as defined in TS 38.213, it can be up to 0.52 us or 0.26us for 15 kHz SCS.

  - Option 2 (Huawei, Ericsson):

    - UL interrupted OFDM symbols shall consider the impact of MTTD, however directly using MTTD value to derive impacted UL OFDM symbols are not always correct

    - The third factor “Timing and measurement error” is already included in MTTD

Proposed: Impacted UL OFDM symbols= Ceiling (UL switching period/symbol length) +1 OS

  - Option 3 (Samsung): Confirm the three factors should be considered in RF session for UL outage time:

    - (1) UL switching time (UE capability),

    - (2) the difference between the TA on the two TAGs, up to MTTD,

    - (3) timing and measurement error: typical value is 27ns

Note: other options are not precluded.

RAN4#105

<R4-2220546>

Sub-topic 1-1: Exact value of switching period for Tx switching across 3/4 bands

Issue 1-1: Exact value of Tx switching period

Way forward:

For the exact value of Tx switching period for each band pair, select one of the two options in RAN4 #106:

- Option 1: For Rel-18 UE, for a band pair within a band combination supporting Tx switching among 3/4 bands, the switching period reported by UE for Rel-18 3/4-band Tx switching is same with the switching period for Rel-16/17 2-band switching operations.

  - Note: With the understanding that the switching period in Rel-18 could be different for different band pairs, according to the granularity of per band pair per BC agreed in RAN4 #104e.

- Option 2: For Rel-18 UE, for a band pair within a band combination supporting Tx switching among 3/4 bands, the switching period reported by UE for Rel-18 3/4-band Tx switching can be the same or different from the switching period for Rel-16/17 2-band switching operations.

  - Note: the set of candidate values is still the same, i.e., {35 us, 140 us, 210 us}, according to the agreement in RAN4 #104e.

Sub-topic 1-2: Impact from switching of one Tx chain on the other Tx chain

Issue 1-2-1: Impact on the band with the number of Tx chain unchanged due to switching

Way forward:

- For the band with the number of Tx chain unchanged due to switching, in addition to the baseline UE assumption agreed in RAN4 #104e, introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged (i.e., one Tx chain is maintained on the band) during UL switching.

  - Down-select and decide the granularity of the optional UE capability in RAN4 #106:

    - Option 1a: per band pair per BC

    - Option 1b: per band per band pair per BC

    - Other options are not precluded

  - Note: this optional advanced UE ability is not considered for the following case, as per the RAN4 #104e agreement

    - The switching is between band A and B, one of Tx chain is switched between band A and B, the other Tx chain keep unchanged with band B.

Issue 1-2-2: Issue of two Tx chains switched between two different band pairs

Way forward:

- When two Tx chains are switched between two different band pairs with different lengths of switching periods (denoted as Tswitch_1 and Tswitch_2 for the switching periods of Tx chain #1 and Tx chain #2 respectively, and Tswitch_1 < Tswitch_2), select one of the two options in RAN4 #106:

  - Option 1: In addition to the baseline UE assumption, introduce advanced optional UE ability to allow the Tx chain #1 to be used for transmission during the time duration of (Tswitch_2 - Tswitch_1)

    - Further discuss the granularity of the optional UE capability:

  - Option 1a: per band pair per BC

  - Other options are not precluded

  - Option 2: Do not introduce the advanced optional UE ability.

Issue 1-2-3: Ambiguity issue when two Tx chains are switched between two different band pairs

Way forward:

- For Rel-18 UL Tx switching among 4 bands, when switching from 1T+1T on band A and B to 1T+1T on band C and D is performed, it is not clear whether UE performs Tx switching {from band A to C + B to D} or {from band A to D + B to C}:

  - As baseline UE assumption, no need to resolve the ambiguity issue of the switching pattern for each Tx chain and determine the switching gap based on the worst case by default, i.e., neither of the two Tx chains is expected to be used for transmission during the maximum of the four switching periods, i.e., max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}.

    - Note: Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C are the switching periods reported by the UE for band pair A&C, B&D,A&D and B&C, respectively.

Sub-topic 1-3: RAN4 RF requirements

Issue 1-3-1: Requirements for “switched UL”

- Further discuss in the next meeting:

  - Option 1: Define the time mask requirements for bandpair, and the switching requirements should be satisfied for all band pairs capable of Tx switching in a band combination.

  - Other options are not precluded.

Issue 1-3-2: Clarification on the common requirements for “switched UL” and “dual UL” cases

- Further discuss the following options in the next meeting:

  - Option 1: Add the following clarification for switching time mask

    - “The time mask is applicable to uplink transmissions when configured with switchedUL or dualUL.”

  - Option 2: Add the following clarification for switching time mask

    - “The time mask is applicable to uplink transmissions when configured with switchedUL or dualUL for uplinkTxSwtichingOption which is defined in [TS 38.331]. In case of simultaneous transmission in the two bands when configured with dualUL, the time mask are the same.”

  - Other options are not precluded.

Issue 1-3-3: Requirements for “dual UL” case

- In the next meeting, bring CRs for options for further discussion:

  - Option 1: For “dual UL”, define switching time mask requirements to cover the following cases with two Tx chains switched between two different band pairs, i.e.,

    - For Tx switching across 3 bands or Tx switching across 4 bands with concurrent UL transmission supported on only one band pair, the two Tx chains are in band A before switching, and are in band B and C respectively after switching.

    - For Tx switching across 4 bands with concurrent UL transmission supported on at least two band pairs, the two Tx chains are in band A and B respectively before switching, and are in band C and D respectively after switching.

    - Other cases are not precluded.

  - Option 2:

    - Transmission requirements for Tx switching between band pair part of switching and requirements for transmissions for bands with tx chains unchanged due to switching for baseline/advanced feature for scenarios described in issues 1-2-1, 1-2-2 and 1-2-3.

  - Other options are not precluded.

Issue 1-3-4: CR language

- Further discuss the language based on the CR in the next meeting:

  - Option 1:

    - UE requirements are written assuming known network behaviour i.e., not using “UE is not expected to ..” language.

    - Language to be used for specifying UE behaviour for concurrent switching periods of different lengths:

“UE shall be capable of starting transmission after the end of switching periods on all bands that are configured for uplink in tx switching. Transient periods are not included in the switching periods.”

  - FFS other options

Sub-topic 1-4: Applicability of DL interruption

Issue 1-4-1: DL interruption for SUL+TDD+FDD combinations

Way forward:

- For SUL+TDD+FDD combinations, when Tx switching is performed between SUL and TDD bands, whether or not to mandate no DL interruption on the SUL and TDD bands can be discussed in a case by case (i.e., per band combination basis) manner.

Issue 1-4-2: Issues of different band pairs having different capabilities of DL interruption

Way forward:

- Further discuss in the next meeting that:

  - RAN4 clarify applicability of DL interruption when different band pairs have different capability of DL interruption and Tx switching is performed for both band pairs simultaneously.

Issue 1-4-3: On previous agreements for combinations of SUL+TDD and TDD+TDD CA band combinations

Way forward:

- For synchronized CA between SUL configuration (with a SUL band and a TDD band) and TDD band(s) with the same UL-DL pattern across all TDD bands:

  - Option 1: DL interruption is not required

  - Option 2: whether to mandate no DL interruption can be discussed in a case by case (i.e., per band combination basis) manner.

- For synchronized CA among 3 or 4 TDD band(s) [‘with the same UL-DL pattern’ or ‘without mandatory simultaneous Rx/Tx requirement’] across all TDD bands, DL interruption is not required.

<R4-2220547>

Sub-topic 2-1: RF specification impact

Issue 2-1-1: Need of new time mask requirements for 2-TAG case

Option 1:

Time mask with an UL time difference covering all possible time delays (R4-2218827), example for 2 bands but also relevant for 3-4 bands for both SUL and CA configurations with dual TAG.

Option 2:

Do not modify the time mask for Tx switching for multiple TAGs. The impact of Tx switching with multiple TAGs can be considered as scheduling restriction. Example below (R4-2219410) for two bands:

General understanding: only full OS are “blanked” (not scheduled or configured for transmission)

WF:

TBD

Sub-topic 2-2: UL outage time

Issue 2-2-1: Discussion and specification for UL outage time

Wed agreement:

Agreement: outage time will be discussed in RF session.

WF:

TBD

Sub-topic 2-3: Location of switching period in relation to time T0

WF: the switching period is located immediately before time T0 (start of transmission on a carrier after the switch)]

- Other options no precluded

Sub-topic 2-4: Scenario

- Proposals:

  - Option 1: Clarifications on the multi-TAG scenario are needed for UL transmission on a carrier without corresponding DL carrier. RAN4’s CR should reflect all the possible multi-TAG scenarios. (ZTE)

    - There are two interpretations when there is a SUL band included in UL Tx switching across 3/4 bands. For example, there are four bands A+B+C+D, where D is SUL band, C is the corresponding NUL band, and A and B are the other two NUL bands

  - Interpretation 1: All the bands belong to one TAG.

  - Interpretation 2: bands C and D belong to one TAG, and bands A and B belong to another TAG.

- Recommended WF

  - Further discuss.

WF:

the SUL case included; the SUL has an associated NUL (the same uplink cell) in the same TAG, a NUL of another cell can be in the other TAG

<R4-2220548>

LS from RAN4 to RAN1/2 (R4-2220548) was approved

<R4-2220417>

Topic #1:	DL interruption for Tx switching across 3/4 bands with 2TAGs

<Agreement>: Issue 1-1: RTD for non-collocated inter-band scenario for Tx switching with 2 TAGs

Agreements on GTW:

  - Use RTD=6us side condition for non-collocated inter-band case to derive DL interruption length for 2TAGs case.

  - FFS if requirements for RTD > 6us is needed

  - Note: RTD value will be captured in the specification as an informational note in the table for DL interruption length for 2TAGs as an assumption to derive the values.

<Agreement>: Issue 1-2: The principle of deriving DL interruption length for 2TAGs

As R16/R17 the interruption length in symbols due to UL TX switching is define as,

Ceil((switching period+2*TA adjustment uncertainty+2*RTD-CP length)/symbol duration)+1

<Agreement>: Issue 1-3: DL interruption length for 2TAGs

Agreements on GTW:

- Use the following DL interruption length (in symbols) for R18 Tx switching across 3/4 bands with 2 TAGs for RTD = 6us

|  | NR Slot length (ms) | Uplink Tx switching period |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | [7 or 8] |
| 2 | 0.25 | 4 | 10 | 14 |

<Agreement>: Issue 1-4: Applicable scenario of DL interruption requirements

It is recommended not to further discuss this issue in RRM session. RRM can directly use the conclusion of RANP or RF session.

Topic #2: UL outage time for TX switching with 2 TAGs

This topic is closed as the below agreement is achieved in RF session.

Agreement: outage time will be discussed in RF session.

RAN4#106

RF

R4-2303719	CR for 38.101-1: Time mask for switching across three or four uplink bands

R4-2303719 CR for 38.101-1: Time mask for switching across three or four uplink bands => Endorsed
R4-2303693 WF on Multi-carrier enhancements for NR => Approved
R4-2303507	LS on Rel-18 Multi-carrier enhancement for NR=> Approved

The following agreement is captured in RAN4#106 chairman’s note

Agreement:

- RAN4 proposes to extend the work item for Multi-carrier enhancements for NR

The following contents are captured in approved WF(R4-2303693)

Topic 1: Tx switching across 3/4 bands with single TAG

Issue 1-1-3: Fallback of Rel-18 Tx switching to Rel-16/17 Tx switching

Candidate options:

- Option 1:

UE will report the 3/4 band combination with Tx switching capability. It is expected that this gives the network sufficient information on UE capability for Tx switching across all fallback combinations.

- Option 2:

For a band pair supported Rel-18 1T-2T switching, Rel-16 1T-2T switching is supported as well.

For a band pair supported Rel-18 2T-2T switching, Rel-17 2T-2T switching is supported as well.

- Other options are not precluded

Sub-topic 1-3: Applicability of DL interruption

Issue 1-3-1: Issues of different band pairs having different capabilities of DL interruption

Way forward:

- If it is possible for two different band pairs conducting Tx switching across the same time and with overlapping switching period, and if the UE capability of DL interruption on any of the band(s) is different, the worst case for each band should be considered, i.e., DL interruption on one band is allowed if it is allowed when Tx switching is conducted on one of the two band pairs.

Issue 1-3-2: On previous agreements for combinations of SUL+TDD and TDD+TDD CA band combinations

- Way forward for “synchronized CA between SUL configuration (with a SUL band and a TDD band) and TDD band(s) with the same UL-DL pattern across all TDD bands”

  - Whether to mandate no DL interruption can be discussed in a case by case (i.e., per band combination basis) manner.

- Way forward for “synchronized CA among 3 or 4 TDD band(s) [‘with the same UL-DL pattern’ or ‘without mandatory simultaneous Rx/Tx requirement’] across all TDD bands, DL interruption is not required”

  - No consensus on whether to use ‘with the same UL-DL pattern’ or ‘without mandatory simultaneous Rx/Tx requirement’, and it is recommended to discuss whether to mandate no DL interruption in a case by case (i.e., per band combination basis) manner.

Sub-topic 1-4: CR related issues

Issue 1-4-1: 2-layer UL-MIMO support for carrier(s) capable of 2Tx

Way forward:

- Further discuss on whether or not to mandate 2-layer UL-MIMO support for carrier(s) capable of 2Tx.

Issue 1-4-2: Location of switching period in relation to time T0

Way forward:

- Further discuss on whether or not to define the time relationship between the location of the switching period and T_0 in RAN4 specification, taking into account the RAN1 agreement.

  - Note: Similar change to Rel-16/17/18 if any change is needed.

Issue 1-4-3: Time mask for dualUL related switching scenarios

Way forward:

- Further discuss whether and how to cover the following scenario based on the RAN1/2 further agreement.

- Further discuss whether and how to cover the following scenario based on the RAN1/2 further agreement.

Time mask for one transmitter switching between band X and band Z, and between band Y and band Z, where UE is capable of uplink transmission on band Y during time period of {switching period #1 - switching period #2}, i.e., the UE indicates [TBD-1] in the capability [TBD tx-on-non-affected-band]

Time mask for one transmitter switching between band X and band Z, and between band Y and band Z, where UE is not capable of uplink transmission on band Y during time period of {switching period #1 - switching period #2}, i.e., UE does not indicate [TBD-1] in the capability [TBD tx-on-non-affected-band]

Topic 2: Tx switching with dual TAGs

Sub-topic 2-1: RF specification impact

Way forward:

- RAN4 can further discuss the following two options in RAN4 #106e-bis meeting:

  - Option 1: Introduce new time mask requirements

    - Option 1a: To ensure that UE functionality can be properly tested, modify the existing time masks such that Tx switching with uplink slot timing difference between carriers is verified when symbols containing the switching period preceding the time T0 are not scheduled by the test system on either carrier to avoid UE malfunction in the field.

  - Option 2: Do not modify the time mask for Tx switching for multiple TAGs. The impact of Tx switching with multiple TAGs can be considered as scheduling restriction.

Sub-topic 2-2: Clarificaiton on UE behaviour in 2-TAG case

Way forward:

- For UEs declaring the Tx switching support with 2-TAG, the baseline and advanced UE behaviors applies to both 1-TAG and 2-TAG cases.

RRM

<Agreement>: Issue 1-1: whether to define requirements for RTD>9us in non-collocated inter-band scenario for Tx switching with 2 TAGs

For information: R4-2300818 from CMCC provided the maximum RTD that can be supported by the DL interruption lengths (which is derived from RTD=6us). It is observed that the supported RTD values can be more than 9us.

|  | NR Slot length (ms) | Uplink Tx switching period |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 (20us RTD) | 3 (33us RTD) | 4 (33us RTD) |
| 1 | 0.5 | 3 (19us RTD) | 6 (21us RTD) | 8 (22us RTD) |
| 2 | 0.25 | 4 (9us RTD) | 10 (11us RTD) | 14 (11us RTD) |

Define the interruption requirements for RTD =< 9 us. Do not define RRM requirements for RTD > 9 us.

<Agreement>: Issue 1-2: DL interruption requirements for Tx switching with 2 TAGs

Background: The principle of deriving DL interruption length for 2TAGs was agreed in R4-2220417.

<Agreement>: Issue 1-2: The principle of deriving DL interruption length for 2TAGs As R16/R17 the interruption length in symbols due to UL TX switching is define as, Ceil((switching period+2*TA adjustment uncertainty+2*RTD-CP length)/symbol duration)+1

DL interruptions for R18 Tx switching across 3/4 bands with 2 TAGs are as below,

|  | NR Slot length (ms) | Uplink Tx switching period |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | 8 |
| 2 | 0.25 | 4 | 10 | 14 |
| Note: RTD=9us is assumed to derive the DL interruption length. |  |  |  |  |