3GPP TSG RAN WG1 #112bis-e			R1- 2303743

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.3.3

Source: 	LG Electronics

Title: 	Study on Potential enhancements on dynamic/flexible TDD

Document for:	Discussion and decision

Introduction

In RAN#94-e meeting, a study item on evolution of NR duplex operation is approved and the corresponding description is provided in [1]. According to the SID, the subband non-overlapping Full Duplex (SB-FD) and potential enhancements on dynamic/flexible TDD are studied. Also, identification of deployment scenarios and developing evaluation methodology are also included in the scope as follows.

In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we discuss on potential enhancements on dynamic/flexible TDD. Specifically, discussion is focused on the cross link interference (CLI) in terms of UE-to-UE CLI.

UE-to-UE CLI handling

Before going into detailed discussions, it is necessary to examine the scenarios in which UE-to-UE CLI occurs. First, the link direction between the UEs should be different, and secondly, the distance between the two UEs must be close enough for the victim UE to receive the aggressor UE's signal. This can be divided into intra-cell UE-to-UE CLI and inter-cell UE-to-UE CLI. Intra-cell UE-to-UE CLI occurs when the link direction between UEs within a cell is different. This can occur when some UEs fails to receive change of TDD configuration properly in cells that operate flexible TDD, but is mainly related to the SBFD environment. On the other hand, inter-cell UE-to-UE CLI occurs in both dynamic/flexible TDD and SBFD environments. In this case, it can be assumed that both the victim and aggressor are located at the cell edge in order for the distance between the two UEs to be close enough. Therefore, it is perfectly reasonable to focus on the environment where both the victim and aggressor UEs are located at the cell edge in inter-cell UE-to-UE CLI.

Observation 1. Inter-cell UE-to-UE CLI with both of victim and aggressor UE located at the cell edge is common scenario to both SBFD and dynamic/flexible TDD.

In the case of flexible/dynamic TDD and changing link directions between gNBs, periodic resource measurements have limited reliability. It is because the link direction can be changed dynamically and/or flexibly. Particularly the uplink transmissions are not always periodic and have limited repetition numbers. Therefore, performance degradation of downlink reception due to UE-to-UE CLI experienced by the victim UE cannot be reflected in long-term characteristics or may only be reflected in some parts, making it insignificant. In other words, UE-to-UE CLI in dynamic/flexible TDD has short-term characteristics, and thus, L1/L2 based measurement and report that are tailored to this characteristic are necessary. In such context, discussion provided is focusing on L1/L2 based measurement and report.

Measurement for L1/L2 based UE-to-UE CLI

In this section, measurement resource itself and further information to enhance more accurate measurement, i.e., spatial domain information and timing information for measurement are discussed.

Measurement resource

For the conventional L3 based CLI measurement, only periodic resource can be configured for both of SRS and CLI. Since the target of L1/L2 based UE-to-UE CLI enhancement is to deliver timely manner CLI information to gNB, it is natural that aperiodic resource should be considered for time domain configuration of measurement resource. If the periodic resource is considered for L1/L2 based CLI measurement resource, since the conventional configurable periodicity is between 120 ms and 30 min, which is dramatically long, could be shortened.

Proposal 1. For L1/L2 based UE-to-UE CLI measurement resource, consider aperiodic measurement resource configuration and/or shortened periodicity.

L1/L2 based UE-to-UE CLI measurements are intended to estimate short-term CLI, and thus they may be more vulnerable to environmental changes than L3 measurements. To suppress this effect, configuration or indication of additional information for L1/L2 based CLI measurements can be considered.

Spatial domain information

In order to achieve more accurate and robust CLI measurement, beam information can be taken into consideration. While it is practically impossible for the victim UE to receive beam information from the aggressor UE or to apply a beam directed to aggressor UE since UE-to-UE measurement/signaling is not possible, it is highly likely that the direction in which the victim UE is facing is similar to or the same as the gNB serving the aggressor UE. Therefore, the beam information (e.g., SSB, CSI-RS) of the serving cell can be considered as the beam information for CLI measurement. For the configuration of beam information for cells other than the serving cell, there have been similar discussions in Rel-17 ICBM (Inter-cell beam management) and being discussed in Rel-18 Further NR mobility enhancements. In these items, the beam information configuration for candidate cells for beam change or cell change was discussed, and this concept can be applied to CLI by replacing the candidate cell to the (potential) aggressor UE belongs. This can provide relatively accurate information, especially in the case of CLI-RSSI measurement rather than sequence-based CLI measurement (SRS-RSRP). Although it is discussed in following section 2.3, it is considered to be targeting on channel measurement rather than interference measurement.

Proposal 2. For L1/L2 based UE-to-UE CLI measurement resource, consider spatial domain configuration.

Figure 1. Potential UE-to-UE CLI scenario

Timing information

Existing CLI measurement timing depends on UE implementation. However, setting the timing is important in terms of accurate CLI measurement. According to agreements in Rel-18 NR mobility enhancement, TA acquisition of candidate cells is supported prior to the L1/L2 cell switch command. Similar to gNB-to-gNB CLI, a candidate cell can be considered an adjacent cell that the UE can potentially handover to, which can be the serving cell of the aggressor UE in the inter-cell UE CLI scenario previously mentioned, assuming that the UE is at the cell edge. This means that UEs can be divided into two groups; UEs with and without the capability to obtain TA information of the candidate cell in the network. For UEs without this capability, it is difficult to consider additional enhancements since there is no way for such UE to obtain UE-to-UE timing information or infer it from any other available information. On the other hand, UEs with the capability can differentiate the CLI measurement value of each candidate cell based on the UL TA of the candidate cells. For an inter-cell CLI scenario where the victim UE and the aggressor UE are located at the cell edge, an example is shown in Figure 1.

Figure 2. Example of timeline UEs and gNBs

In the figure, gNB1 is performing DL and UE1, which is being served by gNB1, is the victim and receiving the signal from gNB1. gNB2 and gNB3 both have UL as their link direction, and UEs served by gNB2 and gNB3 are both aggressors performing UL. UE1 has the capability to obtain TA information for candidate cells (i.e., gNB2, gNB3). UE1, UEs served by gNB2, and UEs served by gNB3 are all adjacent enough to generate CLI. If they are not adjacent, the probability of experiencing CLI is low because UE's transmit power is not strong enough. An example of the timeline of the transmission and reception timing of UEs and gNBs is shown in Figure 2.

As seen from the figure, the timing of the CLI received by UE1 from UEs served by gNB2 is similar to the timing advance from UE1's perspective to gNB2, and the timing of the CLI received by UE1 from UEs served by gNB3 is similar to the timing advance from UE1's perspective to gNB3. This is because the distance between the UEs is close. In other words, the TA value to gNB 2 known by UE1 is the value advanced from the reception time of gNB2 to UE1 and gNB2's channel propagation delay, and the time UEs served by gNB2 transmit is the value advanced from the reception time of gNB2 to UEs served by gNB2 and gNB2's channel propagation delay. The channel propagation delay between UE1 and gNB2 and between UEs served by gNB2 and gNB2 can be considered similar. Therefore, TA values for candidate cells can be used to more accurately measure the CLI of the aggressor UE belonging to that cell. This can also be used to differentiate which cell the aggressor UE belongs to, depending on the cell deployment.

Proposal 3. For UEs capable of TA acquisition of candidate cell, TA of candidate cell can be used to accurate inter-cell UE-to-UE CLI measurement.

Report for L1/L2 based UE-to-UE CLI

An important part of L1/L2 UE-to-UE CLI is the triggering method. If we follow the same methodology for L3 measurements, two types can be considered: gNB indicated report and event-triggered report. Similar discussions can be found for L1 measurement report for Rel-18 L1/L2 mobility. The result of this discussion is that for gNB scheduled L1 measurement report for Rel-18 L1/L2-triggered mobility, report as UCI (semi-persistent report on PUSCH, and aperiodic report on PUSCH) is supported and periodic and semi-persistent PUCCH is remained to be FFS. For event-triggered reports, discussions are ongoing, considering factors such as the definition of events in L1. Based on them, gNB indicated report, event triggered report is discussed.

gNB indicated report

For gNB indicated reports, it would be natural to report the L1/L2 CLI report contents as a UCI to reuse the existing reporting mechanism. To reduce spec impact and considering report on both PUCCH and PUSCH, treating the UCI for L1/L2 CLI reports content as the UCI for CSI would be one option. Alternatively, to minimize the spec impact, a new type of UCI that reports only on PUSCH could be considered. That is, a new type of UCI could be sent on PUSCH with a payload that is added to or combined with the payload of the existing UCI, or a bit for CLI reports on PUSCH could be punctured to report.

On top of that, it was agreed in RAN1#112 meeting that using existing CSI framework as the baseline is L1/L2 CLI measurement and report. The details regarding using existing CSI framework is discussed in following section. Besides that, how the bits for CLI reporting contents is going to be treated should be discussed. To enable L1/L2 report, it would be natural that CLI report is delivered via UCI. The existing types of UCIs are SR, HARQ-ACK, CSI part 1 and CSI part 2. In terms of “using existing CSI framework”, UCI for CSI part 1 and CSI part 2 can be considered. It should be noted that the UCI of CSI part 1 is transmitted by concatenating the generated bits, whether they are transmitted via PUSCH or PUCCH, according to the existing rule. On the other hand, CSI part 2 is subject to the omission rule, which means that some reports may be omitted. In terms of minimum spec impact and delivering CLI estimation in timely manner perspective view, UCI of CSI part 1 seems proper for L1/L2 CLI report. On the other hand, UCI of CSI part 2 can be considered to be proper for CLI report since CSI report is more important than the CLI measurement report. Another approach can be introducing new type of UCI for L1/L2 CLI report to be free from the aforementioned issues.

Proposal 4. For gNB indicated report, consider L1/L2 based UE-to-UE CLI measurement is reported via UCI

- UCI for CSI part 1, UCI for CSI part 2, new type of UCI can be considered.

Event-triggered report

The existing definition of an event for L3 measurement is a mechanism that reports when a configured threshold is exceeded. For L1/L2 CLI, it is necessary to define the event based on L1/L2 requirements. The definition of the L3 measurement event can be used as a basis, where the UE performs CLI monitoring for the configured resources, and an event can be defined as the received CLI exceeding a configured threshold. Two options can be considered in this case, based on whether the UE performs the report based on event occurrence. If an event occurs, the UE can perform CLI measurement report through PUSCH or PUCCH without scheduling. This method allows the gNB to easily monitor the UE's CLI environment within the cell, even without the gNB indicating the UE to report the CLI measurement.

Alternatively, we could consider separating event occurrence and reporting. If L1/L2 measurements are reported to the UCI, specific bits of the UCI can be reserved, and when an event occurs, the CLI measurement results can be inserted into the corresponding bit. If no event occurs, the bit can be left empty, and reporting can follow the gNB indicated report. In other words, the UE can hold the measurement results based on event occurrence until the gNB indicates the CLI measurement report to be sent. While this approach may make it difficult for the gNB to sequentially understand the UE's CLI environment, it can prevent frequent transmission by the UE without scheduling.

L1/L2 CLI measurement and report based on CSI framework

It was agreed in RAN1#112 meeting that using existing CSI framework as the baseline is L1/L2 CLI measurement and report. The common understanding of “existing CSI framework” is required in this regard. Although it is one of the most complicated framework, it can be briefly summarized.

The procedures of CSI reporting is that the CSI reporting is triggered by DCI, by applying the RRC pre-configured parameter. The time and frequency resources that can be used by the UE to report CSI are controlled by the gNB. Multiple CSI-ReportConfig Reporting Settings and multiple CSI-ResourceConfig Resource Settings, and one or two list(s) of trigger states is configured to UE by RRC. Each trigger state contains a list of associated CSI-ReportConfigs indicating the Resource Set IDs for channel and optionally for interference. Each trigger state contains one associated CSI-ReportConfig.

It should be noted that the CSI reporting configuration plays a role in linking the measurement resource, reporting configuration, and reporting resource. That is, CSI reporting configuration contains resource (CSI-RS, CSI-IM and/or SSB) for channel measurement (and interference measurement), report quantity (RI, PMI, CQI, etc.), CSI encoding (subband/wideband), information required for transmission of measurement report, etc. In addition, CSI reporting configuration types have the following constraints. Periodic CSI reporting is reported on PUCCH, semi-persistent CSI reporting is reported on PUCCH when activated/deactivated by MAC-CE, and on SPS PUSCH when activated/deactivated by DCI. Aperiodic CSI reporting is triggered by DCI and transmitted on PUSCH.

When applying L1/L2 CLI measurement and report to the CSI framework, two aspects need to be considered: the measurement resource configuration aspect and the report configuration aspect. When considering the measurement resource configuration aspect, it seems possible to have separate configurations for CSI measurement resources and CLI measurement resources. However, there are significant differences between them. From the perspective of measurement resource configuration, it is possible to have separate configurations for CSI and CLI measurements, but there are significant differences between them. The CSI measurement resource is a BWP-specific parameter set in the DL BWP, and the UE only performs measurements within the active DL BWP. On the other hand, the CLI configuration is not a BWP-specific parameter and is not limited to measurements within the active DL BWP. Therefore, if the CLI configuration is reused in the CSI framework, some adjustments may be necessary to align them.

Next, we discuss the aspect of report configuration. As there is no existing L1/L2 CLI report configuration, it is possible to create a new report configuration, which leads to two possible interpretations. Firstly, there could be a way to add a configuration for L1/L2 CLI report to the existing CSI reporting configuration, and secondly, it could be possible to mimic the CSI reporting configuration to set up a new configuration for CLI report. The first method has the disadvantage of using a limited CSI Reporting configuration ID for CLI measurement, which may lead to resource exhaustion for CSI reporting. However, if the purpose is to combine CSI and CLI measurement in one report, this approach may be appropriate. On the other hand, the second approach would be to create a new CLI report configuration that mimics the CSI reporting configuration without consuming CSI reporting configuration resources. However, this approach still consumes UE measurement and/or report capabilities, and since separate configurations exist for CSI and CLI, conflicts could arise in the UE's measurement and/or report settings, requiring additional handling. In such case, it is important to note that CSI reports should take precedence over CLI reports in terms of report importance.

Proposal 5. L1/L2 CLI measurement and report reusing existing CSI framework should be investigated in following aspects;

- Measurement configuration perspective

- Report configuration perspective

More importantly in previous meeting, there was an argument whether L1/L2 CLI measurement should be interpreted as a channel measurement or interference measurement, or even both should be considered. As far as our concern, it is very generic question should be discussed. Since CSI framework has two functionality that channel measurement report and interference measurement report, our view based on it is provided.

L1/L2 CLI measurement as channel measurement in terms of CSI framework

The channel measurement in CSI framework is targeting to receive other signals transmitted from the same transmitter. To achieve this goal, channel measurements are performed by transmitting a reference signal from the transmitter and estimating the channel quality and/or coefficients at the receiver using a given sequence (or even using a codebook). Based on these measurements, the receiver can estimate the subsequent transmitted signal. If we apply this concept to CLI, treating CLI measurement as channel measurement means that the victim UE performs channel measurement based on the reference signal transmitted by the aggressor UE, and the victim UE aims to receive the subsequent transmission signal (e.g., PUSCH, PUCCH) based on the channel measurement results. To perform channel measurement, it is necessary to use RSRP based on the SRS resource, not RSSI measurement based on the CLI resource. Additionally, the victim UE should be able to distinguish between each aggressor UEs, and the beam being measured should be directed towards the beam that each aggressor UE is targeting, in order to perform more accurate measurement. In this regard, using a beam that points to the gNB serving the aggressor UE, as mentioned earlier, could be considered, as using a beam that directly points to the aggressor UE is practically difficult. Therefore, using a beam that points to adjacent cells mentioned in section 2.1 can be considered in terms of channel measurement. Furthermore, the victim UE needs to differentiate and perform measurement for each aggressor UE. The only way for the victim UE to use the signal transmitted by the estimated aggressor is through interference cancellation at the receiver. In summary, channel measurement for L1/L2 CLI measurement is considered to be based on the assumption that victim UE is capable of advanced receiver.

In terms of channel measurement for L1/L2 CLI measurement, the necessary information and actions required by the network are as follows. First, the gNB needs to aware of the pair information between the aggressor UE and the victim UE in advance. Then, the gNB needs to indicate the victim UE to perform CLI measurement and indicate the aggressor UE to transmit SRS in the time resource specified for CLI measurement in victim UE’s perspective.

The coordination for inter-cell UE can be challenging for the following reasons. Firstly, it requires close coordination between gNBs. Secondly, in the case of inter-cell UE interference, both aggressor and victim UEs are likely to be cell-edge UEs, so the time difference between the victim UE's DL reception timing and the aggressor UE's UL transmission timing is likely to be significant due to the aggressor UE's uplink timing advance. To interpret L1/L2 CLI as channel measurement, it is necessary to consider the issues discussed above.

Proposal 6. Followings need to be taken into account when UE-to-UE L1/L2 CLI measurement is considered as channel measurement.

- The aggressor UE should be indicated to transmit reference signal when victim UE is indicated for L1/L2 CLI measurement.

- The victim UE with advanced receiver (e.g., IRC) and capable of distinguishing aggressor UEs is assumed.

L1/L2 CLI measurement as interference measurement in terms of CSI framework

It is considered the case of interference measurement within the CSI reporting framework. The resources that can be configured for interference measurement are NZP CSI-RS and CSI-IM, where NZP CSI-RS is intended for intra-cell interference measurement, and CSI-IM is mainly for inter-cell interference measurement. The resources for interference measurement are all related to the resources for channel measurement and have a QCL relationship. (The UE may assume that the two CSI-RS resources for channel measurement in a Resource Pair and the associated CSI-IM resource for interference measurement are resource-wise QCLed with respect to 'typeD'.) First of all, using NZP CSI-RS for interference measurement involves performing residual interference measurement, where term residual stems from the fact that interference measurement is obtained after subtracting the desired signal from the overall received signal. On the other hand, using CSI-IM for interference measurement directly measures the interference power from the indicated resource elements (REs), which is the same as in LTE.

Based on the previous discussion, considering the reuse of CSI reporting framework for L1/L2 CLI measurement, it would be more appropriate to measure interference directly instead of residual interference measurement. When the residual interference measurement method is considered for L1/L2 CLI, it cannot be measured only but it can be measured during the CSI measurement which is undesirable. To enable direct interference measurement, configuration of an SRS resource or CLI resource instead of using the resources for CSI measurement and reusing reporting configuration of CSI report can be considered. Since it is direct measurement of interference, both of RSRP and RSSI can be considered following conventional CLI measurement metric.

The main difference between this method and channel measurement is that it does not aim to receive signals from the aggressor in the future. Instead, the purpose of it is to estimate the presence or level of interference in the channel through which the victim UE receives the desired signal from the serving cell. Based on this approach, the victim UE should measure the RSRP or RSSI from the measurement resource based on the beam used for downlink reception (the TCI state ID that is already set for the NZP CSI-RS) instead of the beam towards the aggressor UE. Since the measurement is based on interference instead of channel, the interference level may vary even if the aggressor UE is not always transmitting on the victim UE's allocated resources, but it does not significantly affect the quality of the measurement. In that sense, even for inter-cell UE interference, tight coordination between gNBs may not be necessary.

Proposal 7. Followings need to be taken into account when UE-to-UE L1/L2 CLI measurement is considered as interference measurement.

- Victim UE applies beam used for desired signal from gNB when L1/L2 CLI measurement is indicated.

Summary

In this contribution, we have discussed on potential enhancements on dynamic/flexible TDD. From the discussion, we obtained following proposals and an observations:

Observation 1. Inter-cell UE-to-UE CLI with both of victim and aggressor UE located at the cell edge is common scenario to both SBFD and dynamic/flexible TDD.

Proposal 1. For L1/L2 based UE-to-UE CLI measurement resource, consider aperiodic measurement resource configuration and/or shortened periodicity.

Proposal 2. For L1/L2 based UE-to-UE CLI measurement resource, consider spatial domain configuration.

Proposal 3. For UEs capable of TA acquisition of candidate cell, TA of candidate cell can be used to accurate inter-cell UE-to-UE CLI measurement.

Proposal 4. Consider L1/L2 based UE-to-UE CLI measurement is reported via UCI

- UCI for CSI part 1, UCI for CSI part 2, new type of UCI can be considered.

Proposal 5. L1/L2 CLI measurement and report reusing existing CSI framework should be investigated in following aspects;

- Measurement configuration perspective

- Report configuration perspective

Proposal 6. Followings need to be taken into account when UE-to-UE L1/L2 CLI measurement is considered as channel measurement.

- The aggressor UE should be indicated to transmit reference signal when victim UE is indicated for L1/L2 CLI measurement.

- The victim UE with advanced receiver (e.g., IRC) and capable of distinguishing aggressor UEs is assumed.

Proposal 7. Followings need to be taken into account when UE-to-UE L1/L2 CLI measurement is considered as interference measurement.

- Victim UE applies beam used for desired signal from gNB when L1/L2 CLI measurement is indicated.

Reference

- CMCC, “New SI: Study on evolution of NR duplex operation,” RP-213591, e-Meeting, December 6th – 17th, 2021

- RAN1 Chair’s Notes, RAN1 #109-e, e-Meeting, May 9th – 20th, 2022

- RAN1 Chair’s Notes, RAN1 #110, Toulouse, France, August 22nd – 26th, 2022

- RAN1 Chair’s Notes, RAN1 #110-bis-e, e-Meeting, October 10th  – 19th, 2022

- RAN1 Chair’s Notes, RAN1 #111, Toulouse, France, November 14th – 18th, 2022

- RAN1 Chair’s Notes, RAN1 #112, Athens, Greece, February 27th – March 3rd, 2023

- Annex

Following agreements were made in RAN1#109-e meeting [2].

Agreement - For discussion in AI 9.3.3, consider the deployment scenarios for dynamic/flexible TDD which are agreed for evaluation purpose under AI 9.3.1 in RAN1#109-e. - Under AI 9.3.3., no more discussion about the deployment scenario for potential enhancement on dynamic/flexible TDD Agreement At least, following interference scenarios can be considered for study of dynamic/flexible TDD: - gNB-to-gNB inter-cell co-channel interference - UE-to-UE inter-cell co-channel interference Guideline for future meetings - Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD. - Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD. Agreement For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of gNB-to-gNB CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings: - gNB-to-gNB CLI measurement and reporting - Coordinated scheduling - Spatial domain enhancements - Advanced receiver - UE and gNB transmission and reception timing - Power control based solution - Potential enhancements to Rel-16 RIM - Sensing based mechanism - Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified - Note: Any other scheme(s) for inter-gNB CLI handling is/are not precluded. - Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. - Note: Potential enhancements specific for SBFD will be discussed in 9.3.2 Agreement For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of UE-to-UE CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings: - Potential enhancements to UE-to-UE CLI measurement/reporting - Coordinated scheduling - Spatial domain enhancements, - Advanced Receiver - UE and gNB transmission and reception timing - Power control based solution - Sensing based mechanism - Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified - Note: Any other scheme(s) for UE-to-UE CLI handling is/are not precluded. - Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. - Note: Potential enhancement specific for SBFD will be discussed in 9.3.2 Conclusion The following self-interference scenario and inter-subband CLI scenarios are not considered under AI 9.3.3 (Potential enhancements on dynamic/flexible TDD). - gNB self-interference - UE-to-UE intra-cell co-channel inter-subband CLI - UE-to-UE inter-cell co-channel inter-subband CLI - gNB-to-gNB inter-cell co-channel inter-subband CLI

Also, following agreements were made in RAN1#110 meeting [3].

gNB-to-gNB CLI 1. gNB-to-gNB CLI measurement Agreement Study the feasibility and potential benefits of gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Measurement resource configuration - Measurement details - Relevant information exchange - Usage of measurement 2. Coordinated scheduling Agreement Study the feasibility and potential benefits of coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, the study at least includes: - Details of coordinated scheduling for time/frequency resources - Relevant information exchange 3. Spatial domain enhancements Agreement Study the feasibility and potential benefits of spatial domain coordination method for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, the study at least includes: - Details for spatial domain coordination - Relevant information exchange Note1: Study can include method for FR1 and FR2

UE-to-UE CLI 1. Potential enhancements to UE-to-UE CLI measurement/reporting Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB 2. Coordinated scheduling Agreement Study the feasibility and potential benefits of coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Details of coordinated scheduling for time/frequency resources - Relevant information exchange (if needed) 3. Spatial domain enhancements Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI handling based on spatial domain coordination method which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic /flexible TDD, at least includes: - Details for spatial domain coordination by gNB - Relevant information exchange (if needed) Note1: Study can include method for FR1 and FR2

Following agreements were made in RAN1#110-bis-e meeting [4].

gNB-to-gNB CLI 1. gNB-to-gNB CLI measurement Agreement For gNB-to-gNB co-channel CLI measurement, the potential benefit of uplink resources muting can be studied further. Note: Proponents of uplink resource muting are encouraged to provide evaluation result for comparison of performance between two cases when uplink resource muting based gNB-gNB CLI handling schemes including both UE transparent and non-UE transparent schemes is applied or not. Agreement For gNB-to-gNB co-channel CLI measurement, consider as baseline reusing existing DL channel(s)/signal(s)/measurement_resource(s) - For example, SSB, NZP/ZP-CSI-RS, DMRS for PDCCH/PDSCH, CSI-IM, RSSI measurement resource, etc. - FFS: Which type of DL channel(s)/signal(s) can be used for gNB-to-gNB co-channel CLI measurement - FFS: How resources are used/configured 3. Spatial domain enhancements Agreement For details of spatial domain coordination method for gNB-to-gNB co-channel CLI handling, at least followings can be studied. - Recommended/restricted Beams between gNBs - Beam nulling between gNBs - Beam pairing between gNBs - Other schemes are not precluded. 7. Potential enhancements to Rel-16 RIM Conclusion No further discussion for potential enhancement to Rel-16 RIM for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD. 8. Sensing based mechanism Conclusion No further discussion for sensing based mechanism for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD.

UE-to-UE CLI 1. Potential enhancements to UE-to-UE CLI measurement/reporting Agreement For UE-to-UE co-channel CLI measurement, consider as baseline reusing existing channel(s)/signal(s)/measurement_resource(s) - For example, SRS resources defined in Rel-16 for SRS-RSRP measurement, CLI-RSSI resources defined in Rel-16 for CLI-RSSI measurement - FFS potential enhancements Agreement For UE-to-UE co-channel CLI handling, study L1/L2 based UE-to-UE CLI measurement and reporting - Note: Accounting for UE processing/reporting delay – companies to share their assumptions - Note: Proponents are encouraged to provide the mechanism of L1/L2 based CLI measurement and reporting, and to provide the benefits of L1/L2 based CLI measurement and reporting compared with existing L3 CLI/CSI measurement and report with evaluation result - Note: Accounting for information exchange delay between gNBs (if applicable) 4. Advanced Receiver Conclusion Under AI 9.3.3, no further discussion on UE side advanced receiver for UE-to-UE co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD 7. Sensing based mechanism Conclusion No further discussion for sensing based mechanism (i.e. LBT) for UE-to-UE co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD

Following agreements were made in RAN1#111 meeting [5].

gNB-to-gNB CLI 1. gNB-to-gNB CLI measurement Agreement For gNB-to-gNB co-channel CLI measurement and/or channel measurement, at least periodic NZP CSI-RS/SSB is the baseline in RAN1 study. - FFS: Whether SSB is CD-SSB or NCD-SSB In the study RAN1 assumes that exchange of configuration for NZP CSI-RS /SSB can be an enabler for gNB-to-gNB CLI measurement and/or channel measurement. Agreement For gNB-to-gNB co-channel CLI handling, beam level (i.e., based on measurement result per SSB resource and/or per CSI-RS resource) CLI measurement can be considered for study. 3. Spatial domain enhancements Agreement For spatial domain coordination, the exchange of beam related information among gNB(s) (e.g., victim gNB(s) and aggressor gNB(s)) can be an enabler for inter-gNB co-channel CLI management. - For example 1 (from aggressor gNB to victim gNB), DL beam indication from aggressor gNB(s) - For example 2 (from victim gNB to aggressor gNB), preferred/restricted DL beam and associated resource configuration, beam based inter-gNB co-channel CLI measurement result from victim gNB - FFS: how to define DL beam indication - FFS: how to define DL beam Note: The above examples are only provided as starting point for further discussions

UE-to-UE CLI 1. Potential enhancements to UE-to-UE CLI measurement/reporting Agreement For the purpose of UE-to-UE CLI mitigation, consider the following potential enhancements: - For L1/L2 UE-to-UE CLI reporting, periodic, semi-persistent, aperiodic or event triggered reporting. - For L1/L2 UE-to-UE CLI measurement, periodic, semi-persistent, or aperiodic measurement resource. Companies are encouraged to bring additional details and evaluation results to determine the benefit of the above potential enhancements. 6. Power control based solution Agreement For UE-to-UE co-channel CLI handling, study whether/how to enhance UL power control mechanism. - Existing UL power control mechanism is baseline

Following agreements were made in RAN1#112 meeting [6].

gNB-to-gNB CLI 1. gNB-to-gNB CLI measurement Agreement For the study of gNB-to-gNB co-channel interference measurement, it is assumed that both CD-SSB and NCD-SSB can be used for gNB-to-gNB CLI measurement. Agreement Study the benefit of knowledge among gNBs of configurations such as - SBFD time/frequency configuration 3. Spatial domain enhancements Agreement For spatial domain enhancement of gNB-to-gNB co-channel CLI handling, DL Tx beam information of the gNB can be exchanged between gNBs. Reference signal resource ID (e.g., NZP-CSI-RS resource ID, SSB index) can be used as beam information exchange between gNBs. Agreement For spatial domain enhancement of gNB-to-gNB CLI handling, study the benefit and the procedure of the information exchange of at least the preferred/non-preferred DL beams of the aggressor gNBs, based on the beam information exchanged between gNBs

UE-to-UE CLI 1. Potential enhancements to UE-to-UE CLI measurement/reporting Agreement For L1/L2 based UE-to-UE CLI measurement, SRS-RSRP and CLI-RSSI are to be further studied as baseline metrics. Agreement For the study of L1/L2 based UE-to-UE co-channel CLI measurement, measurement resource for CLI-RSSI measurement as defined in Rel-16 and SRS resource for SRS-RSRP measurement as defined in Rel-16 can be considered. Enhancement of measurement resource can be studied. Agreement For L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism, study the following measurement and report framework. - Use existing CSI framework as the baseline. - Others are not precluded.