3GPP TSG RAN WG1 #112bis-e			R1- 2303747

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.5.5

Source: 	LG Electronics

Title: 	Discussion on positioning support for RedCap UEs

Document for:	Discussion and decision

Introduction

In the last #98 RAN plenary meeting, the following objective for positioning for RedCap UEs was agreed upon as WID [1]:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

Additionally, target performance requirement for positioning for RedCap UEs was captured in [2]:

For commercial use cases for both indoor and outdoor scenarios - Horizontal positioning accuracy: (< 3 m) for 90% of UEs - Vertical positioning accuracy: (< 3 m) for 90% of UEs. For IIoT use cases: - Horizontal positioning accuracy: (< 1 m) for 90% of UEs - Vertical positioning accuracy: (< 3 m) for 90% of UEs.

Based on the performance evaluations of positioning for Redcap UEs in IIOT scenarios provided by a majority of sources, for InF-SH in FR1, the horizontal positioning requirement for IIOT use cases is not achieved by Rel.17 solutions using 5 MHz or 20 MHz of bandwidth. To solve this, enhancements to DL PRS Rx frequency hopping and UL SRS for positioning (SRS-pos) Tx frequency hopping has been studied.

Discussion of DL-PRS Frequency Hopping

In the last RAN1 #112 meeting, it has been agreed to support measurement gap (MG) based measurement on DL PRS Rx hopping.

Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW

If the RedCap UE performs DL PRS Rx hopping, collisions with other signals and channels may occur more frequently. Although PPW can also be considered to handle the collisions, it may not be suitable for the following reasons: UEs are allowed to perform DL PRS measurements inside the active DL BWP using PPW. If measurements using PPW are supported, once a UE measures the first frequency hop of the DL PRS within the active DL BWP, it moves to the next second hop outside the active DL BWP at the next time instance, which means it cannot see any other signals or channels. Although switching of PPW at every hopping can be considered as an alternative, this would require switching of the BWP at every hop since the PPW is associated with the BWP. Therefore, the PPW is not suitable for DL PRS Rx hopping, and it is sufficient to measure based on MG supporting measurement even outside active DL BWP.

Proposal 1: Deprioritize DL-PRS frequency hopping outside MG.

Discussion of UL-SRS for positioning Frequency hopping

In the last #112 RAN1 meeting, it has been agreed to support SRS for positioning frequency hopping by using a configuration separate from the existing BWP configuration.

Agreement For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration - FFS: hopping is configured within a SRS resource or across SRS resources

Configuration for UL SRS pos frequency hopping

It has been decided to introduce frequency hopping for RedCap UEs. Since the UL SRS-pos frequency hopping needs to be performed over a wider frequency range than the UL BWP of the RedCap UE, it can be different from other existing frequency hopping configurations.

Before discussing the configuration method, discussion regarding the parameters required for configuration is required. Firstly, information about the number of hops and the bandwidth of each hop will be needed. Moreover, since the hopping procedure has to be performed in a large frequency range outside the BWP, a switching gap should be required for RF retuning every time each hop is changed.

Proposal 2: Following parameters should be included in SRS-pos frequency hopping configuration

- Number of hops, bandwidth of each hop, switching gap

The UL SRS-pos frequency hopping for Redcap UEs perform outside the UL BWP, requiring a new configuration method. Since the existing SRS configurations available for UE include SRS-MIMO and SRS-pos, both can be considered as a baseline for configuration of SRS for positioning frequency hopping.

Firstly, we will discuss the SRS-MIMO configuration-based approach. The existing SRS-MIMO configuration is primarily designed for UL beam management and offers high flexibility, including support for frequency hopping. Therefore the SRS-MIMO configuration can be modified by adding the necessary functionalities to configure SRS-pos frequency hopping more easily. However, the SRS for MIMO does not support partially overlapping frequency hopping patterns in the frequency range under discussion. Therefore, even if we consider this as the basis for the configuration, we need to take this into account.

Secondly, consider the SRS-pos configuration as the basis. In this case, there is no pre-defined configuration parameter for frequency hopping. Therefore, new frequency hopping configuration should be designed from the scratch. However, there may be an advantage in building a more flexible frequency hopping configuration so that various issues that arise from performing SRS-pos frequency hopping outside the active UL BWP can be handled.

Proposal 3: For configuration of frequency hopping for SRS-Pos, the followings can be considered:

- Alt. 1) based on the frequency hopping principles for SRS-MIMO resource configuration

- Alt. 2) new frequency hopping configuration dedicated for the SRS-pos resource configuration

To satisfy the target performance accuracy, frequency hopping that includes a frequency range larger than the active UL BWP has been introduced. However, considering the existing behavior of UEs where all transmissions occur only within the active UL BWP, it is necessary to consider the relationship between the active UL BWP and frequency hopping configuration, and two approaches can be considered.

Firstly, joint configuration for SRS-Pos and frequency hopping can be considered. In this case, it can be configured to be dependent on the active UL BWP since the SRS-Pos configuration is BWP-specific parameter. This has the advantage that the first hop transmission always exists within the active UL BWP, which is in line with the legacy behavior of UEs. In such a case, since the location of the active UL BWP changes with BWP change, it would be appropriate to express the frequency location of each hop relative to the position of the first hop. As detail method, for example, a pre-defined frequency hopping configuration ID can be associated with legacy SRS-pos resource configuration and RedCap UEs can perform SRS-pos Tx frequency hopping using the configuration information.

For the other case, separate configurations for SRS-Pos and frequency hopping can be considered. While maintaining configuration of legacy SRS-pos associated with BWP configuration as it is, newly define frequency hopping configuration separate from the BWP configuration can be considered. In this case, since the location of the active UL BWP is independent from the SRS-Pos for frequency hopping, it would be appropriate to express the frequency location of each hop by absolute value of frequency position.

Proposal 4: As parameter configuration method for determining the frequency hop of the SRS for positioning, the following alternatives can be considered.

- Alt. 1) Separately configured with SRS-Pos configuration.

- Alt. 2) Jointly configured with SRS-Pos configuration.

Details of configuration

When the transmission of frequency hopped SRS-pos is performed inter-slot, i.e., single SRS transmission occasion in single slot, time duration to complete the frequency hopped SRS-pos will be identical to the number of hops. During those time duration, transmission of other signals/channels will be dropped or postponed which lead latency increment. To avoid it, shorten the frequency hopping duration should be considered. One possible approach will be enabling multiple SRS transmission in a single slot, i.e., the intra-slot + inter-slot UL SRS-pos Tx hopping mechanism so that the UL SRS-pos resource hops can be connected contiguously between inter-slots during frequency hopping. The example is shown in Figure 1.

Depending on the number of hops and the number of symbols per hop, there could be a case where a hop might coincide with the slot boundary. It would lead problems such as partial SRS resource transmission by symbol-level drop and sequence mapping of such partial SRS resource, it is undesirable to allow the cross over the slot boundary.

Proposal 5: The intra-slot + inter-slot SRS-pos Tx hopping mechanism should be considered.

Figure 1. Example of intra-slot + inter-slot contiguous SRS-pos Tx hopping

Associating the frequency hopping related configuration parameters to every SRS-pos resource configuration can cause parameter overhead problem. The mechanism that after allocating predefined frequency hopping related RRC configuration parameters as configuration ID, directly indicate the frequency hopping configuration ID when indication of frequency hopping is necessary can be considered. Therefore it can be reduced by switching the configuration of frequency hopping using the configuration ID.

It needs to be specified how to configure partial overlapped frequency hopping pattern of SRS-pos if supported. It is natural that overlapped size in frequency domain is configured in PRB units, and it can be considered the overlapped size between hops can be same or different. However to allow different amount of frequency overlapping configuration between hops, the overlapped size for each hop should be individually configured for each hops which can lead configuration overhead and the performance gain due to it is questionable. Rather than that, configuring the common overlapped size for each hop in one SRS-pos resource is desirable for simplicity.

Proposal 6: For SRS for positioning frequency hopping configuration mechanism, following should be specified:

- Switching mechanism for SRS-pos frequency hopping configuration

- Configuration of overlap size

Last but not least, the question of whether and how to activate or deactivate frequency hopping for SRS-Pos should be discussed. It can be argued that the frequency hopping for SRS-Pos is configured by RRC. However, from the perspective of the transmitting UE, it could cause latency due to the cancellation or postponement of uplink transmission to the serving cell that cannot be resolved until deactivated. From a resource management perspective, the time and frequency resources that the UE uses to transmit frequency hopped SRS-Pos are considered to be occupied by that UE until it is deactivated. Moreover, it could cause interference in the network, which also cannot be resolved until the feature is deactivated. In a nutshell, the frequency hopping feature of SRS-Pos should support activation and deactivation, and dynamic indication should be supported considering the aforementioned reasons.

Proposal 7: Support frequency hopping of SRS-pos is activated/deactivated by MAC-CE or DCI.

Collision handling

The collisions here is defined as cases where UL Tx to be transmitted to the gNB and frequency hopping SRS-Pos mentioned earlier are indicated in the same time resource, or UL Tx is indicated during the time period outside of the active UL BWP for continuous transmission of SRS-Pos, even if the UE does not actually transmit the frequency hopping SRS-Pos. For collision handling, introducing UL time domain window to ensure frequency hopping SRS-Pos transmission or collision handling rule with other signal/channel for FDD and HD-FDD can be considered. Both cases are discussed in detail respectively.

UL time domain window - Transmission Gap (TG)

Collision with other signals/channels would occur more frequently due to UL SRS-pos frequency hopping operation outside active BWP. If the SRS-pos resource has lower priority than other signals/channels, it may cause positioning performance degradation, and if it has higher priority, it may adversely affect other data transmission performance. Therefore, similar to measurement gap (MG) for positioning for transmission of DL PRS, transmission gap (TG) for positioning for UL SRS-pos that does not expect transmission and reception of other signals/channels can be consider to prevent the collisions.

Two options methods can be considered for setting TG: To define a separate TG configuration, request, and indication method; Associating the TG configuration to SRS-pos resource configuration. The first method is to apply the TG configuration, request, and indication method similarly to the MG setting, request, and indication method currently supported by NR Rel-17. It can be configured through RRC message, requested through UL MAC CE by UE, NRPPa by LMF, and indicated through DL MAC CE. In addition, similar to the priority rule of MG, TG does not expect Tx/Rx of other DL or UL signals/channels within the time window, and Tx hopping of resources can be allowed even in outside active UL BWP.

The second method is to associate the configuration of the TG with the RRC configuration for SRS-pos resource frequency hopping to be specified later in NR Rel-18 and follow the request and instruction of the TG to the request and instruction of the SRS-pos resource frequency hopping operation. If TG is configured and SRS-pos frequency hopping is performed, collision with other DL signal/channels or UL signal/channels can be completely prevented, and transmission of SRS-pos resource is guaranteed. Also high positioning accuracy performance can be guaranteed.

Collision handling rule with other signal/channel for FDD and HD-FDD

Applying the existing SRS-pos collision handling rule to the SRS-p frequency hopping operation may occur several problems due to the characteristics of the frequency hopping and RedCap UEs. First, when the frequency hopping of SRS-p for RedCap UEs is supported, due to the limitation of the BW capability of the RedCap UEs, the UEs may need to perform operations including RF retuning at each hop change location. In the case of general signal/channels to which frequency hopping is not applied, the location of the BW where transmission/reception is performed may not be the same as the location on the frequency axis of the hop where frequency hopping is performed. In this case, the dropping rule of SRS-p performed at the symbol level may cause frequent RF retuning operations of the RedCap UEs, which may cause complexity and power consumption of the UEs.

Second, for high positioning accuracy performance it may be advantageous to arrange continuous RSs in the frequency domain. But if some symbols of SRS-p are dropped and SRS-p are discontinuously transmitted, positioning performance may be significantly deteriorated. Considering those, when a collision between SRS-p and another signal/channel or a collision between SRS-p occurs when transmission and reception of SRS-p to which frequency hopping is applied is performed, the enhanced collision handling methods must be considered.

(a)                                                                                   (b)

Figure 2. Examples of dropping rule for SRS-pos frequency hopping

To determine the collision handling rule, a priority rule and a dropping rule should be specified. The priority rule can be defined about the relationship between SRS-pos frequency hopping and other SRS-pos-frequency hopping/SRS-pos /SRS-MIMO with the same resource type, between SRS-pos frequency hopping and SRS-MIMO/SRS-pos /SRS-posfrequency hopping with different resource types, and between SRS-pos frequency hopping and PUCCH/PUSCH. When collision between the SRS-pos frequency hopping and other signals/channels occurs, dropping is to be performed and the following options can be considered and specified: first, dropping only a hop including the positions of overlapped symbols (Figure 2 (a)) can be considered. Alternatively, dropping the hop where overlapped symbol(s) located in and all previous or subsequent hops of it. (Figure 2 (b)) can be considered. There could be various types of dropping rules, which required to be discussed. It should be noted that, the collision handling rule for HD-FDD is also should be considered since support of FD-FDD is optional for RedCap UE.

Proposal 8: Consider following for handling of collision between frequency hopping SRS transmission and other UL transmission

- Opt. 1) UL time domain window where UE is expected to transmit only SRS for positioning

- Opt. 2) Priority rule between SRS for positioning and other UL transmission

Performance evaluation

Simulation assumptions

To investigate the positioning accuracy for the different mobility cases and different frequency hopping patterns, the TDoA-based positioning was evaluated in the InF-SH scenario. The basic assumptions are:

- Modified TDoA Chan algorithm with selection of the several TRP subsets and weighting-based on residuals is used for absolute position estimation

- RSTDs between UE and TRPs are measured in the baseband with the super-resolution MUSIC algorithms or Peak algorithms for every pair of stations

- Since baseline MUSIC algorithms implementation requires contiguous samples, the Reference Signals grid on the consecutive symbols is united in the single symbol with full band fill. To do this of UL SRS, the comb factor should be equal to repetition parameter.

Coherent stitching of hops

- Before processing MUSIC or Peak by the algorithm, all hops signals are combined and form a full bandwidth.

- Phase continuity between hops is assumed, although Doppler-induced phase drift are not compensated.

Non-coherent combining of the correlation matrix R or channel impulse response (CIR)

- For each hop, correlation matrix or CIR is evaluated separately and averaged over multiple slots/hops.

- No phase continuity is assumed, and random phase shift is introduced between hops.

Non-coherent combining of final distance estimates

- For each hop, final distance estimation is evaluated and then averaged over multiple slots/hops.

- No phase continuity is assumed, and random phase shift is introduced between hops.

Formal simulation assumptions are summarized in Table 1, while Figure 3 shows visualization of scenarios with frequency hopping.

Table 1. Simulation assumptions

| Parameter | Values |
| --- | --- |
| Carrier frequency | 3.5 GHz |
| Scenario | InF-SH |
| Transmission Bandwidth | 86.4 MHz: (240 RB) |
| Type of reference signal | UL SRS |
| Frequency hopping pattern configuration | 17.3MHz(48RBs) x 5slots (CSRS= 55, BSRS = 1, Bhop = 0) |
| SCS | 30 kHz |
| TX/RX Antenna configuration | [1,1,1,1,1] |
| Deployment | Total number of UEs = 500 Vehicle speed = 3 km/h; 30 km/h; 60 km/h; |
| Distance algorithm | MUSIC/ Peak |
| Positioning algorithm | TDoA(Chan) |
| Phase between hops | Uniform random, (0, 2π) |
| Phase compensation between hops | Ideal compensation |
| Types of joint processing hops | - Coherent stitching of hops - Non-coherent combining of the correlation matrix R or channel impulse response(CIR) - Non-coherent combining of final distance estimates |

Figure 3. Visualization of scenarios with frequency hopping

Evaluation results

Full set of simulation result summarizing the performance of three considered combining techniques for different mobility scenarios is shown in Table 2 for super-resolution MUSIC TOA estimation algorithm and in Table 3 for the peak approximation ToA estimation.

Table 2. Horizontal accuracy (MUSIC algorithm)

| Test case assumptions Common: InF-SH, FR1, MUSIC,TDOA,UL SRS |  | Error 50% |  | Error 67% |  | Error 80% |  | Error 90% |  | Whether meet the requirement of commercial use cases(<3m) |  | Whether meet the requirement of IIoT use cases(<1m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 86.4MHz x 1 slots, V=3km/h |  | 0.03 |  | 0.05 |  | 0.08 |  | 0.13 |  | Yes |  | Yes |  |
| Coherent stitching of hops |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.3MHz x 5 slots, V=3km/h |  | 0.05 |  | 0.07 |  | 0.1 |  | 0.17 |  | Yes |  | Yes |  |
| 17.3MHz x 5 slots, V=30km/h |  | 0.32 |  | 0.37 |  | 0.43 |  | 0.55 |  | Yes |  | Yes |  |
| 17.3MHz x 5 slots,V=60km/h |  | 0.93 |  | 1.35 |  | 1.94 |  | 2.84 |  | No |  | No |  |
| Non-coherent combining of the CM |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.3MHz x 5 slots, V=3km/h |  | 0.05 |  | 0.07 |  | 0.12 |  | 0.19 |  | Yes |  | Yes |  |
| 17.3MHz x 5 slots, V=30km/h |  | 0.05 |  | 0.08 |  | 0.12 |  | 0.2 |  | Yes |  | Yes |  |
| 17.3MHz x 5 slots, V=60km/h |  | 0.05 |  | 0.09 |  | 0.14 |  | 0.23 |  | Yes |  | Yes |  |
| Non-coherent combining of final distance estimates |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.3MHz x 5 slots, V=3km/h |  | 0.16 |  | 0.26 |  | 0.4 |  | 0.68 |  | Yes |  | Yes |  |
| 17.3MHz x 5 slots, V=30km/h |  | 0.17 |  | 0.28 |  | 0.42 |  | 0.72 |  | Yes |  | Yes |  |
| 17.3MHz x 5 slots, V=60km/h |  | 0.19 |  | 0.3 |  | 0.48 |  | 0.8 |  | Yes |  | Yes |  |
| Table 3. Horizontal accuracy (Peak algorithm) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test case assumptions Common: InF-SH, FR1, Peak,TDOA,UL SRS |  | Error 50% |  | Error 67% |  | Error 80% |  | Error 90% |  | Whether meet the requirement of commercial use cases(<3m) |  | Whether meet the requirement of IIoT use cases(<1m) |  |
| 86.4MHz x 1 slots, V=3km/h |  | 0.14 |  | 0.2 |  | 0.29 |  | 0.45 |  | Yes |  | Yes |  |
| Coherent stitching of hops |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.3MHz x 5 slots, V=3km/h |  | 0.15 |  | 0.21 |  | 0.31 |  | 0.49 |  | Yes |  | Yes |  |
| 17.3MHz x 5 slots, V=30km/h |  | 0.37 |  | 0.44 |  | 0.52 |  | 0.65 |  | Yes |  | Yes |  |
| 17.3MHz x 5 slots,V=60km/h |  | 3.1 |  | 5.5 |  | 8.1 |  | 11.4 |  | No |  | No |  |
| Non-coherent combining of the CIR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.3MHz x 5 slots, V=3km/h |  | 0.23 |  | 0.36 |  | 0.56 |  | 0.94 |  | Yes |  | Yes |  |
| 17.3MHz x 5 slots, V=30km/h |  | 0.23 |  | 0.36 |  | 0.6 |  | 0.99 |  | Yes |  | Yes |  |
| 17.3MHz x 5 slots, V=60km/h |  | 0.24 |  | 0.37 |  | 0.6 |  | 0.99 |  | Yes |  | Yes |  |
| Non-coherent combining of final distance estimates |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17.3MHz x 5 slots, V=3km/h |  | 0.58 |  | 0.9 |  | 1.35 |  | 2.3 |  | Yes |  | No |  |
| 17.3MHz x 5 slots, V=30km/h |  | 0.58 |  | 0.92 |  | 1.38 |  | 2.4 |  | Yes |  | No |  |
| 17.3MHz x 5 slots, V=60km/h |  | 0.58 |  | 0.92 |  | 1.39 |  | 2.3 |  | Yes |  | No |  |

Baseline case: V = 3 km/h

Figure 4. MUSIC/Peak performances for 5 hopsfor different types of joint processing

The coherent stitching and the intermediate metrics (correlation matrix or CIR) accumulation shows much better performance than accumulation of the final distance metrics. The difference is especially significant in case of using low complex algorithm.

In low mobility case, the non-coherent combining of the correlation matrix shows comparable performance to the coherent combining. However, when low complexity algorithm is used, coherent combining outperforms others in a low mobility scenario.

Observation 1: Performance non-coherent combining based on per hop measurement shows significantly worse than coherent stitching and non-coherent combining of the correlation matrix or CIR.

Mobility dependency: v=3-300 km/h

MUSIC algorithm:

| Coherent stitching of hops | Non-coherent combining of the CM | Non-coherent combining of final distance estimates |
| --- | --- | --- |

Figure 5. Performances of three combining methods (w/ MUSIC algorithm) for different number of hops

Peak algorithm:

| Coherent stitching of hops | Non-coherent combining of the CM | Non-coherent combining of final distance estimates |
| --- | --- | --- |

Figure 6. Performances of three combining methods (w/ Peak algorithm) for different number of hops

Under the phase continuity assumption, degradation strongly depends on the user mobility.

For the low mobility case (3km/h), although performance depends on algorithm used, non-coherent accumulation of the correlation matrix or CIR shows very little degradation in comparison with the ideal coherent/full BW case.

Observation 2: For low mobility case (3km/h), degradation of performance between coherent combining and non-coherent combining is negligible over 5 hops when using the MUSIC algorithm.

While coherent stitching cannot meet the performance requirement for the velocity 60km/h, non-coherent combining of the correlation matrices in the MUSIC algorithm satisfies the performance requirements for the velocities up to 60 km/h (even up to 300km/h).

Observation 3: As speed increased, coherent stitching experiences drastic performance loss due to uncompensated phase drift, while non-coherent algorithms demonstrate robust behaviour.

Referring to our evaluation results, in the case of low mobility, non-coherent combining according to the algorithm used by the receiver is close to the performance of coherent combining assuming ideal phase compensation (purpose of partially overlapped frequency hopping structure). Moreover, as speed increased, coherent stitching experiences drastic performance loss due to uncompensated phase drift, while non-coherent algorithms demonstrate robust behaviour. Therefore, non-coherent combining that does not require overlapping should also be considered.

Proposal 9: Support both of partially overlapped frequency hopping structure and non-overlapped one.

Conclusion

Based on the above discussion, the following proposals and observations are provided.

Proposal 1: Deprioritize DL-PRS frequency hopping outside MG.

Proposal 2: Following parameters should be included in SRS-pos frequency hopping configuration

- Number of hops, bandwidth of each hop, switching gap

Proposal 3: For configuration of frequency hopping for SRS-Pos, the followings can be considered:

- Alt. 1) based on the frequency hopping principles for SRS-MIMO resource configuration

- Alt. 2) new frequency hopping configuration dedicated for the SRS-pos resource configuration

Proposal 4: As parameter configuration method for determining the frequency hop of the SRS for positioning, the following alternatives can be considered.

- Alt. 1) Separately configured with SRS-Pos configuration.

- Alt. 2) Jointly configured with SRS-Pos configuration.

Proposal 5: The intra-slot + inter-slot SRS-pos Tx hopping mechanism should be considered.

Proposal 6: For SRS for positioning frequency hopping configuration mechanism, following should be specified:

- Switching mechanism for SRS-pos frequency hopping configuration

- Configuration of overlap size

Proposal 7: Support frequency hopping of SRS-pos is activated/deactivated by MAC-CE or DCI.

Proposal 8: Consider following for handling of collision between frequency hopping SRS transmission and other UL transmission

- Opt. 1) UL time domain window where UE is expected to transmit only SRS for positioning

- Opt. 2) Priority rule between SRS for positioning and other UL transmission

Proposal 9: Support both of partially overlapped frequency hopping structure and non-overlapped one.

Observation 1: Performance non-coherent combining based on per hop measurement shows significantly worse than coherent stitching and non-coherent combining of the correlation matrix or CIR.

Observation 2: For low mobility case (3km/h), degradation of performance between coherent combining and non-coherent combining is negligible over 5 hops when using the MUSIC algorithm.

Observation 3: As speed increased, coherent stitching experiences drastic performance loss due to uncompensated phase drift, while non-coherent algorithms demonstrate robust behaviour.

Reference

- RP-223549, “New WID on Expanded and Improved NR Positioning”

- 3GPP TR 38.859, “Study on expanded and improved NR positioning”