3GPP TSG RAN WG1 #112bis-e		                     R1- 2303748

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.9.1

Source:	LG Electronics

Title:	Discussion on coverage enhancement for NR NTN

Document for:	   Discussion and Decision

- Introduction

In RAN1 #112 meeting, the following agreements and observation were made for coverage enhancement of NR NTN WI as in [1]:

Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit Working assumption For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE. - Option A: PRACH preamble and/or occasion - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report] - FFS: whether or not indication of repetition factor is assumed - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings - Option B: Higher layer signaling in Msg3 PUSCH - FFS: which signaling is used - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2. - Option C: Physical layer signaling in Msg3 PUSCH - FFS: which signaling is used, e.g. DMRS ports Agreement For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB. - Alt 1: Field in DCI scheduling the Msg4 PDSCH - Alt 1-1: One or two bits of the existing field - Alt 1-1a: MCS field - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource) - Alt 1-1c: HARQ process number filed - Alt 1-1d: DAI field - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field - Alt 1-2: New field with one or two bits - Alt 2: Field in DCI scheduling Msg3 PUSCH - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor - Note: it is assumed that there is impact on DCI design - Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc. - Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design) Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

In this contribution, we discuss on the PUCCH enhancement for Msg4 HARQ-ACK and DMRS bundling for PUSCH for Rel-18 NR NTN enhancement.

- Discussion

PUCCH enhancement for Msg4 HARQ-ACK

RAN1 agreed to support the PUCCH repetition for Msg4 HARQ-ACK in RAN1 #110b-e meeting. However, in order for the UE to perform PUCCH repetition, it is necessary to provide the repetition factor to the UE. Therefore, as a next step, RAN1 need to discuss how to provide the repetition factor of PUCCH for Msg4 HARQ-ACK. As one option, it can be considered to provide a cell-specific repetition factor to the UE via SIB. Although this option is the simplest way, but because each UE may have different channel environments, it may not be efficient to provide cell-specific repetition factor in terms of network resource management.

As another option, it can be considered that gNB can provide the set of repetition factors via SIB, and the exact repetition factor can be configured by another indication field. Here, the indication field may be a specific DCI field of DCI format 1_0 that schedules Msg4 PDSCH (e.g., MCS field or DAI field), or may be included in the Msg4 PDSCH itself.

Moreover, it can be also considered that the repetition factor of PUCCH for Msg4 HARQ-ACK can be provided simultaneously via an indication field that provides the repetition factor of Msg3 PUSCH. In this case, the repetition factors of PUCCH for Msg4 HARQ-ACK may be set independently of the repetition factors of Msg3 PUSCH, and it may be considered to pair each repetition factor so that gNB can provide each repetition factor using a single indication field.

Proposal 1. Repetition factor(s) of PUCCH for Msg4 HARQ-ACK is provided via SIB and following options can be considered.

- Option 1: Cell specific single value of repetition factor can be provided via SIB.

  - The value is one of {2, 4, 8}

- Option 2: Set of repetition factors is provided via SIB.

  - The value is one of {1, 2, 4, 8}

  - Option 2-1: Exact repetition factor can be configured by DCI field (e.g., MCS field or DAI field) scheduling the Msg. 4 PDSCH

  - Option 2-2: Implicit mapping between PUCCH for Msg4 HARQ ACK repetition factor and indication of Msg. 3 PUSCH repetition

In RAN1 #111 meeting, a working assumption was made on how to configure the PUCCH repetition factors as captured below.

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - One or more repetition factors may be configured via SIB - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor - FFS: whether UE requests repetition or indicates repetition capability - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB - FFS: whether UE requests repetition or indicates repetition capability - FFS: whether repetition factor is indicated by UE - FFS: UE behavior when repetition factor is not configured via SIB - FFS: whether one or more UE capabilities are needed for the above is for further discussion

Regarding to the second last FFS point of this working assumption, the UE behavior when repetition factor is not configured via SIB can be further discussed. In general, since the PUCCH repetition scheme is an optional feature, so the PUCCH repetition is desirable to operate when additional signaling (e.g., one or multiple repetition factor(s)) is provided via SIB. In other words, if additional signaling for PUCCH repetition is not provided via SIB, transmitting a single PUCCH (i.e., legacy UE behavior) is the simplest and the most natural UE behavior. Therefore, we propose that if the PUCCH repetition factor is not configured via SIB, the UE shall determine the repetition factor as 1 (i.e., a single PUCCH transmission).

Proposal 2. If the repetition factor is not configured via SIB, the UE shall determine the repetition factor as 1 (i.e., a single PUCCH transmission).

In addition, in the Option 2 of Proposal 1, multiple repetition factors would be simultaneously supported in a specific cell. Here, gNB should allocate PUCCH resources that may be used by each repetition factor. So, if the legacy behavior is reused without any changes, the PRB location based on PRB offset (Table 9.2.1-1 in TS38.213 [2]) must be applied equally regardless of the repetition factor. Thus, gNB is forced to allocate resources for PUCCH repetition using different time domain resources (because sequence domain resources are limited), resulting in an increase in the average time delay of UEs in the cell.

Therefore, in order to handle this issue, if multiple repetition factors are simultaneously supported in a specific cell, additional PRB offsets can be considered for each PUCCH repetition factors. To be specific, the additional PRB offsets can be configured by gNB, or can be predefined for each repetition factors in specification.

Proposal 3. Additional PRB offsets can be supported for each PUCCH repetition factors for Msg4 HARQ-ACK.

In RAN #99 meeting, the terminology of PUCCH for Msg4 HARQ-ACK was discussed. During the meeting, several companies tried to clarify the terminology, but failed to reach a consensus. However, if RAN1 identifies whether this issue led to any missing cases or coverage holes, it was proposed to discuss this issue again at the RAN #100 meeting. Therefore, we think this issue should be discussed as a high priority in this meeting.

If PUCCH repetition for Msg4 HARQ-ACK is configured, it is preferred that other PUCCHs also perform repeated transmissions when dedicated PUCCH resource configuration is not provided. This is because if the PUCCH repetition is only supported for Msg4 HARQ-ACK, and is not supported for PDSCHs received before acquiring the dedicated PUCCH resource sets, the PUCCH for Msg4 HARQ-ACK may be received well, but the PUCCH for other PDSCHs may not be received well at the gNB. After all, we think it is a missing case or a coverage hole, which is undesirable UE/gNB behavior.

In summary, it is necessary to clarify the terminology of PUCCH repetition for Msg4 HARQ-ACK, and we prefer to clarify the terminology ‘enhancement of PUCCH for Msg4 HARQ-ACK’ as ‘enhancement of PUCCH resource sets to be used when dedicated PUCCH resource configuration is not provided’.

Proposal 4. For coverage enhancement of PUCCH for Msg4 HARQ-ACK, RAN1 discuss enhancement for PUCCH to be used when dedicated PUCCH resource configuration is not provided.

In RAN1 #112 meeting, a working assumption was made on how to indicate the repetition request or capability report by UE. Regarding this working assumption, we think it can be operated based on RACH partitioning which is RAN2’s Rel-17 WI. This is because, RACH partitioning is a technique in which a gNB informs which RACH resource (e.g., preamble indexes) a UE should select when the UE has a specific feature or when the UE tries to request the feature. Thus, in order to allocate the RACH resources for PUCCH repetition for Msg4 HARQ-ACK through RACH partitioning, it is necessary to introduce a PUCCH repetition for Msg4 HARQ-ACK feature first.

First of all, it can be considered to introduce a new parameter (e.g., pucch-Repetitions-r18) for the PUCCH repetition for Msg4 HARQ-ACK feature into the SIB (e.g., in FeatureCombination-r17). To be specific, one of the spare parameters in FeatureCombination-r17 can be replaced with a new parameter for the PUCCH repetition for Msg4 HARQ-ACK feature as shown in Figure 1.

FeatureCombination-r17 ::= SEQUENCE {

redCap-r17                 ENUMERATED {true}                          OPTIONAL,  -- Need R

smallData-r17              ENUMERATED {true}                          OPTIONAL,  -- Need R

nsag-r17                   NSAG-List-r17                              OPTIONAL,  -- Need R

msg3-Repetitions-r17       ENUMERATED {true}                          OPTIONAL,  -- Need R

pucch-Repetitions-r18      ENUMERATED {true}                          OPTIONAL,  -- Need R

spare3                     ENUMERATED {true}                          OPTIONAL,  -- Need R

spare2                     ENUMERATED {true}                          OPTIONAL,  -- Need R

spare1                     ENUMERATED {true}                          OPTIONAL   -- Need R

}

Figure 1. Example of new parameter for PUCCH repetition for Msg4 HARQ-ACK in FeatureCombination-r17.

On the other hands, it can be considered to reuse one of the legacy parameters in FeatureCombination-r17 as a PUCCH repetition for Msg4 HARQ-ACK feature. For example, since PUCCH repetition for Msg4 HARQ-ACK and Msg3 PUSCH repetition would generally be required at the same time, “msg3-Repetitions-r17” can be used to indicate the PUCCH repetition for Msg4 HARQ-ACK feature in addition to the existing Msg.3 PUSCH repetition feature.

In addition, gNB can set a preamble index group corresponding to the PUCCH repetition for Msg4 HARQ-ACK feature, and the UE with the PUCCH repetition for Msg4 HARQ-ACK feature can perform a repetition request or capability report by using one preamble index within configured preamble index group. As a result, we prefer Option A (i.e., PRACH preamble and/or occasion) in the working assumption in RAN1 #112.

Proposal 5. For PUCCH repetition for Msg4 HARQ-ACK, PRACH resources (e.g., RACH preambles or RACH occasion) is considered as a container of the repetition request or capability report indicated by UE. One of the following options can be supported for feature combination.

- Option 1: Introduce a new parameter (e.g., pucch-Repetitions-r18) for the PUCCH repetition for Msg4 HARQ-ACK feature into the SIB (e.g., in FeatureCombination-r17).

- Option 2: “msg3-Repetitions-r17” in FeatureCombination-r17 is used as a PUCCH repetition for Msg4 HARQ-ACK feature in addition to the existing Msg3 PUSCH repetition feature.

Moreover, regarding the PUCCH resource sets in Table 9.2.1-1 in TS38.213, the PUCCH format 1 with 14 OFDM symbols was considered as an enhancement target of PUCCH for Msg4 HARQ-ACK. Furthermore, if PUCCH format 0, PUCCH format 1 with 4 OFDM symbols, or PUCCH format 1 with 10 OFDM symbols are considered for PUCCH repetition, additional specification works may be required, e.g., defining additional starting OFDM symbol indexes for each PUCCH format. To be specific, Figure 2 shows an example of introducing additional starting OFDM symbol indexes for PUCCH format 1 with 4 OFDM symbols when repetition factor is 8. In this example, we assume that OFDM symbol index 2 and index 6 are additional starting OFDM symbol indexes for PUCCH format 1 with 4 OFDM symbols.

Figure 2. Example of multiple PUCCH transmissions using PUCCH format 1 with 4 OFDM symbols

So, to avoid doing this additional specification works, if the PUCCH repetition for Msg4 HARQ-ACK is supported in Rel-18 NR NTN, the higher layer parameter “pucch-ResourceCommon” should indicate one of the indexes consisting of PUCCH format 1 with 14 OFDM symbols (i.e., from index 11 to 15) among 16 indexes in Table 9.2.1-1 in TS38.213. In other words, the NTN UE expects that the higher layer parameter "pucch-ResourceCommon" to indicate one of indexes 11 to 15 in Table 9.2.1-1 in TS38.213, if the PUCCH repetition for Msg4 HARQ-ACK is configured.

Proposal 6. The NTN UE expects the parameter "pucch-ResourceCommon" to indicate one of indexes 11 to 15 in Table 9.2.1-1 in TS38.213, if the PUCCH repetition for Msg4 HARQ-ACK is configured.

Alternatively, it is possible to consider introducing a new table consisting only of PUCCH format 1 with 14 OFDM symbols for Rel-18 NR NTN UE. For example, it is possible to consider creating a new table consisting of 8 indexes by adding 3 new indexes to the existing indexes 11-15 in Table 9.2.1-1 in TS38.213. Or, it is also possible to consider creating a new table consisting of 16 indexes by adding 11 new indexes to the existing indexes 11-15 in Table 9.2.1-1 in TS38.213. If this new table is introduced, it is also necessary to determine which legacy or new table the UE should refer to.

Proposal 7. It is possible to consider creating a new PUCCH resource set table consisting only of PUCCH format 1 with 14 OFDM symbols for Rel-18 NR NTN UE.

In RAN1 #112 meeting, it was discussed whether intra-slot frequency hopping for PUCCH repetition for Msg4 HARQ-ACK is automatically supported or not. We think it is natural to support intra-slot frequency hopping for PUCCH repetition for Msg4 HARQ-ACK since this scheme was already supported for a single PUCCH transmission for Msg4 HARQ-ACK. Anyway, if explicit agreement is necessary for clarification, we can support that intra-slot frequency hopping for PUCCH repetition for Msg4 HARQ-ACK.

Proposal 8. Support the intra-slot frequency hopping for PUCCH repetition for Msg4 HARQ-ACK.

Moreover, because it is advantageous in terms of frequency diversity, the inter-slot frequency hopping was introduced in Msg3 PUSCH repetition in Rel-17 NR CE. Similarly, we think it is desirable that inter-slot frequency hopping is supported for PUCCH repetition for Msg4 HARQ-ACK. Therefore, it is possible to consider introducing additional higher layer parameter to indicate enabling/disabling the inter-slot frequency hopping for PUCCH repetition for Msg4 HARQ-ACK.

Proposal 9. Introduce higher payer parameter to indicate the inter-slot frequency hopping for PUCCH repetition for Msg4 HARQ-ACK.

In addition, if the inter-slot frequency hopping is supported and enabled, RAN1 need to further discuss on which slots are applied. As one option, it can be considered that inter-slot frequency hopping operates between the first half slots and the last half slots of the total PUCCH repetition slots. As another option, it can be considered that inter-slot frequency hopping operates between even-numbered slots and odd-numbered slots of the total PUCCH repetition slots.

Proposal 10. Further discuss on which slots are applied if the inter-slot frequency hopping for PUCCH repetition for Msg4 HARQ-ACK is enabled.

- Option 1: Operates between the first half slots and the last half slots

- Option 2: Operates between even-numbered slots and odd-numbered slots

Furthermore, in RAN1 #112 meeting, a working assumption was made regarding the RSRP threshold values for PUCCH repetition for Msg4 HARQ-ACK. In this working assumption, the RSRP threshold value can be provided in three cases as follows:

- Case 1: The case when the configured RSRP threshold is not maximum value

- Case 2: The case when the configured RSRP threshold is maximum value.

- Case 3: The case when the RSRP threshold is not configured.

In addition, the RSRP threshold for PUCCH repetition for Msg4 HARQ-ACK can be defined among two alternatives as follows:

- Alt A: The same RSRP threshold as R17 Msg3 repetition is used.

- Alt B: New RSRP threshold is introduced.

Here, Table 1 shows that whether the combinations of the above cases and alternatives can be supported at the same time.

Table 1. Example of combinations between RSRP threshold values and its definition.

|  | Alt A: The same RSRP threshold as R17 Msg3 repetition is used | Alt B: New RSRP threshold is introduced. |
| --- | --- | --- |
| Case 1: The case when the configured RSRP threshold is not maximum value | Possible | Possible |
| Case 2: The case when the configured RSRP threshold is maximum value | Possible | Possible |
| Case 3: The case when the RSRP threshold is not configured | Impossible | Possible |

As shown in Table 1, most combinations can be supported at the same time, but only one combination (i.e., combination between Case 3 and Alt A) may not be possible. In other words, it seems to be impossible not to configure the corresponding RSRP threshold parameter while reusing the legacy RSRP threshold parameter (i.e., rsrp-ThresholdMsg3-r17).

Regarding this issue, it can be discussed first whether to reuse the legacy RSRP threshold or to introduce new RSRP threshold. We think that it is inefficient to provide multiple RSRP thresholds with similar functions to determine whether to perform repeated transmissions of UL channels included in RACH procedure in terms of signaling overhead at gNB side. Therefore, we prefer to support to reuse the same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17). Also, we do not support the case when the RSRP threshold is not configured as well.

Observation 1. If the same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used for PUCCH repetition for Msg4 HARQ-ACK, it is impossible to support the case when the RSRP thresholds is not configured.

Proposal 11. For PUCCH repetition for Msg4 HARQ-ACK, the same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used.

Proposal 12. Do not support the case when the RSRP threshold is not configured for PUCCH repetition for Msg4 HARQ-ACK.

DMRS bundling for PUSCH

The repetition and DMRS bundling were considered as key features for PUSCH to meet the coverage requirement under NTN environment. From RAN1 #110b-e meeting, DMRS bundling for PUSCH has been discussed and it was concluded that further discussion is necessary for following two aspects: 1) phase difference due to timing drift and/or Doppler shift and 2) an event which causes power consistency and phase continuity.

According to TS 38.101-1, UE needs to maintain phase differences within 25 degrees between two consecutive repetitions during the DMRS bundling window less than 8 slots. Since phase consistency can suffer from carrier frequency noise and timing drift, it should be carefully investigated with the consideration of NTN environment. In NTN environment, uplink timing should be adjusted according to the movement of UE and satellite, however, for DMRS bundling, TA adjustment may be limited due to maintaining phase continuity. This would make additional timing drifts comparing to TN UE, so it becomes more difficult for UE to maintain phase during DMRS bundling window.

According to companies’ contribution in the last meeting, it seems common understanding that the phase difference limit is bottleneck of DMRS bundling. However, the problem is not a newly arisen one. The limitation has been existed, and UE has been handled those problems. In NTN, this problem is magnified as it is highly dependent on NTN platform, rather than whether the UE can manage it or not. It is highly recommended to consider UE capabilities regarding NTN platforms and those properties.

Therefore, we propose to introduce new UE capabilities for the NTN environment, distinct from the current UE capabilities for DMRS bundling. As mentioned above, the need for UL power control and TA pre-compensation is relevant to the relative distance/movement between the UE and gNB. Considering this characteristic, introducing DMRS bundling capabilities according to a type of NTN platforms may be considered.

Proposal 13: For DMRS bundling in NTN environment, introduce a new UE capability based on types of NTN platforms.

On the other hand, the fundamental design principle of Rel-17 DMRS bundling can be adopted for NTN environment. However, there could be NTN-specific issues related to TA, as UE shouldn’t change TA and transmit power at least in a DMRS bundling window in order to maintain phase continuity and power consistency. The existing principle of DMRS bundling window, known as TDW, is designed to ensure phase continuity and power consistency. Therefore, it is highly recommended to use same concept to support PUSCH DMRS bundling for NTN environment.

Proposal 14. Support DMRS bundling for PUSCH coverage enhancement in Rel-18 NR NTN.

- The design of DMRS bundling in Rel-17 CovEnh is a baseline as following:

  - UE is expected to maintain its timing advance at least during TDW

  - Support keeping total TA, which includes both common TA and UE specific TA, during actual TDW where transmitting PUSCHs across multiple slots

One approach to maintain TA and UL transmit power would be just to prohibit UE’s pre-compensation during TDW, once TDW is determined by semi-static configuration. However, it could result in performance degradation, especially when significant TA drift is predicted.

To resolve this kind of pre-compensation issues in NTN, UE may require finer actual TDW up to UE and satellite movement. However, to obtain performance gain from DMRS bundling, it is important to have same determination on actual TDW between UE and gNB. Therefore, it should be known for gNB, if UE changes actual TDW for PUSCH transmission according to TA drift.

To facilitate this, it could be necessary for UE to indicate / declare TA pre-compensation from UE to the gNB, in order to have same and deterministic TDW structure.

Since the ephemeris information are given by gNB, each entity can predict upcoming critical TA drift based on the ephemeris information. Alternatively, gNB may configure pre-compensation occasion for UE, and UE can perform/indicate pre-compensation only on that occasion.

Proposal 15: Discuss how UE declare TA pre-compensation from UE to gNB side before the PUSCH transmission if UE will restart actual TDW within configured TDW for PUSCH transmission according to expected TA drift, in order to have same and deterministic TDW structure between UE and gNB.

Otherwise, UE can indicate its TA pre-compensation via DM-RS or separated uplink channel so that gNB knows where the actual TDW ends. For example, UE may use different DMRS sequence for each TDW changes. Or, UE can directly indicate how many PUSCH transmission needs to be combined on piggybacked UCI. If UE can indicate the timing of TA pre-compensation, gNB and UE can concurrently assume same TDW boundary, without ambiguity. Then, the UE initiated TA pre-compensation can be considered dynamic event.

Proposal 16: Discuss how UE indicate TA pre-compensation from UE to the gNB during the PUSCH transmission if UE restarts actual TDW within configured TDW for PUSCH transmission according to TA drift, in order to have same and deterministic TDW structure between UE and gNB.

Other considerable issues

In RAN #97-e meeting, the PRACH enhancements were agreed to be discussed in Rel-18 NR CE WI. After that, if there is NTN-specific issues, it may be discussed in Rel-18 NR NTN enhancement WI. From this point of view, the following issues may be considered for NTN specific PRACH enhancements.

- RACH occasion configuration for low power class UE

In NTN environment, different types of UE (e.g., Handheld, VST, ESIM, etc) are considered. Those UEs would have different UE power class, which means the maximum UE transmit power would different UE by UE. In the RRC-connected state, it may not be an issue since gNB would know UE power headroom and can schedule and power control for those UE. However, it would be an issue during initial access, since a number of UE could be collide in the same RACH occasion with different transmit power capabilities. To mitigate the issue especially for handheld UE like smartphone, it could be considered for gNB to configure set of preamble index or RACH occasion group for different type of UEs.

Proposal 17. For NTN coverage enhancement, it can be considered to configure exclusive RACH occasion for low power class UE suffering high path loss.

- The open loop TA enhancement for NTN

In case when it is necessary to modify the open loop TA (e.g., UE specific TA and/or common TA) during repeated transmission of UL signal/channel (e.g., PRACH), one of the following solutions may be considered. First of all, the UL segment introduced in Rel-17 IoT NTN can be considered for Rel-18 NTN. If it is supported, the UE can change the open loop TA at UL segment boundary. In addition, it may be considered to support update period of open loop TA, so that UE specific TA and/or common TA can be updated periodically.

Proposal 18. It can be discussed whether and how to support handling open loop TA (e.g., UE specific TA and/or common TA) during repeated transmission of UL signal/channel (e.g., PRACH).

- Polarization loss compensation for NTN

During UL power control, it could be considered to introduce additional power offset based on antenna polarization configuration. If gNB’s antenna polarization is provided to UE, UE can consider gNB’s antenna polarization for UL power control by taking into account of UE’s antenna polarization. By doing so, Polarization loss can be mitigated by increased initial transmit power. It would be beneficial for open-loop power control like PRACH preamble transmission.

Proposal 19. If antenna polarization configuration of gNB are provided to UE, It can be necessary to discuss how to consider the antenna polarization for open-loop UL power control.

- Conclusion

In this contribution, we discussed on the PUCCH enhancement for Msg4 HARQ-ACK and DMRS bundling for PUSCH for Rel-18 NR NTN enhancement, we have following proposals.

Proposal 1. Repetition factor(s) of PUCCH for Msg4 HARQ-ACK is provided via SIB and following options can be considered.

- Option 1: Cell specific single value of repetition factor can be provided via SIB.

  - The value is one of {2, 4, 8}

- Option 2: Set of repetition factors is provided via SIB.

  - The value is one of {1, 2, 4, 8}

  - Option 2-1: Exact repetition factor can be configured by DCI field (e.g., MCS field or DAI field) scheduling the Msg. 4 PDSCH

  - Option 2-2: Implicit mapping between PUCCH for Msg4 HARQ ACK repetition factor and indication of Msg. 3 PUSCH repetition

Proposal 2. If the repetition factor is not configured via SIB, the UE shall determine the repetition factor as 1 (i.e., a single PUCCH transmission).

Proposal 3. Additional PRB offsets can be supported for each PUCCH repetition factors for Msg4 HARQ-ACK.

Proposal 4. For coverage enhancement of PUCCH for Msg4 HARQ-ACK, RAN1 discuss enhancement for PUCCH to be used when dedicated PUCCH resource configuration is not provided.

Proposal 5. For PUCCH repetition for Msg4 HARQ-ACK, PRACH resources (e.g., RACH preambles or RACH occasion) is considered as a container of the repetition request or capability report indicated by UE. One of the following options can be supported for feature combination.

- Option 1: Introduce a new parameter (e.g., pucch-Repetitions-r18) for the PUCCH repetition for Msg4 HARQ-ACK feature into the SIB (e.g., in FeatureCombination-r17).

- Option 2: “msg3-Repetitions-r17” in FeatureCombination-r17 is used as a PUCCH repetition for Msg4 HARQ-ACK feature in addition to the existing Msg3 PUSCH repetition feature.

Proposal 6. The NTN UE expects the parameter "pucch-ResourceCommon" to indicate one of indexes 11 to 15 in Table 9.2.1-1 in TS38.213, if the PUCCH repetition for Msg4 HARQ-ACK is configured.

Proposal 7. It is possible to consider creating a new PUCCH resource set table consisting only of PUCCH format 1 with 14 OFDM symbols for Rel-18 NR NTN UE.

Proposal 8. Support the intra-slot frequency hopping for PUCCH repetition for Msg4 HARQ-ACK.

Proposal 9. Introduce higher payer parameter to indicate the inter-slot frequency hopping for PUCCH repetition for Msg4 HARQ-ACK.

Proposal 10. Further discuss on which slots are applied if the inter-slot frequency hopping for PUCCH repetition for Msg4 HARQ-ACK is enabled.

- Option 1: Operates between the first half slots and the last half slots

- Option 2: Operates between even-numbered slots and odd-numbered slots

Observation 1. If the same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used for PUCCH repetition for Msg4 HARQ-ACK, it is impossible to support the case when the RSRP thresholds is not configured.

Proposal 11. For PUCCH repetition for Msg4 HARQ-ACK, the same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used.

Proposal 12. Do not support the case when the RSRP threshold is not configured for PUCCH repetition for Msg4 HARQ-ACK.

Proposal 13: For DMRS bundling in NTN environment, introduce a new UE capability based on types of NTN platforms.

Proposal 14. Support DMRS bundling for PUSCH coverage enhancement in Rel-18 NR NTN.

- The design of DMRS bundling in Rel-17 CovEnh is a baseline as following:

  - UE is expected to maintain its timing advance at least during TDW

  - Support keeping total TA, which includes both common TA and UE specific TA, during actual TDW where transmitting PUSCHs across multiple slots

Proposal 15: Discuss how UE declare TA pre-compensation from UE to gNB side before the PUSCH transmission if UE will restart actual TDW within configured TDW for PUSCH transmission according to expected TA drift, in order to have same and deterministic TDW structure between UE and gNB.

Proposal 16: Discuss how UE indicate TA pre-compensation from UE to the gNB during the PUSCH transmission if UE restarts actual TDW within configured TDW for PUSCH transmission according to TA drift, in order to have same and deterministic TDW structure between UE and gNB.

Proposal 17. For NTN coverage enhancement, it can be considered to configure exclusive RACH occasion for low power class UE suffering high path loss.

Proposal 18. It can be discussed whether and how to support handling open loop TA (e.g., UE specific TA and/or common TA) during repeated transmission of UL signal/channel (e.g., PRACH).

Proposal 19. If antenna polarization configuration of gNB are provided to UE, It can be necessary to discuss how to consider the antenna polarization for open-loop UL power control.

- Reference

[1] RAN1#112 Chairman’s notes

[2] 3GPP TS 38.213 V17.2.0, NR; Physical layer procedures for control (Release 17)