3GPP TSG RAN WG1 #112bis-e		                     R1-2303751

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.12.2

Source:	LG Electronics

Title:	Discussion on Power Domain Enhancements

Document for:	   Discussion and Decision

- Introduction

Power domain enhancements was included as one of the enhancements to be studied and specified in the NR coverage enhancement work item approved (revised) in RAN1#96 [1]:

- Study and if necessary specify following power domain enhancements

  - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1)

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

The following agreements made in the RAN1 #111 and #112 meetings [2][3] shows discussion scope boiling down to gNB awareness of UE’s Tx power to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work.

Agreement #112 Further discussions in RAN1 concerning means to facilitate higher power transmissions in CA and DC, if applicable, can target increasing gNB awareness of UE’s Tx power, e.g., PHR reporting enhancement such as current power class, power class change, or application of P-MPR by UE (subject to RAN4’s input). - FFS: details. Agreement #111 - At least the following enhancements to information exchange between UE and gNB to facilitate higher power transmissions in CA and DC can be considered for study. Enhanced signaling, if necessary and subject to RAN4’s input, to allow: - Determination at gNB of power class change at the UE - Increased awareness at gNB of energy/power availability at the UE, e.g., a budget. - More informative PHR to be sent from UE to gNB, which may include, e.g., P-MPR related information, power headroom for carrier configured for DL but not UL, power class change indication. - More effective scheduling decisions in the context of UL CA, e.g., best band combination, preferred carrier for servicing uplink, adaptive load sharing across sharing, - Other options are not precluded.

In this contribution, we discuss on PHR reporting enhancement methods such as what to potentially carry (e.g., current power class, power class change, or P-MPR) and when to timely trigger for Rel-18 NR coverage enhancement.

- Discussion

The summation of the maximum output power values for the aggregated bands in CA/DC have been allowed based on Rel-17 RAN4 enhancement on maximum transmit power limit. For example, in case of UE supporting PC3 (23 dBm) in one band and PC2 (26 dBm) in another band, it can use maximum composite power of 27.8 dBm from both PAs while Rel-16 UE would drop its full power to meet the limited maximum composite power.

To realize to increase UE power high limit for CA/DC, it seems reasonable that UE supporting a different power class (e.g., PC2) than the default UE power class (e.g., PC3) could enable and perform the higher maximum output power than that of the default power class and not to fall back to the default PC. However, in accordance with the current specification in [4] and [5], this power class change is carried out based on UE capability with maximum percentage of symbols during a certain evaluation period that can be scheduled for uplink transmission so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies wherein the exact evaluation period is no less than one radio frame if not specified. If maxUplinkDutyCycle-PC2-FR1of UE capability is not absent and the percentage of uplink symbols transmitted in the certain evaluation period is larger than the value, UE shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as to fall back to the default power class, PC3. Similarly in CA, if maxUplinkDutyCycle-interBandCA-PC2-r17 of UE capability is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than the value, UE shall apply all requirements for the default power class to the supported power class and set the configured transmitted power such as to fall back to the default power class.

Observation 1. Current specification articulates one of UE behaviors as its power class fallback based on UE’s own evaluation and decision which is not known to gNB.

Given UE operating in its own implementation specific evaluation and measurement with relatively higher visibility, gNB’s perspective is quite different. Power Headroom Report (PHR) procedure is designed to provide gNB with power headroom and maximum transmit power values, and power class related information (e.g. ΔPPowerClass,CA and ΔPPowerClass,CA ) could be implicitly suggested by the configured maximum output power PCMAX changes based on its equations in both upper and lower bound as specified in [5]. As a result, gNB may observe the configured maximum output power changes by monitoring existing regular or event-triggered PHR reports. gNB may analyze whether power drop has been caused by unexpected path loss or power class change based on value changes in maximum transmit power field as long as PHR report has been not too late to deliver the relevant information to it.

Observation 2. Considering power class change implicitly carried in maximum transmit power field of PHR reports, gNB may analyze whether power drop has been caused by unexpected path loss or power class change as long as PHR reports have been not too late to deliver the relevant information to it.

Proposal 1. RAN1 to discuss whether to indicate current power class and/or power class change explicitly in PHR report could help gNB increase awareness of UE’s Tx power for UE power higher limit for CA/DC.

On the other hand, UE capability maxUplinkDutyCycle-FR2 indicates the maximum percentage of symbols during 1s that can be scheduled for uplink transmission at the UE maximum transmission power, so as to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. If this field is present and the percentage of uplink symbols transmitted within any 1s evaluation period is larger than maxUplinkDutyCycle-FR2, UE follows the uplink scheduling and can apply P-MPRf,c for carrier f of serving cell c that was introduced in the PCMAX,f,c equation. UE reports P-MPR value in the PHR procedure in FR2 when measured power reduction is equal to or higher than a threshold (e.g. P-MPR_00) for MPE reporting. MPE field in PHR MAC CE indicates an index to Table 6.1.3.8-3 (Effective power reduction for MPE P-MPR) in [7], and the corresponding measured values of P-MPR levels in dB are specified in Table 10.1.26.1-1 (Mapping of FR2 P-MPR) in [6] as effective power reduction.

Table 10.1.26.1-1 Mapping of FR2 P-MPR

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| P-MPR_00 | 3  PMP-R < 6 | dB |
| P-MPR_01 | 6  PMP-R < 9 | dB |
| P-MPR_02 | 9  PMP-R < 12 | dB |
| P-MPR_03 | PMP-R 12 | dB |

Table 6.1.3.8-3: Effective power reduction for MPE P-MPR

| MPE | Measured P-MPR value |
| --- | --- |
| 0 | P-MPR_00 |
| 1 | P-MPR_01 |
| 2 | P-MPR_02 |
| 3 | P-MPR_03 |

To be specific, a PHR shall be triggered if mpe-Reporting-FR2 is configured, and mpe-ProhibitTimer is not running under the conditions as in [7]:

- The measured P-MPR is equal to or larger than mpe-Threshold for at least one activated FR2 Serving Cell since the last transmission of a PHR in this MAC entity; or

- the measured P-MPR applied to meet FR2 MPE requirements has changed more than phr-Tx-PowerFactorChange dB for at least one activated FR2 Serving Cell since the last transmission of a PHR due to the measured P-MPR applied to meet MPE requirements being equal to or larger than mpe-Threshold in this MAC entity.

This relevant operation and signaling could be applied to increase UE power high limit for CA/DC even in FR1. If necessary, MPE field could be supported more than 2 bits with more ranges. While this aperiodic PHR reports would indicate effective power reduction explicitly based on MPE compliance, it does not prevent triggering a power class change or account for when it occurs.

Observation 3. P-MPR value in PHR report in FR2 for MPE compliance suggests that power reduction was measured at UE and its range is reported to gNB.

Proposal 2. RAN1 to discuss whether to indicate P-MPR relevant information explicitly in PHR report could help gNB schedule uplink transmission while not triggering UE power class fallback.

Provided that UE implementation specific evaluation period when uplink duty cycle is applied, the improvement only either of current power class or power class change information in PHR report may not be enough for gNB to expect eventually when power class change occurs. According to the current specification, PHR shall be triggered in certain conditions pertaining to phr-ProhibitTimer, phr-PeriodicTimer, phr-Tx-PowerFactorChange, etc which are not involved in MPE (Maximum permissible Exposure) compliance. While MPE P-MPR information shows the power backoff to meet the MPE FR2 requirements, gNB is not still aware of how much available power left, and/or how many symbols left within uplink duty cycle before UE conducts power class fallback. Further threshold values and/or RRC configured parameters relevant to MPE compliance could be considered to let gNB expect when power reduction could potentially occur such as not to schedule uplink transmission within remaining uplink duty cycle.

Proposal 3. RAN1 to discuss on triggering events for PHR reports to help gNB anticipate when power reduction could potentially occur such as not to schedule uplink transmission within remaining uplink duty cycle at UE.

- Conclusion

In this contribution, we discussed on the power domain enhancements for Rel-18 NR coverage enhancement, and following observations and proposals are listed up.

Observation 1. Current specification articulates one of UE behaviors as its power class fallback in various conditions.

Observation 2. Considering power class change implicitly carried in maximum transmit power field of PHR reports, gNB may analyze whether power drop has been caused by unexpected path loss or power class change as long as PHR reports have been not too late to deliver the relevant information to it.

Observation 3. P-MPR value in PHR report in FR2 for MPE compliance suggests that power reduction was measured at UE and its range is reported to gNB.

Proposal 1. RAN1 to discuss whether to indicate current power class and/or power class change explicitly in PHR report could help gNB increase awareness of UE’s Tx power for UE power higher limit for CA/DC.

Proposal 2. RAN1 to discuss whether to indicate P-MPR relevant information explicitly in PHR report could help gNB schedule uplink transmission while not triggering UE power class fallback.

Proposal 3. RAN1 to discuss on triggering events for PHR reports to help gNB anticipate when power reduction could potentially occur such as not to schedule uplink transmission within remaining uplink duty cycle at UE.

- Reference

[1] RP-221858 Revised WID on Further NR coverage enhancements, China Telecom, Jun. 2022.

[2] Chair's notes RAN1#111

[3] Chair's notes RAN1#112

[4] TS 38.306 User Equipment (UE) radio access capabilities

[5] TS 38.101 User Equipment (UE) radio transmission and reception

[6] TS 38.133 Requirements for support of radio resource management

[7] TS 38.321 Medium Access Control (MAC) protocol specification