3GPP TSG RAN WG1 #112bis-e		                     R1- 2303752

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.12.3

Source:	LG Electronics

Title:	Discussion on dynamic waveform switching for NR coverage enhancement

Document for:	   Discussion and Decision

- Introduction

In the RAN1 #112 meeting, the following agreements and conclusions were made for dynamic switching between DFT-S-OFDM and CP-OFDM of Rel-18 NR coverage enhancement WI as in [1]:

Agreement For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI. Note: no change of the current size alignment procedure between UL DCI and DL DCI. Conclusion There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18. Agreement Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant. Conclusion The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant.

In this contribution, we discuss on the dynamic waveform switching between DFT-S-OFDM and CP-OFDM for Rel-18 NR coverage enhancement.

- Discussion

According to the current specification, the gNB can determine either the DFT-S-OFDM or CP-OFDM as the UL waveform of PUSCH via higher layer parameter (e.g., transformPrecoder). That is, if the gNB wants to change the UL waveform for UE, the gNB could only change it by updating the higher layer parameter. Therefore, in order to improve this inefficient operation, dynamic waveform transitions will be introduced for Rel-18 coverage enhancement.

In general, the dynamic waveform switching between DFT-S-OFDM and CP-OFDM would be considered to be supported for UE in RRC connected state. For example, if the channel quality of the UE in RRC connected state is degraded, the gNB can dynamically change the UL waveform of the PUSCH (e.g., change from CP-OFDM to DFT-S-OFDM, if the UL waveform is configured as CP-OFDM via higher layer parameter). Therefore, we prefer to support the dynamic waveform switching between DFT-S-OFDM and CP-OFDM for UE in RRC connected state.

Proposal 1. Support the dynamic waveform switching for UE in RRC connected state in Rel-18 NR coverage enhancement.

Meanwhile, if the dynamic waveform switching is supported, the UE with UE capability for dynamic waveform switching should know whether the dynamic waveform switching is enabled or not. Therefore, we prefer to support the gNB can indicate whether the dynamic waveform switching is enabled or not via higher layer signaling (e.g., dedicated RRC signaling)

Proposal 2. Introduce a new higher layer parameter to indicate whether the dynamic waveform switching is enabled or not.

In the RAN1 #112 meeting, an explicit 1-bit field for dynamic waveform indication was finally introduced, but it is still necessary to clarify which DCI format could be compliant. Firstly, there might be no issues for adding 1-bit field in non-fallback DCIs, e.g. DCI format 0_1, DCI format 0_2 (configured), since those DCI formats have configurable characteristic. However, fallback DCI, i.e. DCI format 0_0 which is especially monitored in common search space (CSS) across all UEs, has a restriction to increase the DCI size, hence add the new 1-bit field is not desirable in this case.

On the other hand, some companies may propose that dynamic waveform switching should be supported using other methods instead of adding 1-bit new field for fallback DCI. However, since it is desirable to introduce uniform solution for all DCI formats, we prefer not to support dynamic waveform switching in fallback DCI in Rel-18 CE.

Moreover, it is necessary to make a decision whether the dynamic waveform switching is applicable for Msg. 3 PUSCH (re-)transmission. To be specific, Msg. 3 PUSCH transmission during initial access procedure should be allocated by RAR UL grant, and Msg. 3 PUSCH re-transmission is scheduled by DCI format 0_0. Considering both cases are difficult to secure a new 1-bit indication field, it is preferred not to introduce dynamic waveform switching for Msg. 3 PUSCH (re-)transmission in Rel-18 CE.

Proposal 3. Dynamic waveform switching is not applicable to fallback DCI (e.g., DCI format 0_0) in Rel-18 CE.

Proposal 4. Dynamic waveform switching is not applicable to Msg. 3 PUSCH (re-)transmission in Rel-18 CE.

FL proposal 2-4r2: RAN1 to select one of the following Options for DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2: Option 1: Per-format alignment - Step 1: Determine field sizes of DCI addressed to C-RNTI based on the waveform indicated by DWS field. - Step 2: If DCI is addressed to CS-RNTI, align field sizes to that of DCI addressed to C-RNTI based on the waveform indicated by DWS field using the existing C-RNTI/CS-RNTI alignment clause. - Select between: - Alt. A: - By specification, at least one field preceding NDI has size that depends on waveform (e.g. FDRA). - Note: This implies DWS field must be decoded before finding position of NDI field. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Impact on procedure for PDCCH validation of CG type 2: if DWS field value does not match waveform RRC-configured in ConfiguredGrantConfig for the CG, and NDI=0, it is an error case. - Alt. B: - By specification, no field preceding NDI has size that depends on waveform. - Note: It seems that it is currently the case for DCI format 0_1/0_2 in R17 (may need further checking) - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Note: This means DWS field can be positioned after NDI, but before the first field that depends on waveform. - No impact on procedure for PDCCH validation of CG type 2. UE can ignore value of DWS field in the validation. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). Option 2: Per-field alignment - Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Step 2: If DCI is addressed to CS-RNTI, align field sizes to those of C-RNTI according to existing alignment clause between CS-RNTI and C-RNTI. - Note: DWS field does not need to be decoded before finding position of NDI field or any other field. - Note: No constraint on position of DWS field in DCI (but positioning before any field that has waveform-dependent interpretation may help implementation). Option 3: Per-field alignment only for fields preceding NDI field - Step 1: For fields preceding NDI, apply per-field alignment as per Steps 1 and 2 of Option 2. - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Step 2: For fields following NDI, determine field sizes based on DWS as per Steps 1 and 2 of Option 1. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Note: Option 3 has same features as Option1/Alt-B with respect to impact on PDCCH validation and positioning of DWS field. In case all fields preceding NDI are guaranteed to have size independent of waveform by specification, Option 3 may not be relevant. Option 4: Per-field alignment only for DCI addressed to CS-RNTI - For DCI addressed to C-RNTI: - Step 1: Determine size of fields based on DWS indication. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Step 2: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - For DCI addressed to CS-RNTI: - Apply padding to each field, if necessary, to match the size of the corresponding field for the DCI addressed to C-RNTI and (DWS=CP-OFDM). - Note: This means DWS field does not need to be decoded before finding position of NDI field in the case of CS-RNTI. Note: Field introduced in future specs needs to have size for CP-OFDM at least equal to size for DFT-S-OFDM, or otherwise cannot be used for CS-RNTI and NDI=1.

Furthermore, as above list-up options [2], there was a discussion related to the DCI size alignment by introducing 1-bit indication field. These alternative options of DCI size alignment are mainly divided based on the NDI value that makes different interpretation of DCI for CG-PUSCH Type 2. Not only that, how to define DWS field with two NDI values in CG-PUSCH Type 2 so as to interpret DCI is FFS, i.e., DWS field in case of NDI=0 might be reserved whereas DWS field indicates the waveform in case of NDI=1 as DG-PUSCH. Moreover, there are waveform-dependent fields whose field size are different, e.g., FDRA, TPMI, antenna port, and even same size MCS field carries different look-up table depending on waveform type.

Given the DCI size alignment options, Option 2 will be the straight-forward solution compared to the other options. To be specific, for DCIs addressed to CS-RNTI, there is no additional size comparison step needed between DCIs with different NDI values since those DCIs, i.e., NDI=0 and NDI=1, are size-matched (regardless of DWS field position/availability) to the largest DCI addressed to C-RNTI per-field.

However, the remaining options require more restrictions to be considered compared to the option 2. Option 1 requires per-field alignment for each activation (NDI=0) or retransmission (NDI=1) case of CS-RNTI with the indicated waveform from C-RNTI determined, which creates ambiguity due to the position of DWS field. Option 3 solves this problem by firstly size-matching C-RNTI DCIs between different waveforms, then appending most significant bits with zero to only the fields preceding NDI field in CS-RNTI DCIs until the bit width equals those fields of C-RNTI. Both options then perform a per-format alignment for the dependency fields according to the indicated waveform for the fields following the NDI, some companies claim that there is a benefit to the alignment due to the possibility of introducing new features for a particular waveform, e.g., FDSS-SE for DFT-S-OFDM. But it is not a benefit that directly obtained from the DCI size alignment because the new feature should be confirmed to introduce first. Lastly, Option 4 unnecessarily performs different alignments between RNTIs, which is not beneficial as a reasonable unified solution is preferred.

Accordance with aforementioned comparison, for the other options the benefit of per-format alignment is not exactly obtained at the price of unnecessarily specification effort, while option 2 simply needs to add dynamic indicated waveform in the existing C-RNTI/CS-RNTI alignment clause.

Proposal 5. Support per-field DCI size alignment that can be the simplest solution without any ambiguity caused by DWS field position and NDI values.

On the other hand, a UE implementation of DWS field in CG-PUSCH Type 2 should be further studied. In the case of CG-PUSCH Type 2 with DWS field to be ignored, DCI field information is determined by the waveform configured by the value of “transformPrecoder” in the higher layer parameter. However, if dynamic waveform switching operation is indicated by DWS field, the availability of DWS field may vary depending on whether DCI carries activation or retransmission information. For example, if DWS field is not valid with activation, UE may decide its uplink waveform based on higher layer parameter, otherwise in case of retransmission UE may set its uplink waveform based on the DWS field value.

Proposal 6. If CG-PUSCH Type 2 is configured, waveform to be applied should be decided upon its DWS field availability and NDI values at UE.

There was a discussion of a power control mechanism to assist dynamic waveform switching. That is, the UE transmission power must be appropriately adjusted according to the UL waveform dynamically indicated from gNB. With the power headroom information, i.e. PHR, per its UL waveform reported by UE, gNB will estimate how much available power the UE has.

Considering the power control technique when UL waveform is dynamically indicated as above mentioned, there might require an additional procedure, i.e. closed-loop power control, for gNB to indicate that UE modifies its transmission power level. In other words, a UE transmission power is adjusted after the waveform is changed subsequently, which may result in a delay during the closed-loop power control procedure.

Therefore, in order for the UE to immediately change the transmission power for the dynamic waveform, the gNB can consider providing and individually managing open-loop power control parameters for each waveform, i.e. CP-OFDM/DFT-S-OFDM, respectively. For example, the open-loop power control related parameters, P0 offset for the target received power or  offset for the path-loss compensation can be considered.

Proposal 7. If dynamic waveform switching is enabled, support independent open-loop power control parameter for each waveform.

- Conclusion

In this contribution, we discussed on the dynamic waveform switching between DFT-S-OFDM and CP-OFDM for Rel-18 NR coverage enhancement, we have following proposals.

Proposal 1. Support the dynamic waveform switching for UE in RRC connected state in Rel-18 NR coverage enhancement.

Proposal 2. Introduce a new higher layer parameter to indicate whether the dynamic waveform switching is enabled or not.

Proposal 3. Dynamic waveform switching is not applicable to fallback DCI (e.g., DCI format 0_0) in Rel-18 CE.

Proposal 4. Dynamic waveform switching is not applicable to Msg. 3 PUSCH (re-)transmission in Rel-18 CE.

Proposal 5. Support per-field DCI size alignment that can be the simplest solution without any ambiguity caused by DWS field position and NDI values.

Proposal 6. If CG-PUSCH Type 2 is configured, waveform to be applied should be decided upon its DWS field availability and NDI values at UE.

Proposal 7. If dynamic waveform switching is enabled, support independent open-loop power control parameter for each waveform.

- Reference

[1] RAN1 Chair’s Notes in the 3GPP TSG RAN WG1 #112

[2] R1-2302222, “Summary #4 on dynamic switching between DFT-S-OFDM and CP-OFDM”, Moderator (InterDigital, Inc.)