3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303754

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on RAN4 reply LS for LP-WUR architectures

Document for:	Discussion

# 1	Introduction

In this contribution, we discuss the incoming reply LS from RAN4 [1] for LP-WUR architectures and provide potential RAN1 replies to the clarifications requested by RAN4.

# 2	Discussion

In the LS, RAN4 informed RAN1 of some initial assumptions and agreements to evaluate the RF impacts of LP-WUR architectures, and also asked for some clarifications from RAN1.

In this meeting, to evaluate the RF impacts of LP-WUR architecture, RAN4 make some initial assumption and agreements, e.g.,

- RAN4 assume 1RX architecture for LP-WUR as starting point

- ACS values from current UE specifications are used as a starting point for discussion to evaluate LP-WUR performance

- Consider 1.4MHz and 5MHz WUS bandwidth for FR1 evaluation as the starting point

  - Guard band if needed, can be located within 1.4MHz and 5MHz RF bandwidth

- No impact of LP-WUS on the existing gNB emissions and compliance requirements is baseline

- RF envelop detection architecture is more appropriate for single-band operation

- IF/BB envelop detection is more appropriate for multi-bands operation. Multi-band here still means that only one band at a time is being received.

To further evaluate the RF aspects of LP-WUR architecture, RAN4 would like to know the following clarifications from RAN1:

- Whether IoT/wearables/smartphone UE types are all considered for LP-WUR design

- Power consumption, coverage and SNR targets

- Max occupied RB number in channel bandwidth for LP-WUS, for 1.4MHz and 5MHz RF bandwidth case

- Possible supported SCS for LP-WUS, if applicable

- Whether WUS can be located in a band separate from the UE’s NR band

- Whether FR1 is considered as first priority frequency range

- Whether in-band power boosting of LP-WUS is considered from RAN1 perspective

Below sections list potential RAN1 replies based on the agreements so far.

### Whether IoT/wearables/smartphone UE types are all considered for LP-WUR design

RAN1 reply > The considered cases from RAN1 perspective are reflected in the below RAN1 agreement.

Agreement

The following characteristics for target use cases are considered in the study item:

- IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics,

  - FFS: latency

  - primary for small form devices

  - power-sensitive

  - static, nomadic or limited mobility

- Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc.,

  - FFS: latency

  - primary for small form devices,

  - power-sensitive

  - low/medium speed, FFS: high speed

- eMBB cases including e.g., XR/smart glasses, smart phones and etc.,

  - FFS: latency

  - devices form is various and not restricted

  - power-sensitive

  - low/medium speed, FFS: high speed

Note: other use cases/characteristics are not precluded if any.

### Power consumption, coverage and SNR targets

RAN1 reply > RAN1 has not agreed to targets for these. Below are RAN1 agreements related to power consumption and coverage evaluations of LP-WUS. RAN1 will inform RAN4 if any further progress is made.

Agreement

The following power model for LP-WUR is used for evaluation for FR1,

| Power State | Relative Power (unit) | Transition energy: (unit multiplied by ms) | Ramp-up time TLR, ramp-up (ms) |
| --- | --- | --- | --- |
| Off | 0.001 | [TLR, ramp-up *(PON+POFF)/2] | TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up |
| On | 0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4 FFS: If other values are needed |  |  |

FFS: whether further categorization/sub-categorization is needed and how.

Agreement

For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) is reused as baseline.

- MIL is used as the metric for LP-WUS coverage evaluation

- urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded.

Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830.

Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss

Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS

### Max occupied RB number in channel bandwidth for LP-WUS, for 1.4MHz and 5MHz RF bandwidth case

RAN1 reply > The LP-WUS BW and RB number that are used as starting point for evaluations in RAN1 are given in below RAN1 agreement. RAN1 has not specifically discussed 1.4 MHz RF bandwidth case.

Agreement

For link-level simulation of LP-WUS, the following table is used as starting point,

- FFS for other assumptions if any

- Note: The assumptions are not intended to limit the scope of the study or the design.

Table XX. Simulation assumptions for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz/4GHz/700MHz |
| Waveform | OOK , FSK , OFDM Company to report which option for OOK /FSK /OFDM is used |
| Channel structure | - Option 1: Sync signal /sequence+ payload + CRC, - Option 2: Sequence only, - Option 3: Payload+CRC, - Other options are not precluded - Company to report the sequence length, payload size, CRC length (may or may not be presence). |
| SCS of OFDM generator for NR signal | 30kHz/15KHz |
| Configuration for LP-WUS signal | For OOK/FSK waveform, - Option 1a: M=1 and SCSs = 15kHz (same as NR signal) - Option 1b: M=1 and SCSs = 30kHz (same as NR signal) - Option 2a: M =2/4/8 for SCS = 15KHz (same as NR signal) - Option 2b: M =2/4/8 for SCS = 30 kHz (same as NR signal) - Option 3: M=1 and SCSs = 60kHz/120kHz/240kHz - Note: M is referred to the definition of “M” in the agreements for OOK-1/2/3/4 and FSK-1/2 For OFDM: FFS, e.g., ZC sequence Other options are up to companies to report |
| WUS duration | Number of OFDM symbols: e.g., 1,2,4, 8, 16,24 symbols |
| MDR/FAR assumption | - The miss-detection rate (MDR) of LP-WUS 1%, - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%] - Other values are not precluded for studying, reported by companies - Further discuss on the following alternatives for FAR target - Alt 1: FAR target is determined per single WUS attempt/trial, - Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - FFS: possible values for reference time durations - Companies to report details, e.g., receiver behaviour, how to compute MDR, detection threshold - Companies to report the selected reference time duration values and the associated number of WUS attempts/trials |
| Code scheme | Companies to report, if any, the coding scheme (e.g., Manchester code or any other schemes) and the code rate (e.g., 1/2, 1/4, ….) |
| gNB Channel BW | 20MHz, FFS other values |
| LP-WUS BW | Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS Option 2: - {2.16, 4.32} MHz including subcarriers for guard band - 1.44MHz, 2.88MHz (i.e.{4, 8} RBs) for LP-WUS transmission for 30kHz SCS FFS: other options are up to companies to report GB is symmetrically placed on each side of LP-WUS |
| Filter | X-th Order filter (e.g. Butterworth, Chebyshev, …) with Y MHz bandwidth, - X = {3, 5} - Companies to report Y Companies to report any other assumptions if needed |
| Adjacent subcarrier interference | - PDSCH mapped on resources other than that for WUS and guard band; EPRE of LP-WUS / EPRE of PDSCH =ρ, where ρ=0 dB as baseline, ρ= {3, 6} dB as optional |
| Sampling Rate | - Companies to report. |
| ADC bit width | 1-bit, 4-bit, 8-bit, ideal and other options are not precluded |
| Channel Model | See link coverage assumption table (will copy and paste here) |
| Impairment modelling | - FFS: Frequency and time error model - Phase noise up to company report, e.g. the modelling used for 802.11ba - Other cell interference is up to company to report |

### Possible supported SCS for LP-WUS, if applicable

RAN1 reply > Below is RAN1 agreement related to SCS for LP-WUS generation. Also, LP-WUS SCS values that are used as starting point for evaluations in RAN1 are captured in the agreement cited for previous question (e.g. the row for ‘Configuration for LP-WUS signal)’.

Agreement

For MC-ASK or MC-FSK waveform generation, SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with, study whether SCS can be different, also study

- FDM/TDM multiplexing with other NR transmissions

- link performance

- impact to legacy UEs

- impact on gNB

### Whether WUS can be located in a band separate from the UE’s NR band

RAN1 reply > RAN1 has no agreement that the WUS can be located in a band separate from the UE’s NR band.  However, RAN1 is evaluating at least scenarios (e.g. for coverage evaluations of LP-WUS) where WUS is in UE’s NR band.

### Whether FR1 is considered as first priority frequency range

RAN1 reply > RAN1 has not agreed on prioritization with respect to frequency ranges. However, based on below agreements, at least LP-WUS power consumption values for FR2 and whether FR2 is included in the scope of LP-WUS SI are FFS, and there is no FR2 scenario agreed yet for evaluation of coverage of LP-WUS.

Agreement

- The following power model for LP-WUR/WUS evaluation is considered,

  - Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring:

    - [0.001]

  - Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring:

    - [0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4]

    - Other values are not precluded to be evaluated.

    - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation

  - No additional transition energy and transition time between ‘on’ and ‘off’ state as start point, FFS any transition energy and transition time if needed.

Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver.

Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion.

Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture)

FFS: LP-WUR power consumption values for FR2.

Agreement

For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) is reused as baseline.

- MIL is used as the metric for LP-WUS coverage evaluation

- urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded.

Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830.

Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss

Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS

Agreement

For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further

- whether BW of LP-WUS is configurable (implicitly or explicitly)

- size of guard band [FFS: within or outside of BW X], if any

- whether there is different X for Idle, Connected, Inactive modes

FFS: Whether FR2 is included in the scope of LP-WUS SI

### Whether in-band power boosting of LP-WUS is considered from RAN1 perspective

RAN1 reply > RAN1 has not yet discussed in-band power boosting of LP-WUS and will inform RAN4 if any progress is made on this.

# 3	References

R1-2302287, “Reply LS to RAN1 on LP-WUR architectures” LS from RAN4, 3GPP TSG RAN1#112bis-e, April 2023.

RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR,” 3GPP TSG-RAN#97e, Sept 12 - 16, 2022.