3GPP TSG-RAN WG1 #112b-e		R1-2303755

eMeeting, Apr 17-26, 2023

Source:	Ericsson

Title:	Power scaling of SCell PRACH for UL CA

Agenda Item:	7.1

Document for:	Discussion and Decision

# Introduction

In this document, we discuss correction to SCell PRACH power scaling for UL CA power limited case.

# Discussion

Power scaling for UL CA is captured in 38.213 sub-clause 7.5. For case when there is overlap between periodic SRS transmission on PCell and a PDCCH order triggered PRACH transmission on SCell, the highlighted text below can be interpreted to imply that UE should prioritize periodic SRS over PDCCH order triggered PRACH during power allocation for power limited case.

-------------------- spec excerpt from 38.213 sub-clause 7.5 -------------------------

7.5	Prioritizations for transmission power reductions

For single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cells in a frequency range in a respective transmission occasion  would exceed , where  is the linear value of  in transmission occasion  as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cells in the frequency range is smaller than or equal to  for that frequency range in every symbol of transmission occasion . When determining a total transmit power for serving cells in a frequency range in a symbol of transmission occasion , the UE does not include power for transmissions starting after the symbol of transmission occasion . The total UE transmit power in a symbol of a slot is defined as the sum of the linear values of UE transmit powers for PUSCH, PUCCH, PRACH, and SRS in the symbol of the slot.

-	PRACH transmission on the PCell

-	PUCCH or PUSCH transmissions with higher priority index according to clause 9

-	For PUCCH or PUSCH transmissions with same priority index

-	PUCCH transmission with HARQ-ACK information, and/or SR, and/or LRR, or PUSCH transmission with HARQ-ACK information

-	PUCCH transmission with CSI or PUSCH transmission with CSI

-	PUSCH transmission without HARQ-ACK information or CSI and, for Type-2 random access procedure, PUSCH transmission on the PCell

-	SRS transmission, with aperiodic SRS having higher priority than semi-persistent and/or periodic SRS, or PRACH transmission on a serving cell other than the PCell

In case of same priority order and for operation with carrier aggregation, the UE prioritizes power allocation for transmissions on the primary cell of the MCG or the SCG over transmissions on a secondary cell. In case of same priority order and for operation with two UL carriers, the UE prioritizes power allocation for transmissions on the carrier where the UE is configured to transmit PUCCH. If PUCCH is not configured for any of the two UL carriers, the UE prioritizes power allocation for transmissions on the non-supplementary UL carrier.

-------------------- end spec excerpt ---------------------------------------------------

However, PDCCH order triggered PRACH having higher priority over periodic SRS is the technically correct behavior. The PRACH transmission on SCell is more critical (it is used to get initial TA estimate for the SCell) than a particular instance of periodic SRS. It is also ‘aperiodic’ and general principle for power scaling behavior has been that aperiodic transmissions are prioritized.

The above issue impacts practical deployments, especially the case of FR1 UL CA with a TDD PCell and FDD SCell. For this case, the PCell and SCell are typically in separate TAGs and PDCCH ordered PRACH is required on the SCell to get TA estimates. Periodic SRS is configured on TDD PCell to support reciprocity-based beamforming. For a typical 1T4R configuration of ‘antenna switching’ SRS resource set, the UE could have periodic SRS in every frame and the SRS resources can overlap with possible PRACH occasions for several PRACH configurations. This is possible for several TDD UL/DL configurations and there could also be cases where the SRS resources can overlap with a possible PRACH occasion in every radio frame.

Without spec clarification to correct the above text, there is risk that UEs would scale down/drop the PRACH and frequent dropping of PRACH on SCell would have significant negative impact on UL CA operation.

To address this issue, we propose to correct the specification text as shown in below TP. Associated draft CR is provided in [1].

-------------------- start TP to 38.213 vgd0 sub-clause 7.5 -------------------------

7.5	Prioritizations for transmission power reductions

For single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cells in a frequency range in a respective transmission occasion  would exceed , where  is the linear value of  in transmission occasion  as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cells in the frequency range is smaller than or equal to  for that frequency range in every symbol of transmission occasion . When determining a total transmit power for serving cells in a frequency range in a symbol of transmission occasion , the UE does not include power for transmissions starting after the symbol of transmission occasion . The total UE transmit power in a symbol of a slot is defined as the sum of the linear values of UE transmit powers for PUSCH, PUCCH, PRACH, and SRS in the symbol of the slot.

-	PRACH transmission on the PCell

-	PUCCH or PUSCH transmissions with higher priority index according to clause 9

-	For PUCCH or PUSCH transmissions with same priority index

-	PUCCH transmission with HARQ-ACK information, and/or SR, and/or LRR, or PUSCH transmission with HARQ-ACK information

-	PUCCH transmission with CSI or PUSCH transmission with CSI

-	PUSCH transmission without HARQ-ACK information or CSI and, for Type-2 random access procedure, PUSCH transmission on the PCell

-	Aperiodic SRS transmission, or PRACH transmission on a serving cell other than the PCell

-	Semi-persistent and/or periodic SRS transmission

In case of same priority order and for operation with carrier aggregation, the UE prioritizes power allocation for transmissions on the primary cell of the MCG or the SCG over transmissions on a secondary cell. In case of same priority order and for operation with two UL carriers, the UE prioritizes power allocation for transmissions on the carrier where the UE is configured to transmit PUCCH. If PUCCH is not configured for any of the two UL carriers, the UE prioritizes power allocation for transmissions on the non-supplementary UL carrier.

-------------------- end TP ---------------------------------------------------

While above proposal addresses the critical issue of SCell PRACH being impacted due to collision with periodic SRS, a more optimal approach would be to prioritize SCell PRACH over any SRS on PCell and we are open to also discuss such a change.

# Conclusions

In this document we discuss correction to SCell PRACH power scaling for UL CA power limited case and propose the following

Proposal

- Clarify that UE should prioritize PDCCH order triggered PRACH over periodic/semi-persistent SRS for power allocation in power limited case when there is overlap between periodic/semi-persistent SRS transmission on PCell and PDCCH order triggered PRACH transmission on SCell

- Agree to the draft CR provided in [1] reflecting below TP

-------------------- start TP to 38.213 vgd0 sub-clause 7.5 -------------------------

7.5	Prioritizations for transmission power reductions

For single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cells in a frequency range in a respective transmission occasion  would exceed , where  is the linear value of  in transmission occasion  as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cells in the frequency range is smaller than or equal to  for that frequency range in every symbol of transmission occasion . When determining a total transmit power for serving cells in a frequency range in a symbol of transmission occasion , the UE does not include power for transmissions starting after the symbol of transmission occasion . The total UE transmit power in a symbol of a slot is defined as the sum of the linear values of UE transmit powers for PUSCH, PUCCH, PRACH, and SRS in the symbol of the slot.

-	PRACH transmission on the PCell

-	PUCCH or PUSCH transmissions with higher priority index according to clause 9

-	For PUCCH or PUSCH transmissions with same priority index

-	PUCCH transmission with HARQ-ACK information, and/or SR, and/or LRR, or PUSCH transmission with HARQ-ACK information

-	PUCCH transmission with CSI or PUSCH transmission with CSI

-	PUSCH transmission without HARQ-ACK information or CSI and, for Type-2 random access procedure, PUSCH transmission on the PCell

-	Aperiodic SRS transmission, or PRACH transmission on a serving cell other than the PCell

-	Semi-persistent and/or periodic SRS transmission

In case of same priority order and for operation with carrier aggregation, the UE prioritizes power allocation for transmissions on the primary cell of the MCG or the SCG over transmissions on a secondary cell. In case of same priority order and for operation with two UL carriers, the UE prioritizes power allocation for transmissions on the carrier where the UE is configured to transmit PUCCH. If PUCCH is not configured for any of the two UL carriers, the UE prioritizes power allocation for transmissions on the non-supplementary UL carrier.

-------------------- end TP ---------------------------------------------------

# References

[1] R1-2303756, “Draft CR for 38.213 on correction to SCell PRACH power scaling for UL CA”, Ericsson, RAN1#112b-e, April 2012.

# Annex A

Relevant agreements from RAN1#92

Agreement:

Following working assumption is confirmed with the understanding that the CCs are in the same cell group:

- In Case 2, (CCs/uplinks configured for UE have same or different numerologies and partially overlapping transmissions between different CCs/uplinks and same/different transmission duration and one or two PUCCH group(s)), when the UE is power limited due to simultaneous transmission on multiple serving CCs/uplinks,

  - PRACH of PCell > PUCCH/PUSCH with ACK/NACK and/or SR > PUCCH/PUSCH with other UCIs > PUSCH w/o UCI > SRS/PRACH of Scell

    - Within a same priority level, PCell is prioritized over Scell

    - In case that transmission power exceeds Pcmax, Scaling/dropping is applied to the lowest priority first until the aggregated power is within Pcmax.

    - Note: different priority of SRS used for carrier switching can be discussed further

  - Scaling or dropping of the whole or part(s) of a transmission is left to UE implementation.

- Note: If the aggregated transmission power does not exceed Pc_max within any part of a transmission that overlaps with other transmission(s), the transmission is considered as non-power limited case.

- Note: power control with look-ahead is not required at UE.

FFS: Priority on the UL and SUL

Agreement:

  - For power control, the following priority rules for SRS transmission is adopted

    - A-SRS > SP-SRS/P-SRS

  - Additional prioritization rules for SRS carrier switching cases are not introduced.

# Annex B

As shown in below spec excerpts from 36.213, SCell PRACH transmission is prioritized over SRS for multiple TAGs case in LTE as well.

------------- start spec excerpt from 36.213 sub-clause 5.1.1.1 -----------------------------------

-	If the UE is configured with multiple TAGs, and if the PUSCH transmission of the UE on subframe/slot/subslot  for a given serving cell in a TAG overlaps some portion of the first symbol of the PUCCH transmission on subframe/slot/subslot  for a different serving cell in another TAG the UE shall adjust its total transmission power to not exceed on any overlapped portion.

-	If the UE is configured with multiple TAGs, and if the trigger type 0/1 SRS transmission of the UE in a symbol on subframe/slot/subslot  for a given serving cell in a TAG overlaps with the PUCCH/PUSCH transmission on subframe/slot/subslot or subframe/slot/subslot  for a different serving cell in the same or another TAG the UE shall drop trigger type 0/1 SRS if its total transmission power exceeds on any overlapped portion of the symbol.

-	If the UE is configured with multiple TAGs, and if the trigger type 2 SRS transmission of the UE in a symbol on subframe/slot/subslot i for a given serving cell in a TAG overlaps with the PUCCH/PUSCH transmission on subframe/slot/subslot i, i-1 or i+1 for a different serving cell in the same or another TAG, the UE shall drop the trigger type 2 SRS in the overlapped symbol if its total transmission power exceeds PCMAX on any overlapped portion of the symbol.

-	If the UE is configured with multiple TAGs and more than 2 serving cells, and if the trigger type 0/1 SRS transmission of the UE in a symbol on subframe/slot/subslot  for a given serving cell overlaps with the SRS transmission on subframe/slot/subslot  for a different serving cell(s) and with PUSCH/PUCCH transmission on subframe/slot/subslot or subframe/slot/subslot  for another serving cell(s) the UE shall drop the trigger type 0/1 SRS transmissions if the total transmission power exceeds on any overlapped portion of the symbol.

-	If the UE is configured with multiple TAGs and more than 2 serving cells, and if the trigger type 2 SRS transmission of the UE in a symbol on subframe/slot/subslot i for a given serving cell overlaps with the SRS transmission on subframe/slot/subslot i-1, i or i+1 for a different serving cell(s) and with PUSCH/PUCCH transmission on subframe/slot/subslot i-1, i or i+1 for another serving cell(s), the UE shall drop the trigger type 2 SRS transmission in the overlapped symbol if the total transmission power exceeds PCMAX on any overlapped portion of the symbol.

-	If the UE is configured with multiple TAGs, the UE shall, when requested by higher layers, to transmit PRACH in a secondary serving cell in parallel with SRS transmission in a symbol on a subframe of a different serving cell belonging to a different TAG, drop SRS if the total transmission power exceeds on any overlapped portion in the symbol.

-	If the UE is configured with multiple TAGs, the UE shall, when requested by higher layers, to transmit PRACH in a secondary serving cell in parallel with PUSCH/PUCCH in a different serving cell belonging to a different TAG, adjust the transmission power of PUSCH/PUCCH so that its total transmission power does not exceed on the overlapped portion.

------------- start spec excerpt from 36.213 sub-clause 5.1.1.1 -----------------------------------

It is also aligned with the behavior specified in LTE as shown in Annex B