3GPP TSG RAN WG1#112-bis-e	R1-2303757

Online Meeting, Apr 17th – Apr 26th, 2023

Agenda Item:	9.7.1

Source:	Ericsson

Title:	NW energy saving techniques in spatial and power domains

Document for:	Discussion and Decision

# Introduction

In RAN#98e, it was agreed to start a Rel 18 WI on network energy savings, with the following as one of the objectives.

- Specify the following techniques in spatial and power domains

- Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2]

- Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2]

- Note: Above objectives are only for UE specific channels/signals

- Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

In this paper, we provide input for above objective, i.e., potential techniques on gNB and UE side in spatial and power domains which can improve network energy savings, with the emphasis that the underlying techniques should prioritize No-, Low- and Medium-load scenarios.

# Discussion

## Power domain energy saving techniques

Adapting downlink transmit power for PDSCH transmissions can reduce network energy consumption with little to no impact on UE perceived performance in some scenarios (e.g., for UEs in good coverage). In RAN1#112, the following agreements were made:

Agreement

For adaptation of power offset values between PDSCH and CSI-RS, further study the following

  - Where/how to configure multiple power offset values

    - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception

    - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS

    - Whether other UE report content can be included

Agreement

Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that

  - Whether there is a need for transition time per adaptation (for UE)

  - Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

Here, we discuss aspects relevant for deciding the design for power domain adaptations.

When it comes to gNB transmission power adaptations, mainly CSI-RS and PDSCH have been under discussion during the WI. In our opinion, it is the PDSCH Tx power adaptations that brings most of the energy savings for the gNB. Many times, the transmitted CSI-RSs are shared by more than one UE and occupy only a few resources and we therefore here only assume that it is the PDSCH energy-per-resource element (EPRE) that is dynamically changed. Upon such change, if the EPRE of CSI-RS does not change accordingly, the actual power offset between PDSCH and CSI-RS REs will change as well. If this is not compensated in the UE’s CSI estimates, the CSI accuracy degrades, particularly the accuracy of MIMO rank estimation (RI). Such inaccuracies are not something that the gNB can compensate simply by scaling the CSI reports received from the UE. Therefore, it would enhance CSI estimation accuracy if the UE reports CSI based on the actual power offset between PDSCH to CSI-RS that the gNB may use. In the current specifications, each NZP-CSI-RS resource is configured with one power offset setting (powerControlOffset) between PDSCH and CSI-RS. Note that we don’t address powerControlOffsetSS (offset between CSI-RS RE to SSS RE) since, as mentioned earlier, we do not think changing CSI-RS Tx power brings major savings.

For power domain adaptation techniques, dynamic updates for powerControlOffsetSS are not supported.

The adaptation can be divided into three phases:

- Phase 1 (CSI measurement/report): UE measures and reports on multiple different power offset values (hypotheses) and reports corresponding CSIs (e.g., RI, PMI, CQI)

- Phase 2: gNB selects an offset amongst the hypothesized ones to use for the purpose of future CSI measurements and reports

- Phase 3 (CSI measurement/report): gNB dynamically indicates to UE which power offset it should use for the purpose of future CSI measurements and reports

### Power adaptation phase 1(CSI measurement/report)

For the enablement of phase 1, a UE should be able to support multiple PDSCH-to-CSI-RS power offset (powerControlOffset) values for CSI measurement on a single NZP-CSI-RS resource. We think that the UE could be preconfigured with multiple power offset hypotheses for which aperiodic CSI reports are triggered, e.g., via the “CSI request” field in DCI 0_1/0_2. We think that these hypotheses could suitably be configured as part of the RRC CSI-AperiodicTriggerState or other IEs referenced within it. This structure could be extended with an optionally present list of power offset hypotheses. For example, a separate trigger state could be configured with power offsets [0, -6, -9] dB for a CSI-RS resource in a CSI report setting. Once the trigger state is triggered by a DCI e.g., DCI 0_1, based on presence of the hypotheses list, the UE would understand that it needs to temporarily override the original offset for the configured CSI-RS and instead measure and report CSI corresponding to each of the power offsets.

The number of hypotheses that can be configured and reported as above per UE is subject to UE capabilities such as the number of simultaneous reports per CC (simultaneousCSI-ReportsPerCC: 1...8) and the number over all CCs (simultaneousCSI-ReportsAllCC: 5...32), and the fact that N CSI reports could imply that N CPUs are used from the total UE budget on simultaneous CSI computations.

When it comes to the CSI report, we think that multiplexing multiple reports, aperiodically triggered, on one PUSCH is straightforward. Optimized CSI reporting contents allowing the UE to provide compact CSI feedback can also be studied.

### Power adaptation phase 2

In Phase 2, the gNB decides which power offset hypothesis to use for future CSI reporting. We don’t think anything needs to be specified for this operation; this phase is subject to gNB implementation. Additionally, the gNB can select the suitable power setting for PDSCH transmissions based on the reported CSI(s) as well as other energy saving criteria – we do not think anything needs to be specified for this aspect also.

### Power adaptation phase 3 (CSI measurement/report)

In Phase 3, the gNB indicates to the UE which power offset (powerControlOffset) hypothesis it plans to use for future CSI measurement/report. This can be useful for cases where CSI based on the multiple power offset hypotheses is triggered occasionally and gNB still likes to use one setting (single power offset) for regular CSI reporting.  Study of this aspect is captured in the 2nd RAN1 agreement highlighted below.

Agreement

Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that

  - Whether there is a need for transition time per adaptation (for UE)

  - Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

Our understanding of the scope of this agreement is dynamic signaling since RRC reconfiguration of a PDSCH-to-CSI-RS offset for a CSI-RS resource is already possible in today’s spec. Dynamic signaling can be either through DCI or MAC-CE. However, we think that DCI is a straightforward signaling mechanism with the least amount of standardization effort. For this we think that DCI 0_1/0_2 could be used together with a specific trigger state configuration in which a single hypothesis is configured. This would be a similar approach as in phase 1, except that instead of a list of hypotheses, a single powerControlOffset value is configured. When the UE receives one of these DCIs that point to a single-hypothesis in a trigger state, it would understand that CSI report is requested based on the single hypothesis.

Figure 1 below depicts what is needed for the different phases:

Figure 1: depicts how different powerControlOffset hypotheses are configured in trigger states and indicated by DCI

For power domain adaptation, for aperiodic CSI reporting, support configuration of one or more powerControlOffsets within a trigger state.

When a UE receives DCI with a trigger state indicating multiple powerControlOffsets, the UE transmits one CSI report including CSI results corresponding to each of the powerControlOffsets.

When a UE receives DCI indicating a trigger state including only one powerControlOffset, the UE measures and reports CSI according to that powerControlOffset.

## Spatial domain energy saving enhancements

The energy consumed by the multitude of transceiver chains typical to gNBs’ operations stands for a major part of total consumed network energy. Not all UEs in connected mode benefit from transmissions by all activated transceiver-related circuits, e.g., when the UEs are close enough to the gNB, or when the load is low or medium and thus the capacity which is provided by the higher number of antennas is underutilized. Spatial adaptations (turning off transceivers) can help in achieving major energy savings. At RAN1#112, the following agreements were made relevant to spatial domain adaptations:

Agreement

For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases

- Type 1: all antenna elements associated to a logical antenna port is disabled/enabled

- Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

Agreement

For spatial element adaptation, further study the following

  - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern

    - FFS: Details on how the association is done

  - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns

    - FFS: Details on how the association is done

  - FFS: Details on the definition of “spatial adaptation patterns”

Agreement

For spatial element adaptation, further study the following

  - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern

  - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern

    - FFS: Details of sub-configuration

Agreement

For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g.

  - FFS: gNB indicates to UE which CSI(s) the UE shall report

  - FFS: the UE selects which CSI(s) are reported

  - FFS: multiple CSI(s) are reported in a joint CSI report

  - FFS: Overhead reduction for multiple CSI(s)

Note: UE complexity needs to be taken into account.

Agreement

Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that

  - Whether there is a need for transition time per adaptation (for UE)

  - Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

Like the power domain adaptations, we consider three phases to the spatial domain adaptations:

- Phase 1(CSI measurement/report): UE measures and reports CSI on multiple different spatial element adaptation patterns

- Phase 2: gNB selects a pattern amongst the different reported ones to use for future CSI measurement and reports

- Phase 3(CSI measurement/report): gNB dynamically indicates to UE about the spatial element pattern used for the purpose of future CSI measurements and reports

### Type-1 spatial domain adaptations

gNBs with active antenna systems generally employ many antenna elements arranged in subarrays as exemplified below:

Figure 1: Exemplary antenna arrangement of a gNB with an active antenna system including subarrays of four antenna elements with two polarizations.

Each subarray is typically connected to two transceiver chains, one per polarization.

Figure 2: A typical setup where each polarization of the subarray is connected to a separate Rx/Tx chain.

It is quite common that such gNBs are equipped with 64 and above such Rx/Tx chains, and more are foreseen in the future, especially at higher frequencies. The energy consumed by the multitude of these transceiver chains stands for a major part of total consumed network energy.

A need for increasing number of transceiver chains is foreseen in gNBs in the future, especially at higher frequencies.

For CSI feedback, this also results in a higher number of reference signal transmissions such as CSI-RSs which in turn, due to excessive number of radio wakeups, is quite energy consuming.

For efficient CSI feedback, increased number of transceiver chains results in a higher number of energy consuming components and reference signal transmissions.

Not all UEs in connected mode benefit from all these transmissions and activated transceiver-related circuits, e.g., when the UEs are close enough to the gNB, or when the load is low or medium and thus the capacity which is provided by the higher number of antennas is underutilized. Savings can be achieved by muting a portion of the transceivers whereby the involved circuitry is turned off. Which portion that is muted depends on different deployments, loads, and UE coverage scenarios and therefore it is necessary for the gNB to dynamically adapt the muting pattern.

Figure 3: Different transceiver muting patterns depending on deployment, load, and coverage of UEs.

There is a tradeoff between energy saving gains and UE performance loss. To avoid recurrent reconfigurations and excessive UE performance loss caused by transceiver muting, it is necessary for the gNB to acquire knowledge of what performance the different muting patterns would result in prior to the actual transceiver muting decision. This requires that the UE can report not only CSI for current transceiver configuration, e.g., 32 chains, but also CSI for other candidate configuration(s), e.g., 16, 8, or 4.

To avoid recurrent reconfigurations, it is necessary for the gNB to acquire knowledge of what performance the different muting patterns would result in prior to the actual transceiver muting decision.

When it comes to CSI-RS transmission, irrespective of number of physical antenna subarrays, 3GPP has defined up to 32 CSI-RS antenna ports where each port is typically mapped to a subarray. As per current specifications, it is possible to configure the UE with multiple CSI-RS configurations corresponding to the different transceiver muting layouts and ask the UE to report the CSI for the different configurations. For example, besides configuring a 32-ports CSI-RS resource which is transmitted from a full panel incorporating all transceiver chains, the gNB can also configure the UE with a 16-ports CSI-RS resource which is transmitted from half of the panel. The gNB can then get CSI feedbacks for both 32 ports (full panel) and 16 ports (half panel) and derive the impact on UE performance in case half of the panel is turned off and whether it is suitable to do so.

The issue with configuring the UE with multiple CSI-RS resources employing different number of ports is that it will result in considerable overhead both in terms of gNB energy consumption caused by extra transmissions and resource wastage because of extra time/frequency resource occupation. Furthermore, the UE CSI-RS measurement effort increases for each of these configurations. Note that the examples above only addressed full vs half panel configurations, whereas in reality (as exemplified in Figure 3) there could be other constellations/layouts of interest as well which would contribute to even more CSI-RS configurations and potentially exceed UE capabilities. RRC reconfigurations are currently possible, however they entail signaling overhead and are quite slow for both the network and the UE.

Reference signal reconfigurations via RRC is slow and leads to excessive energy consumption.

One way to avoid the time/frequency resource wastage and extra transmissions is to configure the UE with multiple CSI-RS resources that partially overlap each other on the time/frequency resource grid. This is however possible only to a certain extent where the same scrambling sequence and the same code division multiplexing (CDM) type are used in the overlapping REs. For example, assume that the gNB is operating with a 32-port configuration but needs to have assessment from the UE on 16- and 8-port configurations before the actual configuration change (i.e., before muting of the transceivers). According to 3GPP TS 38.211 (table 7.4.1.5.3.1), e.g., rows 8, 12, and 17 can be used for this purpose as they all employ the same cdm-Type.

Figure 4: highlights 32-/16-/8- port configurations from TS 38.211 Table 7.4.1.5.3-1 where the same CDM-type is used

As an example, based on Row 17 of the table above, the 32-port CSI-RS resource will have the following layout on the resource grid when assuming firstOFDMSymbolInTimeDomain = 5, firstOFDMSymbolInTimeDomain2 = 8, and frequencyDomainAllocation = 110101.

Figure 5: 32-port CSI-RS configuration based on Row17 of TS 38.211 Table 7.4.1.5.3-1 with firstOFDMSymbolInTimeDomain = 5, firstOFDMSymbolInTimeDomain2 = 8, and frequencyDomainAllocation = 101011

And the following layout on the resource grid for the 16-port CSI-RS resource (Row 12) if the same time/frequency resources are pointed out as the 32-port configuration, i.e., firstOFDMSymbolInTimeDomain = 5 and frequencyDomainAllocation = 110101.

Figure 6: 16-port CSI-RS configuration based on Row 12 of TS 38.211 Table 7.4.1.5.3-1 where the same REs are used as the 32-port configuration of Figure 5.

And finally, the following layout for the 8-port CSI-RS resource (Row 8) if again the same time/frequency resources are pointed out as the 32-port configuration, i.e., firstOFDMSymbolInTimeDomain = 5 and frequencyDomainAllocation = 000101.

Figure 7: 8-port CSI-RS configuration based on Row 8 of TS 38.211 Table 7.4.1.5.3-1 where the same REs are used as the 32-port configuration of Figure 5 and the 16-port configuration of Figure 6.

As seen above, the 8 CSI-RS ports 3000-3007 are present and use the same sequence and REs simultaneously in all layouts above. Similarly, the 16 CSI-RS ports 3000-3015 are present and use the same sequence and REs in both the 16- and 32-port configurations. Based on this, the gNB could then configure 3 CSI-RS resources as above and receive report from the UE on each one before deciding on which configuration is good enough for serving the UE.

There are however some drawbacks with using multiple configurations with overlapping REs. Mainly, the scheme only allows for going between configurations with different number of ports as in the example above going from 32- to 16-/8-ports. If instead the gNB running a 32-port configuration wants to have assessment on two other configurations where both have 8 ports (e.g., same number but distinct set of physical antennas), the scheme cannot be used since the ports 3000-3007 would not be distinguishable between two 8-port configurations if transmitted over the same REs simultaneously. In this case one of the 8-port configurations would have to use another set of time/frequency resources. Another issue is that there is a predefined layout for certain number of CSI-RS in spec 38.214 which is shown in table below.

For a gNB supporting 8 port CSI-RS with N1=2 and N2=2, if gNB wants to mute left/right half panel, it cannot get the right CSI feedback from UE for the remaining half panel as the layout of the remaining 4 ports CSI-RS from the panel is different from the predefined layout for 4 ports CSI-RS.

Figure 8. Illustration of layouts for muting of 8 port CSI-RS ports vs 4-port CSI-RS in current spec.

Furthermore, there is predefined way in spec to map each port in a N ports CSI-RS resource to antenna elements/branch. Still using Figure 8 as an example, for an 8 port CSI-RS, P0 to P3 shall be mapped to one polarization of antenna elements/branches, while P4 to P7 shall be mapped to another polarization of antenna elements/branches. However, for a 4 ports CSI-RS, P0 to P1 shall be mapped to one polarization of antenna elements/branches, P2 and P3 shall be mapped to another polarization of antenna elements/branches. In other word, even 8 ports CSI-RS layout is N1=4 and N2=1, half panel is off, there are still problems in ports to antenna elements/branches mapping which also result into incorrect CSI reporting.

gNB may need CSI report on muting pattern candidates with same number of ports. E.g., gNB operating with 32-ports may want to assess one of multiple 8-port configuration candidates (mapped to different antennas) before muting.

In current specifications, there are assumptions about layout for certain number of ports of one CSI-RS resource and numbering of ports for certain number of ports of one CSI-RS resource. Configuring multiple overlapping CSI-RS resources will collide with the assumptions and make the CSI report not usable.

Secondly, even if only different number of ports would be of interest for muting as in the examples in the figures above, the very same set of configuration parameters (e.g., same scrambling, same REs, density, cdm-Type, periodicityAndOffset, etc.) would have to be repeated for every CSI-RS configuration even though there are minimal differences between the configurations. Therefore, it would be good if the UE could provide CSI feedback for different number of CSI-RS ports for the purpose of port (un-) muting in an efficient way employing same time/frequency resources, with efficient configuration, and without the limitation on whether the different muting patterns employ the same number of ports or not.

For the case with overlapping sequences, even if only different number of ports would be of interest for muting, the configuration would be inefficient as the very same set of configuration parameters, with minimal differences, would have to be repeated for every CSI-RS configuration.

Efficient configuration of CSI-RS ports with overlapping time resources for the purpose of port (un-) muting feedback without the limitation on whether the different muting patterns employ different number of ports is beneficial.

When it comes to configuration of CSI reporting for Type-1 adaptations, we think a similar configuration approach as for power domain adaptations can be used. These steps/phases are described below.

#### Type-1 spatial domain adaptations phase 1(CSI measurement/report)

For phase 1, only a single resource set with single CSI-RS resource is needed, e.g., the gNB always transmits a 32 port CSI-RS resource, and the UE measures on various subsets of ports for each muting hypothesis. A UE configured with a CSI-RS resource with N ports should be able to measure and report CSI for one or more subset(s) of the N ports. This corresponds to Alt 1-2) highlighted in the agreements from RAN1#112:

Agreement

For spatial element adaptation, further study the following

  - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern

    - FFS: Details on how the association is done

  - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns

    - FFS: Details on how the association is done

  - FFS: Details on the definition of “spatial adaptation patterns”

For Type-1 spatial element adaptation, each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns (i.e. A1-2 in the RAN1#112 agreement).

For Type-1 spatial element adaptation, a CSI-RS resource set is configured within a CSI Resource Setting (CSI-ResourceConfig) wherein the set contains a single CSI-RS resource associated with one or more spatial element adaptation patterns.

Similar to the power domain adaptations, for the enablement of phase 1, we think that the UE could be configured with multiple hypotheses. In this case a hypothesis corresponds to a CSI-Report sub-configuration in a CSI report configuration. Pointers (list of sub-configuration indices) to the hypotheses could be included in the RRC CSI-AperiodicTriggerState or or other IEs referenced within it. Once the trigger state is triggered by a DCI e.g., DCI 0_1, based on presence of the pointers, the UE would understand that it needs to measure and report CSI corresponding to each of the sub-configurations indicated by the trigger state, see Figure 2. This corresponds to Alt 2-2) highlighted in the agreements from RAN1#112 for the CSI report configuration.

Agreement

For spatial element adaptation, further study the following

  - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern

  - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern

    - FFS: Details of sub-configuration

For Type-1 spatial element adaptation, one CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern (i.e. A2-2 in the RAN1#112 agreement).

#### Type-1 spatial domain adaptations phase 2

In Phase 2, the gNB decides which spatial adaptation hypothesis to use for future CSI reporting. We don’t think anything needs to be specified for this operation; this phase is subject to gNB implementation. Additionally, the gNB can select the suitable spatial pattern for PDSCH transmissions based on the reported CSI(s) as well as other energy saving criteria – we do not think anything needs to be specified for this aspect also.

#### Type-1 spatial domain adaptations phase 3(CSI measurement/report)

In Phase 3, the gNB indicates to the UE which spatial adaptation hypothesis it plans to use for future PDSCH transmissions. Again, we think that DCI is a straightforward signaling mechanism with the least amount of standardization effort and aligned with phase 3 of the power domain adaptations. DCI0_1/0_2 could be used together with a specific trigger state configuration in which a single hypothesis is configured. This would be a similar approach as in phase 1, except that instead of a list of hypotheses, a single index is configured. When the UE receives one of these DCIs that point to a single hypothesis in a trigger state, it would understand that CSI report is requested for only that hypothesis.

Figure 2 below depicts what is needed for the different phases:

Figure 2: depicts how different report config hypotheses are configured for type-1 in trigger states and indicated by DCI

For Type-1 spatial domain adaptation, for aperiodic CSI reporting, support configuration of one or more indicators within a trigger state, where an indicator points to a sub-configuration within a CSI-ReportConfig.

When a UE receives DCI indicating a trigger state with multiple sub-configuration indicators, the UE transmits one CSI report including CSI results corresponding to each of indicated sub-configurations.

When a UE receives DCI indicating a trigger state including only one sub-config indicator, the UE measures and reports CSI according to the indicated sub-configuration only.

For Type 1 spatial domain adaptation, a CSI-Report sub-configuration includes the following information for CSI measurement and reporting

  - A number of antenna ports

  - Indicator(s) of a subset of antenna ports within a codebook

### Type-2 spatial domain adaptations

One inherent aspect of Type-2 spatial element adaptation is that turning off a subset of antennas also affects the transmission power. It shall however be noted that the resulting transmission power changes shall not affect the cell-specific signals as pointed out by the WID:

- Specify the following techniques in spatial and power domains

- Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2]

- Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2]

- Note: Above objectives are only for UE specific channels/signals

- Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

Hence, we can assume that in these symbols/slots used for transmission of cell-specific signals, all antenna elements associated with each logical antenna port should be (re-)enabled.

According to the WID, the enhancements for Type-2 spatial element adaptation are not applicable for SSB and for PDCCH in symbols that UEs monitor for common search spaces (CSS).

However, as CSI-RS is a UE-specific signal, it is in the scope of the WID for the Type-2 adaptations. The associated side effects of power changes when turning off antennas according to the Type-2 scheme for CSI-RS transmission may result in that the RRC configured powerControlOffsetSS becomes incorrect. For the case of periodic CSI-RS, this may introduce complications in the following areas:

- UL Power Control:

For UL power control, either SSB or periodic CSI-RS can be used as a pathloss reference. If periodic CSI-RS is used as the pathloss reference, and the powerControlOffsetSS is incorrect, the pathloss estimated by the UE that is used in the UE Tx calculation becomes inaccurate.

- Beam Failure Recovery (Link Recovery)

For beam failure recovery the radio link quality is monitored based on SSB or periodic CSI-RS. For the case of periodic CSI-RS, the UE measures L1-RSRP and compares it to a threshold after scaling the CSI-RS reception value by the configured value of powerControlOffsetSS which would be different than the actual value as mentioned above.

Based on this discussion, we think the simplest solution is to, like the SSBs, avoid Type-2 adaptation for periodic CSI-RS.

Type-2 spatial element adaptation enhancement is not supported in symbols configured with periodic CSI-RS. I.e., updates to powerControlOffsetSS are not supported for periodic CSI-RS.

Based on the above, in what follows we assume that Type-2 adaptation is only supported in symbols with PDSCH, UE specific PDCCH, or aperiodic CSI-RS. These steps/phases related to Type-2 spatial domain adaptations are described below.

#### Type-2 spatial domain adaptations phase 1(CSI measurement/report)

In contrast to Type-1 spatial domain adaptations where a multi-port CSI-RS transmission would suffice for UE measurement and reporting on different hypotheses, for Type-2 the gNB needs to transmit the CSI-RS resources with different antenna muting patterns in different OFDM symbols. This corresponds to Alt 1-1) highlighted in the agreements from RAN1#112:

Agreement

For spatial element adaptation, further study the following

  - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern

    - FFS: Details on how the association is done

  - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns

    - FFS: Details on how the association is done

  - FFS: Details on the definition of “spatial adaptation patterns”

For Type-2 spatial element adaptation, each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern (i.e. A1-1 in the RAN1#112 agreement).

For this phase we think the simplest approach is to configure an aperiodic CSI-RS resource set within a single CSI Resource setting (single CSI-ResourceConfig) which contains multiple CSI-RS resources, each associated with only one spatial adaptation pattern. By implementation the gNB transmits each different CSI-RS resource in the set with a different spatial element pattern, i.e., different subset of disabled antenna elements. The different CSI-RS resources in the set are configured during different OFDM symbols since only one spatial element pattern can be transmitted at a time.

For Type-2 spatial element adaptation, a CSI-RS resource set is configured within a CSI Resource Setting (CSI-ResourceConfig) wherein the set contains multiple CSI-RS resources each one associated with a spatial element adaptation pattern.

For reporting of CSI for each hypothesis we think the simplest approach is that multiple CSI reporting corresponding to the different spatial element adaptation patterns are configured within a single CSI-ReportConfig, see Figure 3. This corresponds to Alt 2-2) highlighted in the agreements from RAN1#112:

Agreement

For spatial element adaptation, further study the following

  - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern

  - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern

    - FFS: Details of sub-configuration

However, we note that in contrast to Type-1, there appears to be no strong need to introduce sub-configurations, since the multiple Type-2 spatial element adaptation patterns are associated with the different CSI-RS resources in the CSI-RS resource set as discussed above. Hence, while we support Alt-2, since we think it can be used as a general framework for power domain adaptation and Type-1 and Type-2 spatial domain adaptation.

For Type-2 spatial element adaptation, support a variant of A2-2 in the RAN1#112 agreement in which one CSI report configuration is associated with multiple spatial adaptation patterns, but sub-configurations are not used. The association can be based on current specifications by configuration of a CSI-RS resource set in CSI-ResourceConfig with multiple CSI-RS resources.

Based on the above, for Type-2, the UE does not need to know what subset of antennas is disabled; it simply reports CSI for each different CSI-RS resource in the resource set associated with the CSI-ReportConfig. What is required, however, is to inform the UE that multiple reports are needed, one per CSI-RS resource in the CSI-RS resource set. To enable this, one way could be to add a new CSI reporting quantity, e.g., through an additional RRC parameter reportQuantity2 = ‘Multi-RI-PMI-CQI’. This is needed since in today’s spec, the closest CSI reporting quantity is ‘CRI-RI-PMI-CQI’ in which case the UE would report only one CSI corresponding to the “preferred” CSI-RS resource in the resource set, with CRI pointing to the preferred CSI-RS resource. For the purposes of spatial element adaptation, where multiple CSI reports are needed, there is no need for the UE to report CRI since it would report a separate CSI for each resource in the set, in the same order. With this approach, the single CSI-ReportConfig can be associated with a single aperiodic CSI trigger state as in the current specifications. The main difference is that the parameter reportQuantity2 = ‘Multi-RI-PMI-CQI’ within the CSI-ReportConfig informs the UE to report multiple CSIs in case the CSI-RS resource set has multiple CSI-RS resources. If a set has only one CSI-RS resource, then the UE would use the legacy parameter reportQuantity.

For Type-2 spatial element adaptation, support a new RRC parameter within CSI-ReportConfig that indicates to the UE that it should report multiple CSIs in case a trigger state points to a CSI-RS resource set within CSI-ResourceConfig that contains multiple CSI-RS resources. For example, the new parameter could be named reportConfig2 with new value ‘multi-RI-PMI-CQI.’ The UE uses the legacy parameter reportConfig if the trigger state points to a CSI-RS resource set with only a single CSI-RS resource.

#### Type-2 spatial domain adaptations phase 2

We don’t think anything needs to be specified for this phase; phase 2 is subject to gNB implementation.

#### Type-2 spatial domain adaptations phase 3(CSI measurement/report)

As discussed previously, for Type-2 adaptation, if a subset of antennas associated with a logical antenna port is turned off, then the power offset between CSI-RS and SSB becomes larger than the offset configured by the RRC parameter powerControlOffsetSS within a configured CSI-RS resource since the SSB power does not change. We observe that for the above approach for CSI-RS configuration and CSI reporting for Type-2, each spatial element adaptation pattern corresponds to a different CSI-RS resource. Hence by implementation, the gNB can configure powerControlOffsetSS differently for each CSI-RS resource in the CSI-RS resource set corresponding to each pattern. Then, when aperiodic CSI reporting based on aperiodic CSI-RS is requested after adaptation, the gNB can simply trigger CSI reporting based on a single CSI-RS resource with the correct configuration of powerControlOffsetSS.

Figure 3 below depicts what is needed for the different phases:

Figure 3: depicts how different report config hypotheses are configured for type-2 in trigger states and indicated by DCI

# Conclusion

In previous sections, the following observations and proposals were made:

Observation 1	A need for increasing number of transceiver chains is foreseen in gNBs in the future, especially at higher frequencies.

Observation 2	For efficient CSI feedback, increased number of transceiver chains results in a higher number of energy consuming components and reference signal transmissions.

Observation 3	To avoid recurrent reconfigurations, it is necessary for the gNB to acquire knowledge of what performance the different muting patterns would result in prior to the actual transceiver muting decision.

Observation 4	Reference signal reconfigurations via RRC is slow and leads to excessive energy consumption.

Observation 5	gNB may need CSI report on muting pattern candidates with same number of ports. E.g., gNB operating with 32-ports may want to assess one of multiple 8-port configuration candidates (mapped to different antennas) before muting.

Observation 6	In current specifications, there are assumptions about layout for certain number of ports of one CSI-RS resource and numbering of ports for certain number of ports of one CSI-RS resource. Configuring multiple overlapping CSI-RS resources will collide with the assumptions and make the CSI report not usable.

Observation 7	For the case with overlapping sequences, even if only different number of ports would be of interest for muting, the configuration would be inefficient as the very same set of configuration parameters, with minimal differences, would have to be repeated for every CSI-RS configuration.

Observation 8	Efficient configuration of CSI-RS ports with overlapping time resources for the purpose of port (un-) muting feedback without the limitation on whether the different muting patterns employ different number of ports is beneficial.

Observation 9	According to the WID, the enhancements for Type-2 spatial element adaptation are not applicable for SSB and for PDCCH in symbols that UEs monitor for common search spaces (CSS).

Proposal 1	For power domain adaptation techniques, dynamic updates for powerControlOffsetSS are not supported.

Proposal 2	For power domain adaptation, for aperiodic CSI reporting, support configuration of one or more powerControlOffsets within a trigger state.

Proposal 3	When a UE receives DCI with a trigger state indicating multiple powerControlOffsets, the UE transmits one CSI report including CSI results corresponding to each of the powerControlOffsets.

Proposal 4	When a UE receives DCI indicating a trigger state including only one powerControlOffset, the UE measures and reports CSI according to that powerControlOffset.

Proposal 5	For Type-1 spatial element adaptation, each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns (i.e. A1-2 in the RAN1#112 agreement).

Proposal 6	For Type-1 spatial element adaptation, a CSI-RS resource set is configured within a CSI Resource Setting (CSI-ResourceConfig) wherein the set contains a single CSI-RS resource associated with one or more spatial element adaptation patterns.

Proposal 7	For Type-1 spatial element adaptation, one CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern (i.e. A2-2 in the RAN1#112 agreement).

Proposal 8	For Type-1 spatial domain adaptation, for aperiodic CSI reporting, support configuration of one or more indicators within a trigger state, where an indicator points to a sub-configuration within a CSI-ReportConfig.

Proposal 9	When a UE receives DCI indicating a trigger state with multiple sub-configuration indicators, the UE transmits one CSI report including CSI results corresponding to each of indicated sub-configurations.

Proposal 10	When a UE receives DCI indicating a trigger state including only one sub-config indicator, the UE measures and reports CSI according to the indicated sub-configuration only.

Proposal 11	For Type 1 spatial domain adaptation, a CSI-Report sub-configuration includes the following information for CSI measurement and reporting

a.	A number of antenna ports

b.	Indicator(s) of a subset of antenna ports within a codebook

Proposal 12	Type-2 spatial element adaptation enhancement is not supported in symbols configured with periodic CSI-RS. I.e., updates to powerControlOffsetSS are not supported for periodic CSI-RS.

Proposal 13	For Type-2 spatial element adaptation, each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern (i.e. A1-1 in the RAN1#112 agreement).

Proposal 14	For Type-2 spatial element adaptation, a CSI-RS resource set is configured within a CSI Resource Setting (CSI-ResourceConfig) wherein the set contains multiple CSI-RS resources each one associated with a spatial element adaptation pattern.

Proposal 15	For Type-2 spatial element adaptation, support a variant of A2-2 in the RAN1#112 agreement in which one CSI report configuration is associated with multiple spatial adaptation patterns, but sub-configurations are not used. The association can be based on current specifications by configuration of a CSI-RS resource set in CSI-ResourceConfig with multiple CSI-RS resources.

Proposal 16	For Type-2 spatial element adaptation, support a new RRC parameter within CSI-ReportConfig that indicates to the UE that it should report multiple CSIs in case a trigger state points to a CSI-RS resource set within CSI-ResourceConfig that contains multipe CSI-RS resources. For example, the new parameter could be named reportConfig2 with new value ‘multi-RI-PMI-CQI.’ The UE uses the legacy parameter reportConfig if the trigger state points to a CSI-RS resource set with only a single CSI-RS resource.

# References

RP-223540, 3GPP TSG RAN Meeting #98e, Electronic Meeting, December 12-16, 2022; New WID: Network energy savings for NR