3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303761

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.11.3

Source:	Ericsson

Title:	L1 signal design and procedure for low power WUS

Document for:	Discussion

# 1	Introduction

In this contribution, we discuss the signal design and procedures of WUS/WUR according to the SID and previous agreements.

# 2	General design criteria

From a network vendor and ecosystem perspective, it is important that the gNB should be able to transmit the LP-WUS using existing gNB hardware and not require any new emissions or compliance requirements. Otherwise, it is difficult to enable widespread WUS support in existing deployments.

It should be possible to generate LP-WUS transmissions using existing gNB hardware and not trigger any new emissions or compliance requirements.

In order to not negatively impact network capacity, it is important that the LP-WUS can be multiplexed with other NR transmissions in time and frequency, and that any unused LP-WUS resources can be reused for dynamic scheduling.

It should be possible to multiplex the LP-WUS with other NR transmissions in time or frequency domain without causing interference.

It should be possible to reuse any unused LP-WUS time and frequency resources for other transmissions.

Regarding coverage, we think it is important from a system perspective that the coverage is not worse than for existing Paging PDCCH. Note that Rel-17 PEI is transmitted using PDCCH and DCI format 2_7.

Target the same coverage for LP-WUS as for Paging PDCCH.

# 3	WUS subcarrier spacing

Based on RAN1#112 agreements, it needs to be further studied whether subcarrier spacing (SCS) of WUS and other NR transmissions can be the same or different:

Agreement For MC-ASK or MC-FSK waveform generation, SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with, study whether SCS can be different.

In general, the support of mixed numerologies in the same band requires careful considerations to minimize inter-subcarrier interference and performance degradation. To avoid further increasing the complexity of gNB and UE and better coexistence with different NR transmissions, it is desired to have the same SCS for WUS and other NR transmissions (in the same configured DL BWP).

Another discussion point is that whether using a higher WUS SCS (compared to other transmissions) can be enable the multi-bit OOK-WUS generation (within one OFDM symbol). For example, if the SCS for NR transmissions is 30 kHz (nominal SCS), WUS may have 60 kHz SCS by modulating every other subcarrier for WUS. Then, in the time domain, WUS is repeated within one OFDM symbol with the nominal SCS (as illustrated in Figure 1). To transmit two OOK bits in one OFDM symbol, time-domain masking (setting samples to zero) can be applied to the first half or second half of the generated samples at the output of IFFT. However, with a single IFFT block, this approach results in the coexistence issue and impact on other NR transmissions due to the time-domain operation.

With a single IFFT block, using a larger SCS for WUS for generating multi-bit OOK signal results in the coexistence issue and impact on other NR transmissions due to the time-domain operation (e.g., masking parts of the samples).

Figure 1: Generating multi-bit OOK WUS by using every K subcarriers.

To multiplex multi-bit OOK WUS with other NR transmissions, one approach could be to use two IFFT blocks. In this case, WUS SCS needs to be an integer multiple of the SCS used for other transmissions (i.e., WUS SCS=M*NR SCS, M is integer). However, gNB complexity significantly increases dur to multiple IFFTs. Therefore, different subcarrier spacings for WUS and other NR transmissions is results in a limitation for FDM multiplexing between WUS and other NR transmission.

If WUS SCS is different from SCS used for other NR transmissions, FDM multiplexing between WUS and other NR transmissions cannot be enabled without significant gNB impact.

# 4	OOK generation with OFDM

In RAN1#112, it was agreed to further study options OOK-1 to OOK-4.

### 4.1	Option OOK-1: Single-bit in 1 OFDM symbol

OOK WUS can be generated by transmitting one bit (0 or 1) per OFDM symbol. In this case, to generate “1” WUS subcarriers carry random data (e.g., with QPSK modulation) while “0” is generated by having zero-power WUS subcarriers, as illustrated in Figure 2.  To simplify receiver complexity, Manchester coding can be used, where ON and OFF-symbols are transmitted in pairs of either (ON, OFF) or (OFF, ON). This way the receiver can compare the received energy in the first and second half of the pair to make a decision.

Figure 3 shows the time domain of the generated WUS signal (only WUS is shown) over one slot. Also, the power spectral density (PSD) of the transmitted signal is shown considering both WUS (with random QPSK input) and non-WUS (with 64 QAM input) transmissions. As we can see, single-bit OOK generation of ON/OFF signal with random population of subcarriers (e.g., with QPSK) is straightforward with minimum impact on the OFDM transmitter.

Figure 2: Single-bit OOK generation using OFDM with random QPSK subcarriers.

Figure 3: Time domain representation (only WUS) and PSD of single-bit OOK generated with OFDM (WUS subcarriers populated with random QPSK).

- OOK WUS generation with single bit per OFDM symbol is straightforward with minimum impact on the OFDM transmitter.

Consider the case of one (coded) bit per OFDM symbol as baseline for evaluation of OFDM based OOK waveforms.

### 4.2	Option OOK-2: Parallel M-bit OOK in frequency domain

In this method, WUS subcarriers are divided into M segments possibly with guard-bands in-between segments and/or around the entire WUS subcarriers. Each segment represents one bit by modulating all subcarriers of the segment (for transmitting 1) or having zero power on all those subcarriers for transmitting 0. On the receiver side, parallel OOK chains are needed to filter out different segments. Figure 4 illustrates the parallel M-bit OOK generation method. Also,  Figure 5 shows the time domain of the generated WUS signal (only WUS is shown) over one OFDM symbol and the PSD of the transmitted signals.

Figure 4: Parallel M-bit OOK generation in frequency domain.

Figure 5: Time domain representation (only WUS)  and PSD of parallel multi-bit OOK in frequency domain generated with OFDM (coded bits in one OFDM symbol: [0,1,0,1]).

### 4.3	Option OOK-3: Multi-tone single-bit OOK

In this method, WUS subcarriers are divided into L segments possibly without guardband in-between segments. For each UE/UE group indication, one subcarrier of each segment is used which is to the UE/UE group (e.g., the first subcarrier of each segment is used for targeting a specific UE/UE group). With M segments each containing N subcarriers, it is possible to indicate N different UEs/UE groups while allocating L subcarriers for each WUS transmission. Figure 6 illustrates the multi-tone single-bit OOK generation method. Also,  Figure 7 shows the time domain of the generated WUS signal (only WUS is shown) over one OFDM symbol and the PSD of the transmitted signals. Here, four segments are considered and only one UE/UE group is targeted.

Figure 6: Multi-tone single-bit OOK generation block diagram.

Figure 7: Time domain representation (only WUS) and PSD of multi-tone single-bit OOK generated with OFDM.

### 4.4	Option OOK-4: Transform M-bit OOK in time domain

In RAN1#112, the option of transmitting multiple OOK bits within one OFDM symbol was also discussed. One approach for this is least square (LS) waveform fitting based on ‎[3]. Another approach is based on DFT precoding.

Approach 1: Waveform fitting approach

Figure 8: Multi-bit OOK generation with least square waveform approximation approach.

Waveform fitting approach can be described as follows:

Let K be the number of subcarriers used for WUS generation. As an example, K subcarriers are used for frequency domain input to the IFFT block denoted by  X l, l=a,…,a+K-1,  with a being the index of the first subcarrier. The output of an N-point IFFT is the time domain samples of length N:  Yn, n=0,…,N-1. The objective is to find optimal input values X for which the output Y is as close as possible to a desired output Ytarget. One example of Ytarget is an OOK sequence. In addition, there can be some constraint on the input values X from implementation and power spectral density perspective.

Xopt=argminX  L (Y,Ytarget)
    -  subject to constraints on values of X

where L (.) is a loss/error function.

In this case, a least square (LS) waveform fitting approach can be used which is summarized in Figure 8Figure 3.  Figure 9 shows the time domain OOK symbols representing WUS waveform [0,1,0,1] within one OFDM symbol (including cyclic prefix) generated using this approach. 12 PRBs for WUS and 2048 IFFT size are assumed for this case.

In addition, Figure 9 shows PSD of WUS signal and other non-WUS transmissions for the waveform fitting approach.

Figure 9: Time domain representation (only WUS, one OFDM symbol) and PSD of multi-bit OFDM-based OOK generation with waveform fitting (coded bit in one OFDM symbol: [0,1,0,1]).

Approach 2: DFT based approach

Another possible approach for generating a multi-bit OOK waveform is based on DFT, as summarized in Figure 10. In this case, an M-point DFT is used to convert a set of time samples to frequency domain equivalence on WUS subcarriers used as inputs to the IFFT.

Figure 11 shows the time domain OFDM-based OOK generation representing WUS waveform [0,1,0,1] within one OFDM symbol (including cyclic prefix) for the DFT based approach.  Also, it shows PSD of WUS signal and other non-WUS transmissions.

Figure 10: Multi-bit OOK generation with DFT based approach.

Figure 11: Time domain representation (only WUS, one OFDM symbol) and PSD of multi-bit OFDM-based OOK generation with DFT based approach (codded bit in one OFDM symbol: [0,1,0,1]).

Note that with OOK-4 (DFT-s or waveform fitting approaches) the gNB has to support additional DFT (or another transform for LS approach) and input of IFFT may not fall within existing constellations (e.g., M-QAM). This increases gNB complexity. Mapping the generated frequency domain values (before IFFT) to existing sequences/QAM modulations should considered for this approach to reduce the impact on gNB.

M-bit OOK WUS generation (with multiple bits per OFDM symbol) using waveform fitting or DFT-based approach increases gNB complexity compared to single bit per OFDM symbol OOK WUS generation.

For M-bit OOK generation using waveform fitting or DFT-based approach, consider mapping of generated frequency domain values (before IFFT) to existing sequences and QAM constellations (e.g., 64-QAM) to minimize the gNB impact.

As can be seen from the time domain signals, generating M-bit OOK (with OOK-4 method) may increase the PAPR which has impacts on the gNB. Hence, the PAPR aspect needs to be further studied.

For M-bit OOK generation with DFT-s or waveform fitting (i.e., least square), the PAPR aspects should be studied.

# 5	FSK generation with OFDM

In RAN1#112, the following was agreed:

Agreement For M-bit MC-FSK generation study further the following options - Option FSK-1: N SCs of LP-WUS are separated to M pairs of segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - in a pair of segments one segment is modulated, other segment is zero power (from base-band point of view) - Option FSK-2: N SCs of LP-WUS are separated to 2^M segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - one segment from 2^M segments is modulated, other segments of SCs are zero power (from base-band point of view) - M >0 - N >1 - Study how to generate segment in time domain, e.g. OOK-1 or OOK-4 - Other options are not precluded.

### 5.1 Option FSK-1: M pairs of segments in frequency domain for M-bit information

To transmit M-bit information, FSK WUS can be generated by separating N SCs to M pairs of segments in one OFDM symbol. Each pair of segments carries one bit information, and one segment in one pair is modulated while the other segment is zero power based on the bit carried is 1 or 0. Each segment comprises one subcarrier or multiple contiguous subcarriers.

Figure 12:  FSK signal generation of M pairs of segments in frequency domain for M-bit WUS transmission.

Figure 13 shows the power in frequency domain before OFDM modulation, the other transmissions and WUS transmission are QPSK symbols. Each segment comprises several contiguous subcarriers. 4 pairs, 8 segments and one certain guard band between adjacent segments are shown.

Figure 13: Power allocation for 4 pairs of segments in frequency domain.

Considering the receiver, each segment is filtered to detect and decide if there is WUS transmission in time domain. Let’s see the time domain signal of one segment. To be simple, take M=1 for example in the following analysis. Based on the number of SCs per segment, there are two options that are each segment comprises one subcarrier or multiple contiguous subcarriers. Assume the symbol(s) mapped on the subcarriers for WUS transmission is fixed QPSK symbols. When M=1, WUS can be considered as 2FSK modulation signal. The time domain signal of WUS after OFDM modulation is shown below.

Figure 14: Discontinuous phase FSK modulation caused by CP insertion.

WUS is phase discontinuous FSK modulation caused by CP insertion, which causes power leakage and FSK demodulation performance. To solve this problem, we could compensate some certain phase shift before OFDM modulation. The complex sequence mapped on L contiguous subcarriers for i-th OFDM symbol can be denoted as Bi=bi1,bi2,…,biL. The corresponding subcarrier index vector is denoted as Ki=ki1,ki2,…,kiL. As a result, the WUS signal for i-th OFDM symbol is,

swusin=1NIFFTl=1Lbilej2πnkilNIFFT,n=0,1,…,NIFFT

Let the length of CP corresponding to WUS transmission spanning I OFDM symbols be denoted by a constant vector Ncp={Ncp1,Ncp2,…,NcpI}, where Ncpi,i∈{1,2,…,I} is the CP length of the i-th OFDM symbol in one WUS transmission. To eliminate the phase discontinuous problem caused by CP, the complex sequence mapped on L contiguous subcarriers for i+1-th OFDM symbol should meet,

Bi+1=ej∆Θi+1.*Bi=[ej∆θi+11bi1,ej∆θi+12bi2,…,ej∆θi+1LbiL]

Where the phase shift element is,
∆θi+1l=2πNcpi+1ki+1lNIFFT

The figure shows that the phase is continuous after CP insertion in case of bit 0 and bit 1 are modulated on single subcarrier or multiple subcarriers.

Figure 15: Continuous phase FSK modulation based on OFDM system.

Phase discontinuous FSK modulation caused by CP, causes power leakage and higher requirement of timing synchronization in receiver.

For FSK-1, compensation of the phase shift caused by CP before OFDM modulation may be needed.

When the subcarrier is single, the envelope is flat. When the subcarriers are multiple, the sequence impacts the envelop of the time domain WUS signal. The below figure shows the time domain signal of WUS when the number of contiguous subcarriers is 8, the complex sequence is ones or some 256QAM symbols.

Figure 16: Envelop of time domain WUS signal depends on the sequence mapped on multiple subcarriers.

For the case of each segment comprises multiple subcarriers, the sequence mapped on the subcarriers impacts the envelop of time domain WUS signal. For the energy detection method such as envelop detector in the receiver, flat envelop signal should be designed to improve the detection performance.

For FSK-1, the complex sequence mapped on the subcarriers for WUS transmission should be studied further.

### 5.2	Option FSK-2: 2^M segments in frequency domain for M-bit information

Figure 17: FSK signal generation of 2^M pairs of segments in frequency domain for M-bit WUS transmission.

To transmit M-bit information, FSK WUS can be generated by separating N SCs to M2 segments in one OFDM symbol. Each segment carries one kind of bit group, such as segment 1 has power only when the M-bit is ’00..0’, otherwise it has no power. Each segment comprises one subcarrier or multiple contiguous subcarriers.

FSK-2 needs more segments than FSK-1 when M≥3 to carry same information.

# 6	OFDMA-based WUS

In this section, we briefly discuss OFDM-based WUS generation using existing NR sequences. For example, in DL, there exist PSS and SSS as part of the synchronization signal block (SSB) used e.g. for time and frequency synchronization during the cell search procedure. PSS is generated based on BPSK modulated m-sequence of length 127, while SSS is generated based on BPSK modulated Gold-sequence of length 127. When mapping onto time and frequency domain resources, each PSS and SSS in an SSB occupies 127 subcarriers in one OFDM symbol. In UL, low PAPR Zadoff-Chu sequences are used as root sequences, e.g., for generating sequences for PUCCH format 0 and format 1 as well as SRS, where different sequence lengths can be generated depending on the configurations.

The above sequences have “good” auto-correlation and cross correlation properties, making it possible to perform correlation-based detection both in time and frequency domain, and can thus be considered as potential candidates to use as Rel-18 LP-WUS. Given that WUS transmission is in DL, it is especially reasonable to consider existing DL NR sequences. In Section 7, detection performance of OFDM-based WUS using PSS/SSS are provided.

# 7	Link-level evaluations

Our main assumptions for the link-level evaluations are summarized in Table 1.

Table 1: General simulation parameters for link-level evaluations.

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2.6 GHz |
| Channel | TDL-C |
| SCS | 30 kHz |
| Delay spread | 300 ns |
| UE speed | 3 km/h |
| UE Rx antennas | 1 for WUR |
| WUR sampling rate | 8 Mbps |
| WUS bandwidth | For OOK WUS: - 5 MHz (for 30 kHz SCS: 12 PRBs+ ~ 1 PRB guard band each side of WUS) For SSS-based WUS: - 3.81 MHz (for 30 kHz SCS: 127 subcarriers for SSS) |
| Number of ADC bits | 8 |
| Receiver filter | 3rd order Butterworth, filter BW= 1.2*WUS BW (using filter BW larger than WUS BW compensates the frequency error (e.g., 50 ppm)) |
| Non-WUS transmissions | Adjacent channel interference from other NR transmissions is considered. |
| Max. false alarm due to noise or random symbols | 0.1% |
| Frequency offset (ppm) | For OOK WUS: - 50 ppm maximum frequency error For SSS-based WUS - 10 ppm maximum frequency error |
| WUS payload/sequence | For payload based OOK WUS: - See payload size and CRC bits in respective results below For sequence-based OOK WUS: - 14 OFDM symbols: 0 1 0 1 1 0 0 1 1 0 1 0 1 0 - 6 OFDM symbols: 0 1 1 0 1 0 - 4 OFDM symbols: 0 1 1 0 For SSS-based WUS: - 127 subcarriers of SSS sequence in frequency-domain mapped onto 1, 2, or 4 OFDM symbols |
| Rx approach | Sequence based: Correlation based detector with desired WUS sequence known a priori to UE and detection of known sequence indicates wake-up. Payload based: based on Manchester decoding. |
| PDCCH as baseline for coverage comparison | 48 PRBs, 2-symbol CORESET, AL {8, 16} TDL-C channel, {1-Rx, 2-Rx, 4-Rx} |

## 7.1	WUS based on OOK

### 7.1.1 sequence based

Figure 18 shows initial link performance evaluation results of sequence-based OOK WUS detection. Here OOK WUS sequence is generated based on OOK-1 (See Section 4). Evaluation assumptions are given in Table 1. Evaluations show missed detection rate (MDR) for a correlation-based detector with FAR target of 0.1%.

Figure 18: Missed detection performance of OOK based WUS.

Impact of other cell interference

Similar to the case of payload-based OOK WUS, the impact of adjacent cell interference (ACI) onto WUS subcarriers can be shown. In Figure 19, we show the missed detection performance impact due to other-cell interference where, when the interference is present, the signal power relative to the interference power ratio (SIR) varies from -3 dB to 0 dB. As we can see, the impact can be significant when the interference is stronger than the signal. The impact can however be reduced with longer duration of OOK WUS in time domain.

Figure 19 Missed detection performance of OOK based WUS with different SIR assumptions.

### 7.1.2 Payload based

For payload-based detection, an error detection code is needed if a certain error detection probability (or false alarm probability) needs to be satisfied. With an n-bit CRC added to the data, the error detection probability is (1-2-n) thus the false alarm probability is 2-n. For example, considering a false alarm probability target 10-3, at least a 10-bit CRC is needed.

#### 7.1.2.1 OOK with Single bit per OFDM symbol

Here, we show the link-level performance of single-bit OOK and evaluate the impact of various parameters.

Raw BER results

SSS sequence vs. random QPSK

For single-bit OOK approach, subcarriers corresponding to ON symbols can be populated with different values, for example, random QPSK or a sequence. Figure 20 shows the BER performance of random QPSK and SSS sequence for modulating WUS subcarriers (127 subcarriers are considered here). As expected, for the same signal power the BER performances of these two cases are the same as the OOK receiver only uses amplitude (or energy) of the received signal for WUS detection.

Figure 20: SSS sequence vs. random QPSK for ON symbols (127 WUS subcarriers).

For modulating ON symbols in OOK WUS, a sequence and random QPSK provide the same BER performance for the same signal power.

Other-cell interference:

Figure 21 shows the impact of adjacent cell interference on the BER. In this result, the signal-to-interference ratio (SIR) varies from 6 dB to -3 dB. As we can see, the impact of adjacent cell interference can be significant when the interference is stronger than the signal (e.g., SIR= -3 dB).

Figure 21: Impact of adjacent cell interference on BER.

Adjacent cell interference can result in a significant coverage degradation when the interference is stronger than the signal.

ADC bits:

Here we evaluate the impact of ADC bit width on the coverage. Increasing ADC bits is beneficial for reducing the quantization error thus improving the coverage. Also, a larger ADC bit width improves the dynamic range which is the range of signal amplitude that ADC can resolve/detect. For an N bit ADC, the dynamic range is around 6.02×N [dB]. Therefore, every additional ADC bit expands the dynamic range by 6 dB. Moreover, increasing ADC bit width improves interference mitigation. However, the receiver power consumption increases with more ADC bits.

Figure 22 shows the impact of ADC bit width on the link performance. This result shows that the performance of 1-bit ADC is extremely poor.

Figure 22: Impact of number of ADC bits on BER.

Increasing ADC bit width is beneficial in terms of coverage, interference mitigation, and dynamic range of signal reception.

The coverage performance of single-bit ADC is extremely poor.

WUR should support multi-bit ADC (e.g., bit width 4 or 8) to improve its sensitivity.

UE speed:

In Figure 23 we evaluate the impact of UE speed on the link performance for the Rural scenario (700 MHz carrier frequency). As we can see, the coverage performance slightly degrades (~ 1 dB) when the UE speed increases from 3 km/h to 120 km/h. Note that the impact of UE speed on the coverage is small as the channel coherence time with 120 km/h is around 5 ms (i.e., several slots) which does not cause any considerable fast fading effect.

Channel coherence time is given by Tc≈0.42fm, where  fm=vcFo,  v is the speed of UE, and c=3×108 m/s is the speed of light. Channel coherence time with 120 km/h is around 5 ms (i.e., several slots).

Figure 23: Impact of UE speed on BER.

UE speed has a minor impact on the link performance considering a relatively large channel coherence time.

BLER Results

Here we present BLER results for different block sizes. In Figure 24 we consider the following cases with Manchester encoding, and single codded bit per OFDM symbol:

- 8 bits information+10 bits CRC

- 48 bits information+10 bits CRC

- 48 bits information+24 bits CRC

In the above cases, 8 bits and 48 bits to can be used to indicate group IDs and UE IDs, respectively.

Figure 24: BLER performance for different block sizes.

Repetition:

In order to match WUS coverage with PDCCH, repetitions may need to be applied. Figure 25 shows the BLER performance with WUS repetitions {2,4,6} for different payload sizes. Note that for the WUS case, 1Rx antenna is assumed. As we can see, a repetition factor 2 improves the BLER performance by around 2 dB (considering non-coherent combining). Also, the number of required repetitions depend on the noise figure difference between WUR and main receiver.

Figure 25: BLER performance with repetition for WUS.

For non-coherent WUS detection, every repetition factor of 2 improves the link performance by around 2 dB.

For payload-based OOK WUS a relatively large number of WUS repetitions might be needed to match the PDCCH coverage. The total WUS duration with repetition is in order of tens of slots.

#### 7.1.2.2 OOK with M-bit per OFDM symbol

Here, we present the link-level results for M-bit OOK with OOK-2 and OOK-4 methods.

Raw BER results

Figure 26 and Figure 27 show the raw BER for M-bit OOK generation with least square (LS) and DFT-s approaches. For M-bit OOK generation, the link-level performance significantly degrades when M increases to 8.

Also, Figure 28 shows the performance of M-parallel-OOK approach (OOK-2) with and without guardband (GB) between segments (with GB, 6 subcarriers on each side of each segment are used for guardband).  As we can see, guardband between segments is needed to improve the link performance especially for M>2.

Figure 26: BER performance of least square approach (OOK-4).

Figure 27: BER performance of DFT-s approach (OOK-4).

Figure 28: BER performance of parallel-OOK approach (OOK-2).

Note that for M-parallel OOK method (OOK-2), the total power can be smaller than OOK-4 if the same PSD needs to be maintained (i.e., no power boosting). Figure 29 shows results for OOK-2 with power scaling (to have the same total power as OOK-4 but affecting PSD) and without power scaling.

Figure 29: BER performance of different M-OOK approaches.

For M-bit OOK generation, the DFT-based method and least square method (OOK-4) have a similar coverage performance and outperform parallel M-bit OOK method (OOK-2).

BLER results

In Figure 30 we show the BLER results for different WUS block sizes. We consider multi-bit OOK generation using DFTs approach. The following cases with Manchester encoding:

- 8 bits information+10 bits CRC

- 48 bits information+10 bits CRC

- 48 bits information+24 bits CRC

In the above cases, 8 bits and 48 bits to can be used to indicate group IDs and UE IDs, respectively.

Comparing 1-bit OOK with multi-bit OOK shows that the BLER performance degrades by increasing the number of OOK bits per OFDM symbol. Therefore, multi-bit OOK does not reduce the overhead needed to reach the PDCCH coverage (see Annex for PDCCH link-level results).

Multi-bit OOK WUS does not reduce the overall time-frequency resources needed to reach the PDCCH coverage. This is because the link performance degrades by increasing the number of OOK bits per OFDM symbol.

Figure 30: BLER for different block sizes with M-OOK using DFTs approach.

### 7.2.2	M-bit OOK

Here, we present the link-level results for M-bit OOK with OOK-2 and OOK-4 methods.

Raw BER results

Figure 26 and Figure 27 show the raw BER for M-bit OOK generation with least square (LS) and DFT-s approaches. For M-bit OOK generation, the link-level performance significantly degrades when M increases to 8.

Also, Figure 28 shows the performance of M-parallel-OOK approach (OOK-2) with and without guardband (GB) between segments (with GB, 6 subcarriers on each side of each segment are used for guardband).  As we can see, guardband between segments is needed to improve the link performance especially for M>2.

Figure 31: BER performance of least square approach (OOK-4).

Figure 32: BER performance of DFT-s approach (OOK-4).

Figure 33: BER performance of parallel-OOK approach (OOK-2).

For M-bit OOK generation, the link-level performance significantly degrades for M=8.

For M-parallel-OOK approach (OOK-2), guardband between segments is needed to improve the link performance especially when M >2.

Note that for M-parallel OOK method (OOK-2), the total power can be smaller than OOK-4 if the same PSD needs to be maintained (i.e., no power boosting). Figure 29 shows results for OOK-2 with power scaling (to have the same total power as OOK-4 but affecting PSD) and without power scaling.

Figure 34: BER performance of different M-OOK approaches.

For M-bit OOK generation, the DFT-based method and least square method (OOK-4) have a similar coverage performance and outperform parallel M-bit OOK method (OOK-2).

BLER results

In Figure 30 we show the BLER results for different WUS block sizes. We consider multi-bit OOK generation using DFTs approach. The following cases with Manchester encoding:

- 8 bits information+10 bits CRC

- 48 bits information+10 bits CRC

- 48 bits information+24 bits CRC

In the above cases, 8 bits and 48 bits to can be used to indicate group IDs and UE IDs, respectively.

Comparing 1-bit OOK with multi-bit OOK shows that the BLER performance degrades by increasing the number of OOK bits per OFDM symbol. Therefore, multi-bit OOK does not reduce the overhead needed to reach the PDCCH coverage.

Multi-bit OOK WUS does not reduce the overall time-frequency resources needed to reach the PDCCH coverage. This is because the link performance degrades by increasing the number of OOK bits per OFDM symbol.

Figure 35: BLER for different block sizes with M-OOK using DFTs approach.

## 7.2	WUS based on existing signal structure

In this section, we evaluate detection performance of OFDMA-based WUS (see discussion in Section 6) using the existing NR SSS structure. Results are provided for the detection of SSS in SSB structure based on OFDMA-based WUR and the detection of SSS time domain samples based on WUR capable of processing I/Q samples in time-domain.

Performance of existing SSS-based signal detection based on OFDMA-based WUR

Figure 36 shows initial link performance evaluation results for sequence-based WUS based on existing signal (SSS in SSB) structure. Evaluation assumptions are given in Table 1. Evaluations show missed detection rate (MDR) for a correlation-based detector with FAR target of 0.1%. Figure 37 shows the impact of frequency offset error on the detection performance of WUS based on SS-based signal.

Figure 36: Link performance of WUS based on existing signal (SSS) structure.

Figure 37 Impact of frequency offset error on the link performance of WUS based on existing signal.

Performance of existing SSS-based signal detection based WUR capable of processing I/Q samples in time-domain

In this part, we consider existing SSS-based signal detection in time domain. That is, we assume that WUR is capable of receiving and processing I/Q samples in time-domain, but not necessarily capable of performing FFT. Figure 38 shows initial link performance evaluation results for sequence-based WUS based on existing signal (SSS). Evaluation assumptions are given in Table 1. Evaluations show missed detection rate (MDR) for a time-domain correlation-based detector with FAR target of 0.1%. In Figure 39, we show the impact of adjacent cell interference on the detection performance with different signal to interference power ratio (SIR) from -3 dB to 0 dB.

Figure 38 Link performance of SSS-based signal detection in time domain

Figure 39 SS Link performance of SSS-based signal detection in time domain with different SIR assumptions.

## 7.3	Overhead impact comparison

Based on results in previous sections, in Table 2 below we show the total WUS duration (with repetitions) in terms of number of slots needed to match PDCCH performance for different cases (see Annex for PDCCH link-level results).

Table 2: WUS duration for matching PDCCH coverage for different cases.

| WUS payload(modulation) | Duration (slots) to match PDCCH 1Rx AL16 (for Redcap) at 1% BLER | Duration (slots) to match PDCCH 2Rx AL16 at 1% BLER | Duration (slots) to match PDCCH 4Rx AL16 at 1% BLER |
| --- | --- | --- | --- |
| 1bit (OOK) | 10 | 30 | 83 |
| 8bit (OOK) | 58 | 164 | 465 |
| 48bit (OOK) | 315 | 890 | 2500 |
| 1bit (OFDM) | 6 symbols | 11 symbols | 23 symbols |
| Note1: MDR = 1% and FAR = 0.1% assumed for above comparison Note2: for 1bit payload sequence-based detection is assumed. For 8bit and 48bit payload Manchester encoding is used (with 10bit CRC), and ideal timing is assumed (additional overhead for e.g., preamble to acquire timing not modeled). 10ppm freq. error for OFDM and 50ppm freq. error for OOK. Note3: LP-WUR assumed to have 3dB worse NF than MR for OFDM-WUR and 6dB worse NF than MR for OOK-WUR Note4: Inter-cell interference impact not modeled. |  |  |  |

# 8	LP-WUS-related procedures

In this section, we discuss L1 procedures related to operation of LP-WUS/WUR.

## 8.1	LP-WUR monitoring behaviour

In RAN1#112, the following agreements were reached:

Agreement Study further pros and cons of the following monitoring behaviors of LP-WUR - Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states - Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time

That is, in general, WUR operation can either be duty-cycled, i.e., as for Rel-15/16 WUS for NB-IoT/LTE-M and Rel-17 PEI, with the WUR switching between an active and a sleep mode, or continuously monitoring the DL using WUR, with the WUR constantly in the active mode. The main benefit of the ‘continuous-WUR’ is thought to be reduced DL latency. That is, the UE can we woken up at any time. However, as discussed above, the start up time of the main receiver (400ms transition time from ultra-deep sleep an 20ms transition time for regular deep sleep) anyway put a lower cap on the achievable DL latency. Therefore, a duty-cycle considerably shorter that the main receiver transition time, e.g. 50ms when the main-receiver is in ultra-deep sleep with a 400ms transition time, only increases the energy consumption without any noticeable improvement for the DL latency. Therefore, it may the difficult to motive continuous-WUR, especially since also the specification impact would be considerably larger. E.g. for Idle/Inactive, the paging monitoring framework with DRX, paging occasions, and paging frames in TS 38.304 cannot be reused and new start/stop conditions for DL monitoring using WUR must be defined.

The main receiver transition time, 400 ms in ultra-deep sleep and 20 ms in deep sleep, is a lower bound for the achievable DL latency. Much more frequent WUR monitoring that this is only expected to increase the UE energy consumption without any noticeable improvement for the DL latency.

The main problem with Continuous-WUR is that since it is monitoring continuously it is very sensitive to false alarms, which starts up the MR and consumes a lot of energy. Therefore, the energy consumption for Continuous-WUR does not decrease with an increasing DRX cycle length in the same way as for the DRX baseline or the Duty-cycled WUR. Due to this false-alarm problem the power saving gain of Continuous-WUR is always smaller than for Duty-cycled WUR, and even becomes smaller than the DRX baseline at longer DRX cycle lengths.

False-alarms are problematic for Continuous-WUR and cause the energy consumption reduction to be smaller than for Duty-cycled WUR, and even smaller than the DRX baseline for longer DRX cycles.

## 8.2	WUS triggered UE behaviour

The UE behavior upon receiving WUS will depend on the content of the WUS. The WUS content can range from being a simple 1-bit indication (cf. Rel-15 WUS for NB-IoT/LTE-M), to including more information like the UE subgroup (cf. Rel-16 GWUS for NB-IoT/LTE-M, and Rel-17 PEI), all the way to including a unique UE identifier. As a baseline. the WUS must trigger the UE to continue monitoring of the legacy paging procedure in the paging occasion (PO). This since the UE must receive the paging message on PDSCH and from the included paging records determine if it is being paged, or if paging is for some other UE sharing the PO (as outlined in below figure).

Figure 40: Triggered UE behaviour for WUS without UE identifier.

In RAN1#111 it was discussed that If WUS carries the UE identifier, the UE can from WUS reception determine if it is being paged, and if it is, it can therefore directly trigger random access (as outlined in below figure). WUS with larger payload would negatively impact coverage. Also, additional specification efforts are required as this approach does not use legacy paging procedure.

Figure 41: Triggered UE behaviour for WUS with UE identifier.

Consider WUS with small payload (e.g., one bit or few bits) triggering legacy paging procedure as baseline for the study.

## 8.3 	RRM measurements

In RAN1#111 it was agreed to study ways to reduce the RRM contribution to the overall UE energy consumption to maximize the WUR gains:

Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighbouring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE)

In addition, the following agreement was reached in RAN1#112:

Agreement Study potential measurement metric used for RRM measurements performed by LP-WUR. - examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal

Mobility measurements include serving cell measurements, neighbour cell measurements which can be intra or inter-frequency measurements. Serving cell measurements needs to be performed most frequently and will therefore have the biggest impact on UE energy consumption. Further, intra- and inter-frequency cell measurements are only performed when the UE’s serving cell is weak and the searches for a new serving cell. In a well deployed network this is only temporary and for long-term steady-state it is therefore serving cell measurements that are most relevant for UE energy consumption evaluation.

For evaluation of RRM measurement impact on UE energy consumption, serving cell measurements are considered.

Serving cell measurements are defined in TS 38.133 Clause 4.2.2.2, and an extract is given below:

The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where: M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, otherwise M1=1.

SS-RSRP and SS-RSRQ are measured on SSS and in FR1 the Scaling Factor N1 is equal to 1. Therefore, a UE must perform serving cell measurements either every DRX cycle or every 2nd DRX cycle in FR1 (somewhat more relaxed in FR2).

In general, there are three alternative solutions for RRM measurements with LP-WUR operation:

- WUR measurement on legacy SSB (no measurements by MR)

- WUR measurement on a new WUR-specific reference signal (LP-SS)

- MR measurement on legacy SSB (no measurement by WUR)

Solution A is applicable for WUR capable of receiving legacy SSS (i.e., a somewhat more capable WUR). Solution B requires the introduction of a new WUR-specific reference signal, e.g., the LP-SS as discussed in previous meetings. This implies introduction of new always-on broadcast signals for this feature, and possibly new measurements requirements and would also result in additional NW overhead/energy consumption. Avoiding new always on signals is one of the key considerations from NW operation perspective. Solution C means the main receiver is started-up periodically to perform the RRM measurements. As shown in Section 4.1.3 ‘Impact of RRM measurements’ in [4], measurement relaxations compared to current requirements are needed for LP-WUR to provide power savings gain with this approach. For NR no serving cell measurement relaxations have been introduced, but for NB-IoT and LTE-M serving cell measurements of up to every 8th DRX cycle was introduced for more stationary UEs. However, as seen from [4] not even that is enough to achieve larger power saving gains for WUR.

RRM measurements by LP-WUR using existing OFDMA based signals (SSB) should be considered.

Reusing SSB for RRM measurement purposes would have zero additional overhead from broadcast, unlike the new LP-SS. To limit the negative impact on overhead, some companies have claimed that a ~1s periodicity would be sufficient for the LP-SS. However, 38.133 states that UEs should filter over at least 2 samples per serving cell measurement, and even with a 1.28s DRX cycle Ues would be distributed over several paging frames making the LP-SS reception very disjoint from the paging occasion for some Ues. Further, UEs must also be synchronized before the RRM measurements can start which may take a long time with a 1s LP-SS periodicity, and with such a long LP-SS periodicity as 1s the UEs may even drift in between.  These aspects, and potentially more, should be considered for determining a feasible range of LP-SS periodicities which can be considered for evaluation.

Feasibility for LP-SS periodicity should be determined from, at least, RRM measurement considerations, UE distribution over paging frames, and synchronization requirements.

# 9	Conclusion

In the previous sections we made the following observations:

Observation 1	With a single IFFT block, using a larger SCS for WUS for generating multi-bit OOK signal results in the coexistence issue and impact on other NR transmissions due to the time-domain operation (e.g., masking parts of the samples).

Observation 2	If WUS SCS is different from SCS used for other NR transmissions, FDM multiplexing between WUS and other NR transmissions cannot be enabled without significant gNB impact.

Observation 3	OOK WUS generation with single bit per OFDM symbol is straightforward with minimum impact on the OFDM transmitter.

Observation 4	M-bit OOK WUS generation (with multiple bits per OFDM symbol) using waveform fitting or DFT-based approach increases gNB complexity compared to single bit per OFDM symbol OOK WUS generation.

Observation 5	Phase discontinuous FSK modulation caused by CP, causes power leakage and higher requirement of timing synchronization in receiver.

Observation 6	For FSK-1, compensation of the phase shift caused by CP before OFDM modulation may be needed.

Observation 7	For the case of each segment comprises multiple subcarriers, the sequence mapped on the subcarriers impacts the envelop of time domain WUS signal. For the energy detection method such as envelop detector in the receiver, flat envelop signal should be designed to improve the detection performance.

Observation 8	For FSK-1, the complex sequence mapped on the subcarriers for WUS transmission should be studied further.

Observation 9	FSK-2 needs more segments than FSK-1 when M≥3 to carry same information.

Observation 10	For modulating ON symbols in OOK WUS, a sequence and random QPSK provide the same BER performance for the same signal power.

Observation 11	Adjacent cell interference can result in a significant coverage degradation when the interference is stronger than the signal.

Observation 12	Increasing ADC bit width is beneficial in terms of coverage, interference mitigation, and dynamic range of signal reception.

Observation 13	The coverage performance of single-bit ADC is extremely poor.

Observation 14	UE speed has a minor impact on the link performance considering a relatively large channel coherence time.

Observation 15	For non-coherent WUS detection, every repetition factor of 2 improves the link performance by around 2 dB.

Observation 16	For payload-based OOK WUS a relatively large number of WUS repetitions might be needed to match the PDCCH coverage. The total WUS duration with repetition is in order of tens of slots.

Observation 17	For M-bit OOK generation, the DFT-based method and least square method (OOK-4) have a similar coverage performance and outperform parallel M-bit OOK method (OOK-2).

Observation 18	Multi-bit OOK WUS does not reduce the overall time-frequency resources needed to reach the PDCCH coverage. This is because the link performance degrades by increasing the number of OOK bits per OFDM symbol.

Observation 19	For M-bit OOK generation, the link-level performance significantly degrades for M=8.

Observation 20	For M-parallel-OOK approach (OOK-2), guardband between segments is needed to improve the link performance especially when M >2.

Observation 21	For M-bit OOK generation, the DFT-based method and least square method (OOK-4) have a similar coverage performance and outperform parallel M-bit OOK method (OOK-2).

Observation 22	Multi-bit OOK WUS does not reduce the overall time-frequency resources needed to reach the PDCCH coverage. This is because the link performance degrades by increasing the number of OOK bits per OFDM symbol.

Observation 23	The main receiver transition time, 400 ms in ultra-deep sleep and 20 ms in deep sleep, is a lower bound for the achievable DL latency. Much more frequent WUR monitoring that this is only expected to increase the UE energy consumption without any noticeable improvement for the DL latency.

Observation 24	False-alarms are problematic for Continuous-WUR and cause the energy consumption reduction to be smaller than for Duty-cycled WUR, and even smaller than the DRX baseline for longer DRX cycles.

Based on the discussion in the previous sections we propose the following:

Proposal 1	It should be possible to generate LP-WUS transmissions using existing gNB hardware and not trigger any new emissions or compliance requirements.

Proposal 2	It should be possible to multiplex the LP-WUS with other NR transmissions in time or frequency domain without causing interference.

Proposal 3	It should be possible to reuse any unused LP-WUS time and frequency resources for other transmissions.

Proposal 4	Target the same coverage for LP-WUS as for Paging PDCCH.

Proposal 5	Consider the case of one (coded) bit per OFDM symbol as baseline for evaluation of OFDM based OOK waveforms.

Proposal 6	For M-bit OOK generation using waveform fitting or DFT-based approach, consider mapping of generated frequency domain values (before IFFT) to existing sequences and QAM constellations (e.g., 64-QAM) to minimize the gNB impact.

Proposal 7	For M-bit OOK generation with DFT-s or waveform fitting (i.e., least square), the PAPR aspects should be studied.

Proposal 8	WUR should support multi-bit ADC (e.g., bit width 4 or 8) to improve its sensitivity.

Proposal 9	Consider WUS with small payload (e.g., one bit or few bits) triggering legacy paging procedure as baseline for the study.

Proposal 10	For evaluation of RRM measurement impact on UE energy consumption, serving cell measurements are considered.

Proposal 11	RRM measurements by LP-WUR using existing OFDMA based signals (SSB) should be considered.

Proposal 12	Feasibility for LP-SS periodicity should be determined from, at least, RRM measurement considerations, UE distribution over paging frames, and synchronization requirements.

# References

RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR,” 3GPP TSG-RAN#97e, Sept 12 - 16, 2022.

“IEEE Standard for Information Technology--Telecommunications and Information Exchange between Systems--Local and Metropolitan Area Networks-Specific Requirements--Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications - Amendment 3: Wake-Up Radio Operation”, 2021.

Mazloum, Nafiseh, and Ove Edfors. "Interference-free OFDM embedding of wake-up signals for low-power wake-up receivers." IEEE Transactions on Green Communications and Networking 4.3 (2020): 669-677.

R1-2303759, “Low power WUS evaluations,” 3GPP TSG-RAN#112bis-e, April 2023.

R1-2303760, “Low power WUS receiver architectures,” 3GPP TSG-RAN#112bis-e, April 2023.

# Annex

## PDCCH performance

Here, we show the PDCCH BLER performance for the baseline scenario in which the main receiver decodes the PDCCH. We consider PDCCH aggregation levels (ALs) {8, 16} and {1Rx, 2Rx, 4Rx} UEs.

Figure 42: PDCCH BLER performance.

Table 3: PDCCH BLER performance for difference cases (baseline).

| Case | SNR at 1% BLER for PDCCH |
| --- | --- |
| AL8, 1Rx device | -0.3 dB |
| AL16, 1Rx device | -3 dB |
| AL8, 2Rx device | -3.5 dB |
| AL16, 2Rx device | -6 dB |
| AL8, 4Rx device | -6.5 dB |
| AL16, 4Rx device | -9.2 dB |