3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303762

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.17.10

Source:	Ericsson

Title:	UE features for MCE

Document for:	Discussion

# Introduction

In this paper, our comments on some UE feature aspects for multi-carrier enhancements are provided.

# Discussion

Below we provide a few initial comments.

- RAN1 agreed to following two types of indication of co-scheduled cells via DCI 1_X/0_X.

  - Alt 1: Explicit field for indication of co-scheduled cells

  - Alt 2: Indication Via FDRA field

As discussed during WI discussions, either all UEs supporting DCI 1_X/0_X based scheduling should support both indication mechanisms or at least one common mechanism to avoid unnecessary implementation complexity (e.g. at the gNB). Considering this and UE impact, we think Alt 2 should be considered for support in the basic functionality.

For Indication of co-scheduled cells via DCI 1_X/0_X, at least ‘Indication via FDRA’ is supported in the basic functionality.

- RAN1 also agreed to support multiple sets of serving cells that can be scheduled using DCI 1_X/0_X from a same scheduling cell. The basic functionality would support one set per scheduling cell for downlink/uplink and additional values can be reported using additional capability signaling.

Maximum number of sets that can be supported per scheduling cell is included in the FG.

- Regarding pre-requisites (especially legacy cross-carrier scheduling), it is preferable to discuss on a case-by-case basis. For example, a UE may support DCI 1_X to support up to four cells while the UE may or may not have the capability (e.g. due to BDs) to support cross-carrier scheduling using legacy CIF for the same case. If UE indicates support for both multi-cell scheduling via DCI 1_X/0_X and cross-carrier scheduling, the UE should support DL/UL reception on a cell in the set also using legacy DCI formats with CIF.

Support of DL (UL) reception on a cell within the set of cells via cross-carrier scheduling with DCI 1_1/1_2 (0_1/0_2) with CIF from the scheduling cell, if UE indicates support for cross-carrier scheduling.

- At least slot-based monitoring for DCI 1_X/0_X should be in the basic functionality.

Slot-based PDCCH monitoring on the scheduling cell on which DCI 1_X/0_X is monitored should be assumed in basic functionality.