3GPP TSG-RAN WG1 #112b-e		R1-2303764

eMeeting, Apr 17-26, 2023

Source:	Ericsson

Title:	UE features for Rel18 DSS Enhancements

Agenda Item:	9.17.14

Document for:	Discussion and Decision

# Introduction

Below two features are agreed to be introduced in Rel18 DSS Enhancements WI

- Reception of NR PDCCH candidates that overlap with LTE CRS REs

- Enabling UEs to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP

In this document, we provide our view on UE capability details for reception of NR PDCCH candidates that overlap with LTE CRS REs. Capability details for the other feature were agreed in RAN1#109-e.

# Discussion

Following was agreed in RAN1#110:

Agreement

Reception of NR PDCCH candidates that overlap with LTE CRS REs is supported by Rel18 UEs

PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15 from UE side.

Note: depends on UE capability

Following options can be used for PDCCH-DMRS channel estimation

- legacy CE assumption

  - RAN1 consider support this, if no RAN4 performance requirements are defined

- CE on clean symbol(s) only (this channel estimation option does not apply for 1 symbol CORESET)

Note: Restriction on the symbols and/or LTE CRS patterns applicable for above agreements can be considered during UE capability session.

Conclusion

RAN1 understands above agreements applies to 15kHz SCS only.

Agreement

If the feature “Reception of NR PDCCH candidates that overlap with LTE CRS REs “is supported, it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation.

Some aspects related to UE capabilities were discussed in [110bis-e-R18-Others-01] email thread but no agreements/conclusions were made [1].

One aspect is restriction on symbols in which reception of NR PDCCH candidates that overlap with LTE CRS REs is supported. As discussed in [2] (and shown in Figure 2-1 below), supporting this feature in second OFDM symbol (s1) enables (2sym + 11sym) or (3sym+10sym) combination of (NR PDCCH symbols +NR PDSCH symbols). Therefore, the feature should be supported at least for s1. Among other candidates (s0, s4, s7, s8, s11), we see some benefit in supporting the feature also for s0 and s4 as they enable additional (LTE PDCCH+ NR PDCCH + NR PDSCH) combinations. These can be supported via additional capability indications.

Proposal 1

- A capability indication for UE support of reception of NR PDCCH candidates that overlap with LTE CRS REs in 2nd OFDM symbol (s1) of a slot should be supported.

- Separate capability indication for UE support in {s0,s1}, {s1,s4}, {s0,s1,s4} can be additionally supported.

Figure 2-1 – Example configuration of LTE and NR transmissions on a DSS carrier with Rel18 Enhancement.

Another aspect discussed in [1] was restriction on Type 0/0A/1/2 CSS sets from being used with this feature. We do not see need to introduce such a restriction. Even from UE perspective, there is little/no impact to implementation if legacy CE is used. Even with clean symbol CE, if the UE can support the feature for USS there no additional complexity to support it for other SS sets as well.

Two channel estimation (CE) options (‘legacy CE’ and ‘clean symbol based CE’) for supporting the feature were included in the agreement. In [2] we provided performance evaluations for different CORESET durations and channel estimation options and summary of results is included in Annex A for reference. Supporting the feature with 1-symbol CORESET and legacy CE provides the highest capacity gain among all the evaluated scenarios. Therefore, the feature should be supported at least for this combination.

Typically, channel estimation is considered as UE receiver implementation aspect and is transparent to the specifications. However, the case of ‘clean symbol based CE’ can only be supported with 2-symbol or 3-symbol CORESET durations while the ‘legacy CE’ is applicable to all possible CORESET durations. Given this, if UEs that support only ‘clean symbol CE’ are to be supported, a capability indication to distinguish them from UEs supporting ‘legacy CE’ is required.

Proposal 2

- A capability indication for UE support of reception of NR PDCCH candidates that overlap with LTE CRS REs using ‘legacy channel estimation’ (and all possible CORESET durations) should be supported.

- If there are UEs that only support the ‘clean symbol CE’ channel estimation option, separate capability indication for this should be supported.

As also discussed in [3], if UE capability signalling is introduced such that a UE can indicate that it supports the feature with two channel estimation methods (‘legacy CE’ and ‘clean symbol CE’), then an RRC parameter would be needed to explicitly configure the UE to use one of the two supported channel estimation methods.

On the applicable CRS patterns, the straightforward option is to support the feature for the case of non-overlapping CRS patterns (one pattern for case of Rel15 FG 5-28 and up to 3 non-overlapping patterns for case of Rel16 FG 14-1). Then support for the feature with overlapping CRS patterns (introduced in Rel16 or Rel18) can be considered separately.

Proposal 3

- A capability indication for UE support of reception of NR PDCCH candidates that overlap with LTE CRS REs using one or multiple non-overlapping CRS patterns should be supported.

- Separate capability indication for UE support with two overlapping CRS patterns can be additionally supported .

# Conclusions

In this document we discuss UE capability details for reception of NR PDCCH candidates that overlap with LTE CRS REs and propose the following

Proposal 1

- A capability indication for UE support of reception of NR PDCCH candidates that overlap with LTE CRS REs in 2nd OFDM symbol (s1) of a slot should be supported.

- Separate capability indication for UE support in {s0,s1}, {s1,s4}, {s0,s1,s4} can be additionally supported.

Proposal 2

- A capability indication for UE support of reception of NR PDCCH candidates that overlap with LTE CRS REs using ‘legacy channel estimation’ (and all possible CORESET durations) should be supported.

- If there are UEs that only support the ‘clean symbol CE’ channel estimation option, separate capability indication for this should be supported.

Proposal 3

- A capability indication for UE support of reception of NR PDCCH candidates that overlap with LTE CRS REs using one or multiple non-overlapping CRS patterns should be supported.

- Separate capability indication for UE support with two overlapping CRS patterns can be additionally supported.

# References

[1] R1-2210358 – “Moderator Summary for Rel. 18 NR DSS”, Moderator (Intel Corporation), RAN1#110bis-e, October 2022.

[2] R1-2207439 – “NR PDCCH reception in symbols with LTE CRS REs”, Ericsson, RAN1#110, Toulouse, France, August 2022.

[3] R1-2303766 – “Discussion on MCE, eDSS and NCR for Rel18”, Ericsson, RAN1#112bis-e, eMeeting, April 2023.

# Annex A

Below we provide summary of performance evaluations for different CORESET configurations and channel estimation options from R1-2207439 ([2]). For detailed evaluation results and evaluation assumptions please refer to [2].

Table 2.3.0-1. Summary of PDCCH capacity evaluations (Scenario #1A)

|  | Evaluated case | Description 2 | PDCCH capacity (Avg. #DCIs per slot) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Baseline1 | R18 enh. | Gain (%) |
| 1 | #1A-O2 | 1sym CS/SS in s1, legacy RE mapping, legacy Rx processing for PDCCH REs and PDCCH DMRS REs and legacy CE. More details in section 2.3.1 and Observation 6 | 2.9 | 4.3 | 47 |
| 2 | #1A-O2a | 1sym CS/SS in s1, legacy RE mapping, legacy Rx processing for PDCCH DMRS REs and legacy CE, PDCCH REs overlapping with CRS punctured/nulled by Rx. More details in section 2.3.1 and Observation 6 | 2.9 | 4.8 | 67 |
| 3 | #1A-O2b | 1sym CS/SS in s1, legacy RE mapping, PDCCH REs and PDCCH DMRS REs overlapping with CRS punctured/nulled by Rx. More details in section 2.3.1 and Observation 6 | 2.9 | 5.1 | 77 |
| Notes: 1. 1sym NR PDCCH in s2 used as baseline with s0, s1 reserved for LTE PDCCH and remaining 11 symbols for data. 2. Details such as coexistence assumptions and % UEs that can support the Rel18 enhancement are provided in the corresponding Observation 6 mentioned in the description. CS/SS implies CORESET/Search space. |  |  |  |  |  |

Table 2.3.0-2. Summary of PDCCH capacity evaluations (Scenario #2)

|  | Evaluated case | Description 2 | PDCCH capacity (Avg. #DCIs per slot) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Baseline1 | R18 enh. | Gain (%) |
| 4 | #2- O2 | 2sym CS/SS in s1+s2, legacy RE mapping, legacy Rx processing for PDCCH REs, PDCCH DMRS REs and legacy CE. More details in Observation 7, section 2.3.2 | 2.9 | 3.5 | 19 |
| 5 | #2- O1-1a | 2sym CS/SS in s1+s2, legacy RE mapping, legacy Rx processing for PDCCH DMRS REs and legacy CE, PDCCH REs overlapping with CRS punctured/nulled by Rx. More details in Observation 7, section 2.3.2 | 2.9 | 3.8 | 33 |
| 6 | #2- O1-1b | 2sym CS/SS in s1+s2, legacy RE mapping, Rx only uses for PDCCH DMRS REs in s2 and legacy CE, PDCCH REs overlapping with CRS punctured/nulled by Rx. More details in Observation 7, section 2.3.2 | 2.9 | 3.5 | 19 |
| 7 | #2- O1-2 | 2sym CS/SS in s1+s2, new PDCCH/PDCCH DMRS RE mapping (no PDCCH DMRS on the symbol with LTE CRS), Rx only uses for PDCCH DMRS REs in s2 and legacy CE, Rx assumes PDCCH REs punctured/nulled. More details in Observation 7, section 2.3.2 | 2.9 | 3.6 | 25 |
| 8 | #2- O2b | 2sym CS/SS in s1+s2, legacy RE mapping, Rx assumes PDCCH REs and PDCCH DMRS REs are punctured/nulled. More details in Observation 7, section 2.3.2 | 2.9 | 4.0 | 38 |
| Notes: 1. 1sym NR PDCCH in s2 used as baseline with s0, s1 reserved for LTE PDCCH and remaining 11 symbols for data. 2. Details such as coexistence assumptions and fraction of UEs that can support the Rel18 enhancement are provided in the corresponding Observation 7 mentioned in the description. CS/SS implies CORESET/Search space |  |  |  |  |  |

Table 2.3.0-3. Summary of PDCCH capacity evaluations (Scenario #3A)

|  | Evaluated case | Description 2 | PDCCH capacity (Avg. #DCIs per slot) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Baseline1 | R18 enh. | Gain (%) |
| 9 | #3A-1s-O2 | 1sym CS/SS in s1, legacy RE mapping, legacy Rx processing for PDCCH REs, PDCCH DMRS REs and legacy CE. More details in Observation 8, section 2.3.3 | 5.8 | 7.1 | 24 |
| 10 | #3A-1s-O2a | 1sym CS/SS in s1, legacy RE mapping, legacy Rx processing for PDCCH DMRS REs and legacy CE, PDCCH REs overlapping with CRS punctured/nulled by Rx. More details in Observation 8, section 2.3.3 | 5.8 | 7.8 | 34 |
| 11 | #3A-1s-O2b | 1sym CS/SS in s1, legacy RE mapping, PDCCH REs and PDCCH DMRS REs overlapping with CRS punctured/nulled by Rx. More details in Observation 8, section 2.3.3 | 5.8 | 8.0 | 39 |
| 12 | #3A-2s- O2 | 2sym CS/SS in s1+s2, legacy RE mapping, legacy Rx processing for PDCCH REs, PDCCH DMRS REs and legacy CE. More details in Observation 8, section 2.3.3 | 5.8 | 6.64 | 15 |
| 13 | #3A-2s- O1-1a | 2sym CS/SS in s1+s2, legacy RE mapping, legacy Rx processing for PDCCH DMRS REs and legacy CE, PDCCH REs overlapping with CRS punctured/nulled by Rx. More details in Observation 8, section 2.3.3 | 5.8 | 7.21 | 24 |
| 14 | #3A-2s- O1-1b | 2sym CS/SS in s1+s2, legacy RE mapping, Rx only uses for PDCCH DMRS REs in s2 and legacy CE, PDCCH REs overlapping with CRS punctured/nulled by Rx. More details in Observation 8, section 2.3.3 | 5.8 | 6.63 | 14 |
| 15 | #3A-2s- O1-2 | 2sym CS/SS in s1+s2, new PDCCH/PDCCH DMRS RE mapping (no PDCCH DMRS on the symbol with LTE CRS), Rx only uses for PDCCH DMRS REs in s2 and legacy CE, PDCCH REs overlapping with CRS punctured/nulled by Rx. More details in Observation 8, section 2.3.3 | 5.8 | 6.87 | 19 |
| 16 | #3A-2s- O2b | 2sym CS/SS in s1+s2, legacy RE mapping, PDCCH REs and PDCCH DMRS REs overlapping with CRS punctured/nulled by Rx. More details in Observation 8, section 2.3.3 | 5.8 | 7.45 | 29 |
| Notes: 1. 2symbols of NR PDCCH in s2, s3 used as baseline with s0, s1 reserved for LTE PDCCH and remaining 10 symbols for data. 2. Details such as coexistence assumptions and fraction of UEs that can support the Rel18 enhancement are provided in the corresponding observation mentioned in the description. CS/SS implies CORESET/Search space |  |  |  |  |  |

Table 2.3.0-4. Summary of PDCCH capacity evaluations (Scenario #3)

|  | Evaluated case | Description2 | PDCCH capacity (Avg. #DCIs per slot) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Baseline1 | R18 enh. | Gain (%) |
| 17 | #3-3s- O2 | 3sym CS/SS in s1+s2+s3, legacy RE mapping, legacy Rx processing for PDCCH REs, PDCCH DMRS REs and legacy CE. More details in Observation 9, section 2.3.4 | 5.8 | 6.51 | 11.7 |
| 18 | #3-3s- O1-1a | 3sym CS/SS in s1+s2+s3, legacy RE mapping, legacy Rx processing for PDCCH DMRS REs and legacy CE, PDCCH REs overlapping with CRS punctured/nulled by Rx. More details in Observation 9, section 2.3.4 | 5.8 | 6.972 | 19.64 |
| 19 | #3-3s- O1-1b | 3sym CS/SS in s1+s2+s3, legacy RE mapping, Rx only uses for PDCCH DMRS REs in s2 and legacy CE, PDCCH REs overlapping with CRS punctured/nulled by Rx. More details in Observation 9, section 2.3.4 | 5.8 | 6.672 | 14.48 |
| 20 | #3-3s- O1-2 | 3sym CS/SS in s1+s2+s3, new PDCCH/PDCCH DMRS RE mapping (no PDCCH DMRS on the symbol with LTE CRS), Rx only uses for PDCCH DMRS REs in s2 and legacy CE, PDCCH REs overlapping with CRS punctured/nulled by Rx. More details in Observation 9, section 2.3.4 | 5.8 | 6.835 | 17.28 |
| 21 | #3-3s- O2b | 3sym CS/SS in s1+s2+s3, legacy RE mapping, PDCCH REs and PDCCH DMRS REs overlapping with CRS punctured/nulled by Rx. More details in Observation 9, section 2.3.4 | 5.8 | 7.105 | 21.92 |
| Notes: 1. 2 symbols of NR PDCCH in s2, s3 used as baseline with s0, s1 reserved for LTE PDCCH and remaining 10 symbols for data. 2. Details such as coexistence assumptions and fraction of UEs that can support the Rel18 enhancement are provided in the corresponding observation mentioned in the description. CS/SS implies CORESET/Search space |  |  |  |  |  |