3GPP TSG RAN WG1 #112bis-e                              R1-2303769

e-Meeting, April 17th – April 26th, 2023

Source:	Sharp

Title:	Discussion on physical channel design framework for NR sidelink evolution on unlicensed spectrum

Agenda Item:	9.4.1.2

Document for:	Discussion and Decision

Introduction

In RAN#97e meeting, Rel-18 work item on support of [1] has the following objectives to study and specify support of sidelink on unlicensed spectrum.

2. Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98. - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

In this contribution, we share some views on modifications of physical channel design for adapting sidelink communication on unlicensed spectrum.

Discussions

Physical layer structure

Frequency resource allocation

Mapping between sub-channel and interlace for interlace RB-based PSCCH/PSSCH transmission has been agreed in last RAN1 meeting as below.

Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

It is known that mapping between sub-channel and interlace is associated with how to design the frequency domain resource indication, that is, whether to reuse NR-SL frequency domain resource indication or NR-U frequency domain resource indication. According to the above agreed mapping between sub-channel and interlace (i.e., option 1 in previous agreement regarding mapping between sub-channel and interlace), a sub-channel with a same sub-channel index would appear at different RB sets in the resource pool. Consequently, when referring to a sub-channel in a resource pool but without explicitly indicating RB set index, specific resource blocks for the sub-channel would be of ambiguity. Explicit RB set index is needed for indicating resource blocks in which RB set(s) for frequency domain resource indication. Moreover, the motivation of the agreed option 1 aims to align with NR-U design for frequency domain resource indication. Therefore, in view of the fact what we have agreed for mapping between sub-channel and interlace, the existing NR-U frequency domain resource allocation including interlace indication and RB set indications can be considered as baseline for frequency domain resource indication for interlace RB-based PSCCH/PSSCH transmission.

Proposal 1: Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

  - support explicitly indicating the used sub-channel index(s) and RB set index(s).

With the consideration of not causing RAN4 standard effort too much, the principle of NR-U based frequency domain resource allocation, i.e., same number(s) and same index(s) of interlaces in different RB sets are indicated, can be reused.

Proposal 2: Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

  - support the used interlace index(s) in different RB sets are always the same.

In addition, in NR-U, RIV is used for indicating a starting interlace index and number of contiguous interlace indices for 15kHz SCS case where there are M = 10 interlaces in an RB set, while bitmap is used for indicating interlace combinations for 30kHz SCS case where there are M = 5 interlaces in an RB set. The same frequency allocation scheme can be reuse as much as possible for SL-U. That is, RIV can be used as well for 15kHz SCS case where there are M = 10 subchannels in an RB set, while bitmap can be used for 30kHz SCS case where there are M = 5 subchannels in an RB set. Then, same frequency allocation scheme used for NR-U and SL-U can achieve an efficient coexistence between NR-U and SL-U. In addition, it has to be noted that, in RAN1#110b-e, it is agreed to support 1 sub-channel equals to K=2 interlaces for 15kHz SCS, which means there are only 5 sub-channels in an RB set. In this case, for simplicity, as same as 30kHz SCS case, bitmap can be also used for indicating which combination of sub-channels in an RB set is allocated to the UE.

Proposal 3: For interlace RB-based PSCCH/PSSCH transmission in SL-U in a resource pool:

- Support NR-U based frequency domain resource allocation for sub-channels within an RB set, i.e., X bits, as baseline, where

  - X bits are used for indicating which combination of sub-channels among M sub-channels in an RB set is allocated to the UE where M is the number of sub-channels contained in an RB set of the resource pool.

    - Support X= 6 bits (RIV indication) in for M = 10.

    - Support X= 5 bits (bitmap) for M = 5.

  - The frequency domain resource allocation is determined as an intersection of the resource blocks of the indicated interlaces and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets.

Agreement For PSCCH and PSSCH resource indication in time/frequency domain: - For time domain: R16 NR SL TRIV is reused as baseline - For frequency domain: - further study sub-channel indexing and resource indication - FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

As agreed in RAN1#110 below, the NR 16 SL TRIV is reused as baseline for PSCCH and PSSCH resource indication, which means up to 3 actual resources may be scheduled by a SCI for PSCCH/PSSCH transmission. Furthermore, 3 actual resources can be independently scheduled in time and frequency domain. The existing NR-U frequency domain resource indication is to provide one frequency resource allocation. Therefore, it is necessary to discuss whether a same frequency domain resource allocation is applied to all possible 3 actual resources for simplicity or separate frequency domain resource allocations for respective actual resources can be allowed for scheduling flexibility. It has to be noted that the scheduling flexibility has to come at the expenses of SCI overhead.

Proposal 4: Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

- down-select one of the followings,

  - Option 1: Same frequency domain resource indication is applied to all scheduled actual resources.

  - Option 2: Seperate frequency domain resource indications can be applied to respective actual resources.

Time resource allocation for MCSt

In RAN1#110 meeting, the following regarding time domain resource indication for PSSCH transmission was agreed.

Agreement For PSCCH and PSSCH resource indication in time/frequency domain: - For time domain: R16 NR SL TRIV is reused as baseline - For frequency domain: - further study sub-channel indexing and resource indication - FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

One FFS in above agreement is whether to enhance the R16 NR SL TRIV/FRIV if new feature MCSt is introduced in SL-U. As known, it was agreed in AI 9.4.1.1 that multi-consecutive slots transmission is supported for Mode 1 and Mode 2 resource allocation in SL-U.

Agreement Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U. - FFS details

However, the existing time resource assignment in Rel-16 only supports 2 or 3 consecutive or non-consecutive slots transmission. Up to 3 consecutive slots seems restrictive and is not suitable to adapt to multi-consecutive slots transmission.

Proposal 5: Enhance on R16 NR-SL time resource assignment to support more than 3 slot consecutive transmission for multi-consecutive slots transmission in SL-U.

TBS determination

In SL-U, a number of aspects impact upon TBS determination, e.g.

- With the support of two candidate starting symbols within a slot, the total number of REs used for two (re)transmissions of a TB would be remarkably different if one uses the first candidate starting symbol and the other uses the second candidate starting symbol.

- In interlace RB-based PSSCH transmissions, since an interlace may contain 10 or 11 RBs within one RB Set, there may be more than one sub-channel sizes in a resource pool. Therefore, (re)transmissions of a given TB, even with a same number of sub-channels, may correspond to different total number of RBs.

- Use of intra-cell guard band (i.e. when the two adjacent RB Sets are both used for transmission) may add additional RBs for some of the (re)transmissions of a TB.

Reference symbol length for TBS determination

Regarding how to determine reference symbol length for TBS determination in a resource pool with 2 candidate starting symbols, option 4 is agreed as working assumption in last RAN1#112 meeting.

Working assumption If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - At least for COT initiation, TBS is determined based on a reference number of symbols as follows: - Option 4: The reference number of symbols is determined by (pre-)configuration - FFS: value range - FFS: whether a different reference number of symbols is needed for transmission in a shared COT

As in above working assumption, value range of the reference number of symbols is kept as FFS. Considering both locations of 1st candidate starting symbols and 2nd starting symbols are (pre-)configured, value range of the reference number of symbols naturally varies depending on locations of the 1st and 2nd starting symbols and sl-LengthSymbols. According to the agreed values for locations of the 1st and 2nd starting symbols, the value range of the reference number of symbols varies from 5 symbols to 12 symbols. The (pre-)configuration provides a value for the reference number of symbols corresponding to either the 1st starting symbol or the 2nd starting symbol.

The other FFS point is to whether to use a different reference number of symbols for PSSCH transmission in a shared COT. In our view, a PSSCH transmission for COT initiation can be either an initial transmission or an retransmission, while a PSSCH transmission in a shared COT can be either a retransmission or initial transmission. If a PSSCH transmission in a shared COT is the retransmission of the PSSCH transmission for COT initiation, TBS determination based on different reference numbers of symbols would cause different TB sizes between the initial transmission and the retransmission. In addition, it is not clear whether a UE can be always aware of shared COT information in advance before generating TB size. Therefore, a different reference number of symbols is not needed for transmission in a shared COT.

Proposal 6: Confirm the working assumption with the following updates:

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- At least for COT initiation, TBS is determined based on a reference number of symbols as follows:

  - Option 4: The reference number of symbols is determined by (pre-)configuration

  - FFS: value range is from 5 symbols to 12 symbols.

  - FFS: whether a different reference number of symbols is NOT needed for transmission in a shared COT.

Reference number of PRBs for TBS determination

On the issue of interlaced-RB based PSSCH transmissions and use of intra-cell guard band, this can be addressed by using a reference total number of RBs, for example, by calculating the total number of RBs in TBS determination by using the size of the lowest sub-channel in the resource pool. This simple approach also has the benefit of being backward compatible with legacy NR SL.

Proposal 7: When determining TBS in SL-U, the total number of RBs are calculated by multiplying the size of the lowest sub-channel in the resource pool and the total number of allocated sub-channels.

PSFCH transmission for SL-U

PSFCH transmission under 15 kHz and 30kHz

The following related to PSFCH transmission under 15kHz and 30kHz was agreed in RAN1#112 meeting.

Agreement Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace - Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U - Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - FFS: value of K2, K4 - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case - FFS: whether/how to reduce PAPR of PSFCH transmission Agreement To address PSFCH transmission dropping due to LBT failure: - Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Down-select one or support both of the followings - Option 1: Such PSFCH occasion(s) are (pre-)configured - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc. - FFS other details

It was agreed as below that the interlaced transmission is supported for PSFCH in SL-U and at least there is 1 PSFCH occasion per PSCCH/PSSCH transmission. Interlace is a concept across all RB sets in a SL BWP. Regarding one PSFCH resource, each PSFCH should be confined in an interlace in an RB set to reduce the probability of LBT failure.

Proposal 8: For interlaced transmission for PSFCH in SL-U,

- Frequency resource of a PSFCH is configured in an interlace in an RB set.

One controversial discussion point is how to support RB-based interlace for PSFCH transmission to meet OCB and PSD requirement. Three groups of alternatives regarding PSFCH transmission as above were listed for further study.

Alt.1-1a is intended to solve the PSFCH capacity issue given that the whole PRBs of an interlace needs to be used as one PSFCH resource. Alt.1-1a utilizes a common interlace and K3 dedicated PRB(s) where the common interlace is only used to meet the OCB requirement and does not carry any HARQ-ACK information. With Alt.1-1a, K3 dedicated PRBs are used to carry HARQ-ACK information, while most of the transmission power would be wasted in the common interlace which carry no valid HARQ-ACK information. Moreover, it is inevitable that a dedicated PRB coexists with an RB of a common interlace within a same 1MHz bandwidth. Furthermore, if the value of K2 supports more than 1, how to distribute K3 dedicated PRBs should be further discussed to alleviate the power limitation issue. Otherwise, contiguous dedicated PRBs would aggravate the power limitation issue.

Observation 1: Alt.1-1a with using common interlace may have power limitation issue, and K3>1 dedicated PRBs may aggravate the power limitation issue if not equally distribute K3>1 dedicated PRBs to different frequency bandwidth.

The group of Alt.2 (i.e., Alt 2-2a, Alt 2-3a, Alt 2-4a) is to use dedicated interlace to carry valid HARQ-ACK information. For an interlace with 15kHz or 30kHz, frequency interval between two consecutive resource blocks of one interlace exceeds 1MHz bandwidth. The alternatives in the group of Alt.2 do not have power limitation issue which may exists in Alt.1-1a.

For Alt 3-2a, K2 common PRBs are used to meet the OCB requirement and appropriate distribution of K4 dedicated PRB(s) if K4 >1 would also not cause power limitation issue which may exists in Alt.1-1a.

Overall, PSFCH performance by using the group of Alt.2 and alt.3-2a would be better than that by using Alt.1-1a.

The group of Alt.2 is divided into sub-alternatives to take PSFCH capacity issue into consideration. And alternative 3-2a aims to solve the PSFCH capacity issue as well. Therefore, it is important to firstly conclude whether the PSFCH capacity issue exists or not. Regarding the PSFCH capacity issue, according to the Rel-16/17 PSFCH design principle, the required quantity of the PSFCH resources in a resource pool depends on the number of sub-channels in the resource pool, the configured period, the number of cyclic shift pair indexes. As in the WID of Rel-18 SL, the primary motivation of supporting sidelink over unlicensed spectrum is to increase sidelink data rate. Therefore, it is foreseeable that a sub-channel would occupy several interlaces or a PSSCH would occupy several sub-channels. In this regard, the PSFCH capacity issue can be alleviated in SL-U.

Observation 2: Supporting increased SL data rate over unlicensed spectrum may, in a sense, alleviate the PSFCH capacity issue from interlaced PSFCH transmission.

In addition, TS38.321 in NR SL specifies that, for groupcast, if a group size of recipient of a PSSCH is greater than the number of candidate PSFCH resources, the groupcast HARQ feedback option 1 would be selected. Therefore, same principle can be reused in SL-U if a group size of recipient of a PSSCH is larger than the number of candidate PSFCH resources.

Proposal 9: For groupcast HARQ feedback, if a group size of recipient of a PSSCH is larger than the number of candidate PSFCH resources, a same principle in NR Rel-16 SL is followed, i.e., HARQ-ACK feedback option 1 is used.

Proposal 10: Regarding PSFCH transmission under 15kHz and 30kHz, the following alternative 2-3a and 2-4a are supported:

- Alt 2-3a: each PSFCH transmission occupies 1 interlace.

- Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U.

S-SSB transmission

In RAN1#110bis-e, there is an FFS regarding the usage of PRBs within intra-cell guard-bands for S-SSB transmission as below.

Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

Regarding the FFS on S-SSB, our view is that the same principle for other channels/signals (and the same principle in NR-U) should be followed, i.e. such PRBs should not be used for S-SSB transmission, or else there will be a problem in coexisting with NR-U.

Proposal 11: PRBs within intra-cell guard band of two adjacent RB sets are not used for S-SSB transmissions.

As has been debated in last several RAN1 meetings, it is difficult to conclude whether temporary exemption of OCB requirement is applicable for S-SSB transmission. The following was agreed in RAN1#112 to accommodates both solutions to meet OCB and PSD requirement and solution to apply OCB exemption to S-SSB transmission.

Agreement For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc. - FFS gap of 0 - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS.

Regarding down-selection among solutions to meet OCB and PSD requirements, we think the following design principles should be followed:

- Principle 1: An S-SSB transmission should be completely accommodated by one RB Set. Otherwise transmission of S-SSBs would only be possible in wideband operation, and the probability of dropping an S-SSB transmission due to LBT failure would be significantly increased.

- Principle 2: The time/frequency resources for S-SSBs should be configured within a SL BWP rather than within a resource pool. Otherwise UE behaviours for transmission/reception of S-SSBs would be quite uncertain, e.g. it may depend on which resource pool is selected for data transmission.

- Principle 3: Detection of S-SSBs should not be more complex than that in Rel-16. For example, the time/frequency locations of all candidate S-SSBs should be fully determined prior to detection of S-SSBs.

- Principle 4: A unified solution is used for all supported SCSs and all supported carrier bandwidths, regardless of the number of RB Sets configured in a SL BWP.

For Option 1-1, the 1-slot/11-RB structure of Rel-16 S-SSB can only be directly reused for some of the channel bandwidth and SCS combinations (e.g. 20 MHz with 15 kHz SCS where some interlaces have 11 RBs in one RB Set), and not for others (e.g. 20 MHz with 30 kHz SCS where all interlaces have 10 RBs in one RB Set); And another problem of Option 1-1 is that if interlaced transmission is agreed for 15kHz and 30kHz, a different solution should be further studied given there is no definition of interlace for cover 60 kHz SCS. Different designs of S-SSB for different SCS require additional specification workload.

In addition, option 3-1 seems to be a more straightforward solution and require less specification workload with only some discussion to define repetition number N and gap between each repetition in the frequency domain. For 60 kHz SCS, option 3-1 can be applied as well.

Proposal 12: For S-SSB transmission, the following option is supported to meet OCB and PSBCH requirement for 15 kHz and 30 kHz SCS:

- Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

  - FFS gap of 0

On the other hand, if the temporary exemption of OCB requirement can be concluded to be applicable for S-SSB transmission, specification effort regarding S-SSB format would be minimized. According to ETSI EN 301.893, short transmissions during a COT can be temporarily allowed to not comply with the OCB requirement. In order to transmit the S-SSB, the UE might perform a LBT to initiate a COT. In our view, during the initiating COT, the S-SSB transmission can be considered as a short and temporary transmission and the temporary exemption of OCB requirement can be applicable for the S-SSB transmission.

In addition, if temporary exemption of OCB requirement is supported for S-SSB transmission, the case of S-SSB transmission with 15kHz needs to further study. The relevant FL proposal (i.e., Proposal 5-6 from [2]) for discussion was captured as following.

Proposal 5-6: Under temporary exemption of OCB requirement, regarding how to meet the minimum of 2 MHz requirement for 15 kHz SCS, RAN1 further study the followings: - Alt 1: repeat all or part of S-PSS/S-SSS/PSBCH - Alt 2: PSBCH spans over 12 PRBs, and is wrapped around S-PSS and S-SSS - [Alt 3: use higher SCS than 15kHz for S-PSS/S-SSS/PSBCH in a SL BWP with 15kHz] - FFS details

That is because S-SSB transmission with 11 RBs under 15kHz SCS occupies 1.98MHz channel bandwidth, which cannot meet the minimum of 2 MHz requirement. Three alternatives were listed in proposal 5-3d to solve the issue. We provide pros/cons on these three alternatives from our perspective.

- In terms of S-SSB format, both Alt.1 and Alt.2 require change to the legacy Rel-16 S-SSB format.

- In terms of coverage performance, Alt.3 provides wider bandwidth in the frequency domain than Alt.1 and Alt.2, which a better coverage is provided if considering the PSD limit with 10dBm/MHz.

- In terms of time duration, Alt.1 and Alt.2 span 1ms duration while Alt.3 spans 0.5ms or 0.25ms duration depending on the used SCS.

- In terms of UE complexity, Alt.3 would require a SCS switching between S-SSB and PSCCH/PSSCH. The total time of SCS switching and TX/RX switching can be accommodated in a gap symbol with 15kHz prior to S-SSB transmission and in remaining duration (e.g., 0.5ms or 0.75ms) within a S-SSB slot after S-SSB. It should be noted that a Rel-15 UE can be configured with different SCS of SSB from that of BWP. Moreover, SCS switching is not equivalent to BWP switching where most of the BWP switching delay are caused due to adapting different RRC configurations.

- In terms of specification impact, Alt.3 should have less impact on specification compared to Alt.1 and Alt.2.

Therefore, Alt. 3 is a feasible and straightforward solution.

Proposal 13: If temporary exemption of OCB requirement for S-SSB transmission is concluded to be supported, regarding how to meet the minimum of 2 MHz requirement under 15 kHz SCS, the following option is supported:

- Use higher SCS than 15kHz for S-PSS/S-SSS/PSBCH in a SL BWP with 15kHz.

Regarding the following RAN1 agreement, we think there is no need to transmit S-SSBs in more than one RB set just for the purpose of “maintaining a COT” (otherwise, with the same purpose, it seems the UE has to keep transmitting in each and every RB set and in each and every slot of the COT). On the other hand, we think it is beneficial in allowing S-SSB transmission in more than one RB set, for example, in case of channel access failure in one RB set, the UE can choose another RB set for S-SSB transmission (rather than drop the S-SSB transmission and wait for the next S-SSB slot).

Agreement RAN1 further study the followings: - Whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s), e.g., whether to transmit S-SSB repetitions in more than one RB set, etc.

Proposal 14: More than one frequency locations can be (pre-) configured for S-SSB transmission, each in one RB set.

SL CSI-RS transmission

In a resource pool (pre-) configured with two candidate starting symbols, the set of symbols for a PSSCH transmission (plus AGC and GAP symbols) can be one of the following:

- Case 1: all SL symbols except those for PSFCH (in a slot with PSFCH).

- Case 2: all SL symbols starting from the 1st candidate starting symbol (in a slot without PSFCH, and with the use of the 1st candidate starting symbol).

- Case 3: all SL symbols starting from the 2nd candidate starting symbol (in a slot without PSFCH, and with the use of the 2nd candidate starting symbol).

On the other hand, the SL CSI-RS symbol number is signaled as a symbol within a slot. Therefore, it is likely that a symbol number is valid for SL CSI-RS for one of the above cases, but not other(s), e.g. collision with symbols for DM-RS or 2nd-SCI or PTRS. Hence we propose that SL CSI-RS symbol is separately indicated for the 2 candidate starting symbol, in order not to restrict the flexibility of CSI acquisition due to the introduction of two candidate starting symbols.

Proposal 15: In a resource pool (pre-) configured with two candidate starting symbols, the CSI-RS symbol is separately (pre-) configured for the 1st candidate starting symbol and the 2nd candidate starting symbol.

Physical layer procedure

Congestion control

In legacy SL, SL-RSSI is measured on a sub-channel in the frequency domain and the OFDM symbols configured for PSCCH/PSSCH in the time domain. However, in a resource pool configured with two candidate starting symbols, the actual OFDM symbols used for a PSCCH/PSSCH transmission may be autonomously determined by a TX UE, i.e. the information is not available at another UE performing SL-RSSI measurement in the slot. If the same SL-RSSI definition is used, the total received power may be underestimated (e.g. some UEs use the 1st candidate starting symbols while others use 2nd candidate starting symbols).

Therefore, we propose that in such a resource pool, SL-RSSI measurement is only performed on OFDM symbols corresponding to PSCCH/PSSCH transmissions using the 2nd candidate starting symbols.

Proposal 16: In a resource pool (pre-)configured with two candidate starting symbols, SL-RSSI measurement is performed on OFDM symbols corresponding to PSCCH/PSSCH transmissions using the 2nd candidate starting symbols.

Resource subset determination

In legacy SL, a “candidate resource” for PSCCH/PSSCH transmission corresponds to a number of contiguous SL symbols in a slot and a number of contiguous sub-channels. However, for interlace RB-based transmissions, a “candidate resource” may correspond to e.g. a number of groups of sub-channels with each group in one RB set, and the sub-channels in different RB sets may not be contiguous. Therefore, RAN1 needs to discuss what a “candidate resource” is defined in that case.

For contiguous RB-based PSCCH/PSSCH transmissions, due to the reuse of sub-channel mapping in legacy SL, there may be a case that one or more RBs of the lowest sub-channel in the lowest RB set are in an intra-cell guard-band, and are not suitable for PSCCH transmission. In that sense, the definition of a “candidate resource” should also be updated (e.g. the lowest PRBs of a “candidate resource” should not contain any RB in an intra-cell guard band).

Proposal 17: RAN1 to discuss update of the definition of a “candidate resource” for both interlace RB-based PSCCH/PSSCH transmissions and contiguous RB-based PSCCH/PSSCH transmissions in SL-U.

Conclusion

In this contribution, we have discussed our views on physical channel design for adapting sidelink communication on unlicensed spectrum and have the following proposals and observation.

Proposal 1: Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

  - support explicitly indicating the used sub-channel index(s) and RB set index(s).

Proposal 2: Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

  - support the used interlace index(s) in different RB sets are always the same.

Proposal 3: For interlace RB-based PSCCH/PSSCH transmission in SL-U in a resource pool:

- Support NR-U based frequency domain resource allocation for sub-channels within an RB set, i.e., X bits, as baseline, where

  - X bits are used for indicating which combination of sub-channels among M sub-channels in an RB set is allocated to the UE where M is the number of sub-channels contained in an RB set of the resource pool.

    - Support X= 6 bits (RIV indication) in for M = 10.

    - Support X= 5 bits (bitmap) for M = 5.

  - The frequency domain resource allocation is determined as an intersection of the resource blocks of the indicated interlaces and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets.

Proposal 4: Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

- down-select one of the followings,

  - Option 1: Same frequency domain resource indication is applied to all scheduled actual resources.

  - Option 2: Seperate frequency domain resource indications can be applied to respective actual resources.

Proposal 5: Enhance on R16 NR-SL time resource assignment to support more than 3 slot consecutive transmission for multi-consecutive slots transmission in SL-U.

Proposal 6: Confirm the working assumption with the following updates:

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- At least for COT initiation, TBS is determined based on a reference number of symbols as follows:

  - Option 4: The reference number of symbols is determined by (pre-)configuration

  - FFS: value range is from 5 symbols to 12 symbols.

  - FFS: whether a different reference number of symbols is NOT needed for transmission in a shared COT.

Proposal 7: When determining TBS in SL-U, the total number of RBs are calculated by multiplying the size of the lowest sub-channel in the resource pool and the total number of allocated sub-channels.

Proposal 8: For interlaced transmission for PSFCH in SL-U,

- Frequency resource of a PSFCH is configured in an interlace in an RB set.

Observation 1: Alt.1-1a with using common interlace may have power limitation issue, and K3>1 dedicated PRBs may aggravate the power limitation issue if not equally distribute K3>1 dedicated PRBs to different frequency bandwidth.

Observation 2: Supporting increased SL data rate over unlicensed spectrum may, in a sense, alleviate the PSFCH capacity issue from interlaced PSFCH transmission.

Proposal 9: For groupcast HARQ feedback, if a group size of recipient of a PSSCH is larger than the number of candidate PSFCH resources, a same principle in NR Rel-16 SL is followed, i.e., HARQ-ACK feedback option 1 is used.

Proposal 10: Regarding PSFCH transmission under 15kHz and 30kHz, the following alternative 2-3a and 2-4a are supported:

- Alt 2-3a: each PSFCH transmission occupies 1 interlace.

- Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U.

Proposal 11: PRBs within intra-cell guard band of two adjacent RB sets are not used for S-SSB transmissions.

Proposal 12: For S-SSB transmission, the following option is supported to meet OCB and PSBCH requirement for 15 kHz and 30 kHz SCS:

- Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

  - FFS gap of 0

Proposal 13: If temporary exemption of OCB requirement for S-SSB transmission is concluded to be supported, regarding how to meet the minimum of 2 MHz requirement under 15 kHz SCS, the following option is supported:

- Use higher SCS than 15kHz for S-PSS/S-SSS/PSBCH in a SL BWP with 15kHz.

Proposal 14: More than one frequency locations can be (pre-) configured for S-SSB transmission, each in one RB set.

Proposal 15: In a resource pool (pre-) configured with two candidate starting symbols, the CSI-RS symbol is separately (pre-) configured for the 1st candidate starting symbol and the 2nd candidate starting symbol.

Proposal 16: In a resource pool (pre-) configured with two candidate starting symbols, SL-RSSI measurement is performed on OFDM symbols corresponding to PSCCH/PSSCH transmissions using the 2nd candidate starting symbols.

Proposal 17: RAN1 to discuss update of the definition of a “candidate resource” for both interlace RB-based PSCCH/PSSCH transmissions and contiguous RB-based PSCCH/PSSCH transmissions in SL-U.

References

RP-221938 “WID revision: NR sidelink evolution”, RAN#97e, OPPO.

R1-2212649 “FL summary#5 for AI 9.4.1.2 SL-U physical channel design framework”, RAN1#111, Moderator (Huawei).