3GPP TSG RAN WG1 #112bis-e                              R1-2303771

e-Meeting, April 17th – April 26th , 2023

Source:	Sharp

Title:	Discussion on enhanced sidelink operation on FR2 licensed spectrum

Agenda Item:	9.4.3

Document for:	Discussion and Decision

Introduction

In RAN#98e meeting, Rel-18 work item on support of [1] has the following objectives to study enhanced sidelink operation on FR2 licensed spectrum.

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

In this contribution, we share some initial views regarding support of sidelink beam management on FR2 licensed spectrum.

Discussions

Sidelink beam management

In last RAN1 meeting, agreements as below regarding sidelink bema management on FR2 licensed spectrum were reached.

Agreement For sidelink beam management, RAN1 is to study - how transmit beam(s) training and/or receive beam(s) training is performed - whether and how spatial related information (e.g., TCI, QCL, beam ID, etc) information could be identified - the relationship between PC5 unicast link establishment and sidelink initial beam pairing (e.g., whether initial beam pairing procedure starts before, during or after sidelink unicast link establishment procedure.) Agreement RAN1 is to study sidelink beam measurement and reporting schemes (e.g., periodicity, contents, container, timing, procedure, etc.) for sidelink beam maintenance. Agreement RAN1 is to study sidelink beam indication and switching schemes (e.g., framework, general procedure, contents, signaling, timing, etc.) for sidelink beam maintenance. Agreement RAN1 is to study the information related to a sidelink beam failure instance that the PHY layer provides to the MAC layer.

Timing of initial beam-pairing

Regarding the relationship between PC5 link establishment and sidelink initial beam pairing, there was a discussion regarding when the sidelink initial beam pairing should be applied relative to sidelink unicast link establishment procedure. At least the case that initial sidelink beam pairing procedure starts after sidelink unicast link establishment should be deprioritized. Without applying beamform based operation, it is concerned whether message exchange between direct communication request (DCR) message and direct communication accept (DCA) message for unicast link establishment could succeed, which needs to be evaluated. On the other hand, in our view, whether to apply initial beam pairing procedure before or during sidelink unicast link establishment procedure depends on how to design the initial beam paring, which can be further studied.

Proposal 1: For sidelink beam management, study of applying initial beam pairing procedure before or during sidelink unicast link establishment procedure is prioritized.

Reference signal for initial beam-pairing

In last RAN1 meeting, there was a discussion around which candidate options (S-SSB, sidelink CSI-RS, PSCCH/PSSCH DMRS, PSSCH/PSCCH and corresponding PSFCH) can be enhanced for studying sidelink initial beam pairing. Some arguments mentioned that the option of PSSCH/PSCCH and corresponding PSFCH does not accord with WID description regarding reusing Uu beam management concepts. In NR Uu operation, Uu beam management concept is to utilize associations between SSBs and RACH occasions for establishing initial beam pairing between gNB and UEs. Specifically, the UE measures beamformed SSBs from gNB and then can determine which RX beam is appropriate for subsequent reception for subsequent transmission according to measurement results. And the UE transmits PRACH in a PRACH occasion associated with its selected SSB. Upon reception of the PRACH and based on the association between SSB and RACH occasion, gNB can be aware of which TX beam can be applied for subsequent transmission to the UE. Therefore, utilizing association between PSSCH and corresponding PSFCH should accord with the Uu beam management concept and the minor difference is no more than channels.

Observation 1: Utilizing association between PSSCH and corresponding PSFCH for sidelink initial beam pairing accords with the Uu beam management concept where association between SSB and corresponding RACH occasions are used for initial beam pairing between gNB and UEs.

Regarding only using S-SSB for sidelink initial beam pairing, several factors should be taken into consideration. Firstly, in current NR sidelink operation, S-SSB with synchronization information is to extend the synchronization coverage. Resources for S-SSB transmission are common to syncRef UEs. If S-SSB is enhanced for sidelink beam management, how to configure UE-specific resources specific to a destination Layer-2 ID should be studied. Furthermore, considering the association between SSB and RACH, by which gNB can know TX beam for the UE, does not exist in sidelink, both UE-1 and UE-2 (a pair of peer UEs for unicast) need to beam sweep S-SSBs to each other for beam measurement. It has to be noted that, sidelink operation is a distributed system and, therefore, (pre-)configuring UE specific resources for S-SSBs with multiple beam directions specific to unicast UEs with a 24-bit destination Layer-2 ID would cause inefficient resource utilization. Secondly, SL UEs needs to know S-SSBs are transmitted from which UEs. How to include source Layer-2 ID and destination Layer-2 ID in S-SSB should be studied. Thirdly, start and termination of beam sweeping of S-SSBs should be studied as well. Above all, the feasibility and necessity of S-SSB enhancement for initial beam-pairing needs to be studied.

Proposal 2: Study the feasibility and necessity of S-SSB enhancement for initial beam-pairing including following aspects.

- How to configure and identify resources of S-SSB specific to a pair of UEs with the destination Layer-2 ID.

- How to notify SL UE ID information by S-SSB.

- When the pair of UEs start or terminate beam sweeping of S-SSBs.

Utilizing association between PSSCH and corresponding PSFCH is a feasible option for initial beam pairing. The following Figure 1 is used for the illustration. Figure 1 is a simplified example of establishing initial beam-pairing between a pair of peer UEs for sidelink unicast communication. UE-1 (i.e., UE transmitting DCR message) can select PSSCH resources according to existing NR SL Rel-16 scheme and can send the DCR message on selected PSSCH resources by applying multiple TX beams. The beam sweeping of DCR message on different PSSCH resources can help UE-2 (i.e., UE transmitting DCA message) to receive the DCR message with an appropriate RX beam.

As long as the UE-2 successfully received the DCR, the UE-2 can use a corresponding TX beam to send feedback on PSFCH occasion associated with the PSSCH resource where DCR message is sent. Herein, rather than directly sending the DCA message to UE-1 via multiple beams, use of the PSFCH transmission is to notify UE-1 of which TX/RX beams are appropriate beams to UE-2, which plays a same role as PRACH in NR Uu operation.

As illustrated in Figure 1, different PSSCH resources are mapped to respective PSFCH resources in time domain. The UE-1 is aware of association between PSSCH resource and PSFCH resource. Then according to the association between PSSCH resource and corresponding PSFCH, UE-1 can determine a RX beam for PSFCH reception based on its TX beam on associated PSSCH resource. Upon detection of PSFCH from UE-2, UE 1 can use the RX beam to receive DCA message from UE-2. Therefore, according to the association between PSSCH and corresponding PSFCH, initial beam pairing is achieved together with unicast link establishment.

Figure 1: Example of initial beam paring according to association of PSSCH and PSFCH.

Proposal 3: Study initial beam pairing according to association of PSSCH resources and PSFCH resources by considering following steps.

- UE-1 sends DCR message on multiple PSSCH resources with multiple TX beams and monitors potential PSFCH on PSFCH occasions associated to PSSCH resources.

- UE-2 receives a DCR message from beam swept DCR messages from UE-1 by an appropriate RX beam.

- UE-2 determines a TX beam corresponding to the appropriate RX beam to send PSFCH on PSFCH occasion associated with the PSSCH resource where DCR message is received.

- Upon detection of PSFCH from UE-2, UE-1 can determine the RX beam for subsequent DCA message reception.

- UE-2 sends DCA message with the determined TX beam on a PSSCH resource.

CSI-RS configuration for beam management

In existing NR sidelink CSI framework, a SL UE may set sidelink CSI-RS resource configuration and provided to an RX UE. In existing NR sidelink CSI framework, the sidelink CSI-RS is configured only for sidelink CSI acquisition for link adaption. To support of beam management on FR2 licensed spectrum, it is necessary to configure sidelink CSI-RS resource configuration for beam management. Basically a greater number of CSI-RS resource for beam management is beneficial to achieve a more precise candidate beam refinement with low latency among SL UEs. Therefore, it is necessary to study how many sidelink CSI-RS resources can be supported for beam management by considering SL UE’s antenna configuration on FR2 licensed spectrum.

In addition, existing NR sidelink CSI framework specifies sidelink CSI-RS should be transmitted within a PSSCH transmission region in a slot where a CSI request is triggered. For a resource pool, PSSCH transmission in a slot can be configured to down to 6 symbols. Furthermore, according to existing NR sidelink CSI framework, sidelink CSI-RS cannot be configured on a same symbol where PSSCH DM-RS and 2nd-stage SCI are located. That is, there is a risk that symbols available for sidelink CSI-RS configuration in a slot may be not sufficient for beam management. In Rel-16/17 NR sidelink, one SCI format can schedule up to 2 slots or up to 3 slots according to resource pool configuration. These PSSCH transmission regions in scheduled slots can be considered for sidelink CSI-RS for beam management.

Proposal 4: Study the number of sidelink CSI-RS resources for beam management on FR2 licensed spectrum.

Proposal 5: Study the necessity of not limiting CSI-RS resource to 1 slot in time domain for beam management on FR2 licensed spectrum.

CSI report for sidelink beam management

In existing NR sidelink CSI framework, sidelink CSI report is supported for link adaptation to increase transmission efficiency and reliability for unicast communication. This kind of CSI report consists of only CQI and RI. In NR Uu operation, reporting parameter L1-RSRP is used to indicate the power of a downlink TX beam from gNB and CRI is used to indicate the CSI-RS resource indicator associated with the L1-RSRP. Similarly, to support sidelink beam management and beam indication on FR2 licensed spectrum, Sidelink CSI report should include the reporting parameters CRI and RSRP. The SL UE can estimate RSRPs of different sidelink CSI-RS resources and feedback a best estimated RSRP and a CRI associated with the best estimated RSRP for beam indication.

Proposal 6: For enhanced sidelink operation on FR2 licensed spectrum, sidelink CSI report includes reporting parameters CRI and RSRP for beam reporting.

Conclusion

In this contribution, we have discussed our views on support of beam management on FR2 licensed spectrum and have the following proposals and observation.

Proposal 1: For sidelink beam management, study of applying initial beam pairing procedure before or during sidelink unicast link establishment procedure is prioritized.

Observation 1: Utilizing association between PSSCH and corresponding PSFCH for sidelink initial beam pairing accords with the Uu beam management concept where association between SSB and corresponding RACH occasions are used for initial beam pairing between gNB and UEs.

Proposal 2: Study the feasibility and necessity of S-SSB enhancement for initial beam-pairing including following aspects.

- How to configure and identify resources of S-SSB specific to a pair of UEs with the destination Layer-2 ID.

- How to notify SL UE ID information by S-SSB.

- When the pair of UEs start or terminate beam sweeping of S-SSBs.

Proposal 3: Study initial beam pairing according to association of PSSCH resources and PSFCH resources by considering following steps.

- UE-1 sends DCR message on multiple PSSCH resources with multiple TX beams and monitors potential PSFCH on PSFCH occasions associated to PSSCH resources.

- UE-2 receives a DCR message from beam swept DCR messages from UE-1 by an appropriate RX beam.

- UE-2 determines a TX beam corresponding to the appropriate RX beam to send PSFCH on PSFCH occasion associated with the PSSCH resource where DCR message is received.

- Upon detection of PSFCH from UE-2, UE-1 can determine the RX beam for subsequent DCA message reception.

- UE-2 sends DCA message with the determined TX beam on a PSSCH resource.

Proposal 4: Study the number of sidelink CSI-RS resources for beam management on FR2 licensed spectrum.

Proposal 5: Study the necessity of not limiting CSI-RS resource to 1 slot in time domain for beam management on FR2 licensed spectrum.

Proposal 6: For enhanced sidelink operation on FR2 licensed spectrum, sidelink CSI report includes reporting parameters CRI and RSRP for beam reporting.

References

RP-222806 “WID revision: NR sidelink evolution”, RAN#98e, OPPO.