3GPP TSG RAN WG1 Meeting #112bis-e 	R1-2303774

e-Meeting, April 17th – April 26th, 2023

Agenda Item: 	9.5. 2 NR DL and UL carrier phase positioning

Source: 	Locala, Inc.

Title: 	Discussion on integer number solutions

Document for: 	Discussion & Decision

# Introduction

In the last #98  RAN plenary meeting following text was agreed upon as WID in carrier phase positioning [5].

===========================================================

- Specify physical layer measurements and signaling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4].

  - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements.

  - Specify measurements that are limited to a single carrier/PFL.

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

==========================================================

According to the above agreement, the integer ambiguity solution that can support NR  DL/UL carrier phase positioning should be based on a single carrier. In this contribution, we propose a method of using a subcarrier solution within a single carrier to resolve the integer ambiguity, synchronize gNB/TRPs and achieve accurate Downlink/Uplink carrier phase positioning.

# Network indication of phase synchronized PRS transmission

Carrier phase positioning system measures the distance using the phase information of the received signal. Therefore, it is important to synchronize the initial carrier phase of the PRS signals transmitted by multiple TRPs. Otherwise, UEs receiving the PRS may have to perform complicated procedures to compensate for the initial phase offset errors of different TRPs.

When the network is configured so that the TRP devices all share one external clock source , the TRPs can be perfectly synchronized. Since the PRS signals transmitted by TRPs are also carrier phases synchronized as if they were transmitted in one system, the UEs receiving them do not need to perform additional procedures to correct the initial phase offset error. High-precision positioning can be achieved just by comparing the PRS carrier phase.

Figure 1 below shows an example of synchronization network that shares an external clock source and the UE achieves UL/DL carrier phase positioning using phase synchronized PRS . For a detailed discussion of this, see R1-2211259.

Figure 1. UL/DL carrier phase positioning in perfectly synchronized network

The above synchronization network configuration method is simple and can be configured at a low cost in a relatively small indoor environment , so this application method must be supported by the 3GPP system .

However, the double-difference method currently being discussed in RAN1 may hinder efficient positioning of the UE in such synchronized network. The UE may always have to wait to receive unnecessary phase correction information even when the network already corrected the initial phase offset using the TRP synchronization function.

The double-difference method is originally designed to compensate for the problem of the GPS carrier phase system. This method has several problems that are not suitable for more advanced 3GPP systems. For example, as discussed in R1-2209543 and R1-2211259, this method can generate a huge traffic overhead in the 3GPP system, and in the process of broadcasting the correction information measured by the PRU, latency increases and performance deteriorates.

Nevertheless, in RAN1 there are still some companies supporting the double-difference method. Therefore, there must be a way to explicitly distinguish between the case where double-difference is used and the case where TRP synchronization is used.

If such distinction is not provided, it may cause great inefficiency in the synchronized network as in Figure 1. In other words, the UE should be able to receive network indication so that it can perform the position evaluation procedure immediately upon receiving the PRS, when the TRPs are sufficiently synchronized.

(Observation 1) If the network is configuraed to share external clock source, it is not necessary to employ post-phase correction method such as the double-difference method.

(Proposal 1) Network notification should be delivered to UE when a separate means of PRS phase synchronization is provided, so that the UE does not need to wait for additional initial phase error correction information.

3 . TRP synchronization method using PRS

In an outdoor environment, the cost for connecting TRPs through a separate wired synchronization network may be high. A cost-effective alternative is using a wireless synchronization network. For example, in R1-2209543, and R1-2211259, PRS listening method between neighboring TRPs, and use of the clock phase information for synchronizing the carrier phase of PRS were presented. Such capability of listening PRS signal transmitted by neighboring TRPs is possible in TDD mode if the tx slot time and rx slot time between TRPs are matched. In particular, when this function is combined with the existing muting sequence, the PRS listening function can be implemented with minor spec modification.

In this chapter, we explain in more detail the solution for TRP synchronization using the PRS signal.

(Observation 2) In the TDD system, it is possible to implement the PRS listening between neighboring TRPs by adjusting the tx / rx slot time. In addition, when combined with existing PRS muting sequence , it can be supported with minor impact.

(Proposal 2) At least in the TDD system, a method that can calculate the relative clock phase offset between TRPs by listening to the PRS signal transmitted between TRPs must be supported.

FFS: How to harmonize the PRS muting procedure and the PRS listening procedure

Figure 2. PRS listening and phase synchronization between neighboring TRPs in TDD mode

In Figure 2 above, two neighboring TRP-A, B are operating in TDD mode. Each of these neighboring TRP-A and TRP-B use unsynchronized internal clock.  Assuming that they each have an error of ψA and ψB compared to the ideal absolute time t, their local  time can be expressed as (t-ψA) and (t-ψB), respectively. Therefore the PRS signal periodically transmitted by the TRP-A can be represented by the local time as in Equation (1) below.

(1)

PRSAt=A1ejω1t-ψA+A2ejω2t-ψA+…+Amejωmt-ψA=q=1mAqejωqt-ψA

In Equation (1) above, w1, w2, … wm means the subcarrier frequency at which the PRS is allocated.  In the comb-N configuration, these subcarriers can be placed at regular intervals (subcarrier space x N). A1, A2, … Am is the initial phase sequence of each subcarrier constituting the PRS signal. For example, a Gold  sequence can be used, and the sequence value can be removed at the receiver by multiplying the conjugate of this sequence.

The PRS signal is periodic and can be repeatedly transmitted in k times the  slot duration, so it can be expressed as Equation (2) below.

(2)

PRSAt=PRSAt+kL ,   L=slot length

In  Equation (2) above, L is the length of one slot, and k is the PRS signal iteration cycle. Due to the periodicity of the phase signal, the phase value measured at time (t+kL) is the same as the phase value measured at time t. This means that the PRS signal phases measured at different slot times can be considered as signals measured simultaneously at the same slot  time t , and directly compare the phases regardless of slot location.

The baseband PRS signal of Equation (1) above is up-converted by mixing with the carrier frequency wc as shown in Equation (3) below.

(3)

PRSApasst=ejωct-ψA∙PRSAt=q=1mAqejωc+ωqt-ψA

Since the  local timing error of A is ψA, the same  timing offset is applied  to carrier  signal and multiplied by (t - ψA), in  Equation (3) above.

The passband signal in (3) above reaches TRP-B after the tAB delay. This can be represented by Equation (4) below. To simplify the description, equation (4) does not include a multipath channel expression, but just shows a single transmission delay on a line-of-sight path.

(4)

PRSAcht=e-jωc+ωqτAB∙PRSApasst=q=1mAqejωc+ωqt-ψA-τAB

Looking closely at Equation (4) above, it can be seen that the PRS signal received from UE-B has a signal phase rotated by the propagation delay tAB between A and B. The receiver can measure this phase rotation to obtain propagation delay and distance information.

The target UE-B multiplies the received passband PRS signal by the inverse carrier frequency to down-conversion. In Equation (5) below, the local clock error of UE-B is ψB, so the carrier signal is multiplied by (t-ψB).

(5)

yBt=e-jωct-ψB∙PRSAcht=qAqejωcψB-ψA-τAB+ωqt-ψA-τAB

TRP-B collects the downconverted baseband signal samples of Equation (5) above and decomposes it using FFT. The frequency domain samples corresponding to the PRS subcarriers w1, w2, … wm  are  extracted. This is expressed in the vector form of Equation (6). The detailed DFT decomposition formula is not described further.

(6)

YB=N∙A1ejωc+ω1ψB-ψA-τABA2ejωc+ω2ψB-ψA-τAB⋮Amejωc+ωmψB-ψA-τAB+σB

The carrier phase vector YB in Equation (6) above contains both local clock error (𝜓B - 𝜓A) and path delay tAB in the exponent part of the complex number.  σB represents the residual error that remains after the FFT computation.

Note that the carrier phase vector YB includes the initial phase sequence A1, A2, … Am of the PRS signal sent by anchor UE-A. It can be removed if the UE-B knows the sequence.

Summing up all complex elements of the YB vector from which the initial phase sequence value of A has been removed, and applying the arc-tangent (i.e. angle) function, the carrier phase formula for the carrier frequency can be extracted as shown in Equation (7) below .

(7)

argsumYB≅argejωc+ω∆ψB-ψA-τAB=ωc+ω∆ψB-ψA-τAB-2πNcB

In the  carrier phase equation of (7) above, wc is the carrier frequency, wΔ is the median value of  w1, w2, … wm , that is the mean frequency.  (wc + wΔ)  is the sum of the carrier frequency and the subcarrier frequency, which can be a very large number outside the +/-π . However, since the  arg(...)   function calculated by the arc-tangent function returns only values between -π ~ +π , the formula must be compensated by estimating the number of lost carrier cycles. This is a well-known  Integer Ambiguity  problem. NcB is the Integer Number (i.e. number of phase cycles) to estimate.

If  TRP-A, B are positioned and the propagation delay tAB between the two is known in advance, we can calculate the unknown integer number NcB  in advance as shown in Equation (8) below.

(8)

NcB=roundωc+ω∆τAB/2π

In Equation (8) above, round(…)  is an integer number conversion function.

By applying the NcB calculated by Equation (8) to Equation (7) above and rearranging the equation, we can obtain local  timing difference  (𝜓B - 𝜓A) between the TRP-A and B , as in Equation (9).

(9)

εBA=ψB-ψA=argsumYB+2πNcBωc+ω∆+τAB

The  εBA in Equation (9) above represents the difference between the  local time of TRP-B and  the local time of TRP-A. Therefore, when TRP-B rotates the subcarrier phase of the PRS signal by the amount of εBA ,  passband signal phase of PRSB can be synchronized to be the same as PRSA. The vector used for rotating the phase of the PRS signal is called curl-vector, as shown in Equation (10) below.

(10)

curlvectBA=ejωc+ω1εBAejωc+ω2εBA⋮ejωc+ωmεBAT=ejωc+ω1ψB-ψAejωc+ω2ψB-ψA⋮ejωc+ωmψB-ψAT

The TRP-B multiplies the curl-vector in Equation (10) above to each subcarriers of PRS. This process can also be expressed as inner product of the curl-verctor with the PRS symbol vector, as in Equation (11)  below. This gives the effect of rotating the PRS phase by the amount of εBA.

(11)

PRSBt=curlvectBA∙B1ejω1t-ψBB2ejω2t-ψB⋮Bmejω3t-ψB=ejωc+ω1ψB-ψAejωc+ω2ψB-ψA⋮ejωc+ωmψB-ψAT∙B1ejω1t-ψBB2ejω2t-ψB⋮Bmejω3t-ψB

=q=1mBqejωqt-ψA+ωcψB-ψA

In Equation (11) above, w1, w2, … wm is the comb-N subcarrier frequency allocated for the PRSB.  B1, B2, … Bm is the initial phase sequence of each subcarrier of B, and (t-ψB)  is the local time of B. The phase-rotated  PRSB  of  Equation (11) above is mixed with the carrier frequency wc as shown in Equation (12).

(12)

PRSBpasst=ejωct-ψB∙q=1mBqejωqt-ψA+ωcψB-ψA=q=1mBqejωc+ωqt-ψA

Note in (12) above, the carrier phase of upconverted passband PRSB becomes (t-ψA)  which is the same as the phase of PRSA because of the phase rotation. This phase synchronization effect can clearly be seen in (13) below where the passband phase representation of PRSA is compared with PRSB.

(13)

PRSBpasst=q=1mBqejωc+ωqt-ψA

PRSApasst=q=1mAqejωc+ωqt-ψA

In Equation (13) above, it is observed that the passband phase (t-ψA)   of PRSB is idential to passband phase of PRSA , except the coefficient part. As a result, TRPs may transmit phase coordinated PRS signals.

(Observation 3) Using the curl-vector of Equation (10) and rotating the subcarrier phase value of the PRS signal to be transmitted, the PRS phases of neighboring TRPs can be synchronized.

(Proposal 3) RAN1 should study the benefit and efficiency of the proposed TRP synchronization method, and also application of the curl-vector for removing initial phase offset errors between TRPs.

In the following section, we present further the effect of the PRS phase synchronization method in terms of the UE side procedure.

Equation (14) below shows the phase-rotated PRSA signal reaching UE-R after the propagation delay of the tRA. It also shows the PRSB signal synchronized to the phase of the PRSA and reaching UE-R after the propagation delay of tRB.

(14)

PRSAcht=e-jωc+ωqτRA∙PRSApasst=q=1mAqejωc+ωqt-ψA-τRA

PRSBcht=e-jωc+ωqτRB∙PRSBpasst=q=1mBqejωc+ωqt-ψA-τRB

It is assumed the local clock error of the UE-R is ψR . The UE-R downconverts the received passband PRS signals as shown  in Equation (15) below.

(15)

yRAt=e-jωct-ψR∙PRSAcht=qAqejωcψR-ψA-τRA+ωqt-ψA-τRA

yRBt=e-jωct-ψR∙PRSBcht=qBqejωcψR-ψA-τRB+ωqt-ψA-τRA

UE-R collects the downconverted baseband signal samples and extracts the the PRS subcarriers w1, w2, … wm via FFT process. The subcarrier vectors of the received PRSs are shown in Equation (16 ) below.

(16)

YRA=N∙A1ejωc+ω1ψR-ψA-τRAA2ejωc+ω2ψR-ψA-τRA⋮Amejωc+ωmψR-ψA-τRA+σRA

YRB=N∙B1ejωc+ω1ψR-ψA-τRBB2ejωc+ω2ψR-ψA-τRB⋮Bmejωc+ωmψR-ψA-τRB+σRB

In  Equation (16) above, YRA represents the carrier phase vector of PRSA, and YRB represents the carrier phase vector of PRSB. A1, A2, … Am ,  B1, B2, … Bm represents the initial phase sequence of each vector. Since the initial phase sequence only biases the phase calculation, it is removed by multiplying the conjugate of the known sequence.

The carrier phase difference vector is obtained by element-wise conjugate multiplication of the vector YRA and YRB, as in  Equation (17) below.

(17)

YRAYRB*=N∙ejωc+ω1ψR-ψA-τRAejωc+ω2ψR-ψA-τRA⋮ejωc+ωmψR-ψA-τRA⨁N∙e-jωc+ω1ψR-ψA-τRBe-jωc+ω2ψR-ψA-τRB⋮e-jωc+ωmψR-ψA-τRB=N2∙ejωc+ω1τRB-τRAejωc+ω2τRB-τRA⋮ejωc+ωmτRB-τRA

In Equation (17) above, it is noted that the  initial phase offset errors of TRP-A  and TRP-B are removed at the UE side, and leaves only the path difference components of (tRB - tRA). It is because the passband carrier phase of PRSs are synchronized at the TRPs. This simplifies the downlink procedure for UE based carrier phase positioning.  Therefore, we do not need post-correction method for intial phase error, such as double-difference method.

Now, we can sum-up the elements of the carrier phase vector in (17) and apply the arc-tangent (i.e. angle) function in order to obtain the carrier phase differece equation as in (18) below.

(18)

argsumYRAYRB*≅argejωc+ω∆τRB-τRA=ωc+ω∆τRB-τRA-2πNR

The carrier phase difference Equation (18) above has an unknown integer part of 2πNR.  Estimating this unknown integer number gives a time difference value of (tRB - tRA) , which can be used to calculate the accurate distance difference. Detail is discussed in next section.

4. Solution for integer number resolution using subcarrier

The GPS carrier phase method based  on the code-based positioning method estimates the integer number by applying very complex iteration method such as the Lambda . Some companies in RAN1 are also trying to adopt the GPS method and use the initial UE location for estimating the integer cycle number. However, unlike the GPS which constantly moves earth orbit, gNBs are fixed and it is difficult to find a point of convergence to an integer number through repetition. Legacy positioning methods such as TDoA are not helpful for resolving the integer number of carrier phase in 3GPP system.

Finding the integer number for carrier phase is relatively simple in OFDM system because there’s an integer multiple relationship between carrier and subcarrier wavelengths.

For example, let the frequency of a subcarrier number q is fq Hz, and the carrier frequency that modulates and transmits it is fc Hz. If 𝕔 is the speed of light, the subcarrier wavelength is (𝕔/ fq) meter, which is (fc / fq) times the carrier wavelength.

Suppose two transmitting and receiving UEs A and B are now calculating the distance by measuring the carrier phase as well as the subcarrier phase of a PRS signal. We assume the subcarrier q is the longest subcarrier consisting the PRS signal. If the distance between the UE-A and UE-B is less than 1/2 times the subcarrier wavelength (𝕔/ fq) meter, then we can uniquly measure the distance using the subcarrier wavelength without ambiguity. Let tAB is the propagation delay between A and B, and 𝜙s is the measured subcarrier phase. Then the subcarrier phase can be represented as (19).

(19)

ϕs=fqτAB  ,     τAB=ϕsfq

Simularily, the carrier phase 𝜙c  measured using the PRS can be represented as (20) below.

(20)

ϕc=fcτAB-2πN

In Equation (20), N is the unknown integer number to solve. Substituting tAB in Equation (19) into (20), and rearranging the equation for the integer number N gives the integer number estimation equation (21).

(21)

N=round fc/fq∙ϕs/2π-ϕc/2π

In the above equation (21), round(...) is an integer number conversion function, (fc / fq) is the integer multiple ratio of the carrier wavelength divided by the subcarrier wavelength.  𝜙s/2𝜋  and  𝜙c/2𝜋  are normalized carrier and subcarrier phase value in the range of 0~1 (or  -0.5 ~ +0.5).

The Equation (21) is non-iterative, deterministic solution, which allows to determine both the UE's initial position as well as the unknown integer number without the help of the legacy positioning method.

In the OFDM system, the carrier phase information can be obtained using a FFT decomposed carrier phase vector, such as Equation (16) and (18). In (18), NR is the integer cycle number to estimate. The subcarrier phase information can be extracted from (m-1) length carrier phase vector.

(22)

YRA+1YRB+1*YRA-1YRB-1**=N2∙ejωc+ω2τRB-τRAejωc+ω3τRB-τRA⋮ejωc+ωmτRB-τRA⨁N2∙e-jωc+ω1τRB-τRAe-jωc+ω2τRB-τRA⋮e-jωc+ωm-1τRB-τRA

=N4∙ejω2-ω1τRB-τRAejω3-ω2τRB-τRA⋮ejωm-ωm-1τRB-τRA=N4∙ejωgτRB-τRAejωgτRB-τRA⋮ejωgτRB-τRA

In Equation (22), (YRA+1Y*RB+1) means the conjugate complex of the lower (m-1) length partial vectors of Equation (16) and (YRA-1Y*RB-1)* means the upper (m-1) length partial vectors.

The carrier vector in Equation (16) is expressed as a product of the carrier frequency (wc + wΔ) but the subcarrier vector in Equation (22) above cancels out and is expressed as a product of the subcarrier gap frequency wg (=wm–wm-1).  This gap frequency wg corresponds to (subcarrier  space x N) Hz, the subcarrier placement interval of the com-N PRS. Therefore, determining wg Hz, the subcarrier placement interval of the  PRS reference signal, is an important factor in designing the carrier phase system.

Since all of the (m-1) subcarrier vector elements in Equation (22) above have the same result value, all of them are complexly summed and arg(…).By applying the function, you can obtain a phase measurement for the gap frequency wg  Hz subcarrier.

(23)

argsumYRA+1YRB+1*YRA-1YRB-1**≅argejωgτRB-τRA=ωgτRB-τRA

Equation (23) above is a phase difference value measured using a long subcarrier with a wavelength of (2𝕔/wg) meter.

If the distance difference from UE-R to both TRP-A and B is less than 1/2 of this subcarrier wavelength, the result is always determined only within the range of -𝜋~ + 𝜋. Therefore, unlike the carrier phase equation of Equation (18), the value of (tRB-tRA) can be determined without estimating an unspecified number, and by converting it into a distance difference and applying a three-way position algorithm, etc., the UE can be approximately initial positioned.

(Observation 4) Using the long wavelength subcarrier information, the UE's initial location and integer number can be estimated.

(Proposal 4) The phase-based standalone positioning method should be suppoted

- If it is necessary to report the legacy positioning results, report a ToA or TDoA value that translates the phase information measured using subcarriers.

Using the above subcarrier phase difference equation (23), it is possible to obtain approximate initial location information of the UE-R without the help of the legacy positioning method. However, since the subcarrier phase measurement resolution is not higher than the carrier phase, the error may be large. However, it may be sufficient to estimate the undetermined number of carriers.

By transforming the above formula  (23) into an equation for (tRB-tRA) and substituting it into equation (18) and summarizing the unspecified NR, you can obtain an unspecified number solution of formula (24) below. This is basically the same form as Equation (21).

(24)

NR=roundωc+ω∆ωgargsumYRA+1YRB+1*YRA-1YRB-1**2π-argsumYRAYRB*2π

By substituting the unspecified NR obtained above into Equation (18) again, we can obtain an improved time difference measurement formula for Equation (25) (tRB-tRA).

(25)

τRB-τRA=argsumYRAYRB*+2πNRωc+ω∆

The measurement results using the carrier phase of Equation (25) above are better than those using the subcarrier phase of Equation (23) and are more precise than legacy positioning. The results of the outdoor experiment related to this are reported in R1-2211259.

# Conclusion

In this paper, we introduced how to synchronize the PRS phase by listening to the PRS signals transmitted by TRPs.  A simple procedure called curl-vector multiplies the phase rotation vector by the  PRS subcarrier allows multiple TRPs to synchronize the phase of the PRS signal, especially if the UE's fast positioning performance is important, such as sidelink applications.

Chapter 4 discloses an equation for determining an unspecified number using subcarrier phase measurement. This method can be applied to determine the approximate initial location of the UE, especially to measure the distance between vehicles in a sidelink environment where the legacy positioning method is difficult to apply.

## Observations

(Observation 1) If the network is configuraed to share external clock source, it is not necessary to employ post-phase correction method such as the double-difference method.

(Observation 2) In the TDD system, it is possible to implement the PRS listening between neighboring TRPs by adjusting the tx / rx slot time. In addition, when combined with existing PRS muting sequence , it can be supported with minor impact.

(Observation 3) Using the curl-vector of Equation (10) and rotating the subcarrier phase value of the PRS signal to be transmitted, the PRS phases of neighboring TRPs can be synchronized.

## Proposals

(Proposal 1) Network notification should be delivered to UE when a separate means of PRS phase synchronization is provided, so that the UE does not need to wait for additional initial phase error correction information.

(Proposal 2) At least in the TDD system, a method that can calculate the relative clock phase offset between TRPs by listening to the PRS signal transmitted between TRPs must be supported.

FFS: How to harmonize the PRS muting procedure and the PRS listening procedure

(Proposal 3) RAN1 should study the benefit and efficiency of the proposed TRP synchronization method, and also application of the curl-vector for removing initial phase offset errors between TRPs.

(Proposal 4) The phase-based standalone positioning method should be suppoted

- If it is necessary to report the legacy positioning results, report a ToA or TDoA value that translates the phase information measured using subcarriers.

# References

| [1] | draft TR38.859 v0.2.0, Study on expanded and improved NR positioning (Release 18), RAN1, August, 2022 |
| --- | --- |
| [2] | 3GPP R1-2104880, "Carrier/Subcarrier Phase Based Enhancement for 5G NR Positioning", Dankook University, RAN WG1 #105-e e-Meeting, May 10th – 27th, 2021 |
| [3] | 3GPP R1-2104844, "Carrier Phase Based Downlink Angle of Departure Measurement", Dankook University, RAN WG1 #105-e e-Meeting, May 10th – 27th, 2021 |
| [4] | 3GPP R1-2101470, "Potential Enhancements on DL-AoD positioning", Qualcomm Incorporated, RAN WG1 #104-e e-Meeting, January 25th – February 5th, 2021 |
| [5] | Paul de Jonge, Christian Tiberius, “The LAMBDA method for integer ambiguity estimation: implementation aspects,” LGR-Series, Publications of the Delft Geodetic Computing Centre, August, 1996 |
| [6] | 3GPP R1-2203635, "Continuous PRS for improved carrier phase measurement" , Dankook University , RAN WG1 #109-e e-Meeting, May 9th – 20th, 2022 |
| [7] | https://youtu.be/F0GrAait8Vo |
| [ 8] | www.locaila.com _ |