3GPP TSG RAN WG1 #112bis-e	                           R1-2303784

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.1.1

Source: 	ASUSTeK

Title:  	Discussion on sidelink positioning reference signal

Document for:	Discussion and Decision

- Introduction

In last RAN1 meeting [1], there are some agreements about SL PRS.

Agreement

- For SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID,seqSL-PRS.

- The pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS

Agreement

For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option:

  - Option 1: nID,seqSL-PRS is a higher layer configured parameter

  - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission

  - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission

  - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID

  - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits

  - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined.

Agreement

- Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools.

  - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools.

- For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible .

  - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values.

- FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs

Agreement

TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools.

- FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools.

- FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols.

- FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any

- FFS: which resource allocation schemes are applicable

- FFS: whether or not this is a separate UE capability

In this contribution, we discuss some aspects for SL PRS design.

- Discussion

Regarding to PRS sequence generation, for NR positioning, sequence ID of DL PRS and UL SRS are configured by higher layer parameter. For sidelink positioning, it seems not always operating in network coverage. Furthermore, sidelink positioning may be utilized for different sidelink services and even with different cast types. Considering RX UE needs to decode SCI first and measurement SL PRS transmission scheduled by the SCI, it will be proper to define SL PRS sequence ID based on layer-1 destination ID, layer-1 source ID, or CRC of PSCCH associated with the SL PRS transmission.

Proposal 1:  For defining parameter nID,seqSL-PRS, support one of option 2, 5, 6.

In previous meeting, it agreed to support TDM-based multiplexing of SL PRS in a slot for dedicated resource pools. It means that there can be more than one SL PRS occasion in a slot. Thus, RE level multiplexing and flexibility on SL PRS occasion number are quite different from legacy PSSCH, which is sub-channel level multiplexing and only one PSSCH occasion in a slot. If fixed mapping rule between PSCCH and PSSCH resource is applied, two SL PRS resources in a slot will possibly correspond to the same one resource of PSCCH. It will induce resource collision and severe performance impacts on PSCCH reception and corresponding SL PRS measurement. Thus, resource mapping between PSCCH and SL PRS resource needs re-design for dedicated resource pool. For instance, resource mapping between PSCCH and SL PRS resource needs to consider RE offset and SL PRS occasion index.

Proposal 2:  In dedicated resource pool, resource mapping between PSCCH and SL PRS resource needs further design with consideration on RE offset and SL PRS occasion index.

- Conclusion

In this contribution, we have following proposals about SL PRS design:

Proposal 1:  For defining parameter nID,seqSL-PRS, support one of option 2, 5, 6.

Proposal 2:  In dedicated resource pool, resource mapping between PSCCH and SL PRS resource needs further design with consideration on RE offset and SL PRS occasion index.

- Reference

- RAN1 #112 chairman’s note