3GPP TSG RAN WG1 #112bis-e			R1-2303786

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.1.3

Source: 	ASUSTeK

Title:  	Discussion on resource allocation for SL PRS

Document for:	Discussion and Decision

- Introduction

In previous RAN1 meeting [1], there are some agreements about SL PRS resource allocation.

Agreement

- A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools).

- SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool.

Agreement

For a dedicated resource pool for Positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further.

- Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

Agreement

Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further).

Agreement

Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication).

  - Working assumption: Sensing-based and random selection can be allowed in the same resource pool

  - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool

- FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17.

Agreement

In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options:

- Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers.

  - Note: this also includes higher layer triggering from another UE

- Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A.

  - FFS: Whether lower-layer signaling is SCI or SL MAC-CE

Agreement

For the scheme 2 sensing-based resource allocation,

- Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool.

  - Study if/what changes are needed

- Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool.

  - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc)

- From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective

In this contribution, we discuss some aspects for SL PRS resource allocation.

- Discussion

In NR sidelink, sub-channel based resource allocation is applied for PSSCH. With regards to bandwidth of SL PRS, there are two alternatives. Alt.1 is that bandwidth of SLPRS can be same or smaller than that of the resource pool, which means that sub-channel based resource allocation can be also applied for SL PRS in dedicated resource pool. Alt.2 is that bandwidth of SL PRS shall be same as that of the resource pool. Different bandwidth requirement of positioning/ranging may correspond to SL PRS transmission and measurement in different dedicated resource pool. Thus, Alt.2 may induce more resource pool partition in a carrier and require further resource pool selection procedure. In comparison, Alt.1 can provide more flexibility and efficiency on resource utilization. To mitigate possible SL PRS resource blocking, starting sub-channels can be restricted in some candidate sub-channels for different SL PRS transmission bandwidths, which restriction is similar as SRS resource design.

Proposal 1:  For dedicated resource pool, bandwidth of SL-PRS can be same or smaller than that of the resource pool.

Regarding resource allocation for SL PRS, two schemes are introduced. UE can be configured to perform either Scheme 1 or Scheme 2, applicable to all configured resource pools.

In scheme 1 corresponding to network-centric SL PRS resource allocation, the network will allocate SL PRS resource(s) via SL PRS grant. In previous RAN1 meeting, it is agreed to not further consider SL-PRS resource allocation through higher layers from the LMF. Thus, the main issue in scheme 1 is to design such SL PRS grant, since granularity of SL PRS resource allocation is a little different from PSSCH, e.g., SL PRS resource requires further scheduling information of RE offset and SL PRS occasion indication. Moreover, to differentiate SL grant for PSSCH and SL PRS grant for dedicated resource pool, either RNTI or resource pool index can achieve such differentiation. Regarding whether to include HARQ process, NDI, PUCCH resource indication, it will depend on further study on whether and how to support SL PRS retransmission.

Proposal 2:  In scheme 1, introduce SL PRS grant for allocating SL PRS resource(s) with scheduling information of RE offset and SL PRS occasion indication.

In scheme 2 corresponding to UE autonomous SL PRS resource allocation, it shall support at least sensing-based resource allocation to avoid resource collision. Following principle of legacy sidelink, TX UE needs to provide sidelink control information for indicating/reserving SL PRS resources, which can assist RX UE in SL PRS measurement and also assist other UEs in performing its sensing-based resource allocation. To avoid design complexity, it is better to also keep legacy sidelink principle such that each SL PRS transmission is associated with a SCI transmission in the same slot. Thus, PSCCH which carries SCI associated with SL PRS transmission(s) shall be included in dedicated resource pool for SL positioning.

The sensing-based resource allocation in scheme 2 can be introduced mostly based on sidelink mode 2 design, just with some modification. For instance, one difference is that PSSCH applies sub-channel based resource allocation, while SL PRS resources are differentiated in RE level, which expects to suffer higher interference due to Doppler frequency shift. It is FFS on how to deal with such interference issue in scheme 2.

Observation 1:  SL PRS resource differentiation in RE level will suffer higher interference due to Doppler frequency shift.

Proposal 3:  In scheme 2, sensing-based SL PRS resource allocation is introduced mostly based on sidelink mode 2 design, and further considers interference issue due to resource differentiation in RE level.

- Conclusion

In this contribution, we have following proposals about SL PRS resource allocation:

Proposal 1:  For dedicated resource pool, bandwidth of SL-PRS can be same or smaller than that of the resource pool.

Proposal 2:  In scheme 1, introduce SL PRS grant for allocating SL PRS resource(s) with scheduling information of RE offset and SL PRS occasion indication.

Observation 1:  SL PRS resource differentiation in RE level will suffer higher interference due to Doppler frequency shift.

Proposal 3:  In scheme 2, sensing-based SL PRS resource allocation is introduced mostly based on sidelink mode 2 design, and further considers interference issue due to resource differentiation in RE level.

- Reference

- RAN1 #112 chairman’s note