3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303792

E-meeting, 17 – 26 April, 2023

Agenda Item:	7.2

Source:	Huawei, HiSilicon, CBN

Title:	Remaining issues for Rel-17 MBS

Document for:	Discussion and Decision

# Introduction

FLS#3 on the HARQ-ACK related issues for Rel-17 NR MBS [1] documented the issues discussed in the last meeting. This paper discusses the remaining issues.

# Issue#1: Type-1 CB for ‘fdmed-ReceptionMulticast’

For the case of UE provided the configuration of joint HARQ-ACK feedback for two TRPs or the case of UE provided fdmed-ReceptionMulticast for multiplexing HARQ-ACK for unicast and multicast, the Type-1 CB generation per the current specification is similar. Even though UE does not detect any DCI pertaining to one of TRP or unicast for example, the Type-1 CB should also include the HARQ-ACK information for the TRP or for unicast:

If a UE - is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ackNackFeedbackMode = joint where - a serving cell is placed in a first set S0 of NcellsDL,0 serving cells if the serving cell includes a first CORESET, and - a serving cell is placed in a second set S1 of NcellsDL,1 serving cells if the serving cell includes a second CORESET, and - serving cells are placed in a set according to an ascending order of a serving cell index the UE generates a Type-1 HARQ-ACK codebook for the set S0 and the set S1 of serving cells separately by setting NcellsDL=NcellsDL,0 and NcellsDL=NcellsDL,1 in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set S0 followed by the HARQ-ACK codebook generated for the set S1 to obtain a total number of OACK HARQ-ACK information bits. If a UE is provided fdmed-ReceptionMulticast and the UE is configured to monitor PDCCH for detection of unicast DCI formats and to monitor PDCCH for detection of multicast DCI formats - a serving cell is placed in a first set SU of NcellsDL,U serving cells if the UE is configured to monitor PDCCH for DCI formats 1_0/1_1/1_2 for scheduling on serving cell c, and - a serving cell is placed in a second set SM of NcellsDL,M serving cells if the UE is configured to monitor PDCCH for detection of DCI format 4_1/4_2 for scheduling on serving cell c, and - serving cells are placed in a set according to an ascending order of a serving cell index the UE generates a Type-1 HARQ-ACK codebook for the set SU and the set SM of serving cells separately by setting NcellsDL=NcellsDL,U and NcellsDL=NcellsDL,M in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set SU followed by the HARQ-ACK codebook generated for the set SM to obtain a total number of OACK HARQ-ACK information bits.

For the case of UE provided fdmed-ReceptionMulticast, the draft CR [2] for Type-1 CB generation was not approved and three alternatives were identified for further discussion [1]:

- Alt1: No CR. Follow the current specification.

- Alt2: Change the Type-1 codebook generation as suggested by the draft CR and make the change for the case of mTRP operation as well.

- Alt3: Change the Type-1 codebook generation as suggested by the draft CR only for multiplexing HARQ-ACK for unicast and multicast.

Essentially, the question is whether further reducing the Type-1 HARQ-ACK CB size as in Alt2 or Alt3 is needed in the maintenance stage. Considering the Type-1 CB generation is the same for mTRP and multiplexing unicast and multicast and the case for mTRP was discussed and introduced by a different topic, it is preferred to not change the Type-1 CB generation procedures for the concerned cases.

Proposal 1: No CR is needed for Type-1 CB construction for multiplexing HARQ-ACK for unicast and multicast.

# Issue#2: Disabled HARQ-ACK not applied to SPS ac/dea

According to the current description in clause 18 TS 38.213 regarding disabling HARQ-ACK feedback by the parameter of harq-FeedbackEnablerMulticast:

- when set to ‘enabled’, UE provides HARQ-ACK information for PDSCH reception. What about SPS release? The common understanding should be reporting HARQ-ACK, which should be added for clarity.

- when not provided, UE does not provide HARQ-ACK information for PDSCH reception. Is it applied to the first PDSCH after SPS activation? It is controversial and has been discussed for several meetings yet with no consensus.

Technically, the main concern for no HARQ-ACK information for the first PDSCH after SPS activation is the DCI missing by UE is not aware by network and the subsequent SPS PDSCH transmissions will be wasted in such a case.

18 Multicast Broadcast Services …. A UE can be configured per G-RNTI for multicast or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI for multicast or G-CS-RNTI and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI for multicast or a G-CS-RNTI, the UE provides HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_1 associated with the G-RNTI or the G-CS-RNTI, and determines whether or not to provide the HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_2 based on an indication by the multicast DCI format 4_2 associated with the G-RNTI for multicast or the G-CS-RNTI [4, TS 38.212]. If a UE is provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE does not expect to be provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI. …..

It is also noted that, when the parameter of ‘harq-feedbackEnablingforSPSactive’ is ‘enabled’, UE will report HARQ-ACK for the first PDSCH after SPS activation when UE is provided downlinkHARQ-FeedbackDisabled according to the following description for example in TS38.213 clause 9.1.2:

9.1.2 Type-1 HARQ-ACK codebook determination …… If a UE is provided downlinkHARQ-FeedbackDisabled indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in PDSCH reception occasion m on serving cell c, the UE reports a NACK value for a HARQ-ACK information bit corresponding to the transport block in a Type-1 HARQ-ACK codebook and does not consider the transport block as received in the determination of Nm,creceived in clause 9.1.2.1. If the UE is also provided PDSCH-CodeBlockGroupTransmission, the UE reports NACK values for HARQ-ACK information bits corresponding to CBGs of the transport block in the Type-1 HARQ-ACK codebook and does not consider the CBGs as received in the determination of Nm,creceived,CBG in clause 9.1.2.1. If the UE is also provided harq-feedbackEnablingforSPSactive = 'enabled', the UE considers a HARQ process associated with a transport block in a first SPS PDSCH reception, after an activation of SPS PDSCH receptions, to have enabled HARQ-ACK information and the UE provides a HARQ-ACK information bit according to a decoding outcome for the transport block in the first SPS PDSCH reception. ……

Given the parameter of harq-feedbackEnablingforSPSactive with the following generic description from TS 38.331 can be configured to UE supporting MBS but not NTN as long as UE supports such function, defining an additional UE feature into the MBS UE feature list can be considered for a way forward. In addition, a similar sentence can be added into clause 18 regarding the description of the function of harq-FeedbackEnablerMulticast.

harq-FeedbackEnablingforSPSactive If enabled, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of if HARQ feedback is enabled or disabled corresponding to the first SPS PDSCH after activation. Otherwise, UE follows configuration of HARQ feedback enabled/disabled corresponding to the first SPS PDSCH after activation.

The corresponding draft CRs are provided in [3] and [4].

# Issue#3: PDCCH validation for SPS

Reply LS on SPS configuration for unicast and multicast [5] includes the responses as:

- Reply to Q1: Yes, from RAN1’s perspective, the similar restriction is required when configuring SPS for both unicast and multicast in one BWP, i.e., network can only use SPS-ConfigToAddModList-r16 to configure SPS PDSCH for unicast in this case.

- Reply to Q2: Yes, from RAN1’s perspective, when a single multicast SPS is configured for a group of UEs and there is no SPS configuration for unicast for at least one UE in the group, it’s up to network implementation to configure sps-ConfigIndex to zero or non-zero for the single multicast SPS.

With the above clarification between RAN1 and RAN2, CR is needed for TS38.213 regarding PDCCH validation for multicast SPS. Currently, HPN is used for activation or release PDCCH validation when UE is provided a single multicast SPS configuration, which is NOT correct now due to

- Case 1: UE1 is configured with one multicast SPS configuration but other UEs may be configured with more than one SPS configurations with one configuration shared with UE1.

- Case 2: when unicast DCI is also used to release the single multicast SPS configuration.

For the above two cases, HPN in the DCI is to be used to indicate which SPS configuration index is activated or released, so HPN with value of zero should be removed for PDCCH validation even when UE is configured with only one multicast SPS configuration.

Moreover, regarding joint release SPS, the following agreement was reached in the last meeting:

Agreement

Multicast DCI cannot be used for

- joint SPS unicast and SPS multicast release

- joint SPS unicast release

- FFS: joint SPS multicast release

Joint releasing multiple SPS configurations was not considered for multicast and is suggested not be supported at this late maintenance stage, with the concerns including there is only one parameter of sps-ConfigDeactivationStateList currently used for unicast SPS, setting the SPS configuration for unicast or multicast is up to network, and UE capability for such support, etc.

The corresponding CRs are provided in [6] and [7].

# Issue#4: SR+NACK-only+others

As agreed, on one aspect, for UE multiplexing NACK-only with other HARQ-ACK feedback, CSI or PUSCH, NACK-only is transformed into ACK/NACK; on another aspect, for SR overlapping with NACK-only, it is up to UE to transmit one and drop the other, so that multiplexing SR with NACK-only is not specified.

Agreement: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority, support UE multiplexing the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit. - This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately. - Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others. - FFS the case of PUCCH for SR. Agreement It is up to UE implementation on which one to drop between SR or NACK-only when NACK-only collides with SR when UE would transmit PUCCH with HARQ-ACK according to the second reporting mode.

In the last meeting, more cases involving NACK-only overlapping with SR and other uplink channels are raised as summarized in FLS [1]. The solution proposed is either dropping SR or multiplexing SR with NACK-only and other channels by adopting a corresponding CR.

It was argued that UE has positive SR and reports NACK when failed in decoding at the same time is low probability case so that RAN1 decided to not specify the rule for multiplexing. Following such logic, specifying multiplexing SR, NACK-only and the third channel(s) is also not necessary.

With the current specification descriptions for multiplexing NACK-only only with other HARQ-ACK feedback, CSI or PUSCH, the interpretation is that UE will implicitly drop SR if SR is also overlapped with the other channels wherein.

Overall, no CR is identified as necessary for the concerned cases at this stage.

Proposal 2: No CR is needed for resolving the overlapping of SR, NACK-only and other uplink channels.

# Conclusions

The remaining maintenance issues regarding Type-1 codebook generation, disabling HARQ-ACK for SPS multicast, PDCCH validation for SPS, etc. are discussed in the paper. The corresponding CRs are separately provided. In addition, the proposals are summarized as follows:

Proposal 1: No CR is needed for Type-1 CB construction for multiplexing HARQ-ACK for unicast and multicast.

Proposal 2: No CR is needed for resolving the overlapping of SR, NACK-only and other uplink channels.

# References

R1-2301919, FLS#3 on the HARQ-ACK related issues for Rel-17 NR MBS, Moderator (Huawei), RAN1#112, Athens, Greece, February 27- March 3, 2023.

R1-2301920, Draft CR on Type-1 HARQ-ACK codebook multiplexing for unicast and multicast, Moderator (Huawei), RAN1#112, Athens, Greece, February 27- March 3, 2023.

R1-2303793, Draft CR on disabling HARQ-ACK for multicast SPS, Huawei, HiSilicon, CBN, RAN1#112bis-e, E-meeting, 17-26 April, 2023.

R1-2303876, Draft CR on HARQ-ACK for SPS release, Huawei, HiSilicon, RAN1#112bis-e, E-Meeting, 17-26 April, 2023.

R1-2302209, Reply LS on SPS configuration for unicast and multicast, RAN1#112, Athens, Greece, February 27- March 3, 2023.

R1-2302343, Draft CR on PDCCH validation for multicast SPS, Huawei, HiSilicon, CBN, RAN1#112bis-e, E-Meeting, 17-26 April, 2023.

R1-2303878, Draft CR on joint release for multicast SPS, Huawei, HiSilicon, RAN1#112bis-e, E-Meeting, 17-26 April, 2023.