3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303802

e-Meeting, April 17-26, 2023

Agenda Item:	7.1

Source:	Huawei, HiSilicon

Title:	Discussion on the multiplexing between SR/SPS A/N and CSI

Document for:	Discussion and Decision

# Introduction

This contribution discusses the multiplexing between SR/SPS HARQ-ACK and CSI.

# Discussion

According to the discussion in last meeting [1], two overlapping cases (case 1 and case 2 below) are identified for further discussion.

Case 1: PUCCH with SPS HARQ-ACK overlapping with two non-overlapped PUCCH with CSI reports, and CSI report starting earlier is the reference PUCCH determined by the pseudo code in TS 38.213 Section 9.2.5

Case 2: PUCCH with SPS HARQ-ACK overlapping with two non-overlapped PUCCH with CSI reports, and SPS HARQ is the reference PUCCH determined by the pseudo code in TS 38.213 Section 9.2.5

In the following, we will discuss these cases considering whether multi-CSI-PUCCH-ResourceList is provided or not.

## UE is NOT provided multi-CSI-PUCCH-ResourceList

For case 1, the UE first performs the pseudo code to identify the overlapped group (Set Q). The overlapping PUCCH group (Set Q) includes PUCCH with CSI report 1 and PUCCH with SPS A/N. Then the UE would multiplex the SPS A/N in the PUCCH with CSI report 1. So, the PUCCH with CSI report 2 is also transmitted.

For case 2, the UE first performs the pseudo code to identify the overlapped group (Set Q). The overlapping PUCCH group (Set Q) includes PUCCH with SPS A/N, PUCCH with CSI report 1 and PUCCH with CSI report 2. Then the UE would multiplex the SPS HARQ-ACK in the CSI PUCCH with the higher priority and drop the CSI PUCCH with the lower priority (i.e. drop the PUCCH with CSI report 2). The corresponding spec is below.

### 9.2.5 UE procedure for reporting multiple UCI types …… If - a UE is not provided multi-CSI-PUCCH-ResourceList, and - a resource for a PUCCH transmission with HARQ-ACK information in response to SPS PDSCH reception and/or a resource for a PUCCH associated with a SR occasion overlap in time with two resources for respective PUCCH transmissions with two CSI reports, and - there is no resource for a PUCCH transmission with HARQ-ACK information in response to a DCI format detection that overlaps in time with any of the previous resources, and - the following pseudo code results to the UE attempting to determine a single PUCCH resource from the HARQ-ACK and/or the SR resource and the two PUCCH resources with CSI reports the UE - multiplexes the HARQ-ACK information and/or the SR in the resource for the PUCCH transmission with the CSI report having the higher priority, and - does not transmit the PUCCH with the CSI report having the lower priority

In summary, when the UE is NOT provided multi-CSI-PUCCH-ResourceList the UE behavior for case 1 and case 2 is clear and shown below.

Figure 1 UE behavior for case 1 and case 2 when the UE is NOT provided multi-CSI-PUCCH-ResourceList

## UE is provided multi-CSI-PUCCH-ResourceList

For case 1, the UE first performs the pseudo code to identify the overlapped group (Set Q). The overlapping PUCCH group (Set Q) includes PUCCH with CSI report 1 and PUCCH with SPS A/N. Then the UE would multiplex the SPS A/N in the PUCCH with CSI report 1. So, the PUCCH with CSI report 2 is also transmitted.

For case 2, the UE first performs the pseudo code to identify the overlapped group (Set Q). The overlapping PUCCH group (Set Q) includes PUCCH with SPS A/N, PUCCH with CSI report 1 and PUCCH with CSI report 2. Then the UE needs to resolve this overlapping in the group to determine a single PUCCH.  But it is not clear about the UE behavior in this case since the condition in the spec (marked below) for the multiplexing rule between SPS A/N and two CSI PUCCHs is not satisfied.

### 9.2.5 UE procedure for reporting multiple UCI types …… If - a UE is not provided multi-CSI-PUCCH-ResourceList, and - a resource for a PUCCH transmission with HARQ-ACK information in response to SPS PDSCH reception and/or a resource for a PUCCH associated with a SR occasion overlap in time with two resources for respective PUCCH transmissions with two CSI reports, and - there is no resource for a PUCCH transmission with HARQ-ACK information in response to a DCI format detection that overlaps in time with any of the previous resources, and - the following pseudo code results to the UE attempting to determine a single PUCCH resource from the HARQ-ACK and/or the SR resource and the two PUCCH resources with CSI reports the UE - multiplexes the HARQ-ACK information and/or the SR in the resource for the PUCCH transmission with the CSI report having the higher priority, and - does not transmit the PUCCH with the CSI report having the lower priority

In summary, when the UE is provided multi-CSI-PUCCH-ResourceList the UE behavior for case 1 is clear and shown below, but the UE behavior for case 2 is not clear.

Figure 2 UE behavior for case 1 and case 2 when the UE is provided multi-CSI-PUCCH-ResourceList

And we also found the following Rel-15 agreements below. It only defined the UE behavior for the overlapping between SPS A/N and two non-overlapping CSI PUCCH resources when a UE is NOT configured with multi-CSI-PUCCH report. The overlapping between SPS A/N and two non-overlapping CSI PUCCH resources when a UE is configured with multi-CSI-PUCCH report is missed.

RAN1#94 Agreements:

When a UE is not configured with multi-CSI-PUCCH report,

  - When SR occasion and/or DL SPS HARQ-ACK and two non-overlapping CSI PUCCH resources contained in Resource A and Set X (as defined in RAN1 #93 meeting) decided by the pseudo code (as in 9.2.5 of TS38.213) in a slot

    - DL SPS HARQ-ACK/SR is multiplexed on the CSI PUCCH resource with higher priority, and CSI on the CSI PUCCH resource with lower priority is dropped

      - In case of presence of DL SPS HARQ-ACK, multiplexing occurs if the corresponding timeline requirement is met, otherwise, it is considered as an error case

So, we propose the following TP to resolve this issue. That is, when a UE is provided multi-CSI-PUCCH-ResourceList but the CSI PUCCHs do not overlap in the slot, if the overlapped group includes SPS A/N and two non-overlapped CSI PUCCH resources, the UE still multiplex the SPS A/N and the higher priority CSI, the lower priority CSI is dropped. It is the same behavior as case 2 in figure 1.

Please note that the TP does not change the UE behavior for case 1 since the overlapped group for case 1 still only includes SPS A/N and CSI report 1. Then the UE would multiplex the SPS A/N in the PUCCH with CSI report 1. Thus, the PUCCH with CSI report 2 is also transmitted.

Proposal: To endorse following changes on Rel-16 spec of TS 38.213 Section 9.2.5

--------------------------------------------------Start of the TP--------------------------------------------------

If

-	a UE is not provided multi-CSI-PUCCH-ResourceList or if a UE is provided multi-CSI-PUCCH-ResourceList and PUCCH resources for transmissions of CSI reports do not overlap in the slot, and

-	a resource for a PUCCH transmission with HARQ-ACK information in response to SPS PDSCH reception and/or a resource for a PUCCH associated with a SR occasion overlap in time with two resources for respective PUCCH transmissions with two CSI reports, and

-	there is no resource for a PUCCH transmission with HARQ-ACK information in response to a DCI format detection that overlaps in time with any of the previous resources, and

-	the following pseudo code results to the UE attempting to determine a single PUCCH resource from the HARQ-ACK and/or the SR resource and the two PUCCH resources with CSI reports

the UE

-	multiplexes the HARQ-ACK information and/or the SR in the resource for the PUCCH transmission with the CSI report having the higher priority, and

-	does not transmit the PUCCH with the CSI report having the lower priority

--------------------------------------------------End of the TP--------------------------------------------------

# Conclusions

This contribution discusses the issue of CSI multiplexing, and has the following proposal.

Proposal: To endorse following changes on Rel-16 spec of TS 38.213 Section 9.2.5

--------------------------------------------------Start of the TP--------------------------------------------------

If

-	a UE is not provided multi-CSI-PUCCH-ResourceList or if a UE is provided multi-CSI-PUCCH-ResourceList and PUCCH resources for transmissions of CSI reports do not overlap in the slot, and

-	a resource for a PUCCH transmission with HARQ-ACK information in response to SPS PDSCH reception and/or a resource for a PUCCH associated with a SR occasion overlap in time with two resources for respective PUCCH transmissions with two CSI reports, and

-	there is no resource for a PUCCH transmission with HARQ-ACK information in response to a DCI format detection that overlaps in time with any of the previous resources, and

-	the following pseudo code results to the UE attempting to determine a single PUCCH resource from the HARQ-ACK and/or the SR resource and the two PUCCH resources with CSI reports

the UE

-	multiplexes the HARQ-ACK information and/or the SR in the resource for the PUCCH transmission with the CSI report having the higher priority, and

-	does not transmit the PUCCH with the CSI report having the lower priority

--------------------------------------------------End of the TP--------------------------------------------------

# Reference

- R1-2302139	Summary #2 of discussion on the multiplexing between SR/SPS A/N and CSI	Moderator (HiSilicon)