3GPP TSG RAN WG1 #112bis-e			R1-2303805

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.1.1.1

Source: 	Hyundai Motor Company

Title:	Discussion on unified TCI framework extension for M-TRP operation

Document for:	Discussion and Decision

Introduction

This contribution shows our views for some issues such as PDSCH reception in case of S-DCI based mTRP, beam application time and power control on unified TCI framework extension for MTRP.

Discussion

PDSCH reception in case of S-DCI based mTRP

Regarding PDSCH reception for S-DCI based MTRP, followings are agreed in the previous meetings [1][2].

Agreement On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - FFS: Detail design of the new indicator field Agreement On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 - FFS: Detail of the capability of two default beams for S-DCI based MTRP - FFS: The threshold value

In rel-17 STRP case, the offset between the reception of the DL DCI and the corresponding PDSCH was discussed and it was discussed with the time threshold (e.g. timeDurationForQCL) together. Considering the motivation of the time threshold, we think following the existing rule seems quite reasonable. However, since new indicator field such as TCI selection filed was introduced in Rel-18, we think that RAN1 needs to study some enhancement to consider it. Furthermore, the repeated transmission of PDSCH such as slot aggregation transmission also needs to be considered to improve performance. Figure #1 shows the example of repeated PDSCH occasion in different time domain. According to the existing rule in Rel-17, UE set its Rx beam to default beam for the first PDSCH occasion since the timing offset between first PDSCH occasion and PDCCH is less than the time threshold (e.g. timeDurationForQCL). But for second PDSCH occasion, in that case, UE can choice either following Rx beam for first PDSCH or following indicated TCI state through TCI selection field. That is, when multiple PDSCHs are repeated in multiple slots, some ambiguity problems can be occurred. To prevent it, we think that RAN1 should consider not only time threshold (e.g. timeDurationForQCL) but also repeated transmission of PDSCH such as slot aggregation transmission.

Figure 1: The paradigm of Rx beam for repeated PDSCHs in accordance with time threshold.

Observation #1:

- When repeated PDSCH occasions in different time domain and some of them are located after time threshold, some ambiguity problems for beam selection can be occurred.

Proposal #1:

- Regarding unified TCI framework for mTRP, RAN1 should consider not only time threshold (e.g. timeDurationForQCL) but also repeated transmission of PDSCH such as slot aggregation transmission.

Beam application time

In Rel-17, beam application was introduced and it is used to indicate first slot that indicated TCI state would be applied when the indicated TCI State is different from the previously indicated one. Regarding the beam application time, we think that some clarification and further enhancements would seem to be required. For the case when scheduled DL and/or UL channels exist during the beam application time, the details of behaviours have not been discussed yet. Intuitively, there can be two ways that UE can do. The one is that UE transmits scheduled physical channel applying previous indicated TCI state and the other is that UE does not to transmit it during the beam application time. For the former case, the performance of scheduled channels might be deteriorated. For the latter case, there would be time delay for transmission of scheduled channels. In addition, there can be overlapped time duration between scheduled DL and/or UL channels and beam application time. Figure 2 demonstrates it where exist TCI state is used for TRP #1 whereas beam application time is required for TRP #2. For the case of TRP#2, we think that some rules such as timing offset considering beam application time seems to be required.

Figure 2: The paradigm of beam application time for indicated TCI state among mTRP.

From this perspective, we think that RAN1 firstly should discuss about how to deal with the scheduled DL and/or UL channels during the beam application time.

Observation #2:

- The details of behaviours for the case when scheduled DL and/or UL channels exist during the beam application time have not been discussed yet.

Proposal #2:

- RAN1 firstly should discuss about how to deal with the scheduled DL and/or UL channels during the beam application time.

UL power control for MTRP with STxMP

Regarding STxMP, some issues that are related with power control were discussed and then sending LS for RAN4 was agreed as shown below:

Agreement On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 - Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 is endorsed in R1-2205639.

In case of STxMP transmission, UE can transmit different UL channels simultaneously through different Tx panel(s) and then related power control parameters (e.g. alpha, p0, closedloop index) also can be provided in accordance with TCI state. In such a case, power should be allocated for each panel and then the sum of allocated power should be no greater than a Pcmax. To consider the issue, we currently consider power limitation in terms of UE or panel as shown in above agreement. Here, we think that there would be still unresolved issues even though discussion on power limitation was already done. In other words, even though RAN1 defines per-panel power limitation, the required power for transmission for each UL channel can be larger than per-panel power limitation and then total required power could be larger than the Pcmax when both of link qualities are bad as shown in the figure #4. Hence, since transmission power is directly calculated depending on which channel is transmitted, RAN1 should consider which channel is mapped into the panel in addition to consideration on the panel.

Figure 3: The paradigm of UL power control for MTRP with STxMP.

Observation #3:

- Even though RAN1 define per-panel power limitation, the required power for transmission for each UL channel can be larger than per-panel power limitation and then total required power could be larger than the Pcmax when both of link qualities are bad.

For this reason, some rules that could be commonly applied for each panels/channels should be considered. The one of the many examples would be prioritization rule between panels/channels, such as either cell operation with two uplink carriers or operation with carrier aggregation. Hence, the power prioritization rule between panels/channels should be further studied.

Proposal #3:

- For MTRP scheme(s) with STxMP, the power prioritization rule between panels/channels should be further studied.

Conclusion

In this contribution, the following conclusions were made:

PDSCH reception in case of S-DCI based mTRP

Observation #1:

- When repeated PDSCH occasions in different time domain and some of them are located after time threshold, some ambiguity problems for beam selection can be occurred.

Proposal #1:

- Regarding unified TCI framework for mTRP, RAN1 should consider not only time threshold (e.g. timeDurationForQCL) but also repeated transmission of PDSCH such as slot aggregation transmission.

Beam application time

Observation #2:

- The details of behaviours for the case when scheduled DL and/or UL channels exist during the beam application time have not been discussed yet.

Proposal #2:

- RAN1 firstly should discuss about how to deal with the scheduled DL and/or UL channels during the beam application time.

UL power control for MTRP with STxMP

Observation #3:

- Even though RAN1 define per-panel power limitation, the required power for transmission for each UL channel can be larger than per-panel power limitation and then total required power could be larger than the Pcmax when both of link qualities are bad.

Proposal #3:

- For MTRP scheme(s) with STxMP, the power prioritization rule between panels/channels should be further studied.

Reference

[1] Chairman Note for RAN1#111

[1] Chairman Note for RAN1#112