3GPP TSG RAN WG1 Meeting #112-bis-e													       R1-2303813

e-Meeting, April 17th – April 26th, 2023

Agenda item:		9.7.1

Title:						Spatial Domain Adaptation for NES

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

The WI for Network Energy Saving (NES) was approved in RAN#98-e [1]. One of its objectives is to specify solutions for spatial and power domain adaptations, which has been approved for 3GPP Rel-18:

- Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

Based on the agreements made during RAN1#112 [2], this contribution will focus on certain aspects identified for further study and specification needed for spatial domain adaptation for NES.

# Adaptation of Spatial Elements

In essence, for NES gains, the recommended specification by the WID (i.e., via necessary enhancements on CSI and beam management related procedures including measurement and report, and signalling) to enable adaptation of active spatial elements should create opportunities for turning off associated hardware components of the inactivated transceiver chains and/or antenna panels in the physical antenna array. In this regard, the number of available spatial elements in the radio unit (RU) of a gNB performing MIMO transmission (or reception) can be viewed as being proportional to the number of (virtual) logical antenna ports and the number of physical antenna elements. Depicted in Figure 1 is the relationship between the number of layers, logical antenna ports and physical antenna elements in a gNB’s RU performing MIMO transmission or reception.

Figure 1: Relationship between the number of layers, logical antenna ports and physical antenna elements in the radio unit of a gNB performing MIMO transmission or reception.

In order to leverage low or medium load scenarios, adaptation of the number of active spatial elements associated with the ports or the physical antenna elements can be addressed either through enhanced CSI-related procedures and/or (when applicable) enhanced beam management procedures.

Considering the first agreement from RAN1#112 [2] (copied below), for the Type 1 spatial adaptations, CSI acquisition-related enhancements are applicable while for the Type 2 adaptations, beam management-related enhancements are applicable, as will be discussed next.

Agreement For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases - Type 1: all antenna elements associated to a logical antenna port is disabled/enabled - Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

- Observation 1: CSI acquisition-related enhancements are applicable to the Type 1 spatial domain adaptations while beam management-related enhancements are applicable to the Type 2 adaptations.

## CSI Acquisition-Related Enhancements for Type 1 Adaptation

When the number of available logical ports Np is relatively high and the number of physical antenna elements Na ≥ Np is comparable to the number of ports, i.e., Na ≈ Np, like in FR1 systems, low or medium load scenarios needing to communicate via fewer MIMO layers might create opportunities for dynamically activating/deactivating only a subset of the available antenna ports. Note that this would be Type 1 adaptation with all antenna elements associated to the inactivated logical ports being disabled.  A sustainable and efficient downscaling of the MIMO precoding—and in turn the number of active ports to minimize energy consumption—with minimal degradation in user performance would require enhanced CSI acquisition procedures.

As indicated, legacy MIMO systems can implement such adaptation only via slow manoeuvres such as RRC reconfigurations of CSI-RS resource(s) and/or obtaining CSI feedback corresponding to CSI-RS resource(s) associated with different numbers/sets of ports at the network node over a period of time and/or several CSI feedback occasions. Such procedures, however, would increase latency and control overhead, and might allow only slower adaptation compared to possibly rapidly changing network conditions in certain scenarios.

For a more dynamic and low-latency adaptation of antenna ports at the gNB, switching between different subsets of antenna ports in the array or falling back to a smaller subset of the array, the network can be enabled to request CSI feedback with regard to a specific subset or a configured number of subsets of ports associated with the CSI-RS resource used by UEs to compute the CSI feedback.

- Observation 2: Enabling the network to request CSI feedback with regard to a specific subset or a configured number of subsets of ports associated with the CSI-RS resource(s) used by UEs to compute the CSI feedback will be beneficial for minimizing network energy consumption through spatial element adaptation.

In order to address the trade-off between NES requirements and other performance metrics (like network and UE throughput) more effectively, the UEs can be enabled to address such trade-off instead of providing a comprehensive CSI feedback pertaining to several or all subsets of ports associated with CSI-RS resource(s) and leaving the decision to the network. Allowing the UE to assess the NES vs performance trade-off, the CSI feedback overhead can be considerably reduced to either one or a very limited number of subsets of ports.

- Observation 3: By enabling UEs to address trade-off between NES requirements and other performance metrics more effectively, the UEs can be allowed to choose a specific subset or a limited number of subsets of ports associated with the CSI-RS resource(s) to perform CSI feedback.

- Proposal 1: RAN1 to discuss mechanisms to enable UEs to perform CSI feedback efficiently and sustainably, based on trade-off between NES (through reducing special elements at gNB) and other performance metrics including feedback overhead.

Considering the following agreement from RAN1#112 [2], further study on the CSI Resource/Resource Set/Resource Setting being associated with the spatial adaptation patterns is needed.

Agreement For spatial element adaptation, further study the following - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern - FFS: Details on how the association is done - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns - FFS: Details on how the association is done - FFS: Details on the definition of “spatial adaptation patterns”

Firstly, as for the definition of “spatial adaptation patterns”, we propose to define each spatial domain pattern as a configured subset of the available antenna ports in an array of antenna ports at the gNB.

- Proposal 2: For definition of “spatial adaptation patterns”, define a spatial adaptation pattern as a configured subset of all available ports in an array of antenna ports at the gNB.

In order to adaptively switch between the different spatial adaptation patterns, e.g., to fall back to a smaller subset of ports from the antenna port array at a gNB, the network needs to be enabled to request CSI feedback with regard to one or more spatial domain patterns or the subsets of antenna ports. For the association of the CSI-RS resource(s) to these spatial domain patterns, there can be the two possibilities as noted in the above agreement:

- In line with A1-1, one way is to have the different configured spatial adaptation patterns (i.e., each configured subset of the available antenna ports) associated with individual CSI-RS resources, for example, as depicted in Figure 2. Then, the UE can provide feedback(s) or report(s) with respect to one or more of the CSI-RS resources so as to aid the network in selecting a specific spatial adaptation pattern, where the reporting can be similar to a beam report or the UE itself may choose a desired subset by reporting one CSI-RS Resource Indicator (CRI).

- In line with A1-2, another way is to have the different spatial adaptation patterns (i.e., each configured subset of the available antenna ports) associated with a single CSI-RS resource, for example, as depicted in Figure 3. Then, different subsets of the CSI-RS resource can be associated to each specific spatial adaptation pattern, i.e., a subset of antenna ports that the network may scale down to, from the CSI-RS resource’s original set of antenna ports. By configuring/indicating ‘valid’ subsets within a CSI-RS resource, the network may set the spatial adaptation patterns that make up the sub-arrays that it would like to get CSI feedback from the UE. Thus, the UE needs to receive configuration of a CSI-RS resource and the subsets, based on which the UE can provide feedback on the all or some subsets.

Figure 2: An example showing the association of antenna array ports to CSI-RS resources as per A1-1, where each spatial adaptation pattern or subset of antenna ports is associated with different CSI-RS resources.

Figure 3: An example showing the association of antenna array ports to a CSI-RS resource as per A1-2, where the CSI-RS subsets configuration correspond to different spatial adaptation patterns or subsets of the available ports.

Observation 4: With A1-1, the UE can provide feedback(s) or report(s) with respect to one or more CSI-RS resources so as to aid the network in selecting a specific spatial adaptation pattern, where the reporting can be similar to a beam report or the UE itself may choose a desired subset by reporting one CRI.

Observation 5: With A1-2, a CSI-RS resource may correspond to the antenna ports associated with the whole antenna array while the subsets of the CSI-RS resource may be associated with distinct (overlapping or non-overlapping) subarrays of the antenna array.

Observation 6: With A1-2, the UE needs to receive along a with a CSI-RS resource setting the subsets corresponding to the associated spatial adaptation patterns.

Proposal 3: Regarding A1-1, for details on the association, multiple spatial adaptation patterns can be associated with individual CSI-RS resource so that UEs can be enabled to provide report(s) with respect to one or more of the CSI-RS resources.

Proposal 4: Regarding A1-2, for details on the association, multiple spatial adaptation patterns can be associated with a single CSI-RS resource while different subsets of the CSI-RS resource can be configured corresponding to each spatial adaptation pattern so that UEs can be enabled to provide report(s) with respect to one or more of the configured subsets of the CSI-RS resource.

Considering the following agreements from RAN1#112 [2], further study is needed on how the CSI report configuration for each spatial adaptation pattern shall be specified and how multiple CSIs are reported, where each CSI corresponds to a spatial adaptation pattern.

Agreement For spatial element adaptation, further study the following - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern - FFS: Details of sub-configuration

Agreement For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g. - FFS: gNB indicates to UE which CSI(s) the UE shall report - FFS: the UE selects which CSI(s) are reported - FFS: multiple CSI(s) are reported in a joint CSI report - FFS: Overhead reduction for multiple CSI(s) Note: UE complexity needs to be taken into account.

In order to adaptively switch between multiple spatial adaptation patterns at the gNB, taking the tradeoff between NES and user performance into account, multiple CSI reports from each UE may be needed. In such case, the above agreement on studying the configuration of multiple CSI reports from each UE would be important. Alternatively, such a need for configuring and feeding back multiple CSIs from each UE can be avoided if the UE is enabled to effectively select one from the multiple CSIs it measures corresponding to the multiple spatial adaptation patterns at the gNB.

Observation 7: The need for configuring and feeding back multiple CSIs from each UE can be avoided if the UE is enabled to effectively select one from the multiple CSIs it measures, corresponding to the multiple spatial adaptation patterns at the gNB.

Considering that the necessary CSI-RS resource/resource set/resource setting needed for a UE to measure the CSIs corresponding to the multiple spatial adaptation patterns at the gNB are given, the UE can be configured to report all, some or one of the measured CSIs. Specifically, with a UE set to measure CSI corresponding to P≥ 1 different (subsets of the available antenna ports in the array) spatial domain patterns at the gNB, it should be configured to report for K≤P CSI(s), where the value of K can be either configured by the network or can be fixed in the specifications.

Observation 8:  Considering that the necessary CSI-RS resource/ resource set/resource setting needed for a UE to measure the CSIs corresponding to the multiple spatial adaptation patterns at the gNB are given, the UE can be configured to report all, some or one of the measured CSIs.

Proposal 5: With a UE set to measure CSI corresponding to P≥ 1 different (subsets of the available antenna ports in the array) spatial domain patterns at the gNB, it should be configured to report for K≤P CSI(s), where the value of K can be either configured by the network or can be fixed in the specifications.

Once the network acquires CSI feedback with respect to the multiple spatial domain patterns, the decision on which subarray to use for transmissions may rest on the network. Different possibilities for the UEs reporting CSI with respect to the multiple subsets shall be addressed as noted for further study in the above agreement. If the network collects a detailed CSI report with respect to each subset, it would result in very high uplink control information overhead. This warrants overhead reduction in the reporting of the multiple CSIs or limiting the number of CSIs K≤P to be reported or a combination of both, however, effectively addressing the tradeoff between NES and user performance.

Regarding the overhead reduction with multiple CSIs being reported, a two-step approach may effectively reduce the details in the reported multiple CSIs as follows:

- The UE first reports a ‘representative information’ with respect to each of the K subsets, which can be a performance metric or a measurement made with respect to each subset

- Based on the representative information reported by the UE, the network then requests detailed CSI for one of the K subsets, which may be selected by the network for an expected tradeoff between UEs’ performance and NES.

Observation 9:  For overhead reduction with multiple CSIs being reported, a two-step approach may effectively reduce the details in the reported multiple CSIs as follows:

- The UE first reports a ‘representative information’ with respect to each of the K subsets, which can be a performance metric or a measurement made with respect to each subset

- Based on the representative information reported by the UE, the network then requests detailed CSI for one of the K subsets, which may be selected by the network for an expected tradeoff between UEs’ performance and NES.

Note that in the above two-step approach in CSI reporting, the first step reduces the overhead of multiple reports while the second step limits the number of detailed CSI report to one.

Without such a two-step approach in CSI reporting, another way is to enable the UE to select which CSI(s) are reported in order to effectively limit the number K, ideally to one. In order to enable the UE to make such selection effectively, the UE should be configured with sufficient information from the gNB so as to take certain NES requirements into account, in addition to the UE performance. For example, to let the UE select K=1 CSI to be reported out of the P measured ones, instead of selecting the CSI corresponding to the spatial adaptation pattern (subset of ports) that gives the best UE performance (which may not be the good for NES), the UE can be configured to select the worst among the ones that satisfy a certain threshold in UE performance, where such threshold can be configured by the network.

Proposal 6:  Regarding the option for letting the UE select which CSI(s) are reported, in order to enable the UE to make such selection effectively, the UE should be configured with sufficient information from the gNB so as to take certain NES requirements into account, in addition to the UE performance.

## Beam Management-Related Enhancements for Type-2 Adaptation

When the number of MIMO layers Ns and in turn the number of logical antenna ports Np≥Ns are relatively low and the number of available physical antenna elements Na ≥ Np is much higher than the number of logical ports, like in FR2, low or medium load scenarios needing to communicate with fewer users might create opportunities for activating only a subset of the available antenna elements. Note that in such scenarios, the Type 2 adaptation makes more sense where only part/subset of antenna elements associated to a logical antenna port is disabled/enabled. This means implementing a beam forming targeted at minimizing energy consumption, through enhanced beam management procedures, with a minimum expected degradation in user performance.

Note that beam management in downlink involves the possibility for gNB beam sweeping via CSI-RS transmissions by fixing already selected UE beam(s), to perform UE measurements followed by reporting of one or more of the gNB beam(s) by the UE to the gNB. This possibility can be leveraged and enhanced for optimizing the gNB beam(s) for NES, for example, by changing the properties of the gNB beams that are swept via CSI-RS transmissions via employing different reduced subsets of physical antenna elements from the available array. Then, UEs can be enabled to measure and report beams considering NES constraints and sufficient link gain. Thus, in beam management-related enhancements for NES in FR2, UEs can be enabled to do beam reporting based on CSI-RS measurements considering NES constraints while maintaining sufficient link gains.

- Observation 10: In beam management-related enhancements for NES in FR2, UEs can be enabled to do beam reporting based on CSI-RS measurements considering NES constraints while maintaining sufficient link gains.

- Proposal 7: RAN1 to discuss mechanisms to enable UEs to perform beam measurement and efficient reporting to meet NES requirements while maintaining sufficient link gains.

# Conclusions

In this contribution, we discussed some aspects of Spatial Domain Adaptations for Network Energy Savings. The following observations and proposals have been made:

- Observation 1: CSI acquisition-related enhancements are applicable to the Type 1 spatial domain adaptations while beam management-related enhancements are applicable to the Type 2 adaptations.

- Observation 2: Enabling the network to request CSI feedback with regard to a specific subset or a configured number of subsets of ports associated with the CSI-RS resource(s) used by UEs to compute the CSI feedback will be beneficial for minimizing network energy consumption through spatial element adaptation.

- Observation 3: By enabling UEs to address trade-off between NES requirements and other performance metrics more effectively, the UEs can be allowed to choose a specific subset or a limited number of subsets of ports associated with the CSI-RS resource(s) to perform CSI feedback.

Observation 4: With A1-1, the UE can provide feedback(s) or report(s) with respect to one or more CSI-RS resources so as to aid the network in selecting a specific spatial adaptation pattern, where the reporting can be similar to a beam report or the UE itself may choose a desired subset by reporting one CRI.

Observation 5: With A1-2, a CSI-RS resource may correspond to the antenna ports associated with the whole antenna array while the subsets of the CSI-RS resource may be associated with distinct (overlapping or non-overlapping) subarrays of the antenna array.

Observation 6: With A1-2, the UE needs to receive along a with a CSI-RS resource setting the subsets corresponding to the associated spatial adaptation patterns.

Observation 7: The need for configuring and feeding back multiple CSIs from each UE can be avoided if the UE is enabled to effectively select one from the multiple CSIs it measures, corresponding to the multiple spatial adaptation patterns at the gNB.

Observation 8:  Considering that the necessary CSI-RS resource/resource set/resource setting needed for a UE to measure the CSIs corresponding to the multiple spatial adaptation patterns at the gNB are given, the UE can be configured to report all, some or one of the measured CSIs.

Observation 9:  For overhead reduction with multiple CSIs being reported, a two-step approach may effectively reduce the details in the reported multiple CSIs as follows:

- The UE first reports a ‘representative information’ with respect to each of the K subsets, which can be a performance metric or a measurement made with respect to each subset

- Based on the representative information reported by the UE, the network then requests detailed CSI for one of the K subsets, which may be selected by the network for an expected tradeoff between UEs’ performance and NES.

- Observation 10: In beam management-related enhancements for NES in FR2, UEs can be enabled to do beam reporting based on CSI-RS measurements considering NES constraints while maintaining sufficient link gains.

- Proposal 1: RAN1 to discuss mechanisms to enable UEs to perform CSI feedback efficiently and sustainably, based on trade-off between NES (through reducing special elements at gNB) and other performance metrics including feedback overhead.

- Proposal 2: For definition of “spatial adaptation patterns”, define a spatial adaptation pattern as a configured subset of all available ports in an array of antenna ports at the gNB.

Proposal 3: Regarding A1-1, for details on the association, multiple spatial adaptation patterns can be associated with individual CSI-RS resource so that UEs can be enabled to provide report(s) with respect to one or more of the CSI-RS resources.

Proposal 4: Regarding A1-2, for details on the association, multiple spatial adaptation patterns can be associated with a single CSI-RS resource while different subsets of the CSI-RS resource can be configured corresponding to each spatial adaptation pattern so that UEs can be enabled to provide report(s) with respect to one or more of the configured subsets of the CSI-RS resource.

Proposal 5: With a UE set to measure CSI corresponding to P≥ 1 different (subsets of the available antenna ports in the array) spatial domain patterns at the gNB, it should be configured to report for K≤P CSI(s), where the value of K can be either configured by the network or can be fixed in the specifications.

Proposal 6:  Regarding the option for letting the UE select which CSI(s) are reported, in order to enable the UE to make such selection effectively, the UE should be configured with sufficient information from the gNB so as to take certain NES requirements into account, in addition to the UE performance.

- Proposal 7: RAN1 to discuss mechanisms to enable UEs to perform beam measurement and efficient reporting to meet NES requirements while maintaining sufficient link gains.

# References

- RP-223540, “New WID: Network energy savings for NR”, Huawei, 3GPP RAN#98-e, December 2022.

- 3GPP Chairman’s Notes, 3GPP  RAN1#112, Athens, Feb-Mar 2022

- TR 38.864, “Study on network energy savings for NR”, V1.0.0, December 2022.