3GPP TSG RAN WG1 #112bis-e	R1-2303816

# e-Meeting, April 17th – April 26th, 2023

Agenda Item:	7.2

Source: 	ASUSTeK

Title:  	Discussion on TBoMS regarding multi-PUSCH

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss joint consideration of multi-PUSCH and TBoMS.

- Discussion

According to TS 38.331, TDRA table for multi-PUSCH does not include numberOfRepetitions-r16 nor numberOfRepetitionsExt-r17.

PUSCH-Allocation-r16 ::=  SEQUENCE {

mappingType-r16                           ENUMERATED {typeA, typeB}                     OPTIONAL,   -- Cond NotFormat01-02-Or-TypeA

startSymbolAndLength-r16                  INTEGER (0..127)                              OPTIONAL,   -- Cond NotFormat01-02-Or-TypeA

startSymbol-r16                           INTEGER (0..13)                               OPTIONAL,   -- Cond RepTypeB

length-r16                                INTEGER (1..14)                               OPTIONAL,   -- Cond RepTypeB

numberOfRepetitions-r16                   ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16} OPTIONAL,   -- Cond Format01-02

...,

[[

numberOfRepetitionsExt-r17                ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16, n20, n24, n28, n32, spare4, spare3, spare2,

spare1}                           OPTIONAL,   -- Cond Format01-02-For-TypeA

numberOfSlotsTBoMS-r17                   ENUMERATED {n1, n2, n4, n8, spare4, spare3, spare2, spare1}   OPTIONAL,   -- Need R

extendedK2-r17                            INTEGER (0..128)                              OPTIONAL    -- Cond MultiPUSCH

]]

}

| Conditional Presence | Explanation |
| --- | --- |
| Format01-02 | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is absent. In pusch-TimeDomainAllocationListDCI-0-1 and in pusch-TimeDomainAllocationListDCI-0-2, the field is mandatory present. |
| Format01-02-For-TypeA | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is absent. In pusch-TimeDomainAllocationListDCI-0-1, the field is optionally present if pusch-RepTypeIndicatorDCI-0-1 is not set to pusch-RepTypeB, Need R. It is absent otherwise, Need R. In pusch-TimeDomainAllocationListDCI-0-2, the field is optionally present if pusch-RepTypeIndicatorDCI-0-2 is not set to pusch-RepTypeB, Need R. It is absent otherwise, Need R. |

However, there is another parameter numberOfSlotsTBoMS-r17 introduced by coverage enhancement agenda in each PUSCH-Allocation-r16. Insofar, there is no clear agreement or conclusion specifying whether numberOfSlotsTBoMS-r17 and multi-PUSCH could be configured simultaneously. From our understanding, numberOfSlotsTBoMS-r17 is configured to make UE to transmit one TB over multiple slots to have very low data rate which functionality is similar to numberOfRepetitions. So, similar restriction that numberOfSlotsTBoMS-r17 is absent in pusch-TimeDomainAllocationListForMultiPUSCH could be applied.

Observation 1: There is no clear agreement and conclusion specifying whether numberOfSlotsTBoMS-r17 and pusch-TimeDomainAllocationListForMultiPUSCH could be configured simultaneously.

With introducing of non-contiguous multi-PUSCHs in Rel-17, it seems feasible to configure both two features once gNB can make sure n-th allocation with numberOfSlotsTBoMS-r17 and (n+1)-th allocation does not overlap in time domain. The prerequisite is to avoid ambiguity of overlapping slot is belonging to n-th allocation or (n+1)-th allocation. Fig. 1 is an example for UE to be configured with both numberOfSlotsTBoMS-r17 and multi-PUSCHs.

Fig. 1 (Illustration of feasibility of configuring both numberOfSlotsTBoMS and pusch-TimeDomainAllocationListForMultiPUSCH)

Observation 2: It’s feasible to configure numberOfSlotsTBoMS in pusch-TimeDomainAllocationListForMultiPUSCH.

It’s worthwhile to achieve RAN1 common understanding whether/how to configure numberOfSlotsTBoMS-r17 and multi-PUSCHs simultaneously.

Proposal: For TBoMS and multi-PUSCHs, down-select one alternative:

- Alt1: a UE does not expect to be configured with both numberOfSlotsTBoMS and pusch-TimeDomainAllocationListForMultiPUSCH.

- Alt2: a UE could be configured with both numberOfSlotsTBoMS and pusch-TimeDomainAllocationListForMultiPUSCH when gNB can make sure that for all n, n-th allocation with numberOfSlotsTBoMS and (n+1)-th allocation does not overlap in time domain.

- Conclusion

In this contribution, we have following proposals:

Observation 1: There is no clear agreement and conclusion specifying whether numberOfSlotsTBoMS-r17 and pusch-TimeDomainAllocationListForMultiPUSCH could be configured simultaneously.

Observation 2: It’s feasible to configure numberOfSlotsTBoMS in pusch-TimeDomainAllocationListForMultiPUSCH.

Proposal: For TBoMS and multi-PUSCHs, down-select one alternative:

- Alt1: a UE does not expect to be configured with both numberOfSlotsTBoMS and pusch-TimeDomainAllocationListForMultiPUSCH.

- Alt2: a UE could be configured with both numberOfSlotsTBoMS and pusch-TimeDomainAllocationListForMultiPUSCH when gNB can make sure that for all n, n-th allocation with numberOfSlotsTBoMS and (n+1)-th allocation does not overlap in time domain.

- Reference