3GPP TSG RAN WG1 #112bis-e		                       R1-2303818

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.4.1

Source: 	ASUSTeK

Title:  	Discussion on UCI multiplexing regarding STxMP

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss about UCI multiplexing considering STxMP.

- Discussion

UCI multiplexing

110b-e Agreement The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements). - Note: No symbol-level power adjustment within a PUSCH transmission occassion in the case of fully/partially overlapping in time domain Agreement Multi-DCI based STxMP PUSCH+PUSCH transmission at least supports the following PUSCH combinations: - DG-PUSCH + DG-PUSCH - CG-PUSCH + DG-PUSCH 111 Agreement Support the SFN scheme for single-DCI based STxMP PUCCH transmission Conclusion There is no consensus on the support STxMP PUCCH+PUCCH in multi-DCI based mTRP system 112 Agreement For multi-DCI based STxMP PUSCH+PUSCH, study enhancements of the UCI multiplexing rule to address the case that one PUCCH overlaps with two overlapped PUSCHs of STxMP PUSCH+PUSCH:

For mDCI based mTRP system, RAN1 has agreed that not supporting STxMP PUCCH+PUCCH. In this sense, for mDCI based mTRP system, gNB needs to schedule two PUCCHs in different symbols within a slot or in different slot to ensure there is no STxMP PUCCH+PUCCH. Regarding UCI multiplexing rule in mDCI based mTRP system, since property of non-ideal backhaul between different TRP is assumed for mDCI based mTRP system, it’s not feasible to directly apply current UCI multiplexing rule without enhancements. In one example, in following Fig. 1, if based on current standard’s UCI multiplexing rule, UCI to TRP1 and UCI to TRP2 are multiplexed into PUSCH to TRP2 assuming timeline requirement is satisfied. However, TRP2 cannot ideally expect there is UCI to TRP1 being multiplexed into this PUSCH transmission and thus TRP2 may be hard to ensure payload size of UCI. Thus, given this situation, we think per TRP’s UCI multiplexing rule can be considered in Rel-18 STxMP especially for mDCI based mTRP system. Regarding WID’s artificial restriction of NOT supporting simultaneous PUCCH and PUSCH transmission, we think dropping rule could be applied for having either PUSCH or PUCCH transmission in one timing.

Fig. 1

Proposal 1: For UCI multiplexing in mDCI mTRP system, UCI multiplexing rule in current standard is applied per TRP.

For sDCI based mTRP system, RAN1 has agreed that SFN scheme for STxMP PUCCH and supporting SDM and SFN scheme for STxMP PUSCH. Since ideal backhaul between different TRP for sDCI based mTRP is assumed, we think per TRP’s UCI multiplexing is not needed. However, due to the fact that there is two PUSCHs for STxMP, RAN1 needs to study which PUSCH is used to multiplex UCI. For example, Fig. 2 considers STxMP PUSCH and STxMP PUCCH and Fig. 3 considers STxMP PUSCH and sTRP PUCCH. It’s worthwhile to determine which PUSCH is used to multiplex UCI for align understanding between gNB and UE.

Fig. 2                                              Fig. 3

Observation: For UCI multiplexing in sDCI mTRP system, there is no need to introduce UCI multiplexing rule per TRP.

Proposal 2: For UCI multiplexing in sDCI mTRP system, when there are two PUSCHs for STxMP which could be candidate PUSCH for UCI multiplexing, RAN1 study which PUSCH is used to multiplex UCI.

- Conclusion

In this contribution, we have following proposals:

Proposal 1: For UCI multiplexing in mDCI mTRP system, UCI multiplexing rule in current standard is applied per TRP.

Observation: For UCI multiplexing in sDCI mTRP system, there is no need to introduce UCI multiplexing rule per TRP.

Proposal 2: For UCI multiplexing in sDCI mTRP system, when there are two PUSCHs for STxMP which could be candidate PUSCH for UCI multiplexing, RAN1 study which PUSCH is used to multiplex UCI.

- Reference