3GPP TSG-RAN WG1 Meeting # 112bis-e 	R1-2303824

# e-Meeting, April 17th – April 26th , 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on IUC parameter for determing preferred resource set in TS38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL-enh-Core |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In subclause 8.1.4A, current specification specifies that, when detemingin a preferred resource set, the higher layer parameter sl-Condition1-A-2 is not set to 'Disabled', which gives the impression that the higher layer parameter sl-Condition1-A-2 is provided and is set to other value than 'Disabled’. However, according to parameter structure of sl-Condition1-A-2 specified in TS38.331 as below, the higher layer parameter sl-Condition1-A-2 can only be set to 'Disabled’. Therefore, the description for higher layer parameter sl-Condition1-A-2 in TS38.214 is not aligned with TS38.331. TS 38.331 SL-InterUE-CoordinationScheme1-r17 ::= SEQUENCE { sl-IUC-Explicit-r17 ENUMERATED {enabled, disabled} OPTIONAL, -- Need M sl-IUC-Condition-r17 ENUMERATED {enabled, disabled} OPTIONAL, -- Need M sl-Condition1-A-2-r17 ENUMERATED {disabled} OPTIONAL, -- Need M sl-Condition1-A-2 Indicates disabling the use of condition of excluding from preferred resource set resource(s) in slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation.. |
| Summary of change: |  | Change ‘set to 'Disabled'’ to ‘provided’ in clause 8.1.4A to accord with description of sl-Condition1-A-2 in TS38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent description for higher layer parameter sl-Condition1-A-2 between TS38.214 and TS38.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.1.4A	UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission

When this procedure is triggered, the following parameters are provided by the higher layer:

-	the resource pool from which the preferred or non-preferred resources are to be determined;

-	the resource selection window [n+T1,n+T2] within which the preferred or non-preferred resources are to be determined;

-	the resource set type (either preferred or non-preferred resource set);

-	if the resource set type indicates preferred set, then the higher layer additionally provides the following parameters:

-	L1 priority, prioTX;

-	the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH;

-	the resource reservation period, Prsvp_TX, if present.

The value of Cresel is determined by the UE according to clause 8.1.5.

When this procedure is triggered by another UE's explicit request, the fields in the request are interpreted as follows:

-	The field 'Resource selection window location' is the concatenation of the starting time location and the ending time location of the resource selection window. The starting and ending time locations of the resource selection window are each encoded in the same way as the reference slot as described in clause 8.1.5A.

-	The field 'Resource reservation period' is encoded in the same way as the field of the same name in SCI format 1-A.

When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4 with the above parameters and the following modifications:

-	Step 6a) The UE excludes candidate single-slot resource(s) belonging to slot(s) where the UE does not expect to perform SL reception of a TB due to half-duplex operation, if all the following conditions are met:

-	the UE is a destination UE of the TB for whose transmission the preferred resource set is being determined;

-	the higher layer parameter sl-Condition1-A-2 is not provided.

When determining a non-preferred resource set, the UE considers any resource(s) within the resource selection window, if indicated by a received explicit request, and satisfying at least one of the following conditions as non-preferred resource(s):

-	resource(s) indicated by a received [SCI format 1-A], satisfying at least one of the following criteria:

-	the RSRP measurement performed, according to clause 8.4.2.1, for the received [SCI format 1-A], is higher than ThprioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in sl-ThresholdRSRP-Condition1-B-1-Option1List, where k=pi.

-	the UE is a destination UE of a TB associated with the received [SCI format 1-A] and the RSRP measurement performed, according to clause 8.4.2.1 for the received [SCI format 1-A], is lower than Th'prioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th'(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in sl-ThresholdRSRP-Condition1-B-1-Option2List, where k=pi.

-	resources(s) in slot(s) in which the UE does not expect to perform SL reception due to half duplex operation, if the UE is a destination UE of a TB for whose transmission the non-preferred resource set is being determined.