3GPP TSG-RAN WG1 #112bis-e	R1-2303827

17th – 26th Apr 2023

Online

Source:                    	DENSO CORPORATION

Title:  	Discussion on XR-specific capacity enhancements

Document for:        	Discussion and decision

Agenda Item:         	9.8.1		XR-specific capacity enhancements

- Introduction

In RAN#99, updated WID on XR enhancements was approved [1]. From RAN1 perspective, the following objectives were specified for XR-specific capacity improvements.

Specify the enhancements related to capacity: - Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2);

In this contribution, we provide the discussion on the potential capacity enhancements for XR.

- Discussion

Compared to dynamic grant (DG), configured grant (CG) is effective in reducing the delay. CG provides a UE with periodic resource allocations without having to send scheduling request (SR) and wait for scheduling DCI. CG is useful for minimizing the transmission delay to meet the packet delay budget (PDB) of XR services. CG has another characteristic that it is suitable for transmitting small and fixed size packets. Therefore, for UL scene/video traffic with large packets, one PUSCH transmission occasion within one CG period may not be enough. Additional dynamic scheduling is required, which causes additional delay. In order to solve this problem, it was agreed to support multiple PUSCH transmission occasions within one CG period, which is called multi-PUSCHs CG. In addition, it was also agreed that the UE can dynamically indicate the unused CG PUSCH occasion(s) to adapt to scene/video traffic whose packet size varies significantly. The indication by the UE allows the gNB to reallocate unused occasion(s) for other transmissions. This enhancement could help to avoid wasting resources when the UE is allocated more CG PUSCH occasions than it needs.

  - Multi-PUSCHs CG

For determination of the time domain resource allocation (TDRA) of CG PUSCHs associated to a multi-PUSCHs CG, it can be considered to reuse the framework for scheduling multiple PUSCHs by a single DCI. In the current specification, for DG, multiple PUSCH occasions can be scheduled by DCI indicating the row index containing multiple SLIV entries in the TDRA table. For simplicity of specification, a similar mechanism can be applied to multi-PUSCHs CG. In other words, scheduling one or more PUSCH occasions within one CG period can be indicated by the TDRA field included in higher layer configurations for CG Type 1 or in activation DCIs for CG Type 2. It is also considered to determine the TDRA based on NR-U CG framework. This is a method of allocating consecutive PUSCH occasions by using parameters similar to cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16. It is also acceptable in that most of the existing mechanisms can be reused.

Observation 1:	For determination of TDRA of CG PUSCHs associated to a multi-PUSCHs CG, the following options can be considered.

- Option.1: Reusing the framework for scheduling multiple PUSCHs by a single DCI

- Option.2: Reusing the framework for NR-U CG

Comparing the two options, Option.1 can provide more flexibility as it can configure different K2 or SLIV per each CG PUSCH occasion. For example, Option.1 can support scheduling multiple PUSCHs in non-consecutive slots by configuring different K2 in the TDRA table.

Observation 2:	TDRA determination based on single DCI scheduling multiple PUSCHs can provide more flexibility as it can configure different K2 or SLIV per each CG PUSCH occasion.

For legacy CG that supports only one PUSCH occasion within a period, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation.

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

If this equation is applied directly to multi-PUSCHs CG, the same HARQ process ID is calculated for all PUSCH occasions in a same period. In this case, the HARQ retransmission process may not work well. Therefore, it would be necessary to study how to assign a different HARQ process ID for each PUSCH occasion of multi-PUSCHs CG. This can be achieved simply by introducing the same approach as DG scheduling multi-PUSCHs. In other words, the HARQ process ID is determined by incrementing the HARQ process ID of the preceding PUSCH in the CG period. However, this approach has the drawback that a large number HARQ process IDs are reserved. Given that multi-PUSCHs CG may have some unused occasions due to the random jitter and various packet size of the XR service, some of the reserved HARQ process IDs could be wasted. Therefore, it seems effective to allow the UE to select the HARQ process IDs for multiple CG PUSCH occasions and to transmit them on UCI, as in NR-U CG. This approach increases the UCI overhead, but can be considered depending on the design of the UCI.

Observation 3:	For determination of HARQ process ID of CG PUSCHs associated to a multi-PUSCHs CG, the following options can be considered.

- Option.1: As in DG scheduling multi-PUSCHs, the HARQ process ID is determined by incrementing the HARQ process ID of the preceding PUSCH in the CG period.

- Option.2: As in NR-U CG, the UE can select the HARQ process IDs for multiple CG PUSCH occasions and transmit them on UCI.

Observation 4:	Given that multi-PUSCHs CG may have some unused occasions due to the random jitter and various packet size of the XR service, it seems effective to allow the UE to select the HARQ process IDs, as in NR-U CG, so as not to reserve too many HARQ process IDs.

  - Indication of unused CG PUSCH occasion(s) based on UCI

With respect to how UCI indicates unused occasion(s), one possible way is to introduce a bitmap where each bit can indicate whether the corresponding PUSCH occasion is unused or not. For example, UCI can include the indication for all PUSCH occasions within a single CG period. In this case, the required bitmap size can be equal to the number of configured PUSCH occasions, regardless of whether they are valid. Another way is to indicate the number of PUSCH occasions that the UE will use. For example, after receiving UCI, the gNB can implicitly assume that the indicated number of consecutive PUSCH occasions are used and the remaining ones are unused. While this way can slightly reduce the UCI size, it may not provide as much flexibility as the indication using a bitmap.

Figure 1: Example of how UCI indicates unused occasion(s)

Proposal 1:	RAN1 to study introducing a bitmap into the UCI, where each bit can indicate whether the corresponding PUSCH occasion is unused or not.

Proposal 2:	The bitmap size for the indication of unused occasion(s) can be equal to the number of configured PUSCH occasions, regardless of whether they are valid.

The enhancement of the UCI indication is intended to allow the gNB to reallocate unused occasion(s) for other transmissions. Therefore, the UE is required to send the UCI as early as possible to allow enough time for the gNB processing. For example, the UCI indication can be included in the first configured PUSCH occasion within a CG period. However, considering UL jitter and the mismatch between CG periodicity and XR frame rate, UL data may not always arrive at the UE before the first configured PUSCH occasion. In this case, the UE may not know if the remaining PUSCH occasions are unused due to the lack of the information on the amount of UL data. Moreover, the first configured PUSCH occasion may not be used because there is no data to be transmitted other than the UCI. From these perspectives, the UCI can be multiplexed in the first transmitted CG PUSCH within a CG period. Alternatively, it seems to be beneficial to include the UCI indication in all transmitted CG PUSCHs to improve reliability.

Observation 5:	Considering UL jitter and the mismatch between CG periodicity and XR frame rate, the UE may not know the amount of UL data to indicate unused occasion(s) in the first configured PUSCH occasion within a CG period.

Proposal 3:	RAN1 to study the following options regarding when the UCI is sent.

- Option.1: The UCI can be multiplexed in the first transmitted CG PUSCH within a CG period.

- Option.2: The UCI can be multiplexed in all transmitted CG PUSCHs.

- Summary and proposal

In this contribution, we provided the following observation and proposals.

Observation 1:	For determination of TDRA of CG PUSCHs associated to a multi-PUSCHs CG, the following options can be considered.

- Option.1: Reusing the framework for scheduling multiple PUSCHs by a single DCI

- Option.2: Reusing the framework for NR-U CG

Observation 2:	TDRA determination based on single DCI scheduling multiple PUSCHs can provide more flexibility as it can configure different K2 or SLIV per each CG PUSCH occasion.

Observation 3:	For determination of HARQ process ID of CG PUSCHs associated to a multi-PUSCHs CG, the following options can be considered.

- Option.1: As in DG scheduling multi-PUSCHs, the HARQ process ID is determined by incrementing the HARQ process ID of the preceding PUSCH in the CG period.

- Option.2: As in NR-U CG, the UE can select the HARQ process IDs for multiple CG PUSCH occasions and transmit them on the UCI.

Observation 4:	Given that multi-PUSCHs CG may have some unused occasions due to the random jitter and various packet size of the XR service, it seems effective to allow the UE to select the HARQ process IDs, as in NR-U CG, so as not to reserve too many HARQ process IDs.

Proposal 1:	RAN1 to study introducing a bitmap into the UCI, where each bit can indicate whether the corresponding PUSCH occasion is unused or not.

Proposal 2:	The bitmap size for the indication of unused occasion(s) can be equal to the number of configured PUSCH occasions, regardless of whether they are valid.

Observation 5:	Considering UL jitter and the mismatch between CG periodicity and XR frame rate, the UE may not know the amount of UL data to indicate unused occasion(s) in the first configured PUSCH occasion within a CG period.

Proposal 3:	RAN1 to study the following options regarding when the UCI is sent.

- Option.1: The UCI is multiplexed in the first transmitted CG PUSCH within a CG period.

- Option.2: The UCI is multiplexed in all transmitted CG PUSCHs.

- References

[1]	RP-230786, Updated WID on XR Enhancements for NR, Nokia, Qualcomm