3GPP TSG RAN WG1 #112bis-e			R1-2303835

e-Meeting, April 17th – 26th, 2023

Source:	WILUS Inc.

Title:	Discussion on enhanced sidelink operation on FR2 licensed spectrum

Agenda item:	9.4.3

Document for:	Discussion

- Introduction

In the previous RAN1#112 meeting, the followings were agreed for enhanced sidelink operation on FR2 licensed spectrum [1].

Agreement at RAN1#112 meeting For sidelink beam management, RAN1 is to study - how transmit beam(s) training and/or receive beam(s) training is performed - whether and how spatial related information (e.g., TCI, QCL, beam ID, etc) information could be identified - the relationship between PC5 unicast link establishment and sidelink initial beam pairing (e.g., whether initial beam pairing procedure starts before, during or after sidelink unicast link establishment procedure.) Agreement at RAN1#112 meeting RAN1 is to study sidelink beam measurement and reporting schemes (e.g., periodicity, contents, container, timing, procedure, etc.) for sidelink beam maintenance. Agreement at RAN1#112 meeting RAN1 is to study sidelink beam indication and switching schemes (e.g., framework, general procedure, contents, signaling, timing, etc.) for sidelink beam maintenance. Agreement at RAN1#112 meeting RAN1 is to study the information related to a sidelink beam failure instance that the PHY layer provides to the MAC layer.

In this contribution, we discuss issues on enhanced sidelink operation on FR2 licensed spectrum and provide our views.

Discussion on enhanced sidelink operation on FR2 licensed spectrum

To perform sidelink operation on FR2 spectrum, it may be critical to specify sidelink beam management process. Hence, in this section, we discuss enhanced sidelink operation on FR2 licensed spectrum with respect to sidelink beam management, including initial beam-pairing, beam maintenance, and beam failure recovery for sidelink unicast communication.

Issues on sidelink initial beam pairing

In NR Uu initial beam pairing process, SSB and PRACH at random access procedure are used for initial beam pairing. Specifically, NR UE transmits PRACH to a gNB at appropriate PRACH occasion. Note that, PRACH occasions are related to beam information, thus the gNB can have the beam information of the NR UE if the gNB detects PRACH. To reuse the NR Uu beam management concepts, S-SSB can be used for sidelink initial beam pairing process. However, there is no PRACH in sidelink, so NR SL UE does not transmit PRACH in response to S-SSB reception to indicate the preferred beam. To cover this problem, RAN1 should further investigate on the process for sidelink initial beam pairing using S-SSB.

- Proposal 1: S-SSB should be used for sidelink initial beam pairing.

- Observation 1: NR sidelink does not have PRACH transmission or concept of PRACH occasion, hence NR SL UE cannot indicate beam related information based on PRACH occasion.

- Proposal 2: RAN1 should further investigate on the process for sidelink initial beam pairing using S-SSB.

Since NR SL UE can be configured on multiple resources for S-SSB transmissions by higher layer, NR SL Tx UE can perform beam sweeping for every configured S-SSB transmissions. However, since the S-SSB period is 16 frames which is quite long for beam sweeping with multiple beams, it might be necessary to introduce new S-SSB periodicity for beam sweeping within a certain short duration as efficient beam operation. Also, for beam sweeping process using S-SSB, number of S-SSBs per period should be (pre-)configured as large number, for example, more than 8 times of S-SSB transmissions can be configured rather than other small values. To reuse the concept of initial beam pairing supported in current NR specification, NR SL also can support more than 8 times of S-SSB transmissions in an S-SSB period for Tx beam sweeping. Note that, current NR specification supports maximum 64 SSB transmissions within an half frame. Then, NR SL Rx UE can measure PSBCH-RSRP for identifying the best Tx beam. As a response indicating the best beam information, NR SL Rx UE can transmit S-SSB repeatedly at configured resources so that the transmitting beam of NR SL Rx UE’s S-SSB is associated with the best receiving beam of the Rx UE.

- Proposal 3: NR SL Tx UE can perform beam sweeping for every configured S-SSB transmissions.

- Proposal 4: For efficient beam sweeping process using S-SSB, shorter period for S-SSB transmission and increased number of S-SSB transmissions per period should be (pre-)configured.

- Proposal 5: NR SL Rx UE can measure PSBCH-RSRP for identifying the best beam.

- Proposal 6: NR SL Rx UE can transmit precoding applied S-SSB repeatedly at configured resources that the transmission beam of NR SL Rx UE’s S-SSB is associated with the best receive beam of the Rx UE.

Issues on sidelink beam maintenance

In sidelink initial beam pairing process, wide Tx beam and wide Rx beam are paired. However, for more reliable sidelink unicast communications, additional beam refinement process might be needed. Similar with NR, for sidelink beam maintenance or sidelink beam refinement process, sidelink CSI-RS can be used in addition to S-SSB. Since current NR sidelink supports aperiodic SL CSI-RS [2], aperiodic SL CSI-RS can be utilized for sidelink beam maintenance purpose.

- Proposal 7: NR SL UE can perform beam maintenance procedure using aperiodic SL CSI-RS which is supported by current NR sidelink.

If NR SL UE uses the aperiodic SL CSI-RS for beam maintenance, it should trigger the transmission of SL CSI-RS. However, current specification [2] supports triggering SL CSI-RS transmission to request sidelink CSI report. Hence, it is important to differentiate purpose of triggering SL CSI-RS transmission between requesting sidelink CSI report and requesting beam report. For example, explicit signalling for beam maintenance can be supported by adding more signalling bits in SCI which triggers SL CSI-RS transmission for beam maintenance. Hence, RAN1 can study the signalling method for triggering the transmission of aperiodic SL CSI-RS for beam maintenance.

- Proposal 8: RAN1 to study triggering mechanisms of aperiodic SL CSI-RS for beam maintenance purpose.

- Proposal 9: RAN1 to study the signalling method for triggering the transmission of aperiodic SL CSI-RS for beam maintenance purpose.

Regarding to the sidelink beam measurement and beam reporting for SL beam maintenance procedure, SL UE can measure SL CSI-RSRP to select the best Tx/Rx beams. Also, NR SL UE can transmit beam report for received SL CSI-RS based on the SCI signalling. For example, SL UE may transmit beam report if both signalling for triggering transmission of SL CSI-RS and beam report request (or CSI-request) are configured. Then, two NR SL UEs may perform sidelink P2 procedure, i.e., sidelink Tx beam refinement. For another example, SL UE may not transmit beam report if the signalling for triggering transmission of SL CSI-RS is configured, but the signalling for beam report request (or CSI-request) is not configured. Then, two NR SL UEs may perform sidelink P3 procedure, i.e., sidelink Rx beam refinement.

- Proposal 10: NR SL UE can transmit beam report if both signalling for triggering transmission of SL CSI-RS and CSI request are configured.

For beam maintenance purpose, there can be two options for beam switching schemes: Intra-slot beam switching and inter-slot beam switching. Intra-slot beam switching is that UE can perform beam sweeping in single NR slot. For example, as in Figure 1, UE can transmit SL CSI-RS using different Tx beams in an NR slot. As depicted in Figure 1, UE can transmit PSCCH with wide beam and PSSCH with narrow beam, since PSCCH contains control or reservation information which has to be received by many UEs. For this procedure, transient gap for intra-slot beam switching should be needed. On the other hand, inter-slot beam switching means that UE can apply one beam in a single NR slot. For example, UE can perform unicast transmission using the paired Tx beam.

Figure 1. Depiction of intra-slot beam sweeping

Since intra-slot beam switching enables to transmit using multiple beams in a single slot, low latency can be achieved if NR SL UE performs sidelink beam maintenance using intra-slot beam switching. Thus, for low latency performance, intra-slot beam switching should be supported for sidelink beam maintenance procedure.

- Observation 2: Supporting intra-slot beam switching can achieve lower latency.

- Proposal 11: RAN1 to study intra-slot beam switching scheme for low latency performance of NR SL.

Inter-slot beam switching also has several advantages compared to intra-slot beam switching. For UE that has limited capabilities, inter-slot beam switching can be used for beam maintenance procedure. Also, since inter-slot beam switching use one beam in a single NR slot, existing PSFCH can be used for beam report. Hence, we propose RAN1 to study inter-slot beam switching scheme and corresponding beam reporting scheme.

- Observation 3: Inter-slot beam switching can be used for UE with limited capability.

- Observation 4: Inter-slot beam switching scheme can use existing PSFCH for beam reporting.

- Proposal 12: RAN1 to study inter-slot beam switching and corresponding beam reporting schemes.

Issues on sidelink beam failure recovery

Sidelink beam failure recovery process should also be defined for the case of beam failure between sidelink UEs. Sidelink beam failure recovery process may consist of sidelink beam failure detection (BFD), sidelink beam failure recovery request (BFRQ), and sidelink UE’s response to the SL BFRQ. First, for SL BFD, S-SSB and SL CSI-RS can be utilized. SL Rx UE can measure PSBCH-RSRP or SL-CSI-RSRP for detection of the SL BFD. Similar to NR beam failure recovery process, SL Rx UE can trigger SL beam failure instance (BFI) at PHY layer if the measurement is lower than the (pre-)configured threshold. Then, SL BFI can be sent to MAC layer. MAC layer may increase SL BFI counter and set SL BFD timer for sidelink beam failure detection. If SL BFD timer expires, SL BFI counter may be initialized. However, similar to NR, if SL BFI counter becomes equal to or larger than the (pre-)configured threshold before SL BFD timer expires, then the Rx UE can transmit BFRQ to Tx UE.

- Proposal 13: For sidelink beam failure recovery process, SL UE can measure PSBCH-RSRP or SL CSI-RSRP for sidelink beam failure detection.

- Proposal 14: PHY layer can provide sidelink beam failure instance to MAC layer if PSBCH-RSRP or SL CSI-RSRP measurement is lower than the (pre-)configured threshold.

- Proposal 15: For beam failure recovery process, RAN1 to study when/how NR SL UE transmits SL beam failure recovery request.

- Proposal 16: Sidelink beam failure recovery request can be transmitted if sidelink BFI counter reaches the (pre-)configured threshold before SL BFD timer expires.

If SL BFI counter reaches to the (pre-)configured threshold before SL BFD timer expires, SL Rx UE should transmit SL BFRQ to SL Tx UE. There may be some options for transmitting SL BFRQ. In our view, SL BFRQ can be transmitted via PSFCH since the SL CSI-RS is transmitted via PSSCH. Using PSFCH for transmitting SL BFRQ has more advantages than using PSCCH/PSSCH, because the PSFCH occasion for SL CSI-RS reception is already configured according to the current specification [3]. It means, Rx UE does not have to allocate resource for transmitting SL BFRQ. Moreover, SL BFRQ may be transmitted for 1 bit, and PSFCH is more appropriate than PSCCH/PSSCH for 1 bit transmission. For more specific information of PSFCH transmission of SL BFRQ, network can configure the occasion for transmitting SL BFRQ (BFRQ occasion) as the subset of PSFCH occasion. For example, if the PSFCH period is configured to 2 slots, then BFRQ occasion may be configured for every 10 slots within the PSFCH occasion. Then, SL Rx UE can use the BFRQ occasion corresponding to the slot for the earliest CSI-RS reception that satisfies ‘SL_BFI_counter≥Threshold.’

- Proposal 17: For sidelink beam failure recovery process, UE can transmit beam failure recovery request via PSFCH transmission.

- Proposal 18: For sidelink beam failure recovery process, occasions for sidelink beam failure recovery request transmission can be configured as a subset of sidelink PSFCH occasion.

Others

Since NR sidelink UE can have multiple sidelink unicast links, we propose RAN1 to study other possible issues caused by multi-unicast links. For example, if NR sidelink UE has multiple sidelink unicast links with other SL UEs, there is a issue for transmission or reception of multiple PSFCHs. Specifically, if NR sidelink UE has to transmit or receive two or more PSFCHs in the same PSFCH occasion, SL UE has to decide appropriate Tx beam or Rx beam for multiple PSFCH transmission or reception respectively. Hence, we propose RAN1 to study PSFCH Tx/Rx beam for simultaneous multi-PSFCH transmission/reception, respectively. Also, since reliability of sidelink is affected by resource allocation schemes, it is necessary to study impact of beam to both resource allocation procedures, i.e., Mode 1 and Mode 2. For example, when NR SL UE allocates PSCCH/PSSCH resource for specific Tx beam, there can be a possible problem if the UE use the information received with the Rx beam which is not associated with the Tx beam. In other words, the NR SL UE can allocate not appropriate resource for PSCCH/PSSCH transmission. Hence, we also propose RAN1 to study impact and enhancement to resource allocation schemes in terms of Tx/Rx beam.

- Proposal 19: PSFCH Tx/Rx beam for simultaneous multiple PSFCH transmission/reception should be studied respectively.

- Proposal 20: Possible impact and enhancement to resource allocation schemes (both Mode 1 and Mode 2) in terms of beam should be studied.

- Conclusion

In this contribution, we have discussed issues on enhanced sidelink operation on FR2 licensed spectrum. As a conclusion, we summarize our view as follows:

- Proposal 1: S-SSB should be used for sidelink initial beam pairing.

- Observation 1: NR sidelink does not have PRACH transmission or concept of PRACH occasion, hence NR SL UE cannot indicate beam related information based on PRACH occasion.

- Proposal 2: New process for sidelink initial beam pairing using S-SSB should be defined.

- Proposal 3: NR SL Tx UE can perform beam sweeping for every configured S-SSB transmissions.

- Proposal 4: For efficient beam sweeping process using S-SSB, shorter period for S-SSB transmission and increased number of S-SSB transmissions per period should be (pre-)configured.

- Proposal 5: NR SL Rx UE can measure PSBCH-RSRP for identifying the best beam.

- Proposal 6: NR SL Rx UE can transmit precoding applied S-SSB repeatedly at configure resources that the transmission beam of NR SL Rx UE’s S-SSB is associated with the best receive beam of the Rx UE.

- Proposal 7: NR SL UE can perform beam maintenance procedure using aperiodic SL CSI-RS which is supported by current NR sidelink.

- Proposal 8: RAN1 to study triggering mechanisms of aperiodic SL CSI-RS for beam maintenance purpose.

- Proposal 9: RAN1 to study the signalling method for triggering the transmission of aperiodic SL CSI-RS for beam maintenance purpose.

- Proposal 10: NR SL UE can transmit beam report if both signalling for triggering transmission of SL CSI-RS and CSI request are configured.

- Observation 2: Supporting intra-slot beam switching can achieve lower latency.

- Proposal 11: RAN1 to study intra-slot beam switching scheme for low latency performance of NR SL.

- Observation 3: Inter-slot beam switching can be used for UE with limited capability.

- Observation 4: Inter-slot beam switching scheme can use existing PSFCH for beam reporting.

- Proposal 12: RAN1 to study inter-slot beam switching and corresponding beam reporting schemes.

- Proposal 13: For sidelink beam failure recovery process, SL UE can measure PSBCH-RSRP or SL CSI-RSRP for sidelink beam failure detection.

- Proposal 14: PHY layer can provide sidelink beam failure instance to MAC layer if PSBCH-RSRP or SL CSI-RSRP measurement is lower than the (pre-)configured threshold.

- Proposal 15: For beam failure recovery process, RAN1 to study when/how NR SL UE transmits SL beam failure recovery request.

- Proposal 16: Sidelink beam failure recovery request can be transmitted if sidelink BFI counter reaches the (pre-)configured threshold before SL BFD timer expires.

- Proposal 17: For sidelink beam failure recovery process, UE can transmit beam failure recovery request via PSFCH transmission.

- Proposal 18: For sidelink beam failure recovery process, occasions for sidelink beam failure recovery request transmission can be configured as a subset of sidelink PSFCH occasion.

- Proposal 19: PSFCH Tx/Rx beam for simultaneous multiple PSFCH transmission/reception should be studied respectively.

- Proposal 20: Possible impact and enhancement to resource allocation schemes (both Mode 1 and Mode 2) in terms of beam should be studied.

References

- Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0

- 3GPP TS 38.214 v17.5.0 (2023-03)

- 3GPP TS 38.213 v17.5.0 (2023-03)