3GPP TSG-RAN WG1 #112bis-e	R1-2303837

e-Meeting, 17th April – 26th April, 2023

Agenda item:		9.5.1.1

Source:	MediaTek Inc.

Title:	Reference signal design for sidelink positioning

Document for:		Discussion

# Introduction

The following agreements were reached in previous meeting,

Agreement SL PRS sequence is generated based on Gold sequence: rn=121-2⋅c2n+j121-2⋅c2n+1, where c(i) is a pseudo-random sequence as defined in Clause 5.2.1 of TS 38.211. Agreement - For SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID,seqSL-PRS. - The pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS Agreement For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option: - Option 1: nID,seqSL-PRS is a higher layer configured parameter - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined. Agreement Range of the parameter nID,seqSL-PRS is: nID,seqSL-PRS∈0,1,…,4095 Agreement A SL PFL is not defined. SL positioning RS are defined directly with respect to and contained within a single SL BWP and carrier. Agreement Support SCS values for SL PRS include: - 15 kHz, 30 kHz, 60 kHz for FR1, and 60 kHz, 120 kHz for FR2 - Which SCS values are required, and which ones are optional follow Rel-16 UE capabilities. Agreement For RE-offset sequence for SL PRS, the RE-offset sequences specified for DL PRS are considered as a starting point. - FFS: Exact RE-offset sequences Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs Agreement For SL PRS in shared or dedicated resource pools, - at least comb sizes (N) 2, 4 are supported. - Comb size 6 is supported at least in dedicated resource pool - FFS: comb size 6 in shared resource pool - Comb size 1 is supported at least in shared resource pool - FFS: comb size 1 in dedicated resource pool - comb sizes (N) > 12 are not supported. - FFS: support of comb sizes (N) of 8, 12. Agreement For SL PRS in shared and dedicated resource pools, - SL PRS patterns with full staggering are supported. - FFS: whether (M,N)=(6,6) is supported - SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs: - (M, 2) with M = {1} - (M, 4) with M = {2} - FFS: constraints on maximum effective comb size - FFS: support of partial staggering for other comb sizes - FFS: Support of SL PRS patterns with M > N at least with full staggering. Agreement TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability Agreement The OLPC framework defined for PSSCH/PSCCH is considered as a starting point for OLPC for SL PRS.

In this contribution, we further express our views on the present issues.

# SL-PRS structure

The multiple comb sizes are agreed to support. Whether to support multiple (M,N) values in a slot may consider several aspects. For example, the potential collision may encounter and in order to avoid the collision, the RE usage efficiency would also be reduced. Some comb sizes may form a hierarchy so that multiple (M,N) values could be realized. For example, comb size 2 and 4, or comb size 2, 4 and 8.

Fig. 2-1 gives an example that,

- The blue one is a comb-2, one symbol occupied at first symbol with RE offset 0

- The red one is also a comb-2, one symbol occupied at second symbol with RE offset 0

- The orange one is a comb-2, two symbols with RE offset (1, 3)

- The green one is a comb-2, two symbols with RE offset (3, 1)

It shows that, when some restrictions are made, it is possible to multiplex comb-2 and comb-4 pattern in the same set of symbols.

Fig. 2-2 and Fig. 2-3 show that,

- For Fig. 2-2, if two UEs allocate the respective comb-2 pattern as shown in the figure, it doesn't allow any UE to further allocate a comb-4 pattern

- For Fig. 2-3, if two UEs allocate the respective comb-4 pattern as shown in the figure, it doesn't allow any UE to further allocate a comb-2 pattern

Based on the above analysis, the feasible arrangement within a slot under TDM based multiplexing could be, a single (M,N) value that is used for a set of symbols, and another (M,N) value that could be used for another set of symbols. The two sets of symbols don't overlap with each other.

Fig. 2-1       Fig. 2-2        Fig. 2-3

Proposal 2-1: M (symbol number) > N (comb size) is not supported

Proposal 2-2: (M,N) = (6,6) is not supported

Proposal 2-3: Comb size 1 in dedicated resource pool is not supported

Proposal 2-4: It could be feasible within a slot under TDM based multiplexing that, a single (M,N) value is used for a set of symbols, and another (M,N) value could be used for another set of symbols. The two sets of symbols don't overlap with each other

Proposal 2-5: It is also feasible within a slot that a single (M,N) value applies

Proposal 2-6: For SL-PRS transmission within the shared resource pool, both the comb based and TDM based multiplexing are not supported

Proposal 2-7: Prefer to also support comb size 8, since SRS for positioning supports this comb size, and also comb size 2, 4 and 8 form the hierarchy

Proposal 2-8: Support comb-6 with 2 symbols to obtain an equivalent comb-3 structure

Proposal 2-9: SL-PRS is also applied with the RE offset sequence for the DL-PRS, and the SL-PRS has the own symbol number set different from that for DL-PRS

# Power control

Under uu interface, the uplink power control is mainly based on

- the RSRP measurement on downlink path-loss RS

- the provided transmission power of the base station

- the expected reception power of the base station

- the transmission bandwidth

Moreover, the sidelink (open loop) power control further considers that

- Both the DL and SL path loss could be used

- For SL path loss estimation, the RX UE may provide the filtered RSRP measurement results based on SL-PRS to the TX UE

The existing SL power control mechanism is also suitable for the UL-AOA method with unicast type, as shown in Fig. 3-1. The UE A may provide (report) the filtered RSRP results based on SL-PRS to another UE. The existing SL power control could also be applied to the SL-RTT method with unicast, as shown in Fig. 3-2. In this condition, UE A and UE B may provide the filtered RSRP results to each other for path loss estimation.

Further for the condition in Fig. 3-2, it is also feasible that a UE provides the filtered transmission power to another UE. For example, UE B provides the filtered RSRP measurement results to UE A for UE A to estimate path loss, and UE A provides the filtered transmission power to UE B for UE B to estimate path loss, since UE B has performed RSRP measurement based on the transmitted SL-PRS from UE A.

Fig. 3-3 shows SL-RTT method with groupcast and unicast types. The UE A may provide the filtered transmission power to other UEs through groupcast data transmission, then the individual RSRP measurement result provided to each of the other UEs within the group could be avoided.

Fig. 3-1                            Fig. 3-2

Fig. 3-3,                      Fig. 3-4,                         Fig. 3-5,

Proposal 3-1: Consider to support both DL path loss and SL path loss for SL-PRS power control

Proposal 3-2: Consider to support the report of the filtered RSRP to the UE from a UE receiving SL-PRS

Proposal 3-3: Consider to support providing the transmission power from the UE to a UE receiving SL-PRS, at least it is feasible for groupcast type

# AGC symbol design

When we consider the following design requirements,

- Support TDM multiplexing of SL-PRS within a slot

- Apply an AGC symbol for SL-PRS transmission, or the AGC for SL-PRS could use the AGC symbol for PSCCH. The AGC symbol for SL-PRS may not always be needed

- Facilitate Doppler and frequency shift estimation

And the corresponding design may consider the following constraints,

- The symbol number for a resource may need to be compact

  - And this could be the value to apply partial staggering

- Doppler and frequency shift estimation may perform the delay correlation between the RS at different symbols and at same subcarriers. The TRS structure could be leveraged

Then in the dedicated resource pool, if the AGC symbol transmits SL-PRS, and the RE offset in the AGC symbol is the same as that in the last symbol in a SL-PRS resource, then it could be up to UE implementation to utilize the AGC symbol for gain control, or for Doppler shift estimation. Fig. 4-1 shows the potential slot structure to satisfy the above-mentioned design requirements.

Proposal 4-1: If AGC symbol for SL-PRS is configured in the dedicated resource pool, the AGC symbol also transmits SL-PRS, and the corresponding RE offset is the same as that in the last symbol in a SL-PRS resource

Fig. 4-1,

# Seed initialization

There are a number of options listed below to determine the ID value within the seed initialization equation. In our view, the seed generation could leverage that for SL CSI-RS, and also, the SL-PRS may be allocated through the associated PSCCH. Then option 2 is considered.

Agreement For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option: - Option 1: nID,seqSL-PRS is a higher layer configured parameter - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined.

Proposal 5-1: For the sequence ID value generation for SL-PRS, option 2 that is based on the CRC of PSCCH associated with the transmission is considered

# Conclusion

Proposal 2-1: M (symbol number) > N (comb size) is not supported

Proposal 2-2: (M,N) = (6,6) is not supported

Proposal 2-3: Comb size 1 in dedicated resource pool is not supported

Proposal 2-4: It could be feasible within a slot under TDM based multiplexing that, a single (M,N) value is used for a set of symbols, and another (M,N) value could be used for another set of symbols. The two sets of symbols don't overlap with each other

Proposal 2-5: It is also feasible within a slot that a single (M,N) value applies

Proposal 2-6: For SL-PRS transmission within the shared resource pool, both the comb based and TDM based multiplexing are not supported

Proposal 2-7: Prefer to also support comb size 8, since SRS for positioning supports this comb size, and also comb size 2, 4 and 8 form the hierarchy

Proposal 2-8: Support comb-6 with 2 symbols to obtain an equivalent comb-3 structure

Proposal 2-9: SL-PRS is also applied with the RE offset sequence for the DL-PRS, and the SL-PRS has the own symbol number set different from that for DL-PRS

Proposal 3-1: Consider to support both DL path loss and SL path loss for SL-PRS power control

Proposal 3-2: Consider to support the report of the filtered RSRP to the UE from a UE receiving SL-PRS

Proposal 3-3: Consider to support providing the transmission power from the UE to a UE receiving SL-PRS, at least it is feasible for groupcast type

Proposal 4-1: If AGC symbol for SL-PRS is configured in the dedicated resource pool, the AGC symbol also transmits SL-PRS, and the corresponding RE offset is the same as that in the last symbol in a SL-PRS resource

Proposal 5-1: For the sequence ID value generation for SL-PRS, option 2 that is based on the CRC of PSCCH associated with the transmission is considered

# Reference