3GPP TSG-RAN WG1 #112bis-e	R1-2303842

e-Meeting, 17th April – 26th April, 2023

Agenda item:		9.5.1.2

Source:	MediaTek Inc.

Title:	Measurement and reporting design for sidelink positioning

Document for:		Discussion

# Introduction

The following agreements were reached in previous meeting,

Agreement SL PRS reference signal received power (SL PRS-RSRP) - is defined as the linear average over the power contributions (in W) of the resource elements that carry SL PRS reference signals configured for RSRP measurements within the considered measurement frequency bandwidth With regard to the reference point - For frequency range 1, the reference point for the SL PRS-RSRP shall be the antenna connector of the UE. - For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches. Agreement SL PRS reference signal received path power (SL PRS-RSRPP), - is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry SL PRS signal configured for the measurement, where SL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. With regard to the reference point - For frequency range 1, the reference point for the SL PRS-RSRPP shall be the antenna connector of the UE. - For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches. Agreement SL-PRS based RTOA TSL-RTOA is defined as the beginning time of SL subframe #i containing SL-PRS received from a UE, relative to the RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where - T0 is the nominal beginning time of SFN 0 or DFN0. - FFS on how to select between SFN 0 or DFN 0 for determination of T0. - FFS: the source for the reference timing - tSL-PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively - FFS on how to select between SFN or DFN Agreement Support both GCS and LCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement. - FFS on the applicable scenario/service for AoA/ZoA relative to LCS without translation of the LCS to GCS Agreement For definition of SL-PRS based Rx-Tx measurement, downselect one of the following alternatives in RAN1# 112b to minimize the impact of UE reference timing offset and mobility - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt2: SL-PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu. Agreement Study measurement report content for both the cases of sidelink positioning measurement reported to LMF and UE. Agreement For SL-PRS based Rx-Tx measurement for double sided RTT, consider sidelink PRS transmission without order restriction between multiple rounds of PRS transmission of involved UEs. - FFS on how to differentiate different PRS transmissions for sidelink PRS Rx-Tx measurement and report - FFS on impact of Scheme 2 resource allocation when the different orders in double sided RTT is considered and whether and how to minimize number of different orders - Aspects related to scheme 2 resource allocation are to be discussed in agenda 9.5.1.3 Agreement Study the necessity and scenarios of including location information and quality information of the location of a UE in sidelink positioning measurement report, considering different measurements and different reporting targets (LMF and UE). Agreement Study the following candidates for identification information in sidelink positioning report, considering different measurements and different reporting targets (LMF and UE): - SL-PRS resource ID/SL-PRS resource set ID if multiple resources/resource sets are configured to a UE - FFS: whether SL-PRS resource set is supported - Source ID and/or destination ID - Other identification information not precluded Agreement LoS/NLoS indicator can be included in a sidelink positioning measurement report, considering different reporting targets (LMF and UE). - LOS/NLOS indicator specified in Rel-17 positioning is reused as much as possible. - No specification impact for how to set this indicator. - From RAN1 perspective, no performance requirements are expected to be defined for setting indicator in Rel-18. Agreement Companies are encouraged to provide expected measurement report content in the following table to facilitate discussion in RAN1 #112bis-e. Note: this does not imply a different measurement report content for reporting to LMF or to UE. Table 6.2 Collection of measurement report content reporting to LMF; reporting to UE. SL-PRS based Rx-Tx measurement. SL-PRS based RSTD measurement. SL-PRS based RSRP measurement. SL-PRS based RSRPP measurement. SL-PRS based RTOA measurement. SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement. etc.

In this contribution, we further express our views on the present issues.

# Measurements for SL-RTT

There are three options listed below for SL RX-TX measurement definition,

Agreement For definition of SL-PRS based Rx-Tx measurement, downselect one of the following alternatives in RAN1# 112b to minimize the impact of UE reference timing offset and mobility - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt2: SL-PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu.

The RX-TX time difference measurement requires the RX (reception) and TX (transmission) timing. The main difference between Alt 1 and Alt 3 is, the Alt 1 applies actual SL-PRS transmission timing, and the Alt 3 applies the SL-PRS transmission timing which is the intended transmission closest in time to the reception timing.

A question for Alt 1 is, what is the actual transmission timing. As shown in Fig. 2-1, UE B performs the measurement at t3, and UE B also performs the transmission at t2, which is earlier than t3, and also at t5 with timing change which is later than t3. Then for the measurement at t3, the actual transmission timing should be that in t2 or t5?

In our view, the actual transmission timing depends on the measurement combination. Again by Fig. 2-1, if the measurement by UE A at t2 is combined with the measurement by UE B at t3, then the actual transmission timing is the timing at t2. Similarly, if the measurement by UE A at t5 is combined with the measurement by UE B at t3, then the actual transmission timing is the timing at t5.

What if the measurements at t2, t3 and t5 are combined to realize the double sided RTT? The solution could be that the UE also reports the timing change quantity at t5. Since the actual transmission timing is not clear and it may depend on the LMF to perform the combination, we consider to Alt 3, with additional signaling to indicate the transmission timing change quantity with a timestamp.

Proposal 2-1: For SL RX-TX time difference definition, reuse the R16/R17 definition in Uu

Proposal 2-2: Consider to have additional signaling to indicate the transmission timing change quantity with a timestamp

Fig. 2-1,

# Conclusion

Proposal 2-1: For SL RX-TX time difference definition, reuse the R16/R17 definition in Uu

Proposal 2-2: Consider to have additional signaling to indicate the transmission timing change quantity with a timestamp

# Reference