3GPP TSG RAN WG1 #112bis-e			R1-2303848

e-Meeting, April 17th – April 26th, 2023

Source:	Mitsubishi Electric

Title:	On sidelink co-channel coexistence issues

Agenda Item:	9.4.2 - Co-channel coexistence for LTE sidelink and NR sidelink

Document for: 	Discussion/Decision

# Introduction

One of the objectives of the WID about NR sidelink evolution [1] is to:

Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4]

- Reuse the in-device coexistence framework defined in Rel-16 as much as possible

In RAN1#110 [6], the group concluded that TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs. Yet, this solution, although feasible, is challenging to deploy and unable to adapt to future traffic changes [8], which also leads to important drawbacks in terms of performance. For these reasons, RAN plenary RAN#97 [7] guides RAN 1 to continue the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A.

In RAN1#110b-e and RAN1#111 meetings there was no significant progress on issues such as how to deal with PSFCH overlapping or higher SCS support, although some proposals exist and were thoroughly debated [9], [10]. In RAN1#112 meeting [11], progress was made on NR SL resource (re)selection procedure and a set of candidates for dealing with AGC issue caused by PSFCH was agreed as a working assumption. RAN#99 plenary meeting, further guidance was given to RAN1 on how to proceed on topics such as support of higher subcarrier spacing and AGC PSFCH handling.

The current contribution gives views on remaining issues related to PSSCH/PSCCH resource (re)selection procedure and PSFCH handling.

# Discussion

## On PSSCH/PSCCH resource (re)selection procedure

In the latest RAN1#112 the following agreement was made:

Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - FFS whether/how LTE SL RSRP is applied

Alt 2 may be a specific of Alt 3 if the SL RSRP threshold list of Alt 3 is the same as the SL RSRP threshold list configured for LTE. It can also fallback to a case of Alt 1 if the same RSRP threshold lists are configured for NR and LTE. From this perspective, we investigated by means of simulation whether Alt 3 brings any gain with respect to Alt 1.

We consider LTE and NR traffic as having the same priority (that is, we consider a single RSRP value in the RSRP list).

The variable Δ globally gives an indication on the degree of sensitivity with which LTE traffic protection is applied (Δ=-∞ meaning that LTE is always prioritized and Δ=+∞ meaning that LTE is never prioritized). This variable can have different exact expressions depending on the use case and the exact meaning will be detailed in each sub-section.

### Simulation results: Alt 3 vs Alt 1 with HARQ disabled

To separate the impact of PSFCH avoidance from the impact of the PSSCH/PSCCH resource allocation related decisions, we first investigate the case where PSFCH is disabled. One, two, three or four (re)transmissions are performed systematically.

Let us define Δ as being the difference between the RSRP thresholds corresponding to LTE and NR respectively. The value Δ (in dB) quantifies the degree of sensitivity with which LTE resources are excluded during NR resource (re)selection:

- Δ=0 (Alt 1) means that LTE resources are treated as the NR resources (same RSRP threshold, Alt 1)

- Δ=-∞ (always prioritize LTE) means that LTE resources are always excluded during NR resource (re)selection, regardless of the RSRP value reported by the LTE module

| Figure 1 - LTE PRR, HARQ disabled, one shot transmission | Figure 2 - NR PRR, HARQ disabled, one shot transmission |
| --- | --- |
| Figure 3 - LTE PRR, HARQ disabled, 2 (re)transmissions | Figure 4 - NR PRR, HARQ disabled, 2 (re)transmissions |
| Figure 5 - LTE PRR, HARQ disabled, 3 (re)transmissions | Figure 6 - NR PRR, HARQ disabled, 3 (re)transmissions |
| Figure 7 - LTE PRR, HARQ disabled, 4 (re)transmissions | Figure 8 - NR PRR, HARQ disabled, 4 (re)transmissions |

Since the number of NR retransmissions is constant, the proportion of LTE traffic vs NR traffic is not modified by the avoidance strategy. There is a (small) beneficial impact on LTE, which is slightly more important when the number of aggressor NR transmissions is higher. Nevertheless, it is clear from the evaluations that, at least when HARQ is disabled, the benefit of applying a different threshold between excluding resources reserved by other LTE SL UE and excluding resources reserved by NR UEs is very limited. Applying a stricter policy for exclusion of resources reserved by LTE only procures a marginal benefit for LTE performance. On the other hand, such a policy severely degrades NR performance. The penalty suffered by NR is not justified given the limited gain for LTE.

Observation 1: For NR SL resource (re)selection procedure, when HARQ is disabled, Alt 3 procures marginal gain for LTE while severely degrading the NR performance with respect to Alt 1.

### Simulation results: Alt 3 vs Alt 1 with HARQ enabled (groupcast option 2)

To separate the impact of PSFCH avoidance from the impact of the PSSCH/PSCCH resource allocation related decisions, we now investigate the case where HARQ ACK/NACK transmission is enabled but the impact of NR PSFCH on LTE is ignored (no specific protection measure is taken for PSFCH AGC issue: accept the collision if it occurs). This is a marginal case under the currently agreed behavior for PSFCH avoidance (conditional avoidance with an extreme threshold, avoidance is never performed). The PSFCH impact is separately investigated in section 2.2, because the strategy applied for PSFCH avoidance may have a severe impact on the performance and may completely mask/bias the differences between different strategies for PSSCH/PSCCH (re)selection.

Δ is defined as in the previous subsection as being the difference between the RSRP thresholds corresponding to LTE and NR respectively. The value Δ (in dB) quantifies the degree of sensitivity with which LTE resources are excluded during NR resource (re)selection:

- Δ=0 (Alt 1) means that LTE resources are treated as the NR resources (same RSRP threshold, Alt 1)

- Δ=-∞ (always prioritize LTE) means that LTE resources are always excluded during NR resource (re)selection, regardless of the RSRP value reported by the LTE module

When HARQ ACK/NACK option 2 is enabled for NR transmission with Alt 3, we also tested different avoidance strategies concerning the LTE reserved resource. The system behavior is quite different from the previous case (Alt 3 with HARQ disabled) because of the effect of NR HARQ process on the global traffic pattern. Avoiding LTE resources decreases the number of resources available for NR and consequently increases the number of NR retransmission. The proportion of LTE traffic vs NR traffic is modified by the avoidance strategy (please refer to the legend labels “NrRrPerTB” indicating the average number of NR (re)transmissions for each TB). When LTE avoidance becomes stricter, the average number of NR (re)trasmission increases (effect of HARQ process), thus the average NR traffic increases, rendering NR traffic visible to LTE via RSSI detection. This backfires on LTE. The detrimental effect on LTE caused by the increased NR traffic (more re-transmissions) is superior to the beneficial effect due to NR avoiding LTE. Alt 1 (Δ=0) performs better.

| Figure 9 – Alt 3: LTE PRR, NR with ACK/NACK groupcast option 2, no PSFCH avoidance measure | Figure 10 – Alt 3: NR PRR, NR with ACK/NACK groupcast option 2, no PSFCH avoidance measure |
| --- | --- |

Observation 2: For NR SL resource (re)selection procedure, when HARQ groupcast option 2 is enabled, Alt 1 outperforms Alt 3.

Since Alt 2 is a specific case of Alt 3, it can be concluded that it cannot outperform Alt 1 either.

Observation 3: For NR SL resource (re)selection procedure, Alt 1 is preferred to Alt 2 and Alt 3.

### Simulation results: Alt 4 vs Alt 1 with HARQ enabled (groupcast option 2)

As in the previous case, to separate the impact of PSFCH avoidance from the impact of the PSSCH/PSCCH resource allocation related decisions, we investigate the case where the impact of NR PSFCH on LTE is ignored (no specific protection measure is taken for PSFCH AGC issue: accept the collision if it occurs). This is a marginal case under the currently agreed behavior for PSFCH avoidance (conditional avoidance with an extreme threshold, avoidance is never performed). The PSFCH impact is separately investigated in section 2.2, because the strategy applied for PSFCH avoidance may have a severe impact on the performance and may completely mask/bias the differences between different strategies for PSSCH/PSCCH (re)selection.

Here, Δ qualifies the difference between the RSRP thresholds for LTE exclusion corresponding to slots with PSFCH and without PSFCH respectively. We consider the same RSRP thresholds for LTE and NR exclusion in slots without PSFCH. The value Δ (in dB) quantifies the degree of sensitivity with which LTE resources are excluded during NR resource (re)selection in slots containing PSFCH:

- Δ=0 (Alt 1) means that LTE resources have the same treatment among slots with and without PSFCH (equivalent to Alt 1 because we also use the same RSRP thresholds for LTE and NR exclusion for slots without PSFCH)

- Δ=+∞ (never prioritize LTE) means that, in PSFCH slots, NR ignores the detected LTE reservations: NR transmission is privileged in the slots with PSFCH, hence pushing LTE away from those slots due to the RSSI mechanism of LTE.

When HARQ ACK/NACK option 2 is enabled for NR transmission, we also tested different strategies concerning the NR transmission concentration in the PSFCH slot. The system behavior is also driven by of the effect of NR HARQ process. We consider that every 4th slot contains PSFCH occasions.

When encouraging NR transmission to concentrate in PSFCH slots by increasing the Δ value there is a double effect:

- the proportion of NR traffic concentrated in PSFCH slots increases (see InPSFCH labels on the NR PRR figures), leading to more collisions in these slots and thus to an increased number of retransmissions (see NTrPerTB labels on the NR PRR figures); part of this NR extra traffic will end up in the slots without PSFCH

- the proportion of LTE traffic on resources in PSFCH slots decreases (see InPSFCH labels on the NR LTE figures); the LTE traffic is pushed into the slots without PSFCH

NR traffic is highly increased in slots with PSFCH, with a detrimental effect on NR. Some extra NR traffic (due to more retransmissions) and some extra LTE traffic (pushed out of the PSFCH slots) end up in slots without PSFCH, degrading LTE PRR also. The detrimental effect on LTE caused by the increased NR traffic (more re-transmissions) and increased LTE traffic (pushed out of the PSFCH slots) is superior to the beneficial effect due to less NR/LTE collisions in the PSFCH slots. Alt 1 (Δ=0) performs better.

| Figure 11 – Alt 4: LTE PRR, NR with ACK/NACK groupcast option 2, no PSFCH avoidance measure | Figure 12 – Alt 4: NR PRR, NR with ACK/NACK groupcast option 2, no PSFCH avoidance measure |
| --- | --- |

Observation 4: For NR SL resource (re)selection procedure, when HARQ groupcast option 2 is enabled, Alt 1 outperforms Alt 4.

Proposal 1: For NR SL resource (re)selection procedure, support Alt 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE based on the NR SL RSRP threshold list (pre)configured in the NR SL resource pool.

## On remaining PSFCH issues

Concerning the AGC issues caused by NR PSFCH transmissions occurring in resources that overlap with resources to be used for LTE SL transmissions, the following decision was made during the plenary RAN#99 and introduced in the WID:

For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered.

Furthermore, the cited working assumption reads:

Working assumption - […] - Option 1-2: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is ensured by the UE transmitting PSSCH - FFS whether/how to define condition(s) of selecting NR SL candidate resources for NR SL PSCCH/PSSCH - […]

It is unclear whether the FFS point in the working assumption still stands in the light of the latest plenary decision. Nevertheless, it is straightforward to understand that whenever the LTE traffic is at least medium, the probability of having at least one LTE transmission in the slot containing PSFCH occasions is close to 1. Henceforth, the candidate resource set for NR transmission is often NULL if avoidance is always applied. When LTE traffic is very light the collision probability is small anyhow and there is no gain margin to be closed.

Observation 5: For PSFCH issue, Option 1-2 needs to apply resource exclusion under limiting conditions only (e.g. exclude only under some RSRP/RSSI and/or priority conditions) in order to avoid excessive resource exclusion and NR resource starving.

On RSRP-based conditions for Option 1-2

The tradeoffs that can be achieved when applying Alt.1 under conditions largely depend on the condition. In subsection 2.2.1 we investigated RSRP-based conditions.

We define a new RSRP threshold RSRPThPSFCH, associated with PSFCH occasions, which is ΔdB away from the RSRP threshold used in the PSCCH/PSSCH resource allocation procedure for candidate exclusion, RSRPTh (there is one such value per subchannel, and values may depend on e.g. priority)

RSRPThPSFCH= RSRPTh+∆(dB)

The PSFCH collision mitigation decision at the transmitter/receiver is based on this value. We proceed in the following manner. For each subchannel i in the set S of subchannels with LTE detection (SCI correctly decoded) in the PSFCH occasion, we compute the RSRP associated to the subchannel, RSRPiPSFCH, and then we determine the index of the subchannel with the most significant value:

i =argmaxi∈SRSRPiPSFCH-RSRPTh, iPSFCH

- For Alt.1-1, if RSRPiPSFCH≥RSRPTh, iPSFCH PSFCH is dropped

- For Alt.1-2, the resource allocation procedure for PSSCH/PSFCH considers, for candidate resource exclusion, both the RSRP measured on PSSCH resources and the one measured on PSFCH resources, that is:

  - If RSRPiPSFCH-RSRPTh, iPSFCH≤RSRP-RSRPTh, resource allocation is based on RSRP and RSRPTh (Rel.16 procedure)

  - If RSRPiPSFCH-RSRPTh, iPSFCH>RSRP-RSRPTh, on RSRP and RSRPTh in Rel.16 resource allocation procedure are replaced by RSRPiPSFCH and RSRPTh, iPSFCH (the RSRP situation on PSFCH resources takes priority in the exclusion process).

In both flavors of Alt.1, the value ∆(dB) controls how much LTE presence in a PSFCH occasion weights in the resource allocation process:

- If ∆→∞, LTE presence is ignored (no avoidance, NR UE never avoids collision)

- If ∆→-∞, LTE presence takes precedence (NR UE always avoids collision)

### Simulation results: PRR performance of Option 1-2 under different RSRP conditions

As explained here-above PSFCH collision is avoided based on RSRP criteria and the value ∆(dB) controls how much the LTE presence in a PSFCH slot weights in the resource allocation process and/or in the PSFCH dropping decision:

- If ∆→∞, LTE presence is ignored (no avoidance, NR UE never avoids collision)

- If ∆→-∞, LTE presence takes precedence (NR UE always avoids collision)

- Other values of ∆ perform a tradeoff between sensitivity to LTE presence and impact on NR

As anticipated, when ∆→-∞, NR is starved and ends up with NULL candidate set. In this case, to avoid systematically dropping all NR transmissions we pick one excluded resource to perform one NR transmission (the one with the less defavourable RSRP from LTE perspective). Of course this doesn’t help much: NR performance is extremely degraded, and only marginal gain is observed on LTE side. Such functioning point has no interest from system perspective.

Intermediate values for ∆ were also tested. In order to avoid NR starvation and the extreme degradation observed in the case  ∆→-∞, we also re-integrate excluded resources to obtain a critical mass of 20% candidate resources: Option 1-2 is thus partially overridden in the re-integration process.

It appears from the simulation results in the following figures that:

- Strict resource exclusion degrades NR performance in an extreme manner, without having significant impact on LTE

- Unexpectedly, excessive resource exclusion also degrades LTE performance. Although AGC problems are avoided for LTE, this strategy degrades the quality of the resource used by the NR and also has an effect of concentrating NR transmissions onto resources that it would not have naturally picked. This leads to an increased number of NR retransmissions: the average number of re(transmissions) increases. This causes more collisions at PSSCH level, and the PRR performance of LTE is also degraded.

- Loose exclusion at TX does not procure enough gain for LTE, who performs roughly as in the absence of Option 1-2.

| Figure 13: PSFCH Option 1-2 + PSSCH Alt 1: LTE PRR | Figure 14: PSFCH Option 1-2 + PSSCH Alt 1: NR PRR |
| --- | --- |

We equally tested whether some combination between PSFCH Option 1-2 and PSSCH Alt 4 might be beneficial. Despite the amount of different tested threshold combination (red curves family) there is no outstanding configuration to outperform Alt 1. This is pretty intuitive, since Alt 4 (PSSCH resource selection) and Option 1-2 (PSFCH avoidance by Tx) push in different directions: the former pushes away LTE from PSFCH slots by enhancing collisions with NR in that specific slot, while the latter avoids collisions PSFCH collisions in the same slots by preventive resource allocation of the PSSCH resource. These antagonist effects have no merit of being combined.

| Figure 15: PSFCH Option 1-2 + PSSCH Alt 4: LTE PRR | Figure 16: PSFCH Option 1-2 + PSSCH Alt 4: NR PRR |
| --- | --- |

Observation 6: There is no merit in combining Alt 4 (for PSSCH resource selection) with Option 1-2 (for PSFCH mitigation).

Observation 7: Applying Option 1-2 in a strict manner without any condition severely degrades NR performance, without any outstanding benefit on LTE.

Observation 8: Conditional PSFCH mitigation under Option 1-2 can limit the penalty suffered by NR while trying to protect LTE.

Observation 9: For conditional PSFCH mitigation under Option 1-2, optimal conditions may vary depending on the dynamic traffic changes.

Proposal 2: For PSFCH avoidance with Option 1-2, conditionally exclude NR resource only if the feedback slot has LTE transmission with RSRP over a configurable threshold.

# Conclusions

The following proposals stand:

Proposal 1: For NR SL resource (re)selection procedure, support Alt 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE based on the NR SL RSRP threshold list (pre)configured in the NR SL resource pool.

Proposal 2: For PSFCH avoidance with Option 1-2, conditionally exclude NR resource only if the feedback slot has LTE transmission with RSRP over a configurable threshold.

As a useful reminder, the full set of observations is summarized here-below:

Observation 1: For NR SL resource (re)selection procedure, when HARQ is disabled, Alt 3 procures marginal gain for LTE while severely degrading the NR performance with respect to Alt 1.

Observation 2: For NR SL resource (re)selection procedure, when HARQ groupcast option 2 is enabled, Alt 1 outperforms Alt 3.

Observation 3: For NR SL resource (re)selection procedure, Alt 1 is preferred to Alt 2 and Alt 3.

Observation 4: For NR SL resource (re)selection procedure, when HARQ groupcast option 2 is enabled, Alt 1 outperforms Alt 4.

Observation 5: For PSFCH issue, Option 1-2 needs to apply resource exclusion under limiting conditions only (e.g. exclude only under some RSRP/RSSI and/or priority conditions) in order to avoid excessive resource exclusion and NR resource starving.

Observation 6: There is no merit in combining Alt 4 (for PSSCH resource selection) with Option 1-2 (for PSFCH mitigation).

Observation 7: Applying Option 1-2 in a strict manner without any condition severely degrades NR performance, without any outstanding benefit on LTE.

Observation 8: Conditional PSFCH mitigation under Option 1-2 can limit the penalty suffered by NR while trying to protect LTE.

Observation 9: For conditional PSFCH mitigation under Option 1-2, optimal conditions may vary depending on the dynamic traffic changes.

# Annex: Simulation parameters

The following simulation parameters were used in the simulations

| Sidelink Frequency | 6 GHz |
| --- | --- |
| BW | 20MHz |
| SCS | 15kHz |
| Number of subchannels | 4 |
| Channel | LOS/NLOSv (from TR 37.885) with 23dBm transmit power |
| Antenna gain | Gt=Gr=3dB |
| UE drop and mobility modeling | Highway, 6 lanes with wraparound Option A with 140 km/h and T=4 (37.375 devices per km) |
| Device type | R18-Type A (dual NR+LTE RAT with NR SPS based also of LTE RAT sensing) R14/15-Type C (mono RAT LTE) |
| Traffic model | Periodic, RRI={50, 100}ms Priority=1 Latency = RRI LTE: 2 blind transmissions NR: Up to 4 non-blind transmissions, from 1 to 4 blind transmissions Probability of reselection = 0.2. Packet size: - NR: @RRI=50ms: 300/190 bytes - LTE: @RRI=100ms: 300/190 bytes |
| Communication Mode (NR) | Groupcast option 1 (NACK only) or groupcast option 2 (ACK/NACK). |
| PSFCH slots properties | Minimum response latency K=2 and period N=4. |
| Recipient set definition | The recipients are all other NR RAT at a distance from the sender smaller than 500 m for small packet size (300/190 bytes). |
| MCS | 1 TB/slot/subchannel: - 16QAM 5/6 with 1200 bytes - 16QAM 3/4 with 800 bytes - QPSK 1/2 for 300 bytes - QPSK1/3 for 190 bytes. |
| RAT number and density | 37.5 active RATs per km, i.e. 12.5 NR RAT/km and 24.9 LTE RAT/km). Global channel load: 498 different TB per s and per km in average. (Medium traffic density LTE RAT in dual RAT does not transmit). |

# References

- RP-220300, WID revision: NR sidelink evolution, 3GPP TSG RAN Meeting #95e.

### RWS-210360, 5GAA input to 3GPP Rel.18 Workshop, 5GAA.

### RWS-21006, On NR-LTE V2X Co-Channel Coexistence, Qualcomm.

### RWS-210244, Enhancements for sidelink operations, LG Electronics.

### RWS-210445, NR sidelink and V2X enhancements, Huawei, HiSilicon.

### Chairman’s notes, RAN1#110, August 2022, Toulouse, France.

### Chairman’s notes, RAN#97-e, electronic meeting.

### RP-222369, Discussion on Co-Channel Coexistence for LTE Sidelink and NR Sidelink, Continental Automotive GmbH, Toyota InfoTechnology Center, Robert Bosch GmbH, Mitsubishi, Volkswagen AG, Deutsche Telekom, Fraunhofer HHI, Fraunhofer IIS/ RAN#97-e, electronic meeting.

### R1-2210298, FL Summary EoM for AI 9.4.2 - Co-Channel Coexistence for LTE and NR Sidelink, RAN1#110b-e.

### R1-2212540, FL Summary EoM for AI 9.4.2 - Co-Channel Coexistence for LTE and NR Sidelink, RAN1#110b-e.

### Chairman’s notes, RAN1#112, February 2023, Athens, Greece.