3GPP TSG-RAN WG1 Meeting #112bis-e	Tdoc R1-2303851

E-meeting, April 17th- April 26th, 2023

Agenda Item:	9.16

Source:	Ericsson, Verizon, Qualcomm

Title:	Proposal for Rel-18 TEI

Document for:	Discussion, Decision

# Introduction

We have proposed a TEI related to multi-PxSCH scheduling with single DCI to the last meeting [1]. In this contribution, we try to address the comments raised during the last meeting [2] to further motivate the request to endorse the proposed.

# TEI-18 related to multi-PxSCH scheduling with single DCI

In Rel-16, the feature multi-PUSCH scheduling was introduced whereby a single DCI 0_1 can schedule up to 8 PUSCHs. The feature is agnostic to subcarrier spacing, frequency range, and whether or not shared spectrum access is required for the frequency band. The only restriction in Rel-16 specifications is that the multiple scheduled PUSCHs occur in contiguous slots. The corresponding Rel-16 UE feature parameter is as follows (see [3]

| multiPUSCH-UL-grant-r16 Indicates whether the UE supports scheduling up to 8 PUSCH with a single DCI 0_1. This capability is also applicable to a frequency band that does not require shared spectrum access. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

In Rel-17, multi-PUSCH scheduling was extended to support non-contiguous slots in addition to contiguous slots. In contrast this feature is not agnostic to frequency range; it is limited to FR2 which includes sub-ranges FR2-1 (up to 52.6 GHz) and the newly introduced FR2-2 (up to 71 GHz). For FR2-1 the UE feature is limited to 120 kHz only. The corresponding Rel-17 UE feature parameter for FR2-1 is given is as follows (see [4]):

| multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 Indicates whether the UE supports multi-PUSCH scheduling by single DCI for the operation with 120kHz SCS in FR2-1 with non-contiguous allocation. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

In our view, it would be beneficial to extend multi-PUSCH scheduling with non-contiguous slots also to FR1. The extension is important for XR uses cases. The XR video traffic in UL includes variable and large packets, arriving in bursts. Therefore, dynamic scheduling is a reasonable approach for data transmission. Considering the large size of the packets, multiple PUSCHs are needed to be scheduled for sending the data. Hence, scheduling these PUSCHs with a single DCI has clear benefit and advantage, over scheduling each PUSCH with a single DCI. A limitation to contiguous slots would be problematic in case of transmission over TDD bands. We note there is no RAN1 specification impact from this; it is only a matter of introducing an additional UE capability. With this capability, multi-PUSCH with either contiguous or non-contiguous slots would then be available in all frequency ranges and all subcarrier spacings defined so far.

Similar to multi-PUSCH scheduling, multi-PDSCH scheduling with single DCI 1_1 for both contiguous/non-contiguous slots was specified in Rel-17. Similarly, this feature was limited to FR2 including both sub-ranges FR2-1 and FR2-2, and for FR2-1 it is limited to 120 kHz only. The corresponding Rel-17 UE feature parameter for FR2-1 is given is as follows (see [4]):

| multiPDSCH-SingleDCI-FR2-1-SCS-120kHz-r17 Indicates whether the UE supports multi-PDSCH scheduling by single DCI for the operation with 120kHz SCS in FR2-1 and HARQ enhancements for both type 1 and type 2 HARQ codebook. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

In our view it is beneficial to also extend multi-PDSCH scheduling for contiguous/non-contiguous slots to FR1. Similarly, to UL for the case of multi-PUSCH, this extension is important for XR uses cases to serve XR video traffic in DL which includes variable and large packets, arriving in bursts.  Again, there is no specification impact from this; it is only a matter of introducing an additional UE capability. With this capability, multi-PDSCH with either contiguous or non-contiguous slots would then be available for all frequency ranges and all subcarrier spacings defined so far.

With respect to the comments raised in the previous meeting, we would like to provide the following clarifications:

The features under discussion here, were developed during the WI for FR2-2. There is no specification impact to enable these features for other frequency ranges. Whether the features would be implemented or not, depends on the market need and vendors. When it comes to deployment scenarios, large variations are often experienced as compared to the scenarios used for the purpose of the design of the feature.

For the argument that this proposal was previously discussed and not agreed, we would like to understand the fundamental reasons for not enabling these features when there is no specification impact.

For the argument that the performance benefit should be shown to relax the restriction, we question that why to start with restriction and limitation of use cases?

Consider this example: XR traffic is to be served using a TDD band in FR1. Hence, it is sensible if PUSCHs with different SLIVs are scheduled in different UL slots. A NW vendor see the benefit to use a single DCI for this purpose for various reasons. But it is simply not possible because of FR1. Was it a correct or necessary decision to deny this possibility?

Moreover, we have experienced cases that a feature is being restricted in a release and next release the restriction is removed, consuming a lot of efforts and time.

Therefore, we make the following observations and proposals. We encourage companies to consider the benefits of lifting the current restriction thus contributing to providing enough flexibility and freedom for the market to decide when and what features to use.

Artificial restrictions to use the features developed by 3GPP are not beneficial.

The market decides when and which features to use.

A TEI-18 is supported to introduce the UE feature for multi-PUSCH scheduling with single DCI 0_1 for non-contiguous slots in FR1 for all defined SCSs. Note: there is no RAN1 impact.

Introduce the UE feature for multi-PDSCH scheduling with single DCI 1_1 for contiguous/non-contiguous slots in FR1 for all defined SCSs in the same TEI-18 as for multi-PUSCH with non-contiguous slots for FR1. Note: there is no RAN1 impact.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Artificial restrictions to use the features developed by 3GPP are not beneficial.

Observation 2	The market decides when and which features to use.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A TEI-18 is supported to introduce the UE feature for multi-PUSCH scheduling with single DCI 0_1 for non-contiguous slots in FR1 for all defined SCSs. Note: there is no RAN1 impact.

Proposal 2	Introduce the UE feature for multi-PDSCH scheduling with single DCI 1_1 for contiguous/non-contiguous slots in FR1 for all defined SCSs in the same TEI-18 as for multi-PUSCH with non-contiguous slots for FR1. Note: there is no RAN1 impact.

# References

R1-2300155	Proposal for Rel-18 TEI, Ericsson

R1-2302103		Summary #3 on Rel-18 TEIs; Moderator (NTT DOCOMO, INC.)

3GPP TS 38.306, “User Equipment (UE) radio access capabilities (Release 16),” v.16.11.0, December 2022.

3GPP TS 38.306, “User Equipment (UE) radio access capabilities (Release 17),” v.17.3.0, December 2022.