3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303857

e-Meeting, 17 – 26 April, 2023

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on RAN2 LS on Comparison of SL-RSRP and SD-RSRP measurements

Document for:	Discussion and Decision

- Introduction

A RAN2 LS was sent to RAN1 [1] regarding measurement report trigger within NR_SL_relay_enh work item, with details:

(below is copied from RAN2 LS R2-2302234) … Event Z2: The measured RSRP of a candidate L2 U2N Relay UE becomes an offset better than measured RSRP of the serving L2 U2N Relay UE This new event requires either the comparison of SD-RSRP (RSRP measurement on broadcast PC5 link used for discovery messages) measurements from different UEs, or the comparison of SL-RSRP (RSRP measurement on unicast PC5 link) and SD-RSRP measurements. During the discussion some companies expressed concerns on direct comparison of SD-RSRP and SL-RSRP due to the power control mechanism applied on PC5 unicast links, while other companies think the network can handle such comparison by using an appropriate offset in the measurement event. Before agreeing to the introduction of this measurement report trigger, RAN2 would like to ask feedback from RAN1 and RAN4 on the comparisons of RSRP measurements:

RAN1 is requested to provide the feedback on question below:

- Q1: Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report?

- Discussions

As shown in Figure 1, SD-RSRP is RSRP measurement on broadcast PC5 link used for discovery message, and SL-RSRP is RSRP measurement on unicast PC5 link used for communication message. The power control for transmitting discovery message and communication message have some slight difference. It is possible to compare SL-RSRP and SD-RSRP measurement if the power difference can be addressed by RAN2.

Figure 1 SL-RSRP vs SD-RSRP performed at UE-A

Regarding the power control for PSSCH transmission, as specified in TS 38.213, there are three main factors that may result in power difference between PSSCH transmissions carrying discovery and communication message:

(copied from TS 38.213) A UE determines a power PPSSCH(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f as: PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PMAX,CBR is determined by a value of sl-MaxTxPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTxPower is not provided, then PMAX,CBR=PCMAX; --- Other parts are omitted --- - if sl-P0-PSSCH-PSCCH is provided and if a SCI format scheduling the PSSCH transmission includes a cast type indicator field indicating unicast or is SCI format 2-C - PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm] - else - PPSSCH,SL(i)=minPCMAX,PPSSCH,D(i) [dBm] where - PO,SL is a value of sl-P0-PSSCH-PSCCH-r17, if using the parameter is supported by the UE and the parameter is provided; else sl-P0-PSSCH-PSCCH-r16 if provided - αSL is a value of sl-Alpha-PSSCH-PSCCH, if provided; else, αSL=1 --- Other parts are omitted ---

1) PMAX,CBR is subject to congestion control and priority. For PSSCH transmission carrying discovery message, the priority is fixed as 1 as per TS 38.331. For PSSCH transmission carrying communication message, the priority can be any value ranges from [1,8]. Thus with different priority, even same sl-MaxTxPower is (pre-)configured, SL-RSRP and SD-RSRP may differ.

2) PO,SL and αSL, are configured per resource pool, and resource pools for communication and discovery are separately (pre-)configured. SL-RSRP and SD-RSRP may differ if these values are different.

3) PSSCH transmission carrying communication message of which cast type is unicast, if sl-P0-PSSCH-PSCCH is provided, the transmission power will be subject to SL pathloss (PLSL). However, PSSCH transmission carrying discovery message is based on broadcast, its transmission power is not subject to SL pathloss.

In summary, it would be possible to compare SL-RSRP and SD-RSRP, when the difference between PSSCH transmission power carrying communication message and discovery message can be addressed. How such difference can be compensated is up to RAN2 design if RAN2 decides to support SL-RSRP and SD-RSRP comparison. So we propose the following reply to RAN2:

Proposal 1:  Reply to RAN2 as follows:

- The comparison of SL-RSRP and SD-RSRP measurement can be used for the purposes of triggering a measurement report, if RAN2 can address the difference between transmission power for PSSCH carrying communication and discovery messages due to different priority, PO,SL , αSL , and subject to PLSL or not.

- Conclusions

SL-RSRP and SD-RSRP may differ given the different transmission power for PSSCH carrying communication and discovery messages, due to possible different PMAX,CBR, PO,SL , αSL and subject to PLSL or not. If RAN2 can address such difference, it’s possible that the comparison of SL-RSRP and SD-RSRP measurement can be used for the purposes of triggering a measurement report.

Proposal 1:  Reply to RAN2 as follows:

- The comparison of SL-RSRP and SD-RSRP measurement can be used for the purposes of triggering a measurement report, if RAN2 can address the difference between transmission power for PSSCH carrying communication and discovery messages due to different priority, PO,SL , αSL, and subject to PLSL or not.

# References

- R2-2302234, “LS on Comparison of SL-RSRP and SD-RSRP measurements”, RAN2, RAN2#121, March, 2023.