3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303858

E-Meeting, 17 – 26 April, 2023

Agenda Item:	9.18

Source:	Huawei, HiSilicon

Title:	Remaining issues for eDSS, NCR, multicarrier enhancements, and BWP without restriction

Document for:	Discussion and Decision

Introduction

In this contribution, firstly, a draft TP on UL Tx switching across 3 or 4 bands in TS 38.214 [1] is proposed. Besides, a draft TP on eDSS in TS 38.213 is proposed. Finally, the remaining issues for NCR are discussed.

Discussion on TP for UL Tx switching across 3 or 4 bands

In our view, RAN1 specify UL Tx switching from following aspects in Rel-16 and Rel-17,

- The value of switching gap

- The determination of switching gap

- The scheduling restriction

- The determination of transmission state for Tx state ambiguity issue.

In Rel-18 UL Tx switching, the agreements have been achieved in RAN1, RAN2 and RAN4 which can be found in Appendix 1 have RAN1 specification impacts. It is worth noting that UL Tx switching across 3 or 4 bands for inter-band CA with SUL bands is supported, which should be specified in 6.1.6.2 in TS 38.214. Therefore, the following TP#1 for TS 38.214 should be adopted.

----------------------------------------------- TP#1 start------------------------------------------------ ==== Unchanged parts ==== ### 6.1.6 Uplink switching The UE may omit uplink transmission during the uplink switching gap NTx1-Tx2 if the conditions defined in this clause are met and the UE is configured with uplinkTxSwitching. The switching gap NTx1-Tx2 is indicated by UE capability uplinkTxSwitchingPeriod2T2T if uplinkTxSwitching-2T-Mode is configured, and uplinkTxSwitchingPeriod otherwise: - If a UE indicated a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination - Configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), or - Configured with uplink carrier aggregation, or - Configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink. the conditions under which the switching gap may be present and the location of the switching gap are defined for each of the cases in clauses 6.1.6.1, 6.1.6.2, and 6.1.6.3 respectively. If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213]. For a UE configured with uplink switching with parameter uplinkTxSwitching, - If the UE is configured with uplink carriers on more than two bands, - The UE does not expect to perform more than one uplink switching in a reference slot with µUL, where the µUL corresponds to the maximum subcarrier spacing of the active UL BWPs of all the configured uplink carriers. - If two uplink switching are triggered and UL transmissions involved in the determination of the two uplink switching are on more than two bands within any two consecutive reference slots, then the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time where the minimum separation time is a maximum of time indicated by UE capability [RRC_X_us] and the switching gap required for the second uplink switching. - Otherwise, the UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL, 1, µUL, 2), where the µUL, 1 corresponds to the subcarrier spacing of the active UL BWP of one uplink carrier before the switching gap and the µUL, 2 corresponds to the subcarrier spacing of the active UL BWP of the other uplink carrier after the switching gap. ==== Unchanged parts ==== #### 6.1.6.2 Uplink switching for carrier aggregation For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch or uplinkTxSwitchingPeriod2T2T for a band combination, and if it is for that band combination configured with uplink carrier aggregation: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): - When the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - When the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - For the UE configured with uplinkTxSwitchingOption set to 'switchedUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on a carrier on the same band and the UE is under the operation state in which 2-port transmission cannot be supported in the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on another band and the UE is under the operation state in which 2-port transmission can be supported in the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', if the UE is configured with uplinkTxSwitching-DualUL-TxState set to 'oneT', when the UE is under the operation state in which 2-port transmission can be supported on one carrier on one band followed by no transmission on any carrier on the same band and 1-port transmission on the other carrier on another band the UE shall consider this as if 1-port transmission was transmitted on both uplinks, otherwise the UE shall consider this as if 2-port transmission took place on the transmitting carrier. - If uplinkTxSwitching-2T-Mode is configured, when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier on one band, and any transmission on another uplink carrier on another band. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. ##### 6.1.6.2.1 void ##### 6.1.6.2.2 Uplink switching for more than two configured uplink bands For a UE configured with uplink carriers on more than two bands, if no more than two bands are involved in the determination of one uplink switching, then the procedures in sub-clause 6.1.6.2 and 6.1.6.3 are applied where the higher layer parameter uplinkTxSwitchingOption within the procedures, if any, should be replaced with [TBD uplinkTxSwitchingOption-r18] that is configured on the two bands involved in the determination of the uplink switching. For a UE configured with uplink carriers on more than two bands, if more than two bands are involved in the determination of one uplink switching and if on any two of the bands the UE is configured with [TBD uplinkTxSwitchingOption-r18] set to 'dualUL', - A effective time duration NTx1-Tx2e is the maximum of NTx1-Tx2 for all band pairs of the band for a preceding transmission and the band for the transmission triggering the uplink switching. - When the UE is to transmit a 1-port or 2-port transmission on one uplink carrier on one band and if the preceding uplink transmissions were a 1-port transmission on a carrier on a second band and a 1-port transmission on a carrier on a third band, then the UE is not expected to transmit for the duration of NTx1-Tx2e on any of the carriers. - When the UE is to transmit two 1-port transmissions on two uplink carriers on two bands, respectively, and if one of the preceding uplink transmissions was a 1-port or 2-port transmission on a carrier on a third band, then the UE is not expected to transmit for the duration of NTx1-Tx2e on any of the carriers. - When the UE is to transmit 1-port transmission on one uplink carrier on one band and no transmission on any uplink carrier on any other band, if the preceding uplink transmissions were a 1-port transmission on a carrier on a second band and a 1-port transmission on a carrier on a third band, - If the UE is not configured with [TBD uplinkTxSwitchingOption-r18] set to 'dualUL' on between any band and the first band, then the UE is not expected to be configured with higher layer parameter [uplinkTxSwitching-DualUL-associatedband] and the UE shall consider this transmission as if 2-port transmission took place on the transmitting carrier when an uplink switching is triggered for any of its succeeding transmission. - Otherwise, if the UE is configured with uplinkTxSwitching-DualUL-TxState set to 'oneT' the UE shall consider this as if 1-port transmission was transmitted on both the uplink carrier of the first band and an uplink carrier of a band that is associated with the first band by higher layer parameter [uplinkTxSwitching-DualUL-associatedband], otherwise the UE shall consider this as if 2-port transmission took place on the transmitting carrier. #### 6.1.6.3 Uplink switching for supplementary uplink For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, - If the UE is to transmit any uplink channel or signal on a different uplink on a different band from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of the two uplinks. ## - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. ==== Unchanged parts ==== ----------------------------------------------- TP end ------------------------------------------------

Proposal 1: Adopt the above TP#1 on UL Tx switching across 3 or 4 bands in TS 38.214.

Discussion on TP for eDSS

An RRC configuration is needed for configuring the feature of receiving PDCCH on the symbol of CRS to UE, after the corresponding UE capability is reported. Otherwise, if such RRC configuration is not introduced, and the gNB does not support transmitting NR PDCCH in symbols with LTE CRS REs, the UE has to receive NR PDCCH in symbols with LTE CRS REs by default after reporting the capability; as a result, the false alarm probability of PDCCH will increase. So, if a UE reports that it supports the UE capability for NR PDCCH reception in symbols with LTE CRS REs, it should not start the related NR PDCCH reception immediately, but should wait for the corresponding RRC configuration to enable this reception.

Based on the above discussion, we prepared the following text proposal.

Proposal 2: Adopt the TP on eDSS for clause 10 of TS 38.213.

10 UE procedure for receiving control information < Unchanged parts are omitted > If a UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in clause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell. If - a UE is not provided [PDCCH-ReceptionWithCRS], and if at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList, or - a UE is provided [PDCCH-ReceptionWithCRS], and if at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList on each symbol of the PDCCH candidate, the UE is not required to monitor the PDCCH candidate. If a UE is provided availableRB-SetsPerCell, the UE is not required to monitor PDCCH candidates that overlap with any RB from RB sets that are indicated as unavailable for receptions by an available RB set indicator field in DCI format 2_0 as described in clause 11.1.1. If the UE does not obtain the available RB set indicator for a symbol, the UE monitors PDCCH candidates on all RB sets in the symbol. < Unchanged parts are omitted >

Remaining issues for NCR

Aperiodic beam indication

In the last meeting, the following agreements were made for the aperiodic indication:

Agreement For each aperiodic beam indication for access link via DCI, Tmax=Lmax is supported. - The time indication and beam indication is sequentially associated with one to one mapping. - The value of Tmax is RRC configurable Agreement For aperiodic beam indication, the bitwidth of each field for beam indication in DCI is explicitly configured by dedicated RRC. Agreement For the aperiodic beam indication, the reference of slot offset for each time resource is defined as the slot n+k where n refers to the slot that NCR-MT receive the DCI carrying the indication and: - Option-2: k refers to the offset value [defined by NCR-MT capability and/or declared by vendor]. - Note: This k is different from the parameter used to define the Slot offset for the time resource. Agreement For aperiodic beam indication, - The value range of slot offset of time resource is (0…14) - The maximum number of time resource is 112. - The maximum bitwidth of beam index field in DCI is 6.

There are several remaining issues: the maximum duration for each time domain resource, the maximum number of Tmax, reference of slot offset k (whether k is defined by NCR-MT capability and/or declared by vendor), the forwarding resource index indication, and the indication within a PDCCH monitoring periodicity.

Based on the agreed number of slot offsets, the required total number of time domain resources can be derived for different duration, as shown by Table 1. It can be observed that even for a duration of 14 OFDM symbols, the required total number of time domain resource is much larger than the agreed maximum value of 112.

Table 1 Required time domain resources for an aperiodic indication with different duration

| Duration (number of OFDM symbols) | Required number of time domain resources {slot offset, starting symbol, duration} for Nso slot offsets | Maximum required number of {slot offset, starting symbol, duration} for Nso=15 |
| --- | --- | --- |
| 14 (1 time slot) | 105×Nso | 1575 |
| 28 (2 time slots) | 406×Nso | 6090 |
| 56 (4 time slots) | 1596×Nso | 23940 |
| 112 (8 time slots) | 6328×Nso | 94920 |

Observation 1: For indicating Nso slot offsets, the required number of time domain resources increases with the square of the duration, which leads to a much larger value than the agreed 112 even at a duration of 14 OFDM symbols.

Proposal 3: For aperiodic beam indication, the maximum duration is 14 OFDM symbols within a time slot.

The above also implies that configuring a single time resource list for all time domain resources is not appropriate, i.e. a much longer list is required in order to support the indication of all possible time domain resources. Furthermore, configuring a single time resource list for aperiodic NCR forwarding control results in a significant increase in the required DCIs. For instance, if a UE is configured with 64 entries in PUSCH-TimeDomainResourceAllocation, a single aperiodic indication (i.e., one forwarding resource set and one DCI) can only indicate one PUSCH of a single UE. To support multiple aperiodic signals, such as CSI-RS, PDCCH, PDSCH, SRS, or PUCCH, or multiple UEs, at least six DCIs are needed for each UE. As a result, the total number of required DCIs for serving N UEs is greater than 6×N, which results in significant overhead for the gNB.

Observation 2: If only a single time resource list is configured for each aperiodic indication, the required DCIs of controlling NCR forwarding will be linear with the number of supported UEs and forwarded signals, which leads to high overhead for the gNB.

To reduce the DCI overhead and at the same time allowing indication flexibility, the following options are proposed.

- Option 1: multiple time resource lists of {slot offset, starting symbol, duration}, each list is with a maximum number of 112 time resources.

- Option 2: one list of {slot offset} with Tmax entries, and one list of {starting symbol, duration} with a maximum number of 112 time resources.

Among the two options, option 2 is simpler and of lower configuration overhead.

Observation 3: For each aperiodic beam indication, multiple time resource lists can be configured: option 1). RRC configures Tmax lists of {slot offset, starting symbol, duration}, each list is with up to 112 time resources, option 2). RRC configures a list of Tmax {slot offset} and a list of up to 105 {starting symbol, duration}. And option 2 is simpler and of lower configuration overhead.

Proposal 4: For each aperiodic beam indication, multiple time resource lists can be configured: RRC configures a list of Tmax {slot offset} and a list of up to 105 {starting symbol, duration}.

gNB may configure multiple sets of aperiodic forwarding configurations for NCR. For example, one aperiodic forwarding resource set is used for CSI-RS/SRS and one aperiodic forwarding resource set is used for PDSCH/PDCCH/PUSCH/PUCCH. Without the resource set information (e.g., resource set identity) in DCI, the NCR cannot determine the correct resource set.

Observation 4: Only the beam and time resource information in DCI is not enough for aperiodic beam indication since multiple aperiodic forwarding configurations may be indicated for different signals/channels.

Proposal 5: For each aperiodic beam indication, the resource set information should be indicated in DCI.

It was agreed that the value of Tmax is RRC configurable but the maximum value of Tmaxhas not been decided. Since that it has been agreed that a maximum of 15 slot offsets can be configured, it is proposed that maximum value of Tmax is 15. And the RRC configuration should guarantee that the total DCI payload is acceptable, e.g., not larger than 128 bits (i.e., the same maximum DCI payload size as SFI).

Proposal 6: The maximum value of Tmax  is 15 for aperiodic beam indication by DCI, i.e., Tmax≤15.

Since aperiodic indication is used to indicate the beam for NCR-Fwd, the reference of slot offset can follow the PDCCH based beam switching in NR, i.e., 14 or 28 OFDM symbols depending on capability.

Proposal 7: The reference of slot offset k for NCR aperiodic indications can be defined by NCR-MT capability, e.g., 14/28 OFDM symbols.

For aperiodic indication, the monitoring periodicity of PDCCH can be up to 2560 slots, and the indicated slots may be very large. However, the indicated beams / time resources for each DCI is quite limited, e.g., 16. Thus, much more DCIs are required and the PDCCH overhead will be increased greatly. To improve signaling efficiency, a DCI indication can repeat within the PDCCH monitoring periodicity until the next PDCCH monitoring occasion. An example is shown by Figure 1. A single DCI is used to indicate 2 beams in a time duration of 5 slots, and the indication repeats until the next PDCCH monitoring occasion.

Observation 5: The maximum number of indicated beams and slots in DCI is can be far less than the PDCCH monitoring periodicity (i.e., up to 2560 slots). Many DCIs are required and the PDCCH overhead will be increased greatly such as to cover the PDCCH monitoring periodicity.

Proposal 8: For each aperiodic beam indication by DCI, the indication is repeated within a PDCCH monitoring periodicity.

Figure 1 DCI indication repeats until the next PDCCH monitoring occasion

Priority rules for conflicted indications

In the last meeting, the following agreement was made for the priority :

Agreement A priority flag is introduced per list of periodic and semi-persistent indications. The flag gives priority to periodic and semi-persistent indications over aperiodic indications. Additionally, the following applies: - If there is conflict among beam indication from different type of indication, the order of priority is defined as: Aperiodic beam indication > semi-persistent beam indication > periodic beam indication. - No conflict is expected between periodic beam indications - No conflict is expected between semi-persistent indications - If there is conflict between two aperiodic indications, the latest indication is prioritized. Agreement For periodic and semi-persistent beam indication, - The maximum number of periodic beam indication is 32. - The maximum supported number of beam is 64 (to determine the value range of beam index). - The maximum number of duration of time resource is 112 symbols.

One remaining issue is how to handle the time domain resources when there is a confliction. An example is shown in Figure 2. In (a), there are three indications with  priority order: indication 1 > indication 3 > indication 3, the indication 1 is overlapped with indication 3 in slot 1, indication 2 is overlapped with indication 3 in slot 4. In such a case, one or both indications should be invalid over the overlapped symbols, and there are three possible behaviors shown in Figure 2 (b):

- 1): NCR-Fwd follows the prioritized indication over conflicted symbols.

- 2): NCR-Fwd follows the prioritized indication over conflicted forwarding resources.

- 3): NCR-Fwd follows the prioritized indication over conflicted slots.

In slot 1, the experienced channel of forwarded signals in {1st~2nd symbols and 7th~14th symbols} and in {3rd~6th symbols} are different at the receiver due to NCR beam switching, thus the forwarding cannot support the PDSCH of indication 3. Even worse, the forwarding may lead to interference due to different observed channels over the symbols. As a result, behavior 1 should be avoided. At slot 3, the signal forwarding for indication 2 is useful and could be valid. The whole forwarding resource is invalid in behavior 2, and it is inefficient in terms of signaling overhead. At slot 4, the 1st~2nd symbols cannot support a single PDSCH forwarding for indication 2, thus the forwarding should be invalid.

In summary, behavior 1 may lead to interference and behavior 2 is less efficient, while behavior 3 achieves good signaling efficiency and performance.

Observation 6: For conflicted forwarding resources of different indications, the forwarding of non-overlapped symbols for a lower priority indication in a time slot with overlapping may lead to interference.

Observation 7: For conflicted forwarding resources of different indications, the forwarding of non-overlapped time slot(s) for a lower priority indication in a time slot without overlapping is still useful.

Proposal 9: For conflicted forwarding resources of different indications, the indicated time in a slot by a lower priority indication is valid if there is no overlapping in the slot, otherwise it is invalid.

Figure 2 Indication conflicts and behaviors

Uplink multiplexing between C-link and backhaul link

The UL of C-link and backhaul link may be indicated simultaneously for an NCR. If an NCR only supports TDM, one signal should be discarded. However, discarding the NCR-Fwd UL forwarding of cell-specific signals should be avoided. For the C-link, the PRACH transmission should be kept since it is used for UL synchronization, and the other UL transmissions of C-link can be of lowest priority.

Observation 8: For an NCR supports UL of C-link and backhaul link in TDM, discarding the forwarding of the cell-specific signals may lead to coverage hole if simultaneous UL transmissions are indicated.

Proposal 10: For an NCR indicated with simultaneous UL of C-link and backhaul link, the priority rule should be: PRACH of C-link > UL of backhaul link indicated by periodic indication > UL of backhaul link indicated by semi-persistent / aperiodic indications > other UL transmissions of C-link.

If an NCR is capable of transmitting in C-link and backhaul link simultaneously, the total of C-link power PC-linki and backhaul link power PTx, Fwdi may be larger than the configured maximum power PTotali in a slot i, i.e., PC-linki+PTx, Fwdi>PTotali. In this case, one of the transmission powers should be reduced. Similar to NR, the reduction in power can be based on the signal priorities of C-link and backhaul link. Specifically, if the C-link transmission is with higher priority, the actual backhaul link power is set as P'Tx, Fwdi=PTotali-PC-linki, otherwise the actual C-link transmission power is set as P'C-linki=PTotali-PTx, Fwdi.

Observation 9: For an NCR supports simultaneous UL of C-link and backhaul link, the total of C-link power PC-linki and backhaul link power PTx, Fwdi may be larger than the configured maximum power PTotali in a slot i, i.e., PC-linki+PTx, Fwdi>PTotali, and power reduction is required.

Proposal 11:  For an NCR supports simultaneous UL of C-link and backhaul link, if the total of C-link power PC-linki and backhaul link power PTx, Fwdi is larger than the configured maximum power PTotali in a slot i, the transmission power is reduced as:

- if the C-link transmission is with higher priority, the actual backhaul link power is set as P'Tx, Fwdi=PTotali-PC-linki,

- otherwise the actual C-link transmission power is set as P'C-linki=PTotali-PTx, Fwdi.

Conclusion

According to the above discussions, we have the following observations and proposals:

Observation 1: For indicating Nso slot offsets, the required number of time domain resources increases with the square of the duration, which leads to a much larger value than the agreed 112 even at a duration of 14 OFDM symbols.

Observation 2: If only a single time resource list is configured for each aperiodic indication, the required DCIs of controlling NCR forwarding will be linear with the number of supported UEs and forwarded signals, which leads to high overhead for the gNB.

Observation 3: For each aperiodic beam indication, multiple time resource lists can be configured: option 1). RRC configures Tmax lists of {slot offset, starting symbol, duration}, each list is with up to 112 time resources, option 2). RRC configures a list of Tmax {slot offset} and a list of up to 105 {starting symbol, duration}. And option 2 is simpler and of lower configuration overhead.

Observation 4: Only the beam and time resource information in DCI is not enough for aperiodic beam indication since multiple aperiodic forwarding configurations may be indicated for different signals/channels.

Observation 5: The maximum number of indicated beams and slots in DCI is can be far less than the PDCCH monitoring periodicity (i.e., up to 2560 slots). Many DCIs are required and the PDCCH overhead will be increased greatly such as to cover the PDCCH monitoring periodicity.

Observation 6: For conflicted forwarding resources of different indications, the forwarding of non-overlapped symbols for a lower priority indication in a time slot with overlapping may lead to interference.

Observation 7: For conflicted forwarding resources of different indications, the forwarding of non-overlapped time slot(s) for a lower priority indication in a time slot without overlapping is still useful.

Observation 8: For an NCR supports UL of C-link and backhaul link in TDM, discarding the forwarding of the cell-specific signals may lead to coverage hole if simultaneous UL transmissions are indicated.

Observation 9: For an NCR supports simultaneous UL of C-link and backhaul link, the total of C-link power PC-linki and backhaul link power PTx, Fwdi may be larger than the configured maximum power PTotali in a slot i, i.e., PC-linki+PTx, Fwdi>PTotali, and power reduction is required.

Proposal 1: Adopt the above TP#1 on UL Tx switching across 3 or 4 bands in TS 38.214.

Proposal 2: Adopt the TP on eDSS for clause 10 of TS 38.213.

Proposal 3: For aperiodic beam indication, the maximum duration is 14 OFDM symbols within a time slot.

Proposal 4: For each aperiodic beam indication, multiple time resource lists can be configured: RRC configures a list of Tmax {slot offset} and a list of up to 105 {starting symbol, duration}.

Proposal 5: For each aperiodic beam indication, the resource set information should be indicated in DCI.

Proposal 6: The maximum value of Tmax  is 15 for aperiodic beam indication by DCI, i.e., Tmax≤15.

Proposal 7 The reference of slot offset k for NCR aperiodic indications can be defined by NCR-MT capability, e.g., 14/28 OFDM symbols.

Proposal 8: For each aperiodic beam indication by DCI, the indication is repeated within a PDCCH monitoring periodicity.

Proposal 9: For conflicted forwarding resources of different indications, the indicated time in a slot by a lower priority indication is valid if there is no overlapping in the slot, otherwise it is invalid.

Proposal 10: For an NCR indicated with simultaneous UL of C-link and backhaul link, the priority rule should be: PRACH of C-link > UL of backhaul link indicated by periodic indication > UL of backhaul link indicated by semi-persistent / aperiodic indications > other UL transmissions of C-link.

Proposal 11:  For an NCR supports simultaneous UL of C-link and backhaul link, if the total of C-link power PC-linki and backhaul link power PTx, Fwdi is larger than the configured maximum power PTotali in a slot i, the transmission power is reduced as:

- if the C-link transmission is with higher priority, the actual backhaul link power is set as P'Tx, Fwdi=PTotali-PC-linki,

- otherwise the actual C-link transmission power is set as P'C-linki=PTotali-PTx, Fwdi.

- References

TS 38.214: “NR; Physical layer procedures for control”.

- Appendix 1

RAN1 #111 Agreement Following new conditions are applicable to dual UL only (i.e., not applicable to switched UL) - When the UE is to transmit a 1-port or 2-port transmission on one uplink carrier on one band (1st band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (2nd and 3rd band) - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 2T on a carrier on another band (3rd band) - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on one of the bands and another different band (1st or 2nd band, and 3rd band) - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (3rd and 4th band) Agreement In Case#2 where two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, if oneT is indicated via uplinkTxSwitching-DualUL-TxState, one Tx chain is switched to band C and associated band for another Tx chain is determined by new RRC parameter which is down-selected from following alternatives. - An associated band is configured for each band so that another Tx chain is associated with the configured band (as associated band for the transmitting band) - E.g., associated band for each transmitting band is configured as {B for A}, {A for B}, {A for C} and {C for D}. - When 1 port transmission on band C is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band B is switched to band C while another Tx chain associated with band A remains unchanged (because band A is associated band for band C) - When 1 port transmission on band D is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band A (or B) is switched to band D while another Tx chain associated with band B (or A) is switched to band C (because band C is associated band for band D) If there is one band where concurrent transmission with any other band is not supported, NW does not configure an associated band for the band. In such case, even if oneT is configured, UE performs switching as twoT is configured when 1 port transmission on the band is scheduled Agreement Following restrictions are applied for Rel-18 UL Tx switching across 3 or 4 bands. - The UE does not expect to perform more than one uplink switching within a reference slot based on µUL = max(µUL, 1, µUL, 2, µUL, 3) in case of 3 bands, µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4) in case of 4 bands, where µUL, 1, µUL, 2, µUL, 3, µUL, 4 are SCSs of active UL bandwidth parts of the bands in the band combination - If there are two consecutive intra-band carriers in one band, µUL, 1 = max(µUL, 1-1, µUL, 1-2), where µUL, 1-1 and µUL, 1-2 are SCSs of active UL bandwidth parts of the carriers in the band RAN1 #112 Agreement Confirm the working assumption with following updates (working assumption) If two uplink switching are triggered and UL transmissions involved in the two uplink switching are on more than 2 bands within any two consecutive reference slots, then the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a maximum of X us and the switching gap required for the second uplink switching. - X us is subject to UE capability with a value set of {0us, 500us} RAN2 #121 configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination) For RRC configuration to clarify ambiguous Tx state, RAN2 should introduce an RRC configuration that associates a band to another band which the unused Tx chain is switched to when the switch is from concurrent transmission on two bands to 1 Tx transmission on another band. RAN4 #104e Agreement: ---------------------- RAN4 Answer to RAN1 Question #2 ----------------------------------- RAN4 Answer: RAN4 has discussed the UE assumption/behavior considering two cases: - Case 1: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on either band A or band B. - For Case 1, RAN4 agreed that neither of Tx chains is expected to be used for transmission during the switching period. - Case 2: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on a different band (named “band C”). - For Case 2, RAN4 agreed that, as baseline UE assumption, neither of Tx chains is expected to be used for transmission on band C during the switching period. RAN4 #104b Way forward: - When two Tx chains are switched between two different band pairs with different lengths of switching periods, as baseline UE assumption, neither of the two Tx chains is expected to be used for transmission during the larger one of the two switching periods. - Whether advanced optional UE ability will be considered is related to the discussion in Issue 1-2. RAN4 #105 Way forward: - For Rel-18 UL Tx switching among 4 bands, when switching from 1T+1T on band A and B to 1T+1T on band C and D is performed, it is not clear whether UE performs Tx switching {from band A to C + B to D} or {from band A to D + B to C}: - As baseline UE assumption, no need to resolve the ambiguity issue of the switching pattern for each Tx chain and determine the switching gap based on the worst case by default, i.e., neither of the two Tx chains is expected to be used for transmission during the maximum of the four switching periods, i.e., max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}. - Note: Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C are the switching periods reported by the UE for band pair A&C, B&D,A&D and B&C, respectively. RAN4 #106 When one of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), the other Tx chain is maintained on a different band (named “band C” or “band D” in the case of 4-band) and the number of Tx chain on band C or band D is unchanged due to the switching, RAN4 agreed the granularity of the optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged during UL switching as follows: - Per band (only for the band(s) in the band combination but not included in the pair of bands before and after switching) for each pair of bands before and after switching in each band combination.