3GPP TSG-RAN WG1 Meeting #112bis-e	 																													 R1-2303864

e-Meeting, April 17 – 26, 2023

Agenda Item:	9.17.14

Source:	Huawei, HiSilicon

Title:	UE features for eDSS

Document for:	Discussion and Decision

Introduction

This contribution discusses the UE features for eDSS based on the agreements in previous meeting.

Discussion on UE features for Rel-18 DSS

NR PDCCH reception in symbols with LTE CRS REs

The related agreements and conclusions of previous meeting are as follows [1]:

RAN1#110 Agreement Reception of NR PDCCH candidates that overlap with LTE CRS REs is supported by Rel18 UEs PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15 from UE side. Note: depends on UE capability Following options can be used for PDCCH-DMRS channel estimation - legacy CE assumption - RAN1 consider support this, if no RAN4 performance requirements are defined - CE on clean symbol(s) only (this channel estimation option does not apply for 1 symbol CORESET) Note: Restriction on the symbols and/or LTE CRS patterns applicable for above agreements can be considered during UE capability session. RAN1#110 Conclusion RAN1 understands above agreements applies to 15kHz SCS only.

Based on above agreements, we propose the following UE features.

Table 1 UE feature for NR PDCCH reception in symbols with LTE CRS REs

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XX. NR_DSS_enh | XX-1 | NR PDCCH reception in symbols with LTE CRS REs | Support of NR PDCCH reception a PDCCH candidate overlaps with at least one RE of LTE CRS REs - at least one symbol of the PDCCH candidate does not overlap with LTE CRS REs - the PDCCH candidate overlaps with LTE CRS REs from one configured LTE CRS pattern - the SCS of the PDCCH candidate is 15kHz |  | Yes | N/A |  | Per Band | N/A | FR1 only | N/A |  | Optional with capability signalling |

Proposal 1: Introduce new UE features XX-1 in Table 1 for NR PDCCH reception in symbols with LTE CRS REs in Rel-18 eDSS.

Two overlapping CRS rate matching patterns

The related agreements of previous meeting are as follows [1][2]:

RAN1#109-e Agreement - Introduce new UE capability(ies) for support of two overlapping LTE CRS patterns in Rel-18 DSS - NW can configure two LTE-CRS overlapping rate matching patterns without coresetPoolIndex only if the UE indicates support of the new capability(ies). - Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16. - Maximum number of LTE-CRS rate matching patterns for PDSCH supported by a UE (i.e., maxNumberPatterns-r16 and maxNumberNon-OverlapPatterns-r16) is kept unchanged. RAN1#110 Agreement The following TP for sub-clause 5.1.4.2 of TS 38.214 h20 is endorsed in principle. ## 5.1.4.2 PDSCH resource mapping with RE level granularity <omitted text> A UE may be configured with any of the following higher layer parameters: - REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - REs indicated by 'RateMatchPatternLTE-CRS' in lte-CRS-PatternList1-r16 or lte-CRS-PatternList3-r18 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - For the UE for broadcast reception, REs indicated by 'RateMatchPatternLTE-CRS' in PDSCH-Config-MCCH or PDSCH-Config-MCCH configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - Each RateMatchPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe. - If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to ‘0’, or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to ‘1’; - otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig. - If the UE is not configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet, and if the UE is configured by higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are declared as not available for PDSCH. - If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList3-r18 if the PDSCH is associated with coresetPoolIndex set to ‘0’, or the CRS pattern(s) in lte-CRS-PatternList4-r18 if PDSCH is associated with coresetPoolIndex set to ‘1’; - otherwise, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, in ServingCellConfig. <omitted text>.

Based on above agreements, we propose the following UE features. If the UE supports this feature and 16-2a-5 (Separate CRS rate matching), then the UE can rate match around lte-CRS-PatternList3-r18 for corset pool Index 0 and lte-CRS-PatternList4-r18 for corset pool index 1 separately. And if the UE supports this feature but does not support 16-2a-5, then the UE can rate match around both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18.

Table 2 UE feature for two LTE-CRS overlapping rate matching patterns

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XX. NR_DSS_enh | XX-2 | Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier | 1. Support of two LTE-CRS overlapping rate matching patterns (i.e. lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18) within a part of NR carrier using 15 kHz SCS overlapping with a LTE carrier | 14-1 | Yes | N/A |  | Per Band | N/A | FR1 only | N/A | Maximum number of LTE-CRS rate matching patterns for PDSCH supported by a UE (i.e., maxNumberPatterns-r16 and maxNumberNon-OverlapPatterns-r16) is kept unchanged. The UE can rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex if the UE also supports 16-2a-5. | Optional with capability signalling |

Proposal 2: Introduce new UE features XX-2 in Table 2 for two overlapping CRS rate matching patterns in Rel-18 eDSS.

Conclusion

According to the above discussions, we have the following proposals:

Proposal 1: Introduce new UE features XX-1 in Table 1 for NR PDCCH reception in symbols with LTE CRS REs in Rel-18 eDSS.

Proposal 2: Introduce new UE features XX-2 in Table 2 for two overlapping CRS rate matching patterns in Rel-18 eDSS.

References

- RAN1 Chairman’s notes, RAN1#110, August 22-26, 2022.

- RAN1 Chairman’s notes, RAN1#109-e, May 9-20, 2022.