3GPP TSG-RAN WG1 Meeting #112-bis-e	R1-2303869
Online, April 17th – 26th, 2023

Agenda Item:	7.2

Source:	Ericsson

Title:	Discussion on reference SCS for availability indication in eIAB

Document for:	Discussion, Decision

# Introduction

In this contribution, we address a remaining issue on the SCS used as reference for Rel-16 time-domain and Rel-17 time- and frequency-domain H/S/NA configuration.

# Discussion

## Configuration of Rel-16 and Rel-17 H/S/NA configuration

Table 1 shows an excerpt from TS 38.473 [1] about information regarding Rel-16 configuration of H/S/NA. As can be seen, the configuration is on slot level associating time resources with H/S/NA.

Table 1: Excerpt from 38.473, 9.3.1.107 gNB-DU Cell Resource Configuration, about Rel-16 HSNA configuration

| HSNA Slot Configuration List |  | 0..1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| >HSNA Slot Configuration Item |  | 1..<maxnoofHSNASlots> |  |  |  |  |
| >>HSNA Downlink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot. | - |  |
| >>HSNA Uplink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot. | - |  |
| >>HSNA Flexible | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot. | - |  |

Table 2 shows an excerpt from TS 38.473 [1] about information regarding Rel-17 configuration of H/S/NA. As can be seen, the configuration is on slot level and per RB set associating time and frequency resources with H/S/NA.

Table 2: Excerpt from 38.473, 9.3.1.107 gNB-DU Cell Resource Configuration, about Rel-17 HSNA configuration

| Frequency-Domain HSNA Configuration List |  | 0..1 |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >Frequency-Domain HSNA Configuration Item |  | 1..<maxnoofRBsetsPerCell> |  |  | EACH | reject |
| >>RB Set Index | M |  | INTEGER (0.. maxnoofRBsetsPerCell-1, …) | Refers to an RB set defined by RB Set Configuration. The RB set indices are consecutive (and increasing) starting at 0. | - |  |
| >>Frequency-Domain HSNA Slot Configuration List |  | 1 |  |  | - |  |
| >>>Frequency-Domain HSNA Slot Configuration Item |  | 1..<maxnoofHSNASlots> |  |  | - |  |
| >>>>Slot Index | O |  | INTEGER (0..5119) | Indicates an index to a slot within the HSNA Transmission Periodicity. | - |  |
| >>>>HSNA Downlink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot, for an RB set. | - |  |
| >>>>HSNA Uplink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot, for an RB set. | - |  |
| >>>>HSNA Flexible | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot, for an RB set. | - |  |

TS 38.213 tries to clarify that “The IAB-DU can assume a same SCS configuration for availabilityCombinations-r16 or availabilityCombinationsRBGroups-r17 for a cell as an SCS configuration provided by gNB-DU Cell Resource Configuration for the cell”. But this can only be true for the time-domain aspects (i.e., slots) of the Rel-16 and Rel-17 HSNA configuration(s), since the Rel-17 frequency-domain related configuration (i.e., RBs in an RB set) has its own reference SCS given by the IE RB Set Configuration, as shown in Table 3.

Table 3: Excerpt from 38.473, 9.3.1.230 RB Set Configuration, about Rel-17 RB set configuration

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the RB set configuration. |

We therefore propose to correct the use of the SCS configuration provided by gNB-DU Cell Resource Configuration as the only reference for availabilityCombinationsRBGroups-r17 by limiting the use as reference for its slot configuration.

In a complementing action, we also propose to clarify the use of the SCS configuration provided by IE RB Set Configuration, by including the following text into TS 38.213 [2]:

The IAB-DU can assume a same SCS configuration for availabilityCombinationsRB-Groups-r17 for RB sets of a cell as a SCS configuration provided by RB Set Configuration for the cell.

Adopt the text proposal in the associated CR R1-2303870, reflecting the above behaviour.

Include the following clarification into TS 38.213:
The IAB-DU can assume a same SCS configuration for availabilityCombinationsRB-Groups-r17 for RB sets of a cell as a SCS configuration provided by RB Set Configuration for the cell.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt the text proposal in the associated CR R1-2303870, reflecting the above behaviour.

Proposal 2	Include the following clarification into TS 38.213: The IAB-DU can assume a same SCS configuration for availabilityCombinationsRB-Groups-r17 for RB sets of a cell as a SCS configuration provided by RB Set Configuration for the cell.

# References

TS 38.473, “NG-RAN; F1 application protocol (F1AP)”, 3GPP, V17.3.0, December 2022

TS 38.213, “NR; Physical layer procedures for control”, 3GPP, V17.5.0, March 2023