3GPP TSG-RAN WG1 Meeting #112-bis-e	R1-2303872

Online, April 17th – April 26th, 2023

Agenda Item:	9.17.4

Source:	Ericsson

Title:	Legacy UE features for NCR

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss our view on required legacy NCR-MT capabilities considering the NCR deployment.

# Discussion

## NCR-MT’s capability

In this contribution, we discuss the RAN1 capabilities which are needed for the basic operation of the operator deployed Rel-18 NCR, i.e., those features without which the system cannot operate. Specifically, RAN1 should not modify existing UE functionalities that have been carefully developed and specified. As such, we think, a repeater node, similar to an IAB-node, is a stationary node that is planned and deployed by an operator. The planning can be expected to result in a stable control/backhaul link between the controlling gNB and the NCR. Hence, many UE features related to mobility and measurements are not needed for NCR-MTs whereas all features necessary for the NCR to connect to the network are. Additionally, a minimalistic feature set will allow for reduced development costs, essential for the commercial success of the NCR. In relation to independent IAB-nodes, an NCR is merely an extension of the gNB, implying that, if anything, the NCR feature set should be reduced instead of increased relative to the IAB set.

As a baseline, the NCR-MT can have the same mandatory features as an IAB-MT agreed in the RAN1#101-e meeting [1] and summarized in Table 1.

Agreement (RAN1#101-e) - Wide-area IAB-MTs support the following Rel. 15 layer-1 mandatory UE features (as defined in TR38.822) - Without capability - 0-1, 0-3, 0-4, 1-1 (only 1 preamble for component 1, component 2, component 3 except paging), 2-1, 2-5, 2-6, 2-12, 2-16, 2-16a, 2-32 (only components 1-4 and 7), 2-50 (only components 1,2), 2-52 (only components 1, 2), 3-1 (only components 1,2,3,4,5), 4-1, 5-1 (only components 1/2/3/4/5/6/7/9/10/12), 6-1, 7-1, 8-3 - With capability signaling which shall be set to '1' - 1-3, 2-22, 4-10 - The rest of Rel-15 layer-1 UE features other than the ones as listed above are optional for wide-area IAB-MTs. - Note: Mandatory MT capabilities are independent from DU capabilities and do not imply a corresponding mandatory DU capability. - The UE feature list for local-area IAB-MTs is FFS

Table 1: Mandatory features for IAB-MT.

| ### Features | Index | Feature group | Mandated components for IAB | Without capability | With capability set to 1 |
| --- | --- | --- | --- | --- | --- |
| 0. Waveform, modulation, subcarrier spacings, and CP | 0-1 | CP-OFDM waveform for DL and UL |  | Yes |  |
|  | 0-3 | DL modulation scheme |  | Yes |  |
|  | 0-4 | UL modulation scheme |  | Yes |  |
| 1. Initial access and mobility | 1-1 | Basic initial access channels and procedures | 1 preamble for component 1, component 2, component 3 except paging | Yes |  |
|  | 1-3 | SS block based RLM |  |  | Yes |
| 2. MIMO | 2-1 | Basic PDSCH reception |  | Yes |  |
|  | 2-5 | Basic downlink DMRS for scheduling type A |  | Yes |  |
|  | 2-6 | Basic downlink DMRS for scheduling type B |  | Yes |  |
|  | 2-12 | Basic PUSCH transmission |  | Yes |  |
|  | 2-16 | Basic uplink DMRS (uplink) for scheduling type A |  | Yes |  |
|  | 2-16a | Basic uplink DMRS for scheduling type B |  | Yes |  |
|  | 2-22 | Aperiodic beam report |  |  | Yes |
|  | 2-32 | Basic CSI feedback | component 1-4 and 7 | Yes |  |
|  | 2-50 | Basic TRS | components 1,2 | Yes |  |
|  | 2-52 | Basic SRS | components 1,2 | Yes |  |
| 3. DL control channel and procedure | 3-1 | Basic DL control channel | components 1,2,3,4,5 | Yes |  |
| 4. UL control channel and procedure | 4-1 | Basic UL control channel |  | Yes |  |
|  | 4-10 | Dynamic HARQ-ACK codebook |  |  | Yes |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation | components 1,2,3,4,5,6,7,9,10,12 | Yes |  |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction |  | Yes |  |
| 7. Channel coding | 7-1 | Channel coding |  | Yes |  |
| 8. UL TPC | 8-3 | Basic power control operation |  | Yes |  |

The deployment situation for an IAB-node and NCR is similar, hence the required features for IAB-MT and NCR-MT are similar.

Use the identified and agreed IAB-MT capabilities as a starting point for repeater-MT capabilities.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The deployment situation for an IAB-node and NCR is similar, hence the required features for IAB-MT and NCR-MT are similar.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Use the identified and agreed IAB-MT capabilities as a starting point for repeater-MT capabilities.

# References

“RAN1 Chairman’s Notes”, 3GPP TSG RAN1 #101-e, June 2020.