3GPP TSG RAN WG1 #112bis-e	R1-2303873

e-Meeting, April 17th - April 26th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion of RAN2 LS on Comparison of SL-RSRP and SD-RSRP measurements R1-2302280(R2-2302234)

Agenda item:		5

Document for:		Discussion

# Introduction

In their LS on Comparison of SL-RSRP and SD-RSRP measurements [1], RAN2 pose the following question:

Q1: Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report?

In this contribution we present our view on this question.

# Discussion

The question, as posed by RAN2, is not really a question that RAN1 is qualified to answer. Obviously, such a comparison can be used to trigger a measurement report; how useful and beneficial that would be is for RAN2 to decide. In RAN1 we can just point out the potential problems with such a comparison.

RAN2 discussed introducing the following measurement event:

Event Z2: The measured RSRP of a candidate L2 U2N Relay UE becomes an offset better than measured RSRP of the serving L2 U2N Relay UE

The purpose is presumably to provide relevant measurement results to the gNB to assist the gNB in deciding whether to perform “indirect-to-indirect path switching”, that is replacing the current serving relay UE with a “better” relay UE. In the context of this measurement event, the issue at hand is whether comparing a measured SD-RSRP from a candidate relay UE to a measured SL-RSRP of the serving relay UE is meaningful to trigger such an event. The LS explains these two measurements as follows:

- SD-RSRP (RSRP measurement on broadcast PC5 link used for discovery messages)

- SL-RSRP (RSRP measurement on unicast PC5 link)

A UE may use different TX power for transmissions over which SL-RSRP is measured versus transmissions of discovery messages, over which SD-RSRP is measured, e.g.

- One obvious difference is that unicast is subject to transmit power control based on SL pathloss if parameter sl-P0-PSSCH-PSCCH is provided, while broadcast is not subject to this power control mechanism. This applies regardless of resource allocation mode.

- Due to CBR-based power control, if configured, transmit power may depend on the priority of the transmission. The priority for discovery messages may be different from the priority used for the transmissions over which SL-RSRP is measured.

- Transmission of discovery messages can take place in dedicated discovery pools; these may be configured differently, e.g. with respect to power control settings, from the pool(s) on which SL-RSRP is measured.

Observation 1: Unicast is subject to transmit power control based on SL pathloss (if configured), broadcast is not.

Observation 2: For CBR-based power control (if configured), transmit power may depend on priority. Priority may be different for discovery messages and the transmissions over which SL-RSRP is measured.

In view of these observations, it seems doubtful that it makes sense to directly compare an SL-RSRP measurement from the serving relay UE with an SD-RSRP measurement from a candidate relay UE. Assuming all else being equal (priority and number of subchannels of the transmission, CBR, DL pathloss, pool configuration, etc), the total transmit power for broadcast will always be greater than or equal to the transmit power for unicast. As a result, SD-RSRP of a candidate relay UE may be considerably higher than the SL-RSRP of the current serving relay UE, even if the candidate and the current relay are equally “good” as relay UEs.

Observation 3: A direct comparison of SL-RSRP and SD-RSRP is not meaningful.

Proposal 1: Respond to RAN2 that a direct comparison of SL-RSRP and SD-RSRP is not meaningful.

# Conclusion

In this contribution we discussed the question raised in RAN2’s LS on Comparison of SL-RSRP and SD-RSRP measurements and made the following observations and proposals:

Observation 1: Unicast is subject to transmit power control based on SL pathloss (if configured), broadcast is not.

Observation 2: For CBR-based power control (if configured), transmit power may depend on priority. Priority may be different for discovery messages and the transmissions over which SL-RSRP is measured.

Observation 3: A direct comparison of SL-RSRP and SD-RSRP is not meaningful.

Proposal 1: Respond to RAN2 that a direct comparison of SL-RSRP and SD-RSRP is not meaningful.

# References

- R1-2302280(R2-2302234)	LS on Comparison of SL-RSRP and SD-RSRP measurements	RAN2