3GPP TSG RAN WG1 #112bis-e		R1-2303874

e-Meeting, April 17th – April 26th, 2023

Title:	[Draft] Reply LS on Comparison of SL-RSRP and SD-RSRP measurements

Response to:	R2-2302234 LS on Comparison of SL-RSRP and SD-RSRP measurements  from RAN2

Release:	Rel-18

Work Item:	NR_SL_relay_enh

Source:	[RAN1], Nokia

To:	RAN2

Cc:	RAN4

Contact person:	Torsten Wildschek

torsten.wildschek@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

RAN1 thanks RAN2 for this LS. In the LS, RAN2 asked the following question:

Q1: Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report?

RAN1 would like to provide the following discussion:

A UE may use different TX power for transmissions over which SL-RSRP is measured versus transmissions of discovery messages, over which SD-RSRP is measured, e.g.

- One obvious difference is that unicast is subject to transmit power control based on SL pathloss if parameter sl-P0-PSSCH-PSCCH is provided, while broadcast is not subject to this power control mechanism. This applies regardless of resource allocation mode.

- Due to CBR-based power control, if configured, transmit power may depend on the priority of the transmission. The priority for discovery messages may be different from the priority used for the transmissions over which SL-RSRP is measured.

- Transmission of discovery messages can take place in dedicated discovery pools; these may be configured differently, e.g. with respect to power control settings, from the pool(s) on which SL-RSRP is measured.

Given these differences, RAN1 would like to provide the following answer to RAN2’s question:

RAN1 response: A direct comparison of SL-RSRP and SD-RSRP is not meaningful.

# 2	Actions

To RAN2: RAN1 respectfully asks RAN2 to take the above response into account in the future work.

# 3	Dates of next TSG RAN WG1 meetings

RAN1#113	22-26 May 2023   	Incheon, KR

RAN1#114	21-25 August 2023   	Toulouse, FR