3GPP TSG RAN WG1 Meeting #112bis-e																	R1-2303889

e-meeting, April 17th - 26th, 2023

Source:	Moderator (CATT)

Title:	FL Summary #2 for NR DL and UL carrier phase positioning

Agenda item:	9.5.2

Document for:		Discussion and Decision

# Introduction

Rel-18 new work item on expanded and improved NR positioning includes the following objective related to carrier phase positioning (CPP) [1]:

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4].

  - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements.

  - Specify measurements that are limited to a single carrier/PFL.

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

RAN1 started working on the aforementioned objective in RAN1#112 [2]. The purpose of this contribution is to provide a summary of the following email discussion of the objective, based on the contributions submitted to this meeting ([3-25]).

[112bis-e-R18-Pos-04] Email discussion on NR DL and UL carrier phase positioning by April 26 – Ren (CATT)

- Check points: April 21, April 26

The discussion will focus on the following aspects:

- Definition of NR carrier phase measurements

- NR carrier phase measurements and reporting

- CPP multipath mitigation

- Differential CPP and PRU

- Integer ambiguity

- Phase error group

- CP measurement quality

- UE measurement capability

- Measurement procedure

- Carrier Phase in DC subcarrier

- Round trip carrier phase positioning

- TRP time synchronization

Please note that in this FL summary, a FL proposal may be designated as (H)(M)(L) to indicate its high, medium or low priority for online or offline discussions in this meeting. Nevertheless, we encourage all interested companies to provide feedback on all FL proposals. The FL may revise the priority of the proposals based on inputs from interested companies during the meeting, if it is deemed necessary.

# Definitions of NR carrier phase measurements

Background

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE

Submitted Proposals:

| Huawei, HiSilicon [3] | - Proposal 1: Support the reference point of the RSCP can be defined as the antenna phase center of the UE or gNB. - Proposal 2: Support the measurement timing of RSCP can be defined as the measurement timestamp, which can be presented by frame/subframe/slot/symbol numbers. - Proposal 3: For UE-assisted LMF-based NR CPP, support to introduce reference signal carrier phase difference (RSCPD) measurement. |
| --- | --- |
| vivo[4] | Proposal 5: - NR DL reference signal carrier phase (RSCPD) (of i-th path) is the DL relative phase difference between the Transmission Point (TP) j and the reference TP k, defined as CPj – CPk, - Where: - CPj is the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement from TP j. - CPk is the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement from TP k. - CPj , CPk are associated with the same RF frequency. |
| BUPT[5] | Proposal 1: The definition of DL CP can be represented by Equation 2. Proposal 2: The definition of DL CPD can be represented by Equation 3. Proposal 3: The definition of the UL CP can be expressed by Equation 5. |
| Spreadtrum Comm[6] | Proposal 1: For DL carrier phase positioning, two methods of reporting carrier phase should be supported. |
| CATT[8] | Proposal 1: Both carrier phase (CP) and carrier phase difference (CPD) measurements should be supported for Rel-18 DL carrier phase positioning. Proposal 3: The following three options can be considered to define the specific RF frequency for DL RSCP/RSCPD measurements. Option 1 should be the default option, while at least one of Option 2 and Option 3 should also be supported: - Option 1: The center RF frequency of a DL CC/PFL by default; - Option 2: The center RF frequency of a configured sub-bandwidth of a DL CC/PFL; - Option 3: The RF frequency of a specific subcarrier within a DL CC/PFL. Proposal 4: The reference point of NR DL carrier phase measurement should be the antenna phase center (APC) of the UE. To account for the relative position between the APC and the legacy RSTD and/or UE Rx-Tx time difference reference point (aka antenna phase center offset, PCO), additional parameters need to be introduced for UE to report the PCO to LMF. Proposal 5: The measurement time t of a DL RSCP/RSCPD should be specified to support both per OFDM symbol reporting and per DL PRS resource reporting. In the case of per OFDM symbol reporting, the measurement time should align with the OFDM symbol. In the case of per DL PRS resource reporting, the measurement time should be defined as either the first or the last OFDM symbol of the DL PRS resource. Proposal 7: In NR DL carrier phase positioning, the DL accumulated RSCP (A-RSCP) can be defined as the sum of DL RSCP and an accumulated number of integer cycles, which accounts for the change of the phase cycles after the receiver begins tracking the DL PRS signals from a TRP, i.e., DL RSCP = DL A-RSCP (modulo 1 cycle). Proposal 8: In NR DL carrier phase positioning, the DL accumulated RSCPD (A-RSCPD) can be defined as the difference between two A-RSCPs: one is measured from the DL PRS signals from target TRP and the other from the reference TRP. Proposal 15: The specific RF frequency for a UL RSCP measurement can be defined in the following options: - Option 1: The center RF frequency of a UL carrier by default; - Option 2: The center RF frequency of a configured sub-bandwidth of a UL carrier; - Option 3: The RF frequency of a specific subcarrier within a UL carrier. Proposal 16: The reference point of NR UL RSCP should be the antenna phase center (APC) of the gNB/TRP. To account for the relative position between the APC and the legacy RTOA and/or gNB Rx-Tx time difference reference point (aka antenna phase center offset, PCO), additional parameters need to be introduced for gNB/TRP to report the PCO to LMF. Proposal 17: The measurement time t of a UL RSCP should be specified to support both per OFDM symbol reporting and per UL SRS resource reporting. In the case of per OFDM symbol reporting, the measurement time should align with the OFDM symbol. In the case of per UL SRS resource reporting, the measurement time should be defined as either the first or the last OFDM symbol of the UL SRS resource. Proposal 19: In NR UL carrier phase positioning, the UL accumulated RSCP (A-RSCP) can be defined as the sum of UL RSCP and an accumulated number of integer cycles, which accounts for the change of the phase cycles after the receiver begins tracking the UL SRS signals from UE or PRU, i.e., UL RSCP = UL A-RSCP (modulo 1 cycle). |
| Intel [9] | Proposal 2 - Introduce DL RSCPD for DL CPP considering its resiliency to initial phase at UE Rx. - FFS: Reporting of RSCP corresponding to a target TRP – e.g., DL RSCPD may be sufficient if reporting of DL CP measurements is defined as an independent positioning method. - LMF configures a target UE with a reference cell that is independent from serving cell. Proposal 4 - For DL CPP, - The measurements are performed on DL PRS received in a subframe SF_j from target TP j and in a subframe SF_i from reference TP i that is closest in time to subframe SF_j. - Multiple DL PRS resources may be used to determine the carrier phase. - LMF may optionally request a target UE to perform measurements that are restricted to within a time window. Proposal 5 - For DL CPP, as default, the “specific RF frequency” for the carrier phase corresponds to the center frequency for the DL PFL. Proposal 6 - For DL CPP, reference point for DL RSCP measurement is the UE receiver antenna connector is assumed as the baseline. - FFS: Further enhancements, including PCO reporting. Proposal 7 - DL RSCPD is defined only with respect to the first path detected in time. Proposal 8 - Support UL RSCP reporting by a gNB as an independent positioning method. - FFS: Exact conditions and associated assistance information to resolve integer ambiguity. Proposal 11 - For UL RSCP, as default, the “specific RF frequency” for the carrier phase corresponds to one of (to be downselected): - the center frequency for the UL carrier, or - RF frequency corresponding to the center of the SRS/SRSp resource on which UL RSCP is measured. Proposal 12 - For UL CPP, reference point for UL RSCP measurement is assumed as defined for UL RTOA or gNB Rx-Tx time difference as the baseline. - FFS: Further enhancements, including PCO reporting. |
| Nokia, NSB[10] | Proposal 1: RAN1 should introduce both DL RSCP and DL RSCPD to specification. |
| Xiaomi[11] | - Proposal 1: The reference point for DL RSTD can be reused for DL RSCP. - Proposal 3: The reference point for UL RTOA can be reused for UL RSCP. |
| Samsung[12] | Proposal 1: For carrier phase positioning measurement, the following DL carrier phase positioning measurements are specified: - DL reference signal carrier phase (DL RSCP). - DL reference signal carrier phase difference (DL RSCPD). Proposal 2: For carrier phase positioning measurement, the DL RSCP is measured at the UE’s antenna connector of an antenna port. Proposal 3: For carrier phase positioning measurement, the UL RSCP is measured at the UE’s antenna connector of an antenna port. Proposal 4: For carrier phase positioning measurement, consider the following options for the reference time of the reported carrier phase: - Option 1: The reference time is that of the PRS symbol. - Option 2: The reference time is at the start of the slot or subframe or frame containing the PRS. Proposal 5: For carrier phase positioning measurement, when combining carrier phase cross multiple symbols, the carrier phase measurement should be independent of the symbol location. Proposal 6: For carrier phase positioning measurement, a UE can be configured to provide the carrier phase difference between the DL PRS received from TRP1 (a reference TRP) and TRP2 (a target TRP) at its antenna interface. - UE to maintain phase continuity between carrier-phase measurement of TRP1 and TRP2. |
| Lenovo[14] | Proposal 1: RAN1 to prioritize DL CP measurements based on DL PRS transmitted from a single TRP. |
| Fraunhofer [16] | Proposal 1: For carrier phase measurements in DL support both single and difference carrier phase measurements. |
| Apple [18] | Proposal 5: RAN 1 should down-select from the following options: - Option 1: for UE-assisted NR carrier phase positioning, the RSCP should be fed back to the LMF. Any differences with a reference TRP can be handled at the LMF. - Option 2: Both RSCP and RSCPD can be supported with the RSCP supported by default. The RSCPD used only if a UE indicates support for double differencing. |
| Ericsson[19] | Proposal 2 For DL carrier phase measurements, use the same reference point as DL-RSTD: For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. Proposal 3 For UL carrier phase measurements, use the same reference point as UL-RTOA. Proposal 4 In downlink, support RSCPD of the first path but not for additional paths. Proposal 5 In uplink, support RSCP of the first path but not for additional paths. |
| Qualcomm[20] | - Proposal 1: Define only DL RSCP and do not define DL RSCPD in TS38.215. UE reports do not include RSCPD. - RAN1 understanding is that there will not be RAN4 requirements on absolute accuracy of RSCP. RAN4 requirements are defined on differences of RSCPs, and the detailed definitions of the differencing operations is upto RAN4 Proposal 2: The reference points for DL and UL RSCP measurements should be the same as those for DL RSTD and UL RTOA respectively - If this is not easily agreeable in RAN1; it should be left to RAN4 to define the reference points. Proposal 3: Do not adopt RSCP definition for the i-th path for i>1 unless clear benefits from this are demonstrated Proposal 4: Measurement timing can be inferred from the existing reporting of NR-TimeStamp and PRS resource measured. |
| LGE[22] | Proposal 1: To enable UE-based and UE-assisted NR CPP, both CP and CPD should be introduced Proposal 2: For determination of RF frequency of DL RSCP/RSCPD reporting of a target UE, following options can be considered - Option 1: Center frequency of a PFL - Option 2: LMF informs the RF frequency of DL RSCP/RSPD measurement per PFL or UE - Option 3: LMF informs the RF frequency list of DL RSCP/RSPD measurement per PFL or per UE, and UE selects a RF frequency in the list for a reporting. - FFS: RF frequency of DL RSCP/RSCPD reporting of a PRU Proposal 3: For determination of RF frequency of UL RSCP reporting for a target UE, following options can be considered - Option 1: Center frequency of a SRS resource - Option 2: LMF informs the RF frequency of UL RSCP measurement per UE or SRS resource (set) - FFS: RF frequency of UL RSCP reporting for a PRU Proposal 4: When the RSCP/RSCPD measurements is reported together with the time-based positioning measurements, M(≥1) samples (or instances) is used for the time-based positioning measurement while one sample (or instance) is used for the RSCP/RSCPD measurements. - Time stamp can be reported per each pair of the RSCP/RSCPD and time-based positioning measurements, and it indicates time information for RSCP/RSCPD measurements. Proposal 5: When the RSCP/RSCPD measurements is reported together with the time-based positioning measurements, one time-based positioning measurements can be associated with multiple RSCP/RSCPD measurements with different time instances - FFS: PEG or phase continuity condition for multiple RSCP/RSCPD measurements associates with the time-based positioning measurement |
| IIT Kanpur, CEWiT[24] | Proposal 1: The carrier phase measurement, at least at the subcarrier level, should be supported for carrier phase positioning. Proposal 4: In Rel 18, for the legacy positioning methods, additional reporting of carrier phase measurements per Rx antennas should be supported. Proposal 5: The carrier phase reported should be mapped to the reference point. The reference point shall be the antenna connector of the UE/TRP. |
| MTK [25] | Proposal 3-1: There is no need to define the measurement timing within the definition of measurement quantity Proposal 3-2: The DL-PRS pattern with small number of symbols could be considered from implementation point of view, and the measurement could be per slot basis even when there is repetition within each instance. Proposal 5-1: Support both the CP and CPD measurements |

## DL RSRP/RSRSP

During RAN1#112, a consensus was reached to introduce DL RSCP and/or DL RSCPD for NR carrier phase positioning, but the decision regarding which one to introduce was not finalized. For this meeting, various companies shared their perspectives on this matter [3-25], which can be summarized as follows.

- Option 1: RSCP

  - Supported by: Xiomi, Lenovo, Qualcomm

- Option 2: RSCPD.

  - Supported by: Huawei, HiSilicon, vivo, Ericsson

- Option 3: Both RSCP and RSCPD.

  - Supported by: BUPT, Spreadtrum, CATT, Intel, Nokia, Samsung, Fraunhofer, Apple, LGE, MTK

It seems most companies show their preference to support both RSCP and RSCPD. In FL's understanding, supporting either RSCP or RSCPD alone should suffice for NR carrier phase positioning. However, supporting both of them may provide convenience and flexibility for implementation for certain situations. For example, reporting DL RSCPD may be a better choice when carrier phase measurements are combined with RSTDs for positioning, while reporting RSCP may be more suitable for other scenarios, e.g., when carrier phase measurements are combined with UE/gNB Rx-Tx time measurements for positioning. Therefore, we may consider supporting both RSCP and RSCPD, and then further discussing the scenarios where DL RSCP or DL RSCPD is used.

(H)(Round 1) Proposal 2.1-1

Introduce both DL reference signal carrier phase (DL RSCP) and NR DL reference signal carrier phase difference (DL RSCPD) for NR carrier phase positioning.

- Note: The scenarios where a UE is requested to report DL RSCP or DL RSCPD will be further discussed.

| Company | comments |
| --- | --- |
| Vivo | We are also okay with RSCP, but we would like to confirm why we need to introduce both if the scenario will be further discussed. To be honest, we believe the CPD can be calculated by CP. So, if no clear scenario and benefit, why do we need to support both? |
| ZTE | We share the same view as vivo, LMF can calculate the CPD with the reported CP. |
| Xiaomi | We are fine with the proposal |
| Nokia/NSB | We support the proposal. In our understanding it enables better synergy with DL-TDOA and Multi-RTT reporting to introduce both. |
| CATT | We are fine either to introduce DL RSCP only or both DL RSCP and DL RSCPD. |
| Lenovo | We are also supportive of RSCP, ok with RSCPD but share the same concern on the necessity if it can be performed by LMF implementation using a pair of RSCP measurements and is not time sensitive. |
| OPPO | We are fine to introduce both measurements. |
| Ericsson | We prefer Option 2, not Option 3 as stated above. It will be convenient to have carrier phase measurement definition that harmonize with DL RSTD. |
| Samsung | We are fine with this proposal. |
| InterDigital | We agree with the proposal in principle. |
| Qualcomm | We prefer Option1. RSCP contains all the info that RSCPD does. Regarding comments from Nokia and Ericsson, we think RSCP is actually easier to harmonize and add to the existing reports both for DL-TDOA and Multi-RTT, in much the same way as RSRP reporting was added to DL-TDOA and Multi-RTT |
| Intel | We do not see the logic in agreeing to both when we are not sure of the scenarios for each. Given that one metric (RSCP) is not measurable and the other offers better robustness, we think introducing reporting of DL RSCPD is more appropriate and sufficient. Regarding Multi-RTT, we do not see any issue if UE reports DL RSCPD with respect to another TRP. In fact, in most cases, this would only improve the robustness of the combined measurements at an LMF. |
| FL | In FL’s view, in this meeting RAN1 should make the decision on whether to support DL RSCP, or DL RSCPD, or both, since this is an essential issue that has the impact on some other proposals. Thus, the FL suggest bring the following proposal to the first GTW session, hopefully we can reach a compromise on the option(s). ### (H)(Round 1) Proposal 2.1-1 (updated for 1st GTW session) Adopt one of the following options for NR carrier phase positioning: - Option 1: Introduce DL reference signal carrier phase (DL RSCP) as DL carrier phase measurement; - Option 2: Introduce NR DL reference signal carrier phase difference (DL RSCPD) as DL carrier phase measurement; - Option 3: Introduce both DL reference signal carrier phase (DL RSCP) and NR DL reference signal carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: The scenarios where a UE is requested to report DL RSCP or DL RSCPD will be further discussed. |
| FL | ### The above proposal was discussed in 1st GTW session in Monday for about 40mins, but we did not reach final agreement. The latest draft from Chairman’s Note is copied in the following: Proposal Adopt one of the following options for NR carrier phase positioning: - Option 1: Introduce DL reference signal carrier phase (DL RSCP) as DL carrier phase measurement; - Note: RAN4 requirements may be defined only based on differences - Option 2: Introduce NR DL reference signal carrier phase difference (DL RSCPD) as DL carrier phase measurement; - Option 3: Introduce both DL reference signal carrier phase (DL RSCP) and NR DL reference signal carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: either DL RSCP or DL RSCPD can be reported - DL RSCP is reported with UE Rx - Tx time difference measurement - DL RSCPD is reported with RSTD measurement - FFS: whether DL RSCP or DL RSCPD can be reported as a standalone measurement report - FFS: whether DL RSCP or DL RSCPD can be reported with other measurement types (if agreed) - Note: The scenarios where a UE is requested to report DL RSCP or DL RSCPD will be further discussed. Due the importance for us to reach a consensus in this meeting, the Chair has specifically assigned an offline (1hr) for further discuss the proposal (and possibly others for AI 9.5.2) in this Wednesday. Since we had a long discussion, I assume the interested companies have not much better understanding with the pros and cons of the options. |

(H)(Round 2) Proposal 2.1-1

Adopt one of the following options for NR carrier phase positioning (down-selection in RAN1#112bis-e):

- Option 1: Introduce DL reference signal carrier phase (DL RSCP) as DL carrier phase measurement;

  - Note: It is up to RAN4 to decide whether the RAN4 requirement is defined for DL RSCP or for the differences of DL RSCP

  - Support (1st choice): ZTE, vivo, Xiaomi, CATT, Qualcomm (5 companies)

  - Support (2nd choice): Nokia/NSB, Spreadtrum, Samsung, Apple (5 companies)

- Option 2: Introduce DL reference signal carrier phase difference (DL RSCPD) as DL carrier phase measurement;

  - Support(1st choice): Huawei, HiSilicon, Intel (3 companies)

  - Support(2nd choice): CATT (1 company)

- Option 3: Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements.

  - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note.

  - DL RSCP can be reported together with UE Rx – Tx time difference measurement

  - DL RSCPD can be reported together with RSTD measurement

  - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed

  - Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed.

  - Support(1st choice): LGE, Nokia/NSB, OPPO, Locaila(w/o Notes 1&2), IIT Kanpur, CWEiT (w/o Notes 1&2), Spreadtrum, Fraunhofer, Xiaomi, InterDigital, Samsung, Lenovo, Apple, NTT DCM (15 companies)

  - Support (2nd choice): ZTE(with Notes 1&2), vivo, CATT (3 companies)

| Company | comments |
| --- | --- |
| LGE | We prefer option 3 As captured in the notes, RSCP would be proper to report together with rx-tx time difference measurement while the RSCPD is proper to report together with RSTD measurements. We believe that RSCP measurement reporting cannot be exactly same to the RSCPD measurement. For single differencing, RSCP pair shall be selected with same timing (or close to each other). In case of RSCPD measurement reporting, UE can select a pair of RSCP measurement with same PEG requirement (e.g. including same measurement timing or similar concept can be used) to measure RSCPD. On the contrary, if UE reports RSCP, there is no guarantee that all RSCP measurements are measured with same PEG condition (or at a same time instance). Which means that it is not guaranteed that initial Rx phase error can be eliminated by reported RSCP measurements. FL: These are avlid points to be considered if we agree to report RSCP. Meanwhile, we shall consider that UE can determine reference TRP by itself and it may not be aligned with the recommendation from LFM. Thus it would be better PRU to report RSCP value instead of RSCPD. Instead, more restricted reporting requirement, such as enforce PRU to report measurement on pre-configured PRS resources at pre-configured timing, can be considered. If so, LMF may have more flexibility to perform double differential technique, and it could be supported with any reference TRP that is selected by the target UE. FL: Reporting RSCP does not require UE to determine reference TRP. It is simpler for UE side. In short, in our view, RSCPD measurement reporting would be beneficial since the error elimination could be guaranteed without additional measurement report overhead, while RSCP measurement would be beneficial since it allows LMF to handle error elimination based on the measurement reports from the multiple UEs. One minor correction, In the main bullet, it shall be “down-selection in RAN1#112bis-e” FL: good catch. |
| Huawei, HiSilicon | We support option 2. We would like to clarify that RSCPD is the only possible way to obtain phase measurement in BB that is according to phase definition whose reference point is at the RF. FL: I may miss something here. I assume if this is a concern to DL RSCP, it applies also for UL RSCP. |
| Nokia/NSB | We prefer option 3 but can live with option 1 also. |
| OPPO | We support Option 3 The RSCP and RSCPD can be reported together with different existing measurement. They are applicable to different scenarios. |
| Locaila | We prefer option 3, without note 1 and note 2. We strongly disagree on note 1 and 2. We only want note 3, that is to consider support of stand-alone method. RX-Tx measurement itself is a lot slow method, and if it needs to be used with the double-difference method, it will almost be useless for vehicle use-cases. FL: I don’t understand the opjection here. The proposal does not preclude the support of standalone carrier phase reporting |
| IIT Kanpur, CWEiT | We prefer option 3, without note 1 and note 2. But also fine with option 1. |
| ZTE | We prefer option 1, or option 3 with note 1 and note 2. For option 1, it’s quite enough for UE to report CP, LMF can calculate CPD when necessary (as commented on round 1 discussion). For option 3, we think it is necessary to keep note 1 and note 2. Because without these notes, extra signalling may be required to configure whether UE should report CP or CPD. With these notes, UE can identify which one can be reported. And for note 3, does “other types of measurements” refer to the UE measurement mentioned in section 5 proposal 5-2? If yes, this can be decided if there’s a agreement on proposal 5-2. Otherwise, we prefer delete this note. FL: Note 3 is a compromise. It means the agreement does not preclude standalone carrier phase reporting. |
| vivo | We prefer option 1 but can live with option 3 |
| Intel | Our preference is Option 2. The coupling with Rx-Tx time difference for RSCP is artificial and at most relevant from a signalling structure perspective as already discussed during the GTW session. Nothing fundamental about it. Option 2 offers a clearly testable metric that RSCP reporting would anyway need to rely on. We just seem to be passing the responsibility to another WG. In terms of practical use of RSCP without resorting to CP differences across TRPs, the only options mentioned are: (1) using tracking based approach with multiple measurements reported by a UE (not always feasible depending on PRS configurations and this was discussed at length during SI phase); (2) using CP-RTT type solutions that fundamentally need phase coherency between the Tx and Rx paths (not a typical case in practice); and (3) use of reports from multiple UEs/PRUs that are sufficiently close-by. None of these reflect the primary means of use of CPP compared to use of RSCPD metrics. Thus, the most natural measurement report to define would be RSCPD rather than defining RSCP where the CP is w.r.t. an undefined reference phase. FL: From UE implementionn point of view, isn’t easier for reporting RSCP than RSCPD, since the UE does not hold the responsibility to select the reference TRP correctly? |
| Spreadtrum | We prefer option 3 and we can live with option 1. For the CPD, although UE only needs to report a difference value, the information available to the network side will be correspondingly less. For example, when UE measures PRS signal(s) from the target TRP and the reference TRP, the two links may have different error values. When only the difference between the two carrier phase measurements is reported, the network side can only make compensation according to the difference. Once an error occurs when UE receives, it is difficult for the network side to find the problem only from the difference value. For the CP, although the overhead is higher than that of the CPD, more information can be obtained by network, which will help the network to find the error generated by the UE side. |
| Fraunhofer | Option 3 is our preferred choice, but we are okay to settle on Option1 if needed. |
| Xiaomi | We are fine with either Option 1 or Option 3. For RSCPD, it is better to make clear that the two RSCP must be measured based on same PEG or can be measured based on different PEG. |
| InterDigital | We support Option 3. |
| CATT | Prefer Option 1. We can live with other options. |
| Samsung2 | We think that the purpose of this proposal is decide which of the RSCP and/or RSCPD is supported. In this regards, we prefer option 3, which is to support both. The second issue this proposal attempts to cover is how to report these measurement (i.e., standalone or with other measurements and which measurements), this is a separate issue, that can be discussed later. Our preference is to go with option 3 without any notes. The notes can be discussed as part of another proposal. As a compromise, we can just keep note 3 and update as follows: Note 31: Whether to support standalone DL RSCP or DL RSCPD reporting, and which measurement reports or DL RSCP/DL RSCPD is reported reporting with other types of measurements, can be further discussed. |

### (Closed)(Round 3) Offline Consensus 2.1-1

- Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements.

  - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note.

  - DL RSCP can be reported together with UE Rx – Tx time difference measurement

  - DL RSCPD can be reported together with RSTD measurement

  - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed

  - Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed.

| Company | comments |
| --- | --- |
| Samsung3 | Support |
| Nokia/NSB | We prefer to not have further discussion on this until GTW since it was offline consensus after 1 hour |
| Qualcomm | Support, and agree with Nokia |

## Reference point of carrier phase measurements

In RAN1#112, the reference point of the carrier phase measurements was discussed without a final conclusion. From the submitted proposals to this meeting, the companies’ views for the reference point of NR carrier phase measurements may be summarised as follows:

- Option 1: the antenna connector (as RSTD/RTOA)

  - Supported by: Intel (baseline), Xiaomi, Samsung, Ericsson, Qualcomm, IT Kanpur, CEWiT

- Option 2: the antenna phase center (APC)

  - Supported by: Huawei, HiSilicon, CATT

- Option 3: Up to RAN4 to decide

  - Supported by: Qualcomm

It seems the majority of companies prefer defining the reference point of NR carrier phase measurements to be the same as the legacy timing measurements. The carrier phases are measured by referring to the antenna phase centre (APC), which is the apparent source of radiation. The APC position may not coincide with the geometric centre of the antenna, or the antenna connector, as explained in detail at  https://gssc.esa.int/navipedia/index.php/Receiver_Antenna_Phase_Centre. If the reference point of the carrier phase measurements is defined with respect to the antenna connector, then we may need to further discuss how UE/gNB maps the measured carrier phase to the reference point of the antenna connector. If we call the relative position between the APC and the reference point as antenna phase center offset (PCO), the mapping measured carrier phase to the reference point of the antenna connector requires to compensate for the phase difference related to the projection of PCO on the LOS direction between the UE and TRP, which may not be a trivial since it requires UE/gNB to determine the LOS direction between the UE and the TRP. Thus, we may need to have more discussions on these options.

(H)(Round 1) Proposal 2.2-1

Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection).

- Option 1: Define the reference point of the UE/TRP carrier phase measurements as the antenna connector of the UE/TRP Rx antenna

  - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting.

- Option 2: Define the reference point of the UE/TRP carrier phase measurements as the antenna phase center of the UE/TRP Rx antenna.

  - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF

  - FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame

- Option 3: Request RAN4 to define the reference point of the UE/TRP carrier phase measurements

- If Option 1 or Option 2 is agreed, RAN1 also needs to send an LS to RAN4 to confirm RAN1’s decision

| Company | comments |
| --- | --- |
| vivo | We also share the majority view, if the carrier phase is reported with other existing measurements, the reference point should be the same as the legacy timing measurements. And the UE side error can be removed by difference if it is due to a common error(e.g., the error between APC and antenna connector) In addition, we think option 1 should be modified to the same reference of existing measurement since the reference of FR1 and FR2 is different as follows For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE. |
| Huawei, HiSilicon | We think that the change of FR2 reference point is motivated by phase centre in the first place, see the discussion in R1- 2001243 and LS in R1-2000172. With that, we think that vivo’s suggestion is reasonable. |
| ZTE | We think if this is to be decided by RAN1, option 1 should be supported. It would be reasonable to re-use the same definition in RSTD/RTOA. It is also feasible to be determined by RAN4. Option 1 and 3 are OK for us. |
| Xiaomi | Prefer to reuse the reference point for the legacy measurements and it should be different for FR1 and FR2 for UE, and different for different type of gNB. |
| Nokia/NSB | We support option 2. |
| CATT | Either Option 1 or Option 2 is fine to us. The mortification suggested by vivo is fine to us. |
| Lenovo | We are fine to follow the reference point for legacy positioning measurements, i.e. Option 1. The differentiation between FR1 and FR2 should also be included as suggested by Huawei. |
| OPPO | Option 1 shall be supported because the carrier phase measurement will be reported together with the legacy time-based measurement. |
| Ericsson | Similar comment as vivo. |
| Samsung | Support option 1. |
| Qualcomm | Similar view as vivo |
| Intel | Same comment as vivo. |
| FL | ### (H)(Round 1) Proposal 2.2-1 (updated for 1st GTW session) Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection). - Option 1: The reference point of the UE/TRP carrier phase measurements is defined as the antenna connector of the UE/TRP Rx antenna for frequency range 1, as the UE/TRP Rx antenna for frequency range 2. - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting. - Option 2: The reference point of the UE/TRP carrier phase measurements is defined as the antenna phase center of the UE/TRP Rx antenna for frequency range 1 and frequency range 2. - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF - FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame - Option 3: Request RAN4 to define the reference point of the UE/TRP carrier phase measurements - If Option 1 or Option 2 is agreed, RAN1 also needs to send an LS to RAN4 to confirm RAN1’s decision |
| LGE | Considering the case where the carrier phase measurements is reported along with the legacy positioning measurements, as commented by other companies, supporting option 1 seems more reasonable |

FL Comments:

The  majority of the companies seem in favor of Option 1 with the modifications proposed by vivo, although some companies prefer Option 2. It seems no company supports Option 3. So, I assume we can remove it. The proposal is updated as follows for further discussion.

The following figure explain my understanding of the difference between Option 1 and Option 2.

As shown in the figure, the carrier phase obtained from DL PRS is referenced to the APC. When Option 2 is used, the UE will report the measured phase and the relative position between APC and antenna connection. If Option 1 is adopted, the UE needs to map the obtained carrier phase to the carrier phase referenced to the antenna connector. For the mapping, the UE needs to derive and compensate the delta phase by the use of the relative position between APC and antenna connection and also the direction information of the DL PRS signals from the TRP, which may not be a simple taks for UE.

(H)(Round 2) Proposal 2.2-1

Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection).

- Option 1: The reference point of the UE/TRP carrier phase measurements is defined as the antenna connector of the UE/TRP Rx antenna for frequency range 1, as the UE/TRP Rx antenna for frequency range 2.

  - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting.

- Option 2: The reference point of the UE/TRP carrier phase measurements is defined as the antenna phase center of the UE/TRP Rx antenna for frequency range 1 and frequency range 2.

  - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF

  - FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame

- If Option 1 or Option 2 is agreed, RAN1 needs to send an LS to RAN4 to confirm RAN1’s decision.

| Company | comments |
| --- | --- |
| Nokia/NSB | Support the proposal. |
| OPPO | Option1 since it is aligned with legacy specification and please note the CPP measurement will be reported together with existing measurement, |
| IIT Kanpur, CEWiT | We prefer option 1. |
| Huawei, HiSilicon | OK |
| ZTE | Support option 1. Agree with OPPO. The previous option 3 is also fine for us. |
| vivo | Option 1 |
| Intel | We suggest to agree on Option 1 as baseline while Option 2 can be studied further as a further enhancement with lower priority at this point. FL: this could be a way to resolve the issue. |
| Spreadtrum | We prefer option 1. |
| Fraunhofer | Prefer option 2. Given the majority view, we find the suggestion from Intel to be acceptable. |
| Xiaomi | Prefer Option 1 to resue the legacy reference point. But for TRP, there are 3 types of TRP with different reference point respectively. So the Option 1 should be updated for TRP at least. And it can also be updated as “Option 1: reuse the reference point of DL RSTD for UE and reuse the reference point of UL RTOA for TRP side” for simple. The reference point for TUL-RTOA shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. FL: With the consideration of Xiaomi’s comment, we may change Option 2 - Option 1: - The reference point of the UE carrier phase measurements is defined to be the same as the reference point of RSTD for frequency range 1 and frequency range 2; - The reference point of the TRP carrier phase measurements is defined to be the same as the reference point of RTOA for frequency range 1 and frequency range 2; |
| CATT | Prefer option 2. We are also fine the suggestion from Intel. |
| Samsung2 | Support option 1. |
| FL | Based on the comments, we may consider the following revision: ### (H)(Round 2) Proposal 2.2-1 Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection). - Option 1: - The reference point of the UE carrier phase measurements is defined the same as the the reference point of RSTD for frequency range 1 and frequency range 2. - The reference point of the TRP carrier phase measurements is defined the same as the the reference point of RSTD for frequency range 1 and frequency range 2. - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting. - Option 2: - The reference point of the UE/TRP carrier phase measurements is defined as the antenna phase center of the UE/TRP Rx antenna for frequency range 1 and frequency range 2. - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF - FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame - If any of the options is agreed, RAN1 needs to send an LS to RAN4 to confirm RAN1’s decision. |

### (H)(Round 3) Proposal 2.2-1

Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection in RAN1#113).

- Option 1:

  - The reference point of the UE carrier phase measurements is defined the same as the the reference point of RSTD for frequency range 1 and  frequency range 2.

  - The reference point of the TRP carrier phase measurements is defined the same as the the reference point of RTOA for frequency range 1 and  frequency range 2.

  - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting.

- Option 2:

  - The reference point of the UE/TRP carrier phase measurements is defined as the antenna phase center of the UE/TRP Rx antenna for frequency range 1 and frequency range 2.

  - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF

  - FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame

- If any of the options is agreed, RAN1 needs to send an LS to RAN4 to confirm RAN1’s decision.

| Company | comments |
| --- | --- |
| Nokia/NSB | Is the intention to downselect at this meeting or to just list options. We are okay to agree to the proposal as options at this meeting. FL: Given that this is an isolated issue, not impacting other proposals, my consideration is to give companies additional time for consideration, so we can have the downselection process until the next meeting. |
| Huawei, HiSilicon | OK for further downselection. |
| vivo | Okay |
| Qualcomm | OK. We think Option1 is enough, but we agree with FL comment that this is not coupled to other issues and we can have some more time to consider it by downselecting later. |
| LGE | Fine with the proposal |
| ZTE | OK to make down-selection in next meeting. May be the second bullet in option 1 should be RTOA, i.e., - The reference point of the TRP carrier phase measurements is defined the same as the the reference point of RSTD RTOA for frequency range 1 and frequency range 2. FL: Thanks. Corrected. |
| Samsung3 | Support Option 1. Prefer the original wording in Round 2 as it clearer: - Option 1: The reference point of the UE/TRP carrier phase measurements is defined as the antenna connector of the UE/TRP Rx antenna for frequency range 1, as the UE/TRP Rx antenna for frequency range 2. - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting. FL: The wording in Round 2 does not match the current definition of reference point for RTOA. |
| Xiaomi | Fine with the proposal and support the update by ZTE. |
| Apple | Fine with proposal. Support Option 1. Noth the repeated word “the” in first two sub-bullets of Option 1. |
| CATT | Support for downselection in the next meeting. |
| Intel | We can accept identifying both options for now and down-select next meeting. |

## RF frequency of carrier phase measurements

It was agreed in RAN1#112 that a DL RSCP/RSCPD or UL RSCP is associated with a specific RF frequency. But, it is undecided what the RF frequency is. From the submitted proposals, we may have the following options to define the specific RF frequency associated with a carrier phase measurement:

- Option 1: The center RF frequency of the DL PFL or a UL carrier of SRS for positioning

  - Supported by: CATT, Intel, LGE

- Option 2: The center RF frequency of the measured DL PRS bandwidth, which can be a sub-band/segment of DL PFL, or a measured UL SRS bandwidth, which can be a sub-band/segment of UL carrier

  - Supported by: CATT, Intel,

- Option 3: The RF frequency of a specific subcarrier within a DL PFL or a UL carrier of SRS for positioning

  - Supported by: CATT, LGE, IIT Kanpur, CEWiT

Maybe we can consider Option 1 as a default option. Whether to support Option 2 and Option 3 can be further discussed after RAN1 makes the decision on whether to support the reporting of the carrier phase measurements with more than one RF frequency within a single DL PFL/UL carrier.

(H)(Round 1) Proposal 2.3-1

- The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default.

| Company | comments |
| --- | --- |
| vivo | support |
| Huawei, HiSilicon | We agree this default RF frequency. However, the centre RF frequency should require some discussion, and could be subject to multiple interpretations. Interpretation 1: the centre RF frequency is the frequency centre of the PRS BW, meaning if PRS RE range is 0 – 47 (4 RB), the centre frequency is between 23 or 24 or 23.5? Interpretation 2: the centre RF frequency is the centre of the RF bandwidth. Since the RF bandwidth could be up to UE implementation, it could be anywhere. FL: Interpretation 1 in my mind. |
| Nokia/NSB | Support. To address Huawei’s comment we could simply remove “RF” after center. |
| CATT | Support. For HW’s comment, our understanding is interpretation 1, e,g., the center frequency of the BW of the DL PFL. |
| Lenovo | Supportive for a common alignment on the definition of a center frequency for a DL CP measurement |
| OPPO | Support. |
| Ericsson | Support. |
| Samsung | We prefer to have more discussion on this proposal. There are cases when the UE reports CP for multiple frequencies. In this case, carrier phase would be for different frequencies. There is also the option that the UE indicates the frequency of the CP measurement. FL: This is for default option. If RAN1 reaches agreement for multiple frequencies, we will further define other RF frequencies. |
| Qualcomm | Support |
| Intel | Support. |
| LGE | We tend to agree with FL’s comment that option 1 can be a default option. But we have different view on option 3; it is also useful when carrier phase measurement with one RF frequency is supported. Actually, we see benefits of option 3 when UE measure the carrier phase measurement outside MG. For example, UE cannot measure PRS resource outsider an active DL BWP when it measures PRS within a PPW, and there could be a case that the center of frequency of PFL is not located within the active DL BWP of the UE. Also, if CPP in RRC inactive state is supported, it should be noted that neither MG for positioning nor PPW is supported. Hence, to avoid the problem of these situation, we think that the use of the RF frequency other than center frequency of the PFL shall be supported. So, our preference is to discuss this proposal together with proposal 2.3-2, at least to support reporting the carrier phase measurement with one RF frequency. FL: I think the comment is valid. I think it is to be discussed in Proposal 2.3-2. My intention is to resolve a simpler case first, and then further discuss the more complicated cases. To address the comment, we may add: “Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can be the specific RF frequency” |

(H)(Round 2) Proposal 2.3-1

- The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default.

  - Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them.

| Company | comments |
| --- | --- |
| Nokia/NSB | Support. Suggest to say “can also be” in the note. |
| IIT Kanpur, CEWiT | We are fine with the proposal. |
| Huawei, HiSilicon | OK |
| ZTE | Support. |
| Intel | OK with the main bullet. However, we should clarify what is implied by “non-default cases” – are these corresponding to the PPW use-case mentioned by LGE, or case of reporting on multiple subbands in context of IAR, or something else? At least for these options, it seems there may not be any “non-default cases” supported at all – so, this should be clarified in the note. FL: Right, we haveb’t agree any other cases. So, there may be no other cases. We can add “if agreed”. - Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them. |
| Spreadtrum | Support. |
| Fraunhofer | Support |
| Xiaomi | Support |
| CATT | Support |
| Samsung2 | OK |

### (H)(Round 3) Proposal 2.3-1

- The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default.

  - Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them.

| Company | comments |
| --- | --- |
| Nokia/NSB | Support. |
| Huawei, HiSilicon | OK |
| Qualcomm | OK |
| LGE | Fine with the proposal. But we think “if RAN1 agrees to introduce them” in the note seems not required. The note says “open to further discussion whether ~” and we think it already includes the meaning that we will discuss whether to introduce additional methods or not. FL: The wording is added to address Intel’s comment in Round 2 discussion. It may seem to be redundant. But, I am hoping it can be accepted. |
| ZTE | OK |
| Samsung3 | Support |
| Xiaomi | Support |
| Apple | Support |
| CATT | Support |
| Intel | OK |

(M)(Round 1) Proposal 2.3-2

- The following options can be further considered for the specific RF frequency associated with a DL carrier phase measurement (down-selection):

  - Option 1: the center RF frequency of a configured DL PRS bandwidth for the carrier phase measurement

  - Option 2: the RF frequency of any specific subcarrier within DL PFL

| Company | comments |
| --- | --- |
| InterDigital | We support the proposal and have a preference for Option 2. The reason is as the carrier phase positioning is intended to be a very high accuracy positioning method, the measurements should be as error free as possible even if it means additional complexity and/or signaling cost. Hence, the frequency selection for carrier phase positioning should be based on the channel conditions. As stated in our proposal, since the UE is aware of the channel conditions (e.g., frequency response of the channel), the UE may be best suited to determine which specific subcarrier to perform measurement on. |
| LGE | As we commented in (Round2) Proposal 2.3-1, this proposal shall be discussed together with the proposal above. Regarding listed option, we prefer to add the another option: Option 3: LMF configures one or more than one candidate RF frequencies for the carrier phase measurement, and one RF frequency in the configuration is selected by the UE for the carrier phase measurement. FL: Yes. We can add the Option. It can also be considered as a special case for Option 2. |

(M)(Round 2) Proposal 2.3-2

- The following options can be further considered for the specific RF frequency associated with a DL carrier phase measurement (down-selection):

  - Option 1: the center RF frequency of a configured DL PRS bandwidth for the carrier phase measurement

  - Option 2: the RF frequency of any specific subcarrier within DL PFL

    - FFS: How the specific subcarrier is determined.

  - Option 3: LMF configures one or more than one candidate RF frequencies for the carrier phase measurement, and one RF frequency in the configuration is selected by the UE for the carrier phase measurement.

| Company | comments |
| --- | --- |
| Nokia/NSB | Our view is that reporting the phase associated with the center frequency of the PFL is sufficient for Rel-18. |
| Locaila | We prefer option 2 with following modification. - The following options can be further considered for the specific RF frequency associated with a DL carrier phase measurement (down-selection): - Option 1: the center RF frequency of a configured DL PRS bandwidth for the carrier phase measurement - Option 2: the RF frequency of any specific (carrier+subcarriers) within DL PFL - FFS: How the specific subcarrier is determined. - Option 3: LMF configures one or more than one candidate RF frequencies for the carrier phase measurement, and one RF frequency in the configuration is selected by the UE for the carrier phase measurement. Some more explanation on our view ... The phase information extracted via FFT is expressed as the function of (carrier frequency + subcarrier frequency) as below YB=N∙A1ejωc+ω1ψB-ψA-τABA2ejωc+ω2ψB-ψA-τAB⋮Amejωc+ωmψB-ψA-τAB+σB In above , wc is the passband carrier frequency used for up/down-conversion in RF-frontend, and w1, w2, … is the subcarrier frequency of the resource element allocated for PRS. UE or gNB can only report based on the FFT decomposed phase information of above vector, and it is the form of (wc + wi) , i.e. (carrier frequency + subcarrier frequency) . The full list of subcarrier phase information is necessary for resolving integer ambiguity in standalone method. Note that center frequency can also be obtained by linear average (or complex sum) of the above vector elements. FL: The RF frequency here means the absolute frequency. Thus, RF frequency of i-th subcarrier is the absolute RF frequency of the carrier, i.e., it is (f_c + i*SCS). |
| IIT Kanpur, CEWiT | We prefer option 2. |
| Huawei, HiSilicon | We prefer another Option: Option 4: LMF configures one or more than one candidate RF frequencies for the carrier phase measurement, and one or more than one RF frequencies in the configuration is selected by the UE for the carrier phase measurement. |
| Intel | As mentioned in response to Proposal 2.3-1, we should first discuss and clarify the different cases for which these additional/alternative definitions of “specific RF frequency” applies to before agreeing on any of these. |
| Spreadtrum | Agree with Intel, we need to first indentify the cases where we need to define the RF frequency. |
| CATT | Share the similar view as Nokia that Option 1 may be good enough. However, we are also open with other options. |
| InterDigital | We support Option 2. We are also ok with the modification suggested by Locaila. |
| Samsung2 | We prefer option 2. |
| FL | We may consider the followng changes from the comments: ### (M)(Round 2) Proposal 2.3-2 - Further consider supporting one or more of the following options for the specific RF frequency associated with a DL carrier phase measurement: - Option 1: the center RF frequency of a configured DL PRS bandwidth for the carrier phase measurement - Option 2: the RF frequency of any specific subcarrier within DL PFL - FFS: How the specific subcarrier is determined. - Option 3: LMF configures one or more than one candidate RF frequencies for the carrier phase measurement, and one RF frequency in the configuration is selected by the UE for the carrier phase measurement. - Option 4: LMF configures one or more than one candidate RF frequencies for the carrier phase measurement, and one or more than one RF frequencies in the configuration is selected by the UE for the carrier phase measurement. |

FL Comments:

My thinking is that we list of the options to be considered in this meeting, and then make the decision in the next meeting.

### (H)(Round 3) Proposal 2.3-2 (Revision 1)

- Further consider supporting one or more of the following options for the specific RF frequency associated with a DL carrier phase measurement:

  - Option 1: the center RF frequency of a configured DL PRS bandwidth for the carrier phase measurement

    - FFS: the details on how the DL PRS bandwidth is configured

  - Option 2: the RF frequency of any specific subcarrier within DL PFL

    - FFS: the details on how the specific subcarrier is determined.

  - Option 3: LMF configures one or more than one candidate RF frequencies for the carrier phase measurement, and one RF frequency in the configuration is selected by the UE for the carrier phase measurement.

    - FFS: the details on on how LMF configures the candidate RF frequencies and how the configuration is selected by UE

- Option 4: LMF configures one or more than one candidate RF frequencies for the carrier phase measurement, and one or more than one RF frequencies in the configuration is selected by the UE for the carrier phase measurement.

    - FFS: the details on on how LMF configures the candidate RF frequencies and how the configuration(s) is selected by UE

| Company | comments |
| --- | --- |
| Nokia/NSB | Support option 1. |
| FL |  |
| Huawei, HiSilicon | We think that UL part should be aligned with this, right? FL: Yes. My thinking is that once we reach the agreement for DL side, it would be easier for us to work on the UL side. We prefer Option 4. |
| Qualcomm | We support option 1. |
| LGE | We support option 3 and ok with the proposal for decision in the next meeting. For more clear understanding, I wonder if option 1 intends reporting carrier phase measruements for multiple RF frequencies. If so, it would be better to add a note to avoid misunderstanding. FL: Option 1 may be extended. - Option 1: the center RF frequency(ies) of the configured DL PRS bandwidth(s) for the carrier phase measurement |
| ZTE | As far as we know, the DL PRS bandwidth is configured per PFL, so the center frequency of the DL PFL and the center RF frequency of a configured DL PRS bandwidth may refer to the same frequency. Keeping this in mind, we didn't see the difference between proposal 2.3-1 and proposal 2.3-1 option 1. FL: I assume ZTE's connment is related to Rel-17 configuration. For Rel-18, as ZTE also proposed, we may consider to configure the measurement in sub-PFL for the carrier phase measurement. Maybe we can add an FFS on how it is configured., - Option 1: the center RF frequency of a configured DL PRS bandwidth for the carrier phase measurement - FFS: the details on how the DL PRS bandwidth is configured |
| FL | We may consider the following changes based on the comments from ### (H)(Round 3) Proposal 2.3-2 (Revision 1) - Further consider supporting one or more of the following options for the specific RF frequency associated with a DL carrier phase measurement: - Option 1: the center RF frequency(ies) of the configured DL PRS bandwidth(s) for the carrier phase measurements - FFS: the details on how the DL PRS bandwidth(s) is configured - Option 2: the RF frequency of any specific subcarrier within DL PFL - FFS: the details on how the specific subcarrier is determined. - Option 3: LMF configures one or more than one candidate RF frequencies for the carrier phase measurement, and one RF frequency in the configuration is selected by the UE for the carrier phase measurement. - FFS: the details on on how LMF configures the candidate RF frequencies and how the configuration is selected by UE - Option 4: LMF configures one or more than one candidate RF frequencies for the carrier phase measurement, and one or more than one RF frequencies in the configuration is selected by the UE for the carrier phase measurement. - FFS: the details on on how LMF configures the candidate RF frequencies and how the configuration(s) is selected by UE |
| Samsung3 | Support option 2. |
| Apple | Support Option 1 |
| CATT | Support Option 1. We are also fine with other options. |
| Intel | Option 3 was proposed by motivating CP measurements outside of MGs and in INACTIVE mode. However, we are wondering why Option 1/2 cannot solve that problem? Also, for Option 2, perhaps we could generalize it similar to the latest update for Option 1 to possibility of multiple subcarriers? - Option 2: the RF frequency of any specific subcarrier(s) within DL PFL - FFS: the details on how the specific subcarrier(s) is/are determined. It’d be good to understand the motivation for Option 4. Is it just an extension of Option 3 to multiple RF frequencies and with the same motivation as Option 3? If so, we have the same question as above regarding delta compared to Option 1. We are also curious on how this proposal relates to Proposal 5-1. Lastly, a couple of small typos for FFS bullets under Options 1/4: should use “is/are”. |

(H)(Round 1) Proposal 2.3-3

- The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center RF frequency of the UL carrier that contains the SRS for positioning purpose.

| Company | comments |
| --- | --- |
| vivo | We would like to confirm the center of RF frequency is the center of SRS transmission other than the center of carrier, that is say the center may be not the same for different SRS configurations. For example, if the SRS bandwidth is smaller than the bandwidth of the UL carrier and the center of SRS transmission is not equal to the UL carrier, the center will be different from the center RF frequency of the UL carrier |
| Huawei, HiSilicon | Same comment as DL part. The centre RF frequency should require some discussion, and could be subject to multiple interpretations. Interpretation 1: the centre RF frequency is the frequency centre of the SRS BW, meaning if SRS RE range is 0 – 47 (4 RB), the centre frequency is between 23 or 24 or 23.5? Interpretation 2: the centre RF frequency is the centre of the RF bandwidth. Since the RF bandwidth could be up to gNB implementation, it could be anywhere. |
| Nokia/NSB | Support. To address Huawei’s comment we could simply remove “RF” after center. |
| CATT | Support. |
| Lenovo | Supportive for a common alignment on the definition of a center frequency for a UL CP measurement |
| OPPO | We think the center freq should be center freq of SRS, i.e., the interpretation 1 by HW |
| Ericsson | similar comment as vivo. |
| Samsung | We prefer to have more discussion on this proposal. There are cases when the TRP reports CP for multiple frequencies. In this case, carrier phase would be for different frequencies. There is also the option that the UE indicates the frequency CP measurement. FL: This is for default option. If RAN1 reaches agreement for multiple frequencies, we will further define other RF frequencies. |
| InterDigital | We support the proposal. |
| Qualcomm | Similar comment as vivo |
| Intel | Support. We are also fine to clarify it as center of SRS/SRSp BW. |
| FL | ### (H)(Round 1) Proposal 2.3-3 (updated for 1st GTW session) - The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as one of the following (down-selection) - Option 1: the center frequency of the UL carrier that contains the SRS for positioning purpose. - Option 2: the center frequency of the bandwidth of the SRS for positioning purpose. |
| LGE | ### We are fine with supporting the proposed option, but we have similar concern as DL case. Our preference is to discuss this proposal together with proposal 2.3-4, at least to support reporting the carrier phase measurement with one RF frequency. ### FL: Please see my response for proposal 2.3-1. |

### (H)(Round 2) Proposal 2.3-3

- The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as one of the following (down-selection)

  - Option 1: the center frequency of the UL carrier that contains the SRS for positioning purpose.

  - Option 2: the center frequency of the transmission bandwidth of the SRS for positioning purpose.

  - Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them.

| Company | comments |
| --- | --- |
| Nokia/NSB | Option 2. |
| OPPO | Option 2. |
| Huawei, HiSilicon | Option 2 |
| ZTE | Support Option 2. Compared to option 1, the center frequency of the transmission bandwidth of SRS is more representative for UL SRS phase measurement as the bandwidth of UL carrier is wider than the transmission bandwidth of SRS. |
| vivo | Option 2 |
| Intel | We are ok with Option 2. However, the note seems a copy-paste from the DL counterpart and should probably be updated in context of UL. In any case, we have similar comments as for Proposal 2.3-1 regarding such a note. FL: - Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them. |
| Spreadtrum | Support option 2. |
| Xiaomi | Support option 2 |
| CATT | Support option 2 |
| InterDigital | We support Option 2. |
| Samsung2 | Option 2 |
| FL | It seems no company support Option 1. In that case, we can remove it. The FL is updated as follows for further comment: ### (H)(Round 2) Proposal 2.3-3 (updated) - The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as - the center frequency of the UL carrier that contains the SRS for positioning purpose. - Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them. |

### (H)(Round 3) Proposal 2.3-3 (Revision 1)

- The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center frequency of the transmission bandwidth of the SRS for positioning purpose.

  - Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them.

| Company | comments |
| --- | --- |
| Nokia/NSB | Support. |
| Huawei, HiSilicon | OK |
| vivo | Maybe we should support option 2 other than option 1 |
| Qualcomm | At round2 there 2 options, all replies were in favor of option 2 and FL agreed to remove option 1 as a result, but the proposal in round3 seems to be a typo where option2 was removed instead of option1. |
| LGE | According to the FL’s summary, majority supports option 2 but it seems like option 1 is captured in revised proposal. Also, as we commeted in Proposal 2.3-1, we think “if RAN1 agrees to introduce them” in the note seems not required. The note says “open to further discussion whether ~” and we think it already includes the meaning that we will discuss whether to introduce additional methods or not. |
| ZTE | Yes, as vivo pointed, the previous comments show no company support option 1, this proposal should be option 2. |
| FL | Thanks for all of the comment. It was my mistake. I have revised the proposal as follows: ### (H)(Round 3) Proposal 2.3-3 (Revision 1) - The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center frequency of the transmission bandwidth of the SRS for positioning purpose. - Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them. |
| Samsung3 | It seems that option 2 that had the majority support was removed. We support the following: Option 2: the center frequency of the transmission bandwidth of the SRS for positioning purpose. |
| Xiaomi | Support the latest updated proposal |
| Apple | Fine with the latest proposal |
| CATT | Fine with the latest proposal |
| Intel | OK |

### (M)(Round 1) Proposal 2.3-4

- The following options can be further considered for the specific RF frequency associated with a UL carrier phase measurement (down-selection):

  - Option 1: the center RF frequency of a configured bandwidth of the SRS for positioning purpose.

  - Option 2: the RF frequency of any specific subcarrier within a UL carrier that contains the SRS for positioning purpose.

| Company | comments |
| --- | --- |
| InterDigital | We support the proposal and have the preference for option 2 for the same reason as for proposal 2.3-2. |
| LGE | ### The bandwidth of the SRS transmitted by the PRU may different from that of the target UE. In order to apply double differential method, the RF frequency between the CPM measured from the SRS transmitted by the target UE and the CPM measured from the SRS transmitted by the PRU should be aligned. To aligned the RF frequency between SRS resources transmitted from different UEs, including PRU, option 1 would be more preferable. However, to support Option 1, all SRS resources should be configured to have a bandwidth that includes the center frequency of the UL carrier. From this perspective we think another option should be considered Option 3: LMF configures the RF frequencies for the carrier phase measurements FL: Please see my responses for Proposal 2.3-2. |

### (M)(Round 2) Proposal 2.3-4

- The following options can be further considered for the specific RF frequency associated with a UL carrier phase measurement (down-selection):

  - Option 1: the center RF frequency of a configured bandwidth of the SRS for positioning purpose.

  - Option 2: the RF frequency of any specific subcarrier within a UL carrier that contains the SRS for positioning purpose.

    - FFS: How the specific subcarrier is determined.

  - Option 3: LMF requests one or more than one candidate RF frequencies for the carrier phase measurement, and one RF frequency in the configuration is selected by the TRP for the carrier phase measurement.

| Company | comments |
| --- | --- |
| Nokia/NSB | Same comment as for 2.3-2. |
| Locaila | Same comment as the proposal 2.3-2. We prefer option 2 with following modification. - The following options can be further considered for the specific RF frequency associated with a UL carrier phase measurement (down-selection): - Option 1: the center RF frequency of a configured bandwidth of the SRS for positioning purpose. - Option 2: the RF frequency of any specific (carrier+subcarriers) within a UL carrier that contains the SRS for positioning purpose. - FFS: How the specific subcarrier is determined. - Option 3: LMF requests one or more than one candidate RF frequencies for the carrier phase measurement, and one RF frequency in the configuration is selected by the TRP for the carrier phase measurement. As for LGE comment, we agree on the point that RF frequency of PRU and UE should be aligned. Otherwise the double-difference method will fail. That’s one additional reason why we do not support double-difference method. |
| IIT Kanpur, CEWiT | We prefer option 2. |
| Huawei, HiSilicon | Same as DL. We prefer to allow LMF to request one or multiple and UE report one or multiple according to the request. |
| Intel | Same comment as for Proposal 2.3-2. |
| Spreadtrum | Same comments as DL carrier phase measurement (Proposal 2.3-2). |
| CATT | Same comment as for 2.3-2. |
| Samsung2 | Option 2 |
| FL | We may consider how to modify the proposal after we conclude the discussion for Proposal 2.3-2. |
| LGE | Same comment as for proposal 2.3-3. |

## Timing information of carrier phase measurements

The issue of whether/how the measurement timing is defined was discussed in RAN1#112 without a final conclusion. From the proposals submitted in this meeting, we have the following:

- Option 1: The measurement timing of carrier phase measurement can be defined as the measurement timestamp, presented by frame/subframe/slot/symbol indexes/numbers.

  - Supported by: Huawei, HiSilicon, CATT, Samsung

- Option 2: The start of the slot or subframe or frame containing the

  - Supported by: Samsung

- Option 3: inferred from the existing reporting of NR-TimeStamp

  - Supported by: Qualcomm, LGE (can be multiple timestamps), MTK

NR-TimeStamp defined in TS 37.355 has the granularity of a slot. If it is used as the timestamp for carrier phase measurement, it implies that it cannot support reporting for the carrier phase per OFDM symbol, which may not be ideal for carrier phase measurements due to the fact that carrier phase for each OFDM symbol in a slot can be different. For instance, if there is a 0.1ppm frequency drift in the UE oscillator, the phase change between two OFDM symbols can be as much as 0.3/14=0.02 Hz for a 3GHz carrier. In addition, if the minimum granularity is a slot, the UE/TRP needs to accurately derive the carrier phase at the start of a slot, based on the carrier phase measurement obtained from the OFDM symbol(s) within the same slot. Sometimes, this can be very difficult, e.g., if there is only one DL PRS symbol in the slot.

(H)(Round 1) Proposal 2.4-1

- Adopt one of the following options:

  - Option 1:

    - The timing of a carrier phase measurement is the start of a slot, or the start of a subframe, which contains the DL PRS or UL SRS used for the carrier phase measurement.

    - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp for carrier phase measurements.

  - Option 2:

    - The timing of a carrier phase measurement is the start of an OFDM symbol, or a slot, or a subframe, which contains the DL PRS or UL SRS for the carrier phase measurement.

    - NR-TimeStamp, currently defined in TS 37.355, should be expanded to include the OFDM symbol index in a slot, as the timestamp for carrier phase measurements.

| Company | comments |
| --- | --- |
| vivo | I am not sure which measurement can be used to calculate the positioning if multiple carrier phases will be reported for a path of a resource. In addition, the difference between symbols is more like an error, we prefer to determine one measurement from a resource |
| Huawei, HiSilicon | Option 2 in general. For option 2, we do not prefer to use “start” because the start of OFDM symbol is CP; “the start of” can be removed. FL: I am wondering if we need to distinguish it. The impacts of UE/TRP oscillator or other factors may have not much difference. Removing “the start of” for an OFDM symbol may be fine. But removing “the start of” for an slot or subframe may be significant. |
| ZTE | Prefer option 1. In general, the carrier phase measurement result is reported together with other measurement results. And the current timestamp in TS 37.355 should be supported and reused for carrier phase measurement. |
| Xiaomi | Prefer Option 1 to reuse the legacy definition. |
| Nokia/NSB | We support Option 2. |
| CATT | We support Option 2. |
| Lenovo | Prefer Option 1 in line with legacy measurements, but can also live with Option 2. |
| OPPO | Prefer Option 1 to reuse the existing definition. Do not see the expanding in Option 2 can bring any benefit. |
| Ericsson | Prefer Option 1 |
| Samsung | We should discuss what we mean by the “start” is for the received signal or a reference time of the UE. In the transmitter of the PRS, carrier phase is “0” after the cyclic prefix (CP). This based on the following equation in TS 38.211: When the signal arrives at the receiving device, the phase is also “0” after the CP. We can define the phase of the received signal at a fixed time – that doesn’t depend on the propagation delay. The phase measurement would then be a function of the propagation delay. |
| Qualcomm | Similar view as vivo, we prefer one measurement per PRS resource, and we think identifying this PRS resource should be sufficient to determine the timing of the measurement, since the precise OFDMsymbols it occupies is then known |
| Intel | Same view as vivo and Qualcomm. |
| FL | It seems we have a clear majority to support Option 1. For vivo’s comment, yes, the difference of carrier phase is caused mainly by oscillator drift. The issue is that the impacts of the error are different for different OFDM symbols (or time instances). If we do not define clearly the timestamp for the measurements, say the impact of the errors may not be cancelled out completely even when double differential method is used. |
| LGE | We are fine with both options, but slightly prefer option 1. The most important issue on the timing information for the carrier phase measurement is double differencing. Instead of narrowing timing information of the timestamp, we can consider to support some time duration where phase continuity of the Tx/Rx side could be guaranteed. If so the level of the timestamp would not be a big matter. Besides, we would like to have clear understanding on the proposal. In the legacy DL-TDOA measurement reporting, the time stamp for the RSTD measurement is reported. If the carrier phase measurement is reported together with the RSTD measurement, we should define more details, whether the time stamp indicates timing for both TOA and RSCP measurement is performed or it indicates one of the measurement timing. This question comes from the observations that the RSTD measurement can be obtained by multiple samples (e.g. M=4) while utilizing multiple samples for a phase measurement would not be useful since the phase variation can be largely varied even with small UE mobility is assumed. From this perspective, we need more discussion on how to convey the time stamp information for the carrier phase measurement reporting, especially for the combined measurement reporting. If it is assumed that the time stamps specifies the time instance at which the TOA and RSCP measurements is performed, option 1 is preferable because it can be easily applicable to the RSTD measurement. However, we still need further discussion how to determine the time stamp if different number of samples are used between RSTD and RSCP/RSCPD measurements. If it is assumed that the time stamp for a combine measurement report indicates timing at which the RSCP/RSCPD measurement is performed, both option 1 and option 2 can be considered. FL: My understanding is that the purpose of the timestamp is to indicate when the measurement is valid. For legacy measurement, the accuracy of the measurement is relatively, it may not make much difference for a timestamp to be the start of the subframe or the end of the subframe, since the difference of the legacy timing measurements between the start and the end of the subframe is within the expected accuracy range (or error range). For CP measurement, it may be much more important, since the difference of the carrier phase start and the end of the subframe can be exceed the accuracy range |

FL Comments:

It seems we have a majority support of reuse existing timestamps for carrier phase measurements with a slot-level granularity. My concern is that for CP measurements, as I commended before, could be considerably different for different OFDM symbols. For NR carrier phase positioning, we do not support carrier phase tracking. Thus, sometimes it may be difficult for a receiver to derive the carrier phase for the start of a slot based on the carrier phase(s) measured from one or more DL PRS/UL SRS OFDM symbols within the slot.

(H)(Round 2) Proposal 2.4-1

- Adopt one of the following options:

  - Option 1:

    - The timing of a carrier phase measurement is the start of a slot, or the start of a subframe, which contains the DL PRS or UL SRS used for the carrier phase measurement.

    - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp for carrier phase measurements.

  - Option 2:

    - The timing of a carrier phase measurement is the start of an OFDM symbol, or the start of a slot, or the start of a subframe, which contains the DL PRS or UL SRS for the carrier phase measurement.

    - NR-TimeStamp, currently defined in TS 37.355, should be expanded to include the OFDM symbol index in a slot, as the timestamp for carrier phase measurements.

| Company | comments |
| --- | --- |
| Nokia/NSB | We prefer option 2 but can accept option 1 either with the first sub-bullet or with the understanding that we will further discuss the timing of the phase measurement. |
| OPPO | Support Option 1. It is to reuse the existing definition. The option 2 introduce extra complexity but we do not see the benefits with the cost of extra complexity |
| Huawei, HiSilicon | We understand that the phase could be different on different symbols with CFO/Doppler, which is why we think a nominal symbol index is needed for the timestamp. However, we think that a symbol index is sufficient, but not “start” of something. Therefore, we support Option 3: - Option 2: - The timing of a carrier phase measurement is an OFDM symbol, or a slot, or a subframe, which contains the DL PRS or UL SRS for the carrier phase measurement. - NR-TimeStamp, currently defined in TS 37.355, should be expanded to include the OFDM symbol index in a slot, as the timestamp for carrier phase measurements. FL: For an OFDM symbol, it may not have much difference of the carrier phase at the start of OFDM symbol or after the cyclix prefix or at the end of the OFDM symbol. But, for a slot or a subframe, the difference of the carrier phase at the start and the end of slot/subframe can be significant. |
| ZTE | Support Option 1. Share the same view with OPPO. CP is generally reported together with other measurement reports, it’s more reasonable to use the same timestamp definition. |
| vivo | Same view as OPPO and ZTE |
| Intel | It’s still not clear how exactly a very accurate reporting of the measurement timing will help the LMF when the measurement instances are subject to the granularity of PRS resource occasions. Thus, simply being able to refer to corresponding PRS resource(s) used for measurements should suffice. FL: My understanding is that the purpose of the timestamp is to indicate when the measurement is valid. For legacy measurement, the accuracy of the measurement is relatively, it may not make much difference for a timestamp to be the start of the subframe or the end of the subframe, since the difference of the legacy timing measurements between the start and the end of the subframe is within the expected accuracy range (or error range). For CP measurement, it may be much more important, since the difference of the carrier phase start and the end of the subframe can be exceed the accuracy range. |
| Spreatrum | Support Option 1. |
| Xiaomi | We can understand that the carrier phase may change more than one cycle in different symbol. We prefer Option 1 to reuse the legacy definition and define a default rule that the timing of a carrier phase measurement is the last symbol of the PRS reousrce. |
| CATT | Support Option 2. |
| Samsung2 | Option 2 can be the starting of further discussion. Option 1 leads different phase shift for different symbols. There should be a fixed time reference in the UE/TRP used to determine the phase (independent of the receive signal timing). |
| FL | ### In FL’s undersatdning, a reported value of the carrier phase measurement may not be valid for whole time in a slot or a subframe. For example, the carrier phases at the start and the end of a subframe can be significant different Thus, it is important to indicate clearly the time when the measurement is valid. The proposal is modified as follows for further discussion. ### (H)(Round 2) Proposal 2.4-1 (Updated) - Adopt one of the following options for the timestamp associated with a carrier phase measurement (down-selection in RAN1#113): - Option 1: - the start of a slot, or the start of a subframe, which contains the DL PRS or UL SRS for the carrier phase measurement. - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp for carrier phase measurements. - Option 2: - the start of an OFDM symbol, or a slot, or a subframe, which contains the DL PRS or UL SRS for the carrier phase measurement. - NR-TimeStamp, currently defined in TS 37.355, should be expanded to include the OFDM symbol index in a slot, as the timestamp for carrier phase measurements. - Note: A timestamp reported with a carrier phase measurement indicates the time at which the reported carrier phase measurement is valid. It does not necessarily mean the time when the DL PRS/UL SRS is received. It is up to UE/TRP’s implementation on how to provide the timestamp for a carrier phase measurement, which is measured from the DL PRS/UL SRS contained in an OFDM symbol, or a slot, or a subframe. |

### (H)(Round 3) Proposal 2.4-1

- Adopt one of the following options for the timestamp associated with a carrier phase measurement (down-selection in RAN1#113):

  - Option 1:

    - the start of a slot, or the start of a subframe, which contains the DL PRS or UL SRS for the carrier phase measurement.

    - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp for carrier phase measurements.

  - Option 2:

    - the start of an OFDM symbol, or a slot, or a subframe, which contains the DL PRS or UL SRS for the carrier phase measurement.

    - NR-TimeStamp, currently defined in TS 37.355, should be expanded to include the OFDM symbol index in a slot, as the timestamp for carrier phase measurements.

- Note: A timestamp reported with a carrier phase measurement indicates the time at which the reported carrier phase measurement is valid. It is up to UE/TRP’s implementation on how to provide the timestamp for a carrier phase measurement, which is measured from the DL PRS/UL SRS contained in an OFDM symbol, or a slot, or a subframe. The timestamp may or may not be the time when the DL PRS/UL SRS is received by a UE/TRP.

| Company | comments |
| --- | --- |
| Nokia/NSB | Gnerally we are okay with the proposal. We prefer option 2 but can accept option 1. The new note is a bit confusing to us. If we go with option 2 and the UE sets a symbol level time stamp wouldn’t that not imply the UE measured the DL PRS in that symbol? FL: There are cases that we may need to consider: 1) if a UE measures a number of OFDM symbols in a slot and the UE provides one carrier phase measurement. In this case, I assume it will be up to UE how to provide timestamp for the measurement and make sure the carrier phase is valid at the timestamp. |
| Huawei, HiSilicon | As FL mentioned, there should be no difference within a OFDM symbol, then we do not need “start” anyhow. For Option 2, the “start of” only applies to OFDM symbol, or also to slot, subframe. FL: For one OFDM symbol, I would agree the indicate of start may not be needed. |
| Vivo | We are not sure the note is needed. And okay to remove “start” FL: If we removing the start, may be we can say the “first symbol”? |
| Qualcomm | We still think the comments by QC and Intel in round1, repeated by Intel in round2, have not been addressed. Regarding FL response to Intel in round2, we agree that if we had PRS placed at the beginning and end of a subframe, the measurements from those PRS may be different. But we only measure the phase using the PRS that are present. For example, if PRS was only present at the end of the subframe, that is all we would be able to measure – and there is no need to ‘extrapolate’ that measurement back to the equivalent measurement for the beginning of the subframe. Any such extrapolation may be via a deterministic formula based on number of OFDM symbols etc, which can equivalently be done by LMF after receiving the report. Thus, it should suffice to indicate which PRS Resource was measured. FL: It is good the Qualcomm agrees the measurements can be different for the beginning and end of a subframe. If we only “indicates which PRS Resource was measured”, then the question is that a PRS resource can occupy up to 12 OFDM symbols. How to the LMF kknows the carrier phase corresponds to which OFDM symbol? |
| LGE | We would like to have more clear understading on this proposal. First, “the time at which the reported carrier phase measurement is valid” means the time where measurement is performed? Or is there any intend meaning? FL: My understanding is not. For example, for UE Rx-Tx timefference measurements are defined based on the timing of start of subframe. But, the UE derived the Rx time based on the DL PRS within the subframe. Second, we are not sure the necessity of “the start of” in the definition. According to the FL’s reply to the Nokia above, the UE may measure multiple OFDM symbols within a slot and UE may select a carrier phase measurement at an OFDM symbol among them. If so, it seems like “the start of” is not a valid expression. FL: If the “start of slot/subframe” is an issue, will it be a better wording by using “the first OFDM symbol”? Finally, as we discussed in our tdoc, details about timestamp information when carrier phase measurements are reported together with a legnacy measurements should be discussed. We think it is related to the timestamp definition for the carrier phase measurement. For example, TOA value can be measured across the slot for higher accuracy, while measuring the RSCP across slot seems not a proper idea. FL: Again, this is related to the issue of how long the measurement is considered to be valid. For TOA value, the TOA changes within a slot is normally much smaller than the TOA error range, and thus, we can consider the TOA value is valid for whole slot. |
| ZTE | We think the note is duplicate for saying “A timestamp reported with a carrier phase measurement indicates the time at which the reported carrier phase measurement is valid.” In current TS 37.355, the IE NR-TimeStamp defines the UE measurement associated time stamp. The NR-TimeStamp contains the measurement time, that definitely means the CP measurement is valid at the reported time stamp. FL: It would be fine to me if we have comment undersatdning that the reported carrier phase is valid the time indicated by the NR-TimeStamp. For that we need to claridy that when the granularity of the NR-TimeStamp is slot, does it mean the reported carrier phase is valid for whol time duration of the slot, or only at a particular time with the slot. We saw FL mentioned that “a reported value of the carrier phase measurement may not be valid for whole time in a slot or a subframe”, but the reported timestamp is a slot or a subframe, how to clarify whether the reported CP is valid or not? FL: That is exactly my conver. If the carrier phase measurement can be valid for whole time duration in a slot or a subframe, we don’t have an issue. If not, the question is carrier phase measurement can be valid at which time (or which OFDM symbol) And a suggestion for the description: FL: My consideration is no need to consider CP since the change of the carrier phase should be very small in a CP duration. It is up to UE/TRP’s implementation on how to provide the timestamp for a carrier phase measurement, which is measured from the DL PRS/UL SRS contained in an OFDM symbol (if option 2 is supported), or a slot, or a subframe. |
| FL | Thanks for all of the comments. I am wondering instead of saying the start of slot/subframe, is it better to say the first OFDM symbol? ### (H)(Round 3) Proposal 2.4-1 (Revised 1) - Adopt one of the following options for the timestamp associated with a carrier phase measurement (down-selection in RAN1#113): - Option 1: - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp for carrier phase measurements. - Note: It implies the reported carrier phase measurement is valid at least at the time of the first OFDM symbol of the slot indicated by the NR-TimeStamp. - Option 2: - NR-TimeStamp, currently defined in TS 37.355, is expanded to include the OFDM symbol index in a slot, as the timestamp for carrier phase measurements. - Note: It implies the reported carrier phase measurement is valid at least at the time of the OFDM symbol indicated by the NR-TimeStamp. |
| Samsung3 | Option 2 can be the starting of further discussion. With option 2, we don’t think that the note is needed. There should be a fixed time reference in the UE/TRP used to determine the phase (independent of the receive signal timing). |
| Xiaomi | With Option 2, if the RSCP is reported together with RSTD, does it mean two timestamp will be reported? From our point of view, legacy definition can be reused by defining a default rule based on UE capability, e.g., N symbols after the first symbol of PRS. We are fine with this proposal but we also think the note is not needed for Option2, so we suggest to move the note to the sub-bullet of Option 1. |
| CATT | Either option in the latest FL revision is fine to us. |
| Intel | Given the discussion and agreement we had on Proposal 2.1-1 to introduce RSCP, we are even less convinced that a super-accurate knowledge of the time of the measurement is very usable at the LMF. Unless PRSs on essentially same symbols are measured and reported by UE(s), it is not going to be helpful anyway. The FL provided an example of a 12-symbol PRS. In this case, what is the idea of the operation and UE behavior? That the UE measures CP only using one of the symbols? If so, why provide such a PRS configuration for CP measurements? On the other hand, if the UE may process up to all 12 symbols, what is the interpretation of a reported CP measurement? On a related note, how exactly is the “validity” determined? To summarize, we are still not convinced that anything beyond a PRS resource level information is going to be of practical use at the LMF. |

# Carrier phase measurement reporting

Background

Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements.

Submitted Proposals:

| Huawei, HiSilicon [3] | - Proposal 4: For NR UE-assisted carrier phase positioning, support a UE to report carrier phase measurements together with RSTD to LMF. Proposal 5: For carrier phase measurements reporting together with UE Rx-Tx timing difference, support UE to report RSCPD. Proposal 8: Phase measurement associated with an SRS port for MIMO SRS is supported. |
| --- | --- |
| Vivo[4] | - No standalone carrier phase positioning is introduced in Rel-18 positioning. - To enable UE-based and UE-assisted NR carrier phase positioning (CPP), both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - Enable a UE to report DL CPD together with the RSTD measurement if CPD is supported - Enable a UE to report DL CP together with the UE Rx-Tx time difference measurement if CP is supported - Note: Rx-Tx carrier phase measurement is not pursued in Rel-18 - Enable a TRP to report UL CP together with the RTOA measurement and TRP Rx-Tx time difference measurement |
| OPPO[6] | Proposal 1: Support both DL carrier phase measurement and DL carrier phase difference measurement: - The DL carrier phase measurement is reported together with DL RSTD measurement - The DL carrier phase difference is reported together with UE Rx-Tx Time difference. Proposal 4: The TRP reports the carrier phase measurement of SRS resource for positioning together with existing positioning measurement. |
| Spreadtrum Comm[7] | Proposal 2: The carrier phase can be reported together with the current positioning measurements, such as RSTD and TDOA. |
| CATT[8] | Proposal 2: Support LMF to request a UE to report the DL CP and/or the DL CPD, or define the fixed association between DL CP/CPD and legacy measurements, i.e., DL CP is reported together with UE Rx-Tx time difference measurement, and DL CPD is reported together with RSTD measurement. Proposal 9: For NR DL CPP, support a UE to report DL carrier phase measurements (RSCP or RSCPD) together with RSTD or UE Rx-Tx time difference to the LMF. Proposal 20: For NR UL CPP, support gNB/TRP to report UL RSCP together with RTOA or gNB Rx-Tx time difference to the LMF. |
| Intel[9] | Proposal 1 - Support DL RSCPD reporting as an independent positioning method. - FFS: Exact conditions and associated assistance information to resolve integer ambiguity. Proposal 2 - Introduce DL RSCPD for DL CPP considering its resiliency to initial phase at UE Rx. - FFS: Reporting of RSCP corresponding to a target TRP – e.g., DL RSCPD may be sufficient if reporting of DL CP measurements is defined as an independent positioning method. - LMF configures a target UE with a reference cell that is independent from serving cell. Proposal 3 - For DL CPP, support reporting of DL RSCPD along with at least RSTD and, if DL RSCPD is defined as independent positioning method, then also along with UE Rx-Tx time difference measurements. - FFS: Other combinations, e.g., if DL RSCP is also supported. Proposal 9 - LMF can request a gNB to report UL RSCP along with UL RTOA and/or gNB Rx-Tx time difference measurements. Proposal 10 - Multiple SRS/SRSp resources may be used to determine the UL carrier phase. - LMF may optionally request a gNB to perform measurements that are restricted to within a time window. |
| Xiaomi[11] | - Proposal 4: Support the reporting of the carrier phase measurements together with the measurement results of DL RSTD from UE, and together with the measurement results of UL RTOA from TRP. |
| Samsung[12] | Proposal 13: For carrier phase positioning measurement, support the following approaches for reporting carrier phases measurements to LMF: - Carrier phase measurements included in legacy measurement reports for RSTD, RTOA and Rx-Tx time difference. - Standalone carrier measurement report. Proposal 14: For carrier phase positioning measurement, consider including the following fields in the measurement report: - Carrier phase measurement - Reference signal ID - Time stamp - Quality indicator - Phase continuity indicator since last measurement Proposal 15: In case of reporting phase measurements of multiple frequencies within a PFL/carrier, support reporting the associated frequency index, FFS, details on how to determine the frequency index. |
| CMCC[13] | Proposal 1: The following information should be considered in the CPP measurement reporting: - The timestamp - The measurement quality - Reference TRP / DL PRS resource set / DL PRS resource(s) ID |
| CMCC[13] | Proposal 4: For NR carrier phase positioning, support to report carrier phase measurements together with DL-RSTD, UL-RTOA, and gNB/UE Rx-Tx time difference to LMF. |
| Lenovo[14] | Proposal 2: If carrier phase difference measurements are supported, consider configuration and reporting along with legacy RSTD measurements. Proposal 3: RAN1 to further discuss the specification of CP measurements in terms of a standalone method for DL and UL CP measurements. |
| ZTE[15] | Proposal 2: Support UE/TRP reporting carrier phase measurements together with the RSTD and/or RTOA measurements to LMF. |
| Fraunhofer [16] | Proposal 2: Support reporting the following measurements: - UL CP along with an UL-RTOA or gNB Rx-Tx measurement - DL CP along with an DL-RSTD or UE Rx-Tx measurement - DL CPD along with a DL-RSTD measurement. |
| InterDigital[[17] | Proposal 1: For DL-based positioning or DL & UL based positioning, support reporting of the carrier phase measurements together with the at least RSTD, UE Rx-Tx time difference measurements and RSRP measurements Proposal 2 : Support reporting of the carrier phase-based measurements alone. Proposal 3: Support using the same reference TRP for joint carrier phase and DL-RSTD measurement reporting. Proposal 8: Support the UE to select and report the reference TRP for phase difference measurements. |
| Apple[18] | Proposal 2: RAN1 should support the following to enable CPP in Release 18: • Support joint legacy and CPP positioning and for a UE, optionally stand-alone CPP Proposal 4: The CP can be reported with RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements. This is subject to UE capability in the case of UE-based positioning methods. |
| Ercisson[19] | Proposal 8 Add support to report DL PRS RSCPD for the first path with the DL-TDOA positioning method. Proposal 9 Add support to report UL SRS RSCP for the first path with the UL-TDOA positioning method. |
| Qualcomm[20] | Proposal 7: UE doesn’t report standalone RSCP measurement report, instead RSCP is reported along with the measurements of DL TDOA or DL RTT positioning methods. Proposal 8: The granularity of the phase reporting should be fine enough so that the error it produces is much smaller than the UE’s positioning error. |
| LGE[22] | Proposal 8: Support enabling a UE/TRP to report carrier phase measurements without the legacy positioning measurements to LMF Proposal 12: Support enabling a UE/TRP to report CPM together with the legacy positioning measurements, including DL RSTD, UL RTOA, and UE/TRP Rx-Tx time difference. |
| IIT Kanpur, CEWiT[24] | Proposal 9: For carrier-phase-based positioning, the difference between the carrier phase of multiple subcarriers, along with the frequency difference with respect to the subcarriers, should be reported, which could be introduced as a new measurement. Proposal 12: The additional information apart from the carrier phase measurement should include. - The specific frequency domain resource(carrier, subcarrier, PFL) corresponding to the carrier phase measurement(s). - Measurement reference point, for example, an antenna ID or an antenna stim ID. - PRS/SRS resource ID and resource set ID. - The receiver beam ID - Inter frequency offset if multiple carrier phase measurements are reported, or carrier phase differences are reported. - LoS/NloS indication - RSRP/RSRPP of the paths corresponding to carrier phase measurement - The error sources corresponding to the measurements, such as Timing Error Group, Phase Error Group. |

## Reporting of carrier phase measurements with legacy measurements

In RAN1#112, it was agreed at least supporting a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF. It is still FFS on which legacy positioning measurements can be reported with carrier phase measurements. Based on the submitted proposals, most of the companies are supportive to report the DL carrier phase measurements with RSTD and UE Rx-Tx timing difference, and UL carrier phase measurements with RTOA and gNB Rx-Tx timing difference.

(H)(Round 1) Proposal 3.1-1

- Support enabling a UE to report DL carrier phase measurements together with RSTD and/or UE Rx-Tx time difference measurements to LMF

  - FFS: the DL carrier phase measurement is RSCP and/or RSCPD

- Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF

| Company | comments |
| --- | --- |
| vivo | We prefer to modify FFS as a sub-bullet so that no other carrier phase measurement will be introduced in Rel-18 - Support enabling a UE to report DL carrier phase measurements together with RSTD and/or UE Rx-Tx time difference measurements to LMF - the DL carrier phase measurement is one or more of following - RSCP - RSCPD FL: It is still FFS on whether RSRCP or RSCPD or both are supported |
| Huawei, HiSilicon | Regarding reporting RSCP/RSCPD with RTT, we would prefer to clarify that reporting phase measurement with RTT positioning method should not require both UE and gNB to report the phase measurement. - Support enabling a UE to report DL carrier phase measurements together with RSTD and/or UE Rx-Tx time difference measurements to LMF - FFS: the DL carrier phase measurement is RSCP and/or RSCPD - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF - Note: The report of DL carrier phase measurement with UE Rx – Tx time difference does not require the report of UL carrier phase measurement with gNB Rx – Tx time difference, and vice versa. |
| Xiaomi | Support |
| CATT | Support. HW’s comment is fine to us. |
| Lenovo | Supportive of the proposal. One CP measurement type should be downselected in the FFS bullet |
| OPPO | Support And The note suggested by HW seems not needed. |
| Ericsson | Support. |
| Samsung | Fine in principle. However, we should not exclude standalone CP reporting. Therefore, we would like to update the proposal as follows: - Support enabling a UE to report DL carrier phase measurements together with RSTD and/or UE Rx-Tx time difference measurements to LMF - FFS: the DL carrier phase measurement is RSCP and/or RSCPD - This doesn’t preclude standalone carrier phase measurements reporting, if agreed. - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF - This doesn’t preclude standalone carrier phase measurements reporting, if agreed. |
| InterDigital | We support the proposal. |
| Qualcomm | Support with the additional notes from vivo, Huawei, and Lenovo. We prefer downselection to RSCP (i.e., remove RSCPD), but this looks like a good first step proposal |
| Intel | Support. |
| FL | ### The proposal is modified based on the comments for 1st GTW session based on Huawei’s comments. For Samsung’s comment, I assume it is okay to add but it may not be necessary, since “standalone” reporting means “standalone”, not combined measurement reporting. ### (H)(Round 1) Proposal 3.1-1 (updated for the 1st GTW session) - Support enabling a UE to report DL carrier phase measurements together with RSTD and/or UE Rx-Tx time difference measurements to LMF - FFS: the DL carrier phase measurement is RSCP and/or RSCPD - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF - Note 1: The report of DL carrier phase measurement with UE Rx – Tx time difference does not necessarily require the report of UL carrier phase measurement with gNB Rx – Tx time difference, and vice versa. - Note 2: This doesn’t preclude standalone carrier phase measurements reporting, if a new type of carrier phase measurement is agreed. |
| LGE | ### We are generally fine with the updated proposal. Regarding the standalone carrier phase measurement report, we are supportive to consider it and fine to capture a note. However, we are not sure what is the intention of “if a new type of carrier phase measurement is agreed”. In my understanding, RSCP/RSCPD reporting without the legacy positioning measurement could be a standalone carrier phase measurement report. ### FL: We can change the note to :” Note 3: Whether to support standalone DL RSCP or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other types of measurements, can be further discussed.” |

FL Comments:

Considering that Proposal 2.1-1 now includes DL  joint reporting, the updated Proposal 3.1-1 will only consider the UL joint reporting.

(H)(Round 2) Proposal 3.1-1

- Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF

- Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference.

- Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting.

| Company | comments |
| --- | --- |
| Nokia/NSB | Support. |
| IIT Kanpur, CEWiT | We support |
| Huawei, HiSilicon | OK |
| vivo | Support and prefer to remove note 2. |
| Intel | OK |
| Spreadtrum | Support the proposal without note 2. |
| Fraunhofer | Support |
| Xiaomi | Support |
| CATT | Support |
| Locaila | We don’t support. We continue to argue that legacy time measurement can not provide sufficient precision to estimate the integer number of the carrier phase even if wide bandwidth of 100 MHz is used. As far as we know, no company has showed the result how accurately the legacy method can determine the exact integer number. Most companies brought the simulation result under the assumption that the ideal integer number can be estimated, and no company verified the performance of the integer number estimation itself. We indirectly proved the precision of the standardone method through experimental results when subcarriers are used. Many companies proposed virtual carrier method (a.k.a wide-lane method) to supplement the inaccuracy of the legacy method. However, such method can only be applied if an operator owns both L1 and L2 carriers, like GNSS. How many operator owns two carrier bands nicely separated more than 200MHz apart, i.e. 3.5GHz and 3.7GHz? If the f1, f2 virtual carriers (i.e. L1 and L2) are derived from a single carrier band (i.e single PFL), (f1 – f2) frequency is nothing more than a subcarrier frequency. This means the virtual carrier method is the same as the standardone method. If so, within the single carrier band (i.e PFL), we can make (f1 – f2) small or large to obtain long subcarrier and short subcarrier. This way, we can narrow down the search range and stimate the integer number. After all, the virtual carrier method can be viewed as one implementation of the standalone methods, if it is used within a single carrier band. We argue whatever ToA, TdoA, and integer number information can be obtained using subcarrier phase measurements in standalone way, and the legacy measurement is not necessary. FL: . RAN1 have made the following agreement. The proposal here is to complete the “FFS”. Where to support standalone is under discussion in Section 3.2. Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements. |
| Samsung2 | Support. Prefer to remove Note 1. FL: The motivation of Note 1 is that we can support Multi-RTT + DL CPP as well as support Multi-RTT + UL CPP. |

### (H)(Round 3) Proposal 3.1-1 (no update in comparison with Round 2)

- Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF

- Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference.

- Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting.

| Company | comments |
| --- | --- |
| Nokia/NSB | Okay. |
| Huawei, HiSilicon | OK |
| Qualcomm | OK |
| LGE | Generally ok, but we prefer to capture Note 3 that you mentioned in a previous round. Note 3: Whether to support standalone DL RSCP or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other types of measurements, can be further discussed FL: I am not ure LGE’s comment applies here. This is TRP reporting.gNB does not report DL RSCP or DL RSCPD. |
| Samsung3 | OK |
| Xiaomi | Ok |
| Apple | OK |
| CATT | OK |
| Intel | OK |

## Standalone reporting of carrier phase measurements

Whether to standalone carrier phase positioning was discussed in RAN1#112 without conclusion. For this meeting, the opinions of the interested companies are:

- No standalone carrier phase reporting/positioning in Rel-18

  - Supported by: vivo, Qualcomm

- Introduce standalone carrier phase reporting/positioning in Rel-18

  - Supported by: Samsung, Apple, LGE, Locaila

The issue is related to “whether to introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier”, which is discussed in Section 6. We can further discussed whether to support standalone carrier phase reporting/positioning in Rel-18 once we reach a conclusion in Section 6. In the meanwhile, we can also check the views from other companies on this issue.

### (Round 1) Q&A 3.2-1

- What is your view for the supporting of standalone carrier phase reporting/positioning in Rel-18?

| Company | comments |
| --- | --- |
| Huawei, HiSilicon | No such need. |
| ZTE | Prefer do not support standalone carrier phase positioning. It is imperative for LMF/UE to estimate integer ambiguity with other legacy positioning method. |
| OPPO | Prefer not to support the standalone. Per the study, using legacy positioning measurement is needed to resolve the integer ambiguity issue. |
| Ericsson | Don’t see the need for it. |
| Samsung | Prefer to support. |
| InterDigital | We support the standalone carrier phase reporting. The details for standalone carrier phase should be FFS including how to resolve integer ambiguity, inter-gNB synchronization, the number of paths to report, reporting of LoS/NloS indicator with it, etc. |
| Qualcomm | No need, and no motivation. First path timing would already have been determined for reporting carrier phase, and there is no benefit from not including the timing information in the report. |
| Intel | Support. We are open to consider associated assistance information to resolve integer ambiguity/provide estimate of pseudorange. The main motivation is that the assistance information to estimate pseudorange can be determined by a UE/TRP using any positioning method and not limited to specific RAT-based ones per configuration. |
| NTT DOCOMO | Not support standalone carrier phase positioning in Rel-18. |
| LGE | Support. First, I would like to emphasize that my understanding on the “standalone carrier phase reporting” is different from the FL’s comment above. Our intention is to support “RSCP/RSCPD only reporting” without RSTD/Rx-Tx time difference measurement report. As we commented in our tdoc, PRU with fixed known location does not require to report other positioning measurements for integer ambiguity resolution. Also, if the target UE has small mobility and the LMF already have fine initial integer search range, reporting additional time-based positioning measurement would not be useful anymore. |
| FL | So far we don’t have a majority support for the standalone carrier phase reporting. Let us continue collecting the comments in this meeting to see if we want to have a conclusion for this issue. |
| Nokia/NSB | No need for standalone in Rel-18. Even GNSS never does only phase. |
| Locaila | Strongly support standalone method. Why do we need to follow GNSS method and make it worse, even when we can do better than GNSS ? For more additional information, we proposed in R1-2303774, unique integer number can be obtained by standalone method, using the combination of subcarrier and carrier phase information as below. N=round fc/fq∙ϕs/2π-ϕc/2π In the above equation, round(...) is an integer number conversion function, (fc / fq) is the integer multiple ratio of the carrier frequency divided by the subcarrier frequency. 𝜙s/2𝜋 and 𝜙c/2𝜋 are normalized carrier and subcarrier phases in the range of 0~1 (or -0.5 ~ +0.5). The method was proven in our field experiment. We welcome any discussion on the above standalone method for integer ambiguity. UE, gNB, LMF may determine the phase cycle number without rely on any legacy time measurement information. Further, we also proposed sidelink application of carrier phase RTT method in R1-2303775. It employs above standalone method to determine the phase-cycle number between vehicles. Considering the vehicle use-cases, standalone method efficient. |
| Spreadtrum | Not support standalone carrier phase positioning. |
| Fraunhofer | we are open to discussing standalone carrier phase positioning. However, given that we need to prioritize our efforts, we suggest that the standalone CPP discussion be given a lower priority for now. |
| CATT | We are open for the discussion. But, we share the similar view as Fraunhofer that this can be a low priority for now. |
| FL | Based on the feedback, the opinions of the interested companies are: - No standalone carrier phase reporting/positioning in Rel-18 - Supported by: Huawei, HiSilicon, ZTE, OPPO, Ericsson, vivo, Qualcomm, NTT DOCOMO, Nokia/NSB, Spreadtrum (11 companies) - Introduce standalone carrier phase reporting/positioning in Rel-18 - Supported by: Samsung, InterDigital, Intel, Apple, LGE, Locaila (6 companies) - Open for discussion with low priority: Fraunhofer, CATT) (2 companies) Since we still don’t have a majority support for the standalone carrier phase reporting, but it does get multiple companies support, let us keep the discission open for now with a low priority in this meeting. In addition, this issue may be related to Proposal 5-2 under discussion. Maybe we can wait to see the progress of Proposal 5-2 to decide how to pproceed with standalone carrier phase reporting. |
| Apple | To clarify, we are supportive of standalone only if it is based on a UE capability and not to be supported by the UE by default. |

# Differential CPP and PRU

Background

Agreement RAN1 will continue discussions on what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple Ues, including a target UE and a PRU; and to support simultaneous transmission of SRS for multiple Ues, including a target UE and a PRU. RAN1 will include the outcome in an LS to SA2 and RAN2, and cc RAN3. Note: Enhancements might or might not have RAN1 specification impact.

Submitted Proposals:

| Huawei, HiSilicon [3] | Proposal 6: For UE-based carrier phase positioning, the PRU CPP measurement can be provided to the UE in the assistance data. Proposal 7: In addition to the PRU CPP measurement, additional information in the assistance data may include PRU location and AoD of PRU to each TRP. - Proposal 8: For SRS transmission, gNB can configure the SRS of the target UE close to the SRS of the PRU in time domain. - Proposal 9: For SRS transmission, LMF can send the recommended configuration information of SRS resources to indicate the gNB to configure neighboring SRS resources for the target UE and the PRU. - Proposal 10: With regards to any new procedures to allow two Ues to measure the same DL-PRS simultaneously, RAN4 may be involved. |
| --- | --- |
| Vivo[4] | Proposal 4: For carrier phase measurement reporting, explicit double difference carrier phase measurement is not needed. Proposal 6: - An MTW can be configured to UE, the parameters of an MTW can include - The MTW length - The start time and periodicity of MTW Proposal 7: - For target UE, the MTW length should be within the MTW length of PRU measurement, the UE_MTW periodicity can be multiple of UE_MTW periodicity - E.g., UE_MTW periodicity=N* PRU_MTW periodicity Proposal 8: - For PRU, the MTW should be configured based on multiple UE requirements, for example, the PRU_MTW periodicity can be the greatest common divisor of UE_MTW periodicity - E.g.,PRU_MTW periodicity= GCD(UE1_MTW periodicity , UE2_MTW periodicity, …,UE N_MTW periodicity) Proposal 9: - For one shot measurement report, adding an indication together with the scheduled location time to indicate the UE needs to report the measurement instance within the nearest MTW instance after the scheduled location time. Proposal 10: - From UE side, the multiple Ues can transmit the SRS simultaneously without RAN enhancement - The potential enhancement for UL simultaneous transmission and measurement is NRPPa enhancement. It may include - adding PRU identity in requested SRS transmission characteristics to request the SRS configuration simultaneously for target UE and the PRU - introduce MTW to align the measurement window of gNBs for multiple Ues |
| CATT[8] | Proposal 14: For simultaneous measurements of the same DL PRS for target UE and PRU(s), at least support the following two procedures: - LMF requests the serving and neighboring gNB/TRPs to transmit the DL PRS within the DL PRS resources (resource sets) with the same periodicity and offset for the target UE and the PRU(s). - LMF or serving gNB/TRP requests the target UE and the PRU(s) to measure the DL PRS within the DL PRS resources (resource sets) within a configured/pre-defined time window and report the measurements. Proposal 25: For simultaneous transmission of SRS for target UE(s) and PRU(s), support LMF to request the serving gNB(s) to configure the target UE(s) and PRU(s) to transmit UL SRS within a relatively small time interval. Proposal 26: For simultaneous measurements of the SRS(s) from the target UE and the PRU(s) by different gNB/TRPs, at least support the following two procedures: - The coordination of between LMF and the serving gNB(s) of the target UE and the PRU(s) to schedule the SRS transmission within a time window, where the granularity of the time window can be OFDM symbol or slot. - LMF requests the serving and neighboring gNB/TRPs of the target UE and the PRU(s) to receive SRS transmitted by the target UE and PRU(s) within a configured time window and to report the corresponding UL RSCP measurements. |
| Intel[9] | Proposal 16 - To enable simultaneous (sufficiently-close-in-time) SRS/SRSp transmissions from a target UE and a PRU, the following objectives are pursued for potential NRPPa enhancements: - Objective 1: LMF should be able to direct a serving gNB to schedule such “sufficiently-close-in-time” transmission of SRS/SRSp from a target UE and PRU. - Objective 2: LMF should be able to direct NG-RAN nodes to perform measurements corresponding to these SRS/SRSp resources. - RAN1 to discuss further on approaches to address these objectives for eventual recommendations to RAN2/SA2. Proposal 17 - To enable simultaneous measurements on same DL PRS by a target UE and a PRU, the following objective is pursued for potential LPP enhancements: - Objective 3: LMF should be able to request a UE (target UE, PRU) to perform measurements on specific DL PRS resources or at least on DL PRS resources occurring within an indicated time window. - RAN1 to discuss further on approaches to address the above objective for eventual recommendations to RAN2/SA2. |
| Nokia, NSB[10] | Proposal 3: For CPP when the UE is requested to measure at a Scheduled Location time that the UE shall do so or send an error message to the LMF indicating it can’t do so (e.g., due to conflict). Send an LS to RAN2. Proposal 4: Ask RAN3 if NRPPa can support Scheduled location time in Rel-18 as part of the CPP work. Proposal 5: Send an LS to RAN3 stating that RAN1 finds it necessary for NR CPP that the TRP shall measure at a Scheduled Location time or send an error message to the LMF indicating it can’t do so (e.g., due to conflict). Proposal 6: Support a target UE requesting that a PRU (via LMF) performs a DL carrier phase measurement at a specific time. Proposal 7: RAN1 should consider introducing a PRU measurement validity in order to limit the signaling overhead of NR CPP. |
| Samsum[12] | Proposal 9: RAN1 considers how to provide PRU or PRU-like functionality under existing system, e.g., how to determine a PRU and apply the PRU during the positioning procedure. |
| CMCC[13] | Proposal 3: For UE-based DL positioning, the phase errors between TRPs should be provided from LMF to a target UE. |
| Lenovo[14] | Proposal 6: Support LMF configuration of phase-related errors for accurate DL/UL CP measurements. Proposal 9: Integer Ambiguity could be resolved using assistance data from neighboring Ues with perfectly known locations when single PFL and carrier is considered. Proposal 10: RAN1 to consider phase errors mitigation using data modeling and interpolation at LMF or virtual PRU as a solution for low density PRU deployments. |
| ZTE[15] | Proposal 11: For DL UE-based carrier phase positioning, the ARP or carrier phase error of the TRP can be delivered by LMF. Proposal 12: For UE-based carrier phase positioning, the carrier phase error can be reported by PRU to LMF. Proposal 13: For UE-based carrier phase positioning, support LMF forwarding the carrier phase measurement results of the PRU to the target UE. Proposal 14: For supporting a mechanism to ensure simultaneous measurements of DL PRS for a target UE and a PRU, the following options can be supported: -LMF configures the same measurement time slot(s)/window(s) for PRU and UE. -LMF configures the same PRS instance(s) to be measured by PRU and UE. Proposal 15: For supporting a mechanism to ensure simultaneous transmissions of UL SRS for a target UE and a PRU, LMF can request a list of slots or windows for SRS transmission. |
| Fraunhofer[16] | Proposal 5: Consider the signaling to the UE phase-coherency information related to the different TRPs to inform on the applicability for the processing carrier measurements. |
| InterDigital[[17] | Proposal 13: Support PRU for NR carrier phase-based positioning. Proposal 14: Study a mechanism to transfer measurements from the PRU to the UE in UE-based positioning. Proposal 15: Study mechanisms to support simultaneous transmission of SRS between UE and PRU for UL-based positioning methods and DL & UL positioning method Proposal 16: Study mechanisms to support simultaneous reception of PRS between UE and PRU for DL-based positioning methods and DL & UL positioning method |
| Apple[18] | Proposal 2: RAN1 should support the following to enable CPP in Release 18: - Support joint legacy and CPP positioning and for a UE, optionally stand-alone CPP - Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF. For the UE, this should be subject to a UE capability. - A UE should be explicitly configured as a PRU for double difference CPP subject to its capability - Support simultaneous measurements of the same DL-PRS for multiple Ues, including a target UE and a PRU; and support simultaneous transmission of SRS for multiple Ues, including a target UE and a PRU. The measurements/transmissions should be time sensitive i.e. at a specific time or within a specific time duration Proposal 7: Feedback from a UE/TRP to an LMF may include CP and Rel-17 positioning feedback from the target UE and CP only from a PRU. Note that support for simultaneous measurement/transmission of PRS/SRS should be enabled. |
| Ericsson[19] | Proposal 10 LMF can provide a Target UE with phase correction factors to enable double differentiation. Proposal 11 It should be possible to provide a UE with updated correction factor with variable frequency. Proposal 12 Investigate how phase correction factors can be provided in a resource efficient way. |
| Qualcomm[20] | Proposal 10: Assistance data for UE-based carrier phase positioning includes phase differences between the PRS resources of the TRPs, including timing (e.g., PRS occasion) of the PRS resources whose phase difference is being reported. Proposal 11: Subject to UE capability, include in RequestLocationInformation, a specific list of PRS resources which UE is required to measure, together with a window duration during which that list is valid. Proposal 12: Introduce signaling for LMF to be able to inform the TRPs to measure specific SRS resources in a specific time-window. Proposal 13: Introduce signaling for LMF to be able to request from the serving TRP of a specific slot-offset to be used for the SRS for Positioning configuration (i.e., enhance the “Requested SRS Transmission Characteristics” signaling from the LMF to the serving gNB). |
| LGE[22] | Proposal 9: To support simultaneous measurements of the same DL-PRS for multiple Ues, including a target UE and a PRU, at least following information should be introduced: - For a target UE - List of PRS resources recommended to monitor - List of recommended measurement timing - For a PRU - List of PRS resources that PRU shall monitor - List of measurement timing that PRU shall perform DL PRS measurements Proposal 10: Support LMF to inform a UE multiple list of PRS resources and timing for simultaneous measurements - For UE assisted positioning, assistance data can be used by LMF to recommend/indicate the UE to determine PRU index amongst the list. - For UE based positioning, LMF can transmit information associated with PRU location to the UE |
| Locaila[23] | (Proposal 1) Network notification should be delivered to UE when a separate means of PRS phase synchronization is provided, so that the UE does not need to wait for additional initial phase error correction information. (Proposal 2) At least in the TDD system, a method that can calculate the relative clock phase offset between TRPs by listening to the PRS signal transmitted between TRPs must be supported. FFS: How to harmonize the PRS muting procedure and the PRS listening procedure |
| MTK[25] | Proposal 2-1: The simultaneous measurements may not be enabled if PRU is not deployed Proposal 2-2: The simultaneous measurements between Ues are to aim at particular measurement types Proposal 2-3: When a UE performs the simultaneous measurements, it is not expected to average over instances for reporting Proposal 2-4: Any sophisticated mechanism to intentionally control UE’s measurement behavior is not preferred. A simple mechanism to indicate the simultaneous measurements could be considered. For example, a configured start time and a duration that each DL-PRS instance within the duration may be measured |

## Simultaneous PRS measurements and SRS transmission

In RAN1#112, it was agred that RAN1 continues discussions on “what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple Ues, including a target UE and a PRU; and to support simultaneous transmission of SRS for multiple Ues, including a target UE and a PRU.”

Regarding the simultaneous measurements of the same DL-PRS, many companies have provided their suggestions (e.g., [3][4][8][9][10][12][15][17][18][20]), which can be summarized as follows:

- Target UE request PRU (via LMF) to measure DL PRS at a specific time

  - Supported by: Nokia,

- LMF requests the gNBs to configure PRS resources (for the target UE and the PRU) within a time window

  - Supported by: CATT, Nokia

- LMF indicates target UE and the PRU to measure the measurement instance within a time or time window:

  - Supported by: vivo, CATT, Intel, Nokia, ZTE, Qualcomm, LGE, MTK

- Ask RAN4 to involve in any new procedures to allow two Ues to measure the same DL-PRS simultaneously:

  - Supported by: Huawei, HiSilicon

Regarding the simultaneous measurements of the same DL-PRS, many companies have provided their suggestions (e.g., [3][4][8][9][10][12][15][17][18][20]), which can be summarized as follows:

- LMF requests the gNBs to configure SRS resources for the target UE and the PRU in a time window

  - Supported by: Huawei, HiSilicon, vivo, CATT, Intel, ZTE, Qualcomm

- Serving gNB configures the SRS resources for the target UE and the PRU in a time window:

  - Supported by: Huawei, HiSilicon, vivo, CATT

- LMF request gNBs to measure SRS from the target UE and PRU in a time window:

  - Supported by: vivo, CATT, Intel, Qualcomm

- Note: No additional enhancement is needed in UE side.

  - Supported by: vivo

Regardless which options are supported, it will have impact on higher layer signalling.

(H)(Round 1) Proposal 4.1-1

- To enable simultaneous measurements on same DL PRS by a target UE and a PRU, the support the following enhancements:

  - Enabling LMF to request the gNBs, whose DL PRS signals may be measured by the target UE and the PRU(s), to configure the transmission of the DL PRS resources within an indicated time window.

  - Enabling LMF to request the Ues, including target UE and PRU(s), to perform measurements on DL PRS resources occurring within an indicated time window.

  - FFS: the details of the configuration of the indicated time window (e.g., the start time, duration, periodicity for the time window), as well as the relationship with the Scheduled Location time.

- Send LS to RAN2/RAN3/SA2 on RAN1’s decision

| Company | comments |
| --- | --- |
| vivo | The PRS is a cell-specific signal, we don’t think the configuration of PRS needs to be enhanced. For us, the enhancement will be about PRS measurement configuration only. So, we suggest removing the first sub-bullet. |
| Huawei, HiSilicon | For the first subbullet, why do we need it. Can it be implemented using on-demand PRS. For the second subbullet, we do not think it can enable simultaneous measurements on the same DL PRS. |
| ZTE | We prefer configure multiple time windows for gNBs/Ues. Because in some cases, the terminal may be occupied with other higher priority measurement/transmission tasks, and a single time window may not be sufficient. Therefore we suggest update “an indicated time window” as “indicated time windows”. |
| Xiaomi | As for the second sub-bullet, what is the relationship with PPW? FL: This may need further discussion. The intention of the time windows here is to coordinate the measurement time between target UE and PRU. PPW is configured for a single UE. |
| CATT | Support. ZTE’s comment is fine to us. |
| Lenovo | Supportive but this would need to be extended to all legacy positioning measurements , not only RSCP to avoid multiple windows configured for different positioning measurements. |
| OPPO | 1st bullet: it is not needed. The on-demand PRS function can provide the function. 2nd bullet: we are fine with ZTE’s comment. |
| Ericsson | Don’t support. The proposal suggests that PRU and Target UE measurements resp. transmissions need to be done close in time because otherwise the PRU information is not useful for double-differentiation. But how close in time is needed? We don’t recall any conclusions on this from the SI. For GNSS-RTK, the shortest possible periodicity of SSR phase corrections is 1s (see TS37.355, ssrUpdateInterval). FL: During SI, the simulation assumption does not consider measurement time gaps between PRU and Target UE. It implies simultaneous measurement in my understanding, |
| Samsung | We don’t see a need for the first sub-bullet. A DL PRS transmitted can be received by multiple devices. For the second sub-bullet, LMF can request the same DL PRS for measurement at the UE and PRU. |
| InterDigital | We support the proposal. |
| Qualcomm | Similar comment as Huawei. Second sub-bullet needs explicit indication of resources to measure, not just a time-window. |
| Intel | Agree with others that the first sub-bullet may not be needed; fine with the rest. |
| LGE | Similar view with other companies that the first sub-bullet is not needed. For the second sub-bullet, we have similar view with ZTE. We prefer to have multiple time windows indication. Beside the timing window, we also need to consider set of the PRS resource indication for simultaneous measurements. Even the time window is configured, PRU may not capable to measure all PRS resources within a time window at once. Hence, for the simultaneous measurements from PRS resource perspective, it seems require to indicate set of PRS resources that PRU measures (or prioritizes to measure) to the Ues. |

FL Comments:

The comments may be summarized as follows:

- For the first bullet, multiple companies think it is not needed. So, my thinking is to remove it for now, assuming the simultaneous DL PRS transmission can be done via implementation.

- For the second bullet, multiple companies (e.g.,Huawei, Qualcomm, Samsung) propose to include indication of DL PRS resources to measure. Some companies (e.g., ZTE, LGE) also propose multiple time windows.

(H)(Round 2) Proposal 4.1-1

- To enable simultaneous measurements on same DL PRS by a target UE and a PRU, the support the following enhancements:

  - Enabling LMF to request the Ues, including target UE and PRU(s), to perform measurements on indicated DL PRS resources occurring within indicated time window(s).

  - FFS: the details of the configuration of the indicated time window (e.g., the start time, duration, periodicity for the time window), as well as the relationship with the Scheduled Location time.

- Send LS to RAN2/RAN3/SA2 on RAN1’s decision

| Company | comments |
| --- | --- |
| Nokia/NSB | We think another option is to ask the UE to measure a particular occurrence/time and then if the UE can’t do so it can report that it didn’t measure phase in that occurrence. FL: Maybe we can add a note that it is still up to UE when/how to measure. |
| OPPO | We do not support to include “indicated” in the first sub-bullet. To measure the DL PRS, the system provides the configuration and request the measurement type, at the UE side, it is up to UE implementation and measurement to pick the ‘best’ DL PRS resource for corresponding positioning measurement. It is not feasible for the system to demand the UE to measure a given PRS resource, which the system does not know if it has good or bad quality. FL: Maybe we can add a note that it is still up to UE when/how to measure. |
| Huawei, HiSilicon | More discussion is needed based on our understanding. |
| ZTE | Suggest revise the wording: - To enable simultaneous measurements on same DL PRS by a target UE and a PRU, the support the following enhancements: - Enabling LMF to request the Ues, including target UE and PRU(s), to perform measurements on indicated DL PRS resources occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s) (e.g., the start time, duration, periodicity for the time window(s)), as well as the relationship with the Scheduled Location time. - Send LS to RAN2/RAN3/SA2 on RAN1’s decision FL: Okay. |
| Vivo | We share the same view with OPPO for the description of ‘indicated’. In addition, we have some concerns about how and why to indicate DL PRS resources, with UE mobility, the measured PRS resource will be changed, so how to indicate the appropriate PRS resources for mobility UE. And the error can be removed when part of the PRU can measure the resources. That means PRU 1 may not measure the resource, but other PRU may measure it, and the error also can be removed. |
| Intel | Mostly OK. We can further consider UE reporting whether or not it performed the measurement within the indicated time window(s) as part of the measurement report. |
| Spreadtrum | We’re still a little confused about how can the time window ensure that PRU and UE are measuring the same PRS? |
| Xiaomi | We think it depends on the length of the time wondow. If it is very short, e.g., 1 slot, and the PRS configuration for UE and TRP are transmitted in the slot, it will be more likely that UE and PRU will measure the same PRS resource. Thus from our point of view, it can be supported by the legacy PPW that a common PPW is configured to both UE and PRU. |
| CATT | Support |
| InterDigital | We have the same view as Intel. |
| Samsung2 | Fine with the direction of the proposal. When we say “LMF to request the Ues … to perform measurements on indicated DL PRS resources”, doesn’t this imply time resources on which he DL PRS is configured. We prefer to remove “occurring within indicated time window(s)”. FFS can also be removed. FL: Ideally, we like to the UE to provide the measurement at the indicated resources. Maybe we can add a note to clarify the UE does not have to follow the request. |
| FL | Multiple companies (Nokia, OPPO, vivo, Intel, etc.) have the similar comments that UE should have a final say about when and how measure DL PRS. The proposals is updated with the consideration of the comments: ### (H)(Round 2) Proposal 4.1-1 - To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the Ues, including target UE and PRU(s), to perform measurements on indicated DL PRS resources occurring within indicated time window(s). - Note: A UE does not have to follow the request from LMF, but should follow the request in a best effort manner. - FFS: the details of the configuration of the indicated time window (s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. - Send LS to RAN2/RAN3/SA2 on RAN1’s decision |

### (H)(Round 3) Proposal 4.1-1 (Revised 1)

- To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements:

  - Enabling LMF to request the Ues, including target UE and PRU(s), to perform measurements on indicated DL PRS resources occurring within indicated time window(s).

    - Note: The UE is expected to make its best effort to comply with the request from LMF, but it is not mandatory.

  - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time.

- Send LS to RAN2/RAN3/SA2 on RAN1’s decision

| Company | comments |
| --- | --- |
| OPPO | We still have concerns on the wording “indicated”. How it works and what is the benefit for “indicated” is not clear. From our understanding, it does not work and it does not provide any benefit. Generally each UE measures the PRS and report the measurement results obtained from the ‘best’ PRS resource. But the ‘best’ would vary from UE to UE and also vary for one single UE from time to time due to the UE mobility and also environment variation. The consequence is the indication from LMF does not provide any valuable information to the UE. For example, LMF indicates PRS resource A. If the A happens to be a good resource for UE #1, UE #1 would report the measurement results obtain from this PRS resource A no matter whether LMF indicates or not. But if the A happens to have poort quality to UE #2, UE #2 would not report the measurement results of A even if LMF indicated A. Therefore, we still suggest to remove indicated. The added Note sub-bullet is applicable to the “indicated time window(s)” we think it can be kept. - To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the Ues, including target UE and PRU(s), to perform measurements on indicated DL PRS resources occurring within indicated time window(s). - Note: The UE is expected to make its best effort to comply with the request from LMF, but it is not mandatory. - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. - Send LS to RAN2/RAN3/SA2 on RAN1’s decision FL: In previous round discussion, some companies made the comment that it is not sufficient to indicate only the time windows. Let us wait to see if other companies’s opinions. |
| Nokia/NSB | Support. |
| Vivo | Same view as oppo |
| LGE | Support the proposal. Also we prefer to keep indicated DL PRS resources. Considering the PRU may not possible to measure all PRS resources at once due to the limited capability, it would be worth to have such information. If “indicated” is not preferealbe, we also fine with “recommended” as a compromise. |
| ZTE | Generally ok for the revised proposal, and also open to further study the details on the configuration. |
| Samsung3 | We still think that the DL PRS resources include time and frequency on which the DL resource is transmitted. Therefore, we prefer the following update - To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resources occurring within indicated time window(s). - Note: The UE is expected to make its best effort to comply with the request from LMF, but it is not mandatory. - FFS: the details of the configuration of PRS resource the indicated time window(s), e.g., the start time, duration, periodicity … for the time window(s), as well as the relationship with the Scheduled Location time. FL: The configured DL PRS resources may be periodic. Thus, The UE may not provide the measurement for each DL PRS respruce periodicicy. |
| Xiaomi | ‘Indicated DL PRS resource’ means it is possible there are multiple PRS resources in the time window? We think it can be avoid by PRS configuration. |
| Apple | Have same view as Oppo. Open to using “recommended” |
| CATT | Support. |
| Intel | We are supportive of this in principle. However, as noted in previous round, and especially in consideration of the new Note, we think the UE should indicate whether or not it performed the measurement within the indicated time window(s) as part of the measurement report. FL: In my view, the LMF knows whether UE has measurement within the indicated time window from the timestamp of the measurements. |
| FL | It seems the main issue is related to the “indicated” DL PRS resources. ### (H)(Round 3) Proposal 4.1-1 (Revised 1) - To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resources occurring within indicated time window(s). - Note: The UE is expected to make its best effort to comply with the request from LMF, but it is not mandatory. - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. - Send LS to RAN2/RAN3/SA2 on RAN1’s decision |

(H)(Round 1) Proposal 4.1-2

- To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements:

  - Enabling LMF to request the serving gNB(s) of the target UE and the PRU to configure the  transmission of the UL SRS resources from target UE and the PRU within an indicated time window.

  - Enabling LMF to request the serving gNB(s) and neighboring gNBs to measure the UL SRS resources from target UE and the PRU within an indicated time window.

  - FFS: the details of the time window (e.g., using new parameters for the start time, duration, periodicity for the time window, or use the existing message of Scheduled Location)

- Send LS to RAN2/RAN3/SA2 on RAN1’s decision

| Company | comments |
| --- | --- |
| Huawei, HiSilicon | We have some suggestions on the two subbullets - Enabling LMF to request the serving gNB(s) of a UE to configure the transmission of the UL SRS resources within the vicinity of a reference SRS configuration. - Note: The reference SRS resources may be the SRS configuration to a PRU. - FFS: Enabling LMF to request the serving gNB(s) and neighboring gNBs to measure the UL SRS resources from target UE and the PRU within an indicated time window. FL: I assume the intention of “vicinity of a reference SRS configuration” is one way to define the time windows. Maybe we can add : - FFS: the details of the configuration of the indicated time window (e.g., within the vicinity of a reference SRS configuration). |
| ZTE | Same as proposal 4.1-1 We prefer configure multiple time windows for gNBs/Ues. Because in some cases, the terminal may be occupied with other higher priority measurement/transmission tasks, and a single time window may not be sufficient. Therefore we suggest update “an indicated time window” as “indicated time windows”. |
| CATT | Support. ZTE’s comment is fine to us. For HW’s comment, maybe we only need a reference time for the time window, instead of reference SRS configuration. |
| Ericsson | Don’t support. The proposal suggests that PRU and Target UE resp. transmissions need to be done close in time because otherwise the PRU information is not useful for double-differentiation. But how close in time is needed? We don’t recall any conclusions on this from the SI. For GNSS-RTK, the shortest possible periodicity of SSR phase corrections is 1s (see TS37.355, ssrUpdateInterval). FL: During SI, the simulation assumption does not consider measurement time gaps between PRU and Target UE. It implies simultaneous measurement in my understanding, |
| Samsung | Fine with the first sub-bullet. Second sub-bullet is not needed, or can be FFS for more discussion. |
| InterDigital | We support the proposal. |
| Qualcomm | Support |
| Intel | Support. To the edits from Huawei – “within vicinity of a reference SRS configuration” is one way to implement a time window and in this regard, prefer the original wording with the change from “the target UE and the PRU” to “a UE”. |

FL Comments:

The comments may be summarized as follows:

- For the first  bullet, it seems majority companies are fine with it with some wording changes.

- For the second bullet, we may add FFS as suggested Huawei and Samsung.

(H)(Round 2) Proposal 4.1-2

- To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements:

  - Enabling LMF to request the serving gNB(s) of the target UE and the PRU to configure the transmission of the UL SRS resources from target UE and the PRU within an indicated time window.

    - FFS: the details of the time window (e.g., using new parameters for the start time, duration, periodicity for the time window, within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location)

  - FFS: Enabling LMF to request the serving gNB(s) and neighboring gNBs to measure the UL SRS resources from target UE and the PRU within an indicated time window.

- Send LS to RAN2/RAN3/SA2 on RAN1’s decision

| Company | comments |
| --- | --- |
| Nokia/NSB | Same comment as UL. FL: UL may be different. gNB schedule when UE to transmit and UE needs to follow it. |
| Huawei, HiSilicon | With regards to the request, we do not need explicitly mention target and UE, while each gNB allocates the SRS resource to its served Ues subject to LMF request on the SRS transmission characteristics, which is coordinated by LMF. In light of this, we prefer to update the proposal as - To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB(s) of a UE to configure the transmission of the UL SRS resources from the UE within an indicated time window. - FFS: the details of the time window (e.g., using new parameters for the start time, duration, periodicity for the time window, within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location) - FFS: Enabling LMF to request the serving gNB(s) and neighboring gNBs to measure the UL SRS resources from target UE and the PRU within an indicated time window. - Send LS to RAN2/RAN3/SA2 on RAN1’s decision FL: Okay |
| ZTE | Same as previous comments, i.e., an indicated time window -> indicated time window(s) FL: |
| Intel | We are ok with HW’s edits regarding “UE” as against target UE and PRU. However, it is not clear why we mark the second bullet as FFS if the LMF cannot ensure that gNBs actually measure these close-in-time SRS transmissions from multiple Ues? FL: I share the similar view. |
| CATT | Support |
| InterDigital | We are ok with the edits proposed by HW. |
| Samsung2 | Fine in principle. |

FL Comments:

The proposal is revised based on the comments. For companies that propose putting “FFS” in frat of “Enabling LMF to request the serving gNB(s) and neighboring gNBs…”, please also explain what is the concern.

### (H)(Round 3) Proposal 4.1-2 (Revision 1)

- To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements:

  - Enabling LMF to request the serving gNB of a UE to configure the transmission of the UL SRS resources from the UE within indicated time window(s).

    - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time

  - FFS: Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the UL SRS resources from the UE within indicated time window(s).

- Send LS to RAN2/RAN3/SA2 on RAN1’s decision

| Company | comments |
| --- | --- |
| Nokia/NSB | Support. |
| Huawei, HiSilicon | OK |
| Qualcomm | OK |
| LGE | fine |
| ZTE | Support. We think the wording for the first FFS can be similar to that of proposal 4.1-1 - FFS: the details of the time window(s), (e.g., using new parameters for the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time) FL: Okay. I make the changes for Revision 1 ### (H)(Round 3) Proposal 4.1-2 (Revision 1) - To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window(s), the start time, duration, periodicity for the time window, within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - FFS: Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the UL SRS resources from the UE within indicated time window(s). - Send LS to RAN2/RAN3/SA2 on RAN1’s decision |
| Samsung3 | Support in principle. |
| Apple | OK |
| CATT | Support. |
| Intel | OK |

## Assistance information for UE-based CPP

During the SI phase, using PRU to aid NR carrier phase positioning was inevstigated. In the downlink, a PRU acts as a UE that receives DL PRS reference signals to provide DL carrier phase measurements to the LMF. The double differential measurements can then be obtained by differencing the DL carrier phase measurements of the target UE with those of the PRU, which eliminates any common measurement errors. Similarly, in the uplink, a PRU acts as a UE that transmits UL SRS signals for positioning. UL carrier phase measurements obtained from the UL SRS signals of both the target UE and the PRU to the LMF by the TRPs.  Double differential measurements can then be obtained by calculating the difference between these UL carrier phase measurements, which eliminates any measurement errors.

The use of PRU measurements for suppprting differential UE-based CPP can be accomplished through two main approaches:

- Option 1: LMF forwards the carrier phase measurements reported from PRU to target UE in the assistance data (e.g., [3][8][15][17][20])

- Option 2: LMF first derives the error correction information based on  carrier phase measurements reported from PRU, and the send the error correction information to target UE for UE-based CPP (e.g.,[13][14][15][16][19]).

The implementation of the first approach is relatively simple and was evaluated during the SI phase. On the other hand, the second approach may be more efficient and reliable, especially when multiple PRUs are available in a PRU network. However, this approach was not evaluated during the SI phase, and further discussion is required regarding the content of the error correction as well as the effectiveness. One reasonable wayforwrd is to consider the first approach as a baseline approach and then further discuss the error corrections for the second approach as an optional feature.

In [23], it proposes network notification should be delivered to UE when a separate means of PRS phase synchronization is provided, and to support the method that can calculate the relative clock phase offset between TRPs by listening to the PRS signal transmitted between TRPs.

### (H)(Round 1) Proposal 4.2-1

- Support enabling LMF to forward the DL carrier phase measurement reported by a PRU and other PRU information to a target UE for UE-based carrier phase positioning in the positioning assistance.

  - Note: Whether DL carrier phase measurement is DL RSCP and/or DL RSCPD depends on which of them is (are) supported.

  - FFS: other PRU information, such as PRU location, to be included in the positioning assistance

| Company | comments |
| --- | --- |
| vivo | Support |
| ZTE | A small correction, the proposal can be updated as: - Support enabling LMF to forward the DL carrier phase measurement reported by a PRU and other PRU information to a target UE for UE-based carrier phase positioning in the positioning assistance data. - Note: Whether DL carrier phase measurement is DL RSCP and/or DL RSCPD depends on which of them is (are) supported. - FFS: other PRU information, such as PRU location, to be included in the positioning assistance data. |
| CATT | Support. |
| Lenovo | Support |
| OPPO | Do not support. PRU for DL carrier phase measurement is out of the scope of WI. It was investigated during SI but not included in the WI. FL: OPPO’s comment is unclear to me. RAN1 has agreed to “continue discussions on what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple UEs, including a target UE and a PRU”. Also, UE-based carrier phase positioning is included in WID. |
| Ericsson | Not support the current proposal. We are open to consider Option 2. Option 1 and Option 2 are mathematically equivalent; hence we don’t think that it is correct of FL to say that Option 2 was not evaluated during the SI. In fact, our implementation during the SI was more like Option 2. FL: Thanks for the comments. It would be great if Ericsson already evaluated the use of the differential corrections for UE for UE-based NR carrier phase positioning during Rel-18 SI. But, I haven’t the use of differential correction from LMF to UE for UE-based NR carrier phase positioning in TR 38.859. We have the following description from TR 38.859: “ The use of Positioning Reference Unit (PRU) to facilitate NR carrier phase positioning is studied. - For DL NR carrier phase positioning, a PRU works as a UE to receive the DL PRS reference signals and provide the DL carrier phase measurements to the LMF, where the double differential measurements can be obtained by the difference of the DL carrier phase measurements from the target UE and those from the PRU for eliminating the measurement errors. - For UL NR carrier phase positioning, a PRU works as a UE to transmit the UL SRS signals for positioning purpose. The TRPs provide the UL carrier phase measurements obtained from the UL SRS signals of the target UE and of the PRU to the LMF, where the double differential measurements can be obtained by the difference of these UL carrier phase measurements for eliminating the measurement errors. Anyway, I made the comment that using differential “may be more efficient and reliable, especially when multiple PRUs are available in a PRU network”. We can further disucss the content of the error correction as well as the effectiveness of the approach, as in Proposal 4.2-2. |
| Samsung | OK in principle. |
| InterDigital | We support the proposal. |
| Qualcomm | We think Proposal 4.2-2 is sufficient and is also in line with existing RTDInfo IE in assistance data for UE-based DL TDOA FL: Proposal 4.2-2 is another option in my view. It requires the phase correction to be accurate to less than a fraction of the cycles in my view to be effective. So, we may need some further discussion on it. |
| LGE | Support |

FL Comments:

It seems the majority companies support the proposal, but some companies consider the method proposed in Proposal 4.2-1 is a better approach.  Maybe we can combine them together for further discussion.

(H)(Round 2) Proposal 4.2-1

- For UE-based carrier phase positioning, support one or both of the following options:

  - Option 1: Enabling LMF to forward the DL carrier phase measurement reported by a PRU and other PRU information to a target UE for UE-based carrier phase positioning in the positioning assistance data.

    - Note: Whether DL carrier phase measurement is DL RSCP and/or DL RSCPD depends on which of them is (are) supported.

    - FFS: other PRU information, such as PRU location, to be included in the positioning assistance data

  - Option 2: Enabling LMF to provide, optionally, the differential corrections to a target UE for UE-based carrier phase positioning.

    - FFS: the details of the correction information, which may include the corrections for TRP timing errors, TRP phase errors, ARP/APC errors, etc.

| Company | comments |
| --- | --- |
| Nokia/NSB | We support option 1. |
| OPPO | We are ok with Option 2. And suggest to change the main bullet from “support one or both of” to “support one or both of …” We do not support Option 1 since the carrier phase measurement for PRU is not part of the WID To FL’s reply in last round: supporting carrier phase measurement in PRU was not agreed and not part of the WID. The agreement you referred to is only for the exsiting PRU function, not including the carrier phase measurement. FL: Regardless which option is used, there is a need to support PRU (or use the term another UE" as Huawei suggested. As a matter of factor, w/o the support of PRU to support double differential positioning, it is unclear to me how the NR CPP will work. If OPPO does not want to have the term PRU, we can consider using the suggestion from Huawei, to say “another UE”, which I am not sure if that is OPPO wants. |
| Huawei, HiSilicon | Option 1. PRU terminology can be replaced by “another UE”. |
| ZTE | Both options are fine for us. But we are wondering how the TRP timing errors can be calculated with the help of PRU’s carrier phase? TRP/ARP errors can be calculated by differential operations with the help of PRU carrier phase measurement. How the timing errors is calculated is our concern. If no solution is provided, we prefer to remove TRP timing errors in the FFS mentioned in option 2. FL: PRU can report other legacy measurements (e.g., RSTD) which can be used to derive the timing error corrections. |
| vivo | Option 1 |
| Spreadtrum | We are fine with two options. |
| Xiaomi | We prefer Option 2 to reduce the UE complexity and the signaling overhead. |
| CATT | We support option 1. But, we are open for further discuss for Option 2. |
| InterDigital | We support the proposal. |

FL Comments:

In [23], it proposes network notification should be delivered to UE when a separate means of PRS phase synchronization is provided, and to support the method that can calculate the relative clock phase offset between TRPs by listening to the PRS signal transmitted between TRPs, which can also be an option.

### (H)(Round 3) Proposal 4.2-1

- For UE-based carrier phase positioning, consider supporting one or more of the following options:

  - Option 1: Enabling LMF to forward the DL carrier phase measurement reported by a PRU and other PRU information to a target UE for UE-based carrier phase positioning in the positioning assistance data.

    - Note: Whether DL carrier phase measurement is DL RSCP and/or DL RSCPD depends on which of them is (are) supported.

    - FFS: other PRU information, such as PRU location, to be included in the positioning assistance data

  - Option 2: Enabling LMF to provide, optionally, the differential corrections to a target UE for UE-based carrier phase positioning.

    - FFS: the details of the correction information, which may include the corrections for TRP timing errors, TRP phase errors, ARP/APC errors, etc.

  - Option 3: Enabling LMF to provide a notification to UE indicating that DL PRS transmission is phase synchronization among TRPs.

| Company | comments |
| --- | --- |
| OPPO | We do not support the proposal since Option 1 is out of scope of WID. It did be dicussed during SI but it was not agreed to include in it in the WID. And the WID is The Option 1 does not align with the WID. We shall remove it from the proposal. - For UE-based carrier phase positioning, consider supporting one or more of the following options: - Option 1: Enabling LMF to forward the DL carrier phase measurement reported by a PRU and other PRU information to a target UE for UE-based carrier phase positioning in the positioning assistance data. - Note: Whether DL carrier phase measurement is DL RSCP and/or DL RSCPD depends on which of them is (are) supported. - FFS: other PRU information, such as PRU location, to be included in the positioning assistance data - Option 21: Enabling LMF to provide, optionally, the differential corrections to a target UE for UE-based carrier phase positioning. - FFS: the details of the correction information, which may include the corrections for TRP timing errors, TRP phase errors, ARP/APC errors, etc. - Option 32: Enabling LMF to provide a notification to UE indicating that DL PRS transmission is phase synchronization among TRPs. |
| Nokia/NSB | Sorry but the comment from OPPO is very confusing. UE-based CPP is clearly in the WID so Option 1 is clearly in the scope of the WID in our view. We do not support option 2 for the following reasons: - It creates new IEs as a function of the PRU measurements and then forwards them to the UE. This is not nearly as clean of a solution as simply forwarding the PRU measurements to the target UE. Whereas forwarding the measurements does not require any new IEs as the PRU measurement report will likely be exactly the same as that of a UE. - It also requires additional specification work as the value range and fields for these new IEs need to be worked out. - It potentially introduces additional errors depending on the granularity of the new IEs and also based on LMF implementation. We do not support option 3 as in our opinion this level of phase synchronization for CPP is not feasible in practice. Option 1 is the only way forward in our view. |
| LGE | Support the proposal, and we have similar view with Nokia that option 1 is in the scope of the WID. |
| ZTE | From our perspective, both option 1 and option 2 are ok for us. The PRU location information and potential errors can be provided by LMF via assistance data. And thanks for FL’s reply on the timing errors in option 2. But according to the timing error calculation method, it seems that this is more like the signaling related to RSTD, instead of CPP. In our opinion, it is out of study scope of CPP. For option 3, we are not sure about the necessity and feasibility, |
| Xiaomi | We prefer Option 2 to reduce the UE complexity and the signaling overhead. |
| OPPO | @Nokia/NSB and LG: Option 1 has “DL carrier phase measurement reported by a PRU” that is not part of the WID. We did not agree to include “carrier phase measurement of PRU” in WI. |
| Apple | Not clear on Option 3. Do you mean “phase synchronized”? |
| CATT | Support Option 1. We are also fine with Option 2. |
| FL | If seems many companies are not supportive for Option 3. One possible wayforward is to say other option is not precluded. ### (H)(Round 3) Proposal 4.2-1 (Revision 1) - For UE-based carrier phase positioning, consider supporting one or both of the following options: - Option 1: Enabling LMF to forward the DL carrier phase measurement reported by a PRU and other PRU information to a target UE for UE-based carrier phase positioning in the positioning assistance data. - Note: Whether DL carrier phase measurement is DL RSCP and/or DL RSCPD depends on which of them is (are) supported. - FFS: other PRU information, such as PRU location, to be included in the positioning assistance data - Option 2: Enabling LMF to provide, optionally, the differential corrections to a target UE for UE-based carrier phase positioning. - FFS: the details of the correction information, which may include the corrections for TRP timing errors, TRP phase errors, ARP/APC errors, etc. - Other option is not precluded |

### (H)(Round 2) Proposal 4.2-2

- Support the LMF to provide, optionally, the differential corrections to a target UE for UE-based carrier phase positioning.

  - FFS: the details of the correction information, which may include the corrections for TRP timing errors, TRP phase errors, ARP/APC errors, etc.

| Company | comments |
| --- | --- |
| Huawei, HiSilicon | With proposal 4.2-1, do we need 4.2-2 for the TRP timing errors, TRP phase errors? |
| CATT | We prefer further study it. |
| Lenovo | Support |
| Ericsson | Support |
| Samsung | More discussion is needed for this proposal. |
| InterDigital | We support the proposal. |
| Qualcomm | Support |
| Huawei, HiSilicon | This TRP phase error is calculated based on gNB ARP and PRU measurement, right? Then we think that the problem here is that gNB ARP may not be accurately obtained, and thus the corresponding TRP phase error correction information may also be biased. FL: I have the same concern. In order to resolve the integer ambiguity, it basically requires the errors in carrier phase measurements to be eliminated completely, which requires a very careful study of the error correction information. |
| FL | Continue the discussion in (H)(Round 2) Proposal 4.2-1 |

# Integer Ambiguity

Background

Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data.

Submitted Proposals:

| Huawei, HiSilicon [3] | Proposal 16: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF. Proposal 17: Do not introduce a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier. Proposal 18: Do not support a UE/TRP to report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF. Proposal 19: Do not support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. |
| --- | --- |
| vivo[4] | Proposal 11: - At least option 4 can be supported. - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. |
| OPPO[6] | Proposal 2: For the issue of integer ambiguity, support Option 1 and Option 4. Proposal 5: The TRP reports the UL carrier phase measurement of multiple different REs in SRS resource for positioning. |
| Spreadtrum Comm[7] | Proposal 3: The range or some alternative values of integer N should be provided/reported for carrier phase positioning. |
| CATT[8] | Proposal 10: For Rel-18 NR DL CPP, at least support a UE to report DL RSCP/RSCPD of multiple RF frequencies within a PFL to LMF (Option 1). Proposal 11: If the RF frequency of a specific subcarrier within a DL PFL is used for definition the specific RF frequency for DL RSCP/RSCPD measurement (Option 3 in Proposal 3), support LMF/gNB to indicate the specific RF frequency(ies) for a UE to report DL RSCP/RSCPD. The specific RF frequency(ies) apply to all DL RSCP/RSCPD measurements regardless the DL RSCP/RSCPD measurements are obtained from which OFDM symbol(s). Proposal 21: For Rel-18 NR UL CPP, at least support a gNB/TRP to report the UL RSCP measurements of multiple RF frequencies within a UL carrier to LMF (Option 1). Proposal 22: If the RF frequency of a specific subcarrier within a UL carrier is used for definition the specific RF frequency for UL RSCP measurement (Option 3 in Proposal 15), support LMF to indicate a gNB to report UL RSCP with the specific RF frequency(ies). The specific RF frequency(ies) apply to all UL RSCP measurements regardless the UL RSCP measurements are obtained from which OFDM symbol(s). |
| Intel[9] | Proposal 15 - For integer ambiguity resolution (IAR), introduce the following options in addition to the option of reporting RSCP/RSCPD with legacy positioning metrics, - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier. - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF. |
| Nokia, NSB[10] | Proposal 8: Support the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. |
| Xiaomi[11] | - Proposal 5: Support positioning report to contain the carrier phase of multiple sub-carriers. |
| Samsung[12] | Proposal 7: For carrier phase positioning measurement, a UE or gNB can be configured to provide a measurement based on the carrier phase of multiple sub-carriers of DL PRS or UL positioning SRS (option 1). Proposal 8: For carrier phase positioning measurement, a UE or gNB can be configured to provide a measurement based on the carrier phase difference of two sub-carriers of DL PRS or UL positioning SRS (option 2). |
| Levono[14] | Proposal 4: Support the measurement of subcarrier phase measurements at UE/gNB. FFS the details of exploiting the subcarrier phases, e.g., use of subcarrier phase differentials, number of considered subcarriers, etc. Proposal 5: RAN1 to further discuss which of the PRS PRBs should be used for performing subcarrier/carrier phase measurements in relation to other signals/channels. Proposal 8: RAN1 to consider the support of reporting IA related metrics such as the actual IA, search ranges, subcarrier phase differentials. |
| ZTE[15] | Proposal 1: For signalling concerning the integer ambiguity, at least support LMF providing the range of integer to UE for UE-based NR carrier phase positioning. Proposal 20: Support measuring and reporting carrier phase of multiple sub-carriers/segments in one carrier/PFL. - center frequency or λ should be informed for each selected sub-carrier/segment. Proposal 21: If a UE/TRP is supported to report the carrier phase measurements of more than one frequencies within a PFL/carrier, the following enhancement can be supported: -LMF configures the number of PRBs/sub-carriers for each carrier phase measurement report. -LMF configures the number of carrier phase measurement reports for each PFL/carrier. |
| InterDigital[[17] | Proposal 5: Support Option 1 “Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF “ Proposal 6: Support the UE to determine the subcarrier(s) to measure and report the carrier phase. Proposal 7: Support Option 2 “Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier “ Proposal 8: Support the UE to select and report the reference TRP for phase difference measurements. Proposal 9: Support Option 3 “Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF” Proposal 10: Support the UE to perform PRS measurement in multiple frequencies in one measurement window. Proposal 11: Support the UE to report the phase measurement in multiple frequencies in one measurement reporting. |
| Apple[18] | Proposal 2: RAN1 should support the following to enable CPP in Release 18: - Support joint legacy and CPP positioning and for a UE, optionally stand-alone CPP - Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF. For the UE, this should be subject to a UE capability. - A UE should be explicitly configured as a PRU for double difference CPP subject to its capability - Support simultaneous measurements of the same DL-PRS for multiple UEs, including a target UE and a PRU; and support simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU. The measurements/transmissions should be time sensitive i.e. at a specific time or within a specific time duration |
| Apple[18] | Proposal 6: RAN1 should identify the information to be fed back and the configuration needed in the case of a single CP measurement within a PFL/carrier, and of multiple CP measurements within a PFL/carrier. |
| Ericsson[19] | Proposal 6 A phase measurement shall be defined and reported for the central carrier frequency. Proposal 7 Do not introduce reporting of phase measurements at more than one frequency within the PFL. |
| Qualcomm[20] | Proposal 5: Support Option 4 “Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data”. For Option 3 “Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF”, clarify how the UE/TRP can compute such an estimate. Proposal 6: Do not support Options 1 and 2 of the RAN1#112 agreement as listed below: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival |
| NTT DCM[21] | Proposal 1: - For solution of integer ambiguity resolution, legacy NR positioning measurements such as RSTD and RTOA can be used as coarse estimation step. Proposal 2: - If either measurement result of UE Rx-Tx time difference or gNB Rx-Tx time difference is enough to resolve integer ambiguity, we should discuss which legacy positioning method can be considered. - E.g., Whether measurement results of UL E-CID (e.g., TA) can be used for integer ambiguity resolution or not can be discussed. Proposal 3: - Details of integer ambiguity resolution based on reporting carrier phase measurements of more than one frequency within a PFL/carrier from UE/TRP to LMF should be clarified. Proposal 4: - For further accurate integer ambiguity resolution, new measurement based on carrier phase difference(s) across multiple subcarriers within a carrier should be considered. Proposal 5: - The benefit of reporting estimated integer ambiguity with uncertainty if any should be clarified. Proposal 6: - Assistance data which provides the expected integer ambiguity range should be considered. |
| Locaila[23] | (Proposal 4) The phase-based standalone positioning method should be suppoted - If it is necessary to report the legacy positioning results, report a ToA or TDoA value that translates the phase information measured using subcarriers. |
| IIT Kanpur, CEWiT[24] | Proposal 1: The carrier phase measurement, at least at the subcarrier level, should be supported for carrier phase positioning. Proposal 2: For UE-assisted carrier-phase-based positioning, carrier-phase measurement over multiple subcarriers should be reported to help mitigate the effects of noise. Proposal 3: The frequency location of the subcarriers and the number of subcarriers used for carrier phase reporting should be conveyed to the TRP and/or to the UE performing the carrier phase (CP) measurement. Proposal 10: For UE-based carrier phase positioning, the integer ambiguity could be conveyed to the UE. Option 1: Single value of Integer ambiguity. Option 2: A possible range of integer ambiguity |
| MTK[25] | Proposal 4-1: support the following two options, - Option 1: support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - Option 3: support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF |

Various potential solutions for integer ambiguity resolution in NR carrier phase positioning were investigated during Rel-18 SI and also further discussed in RAN1#112. Many companies have provided their views on the four options included in the previous agreement:

- Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF

  - Support: Huawei, HiSilicon, OPPO, CATT, Nokia, Xiaomi, Samsung, Levono, ZTE, InterDigital, Apple, IIT Kanpur, CEWiT, MTK

  - Not Support: Ericsson, Qualcomm

- Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier

  - Supported by: Intel, Samsung, InterDigital, DCM, Locaila

  - Not Support: Huawei, Qualcomm

- Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF

  - Support: Spreadtrum, Intel, Nokia, Levono, InterDigital, MTK

  - Not Support: Huawei

- Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data.

  - Supported by: vivo, OPPO, Spreadtrum, Nokia, Levono, ZTE, Qualcomm, IIT Kanpur, CEWiT

  - Not Support: Huawei

From the contributions in this meeting, Option 1 and Option 4 have the majority support of companies, while Option 2 and Option 3 also have support from multiple companies. The main reason for supporting Option 1 is due to the use of virtual carrier phase which is a well-established approach for resolving the integer ambiguity in GNSS carrier phase positioning. However, some companies do not support this option and argue that wider bandwidth provides higher carrier phase measurement accuracy, and that the carrier phase at any RF frequency in a PFL can be derived from the carrier phase at the center frequency and the time of arrival for NR OFDM system. In FL’s understanding, These arguments could be valid in an ideal situation, but in really, there are frequency-dependent and non-linear phase/time delays in the Tx/Rx RF chains that cannot be avoided (see e.g., https://www.dsprelated.com/freebooks/filters/Phase_Group_Delay.html). This makes it impossible to use the carrier phase at the certer RF frequency and the TOA to derive the carrier phase of another RF frequency, which may be significantly different (e.g., more than 10MHz away) from the center frequency of the DL PFL.

The props/cons of these options were extensively discussed during Rel-18 SI and also in RAN1#112 [26]. To make progress on the work item, it is crucial for RAN1 to determine which of these options to support in this meeting. Additionally, for Option 1, it is important to define the maximum number of RF frequencies that can be supported in a single PFL/UL carrier. For Option 2, the proponent should provide more details on the new type of UE/TRP measurement.  As for Option 3 and Option 4, it may be necessary to clarify whether the estimated integer ambiguity and/or search range is related to an original carrier phase measurement or a differential original carrier phase measurement.

(H)(Round 1)Proposal 5-1

For NR carrier phase positioning, subject to UE’s capability, support LMF to divide a single DL PFL to M sub-PFLs and request a UE, to report the carrier phase measurements for each sub-PFL. The RF frequency of each carrier phase measurement is the center frequency of the corresponding sub-PFL.

- FFS: M=[2, 3]

- FFS: the minimum bandwidth of a sub-PFL

| Company | comments |
| --- | --- |
| vivo | We have some concerns about option 1 since the carrier phase accuracy will be dependent on sub-PFL bandwidth based on carrier phase definition. FL: For legacy positioning, the positioning accuracy is roughly proportional to the accuracy of the timing measurement. The same rule may apply to carrier phase positioning only if the integer ambiguity is resolved correctly. For single-shot NR CPP, as we discuss now for Rel-18, the resolution of the integer ambiguity depends more on the carrier wavelength and the search range associated with the other measurements (e.g., RSTD/RTOA), but less on the accuracy of the carrier phase measurement. Let us assume 100 MHz BW we have CP error of 0.01 cycle, and for 3GHz we have 1 cycle = 10cm. Assume the TOA error is more than 5cm, e.g., larger than half cycle, i.e., 5cm, then it will be difficult to determine the integer ambiguity correctly even the CP error is only 0.01 cycle. On the other hand, if we divide 100 MHz BW into two 50MHz, and the CP error is doubled to 0.02 cycles. Assume the virtual wavelength is now also doubled to 20cm (maybe a different number). Then, with the same TOA error is 5cm, we may get correct the integer ambiguity of the virtual career phase, with to 0.02 cycle carrier phase error. I acknowledge my explanation may be over-simplified, since the measurement errors of the virtual career phase can be much larger than 0.02 cycles. Nevertheless, it is will known the use of the virtual career phase is an effective approach for resolving integer ambiguity with single-shot carrier phase measurements. More discussion of the benefits of using more than one RF frequency within a PFL was discussed and evaluated by multiple companies in Rel-18 SI. |
| ZTE | Generally OK for the main bullet. |
| Xiaomi | We are fine with the proposal |
| CATT | We are fine with the proposal |
| Lenovo | Support |
| OPPO | Ok with the proposal |
| Ericsson | Not supported. Reporting of carrier phase measurements at additional frequencies increases the complexity and the signaling overhead for CPP. The “virtual frequency” method for integer ambiguity solution can be used without this feature since the carrier phase at any RF frequency in a PFL can be derived from the center carrier phase and the TOA. This follows from the measurement equations used by companies during the SI. Consequently, the argumentation by FL that this is not possible in non-ideal situations is the same as saying that the simulation assumptions during the SI were not good enough. FL: I like to point out my comment “This makes it impossible to use the carrier phase at the certer RF frequency and the TOA to derive the carrier phase of another RF frequency, which may be significantly different (e.g., more than 10MHz away) from the center frequency of the DL PFL” does not mean that the simulation assumptions during the SI were not good enough. As a matter of fact, in Rel-18 SI, arbitrary Tx/Rx phase offsets were assumed at any RF carrier frequency. Due to double differential, the Tx/Rx phase offsets at any any RF carrier frequency can be eliminated by the use of the carrier phase measurement of the corresponding RF carrier frequency. |
| InterDigital | We support the proposal |
| Qualcomm | Do not support. Each sub-PFL report will be less accurate since it was generated using smaller bandwidth. FL: Please see my response to vivo’s similar comments |
| Intel | Considering additional robustness to frequency offsets and a more “direct” metric offered by carrier phase differentials as in Proposal 5-2, we prefer the option in Proposal 5-2 over this proposal. FL: If direct metric offered by carrier phase differentials is accurate enough, say the error is smaller than 0.5 cycle, which is 5cm for 3GHz carrier frequency, then yes, we can fix the integer ambiguity. |
| NTT DOCOMO | Support. |
| LGE | Not support. |

FL Comments:

It seems the majority companies support the proposal, but some companies still have the concerns.

### (H)(Round 2)Proposal 5-1

For NR carrier phase positioning, subject to UE’s capability, support LMF to divide a single DL PFL to M sub-PFLs and request a UE, to report the carrier phase measurements for each sub-PFL. The RF frequency of each carrier phase measurement is the center frequency of the corresponding sub-PFL.

- M= [2, 3]

- FFS: the minimum bandwidth of a sub-PFL is [10] MHz

| Company | comments |
| --- | --- |
| Huawei, HiSilicon | OK, but this proposal does not mention UL at all. We think that this is also related to the discussion in 2.3-2, and 2.3-4. FL:. Right. There is some relation. My thinking is we focus this one for DL first. We can work on UL if we can have an agreement for DL.. |
| ZTE | Generally OK with the proposal. But how does the explicit value of M is determined, is there any limitation on sub-PFLs? |
| vivo | Thanks for the response from FL, but we remember some companies have provided the simulation which shows the integer number can not be correctly resolved if the carrier phase error is larger than a threshold. In this case, even if the searching scope of IAR can be reduced by sub-band PFL, the IAR may not be resolved if the phase error is larger than a threshold. FL: That is correct. The IRA search algorithm plays important role here. This really depends on the RF environment. The issue here is that, as I explain, depends on the degradation due to smaller BW and the increase of the wavelength. In another word, the approach may not always work for all scenarios, but it should at least work for some scenarios with good RF conditions. |
| Intel | Compared to the option in Proposal 5-2, how it addresses frequency offsets is still not clear to us. FL: Can you explain what the frequency offset you are refering to here? The proposal is simle measure a configured BW to provide the carrier phase measurement. |
| Xiaomi | Support |
| CATT | Support |
| Locaila | We understand FL’s concern and intension here. But, it needs more discussion. Isn’t this, after all, similar to standalone subcarrier method ? Dividing 100MHz to smaller bandwidth is actually the subcarrier frequency. And we may have better calculation method to obtain the ‘vitual carrier phase’ if we calculate the phase differeces between group of subcarriers. FL: In my understanding, reporting only carrier phase with M=2 or 3 carrier frequencies is not enough for supporting standalone carrier phase positioning. On the other hand, if RAN1 can make the agreement to support reporting the carrier phases or carrier difference more subcarriers for standalone approach, then I assume virual carrier phase can also be supported. To Ericsson, we have a question. ========================== Has any company presented a performance evaluation result showing that exact integer number can be “… derived from the center carrier phase and the TOA, … (with sufficient accuracy?)” As far as we know, no company has showed such result how accurately the legacy method can determine the exact integer number. Most companies brought the simulation result under the assumption that the ideal integer number can be estimated, and no company verified the performance of the integer number estimation itself. As FL explained his concern in above, it should be verified how closely the TOA measurement can provide the exact integer number. |

### (H)(Round 3)Proposal 5-1

For NR carrier phase positioning, subject to UE’s capability, support LMF to divide a single DL PFL to M sub-PFLs and request a UE, to report the carrier phase measurements for each sub-PFL. The RF frequency of each carrier phase measurement is the center frequency of the corresponding sub-PFL.

- M= [2, 3]

- FFS: the minimum bandwidth of a sub-PFL, e.g., [10, 20] MHz

| Company | comments |
| --- | --- |
| Locaila | We want more discussion on this subject. We understand the intension here. Perhapse we may find unified solution for 5-1 and 5-2. See our discussion in 5-2. |
| vivo | Based on the response as FL, we think the issue can be dropped in Rel-18 to only support the base feature of CPP. |
| Qualcomm | Same comment as vivo. The gains have not been sufficiently established. |
| LGE | Similar view with vivo |
| ZTE | Support the main bullet. The detailed value of M and bandwidth limitation can be further studied. |
| Xiaomi | Support |
| Apple | The measurement seems to have dependency on Proposal 2.3-2 . If we accept this, we are by default accepting Option 4 in the other proposal. Suggest makding a decision there then here. |
| CATAT | Support |
| Intel | @FL For round 2, we had misread the proposal – if the measurements are performed at the same time, then FO can be addressed. |
| Locaila | Here’s our proposal for unified approach of both 5-1 and 5-2. In our view, 5-1 and 5-2 are in principle the same if both are performed within single carrier/PFL. Let Y the set of phase information extracted via FFT at the receiving UE/TRP. (0) Y = {x[1], x[2], …. x[m]} Y is a set of m subcarriers consisting a PRS, or REs in the PFL. If we divide Y into two subsets Y1 and Y2, and calculate sum of each subset, and calculate the phase difference, we may obtain virtual carrier phase as below - Y1 = {x[1], x[2] , … x[m/2] } , Y2 = {x[m/2+1], x[m/2+2] … x[m]} - Pv = arg( sum(Y1) – sum(Y2) ) In above, (m/2) is the distance between two sets Y1, Y2 and also the frequency of the virtual carrier implied in 5-1. In fact, this is the (m/2) times of subcarrier gap frequency in the PFL. Equation (2) can be reformulated as (3). We denote this as P(m/2) - P(m/2) = arg ( sum { (x[1]-x[m/2+1]) , (x[2]-x[m/2+2]) , … (x[m/2]-x[m]) } ) This is the representation of subcarrier phase of frequency (m/2) in our (standalone) approach. Of course, it can be slightly different to proponents of ‘slope’ method, however, we think the end-result will be the same. Since (2) and (3) are in effect the same, we may find the starting point for unified approach from here. Now, let’s divide Y into two subsets Ys1, Ys2, allowing redundancy as below. - Ys1 = {x[1], x[2] , … x[m-1] } , Ys2 = {x[2], x[3] … x[m]} In above equation (4), the distance between two sets Ys1, Ys2 is only ‘1’ and it is the shortest subcarrier gap frequency in the PFL, meaning the longest length subcarrier wave. We may obtain the longest subcarrier phase as below (5). - P(1) = arg ( sum { (x[1]-x[2]) , (x[2]-x[3]) , … (x[m-1]-x[m]) } ) If the wavelength of P(1) is sufficiently long, we may directly convert the phase value into distance and estimate coarse UE location. Note that an integer number has to be estimated in both (2) and (3) because the subcarrier wavelength of (2) and (3) are far shorter than (5). The proponents of virtual carrier method suggest to use legacy measurement to estimate this integer number. However in standalone approach, (5) is sufficient to estimate the interger number for (2) and (3). Using equation we proposed in R1-2303774 as below (6), we may estimate refined UE location without relying on legacy measurements. - N=round (m/2)∙P(1)/2π-P(m/2)/2π We can iteratively apply above procedure and narrow down until we finally find the integer number for carrier frequency which is the center frequency of Y in equation (0). Above procedure may be a bit different in ‘slope’ method, however the end-result will be the same, in our view. If companies agree on the propose view, we may formulate texts unifying 5-1 and 5-2, and finish arguments against the ‘standalone’ approach. |

(H)(Round 1)Proposal 5-2

For NR carrier phase positioning, support introducing a new type of UE measurement, which can be related to time of arrival, based on carrier phase differentials across multiple subcarriers within a PFL.

- FFS: the details of the new type of UE measurement

| Company | comments |
| --- | --- |
| vivo | Not supported |
| Huawei, HiSilicon | This proposal should be interpreted as to introduce a ToA measurement for the CPP standalone method. We do not agree with the current wording before we agree on introducing standalone CPP method. |
| CATT | It may be better to first understand what the candidates for the new type of UE measurements |
| Lenovo | Support and may be defined in conjunction with a RSCP measurement |
| OPPO | We are open to introduce measurements based on carrier phase differentials across different subcarriers, however, the wording of the proposal is not clear. |
| Ericsson | Not support |
| Samsung | Support |
| Qualcomm | Don’t support sub-PFL reporting, direct or differential. |
| Intel | Support. In our view this is mainly relevant to support of CPP as separate positioning method. To CATT’s question, this can be TOA-like metric that can be defined similar to RSTD with finer granularity and reported along with RSCPD for the same reference TRP. FL: If it is only finer granularity, we can propose RAN4 to include the granularity of RSTD reporting w/o introducing new measurement if the accuracy of the proposed TOA-like metric is same as RSTD. |
| NTT DOCOMO | The details should be clarified. |
| Nokia/NSB | Don’t support. |

FL Comments:

Maybe we can add a note based on Intel’s comment to see if we have a better understanding of the proposal.

(H)(Round 2)Proposal 5-2

For NR carrier phase positioning, support introducing a new type of UE measurement, which can be related to time of arrival, based on carrier phase differentials across multiple subcarriers within a PFL.

- Note: the new type of UE measurement has a finer granularity than current RSTD

- FFS: the details of the new type of UE measurement

| Company | comments |
| --- | --- |
| Locaila | We prefer more discussion on this topic. What’s the existing view on UE measurement and what’s the new type of UE measurement? In our view, there’s not much difference in mathematical analysis. It may be just a different way of calculation. It will be helpful to converge on common understanding if proponent companies suggest clear mathematical definition, rather than text description. |
| Huawei, HiSilicon | As Locaila points out, this is a different way of calculating ToA, which should have no impact on spec. |
| vivo | Low priority |
| Intel | We suggest simplifying the proposal as below. This should also help clarify the questions/comments from Locaila and HW-HiSi. For NR carrier phase positioning, support introducing a new type of UE measurement, which can be related to time of arrival, based on reporting of carrier phase differentials across multiple subcarriers within a PFL. - FFS:Note: tThe new type of UE measurement carrier phase differentials across multiple subcarriers within a PFL is reported using a RSTD-like metric with has a finer granularity than current RSTD - FFS: the details of the new type of UE measurement FL: The main bullet says reporting of carrier phase differentials, the seems contradict the main bullet. Maybe separate into two options? ### (H)(Round 2)Proposal 5-2 (updated) For NR carrier phase positioning, support one of the following options:: - Option 1: reporting of carrier phase differentials across multiple subcarriers within a PFL - Option 2: reporting of RSTD-like metric derived with has a finer granularity than current RSTD, which is derived from carrier phase differentials across multiple subcarriers within a PFL |
| Spreadtrum | We think this proposal needs more discussion. |
| CATT | Share the similar view as Huawei |
| Samsung2 | Support. We are also fine in principle with the updates made by Intel. |

### (H)(Round 3)Proposal 5-2

For NR carrier phase positioning, support one of the following options::

- Option 1: reporting of carrier phase differentials across multiple subcarriers within a PFL

- Option 2: reporting of RSTD-like metric derived with has a finer granularity than current RSTD, which is derived from carrier phase differentials across multiple subcarriers within a PFL

| Company | comments |
| --- | --- |
| Nokia/NSB | We don’t support this proposal. Option 2 is confusing to us as we feel it is not likely that 3GPP would specify “which is derived from carrier phase differentials across multiple subcarriers within a PFL”. |
| Locaila | Current wording is a good improvement, but there’s still more elements to add and elaborate. In our view, proposal 5-1 and 5-2 are in principle the same with slight difference in calculation method. Both use the same phase information extracted from received PRS within the same carrier/PFL. I don’t agree on FL’s view that standalone method may not provide enough accuracy than ‘virtual carrier’ method. I wish FL suggest exact equation for 5-1, then I can find combined solution with 5-2. FL: I believe Locaila has misunderstood my previous response to their comment. To clarify, my previous response was: “FL: In my understanding, reporting only carrier phase with M=2 or 3 carrier frequencies is not enough for supporting standalone carrier phase positioning. On the other hand, if RAN1 can make the agreement to support reporting the carrier phases or carrier difference more subcarriers for standalone approach, then I assume virual carrier phase can also be supported.” What I ment was that solely reporting carrier phase with two or three carrier frequencies (which is the virtual carrier method) is insufficient for supporting standalone carrier phase positioning. However, if RAN1 agrees to support reporting carrier phases or carrier differences with more subcarriers for standalone positioning, then virtual carrier phase can also be supported. Essentially, providing more information through reporting carrier phases or carrier differences with additional subcarriers is necessary for supporting standalone carrier phase positioning and also enables virtual carrier phase support. The amount of calculation and amount of information for 5-2 is larger, meaning that standalone method can provide better accuracy. And more, standalone method provide broder range of information from coarse integer number estimation to fine-grain integer number estimation using long to short subcarrier waves. But the virtual carrier method implied in 5-1 lacks the capability to provide initial UE location (or coarse integer number estimation). That’s why the proponent companies insist to report legacy time measurement together with carrier phase measurement. This is unnecessary and it only makes UE heavy, in my view. I hope, we may converge to a common, generic solution covering both 5-1 and 5-2, and find better wording agreeable to both ‘virtual carrier’ proponents and ‘standalone’ proponents. |
| Vivo | Same view as proposal 5-1, drop the discussion in Rel-18 |
| Qualcomm | Same view as vivo |
| ZTE | We are open to study the report of carrier phase and/or the report of carrier phase difference in multiple subcarriers. Option 1 is fine for us, but option 2 is a little bit confusing, what does RSTD-like metric refers to? Does this report can actually improve the positioning performance? Maybe more discussion and detailed explanation on option 2 is required. Proposal 5-1 mentioned that UE can report carrier phase in different sub-PFL, but 5-2 uses subcarriers, how about unifying the description, like update all “sub-PFL” to “subcarrier”? |
| Apple | Agree with Nokia, Vivo and Qualcomm |
| CATT | We are fine with Option 1. |
| Intel | We are supportive. However, in our understanding, Options 1 and 2 are exactly the same. The suggestion in the previous round was not the clearest causing some misunderstanding for the FL. We give it another attempt below. For NR carrier phase positioning, support reporting of RTOA/RSTD which is derived from carrier phase differentials across multiple subcarriers within a PFL. - The RTOA/RSTD metrics are reported with finer granularity than current RTOA/RSTD. FFS: Details. In the above, RTOA is added in consideration of agreement on support of RSCP as well. Also, a similar approach can be applied to UL. |

(H)(Round 1)Proposal 5-3

Subject to UE’s capability, support enabling a UE to optionally report an estimated integer ambiguity and/or the search range of the integer ambiguity to LMF.

- FFS: the details of the estimated integer ambiguity and/or the search range

| Company | comments |
| --- | --- |
| Huawei, HiSilicon | This proposal should be interpreted as to introduce a ToA measurement quality for the CPP standalone method. We do not agree with the current wording before we agree on introducing standalone CPP method. |
| ZTE | In our view, this proposal can be supported if there are benefits proposed concerning UE/TRP report the estimated integer ambiguity. |
| CATT | We assume UE may be able to provide the estimated integer ambiguity for RSCPD, but not RSCP due to the impact of UE timing error on RSCP is unknown. |
| Lenovo | Support |
| OPPO | Do not support. The UE is not able to estimate a meaningful interger ambiguity or search range without knowing the TRP location, which is not available for UE-assisted positioning. |
| Ericsson | Not support |
| mtk | To HW, this proposal in NOT related to CPP standalone. It is just that during UE measurement, UE could provide a search window. The center of the search window is basically the RSTD measurement result. The search window could also be treated as how confident the UE determines RSTD result. In our view, the search range doesn't need to know UE location. Maybe we can consider to modify the existing IE to support with finer granularity. The search range is multiples of wavelengths. |
| Samsung | More discussion is needed for this proposal. |
| InterDigital | We support the proposal |
| Qualcomm | It is not clear how the UE will estimate this IAR search range. |
| Intel | Support the proposal and prefer to only list “estimated IA” and remove “search range of the IA”. |
| NTT DOCOMO | Is the search range reported after integer ambiguity resolution based on legacy positioning measurements and carrier phase measurements (e.g., proposal 5-1)? |
| LGE | We do not see clear benefits from this method. |
| FL | It seems more discussion is needed for the proponents to present the benefits of the proposal. |
| Nokia/NSB | Support. |
| Spreadtrum | Support this proposal |

FL Comments:

Maybe we can add a note based on MTK’s comment to see if we have a better understanding of the proposal.

### (H)(Round 2)Proposal 5-3

Subject to UE’s capability, support enabling a UE to optionally report an estimated integer ambiguity and/or the search range of the integer ambiguity to LMF.

- FFS: the details of the estimated integer ambiguity and/or the search range, which can be based on the modification of the existing NR-TimingQuality with finer granularity

| Company | comments |
| --- | --- |
| mtk | 1, LMF basically performs blind integer cycle search based on a defined cost function, and the search range for LMF is still unknown so that LMF may define the search range based on its computation capability, or just don't search by directly modifying each DL-RSTD with the RSCPD and then perform position calculation. Note that the search effort will grow exponentially when more DL-RSTD values are taken into the position calculation. 2, UE may observe how confident for each measurement result. It is well known that, a time delay causes the phase shift in frequency domain with a slope. The slope determines TOA and then further determines RSTD. A correlation matrix could be formed, similar to that for MUSIC algorithm. Then for each time delay, it corresponds to have a signature vector within this correlation matrix. UE could form a steering vector by adjusting the slope to check whether the signature vector corresponding to the time delay exists. So the inverse of correlation matrix is needed. It is MPDR beamformer described by Van Tree’s book. When we adjust the phase slope of the steering vector and when it matches with the phase slope of a signature vector within the correlation matrix, there would be a peak. The sharpness of the peak may be determined by noise level, or whether there is another close path which can not be resolved by traditional IFFT method. In our view, the sharpness of the peak could be treated as how confident for the measurement. 3, since LMF performs blind search, if UE is able to provide the guidance from measurement observation, the search at LMF could be at least not so blind. 4, For NTT DOCOMO’s question in above, our view is, proposal 5-1 is also a solution to reduce search effort. But search is still needed. Proposal 5-1 may basically utilize the IFFT method (with smaller IFFT size) to obtain phase measurement at some subcarriers. Smaller IFFT size could be a concern if there is close path. |
| Locaila | In our view, it is just a duplicate information of legacy time measurement. If we divide the legacy time measurement by the period of carrier wavelength, we may obtain the integer number. Why do we have to report same information in difference ways? |
| Huawei, HiSilicon | To MTK, with regards to the confidence of measurement, I think that you are referring to ToA measurement. Then we need to understanding why the current ToA measurement quality is not sufficient. |
| CATT | This may be related to the quality of carrier phase measurements. |
| Samsung2 | More discussion needed for this proposal. |
| Nokia/NSB | Support. |
| FL | Let us wait for more inputs from companies to see how to proceed with the proposal. |
| LGE | It seems pre-mature to make decision. We need further discussion on this topic. |

(H)(Round 1)Proposal 5-4

Support LMF to optionally provide the expected integer ambiguity range to a target UE in the positioning assistance data for UE-based carrier phase positioning.

- FFS: the details of the estimated integer ambiguity search range

| Company | comments |
| --- | --- |
| vivo | Okay |
| ZTE | Support. |
| Lenovo | Support |
| OPPO | Ok |
| Ericsson | Not supported. We think that the current possibilities for LMF to report to UE the Expected RSTD and uncertainty is enough. |
| Samsung | More discussion is needed for this proposal. |
| InterDigital | We support the proposal |
| Qualcomm | Support |
| Intel | Do not support. In context of being useful for CPP, we think much narrower range would be needed which is not quite practical for LMF to know in most practical cases. If this is not the case, then, as mentioned by Ericsson, the existing assistance info on expected RSTD and uncertainty should suffice. |
| NTT DOCOMO | Support. |
| LGE | Share similar view with Ericsson and Intel. |
| FL | In my understanding, expected RSTD and uncertainty is not good enough for integer ambiguity resolution, since the purpose of them are for RSTD search range, which can be in microsecond-level, but not integer ambiguity search range, which is nano-seconds level. |
| Nokia/NSB | Support. |

FL Comments:

We may need to consider whether the expected integer ambiguity range is for RSCP or RSCPD or can be for both.

### (H)(Round 2)Proposal 5-4

Support LMF to optionally provide the expected integer ambiguity range to a target UE in the positioning assistance data for UE-based carrier phase positioning.

- FFS: the details of the estimated integer ambiguity search range

- FFS: the estimated integer ambiguity search range is for RSCP and/or RSCPD

| Company | comments |
| --- | --- |
| ZTE | We support LMF provide the integer ambiguity to UE. But we doubt the necessity for LMF provide the integer ambiguity search range for RSCPD. How does the estimated integer ambiguity search range for RSCPD is defined? If the RSCPD refers to the phase difference between two TRPs, does that mean LMF should provide the estimated integer ambiguity search range(s) of both two TRPs to the targe UE? |
| Spreadtrum | Agree with ZTE. How to provide the integer ambiguity search range for RSCPD should be further considered. |
| CATT | It may be simpler for LMF to provide the estimated distance between UE and TRP and the uncertainty, and let UE to use the information to determine the search range. |
| Samsung2 | More discussion needed for this proposal. |
| Nokia/NSB | Support. |
| FL | Let us wait for more inputs from companies to see how to proceed with the proposal. |
| vivo | OK |
| Qualcomm | OK |
| LGE | It seems pre-mature to make decision. We need further discussion on this topic. |

# Phase error group

Submitted Proposals:

| Xiaomi[11] | - Proposal 6: Report the phase error group ID from UE/TRP for initial phase error mitigation. |
| --- | --- |
| ZTE[15] | Proposal 22: Phase Error Group(PEG) should be defined for carrier phase measurement, where phase measurement errors in a PEG is within a margin value. |
| Fraunhofer[16] | Proposal 4: Consider the signaling of the phase-coherency information related to the different PEGs at a UE/TRP. |
| InterDigital[[17] | Proposal 4: Study assistance information that allows the UE or network to mitigate unknown phase offset in phase measurements, e.g., phase error group. |
| Ercisson[19] | Proposal 13 Reinterpret the definition of a “timing error group” so that it captures both timing and phase errors. Proposal 14 Associate a phase-error margin to each Rx and Tx TEG. |
| Qualcomm[20] | Proposal 9: Study PEG-ID reporting scheme, an extension of TEG-ID reporting concept for phase reports. Measurements/transmissions with same PEG-ID are phase-coherent. - Consider allowing a larger number of PEG-IDs than TEG-IDs. |
| NTT DCM[21] | Proposal 8: - Benefit of introducing PEG in addition to TEG should be clarified firstly. |
| LGE[22] | Proposal 6: For a RSCPD, pair of RSCP should be selected in a same slot or within a same phase continuity window. Proposal 7: A single PRS resource or PRS resources with same PEG can be used as a reference PRS resource(s) for RSCPD measurements of a positioning measurement report. Proposal 13: Support Tx and Rx PEG(Phase Error Group) for CPP, where - Tx PEG: positioning RS with same phase error or error difference smaller than certain margin - Rx PEG: positioning measurement with same phase error or error difference smaller than certain margin Proposal 14: PEG with zero margin can be assumed when TEG with 0 Tc margin is configured. |
| IIT Kanpur, CEWiT[24] | Proposal 4: In Rel 18, for the legacy positioning methods, additional reporting of carrier phase measurements per Rx antennas should be supported. |

FL Comments:

A UE/TRP may have several Tx/Rx antenna panels with different Tx/Rx chains, which may lead to RF signals that are transmitted and/or received through various RF chains, experience distinct group delays. In Rel-17, RAN1 has introduced the Timing Error Group to address the group delays for timing measurements. For carrier phase positioning, some companies suggest to introduce new definition of UE/TRP Rx/Tx phase error groups to address the group delays on the phase measurements (e.g., [11][12][16][17][20][22]). However, since both the timing delays and phase delays are caused by the same RF group delays, some company suggests (e.g., [19]) extending the definition of Rel-17 TEG to captures both timing and phase errors caused by the RF group delays. If FL’s view, it might be useful to either introduce PEG considering that the error margin for timing error and phase error can be significantly different for different RF chains/antennas.

(H)(Round 1) Proposal 6-1

To address the impact of the timing delays on different Tx/Rx RF chains, support introducing the UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals.

- Rel-17 definition of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs.

- FFS: the maximum numbers of UE/TRP Tx/Rx PEGs.

- FFS: the value ranges of the error margins of UE/TRP Tx/Rx PEGs

| Company | comments |
| --- | --- |
| vivo | We prefer to up to RAN4 to discuss the issue since the TEG value is determained by RAN4, so, whether a smaller value can be defined for PEG or a phase value can be defined for PEG needs to be further discussed by RAN4. |
| Huawei, HiSilicon | We do not prefer to introduce PEG. It is not case as TEG where two chains could be grouped in a TEGs, while for phase measurement, different chains should be different PEG. |
| ZTE | Support the definition of PEG. With the help of PEG, UE or LMF can choose appropriate antenna/PEG for location estimation with the measured and/or reported carrier phase. |
| Xiaomi | Support the proposal |
| CATT | We share the similar view as HW that different chains should be different PEGs. So, we assume the PEG is introduced for that purpose. |
| Lenovo | Support the intention of PEG concept. |
| OPPO | Do not support. Technically, it is not feasible to group different chain into one so-called PEGs since different chains would have different phase. |
| Ericsson | We prefer to reuse and reinterpret the TEG IDs. Each TEG should then be associated with one timing error margin and one phase error margin. |
| Samsung | We agree with the comment from Huawei that different chains should belong to different phase error groups. The phase measurement, would be associated with a Tx/Rx chain, we don’t think that we can correlate phase across different Tx/Rx chains. |
| InterDigital | We agree with the proposal in principle. |
| Qualcomm | Support |
| Intel | Not support. Agree with Huawei’s point that it is not clear if different chains can be grouped for phase error as in the case of TEG. If at all, this can be considered by RAN4. |
| NTT DOCOMO | We think it necessary to clarify whether Tx/Rx phase error is independent of Tx/Rx timing error or not. FL: My understanding is not independent. See e.g., https://www.dsprelated.com/freebooks/filters/Phase_Group_Delay.html). |
| LGE | We support to introduce PEG. |
| Nokia/NSB | Support. |

### (H)(Round 2) Proposal 6-1

To address the impact of the timing delays on different Tx/Rx RF chains, consider the following options;

- Option 1: introduce the UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals with one of the following alternatives:.

  - Alt. 1: Rel-17 definition of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs.

  - Alt. 2: Each UE/TRP Tx/Rx RF chain is considered to be an Tx/Rx PEG

  - FFS: the maximum numbers of UE/TRP Tx/Rx PEGs.

  - FFS: the value ranges of the error margins of UE/TRP Tx/Rx PEGs

- Option 2: send an LS to RAN4, request RAN4 to consider whether there is a need to define the UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals and define the UE/TRP Tx/Rx PEGs if it is needed.

| Company | comments |
| --- | --- |
| OPPO | As commented in last round, we do not support Option 1. Technically, it is not feasible to group different chain into one so-called PEGs since different chains would have different phase. FL: Is Alt.2 in Option 1 what OPPO wants? Re option 2: we do not see the need for a LS to RAN4. RAN4 can do the study if they think it is needed. Sending an LS from RAN1 to request RAN4 to consider something that even companies in RAN1 do not think it is a problem seem not necessary. |
| Huawei, HiSilicon | Agree with OPPO. Reporting Rx antenna ID should be sufficient. FL: Need consider both Tx/Rx antenna IDs if we introduce them. |
| ZTE | Support introducing PEG for CPP. |
| vivo | Option 2 |
| Intel | Same view as OPPO and HW-HiSi. |
| Spreadtrum | Option 2 |
| Fraunhofer | We support Option 1 and Alt1 which can be even further simplified such as: UE Tx ‘timing error group’ (UE Tx PEG): A UE Tx PEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose which are phase coherent. FFS: the definition of phase coherency. FL: The difficult is now how to define the phase coherency. |
| Xiaomi | Support Option 1 |

| CATT | Option 1. Atl.2, or Option 2 |
| --- | --- |
| InterDigital | We support Option 1. |
| Samsung2 | Similar view to OPPO and Huawei. |
| Nokia/NSB | Support option 1. |
| Qualcomm | Support Alt1 of Option1. Given TEG was defined in Rel-17, it is quite clear that some form of PEG should be added. The idea of TEG/PEG is to abstract away the details of UE implementation, and Alt2 which discusses UE architecture (number of RF chains) is not desirable from this viewpoint. It is easily possible that two different PRS sent from the same RF chain should be in two different PEGs. |
| LGE | Support option 1 |

# CPP Multipath mitigation

Background

Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement (RAN1#112) Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

Submitted Proposals:

| Huawei, HiSilicon [3] | - Proposal 23: For NR carrier phase positioning, the TRP/UE can report the carrier phase measurements of the first path and the additional paths. |
| --- | --- |
| OPPO [6] | Proposal 3: Define per path RSCPD and the UE reports the per path RSCPD of each additional path with reference to the first path in the additional path reporting of DL TDOA and multi-RTT. |
| Spreadtrum Comm[7] | Proposal 4: For carrier phase positioning, both carrier phases of the first path and the additional paths can be reported. Proposal 5: For multipath mitigation, support the reporting of other existing channel information together with carrier phase positioning. |
| CATT[8] | Proposal 6: In Rel-18, support only reporting DL RSCP/RSCPD of the first path. Proposal 18: In Rel-18, support only reporting UL RSCP of the first path. |
| Intel [9] | Proposal 13 - In consideration of multipath environments, adopt the following solutions: - Measurements: For both DL and UL CPP, limit reporting of carrier phase to that of the first path detected in time. - Assistance information: Rel-17 LOS/NLOS indication can be the baseline. Proposal 14 - Introduce LOS/NLOS reporting as optional assistance information along with reporting of CP(D) measurements for DL/UL CPP as independent positioning method. - Reuse R17 LOS/NLOS reporting quantity/granularity as baseline. |
| Nokia, NSB[10] | Proposal 2: NR CPP should optionally support reporting the carrier phase of additional paths. |
| Samsung[12] | Proposal 11: For carrier phase positioning measurement, a UE or gNB provides a measurement based on the first (LOS) received multi-path. |
| CMCC[13] | Proposal 2: For NR carrier phase positioning, support only the carrier phase of the first path. |
| ZTE[15] | Proposal 24: Support UE/TRP reporting the carrier phase of the first path only at least for Rel-18. |
| Fraunhofer[16] | Proposal 3: Support reporting the phase measurements for the first and additional paths (N-path reporting for the phase) associated with the LOS path. The additional paths can be identified from the correlation function in the delay domain. |
| InterDigital[[17] | Proposal 10: Support the UE to perform PRS measurement in multiple frequencies in one measurement window. Proposal 11: Support the UE to report the phase measurement in multiple frequencies in one measurement reporting. |
| Apple[18] | Proposal 8: the Rel-17 LOS/NLOS agreements for additional paths (when indicated) can be applied to carrier phase measurement(s) in the same report if measurements are reported for additional paths. |
| Ericsson[19] | Proposal 1 Only support carrier phase measurements of the first detected path. |
| Qualcomm[20] | Proposal 3: Do not adopt RSCP definition for the i-th path for i>1 unless clear benefits from this are demonstrated. |
| NTT DCM[21] | Proposal 7: - Support the following option: - Option 2: Report the carrier phase of the first path, and optionally, the additional paths. |
| LGE[22] | Proposal 11: Reuse Rel-17 LOS/NLOS indication for a carrier phase measurements reporting without the legacy positioning measurement. |
| IIT Kanpur, CEWiT[24] | Proposal 6: For carrier phase-based positioning, reporting of carrier phase measurement corresponding to the first path should be supported. The measurement corresponding to the additional path could be optionally supported. Proposal 7: The RSRPP should be reported across all the additional paths if the carrier phase of additional paths is reported. The number of paths reported is UE capability. |

## Carrier phase measurements for additional paths

Regarding the reporting of carrier phase measurements for additional paths, there are differing opinions among companies:

- Option 1: TRP/UE report the carrier phase measurements of the first path and additional paths

  - Supported by: Huawei, HiSilicon, OPPO, Spreadtrum, Nokia, NSB, Fraunhofer, DCM, IIT Kanpur, CEWiT

- Option 2: TRP/UE report the carrier phase measurements of the first path only

  - Supported by: Intel, Samsung, CMCC, ZTE, Ericsson, Qualcomm, CATT

Ther is a slightly larger number of companies are in favor of reporting the carrier phase measurements of the additional paths. However, due to the lack of simulation evaluations on the benefits of reporting carrier phase for additional paths during Rel-18 SI, it may be difficult for proponents to convince other companies to support reporting carrier phase measurements for additional paths. Thus, the FL likes to check companies’s views on whether we should continue the discussion or we should conclude not continuing the discussion so that attention can be redirected to other more critical issues in Rel-18.

### (Round 1) Proposal 7.1-1 (as a conclusion):

- RAN1 concludes no further discussion for the support of the reporting of the carrier phase of the additional paths in Rel-18.

| Company | comments |
| --- | --- |
| Qualcomm | Ok |
| Intel | Support. |
| Nokia/NSB | Do not support. This is not how we should handle this discussion. Clearly there is a majority in favor of additional path reporting… we made dozens of decisions (or more) each release without simulation results so that argument is frankly not a fair way to handle this topic. |
| FL | Let us wait to see if there is any concern for the proposal |
| Fraunhofer | Do not support. due to channel conditions, the device may sometimes be unsure about the first arriving path, resulting in reported phases that do not correspond to the LOS. In such cases, reporting CPP on additional paths can avoid wrong CPP and help to identify the correct LOS and improve the accuracy of the positioning system. |
| CATT | Support. |
| Samsung2 | Support |
| Nokia/NSB_2 | We would like to highlight that for SL there seems to be no concerns to have additional path reporting for timing techniques. We have yet to see any simulation results that this improves the positioning performance in SL. So we think the argument that there are not simulation results is not a valid one. |
| LGE | Fine |

## LOS/NLOS indication for standalone carrier phase positioning

Regarding the LOS/NLOS indication, it was decided in RAN1#112 that when the carrier phase measurement(s) and legacy measurements are in the same report, the Rel-17 LOS/NLOS indication would be applicable when indicated. This covers the cases where legacy measurements are reported alongside carrier phase measurements. If RAN1 reachs the agreement to support standalone carrier phase reporting/positioning, we will further discuss the LOS/NLOS indication for standalone carrier phase phase measurements.

### (Round 1) Proposal 7.2-1

- If standalone carrier phase phase measurement reporting is supported, LOS/NLOS indication developed in Rel-17 can be included in the measurement report to indicate the RF channel status.

| Company | comments |
| --- | --- |
| InterDigital | We support the proposal. |
| Qualcomm | Do not support standalone report, which will avoid such additional work that we see as unnecessary |
| Intel | Support; unlike QC, we do not see any significant additional work here. Most options have been (and are being) discussed at length. |
| LGE | Support the proposal. |
| FL | Let us wait for more feedback to decide the next step. |
| Locaila | Support |
| Spreadtrum | Do not support standalone report. |
| CATT | Okay |

# Additional measurement reporting

Submitted Proposals:

| Samsung[12] | Proposal 12: For carrier phase positioning measurement, a UE or gNB provides a quality (or accuracy) indicator of the carrier-phase measurement based on the relative strength of the LOS multi-path component. |
| --- | --- |
| CMCC[13] | Proposal 1: The following information should be considered in the CPP measurement reporting: - The timestamp - The measurement quality - Reference TRP / DL PRS resource set / DL PRS resource(s) ID |
| Lenovo[14] | Proposal 7: RAN1 to consider the reporting of quality metrics and timestamps associated to DL/UL carrier phase measurements. FFS details of the type of quality metrics. |
| ZTE[15] | Proposal 25: Support UE/TRP reporting the carrier phase measurement quality together with the phase measurement result. |

## Carrier phse measurement quality

Similar with other positioning measurement, it is important for the receiver to provide the measurement quality indication to the LMF, as proposed by [12-15]. For NR timing measurements, NR-TimingQuality is defined in TS 37.355 to represent the quality of a timing value (e.g., of a TOA measurement). NR-TimingQuality includes timingQualityValue for an estimate of uncertainty of the timing value and timingQualityResolution for the resolution used in the timingQualityValue field. For GNSS measurement, TS 37.355 has included the following IEs that are related to GNSS measurement quality:

- cNo: This field provides an estimate of the carrier to noise ratio of the received signal from the particular satellite.

- carrierQualityInd: This field indicates the quality of a carrier phase measurement, e.g., whether if accumulation of the carrier phase has been continuous.

- adrRMSerror: This field contains the ADR root mean squared error value. Scale factor 2-10 metres.

(H)(Round 1) Proposal 8.1-1

- Support UE/TRP to report the carrier phase measurement quality indication for corresponding the phase measurements

  - FFS: the details of the carrier phase measurement quality indication, e.g., the value range, the unit (e.g., meters or cycles) and the resolution

| Company | comments |
| --- | --- |
| vivo | We are okay with introducing an indication. But we would like to confirm how to calculate the quality, can the proponents provide more information for this? |
| ZTE | Support UE/TRP report the measurement quality as legacy measurement quality is also supported in current spec. @vivo: The number of reported PRS carrier phase measurement, physical channel environment and PRS propagation multi-path could affect the accuracy of carrier phase evaluation. These factors can be used as a basis for measurement quality indication. UE can report the uncertainty of phase measurement and further provides the corresponding phase measurement quality or phase measurement confidence, which is similar to the existing timing measurement quality or angle measurement quality. |
| CATT | Support |
| Lenovo | Supportive of FL’s proposal, similar quality metrics to GNSS may be discussed/considered as part of the FFS |
| Ericsson | Similar view as vivo. |
| Samsung | OK in principle. |
| Qualcomm | Need more discussion on details first. |
| Intel | Same vivo as vivo, Ericsson. We are not sure ZTE’s examples can work for defining quality for carrier phase quality. |
| LG | The reason to introduce new indication for measurement quality is not clear to us. Also it depends on what quality indication will be. We think details regarding them should be discussed first before introducing it. |
| FL | Based on the feedback, it seems many companies need more time to study what the quality indicators are and how to obtain them. Based on the feedback, |

(H)(Round 2) Proposal 8.1-1

- Study whether to support a UE/TRP to report the carrier phase measurement quality indication for corresponding the phase measurements. The candidates of the carrier phase measurement quality indication may include:

  - the carrier to noise ratio of the received signal

  - the root mean squared error value

  - other candidates are not precluded

| Company | comments |
| --- | --- |
| Huawei, HiSilicon | The existing quality for ToA/AoA should be the baseline, and then we can discuss the range. |
| ZTE | We are ok to support UE/TRP reporting the carrier phase measurement quality indication. But for the second bullet “the root mean squared error value”, how does the UE/TRP measure the error value? Because UE/TRP only know the CP measurement, they don’t know the explicit UE location, UT/TRP cannot calculate the error value or the difference between the real UE location and estimated UE location. Furthermore, UE cannot calculate it’s location in LMF-based CPP. |
| vivo | We have the same concern as ZTE on the second sub-bullet. In addition, maybe the consistency between those errors and carrier phase errors also needs to be considered. |
| CATT | Share the similar view as Huawei, ZTE and vivo |
| Samsung2 | OK in principle, but no need to list examples. |
| FL | Based on the comments, we may remove the list of example. ### (H)(Round 2) Proposal 8.1-1 (updated) - Study whether and how to support a UE/TRP to report the carrier phase measurement quality indication for corresponding the phase measurements. |

### (H)(Round 3) Proposal 8.1-1

- Further study whether and how to support a UE/TRP to report the carrier phase measurement quality indication for corresponding the phase measurements.

| Company | comments |
| --- | --- |
| Nokia/NSB | Okay |
| Huawei, HiSilicon | OK |
| Qualcomm | OK |
| LGE | Ok with further study |
| ZTE | OK to further study. |

## Additional information for carrier measurements

Additoinal information may need to be included in the carrier phase measurement. For example, the timestamp needs to be included in carrier phase measurement reporting as proposed in [13][14]. If carrier phase measurements are reported together with a legnacy measurements, then the measurements may share the same timestamps. The details of the timestamps will depend on the conclusion of the discussion related to the timing of the carrier phase measurements in Section 2.

| Company | comments |
| --- | --- |
| LGE | As we mentioned in section 2.4, details of the time stamp need to be discussed in more detail level. |

# Measurement procedures

Submitted Proposals

| CATT[8] | Proposal 12: For both UE-based and UE-assisted NR DL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both with and without measurement gap scenarios. - FFS: the details of the enhancements. Proposal 13: From RAN1’s perspective, existing higher layer procedures for NR positioning can be enhanced to support both UE-based and UE-assisted NR carrier phase positioning in RRC_CONNECTED and RRC_INACTIVE states for both with and without measurement gap scenarios. - The details of the enhancements can be decided by RAN2/RAN3. Proposal 23: For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP). - FFS: the details of the enhancements. Proposal 24: From RAN1’s perspective, existing higher layer procedures for NR UL positioning can be enhanced to support NR UL carrier phase positioning in RRC_CONNECTED and RRC_INACTIVE states. - The details of the enhancements can be decided by RAN2/RAN3. |
| --- | --- |

## DL carrier measurement procedures

With the reuse of the existing Rel-16 DL PRS and SRS for positioning for NR carrier phase positioning, the NR carrier phase measurements can be requested, measured and reported together with the existing positioning measurements. Thus, existing physical layer procedures for current positioning methods can be re-used and enhanced for supporting the NR carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, including the measurement configuration, measurement request and measurement report with potential enhancements of the message content (e.g., the configuration related to the NR carrier phase positioning) for both with and without measurement gap scenarios.

(H)(Round 1)Proposal 9.1-1

- For both UE-based and UE-assisted NR DL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both with and without measurement gap scenarios.

  - FFS: the details of the enhancements.

- From RAN1’s perspective, existing higher layer procedures for NR positioning can be enhanced to support both UE-based and UE-assisted NR carrier phase positioning in RRC_CONNECTED and RRC_INACTIVE states for both with and without measurement gap scenarios.

  - The details of the enhancements, if any, can be decided by RAN2/RAN3.

- Send LS to RAN2/RAN3 on RAN1’s decision, and ask RAN2/RAN3 if there is any concern or feedback.

| Company | comments |
| --- | --- |
| Vivo | For the second bullet, the high layer signaling should be enhanced for carrier phase positioning other than high layer procedure. |
| Huawei, HiSilicon | When it comes MG v.s. PPW, we think that RAN4 should be consulted as well. Considering we are expanding the scope, RAN-P should also be informed, but we can wait for the LS to RAN-P after May. FL: My understanding is that we do not expand the scope. WID makes it clear that the limitation of MG is for RAN4 RRM requirements. |
| CATT | Support. |
| Qualcomm | Prefer to only consider the case with measurement gaps, because UE may find it difficult to maintain phase coherence without a gap FL: Maybe Qualcomm can explain what the “phase coherence” refres t, and why “UE may find it difficult to maintain phase coherence without a gap”? We are working on single PFL/carrier in Rel-18. We may also say “at least for with measurement gap scenarios” |
| Intel | Same view as QC. |
| LGE | We are fine with the proposal. |
| Nokia/NSB | Okay with the proposal but not sure how much dedicated work in RAN1 needs to happen for MG vs PPW. This may be up to other groups. |

(H)(Round 2)Proposal 9.1-1

- For both UE-based and UE-assisted NR DL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) at least for with measurement gap scenarios.

  - FFS: the details of the enhancements.

- From RAN1’s perspective, existing higher layer procedures for NR positioning can be enhanced to support both UE-based and UE-assisted NR carrier phase positioning in RRC_CONNECTED and RRC_INACTIVE states at least for both with and without measurement gap scenarios.

  - The details of the enhancements, if any, can be decided by RAN2/RAN3.

- Send LS to RAN2/RAN3 on RAN1’s decision, and ask RAN2/RAN3 if there is any concern or feedback.

| Company | comments |
| --- | --- |
| Huawei, HiSilicon | On the main bullet, do we have measurement gap in RRC_INACTIVE state? FL: Maybe we can separate it into two bullets. |
| Vivo | We are not sure that the second bullet is needed, it can be decided by RAN2. FL: Okay. Let us remove it for now. |
| CATT | Support |
| Samsung2 | OK in principle. |

### (H)(Round 3)Proposal 9.1-1

- Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including

  - UE in RRC_CONNECTED state with measurement gap.

  - FFS: UE in RRC_CONNECTED state without measurement gap

  - UE in RRC_ INACTIVE state

- FFS: the details of the enhancements.

| Company | comments |
| --- | --- |
| Nokia/NSB | We prefer to remove the FFS on PPW. In our view PPW won’t add any additional work to RAN1 so it should be supported from our perspective. |
| Huawei, HiSilicon | OK |
| Qualcomm | Prefer to keep the case without measurement gap as FFS. Note that based on latest WID, RAN4 will only define requirements for the case with measurement gaps. In response to FL question on QC comment in round1, the issue is that within a PPW, there may be other data activity, including UL-DL switching, which may potentially introduce random phase jumps in the UE Rx chain. FL: Okay. Let us put back “FFS” for now |
| LGE | Support, and also fine with removing FFS in the sub-bullet regarding PPW |
| ZTE | Generally ok for this proposal. |
| Samsung3 | OK in principle |
| Xiaomi | Support |
| CATT | Support |

## UL carrier measurement procedures

Similar to DL carrier measurement procedures, it is expected the existing physical layer procedures for current positioning methods can be re-used and enhanced for supporting the NR carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, including the measurement configuration, measurement request and measurement report with potential enhancements of the message content (e.g., the configuration related to the NR carrier phase positioning) for both with and without UL SRS Tx switching.

### (H)(Round 1)Proposal 9.2-1 (Revised)

- For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP).

  - FFS: the details of the enhancements.

- From RAN1’s perspective, existing higher layer procedures for NR UL positioning can be enhanced to support NR UL carrier phase positioning in RRC_CONNECTED and RRC_INACTIVE states.

  - The details of the enhancements, if any, can be decided by RAN2/RAN3.

- Send LS to RAN2/RAN3 on RAN1’s decision, and ask RAN2/RAN3 if there is any concern or feedback.

| Company | comments |
| --- | --- |
| CATT | Support |
| Nokia/NSB | Same as for 9.1-1 FL: I haven’t seen any comment from Nokia for 9.1-1. |
| Huawei, HiSilicon | Support |
| Spreadtrum | Support |
| CATT | Support |
| Samsung2 | OK in principle |
| Nokia/NSB2 | We don’t feel the 2nd bullet is needed and no need to send an LS. FL: Okay. Let us remove it for now as Propsal 9.2.-1 ### (H)(Round 1)Proposal 9.2-1 (Revised) - For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP). - FFS: the details of the enhancements. |
| Vivo | Okay to remove the second and third bullet |
| LGE | Support |
| CATT | Support |

# UE CPP Capability

Submitted Proposals:

| Xiaomi[11] | - Proposal 2: Support UE to report the supported minimum number of symbols for PRS and the supported largest periodicity for PRS. |
| --- | --- |
| Apple [18] | Proposal 1: RAN1 should discuss the capabilities of the UE, TRP and LMF for CPP. This could be one of the following: - UE Capabilities for Carrier Phase positioning (CPP) i.e. can the UE perform standalone CPP or joint Rel-17 positioning with CPP ? - UE Capabilities for Carrier Phase Measurement and reporting i.e. can the UE perform a single frequency measurement or multiple frequency measurements within a PFL for CPP reporting - PRU capability and configuration for double differential CPP i.e. can a UE serve as a PRU for CPP based on the accuracy of its position and measurement capabilities ? Proposal 3: for the UE’s positioning processing capability, the “Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE” should be updated especially in the case of joint traditional positioning with CPP due to the additional measurements needed. |
| IIT Kanpur, CEWiT[24] | Proposal 11: For NR carrier phase positioning, the UE information should include the following. - Frequency range supported by the UE - Frequency granularity supported by the UE to enable carrier phase measurement - DC carrier - Subcarrier - Positioning frequency level - Measurement reference point granularity supported by the UE - Single measurement reference point supported - Multiple measurement reference points supported - Per Rx antenna - Per receiver RF chain - Group of Rx chain/antenna - The supported integer ambiguity resolution method (UE-based CPP) - Virtual carrier method - Psudo range method (in the combination of existing measurements) - Whether the measuring UE supports LoS/NLoS identification (UE-based CPP) - The PRS/SRS configuration supported by the UE |

FL Comments:

NR CPP requires introducing new UE capabilities. These capabilities may include determining the minimum number of PRS symbols supported, the maximum PRS periodicity, the need for one or multiple frequency measurements within a PFL, whether to support standalone CPP, whether it can be a PRU, as pointed out in [10, 18, 24]. Since the discussion of the UE capabilities depends heavily on which features and functionarity are introduced for CPP, the suggestion of FL for this meeting is still focusing on the features and functionarity to introduced for CPP. Nevertheless, it may be beneficial to gather feedback from companies on above proposals.

### (Round 1) Q&A 10-1

- What is your view for the introducing the following UE capabilities for carrier phase reporting/positioning in Rel-18?

  - Carrier phase positioning (CPP) capability

    - Where a UE supports joint CPP with Rel-17 positioning methods or standalone CPP, if standalone CPP is introduced in Rel-18

    - Where a UE supports CPP for RRC_CONNECTED state only, or also supports CPP for RRC_INNECTIVE state

    - Where a UE supports UE-based CPP, or UE-assisted CPP, or both

  - Carrier phase measurements (CPM) reporting

    - Whether a UE supports reporting carrier phase measurements of multiple RF multiple frequencies of multiple sub-PFL(sub-bandwidths) within a PFL for CPP reporting, if the feature is introduced in Rel-18

    - Whether a UE supports reporting carrier phase measurements of RF multiple frequencies of specific subcarriers, if the feature is introduced in Rel-18

  - Other UE reporting capabilities

    - Capabily related to integer ambiguity resolution, such as the search range, if the feature is introduced in Rel-18

    - Capability related to maltipth mitigations, such as reporting CPP for additional paths, if the feature is introduced in Rel-18

    - Capabilities related to PEGs, if the feature is introduced in Rel-18

  - UE DL PRS processing capabilities

    - UE DL PRS processing capabilities for legacy timing measurements can be used as the starting point

  - UE UL SRS transmission capabilities

    - Maybe no change is needed.

| Company | comments |
| --- | --- |
| Huawei, HiSilicon | We prefer to follow the framework of path RSRPP reporting capability introduced in Rel-17 for CPP if CPP is not introduced as a standalone method. |
| Nokia/NSB | Low priority. This can be discussed in future meetings after the overall design has made more progress. |
| CATT | Low priority for this meeting. |
| Samsung2 | Can be discussed after we make progress on other design aspects. |
| FL | From the feedback, it seems the issue should be a low priority in this meeting. |

# Round trip carrier phase positioning

Submitted Proposals:

| Nokia[11] | Proposal 9: RAN1 should support solutions to ensure the same local oscillator is used between DL and UL (at UE/TRP) and that the coherence time of the oscillator drift is met. |
| --- | --- |
| Samsung[12] | Proposal 10: For carrier phase positioning measurement, RAN1 considers providing measurements for round-trip carrier phase measurement. |

FL Comments:

During the Rel-18 SI, the effectiveness of the round-trip carrier phase technique was assessed under the assumption of the phase coherency between the transmit and receive paths of each device as shown in TR 38.859 [26]. For this meeting, proposals were made in [11, 12] to explore the solutions further. For example, it suggests employing the same local oscillator between DL and UL to maintain phase coherence in [11]. it suggests employing the same local oscillator between DL and UL to maintain phase coherence. Interested companies are invited other to provide their views on the above proposals from [11, 12].

### (Round 1) Q&A 11-1 What is your views on the following proposals from [11, 12]?

- (Nokia) Proposal 9: RAN1 should support solutions to ensure the same local oscillator is used between DL and UL (at UE/TRP) and that the coherence time of the oscillator drift is met.

- (Samsung) Proposal 10: For carrier phase positioning measurement, RAN1 considers providing measurements for round-trip carrier phase measurement.

| Company | comments |
| --- | --- |
| Samsung | Support both proposals. |
| Nokia/NSB | Support. |
| Locaila | Support. For your reference, we proposed a solution for phase-based RTT in R1-2303775. It can be a starting point for initial discussion. |
| FL | We have three companies support the proposes. But, not companies has objections. In this case, let us raise the priority for the discussion. Due to the fact that only three companies are interested on this issue, let us rasie the the priority to medium |

### (M)(Round 2)Proposal 11-1

- RAN1 should support solutions to ensure the same local oscillator is used between DL and UL (at UE/TRP) and that the coherence time of the oscillator drift is met.

| Company | comments |
| --- | --- |
| Nokia/NSB | Support |
| Samsung3 | Support |

### (M)(Round 2)Proposal 11-2

- For carrier phase positioning measurement, RAN1 considers providing measurements for round-trip carrier phase measurement.

| Company | comments |
| --- | --- |
| Nokia/NSB | We are supportive of the proposal but in our understanding if DL RSCP is supported then RTT phase is possible by implementation. Maybe Samsung could clarify if additional things are needed to support RTT phase? |
| Locaila | Support. Seeing the current on-going discussion in SL-RTT, there’s a lot issue to discuss. |
| Samsung3 | Support In reply to Nokia, yes supporting DL RSCP (and UL RSCP) is one aspect. Another aspect is the slection of the reference time used for phase measurement in the UE and TRP, so that when the two phases (UL and DL) are added, the result is the round-trip phase. |

# TRP time synchronization

Submitted Proposals:

| Locaila[23] | (Proposal 1) Network notification should be delivered to UE when a separate means of PRS phase synchronization is provided, so that the UE does not need to wait for additional initial phase error correction information. (Proposal 2) At least in the TDD system, a method that can calculate the relative clock phase offset between TRPs by listening to the PRS signal transmitted between TRPs must be supported. FFS: How to harmonize the PRS muting procedure and the PRS listening procedure (Proposal 3) RAN1 should study the benefit and efficiency of the proposed TRP synchronization method, and also application of the curl-vector for removing initial phase offset errors between TRPs. |
| --- | --- |

FL Comments:

For precise positioning techniques like DL-TOA, UL-TDOA, and carrier phase positioning, timing synchronization is crucial. A possible approach for gNB synchronization was proposed using PRS signals in [23]. However, the discussion on how to implement inter-TRP time synchronization may exceed the scope of this WI. Interested companies are welcome to share their views on the above proposals from [23]. Interested companies are invited other to provide their views on the above proposals from [23].

### (Round 1) Q&A 12-1 What is your views on the following proposals from [23]?

- (Locaila ) Proposal 1. Network notification should be delivered to UE when a separate means of PRS phase synchronization is provided, so that the UE does not need to wait for additional initial phase error correction information.

- (Locaila ) Proposal 2. At least in the TDD system, a method that can calculate the relative clock phase offset between TRPs by listening to the PRS signal transmitted between TRPs must be supported.

    - FFS: How to harmonize the PRS muting procedure and the PRS listening procedure

- (Locaila ) Proposal 3. RAN1 should study the benefit and efficiency of the proposed TRP synchronization method, and also application of the curl-vector for removing initial phase offset errors between TRPs.

| Company | comments |
| --- | --- |
| Nokia/NSB | Do not support. |
| Locaila | Support This is an efficient alternative to the double-difference method, and also it is the proven method tested in our field experiments. The 5G double-difference method currently being discussed in RAN1 is already complex and inefficient than GPS. We can't win GPS with such copied idea. Proposed TRP synchronization method is more suitable for 5G. It is better to correct the initial phase error at the network side rather than pushing the burden to UE. No-timestamping, no-simultaneous measurement, no-assistant data, no-LMF/LPP signaling. It is effective method particularly for vehicles where fast and accurate positioning is the critical issue. We hope companies support this method that we can continue this work in RAN1 and produce competitive result than GPS-carrier phase. |
| CATT | Out of the WI scope in nour view. |
| Locaila | On what ground in WI scope ? we think it is in scope. |
| InterDigital | Generally supportive of the proposals. We can consider the proposals to study how to minimize the effect of error sources for NR CP based positioning. |
| Samsung2 | We are supportive of these proposals. In scenarios where synchrionization between TRPs can be achived, this would benefit carrier phase measurements. We think that this is within the scope of the WID as the WID includes signaling for carrier phase measurements. For example, there can be a signal from the network to indicate when the TRPs are synchronized to benefit carrier phase measurements. |
| FL | We have 5 companies provide the feedback this time and three companies are supportive of the proposals. Let us raise the priority level for further discussion. Actually, (Locaila ) Proposal 1 can be seen as one additional option for FL Proposal 4.2-1, and (Locaila ) Proposal 2 is already covered in the second option in FL Proposal 4.2-1. |

### (M)(Round 2)Proposal 12-1

- RAN1 should study the benefit and efficiency of the proposed TRP synchronization method, and also application of the curl-vector for removing initial phase offset errors between TRPs.

| Company | comments |
| --- | --- |
| Nokia/NSB | Do not support. |
| Locaila | Thank you for the companies’ support on this work. We believe this function will greatly simplify the solution for ‘initial TRP phase offset error’ with only minor impact to NR system, and also reduces the potential burden in both UE and network side. In particular for V2X use-cases where fast and accurate UE-based positioning is required, it will definitely be the effective solution. We will bring more detailed proposal from next meeting. |
| Samsung3 | Support |
| Locaila | Support, Here’s some motivation & justification remark on the proposed short study of 12-1 ------------------------------------------------------------------------------------------------------------ During the past SI and WI discussion, we found that initial phase offset errors caused by the oscillator drift of gNB/TRPs had a significant impact on carrier phase measurements, but we do not know how much it will affect the accuracy. We should study effect of the oscillator quality on UL/DL carrier phase positioning. Further, aspects on TRP synchronization method using PRS signal needs to be studied as a solution. System impact and benefit to carrier phase positioning should be discussed. Therefore, we propose some modification on the above 12-1 text as below - RAN1 should study the benefit and efficiency of the proposed TRP synchronization method, and also application of the curl-vector for removing initial phase offset errors between TRPs. - FFS : Measurement of the clock timing difference of TRPs, aspects related to monitoring PRS phases between TRPs, and adjustment of PRS signal phases at the TX baseband using the curl-vector |

# (Closed) Carrier Phase in DC subcarrier

Submitted Proposals:

| Nokia[11] | Proposal 10: RAN1 should further investigate the carrier phase measurement impairment in the case that the configured PRS resources includes the DC subcarriers. . |
| --- | --- |

FL Comments:

In [11], it points out that local oscillator leakage may potentially cause interference and negatively impact the accuracy of carrier phase estimation in DC carrier. According to the FL's understanding, receivers typically can use signals received from all PRS subcarriers in a PFL to estimate the carrier phase of a particular subcarrier, including the DC carrier, rather than relying solely on the signals from that subcarrier. Therefore, it is up to the UE's implementation to determine whether to utilize the signals received in the DC subcarrier for carrier phase estimation if it wants to report the carrier phase of the DC carrier. Interested companies are invited other to provide their views on the above proposal from [11].

(Round 1) Q&A 13-1 What is your views on the following proposal from [11]?

- (Nokia) Proposal 10. RAN1 should further investigate the carrier phase measurement impairment in the case that the configured PRS resources includes the DC subcarriers.

| Company | comments |
| --- | --- |
| FL | No company provide feedback. Thus, the FL suggest closing the discussion of the issue. |

# FL Proposals for 2nd GTW Session

### (H)(Round 3) Proposal 2.2-1

Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection in RAN1#113).

- Option 1:

  - The reference point of the UE carrier phase measurements is defined the same as the the reference point of RSTD for frequency range 1 and  frequency range 2.

  - The reference point of the TRP carrier phase measurements is defined the same as the the reference point of RTOA for frequency range 1 and  frequency range 2.

  - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting.

- Option 2:

  - The reference point of the UE/TRP carrier phase measurements is defined as the antenna phase center of the UE/TRP Rx antenna for frequency range 1 and frequency range 2.

  - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF

  - FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame

- If any of the options is agreed, RAN1 needs to send an LS to RAN4 to confirm RAN1’s decision.

### (H)(Round 3) Proposal 4.1-1 (Revised 1)

- To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements:

  - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resources occurring within indicated time window(s).

    - Note: The UE is expected to make its best effort to comply with the request from LMF, but it is not mandatory.

  - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time.

- Send LS to RAN2/RAN3/SA2 on RAN1’s decision

### (H)(Round 3) Proposal 4.1-2 (Revision 1)

- To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements:

  - Enabling LMF to request the serving gNB of a UE to configure the transmission of the UL SRS resources from the UE within indicated time window(s).

    - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time

  - FFS: Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the UL SRS resources from the UE within indicated time window(s).

- Send LS to RAN2/RAN3/SA2 on RAN1’s decision

### H)(Round 3) Proposal 4.2-1 (Revision 1)

- For UE-based carrier phase positioning, consider supporting one or both of the following options:

  - Option 1: Enabling LMF to forward the DL carrier phase measurement reported by a PRU and other PRU information to a target UE for UE-based carrier phase positioning in the positioning assistance data.

    - Note: Whether DL carrier phase measurement is DL RSCP and/or DL RSCPD depends on which of them is (are) supported.

    - FFS: other PRU information, such as PRU location, to be included in the positioning assistance data

  - Option 2: Enabling LMF to provide, optionally, the differential corrections to a target UE for UE-based carrier phase positioning.

    - FFS: the details of the correction information, which may include the corrections for TRP timing errors, TRP phase errors, ARP/APC errors, etc.

  - Other options are not precluded.

# FL Proposals recommended for email endorsement (1st Checkpoint, April 21)

### (H)(Round 3) Proposal 2.3-1

·          The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default.

o    Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them.

| COMPANY | COMMENTS |
| --- | --- |
| FL | Received on comment from LGE. They “Fine with the proposal.” But suggest removing “), if RAN1 agrees to introduce them”. The FL provided the response, and hopefully it can be accepted. |

### (H)(Round 3) Proposal 2.3-3 (Revision 1)

·          The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center frequency of the UL carrier that contains transmission bandwidth of the SRS for positioning purpose.

o    Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them.

| COMPANY | COMMENTS |
| --- | --- |
| FL | All feedback so far are fine with it (The FL corrected the mistake in copy and paste, which was pointed out by multiple companies.) |

### (H)(Round 3) Proposal 3.1-1 (no update in comparison with Round 2)

- Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF

- Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference.

- Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting.

| COMPANY | COMMENTS |
| --- | --- |
| FL | LGE made a commend. They are “Generally ok,” but propose add a note. The FL provided the response, and hopefully it can be accepted. |

### (H)(Round 3) Proposal 8.1-1

- Further study whether and how to support a UE/TRP to report the carrier phase measurement quality indication for corresponding the phase measurements.

| COMPANY | COMMENTS |
| --- | --- |
| FL | No negative comment so far |

### (H)(Round 3)Proposal 9.1-1

•      Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including

•      UE in RRC_CONNECTED state with measurement gap.

•      FFS: UE in RRC_CONNECTED state without measurement gap

•      UE in RRC_ INACTIVE state

•      FFS: the details of the enhancements.

| COMPANY | COMMENTS |
| --- | --- |
| FL | “FFS:” is added back to the subbullet “UE in RRC_CONNECTED…”. I am hoping this can be accepted to all companies that we will further discuss the issue of UE in RRC_CONNECTED state without measurement gap. |

### (H)(Round 1)Proposal 9.2-1 (Revised)

- For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP).

  - FFS: the details of the enhancements.

- From RAN1’s perspective, existing higher layer procedures for NR UL positioning can be enhanced to support NR UL carrier phase positioning in RRC_CONNECTED and RRC_INACTIVE states.

  - The details of the enhancements, if any, can be decided by RAN2/RAN3.

•      Send LS to RAN2/RAN3 on RAN1’s decision, and ask RAN2/RAN3 if there is any concern or feedback.

| COMPANY | COMMENTS |
| --- | --- |
| FL | Nokia made a suggestion to remove the last two bullets, since to Proposal 9.1-1. I assume the suggestion can be acceptable for all companies. |

# References

- 3GPP RP-223549 New WID on Expanded and Improved NR Positioning Intel Corporation, CATT, Ericsson

- 3GPP RAN1#112 Chairman’s Notes.

- R1-2302380	Measurement and reporting for NR carrier phase positioning	Huawei, HiSilicon

- R1-2302493	Discussion on carrier phase positioning	vivo

- R1-2302524	Discussions on Carrier Phase Measurement for NR Positioning	BUPT

- R1-2302556	Discussions on NR carrier phase positioning	OPPO

- R1-2302608	Discussion on NR DL and UL carrier phase positioning	Spreadtrum Communications

- R1-2302711	Further discussion on NR DL and UL carrier phase positioning	CATT

- R1-2302804	On DL and UL carrier phase positioning	Intel Corporation

- R1-2302934	Views on NR DL and UL carrier phase positioning	Nokia, Nokia Shanghai Bell

- R1-2302991	NR DL and UL carrier phase positioning	xiaomi

- R1-2303136	On NR DL and UL Carrier Phase Positioning	Samsung

- R1-2303242	Discussion on DL/UL carrier phase positioning	CMCC

- R1-2303266	DL & UL Carrier Phase Positioning Discussion	Lenovo

- R1-2303279	Discussion on carrier phase positioning	ZTE

- R1-2303882	NR carrier phase measurements for positioning	Fraunhofer IIS, Fraunhofer HHI

- R1-2303446	Discussion on positioning based on NR carrier phase measurement	InterDigital, Inc.

- R1-2303491	On NR DL and UL carrier phase positioning	Apple

- R1-2303553	NR DL and UL carrier phase positioning	Ericsson

- R1-2303598	NR Carrier Phase Positioning	Qualcomm Incorporated

- R1-2303717	Discussion on NR DL and UL carrier phase positioning	NTT DOCOMO, INC.

- R1-2303944	Discussion on carrier phase positioning in NR	LG Electronics

- R1-2303774	Discussion on integer number resolution	Locaila

- R1-2303821	Discussion on NR UL and DL carrier phase positioning 	IIT Kanpur, CEWiT

- R1-2303841	Solutions for carrier phase positioning		MediaTek (Chengdu) Inc.

- R1-2301830	FL Summary #3 for NR DL and UL carrier phase positioning, CATT

- 3GPP TR 38.859, Study on Expanded and Improved NR Positioning

# Appendix

## Agreements made in RAN1#112

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements. Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. Agreement Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

## Conclusions in Rel-18 SI [TR 38.859]

### 6.3.1 Potential solutions for NR carrier phase positioning #### 6.3.1.1 Reference signals for NR carrier phase positioning Existing DL PRS and UL SRS for positioning can be re-used as the reference signals to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning. Whether to consider enhancements of the existing DL PRS and UL SRS for better positioning performance can be considered further. Note that the use of MIMO SRS for positioning purpose is transparent to UE. #### 6.3.1.2 Physical layer measurements for NR carrier phase positioning The study of the accuracy improvement based on NR carrier phase measurements includes: - UE-based and UE-assisted carrier phase positioning - UL carrier phase positioning and DL carrier phase positioning - NR carrier phase positioning with the carrier phase measurements of one carrier frequency or multiple frequencies - Combination of NR carrier phase positioning with another standardized Rel. 17 positioning method, e.g., DL-TDOA, UL-TDOA, Multi-RTT, etc. For DL UE-assisted NR carrier phase positioning, at least the following options are considered: - The difference between the carrier phase measured from the DL PRS signal(s) of the target TRP and the carrier phase measured from the DL PRS signal(s) of the reference TRP; - The carrier phase measured from the DL PRS signal(s) of a TRP. For UL UE-assisted NR carrier phase positioning, at least the carrier phase measured from the UL SRS for positioning purpose is considered. #### 6.3.1.3 Physical layer procedures for NR carrier phase positioning The impact of integer ambiguity on NR carrier phase positioning and potential solutions to resolve the integer ambiguity when using carrier phase measurements to estimate the propagation delay/distance between transmitting and receiving nodes are studied. Benefits of using the carrier phase measurements of multiple DL PFLs for NR carrier phase positioning, which may include the impact of the time gap between the carrier phase measurements of multiple DL PFLs are studied. NOTE 1: The initial phase error and the frequency error for each PFL can be modelled independently. NOTE 2: For evaluations, the PRSs of all the PFLs of a TRP can be assumed to be transmitted from the same ARP or from different ARPs of the TRP. NOTE 3: The location error for ARPs can be modelled independently. NOTE 4: The timing errors of the PFLs may not be the same for PFLs in different bands or frequency ranges. NOTE 5: In Rel-17, simultaneous reception of DL PRS from multiple frequency layers is not supported. The impact of multipath/NLOS on NR carrier phase positioning is evaluated during the study item. Based on the study, it is concluded that multipath/NLOS deteriorates the performance of carrier phase positioning, and it is necessary to consider multipath mitigation for NR carrier phase positioning. The effectiveness of the following multipath mitigation methods for NR carrier phase positioning is studied: - Identification and separation of the first path and other paths. - Reporting of the carrier phase of the first path, and optionally, the additional paths. - The use of LOS/NLOS indication for the carrier phase measurements. - NOTE: Rel-17 LOS/NLOS indicator can be considered as a starting point. - The report of other channel information, such as existing RSRP/RSRPP. The use of Positioning Reference Unit (PRU) to facilitate NR carrier phase positioning is studied. - For DL NR carrier phase positioning, a PRU works as a UE to receive the DL PRS reference signals and provide the DL carrier phase measurements to the LMF, where the double differential measurements can be obtained by the difference of the DL carrier phase measurements from the target UE and those from the PRU for eliminating the measurement errors. - For UL NR carrier phase positioning, a PRU works as a UE to transmit the UL SRS signals for positioning purpose. The TRPs provide the UL carrier phase measurements obtained from the UL SRS signals of the target UE and of the PRU to the LMF, where the double differential measurements can be obtained by the difference of these UL carrier phase measurements for eliminating the measurement errors. The following approaches for NR carrier phase positioning are studied: - The reporting of the carrier phase measurements together with the existing positioning measurements. - The reporting of the carrier phase-based measurements alone without reporting the existing positioning measurements. Potential solutions for NR carrier phase positioning are evaluated with the consideration of various error sources, which include phase noise (FR2), carrier frequency offset (CFO)/Doppler, oscillator-drift, transmitter/receiver antenna reference point (ARP) location errors, transmitter/receiver initial phase error, antenna Phase Center Offset (PCO) etc. Detailed evaluation methodology and assumptions are presented in Annex A.3 A summary of the evaluation results for the impact of the multipath/NLOS on NR carrier phase positioning are presented in Clause 6.3.2. NR carrier phase positioning performance is evaluated at least with the carrier phase measurements of a single measurement instance. It should be noted that the use of "carrier phase positioning" does not necessarily imply that it may be defined as a standalone positioning method. The potential solutions of integer ambiguity resolution for NR carrier phase positioning were investigated in the study item, which include the following: - Reporting of the carrier phases of more than one frequency from UE/TRP to LMF - NOTE: frequency refers to frequency of carrier or frequency of subcarrier(s) - Reporting of the determined integer ambiguity and/or the search range of the integer ambiguity from UE/TRP to LMF - Reporting of the carrier phase measurements together with the legacy positioning measurements from UE/TRP to LMF - Reporting of the new measurements from UE /TRP to LMF, e.g., based on carrier phase differentials across multiple subcarriers within a carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be equivalent to time of arrival - LMF configure the integer ambiguity range between the TRP and target UE (for UE-based NR CPP). ### 6.3.3 Potential specification impact for NR carrier phase positioning Regarding the reference signals for NR carrier phase positioning: - Existing DL PRS and UL SRS for positioning purpose are recommended as the reference signals to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning if NR CPP is introduced. NOTE: The use of SRS MIMO for NR carrier phase positioning is transparent for UE. Regarding the physical layer measurements for NR carrier phase positioning: - New measurements are recommended to be introduced for supporting UE-based and UE-assisted NR carrier phase positioning, if NR CPP is introduced. The new measurements include, at least, the following: - For DL carrier phase positioning, the following candidate measurements are identified (potential down-selection may be considered during normative work). - The difference between the carrier phase measured from the DL PRS signal(s) of the target TRP and the carrier phase measured from the DL PRS signal(s) of the reference TRP; - The carrier phase measured from the DL PRS signal(s) of a TRP. - For UL carrier phase positioning, the carrier phases measured from the UL SRS for positioning purpose is identified as the UL carrier phase measurements. NOTE: This proposal does not imply which carrier phase measurements are mapped to which positioning technique. Multipath mitigation methods for the carrier phase positioning are recommended to be introduced during normative work, if NR CPP is introduced. The candidate solutions may include, but are not limited to, the following: - Reporting of the carrier phase of the first path - At least reporting of the carrier phase of the first path, and optionally, the additional paths. - The use of LOS/NLOS indication for the carrier phase measurements. - NOTE: Rel-17 LOS/NLOS indicator can be considered as a starting point. - Reporting of other channel information together with carrier phase measurements, such as existing RSRP/RSRPP. At least the double differential technique with PRU is feasible for UE-based, and UE-assisted NR carrier phase positioning, if NR CPP is introduced, at least, for eliminating the impact of the initial phases of the transmitter and the receiver. NOTE 1: How to efficiently enable the use of the PRU for supporting NR double differential carrier phase positioning needs further discussion during normative work. NOTE 2: The required PRU density also needs further discussion during normative work. NOTE 3: Other methods for eliminating the impact of the initial phases of the transmitter and the receiver are not precluded. ## 17.6 NR carrier phase positioning Based on the study, it is concluded that it is feasible to use existing DL PRS and SRS signals to obtain the carrier phase measurements for achieving a horizontal accuracy of up to a few centimeters at least at 50% under certain conditions, including the PRU(s) being located in LOS with TRP(s), and the locations of the PRU(s) and TRPs known with centimeter-level accuracy, in the agreed evaluation assumptions. If NR CPP is introduced, - Existing DL PRS and UL SRS for positioning purpose are recommended as the reference signals to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning. - New measurements are recommended to be introduced for supporting UE-based and UE-assisted NR carrier phase positioning. - Multipath mitigation methods for the carrier phase positioning are recommended to be introduced during normative work.